3GPP TSG-RAN2#94	TDoc R2-164505

Nanjing, P.R. China, May 23 - 27, 2016

Agenda Item:		7.16.2

Source:	Ericsson (Rapporteur)

Title:					Review issue list for ASN.1 freeze

Version:				1.1

Document for:		Discussion and decision

# Introduction

This document provides an overview of list of issues resulting from the review of the PDU specification including their status. For some issues the proposed solution is indicated as well as the company & TDoc introducing this in the standard. For some of the issues this document includes further considerations. The following companies volunteered for the review,

| Companies | Contact persons | Email |
| --- | --- | --- |
| CATT | Xing Yang Fangli Xu | yangxing@catt.cn xufangli@catt.cn |
| Ericsson (review coordinator) | Kai-Erik Sunell Riikka Susitaival Mats Folke | kai-erik.sunell@ericsson.com riikka.susitaival@ericsson.com mats.folke@ericsson.com |
| Huawei | Yi Guo | yi.guo@huawei.com |
| Intel | Hyung-Nam Choi Marta Tarradell | hyung-nam.choi@intel.com marta.m.tarradell@intel.com |
| LG Electronics Inc | SungHoon Jung | sunghoon.jung@lge.com |
| Nokia | Tero Henttonen Jarkko Koskela Tomala Malgorzata | tero.henttonen@nokia.com jarkko.t.koskela@nokia.com malgorzata.tomala@nokia.com |
| NTT DOCOMO | Hideaki Takahashi | hideaki.takahashi.vx@nttdocomo.com |
| Qualcomm | Kitazoe Masato | mkitazoe@qti.qualcomm.com |
| Samsung | Himke Van der Velde | himke.vandervelde@samsung.com |
| ZTE | Sergio Parolari | sergio.parolari@zte.com.cn |

# Discussion

<Moved to separate discussion paper>

# Conclusion & recommendation

This paper includes an overview of list of issues resulting from the review of the PDU specification. RAN2 is requested to endorse the status including the solutions proposed.

# References

[1] 	TS 36.331 E-UTRA RRC specification

# Review issue list (Annex)

Classification: 1: straigthforward clarification/ correction that can be included in next rapporteurs update, 2: small issue i.e. solution expected to be concluded easily e.g. by e-mail, 3: more significant issue i.e. requiring further discussion/ contributions during review meeting, 4: more significant issue i.e. requiring further discussion/ contributions during next RAN2 working group meeting. Abbreviations used: TBD (to be done), TBC (to be confirmed/ concluded)

| No | Clause(s) | Description | Class |  | Details (proposed solution/ discussion) |  | Status/ ref |
| --- | --- | --- | --- | --- | --- | --- | --- |
| General |  |  |  |  |  |  |  |
| N.002 | Many | The indentations look dissimilar in many places, could unify how to make them. | 1 |  | Unify indentation once other review has been finalized. Ericsson: Agree. Coordinator: This should be done when all other corrections are ready. Also indentation of legacy ASN.1 should be unified. Coordinator: It is better to do this during v1310 CR implementation because there are multiple CRs and indentation does not have an y impact on the protocol. Nokia: Nothing done for now, agree with coordinator to come back to this in v1310. Coordinator: This can also be fixed after freezing because there are so many CRs. Coordinator: Fix this issues after completion of NB-IoT work item e.g. during CR implementation. Closed for now. |  | Closed (General CR) |
| N.004 | Several | Do we define spares where they can be defined, or leave them out? Different IEs have different handling for these, would be nice to have a general rule about that. | 3 |  | Discuss whether we need more rules about defining spares when possible. Samsung: Would be good to be (more) consistent. Note that undefined code point has same/ similar handling but we never seem to use them later Ericsson: During the previous ASN.1 review it was concluded that spare values should be defined for system information broadcast but there are no obvious reasons to define them for dedicated signaling. Perhaps this should be captured in the guidelines. ZTE: Not sure if the current using of spare has any ambiguity or problem. Chair: => For DL dedicated signalling we do not define spares (as it is anyway possible to define not used code points). => For system information do define spares if there is defines error handling for reception of spare values (e.g. doesn't make sense for a mandatory field). => Exception handling for UL (common and dedicated signalling) will be discussed at RAN2#93, in context of new establishment cause issue. At least Nokia will submit a contribution. Coordinator: See TDocs R2-160011 and R2-160040. Can be closed when exception handling for UL is discussed at RAN2#93. Coordinator: Can be closed based on RAN2#93 agreements |  | Closed (ASN.1) TDoc Nokia ASN.1 AH |
| H.002 | 5.1.1 (SC-PTM) | SC-PTM section is not listed | 2 |  | Add "subclause 5.8a covers SC-PTM (i.e. MBMS reception via SC-MRB)" Samsung: Should do same for other new main sections Nokia: if we assume that 5.8a is a subset of 5.8, no changes needed. If changes are deemed necessary, then it should be “sub-clauses 5.8 and 5.8a cover MBMS”. Huawei: If a section is numbered e.g. 1.2.3 and there is also a 1.2.4 already, using 1.2.3a is the way to insert a new section between 1.2.3 and 1.2.4, i.e. 1.2.3a is not a subsection of 1.2.3. Nokia: Seems to require discussion whether 5.8a is sub-section of 5.8 or not – this is a general discussion and not related to SC-PTM in particular. Coordinator: TR 21801 “Specification drafting rules”, clause 5.2, describes numbering of divisions and subdivisions. If continuous numbering cannot be maintained, a new element may be inserted in existing text using an appropriate alphanumeric designation that does not disturb the existing numbering scheme. This applies to all elements (e.g. clause, subclause, annex, figure, table, note, list). So, it seems that the insertion of an alphanumeric designation does not change a clause into a subclause and hence 5.8a is not a subclause of 5.8. Coordinator: Can be closed |  | Closed (General CR) |
| S.007 | measConfig | Existing RRM measurement configuration is not so extensible e.g. mandatory fields that shall be ignored. It seems desirable to consider some restructuring to simplify and unify different cases not fitting within original information structure (objects, reporting config) | 4 |  | Requires further analysis and discussion during RAN2#93 Ericsson: Agree. We should consider restructuring because limited extensibility should not become a bottleneck for the radio interface evolution. ZTE: Not sure about the benefits. As the measurement configuration may be sent before the capability enquiry, we should ensure the backward compatibility. DCM: Tend to agree with Samsung. Some extensions were not aligned with the original strucruing. Chair: => General issue of measurements extensibility can be progressed in R14 under TEI14. Coordinator: Closed. See also C.078, S.074 and S.093. |  | Closed (General CR) (ASN.1), TDoc Sam |
| S.003 | Several | In some cases RS-SINR is used in names (prefix) while in other cases SINR is used (tail) | 2 |  | Always use SINR? DCM: Agree to use SINR for simplicity although TS 36.214 defines it as RS-SINR. Chair: => Naming should be consistent with RS-SINR in all cases. Nokia: RS-SINR seems to be always used except in one condition – issue can be closed. |  | Closed (General CR) |
| Z.001 | 3.2 Abbreviations | The abbreviation of MCS needs to be added, as “MCS" is now used in section 5.8a.1.2 and serveral field description. Then the full name of MCS can be removed from the text. | 1 |  | Add abbreviation: MCS Modulation and Coding Scheme Then remove “Modulation and Coding Scheme” from the text. Ericsson: Disagree, MSC is already in 21.905. But to replace "Modulation and Coding scheme" with "MSC" is of course OK. Nokia: Replaced Modulation and Coding scheme" with "MCS" in the specs. The abbreviation has not been added (for now) to be consistent with resolution of E.028. |  | Closed (General CR) |
| E.028 | 3.2 Abbreviations | "USD" is missing from the list of abbreviations (and from 21.905) but is used in this spec. | 1 |  | Add the following to the list of abbreviations: USD User Service Description Intel: OK Nokia: it would be good to avoid duplicating definitions and acronyms and since this is defined in the Stage 2 perhaps adding 36.300 next to 21.905 would solve this. Nokia: Added the following to avoid duplication: For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 36.300 [9] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] or TS 36.300 [9]. |  | Closed (General CR) |
| S.009 | Several e.g. 5.2.2.10 | Do not use capitols when referring to names e.g. of procedures | 1 |  | E.g. perform the E-UTRAN inter-frequency redistribution procedure. Same in field description of excessDelay Nokia: Corrected text referring to procedures to use non-capitalized text in following sections: - 5.2.2.10: "E-UTRAN inter-frequency redistribution procedure“ - 5.2.2.12: "E-UTRAN inter-frequency redistribution procedure“ - 5.3.2.3: "E-UTRAN inter-frequency redistribution procedure“ - 5.3.5.3: "RRC connection re-establishment procedure", legacy text - 5.4.2.2: "handover to E-UTRA procedure“, legacy text - 5.4.3.5: "CS fallback procedure", legacy text - 5.4.5.2: " UL handover Ppreparation Ttransfer procedure", legacy text - 5.6.4.2: "1x parameter transfer procedure ", legacy text - 5.6.6.1: " UE information procedure", legacy text - 5.8.2.5: "MBMS counting procedure", legacy text - 5.8.4.1: " MBMS counting procedure", legacy text - 6.2.2: IE MobilityFromEUTRA field cs-FallBackIndicator, legacy text - 6.2.2: IE Paging field redistrivbutionIndication - 6.3.1: IE SystemInformationBlockType3 field redistrOnPagingOnly AND IE SystemInformationBlockType18 description text - 6.3.5: IE MeasResults field excessDelay, - 6.3.8: IE SL-DiscConfig field discSysInfoReportConfig - 7.3: Stop condition of T330, legacy text Annex C.1: FGI 112 descriptiom, legacy text |  | Closed (General CR) |
| S.112 | Several | Parameters regarding a particular functionality/ feature should be grouped i.e. not just have 6 individual parameters but grouped in a sequence that is absent when the feature is not used | 2 |  | Samsung: Relates to N.005 Coordinator: Same as N.005. Chair: => Examples for potential grouping in Annex should be considered offline. New issue in issue list to be added if not already present. => By default place extensions according to the logical/ functional entity rather than according to a feature/ WI i.e. protocol structure above feature. If there are more than one field related to a logical/functional entity then they may be grouped. => If within a new message/ IE a field is to be included also containing all extension introduced after its conception, this should by default be done by creating a revision of the IE defining the concerned field i.e. an IE in which all extensions are merged. => Conditions may be used in case of a condition related to the contents of a given message (rather than related to the configuration as set by previous messages) and when it is not feasible to reflect this by grouping the fields. Coodinator: See also TDocs R2-160008 and R2-160041. Grouping will be considered as a general issue for all WIs. Closed for now. |  | Closed (ASN.1) |
| S.113 | Several | Parameters (including messages, IEs and fields) should be named consistently | 2 |  | IE names should preferrably be generic merely reflect what is contained while field names may reflect particular use case Specific version of a parameter should be reflected by suffix i.e. at tail of name e.g. measResultWLAN (so that similar parameters appear together) Chair: => Naming principles should be followed whenever we add fields and IEs. Coordinator: No changes needed. Closed. |  | Closed (ASN.1) |
| S.005 | Several | Several REL-13 versions of IEs are introduced that replace earlier versions (i.e. critical extension). Can we confirm the general principle that full configuration is used to revert back to earlier versions | 2 |  | Examples include: neighCellsCRS-Info DCM: It should be analysed carefully case by case. There was a similar discussion on the REL-10 critical extension in the past. Chair: => Confirm the general principle that reverting to a earlier configuration then full configuration is used to revert back to earlier versions. Coordinator: No changes needed. Closed. |  | Closed (ASN.1) |
| N.007 | References in field descriptions missing | There are several occurrences where references are missing related to CA enhancement parameters | 3 |  | Have a contribution addingin missing/wrong field descriptions! CATT: For CA enhancement, we have given the RAN1 reference updated in our checking list. Chair: => Text proposal in R2-160007 l is agreed. Can be included in general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) TDoc Nokia ASN.1 AH |
| H.001 | 3.2 Abbreviations | QCI has been defined in 21.905, why do we need to define it again in 36.331? | 2 |  | Remove the abbreviation of QCI. Ericsson: Agree, but ideally we should treat all abbreviations in the same way. Are we sure QCI is the only one? Coordinator: Same as E.003. ZTE: Most abbrevations in 36.331 are duplications from 25.905. We feel the abbreviations provided in 36.331 can help readers by avoiding searching another specification. Nokia: Closed this since E.003 is also closed (no changes). |  | Closed |
| S.004 | Several | For fields for which an IE is defined, a field description is provided only if there is something to state about this particular case. If not, it is better to omit | 2 |  | Remove field descriptions for such cases e.g. cellSelectionSubPriority Nokia: For the particular case considered here, removed the field descriptions from SIB3, SIB5, RRCConnectionRelease and added the work “fractional” to IE description (that was the only extra information from IE but present in field descriptions), i.e. like this: The IE CellReselectionSubPriority indicates a fractional value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency/ set of frequencies (GERAN)/ bandclass (CDMA2000). Otherwise, needs checking whether there are some other cases where this would be needed. Coordinator: Can be closed. |  | Closed |
| S.006 | Several | Should not use abbreviations excessively, althoug useful to keep names short | 2 |  | If used infrequently, it is better to spell out. E.g. SSTD in field description Nokia: Related to issue S.107, which is closed for now, but SSTD itself is used in many places. Not sure whether to change all of those? Coordinator: Can be closed. |  | Closed |
| N.003 | Several | Should make new types for the following IEs that are used in several places: q-RxLevMinOffset (SIB1, SIB19, SL-SysInfoReport), q-Hyst (SIB3, SIB19, SL-SysInfoReport), discHystMax (SIB19), HystMin (SIB19) | 1 |  | Create new IE types for each of these and use them consistently. Samsung: Agree we should try to do this whenever same type is used multiple times. Even legacy field may then refer to newly introduced REL-13 IE. Intel: The respective types are only referenced few times, so we see no stringent need to define new types. Ericsson: Agree. Nokia: Closed for now since no practical examples or support was indicated. |  | Closed |
| S.008 | Several e.g. 5.2.2.10 | Use convention when referring to section in other specification i.e. list TS and indicate section within brackets | 1 |  | E.g. as specified in TS 36.304 [4, 5.2.4.z] Nokia: Closed – seems only related to D.003, D.004, N.021, D.016 in Rel-13. No changes done for Rel-12 and earlier text. |  | Closed |
| E.182 | 3.2 Abbreviations | PS is used in (at least some places) in this spec to refer to "Public Safety" but there is not such abbreviation defined hence the "Packet Swtiched" abbreviation from 21.905 will be applied. | 1 |  | Add "PS" to list of abbreviations. Ensure that "PS" is not used elsewhere in 36.331 where the meaning is "Packet Switched". If so then fix the collision. Intel: OK Coordinator: There is a collision at least in “5.6.3.3 Reception of the UECapabilityEnquiry by the UE” where GERAN PS domain is mention. So, a new abbreviation is needed for public safety. This should be addressed by ProSe CR issue H.026 because this abbreviation is ProSe related. |  | Closed |
| I.001 | 2 References | The new references [72] and [73] should be added with straight quotes and not curly quotes. | 1 |  | Replace the curly quotes with straight quotes. [72] 3GPP TS 23.179: “Functional architecture and information flows to support mission critical communication services; Stage 2”. [73] 3GPP TS 24.105: “Application specific Congestion control for Data Communication (ACDC) Management Object (MO)”. Coordinator: Same as E.215 Ericsson: Agree. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.215 | 2 References | Wrong quotation marks in references 72 and 73, [72] 3GPP TS 23.179: “Functional architecture and information flows to support mission critical communication services; Stage 2”. [73] 3GPP TS 24.105: “Application specific Congestion control for Data Communication (ACDC) Management Object (MO)”. | 1 |  | (If not already fixed in the v13.0.0 CR implementation) change curly quotations mark to straigth ones; [72] 3GPP TS 23.179: "“Functional architecture and information flows to support mission critical communication services; Stage 2” ". [73] 3GPP TS 24.105: "“Application specific Congestion control for Data Communication (ACDC) Management Object (MO)”". Coordinator: Same as I.001 |  | Closed |
| E.002 | 3.2 Abbreviations | Additional tab for LAA abreviation. | 1 |  | Remove additional tab. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.003 | 3.2 Abbreviations | QCI already defined in 21.905 | 1 |  | Remove QCI from abbreviation list in RRC. Ericsson: This is same as H.001. Coordinator: Same as H.001 |  | Closed |
| E.077 | e.g. 5.10.5, 5.10.6, 5.10.7.2, 5.10.7.4, 5.10.12.4 | “1> “ contains space instead of tab character (“1>\tab”). | 1 |  | Replace space with tab character in “1> “. This should be easy to achieve with search and replace. It also applies to other levels of indentation too. Coordinator: Corrected during v1300 CR implementation. It cannot be done with search and replace because “> “is used other parts of the specification than in procedures, e.g. some tables. |  | Closed |
| N.001 | Many | All suffixes of the form “-v13xy”, “-v13xx” or “-v13x0” should be “-v1300” instead | 1 |  | Replace all suffixes “–v13xy” with “–v1300”. Coordinator: This is normally done during CR implementation. Samsung: While ASN.1 is not yet frozen, we should use –v13xy or v13x0. At ASN.1 freeze we fix the suffix i.e. it would probably become v1320 for most WIs Intel: We can keep the suffices as they are as long as ASN.1 has not been frozen yet. When it will be done in March 2016 then they all will be replaced by “-v1310”. Ericsson: Agree with Coordinator. |  | Closed |
| 5.2 System information |  |  |  |  |  |  |  |
| C.006 | 5.2.1.3 | “Upon receiving a change notification, a UE configured with a DRX cycle that is longer than the modification period acquires the updated system information at the next H-SFN boundary defined by H-SFN mod 256 = 0” Confused expression in the highlight part, should follow more accurate syntax as legacy expression. The same comment in the reset part of the specification. | 2 |  | change to “defined by the H-SFN values for which H-SFN mod 256=0” Intel: By removing "next" the meaning of the sentence changes, so the suggested change might need to be further discussed. Coordinator: The procedure has changed so much that it is not possible to recognize where this change needs to be made. Can be closed for now and treated in a separate CR if still perceived as necessary correction." |  | Closed (General CR) |
| S.113 | 5.2.2.4 | Samsung: Details of modification period boundaries are best kept specified in 5.2.1.3 only | 2 |  | Coordinator: This is already fixed in the specification. Can be closed. |  | Closed (General CR) |
| C.007 | 5.2.1.3 | The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. If the UE receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. The highlight part should clarify the behavior is only applicable for UE configured with DRX cycle longer than modification period. | 3 |  | The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change.If the UE configured with DRX cycle that is smaller than or equal to the modification period receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. Intel: No need to do that change as the systemInfoModification would be applicable for any UE with DRX cycle longer or shorter than the BCCH modification period. The understanding is that a UE with DRX cycle longer than BCCH modification period might not have to re-acquire the SI info when this indication is received but this could be left up to UE implementation. Coordinator: Does not seem to be agreeable. Closed for now. If concerns still remain, they can be addressed during working group meetings. |  | Closed (General CR) |
| H.005 | 5.2.1.3 | Incorrect syntax (double verb) in "If the UE is configured with a DRX cycle longer than the modification period receives a Paging message including the systemInfoModification-eDRX" | 1 |  | Remove "is" Samsung: Also change to ‘a UE’ i.e. If a UE configured Nokia: Change from Samsung also implemented to general CR as follows: “If a UE configured with a DRX cycle longer than the modification period receives a Paging message including the systemInfoModification-eDRX,” |  | Closed (General CR) |
| D.003 | 5.2.2.10 Actions upon reception of SystemInformationBlockType3 | Style error of refering to the other speciication. The numbering library is used in the procedure text although in usual, style font B1, B2, etc is used. | 1 |  | Should be corrected as follows. 1> If redistributionServingInfo is included: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in TS 36.304 [4, 5.2.4.z [4]; Coordinator: The font syle error is already corrected during v1300 CR implementation. Nokia: Corrected to the general CR. |  | Closed (General CR) |
| H.022 | 5.2.2.10 and 5.2.2.12 | "No further UE requirements ... apply other than those specified elsewehere.." is strange English. | 2 |  | Reword as "Other UE requirements related to the contents of this SIB are specified elswehere, e.g." Samsung: Can consider removing the paragraph (don’t have it for any other SIB for which we specify some actions). ZTE: Not sure if the orginal text has any ambiguity. Prefer to keep the original one. Nokia Networks: Agree with Samsung. In all other SIs we have removed the whole sentence of “no UE requirements” once such have appeared. There doesn’t seem to be a point to having the text now that the SIB does contain UE requirements. Nokia: Corrected in general CR by removing the paragraphs from both SIB3 and SIB5 procedural text to align with other SIBs. |  | Closed (General CR) |
| D.004 | 5.2.2.12 Actions upon reception of SystemInformationBlockType5 | Style error of refering to the other speciication. The numbering library is used in the procedure text although in usual, style font B1, B2, etc is used. | 1 |  | Should be corrected as follows. Upon receiving SystemInformationBlockType5, the UE shall: 1> If redistributionInterFreqInfo is included: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in TS 36.304 [4, 5.2.4.z [4] Nokia: Corrected as above in the general CR |  | Closed (General CR) |
| E.007 | 5.2.1.3 | "a" has yellow highlight in the following: Upon receiving a change notification, the UE configured with a DRX cycle that is smaller than or equal to the modification period acquires the new system information immediately from the start of the next modification period. | 1 |  | Make "a" not highlighted in the following: Upon receiving a change notification, the UE configured with a DRX cycle that is smaller than or equal to the modification period acquires the new system information immediately from the start of the next modification period. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.008 | 5.2.1.3 | "is" has yellow highlight in the following: If the UE is configured with a DRX cycle longer than the modification period receives a Paging message including the systemInfoModification-eDRX, it acquires the new system information at the next H-SFN boundary defined by H-SFN mod 256 = 0. | 1 |  | Make "is" not highlighted in the following: If the UE is configured with a DRX cycle longer than the modification period receives a Paging message including the systemInfoModification-eDRX, it acquires the new system information at the next H-SFN boundary defined by H-SFN mod 256 = 0. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.078 | 5.2.1.3 | "with a" has yellow highlight in the following: The UE configured with a DRX cycle smaller than or equal to the modification period verifies that stored system information remains valid by either checking systemInfoValueTag in SystemInformationBlockType1 after the modification period boundary, or attempting to find the systemInfoModification indication at least modificationPeriodCoeff times during the modification period in case no paging is received, in every modification period. | 1 |  | Make " with a" not highlighted in the following: The UE configured with a DRX cycle smaller than or equal to the modification period verifies that stored system information remains valid by either checking systemInfoValueTag in SystemInformationBlockType1 after the modification period boundary, or attempting to find the systemInfoModification indication at least modificationPeriodCoeff times during the modification period in case no paging is received, in every modification period. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.016 | 5.2.2.10 | There is a large "i" and "p" in the following and a missing comma and missing dot: Upon receiving SystemInformationBlockType3, the UE shall: 1> If redistributionServingInfo is included 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4] | 1 |  | Correct as follows: Upon receiving SystemInformationBlockType3, the UE shall: 1> if redistributionServingInfo is included: 2> perform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4]. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.218 | 5.2.2.10 Actions upon reception of SystemInformationBlockType3 | Missing colons in the procedure text | 1 |  | (If not already fixed in the v13.0.0 CR implementation) add the missing characters in Upon receiving SystemInformationBlockType3, the UE shall: 1> If redistributionInterFreqInfo is included: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4]; Coordinator: Corrected during v1300 CR implementation (and this is a duplicated issue): |  | Closed |
| E.219 | 5.2.2.10 Actions upon reception of SystemInformationBlockType4 | Missing colons in the procedure text | 1 |  | (If not already fixed in the v13.0.0 CR implementation) add the missing characters in Upon receiving SystemInformationBlockType5, the UE shall: 1> If redistributionInterFreqInfo is included: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4]; Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.021 | 5.2.2.10 and various locations where 5.2.4.z is referred from 36.304 (MCLD) | Wrong style used in referencing 36.304 in various locations where “as specified in 5.2.4.z [4] | 1 |  | should refer to TS 36.304 [4, 5.2.4.z]. Naturally chapter number should be updated once known. Coordinator: Same as D.003 and H.021 |  | Closed |
| H.021 | 5.2.2.10; 5.2.2.12 | 1> If redistributionServingInfo is included 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4] should be 1> If redistributionServingInfo is included: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in TS 36.304 [5.2.4.z, 4]; | 1 |  | Coordinator: Same as D.003 |  | Closed |
| E.017 | 5.2.2.12 | Same error as issue E.016 above. | 1 |  | Same as for issue E.016. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.012 | 5.2.2.4 | There are spaces instead of indentations in the following as well as a comma is missing after "else": 2> if the UE uses a DRX cycle longer than the modification period: 3> start acquiring the required system information, as defined in 5.2.2.3, from the next H-SFN boundary defined by H-SFN mod 256 = 0; 2> else | 1 |  | Change from spaces to indentations in the following and add a comma after "else": 2><here> if the UE uses a DRX cycle longer than the modification period: 3> start acquiring the required system information, as defined in 5.2.2.3, from the next H-SFN boundary defined by H-SFN mod 256 = 0; 2> <here>else Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.014 | 5.2.2.4 | Gramatical/logical error in the following. With the current wording the if-statement would be true regardless of whether the current stored system information block is "valid" since it would be true if the UE has ever (even some time ago) stored a valid version of this system information block: 3> if schedulingInfoList indicates that SystemInformationBlockType20 is present and the UE does not have stored a valid version of this system information block: | 1 |  | Correct as follows: 3> if schedulingInfoList indicates that SystemInformationBlockType20 is present and the UE does not have stored a stored valid version of this system information block: Coordinator: This formulation is used for other SIBs as well. This correction is probably not needed and this issue could be closed. If there is a reason to make an expection for SIB20, the issue could be re-opened. |  | Closed |
| E.217 | 5.2.2.4 System information acquisition by the UE | There should be TABs between “>”and the procedure text | 1 |  | (If not already fixed in the v13.0.0 CR implementation) add TABs in 2> if the UE uses a DRX cycle longer than the modification period: 3> start acquiring the required system information, as defined in 5.2.2.3, from the next H-SFN boundary defined by H-SFN mod 256 = 0; 2> else 3> start Coordinator: Corrected during v1300 CR implementation |  | Closed |
| 5.3 Connection control |  |  |  |  |  |  |  |
| H.024 | 5.3.1.3 | “In case the UE was configured with DC, the target eNB indicates in the handover message that the UE shall release the entire SCG configuration. Misleading setence, seems the target eNB shall always release the entire SCG upon HO. Prefer to remove this sentence. | 2 |  | Samsung: We should keep the information that it is the target that indicates release to the UE. Sentence can however be rephrased e.g. as follows: In case of SCG release upon handoverhe UE was configured with DC, it is the target eNB that indicates in the handover message that the UE shall release the entire SCG configuration. Intel: Needs further discussion. The previous sentence says “If an SCG is configured, handover involves either SCG release or SCG change.” Even for SCG Change, we may still release and add the same SCG? Ericsson: This is legacy procedure text. What is the consequence if deleted? ZTE: Agree with HW. The sentence is no longer correct, as we have supported the HO while keeping SCG. Nokia Networks: The current sentence is indeed confusing so it either needs to be deleted or modified, but no strong opinion which option to take. A possible way way to modify could be (changed word bolded): In case the UE was configured with DC, the target eNB indicates in the handover message whether the UE shall release the entire SCG configuration Coordinator: Not agreeable. Can be treated in a separate CR. Closed for now. |  | Closed (General CR) |
| Z.007 | 5.3.13 | If an SCell belonging to a PUCCH cell group only sends HARQ feedback via PUSCH of a SCell without PUCCH, and does not send its HARQ feedback on the PUCCH of the indicated serving cell. Upon the release of the PUCCH, the SCell is unable to release cqi-ReportConfigSCell. For example, the pre-scheduling PUSCH with period of 1ms is allowed from an SCell by network implementation. Then the PUCCH may not be used by the UE. | 2 |  | Suggested change by trying to align witht field descripton of pucch-Cell: 1> apply the default physical channel configuration for cqi-ReportConfig for the indicated serving cell as specified in 9.2.4 and release cqi-ReportConfigSCell, for each SCell that sends HARQ PUCCH feedback on the indicated serving cell, if any; CATT: We prefer current description, since there is no PUCCH feedback, but only HARQ feedback, and also the the same description is used in MAC spec. Coordinator: Not agreeable. Can be treated in a separate CR. Closed for now. |  | Closed (General CR) |
| E.221 | 5.3.3.13 Access barring check for ACDC | What is "ACDC barring parameter" and where is it defined? | 2 |  | This is probably supposed to be acdc-BarringConfig , i.e.ACDC barring parameter" should probably be replaced with acdc-BarringConfig everywhere in the procedures. Huawei: ok to change “ACDC barring parameter” into “acdc-BarringConfig”. Ericsson: Also, procedure text in 5.3.3.2 could be simplified, as there is no need to use “ACDC barring parameters” Change from: 2> perform access barring check as specified in 5.3.3.xx, using T3xx as "Tbarring" and acdc-BarringConfig in the BarringPerACDC-Category as "ACDC barring parameter"; To: 2> perform access barring check as specified in 5.3.3.xx, Coordinator: Captured in v13.2.0 CR. Closed |  | Open (General CR) |
| Z.011 | 5.3.5.4 | The initiation of the sidelinkUEInformation message transmission should also include the scenario where the had transmitted the sidelinkUEInformation includeing the request for relay resources, such as commTxResourceInfoReqRelay and discTxResourceReqPS. | 4 |  | The original text is as follows. It does not consider the scenario that UE has send the SidelinkUEInformation for relay resource request: 2> if SystemInformationBlockType18 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message including commRxInterestedFreq or commTxResourceReq during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or 2> if SystemInformationBlockType19 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message including discRxInterest or discTxResourceReq during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo: Suggest to change to: 2> if SystemInformationBlockType18 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message including commRxInterestedFreq or commTxResourceReq or commTxResourceInfoReqRelay during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or 2> if SystemInformationBlockType19 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message including discRxInterest or discTxResourceReq or discTxResourceReqPS during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo: Samsung: Agree Ericsson: Agree, but the added “or”s should not be in italics. Nokia: This seems to belong to Prose CR instead, to be removed from here and moved there. Samsung: The suggested change seems incomplete i.e. in case the UE indicated it no longer requires resources, it should also repeat. In other words, if SIB18 is broadcast, any change concerning sideling communication should be repeated. Likewise for SIB19, but there we should also cover gaps related changes. |  | Closed (ProSe CR) |
| Z.009 | 5.3.3.1a | “ discTxResourcePS” does not exist. It is suggested to use “discTxResourcesPS” to align with the ASN.1 definition. | 1 |  | The original description use an non existing IE as follows: 2> if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated: Suggest to change to: 2> if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS discTxResourcesPS included and set to requestDedicated: Intel: ok, but category should be “1” as it is an editorial correction Ericsson: Agree. Coordinator: Changed from class 2 to class 1. |  | Closed (ProSe CR) |
| Z.010 | 5.3.3.1a | It is suggested to remove the non-relay constraints since this condition also include the scenario where the upper layer configures to transmit relay related sidelink discovery announcement. | 2 |  | The original condition only considers the UE configured to transmit non-relay PS related sidelink discovery. However in the subsequent description, it considers the UE configured to transmit relay related sidelink discovery announcement which in fact will never happen. 2> if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated: 3> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 3> if configured by upper layers to transmit relay related sidelink discovery announcements: 4> if the UE is acting as relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRelayUE, if included in SystemInformationBlockType19 of the cell on which the UE camps; and above discThreshLoRelayUE, if included in the same SystemInformationBlockType19; or 4> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19 of the cell on which the UE camps: Suggest to change the conditions to : 2> if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated: Samsung: Agree Intel: first, we agree it’s not easy to understand this part. However we have different suggestion. It would be clearer if the first bullet 2> (“if the frequency on which the UE is configured to transmit PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common”) is separated into two cases, i.e. one of for relay related PS and the other one is non-relay related PS. Then the bullets 4> will be put into the part of relay related PS Ericsson: This is same issue as E.091. We are not sure the proposed solution is the correct one, as the UE may be configured to transmit non-Relay PS discovery, or configured to transmit PS discovery. We need to discuss which case we are looking at here. Samsung: Covered by agreements from R2-160034 |  | Closed (ProSe CR) |
| N.196 | 5.3.3.1a | What happens after these statements (there is no level 5 after the level 4 conditions!) 3> if configured by upper layers to transmit relay related sidelink discovery announcements: 4> if the UE is acting as relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRelayUE, if included in SystemInformationBlockType19 of the cell on which the UE camps; and above discThreshLoRelayUE, if included in the same SystemInformationBlockType19; or 4> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19 of the cell on which the UE camps: | 2 |  | Discuss what to do and add the corresponding action Ericsson: We understand it as the UE, if all of this evaluates to “true”, initiates an RRC Connection, as shown in the beginning of the procedure: For sidelink discovery an RRC connection is initiated only in the following case: ZTE: This section discuss the conditions for establishing RRC connection for sidelink communication/discovery. So the corresponding action is clear that the UE shall initite the RRC connection establishment. Intel: it is not clear what the problem is. If 4> is met, then it will initiate RRC connection establishment. It is already specified in the top level, i.e. “For sidelink discovery an RRC connection is initiated only in the following case:” So why we need something for level 5? Samsung: There is no action i.e. these are merely conditions for establishing a connection (so no change needed) CATT: This problem does not exist since it has stated in the beginning that “ For sidelink discovery an RRC connection is initiated only in the following case”. But we suggest to reorignze the description. The condition should be first described and then describe the UE behaviour. It will be clearer for understanding. Samsung: No change considered needed |  | Closed (ProSe CR) |
| L.003 | 5.3.3.1a | Remove redundant sentences since double check of the same condition. The removed condition is already checked by ‘ 1> if configured by upper layers to transmit PS related sidelink discovery announcements:’ In order to allow relay and/or remote UE to establish RRC connection without checking the threshold when they are intended to transmit non-relay related sidelink discovery announcement, it is proposed to separate the paragraphs for relay and non-relay. | 2 |  | 2> if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated: 3> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 3> if configured by upper layers to transmit relay related sidelink discovery announcements: Samsung: Removal is not really correct i.e. the intention is that the bullets 4> only apply to the 2nd bullet 3. If unclear, maby 2nd bullet can be removed while lifting both bullets 4> (i.e. change to bullet 3) Ericsson: Same issue as E.091 and Z.010. Intel: see the comment in Z.010 (similar issue as Z.010). ZTE: It is not appropriate to remove the condition. In our opinion, the two ifs in level 3> are used to differentiate the relay and non relay cases, where the relay relevant discovery announcement transmission requires the check of threshold. Samsung: Covered by agreements from R2-160034 |  | Closed (ProSe CR) |
| E.092 | 5.3.3.1a | The term “cell on which the UE camps” is long and cumbersome. | 2 |  | Suggest to change to “serving cell” instead. From the context of this section it is clear that the UE is in RRC_IDLE, hence there is only one serving cell. There are multiple occurrances of this text. Intel: it has been specified from Rel-12 and since there is no problem in principle, it seems better to keep the original sentence. CATT: it is more suitable to use “camped cell” to replace the term “cell on which the UE camps”？ Samsung: As used in REL-12 no change (for now) |  | Closed (ProSe CR) |
| E.275 | 5.3.3.1a | 2> if the UE has a selected relay; and if SystemInformationBlockType19 is broadcast by the cell on which the UE camps and includes discConfigRelay; and if either discThreshHiRemoteUE is not included in SystemInformationBlockType19 or the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE; | 2 |  | This is not in line with 5.10.10.2 (page 171) , since for transmission of sidelink UE info, it should be the UE is selecting/has a selected relay. However to me, the UE should send SL info once the selection has been done. Therefore I would say that the statement in 5.10.10.2 is wrong. Samsung: We assume a remote UE may send a sollicitation message before having selected a relay. Same comment as for E.082 ZTE: We could not find the 5.10.10.2 section. Does it mean the 5.10.4 in page 171? Agree that the UE has already selected relay when it transmits the sidelink communication. Intel: it’s for relay related communication, so it seems more correct the UE has a selected relay UE to trigger RRC connection establishment for relay related communication. So we prefer keeping the current text. Samsung: No change i.e. for communication it should be has a selected relay (only for discovery ‘is selecting’ applies also) |  | Closed (ProSe CR) |
| H.026 | 5.3.3.1a | "PS" is not defined | 1 |  | Add acronym Intel: ok (same as E.182) Ericsson: Agree, "PS" is "Packet switched" in 21.905. But here "PS" means Public Safety. The collision should be addressed. Coordinator: There is a collision at least in “5.6.3.3 Reception of the UECapabilityEnquiry by the UE” where GERAN PS domain is mention. So, a new abbreviation is needed for public safety. This should be addressed by ProSe CR issue H.026 because this abbreviation is ProSe related. Samsung: Easiest change seems to use PuS. However, as this would affect many procedures as well as field names, it seems preferable not to change PS to PuS. Added PS to abbreviation section, clarifying that meaning depends on context i.e. in context of SL it means public safety and otherwise packet switched |  | Closed (ProSe CR) |
| H.027 | 5.3.3.1a | Last bullet of each sentence "For sidelink ..." should be terminated with "." instead of ":". | 1 |  | Samsung: Correction noted, but every bullet should end with ; (not with dot) Intel: no strong view Ericsson: We disagree. Using a colon (:) for these cases seems to be practised on numerous places in the spec. Samsung: No change |  | Closed (ProSe CR) |
| E.020 | 5.3.3.1a | The comma should be a dot in the end of the following: For sidelink communication an RRC connection is initiated only in the following case: 1> if configured by upper layers to transmit non-relay related sidelink communication and related data is available for transmission: 2> if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 does not include commTxPoolNormalCommon; 1> if configured by upper layers to transmit relay related sidelink communication: 2> if the UE is acting as relay; or: 2> if the UE has a selected relay; and if SystemInformationBlockType19 is broadcast by the cell on which the UE camps and includes discConfigRelay; and if either discThreshHiRemoteUE is not included in SystemInformationBlockType19 or the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE; 3> if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 either does not include commTxPoolNormalCommon or does not include commTxAllowRelayCommon<here> | 1 |  | Change from comma to dot. Intel: not sure what the problem is. It is indicated as “:” Is it a problem? Samsung: All final bullets changed to end with ‘;’ (as is convention) |  | Closed (ProSe CR) |
| E.021 | 5.3.3.1a | Same as issue E.020 : For sidelink discovery an RRC connection is initiated only in the following case: 1> if configured by upper layers to transmit non-PS related sidelink discovery announcements: 2> if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps does not include discTxPoolCommon; or 2> if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResources included and set to requestDedicated: 1> if configured by upper layers to transmit PS related sidelink discovery announcements: 2> if the frequency on which the UE is configured to transmit PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common; or 2> if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated: 3> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 3> if configured by upper layers to transmit relay related sidelink discovery announcements: 4> if the UE is acting as relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRelayUE, if included in SystemInformationBlockType19 of the cell on which the UE camps; and above discThreshLoRelayUE, if included in the same SystemInformationBlockType19; or 4> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19 of the cell on which the UE camps<change from comma to dot> | 1 |  | Same as issue E.020. Coordinator: This probably means that issue E.020 is a similar kind of issue as E.021, i.e. E.020 and E.021 are not the same issue. Intel: not sure what the problem is. It is indicated as “:” Is it a problem? Ericsson: Sorry for the confusion. What is meant is that the solution to E.021 is the same as E.020. Samsung: See E.020 |  | Closed (ProSe CR) |
| E.088 | 5.3.3.1a – sidelink discovery | The word “while” can be misunderstood. 2> if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps does not include discTxPoolCommon; or | 1 |  | Change “while” to “and” 2> if the frequency on which the UE is configured to transmit non-PS related sidelink discovery announcements concerns the camped frequency whileand SystemInformationBlockType19 of the cell on which the UE camps does not include discTxPoolCommon; or Intel: ok Samsung: Corrected |  | Closed (ProSe CR) |
| E.079 | 5.3.3.1a sidelink communication | The term “non-relay related sidelink communication”is incorrect. 1> if configured by upper layers to transmit non-relay related sidelink communication and related data is available for transmission: | 1 |  | Change text to 1> if configured by upper layers to transmit one to many non-relay related sidelink communication and related data is available for transmission; or: 1> if configured by upper layers to transmit non-relay related one to one sidelink communication and related data is available for transmission: HV>No change for now (considering comment to E.080). General statement as covered by changes from R2-160015 seems sufficient |  | Closed (ProSe CR) |
| E.080 | 5.3.3.1a sidelink communication | The term “relay-related sidelink communication” is incorrect. 1> if configured by upper layers to transmit relay related sidelink communication: | 1 |  | Change text to: 1> if configured by upper layers to transmit relay related one to one sidelink communication: Intel: not sure what the problem is and what the difference is between the current text and the newly proposed text. Slightly prefer keeping the current one. Samsung: See E.079 |  | Closed (ProSe CR) |
| E.081 | 5.3.3.1a sidelink communication | The UE behaviour with respect to the following statement is ambiguous: 3> if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 either does not include commTxPoolNormalCommon or does not include commTxAllowRelayCommon: Our understanding is that the UE does not perform RRC connection only if commTxAllowRelayCommon is true and commTxPoolNormalCommon is included. This can be clarfied. | 1 |  | Change the text to 3> if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 eitherdoes not include commTxPoolNormalCommon; or: 3> if SystemInformationBlockType18 is broadcast by the cell on which the UE camps; and if the valid version of SystemInformationBlockType18 does not include commTxAllowRelayCommon: The proposed text more accurately captures the case when neither of the parameters are set/included. Samsung: Not sure if this is really needed (could also remove either, if found confusing) Intel: not sure what the difference is between the current text and the newly proposed text. If clarified as same, we’re fine with the either way. Samsung: Kept 1 bullet but removed ‘either’ and second ‘does not include’ |  | Closed (ProSe CR) |
| E.085 | 5.3.3.1a sidelink communication | It may seem that there are two cases as there are two independent if-clauses. 1> if configured by upper layers to transmit non-relay related sidelink communication and related data is available for transmission: 1> if configured by upper layers to transmit relay related sidelink communication: | 1 |  | Change the second if-clause to: 1> else, if configured by upper layers to transmit relay related sidelink communication: Samsung: Isn’t it so that a relay UE may be engaged in both at the same time. Hence, addition of else seems incorrect Intel: we see no real need to change. Samsung: No change |  | Closed (ProSe CR) |
| E.091 | 5.3.3.1a sidelink discovery | The following procedure is incorrect. 2> if the frequency on which the UE is configured to transmit non-relay PS related sidelink discovery announcements (e.g. group member discovery) is included in discInterFreqList in SystemInformationBlockType19 broadcast by the cell on which the UE camps, with discTxResourcePS included and set to requestDedicated: 3> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 3> if configured by upper layers to transmit relay related sidelink discovery announcements: 4> if the UE is acting as relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRelayUE, if included in SystemInformationBlockType19 of the cell on which the UE camps; and above discThreshLoRelayUE, if included in the same SystemInformationBlockType19; or 4> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19 of the cell on which the UE camps: Either the UE is configured to transmit non-Relay PS discovery and if so the latter parts related to relaying can be dropped. Or (more likely) the UE is configured to transmit PS discovery (can be Relaying or group member discovery), and if so, the first if-statement should be corrected. | 2 |  | Discuss and change accordingly Samsung: See Z.010 Intel: see the comments in Z.010 (similar issue) Ericsson: This is same issue as Z.010, also similar to L.003. CATT: we also think this issue exists and need correction. Samsung: See Z.010 |  | Closed (ProSe CR) |
| E.086 | 5.3.3.1a sidelink discovery | The term “non-PS related sidelink discovery” is not clearly defined. | 1 |  | Add the following text to the start of section 5.10.6. “Sidelink discovery consists of Public safety related (PS related) and non-PS related sidelink discovery. PS related sidelink discovery consists of relay related and non-relay related PS related sidelink discovery. Upper layers indicate to RRC whether a particular announcement is PS related or non-PS related.” Intel: if we put the “PS” into abbreviations (see E.182), shouldn’t the current text be ok as it is? Coordinator: See alsoE.087, E.128 where it is commented that also PS related sidelink discovery and the variants of sidelink discovery are missing. It should be enough to discuss this definition proposal only once. Hence E.087 and E.128 are closed for now. Samsung: Covered by changes from R2-160015 |  | Closed (ProSe CR) |
| E.089 | 5.3.3.1a sidelink discovery | It may seem that there are two cases as there are two independent if-clauses. 1> if configured by upper layers to transmit non-PS related sidelink discovery announcements: 1> if configured by upper layers to transmit PS related sidelink discovery announcements: | 1 |  | Change the second if-clause to 1> else, if configured by upper layers to transmit PS related sidelink discovery announcements: Samsung: See remark for E.085 Intel: We see no real need to change. Samsung: See E.085 |  | Closed (ProSe CR) |
| E.090 | 5.3.3.1a sidelink discovery | The word “while” can be misunderstood. 2> if the frequency on which the UE is configured to transmit PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common; or | 1 |  | Change “while” to “and”. 2> if the frequency on which the UE is configured to transmit PS related sidelink discovery announcements concerns the camped frequency whileand SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common; or Intel: ok Samsung: Changed |  | Closed (ProSe CR) |
| S.018 | 5.3.10.3 | Improve wording | 1 |  | Change to: 1> if uplink PDCP delay results are available: 2> set the ul-PDCP-DelayResultLst to include the available delay results; Nokia: The correction section 5.5.5, not 5.3.10.3 The corrected text is slightly different than the proposal from Samsung (to align both 1> and 2> with each other, the highlighted part shows the different): 1> if uplink PDCP delay results are available: 2> set the ul-PDCP-DelayResult to include the available uplink PDCP delay results; |  | Closed (General CR) |
| N.024 | 5.3.11 | This procedural text “> if the detected radio link failure concerns a DRB which QCI is 1:” Probably RLF itself does not concern specific QCI but UE who is configured with DRB for which QCI1 is mapped | 1 |  | Change sentence to “”3> if UE has DRB for which QCI is 1:” . Level 2 covers the point that RLF has been detected => no need to duplicate. Similary update field description to “This field is used to indicate the radio link failure occurred while a bearer with QCI value equal 1 was configured, see TS 24.301 [35].” Nokia: This seems more like level 1 issue, so it is closed with the proposed correction to procedural text adopted, as well as field description in UEInformationResponse-message in section 6.2.2. See also |  | Closed (General CR) |
| S.016 | 5.3.2.3 | The 2nd and 3rd bullet 1 can be combined i.e. action is same | 1 |  | Change to: 1>sysInfoMod-eDRX and UE configured with longer DRX; or 1> sysInfoMod included Nokia: Corrected in general CR as follows: 1> if the systemInfoModification is included; or 1> if the UE is configured with a DRX cycle longer than the modification period and the systemInfoModification-eDRX is included: 2> re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2. |  | Closed (General CR) |
| C.009 | 5.3.2.3 | 1> if the UE uses a DRX cycle longer than the modification period: 2> if the systemInfoModification-eDRX is included: | 1 |  | change to “is configured with” Nokia: Corrected in general CR as proposed (see S.016). |  | Closed (General CR) |
| H.025 | 5.3.2.3 | Similar to comment on 5.2.2.10 1> if in RRC_IDLE, the redistributionIndication is included and the UE is redistribution capable: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4]; should be changed to 1> if in RRC_IDLE, the redistributionIndication is included and the UE is redistribution capable: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in TS 36.304 [5.2.4.z, 4];; | 1 |  | Nokia: Corrected in general CR (same issue as D.005). |  | Closed (General CR) |
| H.030 | 5.3.3.13 | "The UE shall" is missing at the begining | 1 |  | Nokia: Corrected in general CR. |  | Closed (General CR) |
| H.029 | 5.3.3.2 | There is no “ac-BarringForACDC,” in ASN.1 part; Perfer to use “ACDC barring parameter” instead. In addition, for 2> if the ac-BarringForACDC contains a BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers:, could be changed to 2> if the ac-BarringForACDC contains a BarringPerACDC-Category entry correspondingcorresponds to the ACDC category selected by upper layers: | 2 |  | Intel: see I.013, E.211, N.022. For the first issue we prefer to replace ac-BarringForACDC with barringPerACDC-CategoryList. For the second issue we see no need to change it the first issue is corrected as proposed in I.013. Nokia: See I.013 is closed and implemented to general CR, this is also closed as a consequence. |  | Closed (General CR) |
| I.012 | 5.3.3.2 | In the procedural text related to ACDC the reference to “common access barring parameters” is incorrect as the common access barring parameters refer to the access class barring parameters. Instead it should be corrected to ACDC-BarringForCommon. | 1 |  | Replace “common access barring parameters” with ACDC-BarringForCommon. 1> if SystemInformationBlockType2 contains ACDC-BarringPerPLMN-List and the ACDC-BarringPerPLMN-List contains an ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]): 2> select the ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers; 2> in the remainder of this procedure, use the selected ACDC-BarringPerPLMN entry for ACDC barring check (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters ACDC-BarringForCommon included in SystemInformationBlockType2; 1> else: 2> in the remainder of this procedure use the ACDC-BarringForCommon (i.e. presence or absence of these parameters) included in SystemInformationBlockType2 for ACDC barring check; Huawei: agree. Nokia: Closed in the general CR as per above. |  | Closed (General CR) |
| I.013 | 5.3.3.2 | In the procedural text related to ACDC the reference ac-BarringForACDC is not aligned with the corresponding ASN.1 in SIB2 and should be replaced by barringPerACDC-CategoryList. | 1 |  | Replace ac-BarringForACDC with barringPerACDC-CategoryList: 1> if upper layers indicate that the RRC connection is subject to ACDC (see TS 24.301 [35]), SystemInformationBlockType2 contains ac-BarringForACDCbarringPerACDC-CategoryList, and acdc-HPLMNonly indicates that ACDC is applicable for the UE: 2> if the ac-BarringForACDCbarringPerACDC-CategoryList contains a BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers: 3> select the BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers; 2> else: 3> select the last BarringPerACDC-Category entry in the BarringPerACDC-CategoryList; 2> stop timer T308, if running; 2> perform access barring check as specified in 5.3.3.13, using T308 as "Tbarring" and acdc-BarringConfig in the BarringPerACDC-Category as "ACDC barring parameter"; 2> if access to the cell is barred: 3> inform upper layers about the failure to establish the RRC connection and that access barring is applicable due to ACDC, upon which the procedure ends; Huawei: the same to N.022 Intel: see N.022, E.211, H.029 Nokia: Closed in the general CR (same as N.022, E.211, H.029). |  | Closed (General CR) |
| E.082 | 5.3.3.1a | The term “acting as relay” is ambiguous. | 4 |  | Change to “acting as a sidelink relay UE”. Multiple occurances. Samsung: As indicated during e-mail review we will bring a paper to RAN2#93 discussing terminology and modelling of the inter-layer interaction (which is related). Prefer not to make changes at this stage Intel: ok, to be more accurate we may better say a sidelink UE-to-NW relay UE. Samsung: Covered by changes from R2-160015 |  | Closed (ProSe CR) TDoc Sam |
| S.020 | 5.3.11.3 | The text indicates that RLF concerns a particular DRB, which suggests the intention is only to cover the case of an RLC unrecoverable error (which we understand is not intended) | 2 |  | Rephrase to ‘if a DRB with QCI 1 is established (upon detecting RLF detected): Nokia: See N.024 – since the whole section covers actions related to RLF, the extra clarification doesn’t seem needed. |  | Closed |
| D.005 | 5.3.2.3 Reception of the Paging message by the UE | Style error of referring to the other specification. | 1 |  | Should be corrected as follows. 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in TS 36.304 [4, 5.2.4.z [4]; Nokia: Duplicate of H.025 – closed since H.025 is corrected in general CR. |  | Closed |
| H.023 | 5.3.1.3 | An SCG can be established reconfigured Comma is missing; should be An SCG can be established, reconfigured | 1 |  | Intel: Disagree. The comma is NOT missing in CR R2-157064. Coordinator: This is a CR implementation error. |  | Closed |
| E.031 | 5.3.10.3 | Unclear what the following means (xor?): 2> if drb-ToAddModListSCG or drb-ToAddModListLWA is not received or does not include the drb-Identity value (i.e. add MCG DRB): | 1 |  | Change to: 2> if drb-ToAddModListSCG is not received or does not include the drb-Identity value, and; 2> if drb-ToAddModListLWA is not received or does not include the drb-Identity value (i.e. add MCG DRB): Note that there are two occurrences of this. Huawei: Agree.¨ Intel: Addresses same issue as S.017. Slightly prefer S.017. Coordinator: Same as S.017. Qualcomm: See S.017 |  | Closed |
| E.037 | 5.3.11.3 | Spaces instead of tabs in the following: 3><here> if the detected radio link failure concerns a DRB which QCI is 1: 4><here> include the droppedQCI; | 1 |  | Fix this. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.018 | 5.3.2.3 | There are spaces instead of indentations in the following: 1> if the UE uses a DRX cycle longer than the modification period: 2><here> if the systemInfoModification-eDRX is included: 3><here> re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2. 1> else 2> if the systemInfoModification is included: 3><here> re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2. | 1 |  | Change from spaces to indentations in the following: 1> if the UE uses a DRX cycle longer than the modification period: 2><here> if the systemInfoModification-eDRX is included: 3><here> re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2. 1> else 2> if the systemInfoModification is included: 3><here> re-acquire the required system information using the system information acquisition procedure as specified in 5.2.2. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.019 | 5.3.2.3 | Large "p" in the following: 1> if in RRC_IDLE, the redistributionIndication is included and the UE is redistribution capable: 2> Perform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4]; | 1 |  | Change as follows: 1> if in RRC_IDLE, the redistributionIndication is included and the UE is redistribution capable: 2> pPerform E-UTRAN Inter-Frequency Redistribution procedure as specified in 5.2.4.z [4]; Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.220 | 5.3.2.3 Reception of the Paging message by the UE 5.3.5.3 Reception of an RRCConnectionReconfiguration not including the mobilityControlInfo by the UE 5.3.11.3 Detection of radio link failure | There should be TABs between “>”and the procedure text. There should not be any SPACEs between “>” and the procedure text. | 1 |  | (If not already fixed in the v13.0.0 CR implementation) add TABs in the procedures and remove SPACEs. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.022 | 5.3.3 RRC connection establishment | In two places ac-BarringForACDC is wrongly referrenced field | 1 |  | There is no such field in ASN.1. Change to “BarringPerACDC-CategoryList” , alternatively to a) “ac-Barring for ACDC” or b)“acdc-BarringConfig within a “BarringPerACDC-Category” Huawei: The alternative a) is not precise and b) is not correct because the previous “ac-BarringForACDC” should at least include “BarringPerACDC-Category”. So we propose to change “ac-BarringForACDC” into “ACDC-BarringForCommon or ACDC-BarringPerPLMN”. Intel: see I.013, E.211, H.029. We prefer to replace ac-BarringForACDC with barringPerACDC-CategoryList. Ericsson: Same as I.013 Coordinator: Same as I.013 |  | Closed |
| Z.008 | 5.3.3.1a | The redundant semicolon should be removed | 1 |  | if the frequency on which the UE is configured to transmit PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common; or Intel: ok Coordinator: Corrected during v1300 CR implementation. |  | Closed |
| L.002 | 5.3.3.1a | Remove ‘;’ | 1 |  | 2> if the frequency on which the UE is configured to transmit PS related sidelink discovery announcements concerns the camped frequency while SystemInformationBlockType19 of the cell on which the UE camps; includes discConfigPS but does not include discTxPoolPS-Common; or Intel: ok (same as Z.008) Ericsson: Agree Coordinator: Same as Z.008 |  | Closed |
| L.001 | 5.3.3.1a | Remove ‘:’ Replace ‘;’ with ‘:’ | 1 |  | 1> if configured by upper layers to transmit relay related sidelink communication: 2> if the UE is acting as relay; or: 2> if the UE has a selected relay; and if SystemInformationBlockType19 is broadcast by the cell on which the UE camps and includes discConfigRelay; and if either discThreshHiRemoteUE is not included in SystemInformationBlockType19 or the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE;: Intel: ok Ericsson: Agree Coordinator: Corrected during v1300 CR implementation. |  | Closed |
| E.083 | 5.3.3.1a | The term “has a selected relay” is ambiguous. | 1 |  | Change to “has a selected sidelink relay UE”. Multiple occurrances. Samsung: Same comment as for E.082 Intel: ok, to be more accurate we may better say a sidelink UE-to-NW relay UE. Coordinator: Same as E.082 |  | Closed |
| E.084 | 5.3.3.1a | The term “is selecting a relay” is ambiguous. | 1 |  | Change to “is selecting a sidelink relay UE”. Multiple occurrances. Samsung: Same comment as for E.082 Intel: ok, to be more accurate we may better say a sidelink UE-to-NW relay UE. Coordinator: Same as E.082 |  | Closed |
| E.273 | 5.3.3.1a | if configured by upper layers to transmit non-PS related sidelink discovery announcements | 1 |  | PS is not included in the abbreviation list Intel: ok (same as E.182) Coordinator: Same as E.182 |  | Closed |
| E.087 | 5.3.3.1a sidelink discovery | The term “PS related sidelink discovery” is not clearly defined. | 1 |  | Add the following text to the start of section 5.10.6. “Sidelink discovery consists of Public safety related (PS related) and non-PS related sidelink discovery. PS related sidelink discovery consists of relay related and non-relay related PS related sidelink discovery. Upper layers indicate to RRC whether a particular announcement is PS related or non-PS related.” Intel: if we put the “PS” into abbreviations (see E.182), shouldn’t the current text be ok as it is? Coordinator: See alsoE.086, E.128 |  | Closed |
| H.028 | 5.3.3.2 | ":" to be removed in "1> else:" and "2> else:" | 1 |  | Remove ":" Intel: Can’t find the issue addressed. Coordinator: Agree with Intel that the issue cannot be found. Closed for now. Can be reopened if the issue is found. |  | Closed |
| N.023 | 5.3.3.4, 7.3 | Timer T360 description missing in the clause 7.3 | 2 |  | Add timer description for T360 Samsung: See S.109 Coordinator: No solution proposal and hence closed. Address the issue S.109 instead (which is similar and contains a solution proposal). |  | Closed |
| 5.4 Inter-RAT mobility |  |  |  |  |  |  |  |
| 5.5 Measurements |  |  |  |  |  |  |  |
| E.212 | 5.5.3.13 | If-stmt not needed in ACDC barring check | 2 |  | From procedure text in 5.5.3.2, the highlighted if-stmt and else-branch should not be needed #### 5.3.3.13 Access barring check for ACDC 1> if timer T302 is running: 2> consider access to the cell as barred; 1> else if SystemInformationBlockType2 includes "ACDC barring parameter": 2> draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1; 2> if 'rand' is lower than the value indicated by ac-BarringFactor included in "ACDC barring parameter": 3> consider access to the cell as not barred; 2> else: 3> consider access to the cell as barred; 1> else: 2> consider access to the cell as not barred; Huawei: an alternative is that, the second highlighted part can be changed into 2> consider access to the cell as not barred due to ACDC. Intel: Correct subclause is 5.3.3.13. We need the if-statement to cover the case where acdc-BarringConfig is not present. In this case the access to the cell is considered as not barred for the associated ACDC category Coordinator: Not an issue as explained by Intel. Closed for now. |  | Closed (General CR) |
| I.015 | 5.5.4.1 | With regards to initiation of LAA measurement reporting procedure the current condition says: 3> initiate the measurement reporting procedure as specified in 5.5.5 immediately after a first RSSI sample value is reported by the physical layer; However, when the first RSSI sample value is delivered may be different depending on implementation if L1 measurement duration is more than 1 OFDM symbol. In terms of implementation the PHY layer may send 1 sample every L1 averaging duration or N samples every L1 measurement duration. As result, the more reasonable assumption would be that the measurement reporting procedure is initiated when the first L1 measurement duration is completed. | 2 |  | Update the condition for initiating LAA measurement reporting procedure as shown below: 3> initiate the measurement reporting procedure as specified in 5.5.5 immediately after a firstwhen RSSI sample values areis reported by the physical layer after the first L1 measurement duration; Samsung: We are not sure to what extend we need to specify and if this is needed i.e. is it only making a difference if L1 is not doing L1 averaging? Ericsson: Agree, but this particular formuation is maybe not be very clear. Would the RSSI sample really come after the L1 measurement duration? Maybe better to talk about the first samples for the first L1 measurement duration? CATT: No strong view. If needed, we prefer the following description: 3> initiate the measurement reporting procedure as specified in 5.5.5 immediately after a first RSSI sample(s) is reported by the physical layer; Coordinator: Coordinator: Captured in v13.1.0 CR. Closed. |  | Closed (General CR) |
| S.023 | 5.5.4.1, first report | As result of CR merging it is specified that condition for first LAA measurement report and conditions for other first measurement reports may occur simultaneously | 2 |  | Ericsson: Is this a problem? Coordinator: It could be discussed if this is (or may become) a problem Coordinator: No solution proposals. Can be treated in a separate CR. Closed for now |  | Closed (General CR) |
| C.010 | 5.5.5.4 | The report amount may be set to 1 when measRSSI-ReportConfig is included. In this case, the first RSSI measurement reporting initiation procedure needs to wait reportinterval time after a first RSSI sample value is reported by the physical layer. Therefore, descriptions of the first RSSI measurement reporting initiation need to be divided into two cases: the case of reportAmonut>1 and the case of reportAmount =1. In addition, as the suggestion to the field description of measRSSI-ReportConfig, the corresponding triggerType and purpose are periodical and reportStrongestCells respectively for RSSI measurment configuration. Therefore, the correponding description of first RSSI measurement reporting initiation can be merged into the common description that the purpose is set to reportStrongestCells. | 2 |  | Update the descriptions as follows: 2> if measRSSI-ReportConfig is included and if a (first) measurement result is available: 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> initiate the measurement reporting procedure as specified in 5.5.5 immediately after a first RSSI sample value is reported by the physical layer; 2> if the purpose is included and set to reportStrongestCells or to reportStrongestCellsForSON and if a (first) measurement result is available: 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> if the purpose is set to reportStrongestCells and reportStrongestCSI-RSs is not included: 4> if the reportAmount > 1: 5> initiate the measurement reporting procedure as specified in 5.5.5 immediately after the quantity to be reported becomes available for the PCell, or after a first RSSI sample value is reported by the physical layer in case of RSSI and channel occupancy measurements; 4> else (i.e. the reportAmount = 1): 5> initiate the measurement reporting procedure as specified in 5.5.5 immediately after the quantity to be reported becomes available for the PCell and for the strongest cell among the applicable cells, or becomes available for the pair of PCell and the PSCell in case of SSTD measurements, or after reportinterval time when a first RSSI sample value is reported by the physical layer in case of RSSI and channel occupancy measurements; Samsung: Related to I.015 i.e. conclude together Ericsson: Not clear why existing text is not correct. Coordinator: Not agreeable. Can be treated in a separate CR. Closed for now |  | Closed (General CR) |
| S.022 | 5.5.2.1 (and 5.5.3) | The text seems to cover the setup case only (but may not be needed, depending on where ul-DelayConfig is placed). Moreover, there seems a need to state something in 5.5.3 (i.e. to clarify that measurements need to be performed). | 4 |  | CATT: It depends on where ul-DelayConfig is placed. Nokia Networks: We plan to bring a contribution on this to February meeting Coordinator: handled in a separate CR. |  | Closed (General CR)TDoc Nokia RAN2#93 |
| N.025 | 5.5.3 Performing Measurements | RS-SINR addition to the initial procedural instruction on measurments performance seems to suggest that RS-SINR would be a mendatory measurement and to be performed for each serving cell (including SCells). For instance the following procedure would become applicable: 4> perform the corresponding measurements on the frequency and RAT indicated in the associated measObject That brings ambiguous interpretations. | 4 |  | Not clear that was the intention. RAN2#91 bis decision was: RAN2 should introduce signalling for inter-frequency SINR and serving cell SINR measurements. Neighbour cell intra-frequency measurements should be considered as well (depending on RAN4 discussion). Nokia Networks: We plan to bring a contribution on this to February meeting. Coordinator: Corrected in general CR according to the ad hoc meeting notes => Move the requirements related to RS-SINR to the part that is 'For each measid...'. See R2-160019. |  | Closed (General CR) TDoc Nokia RAN2#93 |
| E.226 | 5.5.4.1 | The proceudre text should be readable text and therefore the formulation (even though it is well-understood from mathematics) reportAmount > 1 should be avoided. The same applies for reportAmount=1. | 1 |  | Change “if reportAmount > 1” to “if reportAmount is greater than 1”and the text (in parenthesis) “report amount=1” to “reportAmount is equal to 1”. Samsung: (or ‘exceeds/ equals 1’) Nokia: It seems the RRC is not always consistent, and both proposed options are used. However, “is equal to” and “exceeds” are more frequently used in the current RRC text, so corrected according to those in the general CR. |  | Closed (General CR) |
| S.029 | 5.5.5 | Field name is incorrect i.e. ‘List’ is missing | 2 |  | Change to ul-PDCP-DelayResultLst CATT: Ok with the change. But, how the UE knows there is ul-PDCP-DelayResult availbale? Nokia: Correct the sentence to ul-PDCP-DelayResultList since that is the field name in MeasResults. |  | Closed (General CR) |
| E.039 | 5.5.1 | Incomplete statement that UE also measures serving WLANs: For inter-RAT WLAN, the UE measures and reports on listed cells. | 1 |  | Update as follows: For inter-RAT WLAN, the UE measures and reports on serving and listed cells. Samsung: Isn’t this related to E.040? Moreover, are there cases the serving is not listed? Intel: Disagree. This suggested modification introduces a problem they suggest to discuss in E.040! This is also definatelty not category 1. Nokia Networks: We think the proposed modification is not correct. So far we haven’t had a notion of “serving WLAN cell”, and we are explicitly listing all WLAN identifiers to be measured by UE. That’s why they are only considered as “listed cells”. Whether we can use the term “cell” for WLAN can be discussed RAN2#93. Qualcomm: See E.040; this is not a class 1 issue. It was done this way very intentionally considering the controversy and compromise. Coordinator: Not class 1. Seems to be same as E.040 and hence closed. |  | Closed |
| E.223 | 5.5.2.5 Measurement object addition/ modification 5.5.5 Measurement reporting | There should be TABs between “>”and the procedure text. There should not be any SPACEs between “>” and the procedure text. | 1 |  | (If not already fixed in the v13.0.0 CR implementation) add TABs in the procedures and remove SPACEs. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.042 | 5.5.3.1 | Missing space in the following: 1> whenever the UE has a measConfig, perform RSRP, and RSRQ and RS-SINR (if indicated in the associated reportConfig)<here>measurements for each serving cell as follows: | 1 |  | Fix the error. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| D.007 | 5.5.3.1 Performing measurements – General | A space is missing as shown below. 1> whenever the UE has a measConfig, perform RSRP, RSRQ and RS-SINR (if indicated in the associated reportConfig)measurements for each serving cell as follows: | 1 |  | Should be corrected as follows. 1> whenever the UE has a measConfig, perform RSRP, RSRQ and RS-SINR (if indicated in the associated reportConfig) measurements for each serving cell as follows: Coordinator: Same as E.042 |  | Closed |
| E.211 | 5.5.3.2 | Undefined ac-BarringForACDC used in procedure text. | 2 |  | There is no field called ac-BarringForACDC. Some existing field should be used: 1> if upper layers indicate that the RRC connection is subject to ACDC (see TS 24.301 [35]), SystemInformationBlockType2 contains ac-BarringForACDC, and acdc-HPLMNonly indicates that ACDC is applicable for the UE: 2> if the ac-BarringForACDC contains a BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers: applicable due to ACDC, upon which the procedure ends; Huawei: the same to N.022 Intel: see I.013, H.029, N.022. We prefer to replace ac-BarringForACDC with barringPerACDC-CategoryList. Ericsson: This issue is listed with wrong section number. Issue can be deleted, as problem is already covered by I.012. Coordinator: Already covered by I.012 |  | Closed |
| E.043 | 5.5.4.1 | Editoral errors in the following: 5> if useWhiteCellList is set to TRUE:<extra space> 6> consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is included in the whiteCellsToAddModList defined within the VarMeasConfig for this measId; 5> else<missing comma> | 1 |  | Fix the errors Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.045 | 5.5.4.1 | The following is yellow highlighted: 2> if measRSSI-ReportConfig is included and if a (first) measurement result is available: 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> initiate the measurement reporting procedure as specified in 5.5.5 immediately after a first RSSI sample value is reported by the physical layer; | 1 |  | Remove the highlight. Coordinator: Corrected during v1300 CR implementation. Usually yellow (or any other colour) highlighted text is not implemented in the specification. It would be desirable if such highlights are already removed from final CRs (even though highlighted text may be useful during drafting). |  | Closed |
| 5.6 Other |  |  |  |  |  |  |  |
| E.208 | 5.6.3.3 | Parameters controling Band combination capability signallling | 3 |  | We should re-use the same principle as we did in the Rel-11 that the UE should “mirror” the parameters received in the request UECapabilityEnquiry, and echo them back in the UE-EUTRAcapabilities, similar to 4> indicate in requestedBands the same bands and in the same order as included in the received requestedFrequencyBands; So these “mirrored” parameters are not really capability indicators, but rather information for a “later” eNB (that receives the EUTRA capabilities from MME or other eNB) to get an understanding on what premises the UE originally composed the lists of band combinations. Following the same principle in Rel-11, there should be a more clear separation - UE support bits - Nw-requested “format” of band combinations, and - UE-reported format of band combinations SEE SEPARATE DOCUMENT. Samsung: Indeed requires more discussion. Assuming Ericsson will bring contribution Nokia Networks: We think this is a good and valid point, and agree the Rel-11 principles should be followed here. This topic was brought up only late in the e-mail discussion so it was not captured yet, so we think it would be good to discuss according to contribution(s). Coordinator: Ad hoc meeting notes => UE echos back the number of requested UL and DL CCs. => fallbackCombinationsSkipped echoing is not needed (possible to rely on the per band combination indication that the UE support more than the fallback). => requestedMaximumCCs, skipFallbackComb support indicators are agreed. => Backward compatibility can be further discussed at RAN2#93. See R2-160028. Coordinator: Already fixed in another CR. Can be closed. |  | Closed (General CR) TDoc Eri |
| E.210 | 5.6.3.3 | Procedure text for fallback band combinations | 4 |  | This procedure text need to be improved, I have no proposal now. 6> include CA band combinations with the highest supported number of DL and UL carriers first; 6> if a band combination is the fallback band combination (i.e. a band combination that can be obtained by release of any SCell without requiring reconfiguration of parameters related to UE radio access capabilities for the remaining serving cells, except for release of an SCell from a contiguous CA band configuration that results in a non-contiguous CA band configuration) of an already-included band combination: 7> exclude this band combination from the supported band combination list; 7> if the capabilities of this band combination are different from the already-included band combination for which this band combination is a fallback, include the additionalFallbackSupported in this already-included band combination; 8> include the additionalFallbackSupported in the already-included band combination whose fallback case this band combination is; Nokia Networks: Agree this is not the best wording. We plan to bring a contribution latest to February meeting on this. Coordinator: Handled in a separate CR. |  | Closed (General CR) TDoc Nokia RAN2#93 |
| Z.015 | 5.6.3.3 | The network is not able to know if the UE supports “requestReducedFormat”. No capability bit is included. | 4 |  | Alternatives: - Add one capability signaling to indicate that the UE supports requestReducedFormat. - Reuse “skipFallbackCombinations” to indicate that the UE supports requestReducedFormat. - Clarify in 36.306 that the capaiblity does not need capability signaling. Then the network blindly sends the request to the UE. The UE not supporting the function can just ignore the request. Samsung: Some more discussion seems required Nokia Networks: We assume any UE supporting >5CCs should support reduced format. We don’t see the reduced format as being necessary for other cases. Note that this also relates to E.210, H.040, D.013. Discussions on corrections should be done in RAN2#93. Coordinator: Already fixed in another CR. Can be closed. |  | Closed (General CR) |
| H.040 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | Can requestReducedFormat only be set? What’s the UE behaviour? UECapabilityEnquiry-v13xy-IEs ::= SEQUENCE { requestReducedFormat-r13 ENUMERATED {true} OPTIONAL, skipFallbackCombinations-r13 ENUMERATED {true} OPTIONAL, requestedMaxCCsDL-r13 INTEGER (2..32) OPTIONAL, requestedMaxCCsUL-r13 INTEGER (2..32) OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } | 4 |  | Need more discussion Nokia Networks: Agree this should be discussed. The intention was that the reduced format flag would control which IEs are filled in, but it doesn’t seem to necessarily make sense without any of the other flags. Needs more discussion latest during February meeting. Coordinator: Already fixed in another CR. Can be closed. |  | Closed (General CR) TDoc Nokia RAN2#93 |
| D.013 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | As for the procedure text on additionalFallbackSupported below, 7> if the capabilities of this band combination are different from the already-included band combination whose fallback case this band combination is, include the additionalFallbackSupported in the already-included band combination whose fallback case this band combination is; 8> include the additionalFallbackSupported in the already-included band combination whose fallback case this band combination is; the assumption seems that a fallback combination supports the same capabilities as the band combination with the highest number of DL/UL CCs (callled parent band combination hereafter) as well as different capabilities in addition. For example, the parent band combination supports 2 layer MIMO, while a fallback combination supports 2 layer MIMO and 4 layer MIMO. On the other hand, DCM is wondering if there is a case that there is no overlap between the parent band combination and the fallback combination in terms of their supported capabilities. For instance, the parent combination supports 2 layer MIMO only while the fallback combination only supports 4 layer MIMO only. DCM would like to discuss if such a scenario is valid and how to deal with it. | 4 |  | Nokia Networks: It is not clear whether this would occur in practice, but agree this should be discussed. Coordinator: Can be treated in a separate CR. Can be closed |  | Closed (General CR) TDoc Nokia RAN2#93 |
| D.012 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | The definition of fallback combination is copied and pasted from TS 36.306 as shown below. 6> if a band combination is the fallback band combination (i.e. a band combination that can be obtained by release of any SCell without requiring reconfiguration of parameters related to UE radio access capabilities for the remaining serving cells, except for release of an SCell from a contiguous CA band configuration that results in a non-contiguous CA band configuration) of an already-included band combination: Although it was an outcome from the email discussion, DCM thinks it is a lengthy procedure text. | 4 |  | DCM would like to suggest just to refer TS 36.306 as shown below. 6> if a band combination is the fallback band combination as specified in TS 36.306 [5, 4.3.5.2](i.e. a band combination that can be obtained by release of any SCell without requiring reconfiguration of parameters related to UE radio access capabilities for the remaining serving cells, except for release of an SCell from a contiguous CA band configuration that results in a non-contiguous CA band configuration) of an already-included band combination: ZTE: It seems we defined a new term “the fallback band combination” in 36.331, which is actually not defined in 36.306. Nokia Networks: Agree the term “fallback band combination” has not been defined before. The text attempts to align with the work-around we did in 36.306 earlier. Discussion on corrections should be continued in RAN2#93. Coordinator: Already fixed in another CR. Can be closed. |  | Closed (General CR) |
| C.015 | 5.6.8.2 | If the UE detects IDC problems and the VarLogMeasReport is empty, the UE would not perform logged MBSFN measurements. | 2 |  | Update as follows: 1> perform the logging in accordance with the following: 2> if the UE detects IDC problems and the VarLogMeasReport is empty: 3> suspend adding further logged measurement entries as long as the UE is experiencing IDC problems; NOTE 1: The UE may detect the start of the IDC problem as early as Phase 1 as described in TS 36.300 [9, 23.4]. 2> else if targetMBSFN-AreaList is included in VarLogMeasConfig: 3> if the UE is camping normally on an E-UTRA cell or is connected to E-UTRA; and 3> if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; and Coordinator: The procedure has changed so much that it is not possible to recognize where this change needs to be made. Can be closed for now and treated in a separate CR if still perceived as necessary correction. |  | Closed (General CR) |
| C.014 | 5.6.3.3 | Use the wrong parameter “includedBoth” when only need to indicate maxmim DL or UL CC numbers. Should modify it to “includedDL” or “includedUL” | 1 |  | 5> else if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsDL (i.e. only DL maximum limit is given): 6> include the target band combinations for which the number of CCs in DL is up to the value indicated in the requestedMaxCCsDL; 6> indicate value includedBoth includedDL in requestedMaximumCCs; 5> else if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsUL (i.e. only UL maximum limit is given): 6> include the target band combinations for which the number of CCs in UL is up to the value indicated in the requestedMaxCCsUL; 6> indicate value includedBoth includedUL in requestedMaximumCCs; Nokia: Corrected in general CR. |  | Closed (General CR) |
| D.010 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | For the case where the UE does not support requestedFrequencyBands and supports requestedMaximumCCs, there are a few erroneous values as shown below. 5> if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsDL and the requestedMaxCCsUL (i.e. both UL and DL maximums are given): 6> include the target band combinations for which the number of CCs in DL is up to the value indicated in the requestedMaxCCsDL and for which the number of CCs in UL is up to the value indicated in the requestedMaxCCsUL; 6> indicate value includedBoth in requestedMaximumCCs; 5> else if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsDL (i.e. only DL maximum limit is given): 6> include the target band combinations for which the number of CCs in DL is up to the value indicated in the requestedMaxCCsDL; 6> indicate value includedBoth in requestedMaximumCCs; 5> else if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsUL (i.e. only UL maximum limit is given): 6> include the target band combinations for which the number of CCs in UL is up to the value indicated in the requestedMaxCCsUL; 6> indicate value includedBoth in requestedMaximumCCs; | 1 |  | Should be corrected as follows. 5> if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsDL and the requestedMaxCCsUL (i.e. both UL and DL maximums are given): 6> include the target band combinations for which the number of CCs in DL is up to the value indicated in the requestedMaxCCsDL and for which the number of CCs in UL is up to the value indicated in the requestedMaxCCsUL; 6> indicate value includedBoth in requestedMaximumCCs; 5> else if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsDL (i.e. only DL maximum limit is given): 6> include the target band combinations for which the number of CCs in DL is up to the value indicated in the requestedMaxCCsDL; 6> indicate value includedBothincludedDL in requestedMaximumCCs; 5> else if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsUL (i.e. only UL maximum limit is given): 6> include the target band combinations for which the number of CCs in UL is up to the value indicated in the requestedMaxCCsUL; 6> indicate value includedBothincludedUL in requestedMaximumCCs; Nokia: Duplicate of C.014 – closed (Corrected in general CR). |  | Closed (General CR) |
| E.228 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | An informative note that starts with FFS is most likely completely redundant. | 1 |  | Remove the note if the FFS cannot be removed. Coordinator: Remove the note. Nokia: Corrected in general CR by removing the note – the FFS itself can be discussed in RAN2#93. |  | Closed (General CR) |
| I.022 N.027 | 5.6.8 Measurements logging | > else if introduced after the NOTE 3 intended to cover all other conditions than previously conditioned “if’ and “else if” | 1 |  | Remove the “if” introduced by the CR after the NOTE 3 Nokia: This is a class 1 issue, and the correct placing is after NOTE 2 (new NOTE 0 introduced) - corrected in general CR by removing the “if”. |  | Closed (General CR) |
| S.031 | 5.6.8.2 | Seems preferrable not to refer to phases in stage 2 (not even done in IDC procedure | 1 |  | CATT: Agree. Nokia: Seems more like class 1 issue, so corrected NOTE by removing the words “as early as Phase 1”: NOTE 0: The UE may detect the start of IDC problems as described in section 23.4 of TS 36.300 [9]. |  | Closed (General CR) |
| S.030 | 5.6.8.2 (eMDT, IDC suspension) | The procedural structure for suppression of measurement logging due to IDC sinewhat breaks the current structure, that has one part specifying when the UE reports and another part specifying what the UE reports There is duplication/ redundancy regarding statements on suspension | 3 |  | In the initial bullet 1, add to the beginning: if not suspended due to IDC problems, bla.. After bullet 4 on locationCoordinates, chane bullet 3 as follows; if IDC was detected during the last logging interval, the UE includes the IDC indication and suspends logging CATT: Not sure if the change is needed, More details need to be provided. Coordinator: Corrected in general CR according to ad hoc meeting notes and argeements => Text proposal in R2-160046 is agreed. |  | Closed (General CR) |
| E.331 | 5.6.8.2 | 3> if the UE detects the IDC problems: Improve language | 1 |  | Rephrase to “ 3> if the UE detects the IDC problems:” (“the” removed) Intel: OK Nokia: This seems to have been corrected already in v13.0.0, so closed the issue. |  | Closed |
| S.032 | 5.6.8.2 (eMDT, IDC suspension) | There is an incorrect ‘else if’:. We understand that the else should be removed | 2 |  | Remove ‘if’ i.e. change to ‘else’ Nokia: This seems to be handled by Samsung contribution R2-160046, so closed the issue. |  | Closed |
| C.013 | 5.6.3.3 | Editorial correction | 1 |  | 6> indicate value includedUL in requestedMaximumCCs; 5> if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes skipFallbackCombinations: Coordinator: Corrected during v1300 CR implementation. |  | Closed |
| Z.014 | 5.6.3.3 | Bullet 5 is in the wrong line.Value includedUL should be italic. | 1 |  | Change: 5> else if UE supports requestedMaximumCCs and if the UECapabilityEnquiry message includes the requestedMaxCCsUL (i.e. only UL maximum limit is given): 6> include the target band combinations for which the number of CCs in UL is up to the value indicated in the requestedMaxCCsUL; 6> indicate value includedUL in requestedMaximumCCs; 5> if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes skipFallbackCombinations: Coordinator: Corrected during v1300 CR implementation |  | Closed |
| D.008 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | The following text is removed for the case where the UE supports requestedFrequencyBands. 5>- include all 2DL+1UL CA band combinations, only consisting of bands included in requestedFrequencyBands; - include all other 2DL+1UL CA band combinations; Although it is based on the agreement of changing the existing UE behaviour at RAN2#92, DCM would like to check if there is no backward compability issue. In particular, if the eNB is the legacy one supporting the legacy frequency selective mechanism and the UE supports this new frequency selective machanism, the eNB thinks that the reported 2DL+1UL CA band combinations are all supported by the UE although in fact some are filtered by the UE. | 2 |  | Coordinator: ASN.1 review verifies that agreement s are correctly implemented. Therefore it is better to discuss this during the next working group meeting. Closed for now and come back in Malta meeting. |  | Closed |
| H.039 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | 6> indicate value includedUL in requestedMaximumCCs; 5> if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes skipFallbackCombinations: | 1 |  | Should be 6> indicate value includedUL in requestedMaximumCCs; 5> if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes skipFallbackCombinations: Coordinator: Corrected during v1300 CR implementation |  | Closed |
| D.009 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | Style error on the following procedure text. 6> indicate value includedUL in requestedMaximumCCs; | 1 |  | Should be corrected as follows. 6> indicate value includedUL in requestedMaximumCCs; Coordinator: Corrected during v1300 CR implementation |  | Closed |
| D.011 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | Style error as shown below. 5> if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes skipFallbackCombinations: | 1 |  | Should be corrected as follow. 5> if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes skipFallbackCombinations: Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.227 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE T351 Expiry (WLAN Connection Timeout) 5.6.12a.5 WLAN Status Monitoring 5.10.5 Sidelink discovery monitoring 5.10.6 Sidelink discovery announcement 5.10.7.2 Initiation | There should be TABs between “>”and the procedure text. There should not be any SPACEs between “>” and the procedure text. | 1 |  | (If not already fixed in the v13.0.0 CR implementation) add TABs in the procedures and remove SPACEs. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.330 | 5.6.8.2 | 3> else if: Empty else if witout corresponding criteria | 2 |  | Remove “if” (No additional condition seems to be missing from the addition of this feature) CATT: agree Coordinator: Same as S.032. |  | Closed |
| E.229 | 5.6.8.2 Initiation | A new note is added by changing the numbering of existing notes. This is not according to specification drafting rules. | 1 |  | Renumber the new NOTE1 to NOTE5 and remove the changes on the existing NOTE1. i.e. NOTE 51: The UE may detect the start of the IDC problem as early as Phase 1 as described in TS 36.300 [9, 23.4]. Also add missing TABS to procedure text (if not already fixed in the v13.0.0 CR implementation). Coordinator: Corrected during v1300 CR implementation. The new NOTE is numbered as NOTE 0. |  | Closed |
| 5.8 MBMS |  |  |  |  |  |  |  |
| H.050 | 5.8.1.3 | SIBs are not received from "a carrier" like in the sentence "A UE interested to receive MBMS from a carrier on which dl-Bandwidth included in MasterInformationBlock is set to n6" | 1 |  | Reword "A UE interested to receive a MBMS from a cell on which dl-Bandwidth included in MasterInformationBlock is set to n6” Samsung: In case of MBSFN multiple cells contribute to the reception (but it considers MIB from one cell on that carrier) Nokia: the term carrier was delibarate for MBSFN and 5.8.1.3 only applies to MBSFN → no changes needed. Also this sub-clause describes MCCH acquisition which implicitly means it is used for MBSFN reception of MBMS Coordinator: No change needed. Can be closed |  | Closed (General CR) |
| E.066 | 5.8.1.3 | N6 should be italic in the following A UE that is receiving an MBMS service shall acquire the MCCH information from the start of each modification period. A UE interested to receive MBMS from a carrier on which dl-Bandwidth included in MasterInformationBlock is set to n6 shall acquire the MCCH information at least once every MCCH modification period. A UE that is not receiving an MBMS service, as well as UEs that are receiving an MBMS service but potentially interested to receive other services not started yet in another MBSFN area from a carrier on which dl-Bandwidth included in MasterInformationBlock is other than n6, shall verify that the stored MCCH information remains valid by attempting to find the MCCH information change notification at least notificationRepetitionCoeff times during the modification period of the applicable MCCH(s), if no MCCH information change notification is received. | 1 |  | Make n6 italic. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| H.051 | 5.8.5.2 | Duplicate word ''capable" | 1 |  | Remove duplicate Huawei: First sentence in 5.8.5.2. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.067 | 5.8.5.2 | Should it not be " else if SystemInformationBlockType20 is broadcasted by the PCell "? 3> else if SystemInformationBlockType20 is provided by the PCell: | 1 |  | Change to "broadcasted by the PCell"… maybe? Huawei: already covered by I 0.21 Intel: same as I.021. Coordinator: Same as I.021 |  | Closed |
| E.068 | 5.8.5.2 | Extra "capable" in the following: An MBMS or SC-PTM capable capable UE in RRC_CONNECTED | 1 |  | Remove extra "capable". Huawei: Already covered in H.051. |  | Closed |
| E.022 | 5.8.5.2 | Extra “clause” in first paragraph | 1 |  | Remove extra “clause” Huawei: Is it "capable" rather than "clause"? (covered in H.051). Coordinator: Same as H.051 |  | Closed |
| E.069 | 5.8a.1.3 | Missing space in the following: The modification period boundaries are defined by SFN values for which SFN mod m<here>= 0, where m is the number of radio frames comprising the modification period. | 1 |  | Add the missing space. Huawei: Agree. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| 5.9 RN procedures |  |  |  |  |  |  |  |
| 5.10 ProSe |  |  |  |  |  |  |  |
| E.110 | 5.10.2.3 | The following text does not use correct terminology. 3> if configured by upper layers to transmit relay related sidelink communication; and | 1 |  | Change text to 3> if configured by upper layers to transmit relay related one to many sidelink communication; and Huawei: Need to keep in mind that relay UE could transmit one to many sidelink communication. Intel: ok Qualcomm: No need, there can be eMBMS relay Samsung: Changed to one-to-one (one to many communication is assumed to cover both relay and non-relay). Some confirmation may be required |  | Closed (ProSe CR) |
| L.004 | 5.10.2.1, 5.10.2.2, 5.10.2.3 | Clarifying the purpose of reporting of information of neighbor cell is for sidelink discovery. | 1 |  | Sidelink operations Ssidelink operationsdiscovery Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| E.102 | 5.10.2.2 | To align with the term “non-relay related one to one sidelink communication”, the term “relay related sidelink communication” should be changed to “relay related one to one sidelink communication”. That makes it clear that relay related communication is one to one. 2> if configured by upper layers to transmit relay related sidelink communication: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or 3> if the last transmission of the SidelinkUEInformation message did not include commTxResourceReqRelay; or if the information carried by the commTxResourceReqRelay has changed since the last transmission of the SidelinkUEInformation message: 4> if the UE is acting as relay; or: 4> if the UE has a selected relay; and if SystemInformationBlockType19 is broadcast by the PCell and includes discConfigRelay; and if either discThreshHiRemoteUE is not included in SystemInformationBlockType19 or the RSRP measurement of the PCell is below discThreshHiRemoteUE; 5> initiate transmission of the SidelinkUEInformation message to indicate the relay related sidelink communication transmission resources required by the UE in accordance with 5.10.2.3; 2> else: 3> if the last transmission of the SidelinkUEInformation message included commTxResourceReqRelay: 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require relay related sidelink communication transmission resources in accordance with 5.10.2.3; | 1 |  | Change the text to 2> if configured by upper layers to transmit relay related one to one sidelink communication: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or 3> if the last transmission of the SidelinkUEInformation message did not include commTxResourceReqRelay; or if the information carried by the commTxResourceReqRelay has changed since the last transmission of the SidelinkUEInformation message: 4> if the UE is acting as relay; or: 4> if the UE has a selected relay; and if SystemInformationBlockType19 is broadcast by the PCell and includes discConfigRelay; and if either discThreshHiRemoteUE is not included in SystemInformationBlockType19 or the RSRP measurement of the PCell is below discThreshHiRemoteUE; 5> initiate transmission of the SidelinkUEInformation message to indicate the relay related one to one sidelink communication transmission resources required by the UE in accordance with 5.10.2.3; 2> else: 3> if the last transmission of the SidelinkUEInformation message included commTxResourceReqRelay: 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require relay related one to one sidelink communication transmission resources in accordance with 5.10.2.3; Huawei: fine to keep existing text. There is no explicit indication that the request is for for one to many.sidelink communication. Intel: ok (for clarification, so 1-to-1 communication is only allowed for relay?) Qualcomm: No need as there is eMBMS relay as well. Samsung: Some further discussion seems required during RAN2#93. It is noted that field commTxResourceReqRelay is using IE SL-CommTxResourceReqUC. |  | Closed (ProSe CR) |
| L.008 | 5.10.2.3 | No need to restrict simultaneouls occurrence of the procedure for providing information on sidelink communication/discovery rx/tx, the procedure for requesting gap and/or reporting neighbor cell information for sidelink operation. | 3 |  | 1> if the UE initiates the procedure to indicate it is (no more) interested to receive sidelink communication or discovery or to request (configuration/ release) of sidelink communication or discovery transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure): … 1> else (i.e. UE request transmission and/ or reception gaps): … else if the UE initiates the procedure to report the system information parameters related to Sidelink operations of carriers other than the primary 2> include discSL-SysInfoReportList and set it to report the system information parameter acquired from the cells on those carriers Correspondingly, the depth of each condition needs to be adjusted. Huawei: Not clear why to remove so many parts. Propose to accept the last two changes only. I.e., 1> else if the UE initiates the procedure to report the system information parameters related to Sidelink operations of carriers other than the primary 2> include discSL-SysInfoReportList and set it to report the system information parameter acquired from the cells on those carriers Samsung: When indicating Rx/ Tx resource request information, the UE always provides the complete information i.e. not just what changed compared to the last SidelinkUEInformation message. We discussed and endorsed that when requesting gaps, the UE should not include the full resource request information. We assume the same applies when reporting UE assistance for broadcasting inter-PLMN discovery resource configuration for uncoordinated PLMNs. Hence, we think the if statements should remain. We have no real problem to remove any preceding else, there seems no need to avoid that the unlikely event of multiple simultaneous triggers is handled by separate messages Intel: seems more discussion is needed. Ericsson: We have some sympathy for requesting/releasing several things in one message. We support this. ZTE: agree with the simultaneous occurrence. But the modification needs further discussion. The three level 1> condition are still necessary for better understanding. For the second 1>, it is better to explicitly say “if the UE initiates the procedure to request and/or reception gaps.” Qualcomm: Intention is not clear. Separate discussion paper is needed. Samsung: No changes for now. If there is interest to persue the proposal, a paper should be brought to RAN2#93 |  | Closed (ProSe CR) |
| L.009 | 5.10.4 | The remote UE applies threshold check during discovery operation. The reason for the UE in RRC connected not to apply threshold check during communication operation? Since the conditions of 3> needs to be checked for relay and/or remote UE, reordering of conditions is necessasry. | 3 |  | 1> if the transmission concerns relay communication; and the UE is capable of sidelink relay or remote operation: 2> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19; or 2> if the UE is acting as relay: 2> if the UE is in RRC_IDLE; and if the UE is selecting a relay/ has a selected relay: configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section, only if the following condition is met: 3> if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19; 2> configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section, only if the following condition is met: 3> if the UE configured lower layers with a pool of resources included in SystemInformationBlockType18 (i.e. commTxPoolNormalCommon or commTxPoolExceptional); and commTxAllowRelayCommon is included in SystemInformationBlockType18; or 3> if the UE configured lower layers with resources provided by dedicated signalling (i.e. commTxPoolResources set to scheduled or ue-Selected); and the UE is configured with commTxAllowRelayDedicated while its value is set to true 4> configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section: Huawei: Can further discuss in next meeting if the UE in RRC_CONNECTED applies the threshold, or the eNB can configure the resources based on measurement reports. Samsung: We are not so sure, but agree to change more discussion would be required Intel: not clear understanding on the issue. For connected, the UE will configure lower layers when it receives dedicated resource configuration by the eNB, i.e. not at the moment threshold condition is met. Ericsson: We are not sure we have understood the issue raised by LG. Maybe this can be clarified in a separate contribution somehow? We also have some concerns on this text (issue E.123), but it seems LG has different thoughts of the problem with this text. Qualcomm: Issue is not clear. Need discussion paper. Samsung: No change for now. Paper to RAN2#93 seems beneficial |  | Closed (ProSe CR) |
| E.285 | 5.10.6 | 1> else if, for the frequency used to transmit sidelink discovery on, the UE is configured with dedicated resources (i.e. with discTxResources in discTxInterFreqInfo within sl-DiscConfig); and the conditions for sidelink operation as defined in 5.10.1a are met: 2> if the UE is configured with discTxResources set to scheduled: 3> configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResources; 2> else if the UE is configured with discTxPoolDedicated (i.e. discTxResources set to ue-Selected): 3> select an entry of the list of resource pool entries in discTxPoolDedicated and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; 1> else if the frequency used to transmit sidelink discovery on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, while discTxResources in the corresponding entry of discInterFreqList is set to discTxPoolCommon (i.e. serving cell/ PCell broadcasts pool of resources) and the conditions for sidelink operation as defined in 5.10.1a are met; or 1> else if discTxPoolCommon is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: | 2 |  | In 5.10.1a is not considered the case in which the UE performs SL operations in non-serving cell. Therefore I would say that either we remove the highlighted sentence or we change section 5.10.1a. Probably better to review section 5.10.1a. Intel: what should be new addition to 5.10.1a if we consider SL operations in other carrier? If there is new and different, it would be good to clarify it into 5.10.1a. However at least for PS case, we assume Rel-12 inter-carrier D2D communication also follows 5.10.1a. Seems more discussion is needed. Huawei: Not sure. Even if the UE performs sidelink on non-serving carriers, the UE’s serrving cell should meet the conditons defined in 5.1.10.1a. Qualcomm: Not sure if understood the issue correctly. 5.10.1a was written for serving cell in current frequency. Samsung: As questions are raised, a more concrete proposal by a paper to RAN2#93 seems desirable. My understanding is that the intention is that intention is that 5.10.1a should also cover the case serving cell’s SIB19 indicates value requestDedicated for an inter-freq. |  | Closed (ProSe CR) |
| N.197 | 5.10.7.3 | It is unclear which section covers public safety discovery announcements for in coverage. The levels of conditions does not align with else statements. Reading the section makes it seem like syncTxPeriodic concerns PS as well. Not clear how this fits to this section | 4 |  | Needs further dsicussion – currently how the PS and non-PS features are captured is really unclear. Ericsson: We do not think this is unclear. In fact the first statement handles all sidelink discovery announcements in coverage. 1> if triggered by sidelink discovery announcement and in coverage on the frequency used for sidelink discovery, as defined in TS 36.304 [4, 11.4]: Maybe Nokia can clarify the issue in a contribution? Intel: first would like to see more discussion to clarify the issue. Huawei: The issue is not crystal clear. Can further discuss this if needed. Samsung: We see no need for changes Nokia Networks: Will attempt to clarify the issue in contribution to AH or to RAN2#93. The intent was to consider how we can see which features belong to Public Safety functionality and which do not. Qualcomm: Separate discussion paper is needed, as proposal is not clear. No need to change the text. |  | Closed (ProSe CR) TDoc Nokia RAN2#93 |
| E.289 | 5.10.8.2 | 1> for the frequency used for sidelink communication, if out of coverage on that frequency as defined in TS 36.304 [4, 11.4] and for each other frequency used for sidelink discovery on which the UE is out of coverage, as defined in TS 36.304 [4, 11.4]: | 2 |  | Why each other frequency? Should not be each frequency? Huawei: The frequency used for sidelink communication could be same as that for sidelink discovery. Fine with the orignal wording. Samsung: Intention is to avoid performing the action twice when the frequency is used for both sidelink communication and discovery Intel: Shouldn’t be “the frequency”? Qualcomm: Agree with the issue. Similar to communication, out of coverage discovery is supported on only one frequency (that is why we have only one preconfigured frequency even for discovery). Text can be : 1> for the frequency used for sidelink communication, if out of coverage on that frequency as defined in TS 36.304 [4, 11.4] and for each other frequency used for sidelink discovery on which the UE is out of coverage, as defined in TS 36.304 [4, 11.4]: Samsung: No change (concerns expressed). If still concerns about the current text, a proposal should be brought to RAN2#93. Relates to inter-frequency support for PS |  | Closed (ProSe CR) |
| E.151 | 5.10.12.2 | Wrong terminology in heading #### 5.10.12.2 AS- conditions relay communication transmission for remote UE | 1 |  | Change heading to #### 5.10.12.2 AS-conditions for relay related one to one sidelink communication transmission for sidelink remote UE Huawei: it is true that remote UEs do not transmit one to many sidelink communication, but it is unnecessary to be so specific. Fine with other changes except “one to one”. Intel: ok Qualcomm: Ok. However “one to one” is not required. Samsung: Handled same as E.145 |  | Closed(ProSe CR) |
| E.142 | 5.10.11.1 | Imprecise language, terminology. This procedure is used by a UE supporting sidelink relay operation and involves evaluation of the AS-layer conditions that need to be met in order for upper layers to configure a relay UE to receive/ transmit relay related sidelink discovery/ communication. The AS-layer conditions merely comprise of being configured with radio resources that can be used for transmission. | 1 |  | Change text to This procedure is used by a UE supporting sidelink relay UE operation and involves evaluation of the AS-layer conditions that need to be met in order for upper layers to configure a sidelink relay UE to receive and transmit relay related one to one sidelink communication and PS relay related sidelink discovery. The AS-layer conditions merely comprise of being configured with radio resources that can be used for transmission. Huawei: Agree with changes except “one to one sidelink communication”. Relay UE can also transmit one to many sidelink communication. Intel: no strong view. For clarification, so only 121 communication is allowed for UE-to-NW relay operation? Qualcomm: Ok. However following text is not required, as there can be eMBMS relay as well “one to one sidelink” Samsung: Partially included (generally no distinction one-to-one/ one-to-many communication |  | Closed (ProSe CR) |
| C.034 | #### 5.10.11.1 General | The last sentense of the first paragraph of 5.10.11.1 is not clear and since there is detail description on the contents of the AS-layer conditions, it can be deleted. | 2 |  | #### 5.10.11.1 General This procedure is used by a UE supporting sidelink relay operation and involves evaluation of the AS-layer conditions that need to be met in order for upper layers to configure a relay UE to receive/ transmit relay related sidelink discovery/ communication. The AS-layer conditions merely comprise of being configured with radio resources that can be used for transmission. Huawei: Neutral. There could be some motivation for this sentence. Intel: ok Ericsson: Agree. Qualcomm: The suggested text (for deletion), actually suppose to stich section 5.10.11.1 and 5.10.11.2 so that interlay interaction is somehow handled. Samsung: No change for now. Seems best to be considered (as part of discussion on inter-layer interaction for which Samsung will bring paper) |  | Closed (ProSe CR) |
| E.143 | 5.10.11.2 | 1) Imprecise language, terminology. 2) commTxPoolResources is incorrect, it should be commTxResources. 3) As commTxResources can only be either scheduled or ue-selected, it seems unnecessary to state that. A UE capable of sidelink relay operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met: 1> if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its value is set to true; | 1 |  | Change text to A UE capable of sidelink relay UE operation shall inform upper layers that it is configured with radio resources that can be used for relay related one to one sidelink communication transmission if the following conditions are met: 1> if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its value is set to true; Huawei: Original words “relay related sidelink communication” was correct to us. Should also consider relay one to many sidelink communication. The last change should be “commTxResources set to scheduled or ue-Selected” Intel: ok, but to be more accurate, we may better say sidelink UE-to-NW relay UE operation Qualcomm: Same as Z.038. So CommTxPoolResources should be changed to CommTxResources. However no need to add “one to one” as there can be eMBMS relay as well. General comment: CommTxPoolResources is used in other places as well and can be changed to CommTxResources in those places as well. Samsung: Partially included (generally no distinction one-to-one/ one-to-many communication) |  | Closed (ProSe CR) |
| E.144 | 5.10.11.2 | Heading can be improved #### 5.10.11.2 AS- conditions communication transmission for relay UE | 1 |  | Change heading to 5.10.11.2 AS-conditions for relay related one to one sidelink communication transmission for sidelink relay UE Huawei: Change to “AS- conditions for sidelink communication transmission for relay UE”. Should also consider relay one to many. Intel: ok Qualcomm: Ok. However, no need to add “one to one” Samsung: Partially changed (also other similar titles changed in 5.10.10/ 5.10.11, but generally no distinction one-to-one/ one-to-many communication) |  | Closed (ProSe CR) |
| E.145 | 5.10.11.3 | Heading can be improved 5.10.11.3 AS- conditions discovery transmission for relay UE | 1 |  | Change heading to 5.10.11.3 AS-conditions for PS relay related sidelink discovery transmission for sidelink relay UE Huawei: Fine. Intel: ok, but to be more accurate, it may be better to say sidelink UE-to-NW relay UE operation. Qualcomm: Ok Samsung: See E.144 |  | Closed (ProSe CR) |
| E.146 | 5.10.11.3 | Wrong terminology A UE capable of sidelink relay operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink discovery transmission if the following conditions are met: | 1 |  | Change text to A UE capable of sidelink relay UE operation shall inform upper layers that it is configured with radio resources that can be used for PS relay related sidelink discovery transmission if the following conditions are met: Huawei: Fine. Intel: ok, but to be more accurate, it may be better to say sidelink UE-to-NW relay UE operation. Qualcomm: Ok |  | Closed (ProSe CR) |
| E.147 | 5.10.11.3 | Superfluous text. 1> else if in RRC_CONNECTED; and if discTxResourcesPS is configured (i.e. set to scheduled or ue-Selected); | 1 |  | Change text to 1> else if in RRC_CONNECTED; and if discTxResourcesPS is configured (i.e. set to scheduled or ue-Selected); Huawei: Suggest sticking to the original. Intel: ok Qualcomm: No strong opinion either way. |  | Closed (ProSe CR) |
| C.035 | #### 5.10.11.3 AS- conditions discovery transmission for relay UE | The discConfigRelay is used to configure the relay UE’s Uu RSRP upper/lower threshold. This parameter is optional configurable，Thus there is no need to force this condition must be satisfied。 | 2 |  | #### 5.10.11.3 AS- conditions discovery transmission for relay UE A UE capable of sidelink relay operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink discovery transmission if the following conditions are met: 1> if in RRC_IDLE; and if the UE’s serving cell is suitable as defined in TS 36.304 [4]; and if SystemInformationBlockType19 includes discConfigPS including discTxPoolPS-Common and discConfigRelay: 2> if neither discThreshHiRelayUE nor discThreshLoRelayUE is included in SystemInformationBlockType19; 2> else if AS-layer conditions specified in this section were not met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE by discHystMaxRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE by discHystMinRelayUE, if included in SystemInformationBlockType19 (entry); 2> if AS-layer conditions specified in this section were met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19 (stay); 1> else if in RRC_CONNECTED; and if discTxResourcesPS is configured (i.e. set to scheduled or ue-Selected); Huawei: Do not agree the change. This is to indicate the cell supporting relay operation. Samsung: Field discConfigRelay indicates that relay operation is supported, and hence needs to be checked Intel: we think the original text is correct. Ericsson: We disagree. discConfigRelay must be included for the UE to use relaying in that cell. ZTE: Agree with the problem. However, the modification needs further discussion. Suggest to divide the first “1 level if condition” into two if conditions, one is discConfigRelay included, the other one is discConfigRelay included not included. Qualcomm: No need to change, current text is right. We have agreement that if relay related information is not present then don’t perform relay operation. DiscConfigRelay is the information. Samsung: No change seems needed (note that field is also used to indicate UE is allowed to request relay related resources, as discussed during Ad Hoc) |  | Closed (ProSe CR) |
| N.198 | 5.10.11.3, 5.10.12.3 | For this sentence: “else if AS-layer conditions specified in this section were not met” It is not clear which is the section, which requirements | 1 |  | Clarify which requirements are intended here. Huawei: Agree to improve this sentence. Samsung: There is a hysteresis beteen the conditions for entry and leave of ‘AS conditions for discovery transmission for relay UE’. Wording can possibly be improved, or some kind of state could be introduced (may be covered by contribution on terminology, modelling see E.082) Intel: probably better to clarify what AS-layer conditions mentions here. Qualcomm: current text is fine. Somewhat related to C.034. “else if AS-layer conditions specified in this section were not met” becomes clear when 5.10.11.1/2/3 are read together. Samsung: Merely aligned terminology in both sections to ‘the AS-layer radio conditions). |  | Closed (ProSe CR) |
| E.149 | 5.10.12 | Wrong terminology in heading ### 5.10.12 Sidelink remote operation | 1 |  | Change heading to ### 5.10.12 Sidelink remote UE operation Huawei: Agree. Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| E.150 | 5.10.12.1 | Imprecise language, terminology This procedure is used by a UE supporting sidelink remote operation and involves evaluation of the AS-layer conditions that need to be met in order for upper layers to configure a remote UE to receive/ transmit relay related sidelink discovery/ communication. The AS-layer conditions merely comprise of being configured with radio resources that can be used for transmission, as well as whether or not having a selected relay. | 1 |  | Change text to This procedure is used by a UE supporting sidelink remote UE operation and involves evaluation of the AS-layer conditions that need to be met in order for upper layers to configure a sidelink remote UE to receive and transmit relay related one to one sidelink communication and PS relay related sidelink discovery. The AS-layer conditions merely comprise of being configured with radio resources that can be used for transmission, as well as whether or not having a selected sidelink relay UE. Huawei: Fine to changes except “one to one”. Intel: ok, but to be more accurate, it may be better to say sidelink UE-to-NW relay UE. Qualcomm: Ok. However “one to one” is not required because there can be eMBMS relay. Samsung: Handled same as E.142 |  | Closed (ProSe CR) |
| E.152 | 5.10.12.2 | Wrong terminology A UE capable of sidelink remote operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met: | 1 |  | Change text to A UE capable of sidelink remote UE operation shall inform upper layers that it is configured with radio resources that can be used for relay related one to one sidelink communication transmission if the following conditions are met: Huawei: Fine with the changes except “one to one”. Intel: ok Qualcomm: No need to change. Relay operation can be for eMBMS relay as well |  | Closed (ProSe CR) |
| E.153 | 5.10.12.2 | If statement is incorrect, should be else if 2> if the AS-layer radio conditions specified in this section were previously met; and the RSRP measurement of the PCell is below discThreshHiRemoteUE (stay); | 1 |  | Change text to 2> else if the AS-layer radio conditions specified in this section were previously met; and the RSRP measurement of the PCell is below discThreshHiRemoteUE (stay); Huawei: Suggest changing to 2> if discThreshHiRemoteUE is not included in SystemInformationBlockType19; or 2> if the AS-layer radio conditions specified in this section were previously not met; and if discConfigRelay is included in SystemInformationBlockType19 while the RSRP measurement of the PCell is below discThreshHiRemoteUE by discHystMaxRemoteUE (entry); or 2> if the AS-layer radio conditions specified in this section were previously met; and if discConfigRelay is included in SystemInformationBlockType19 while the RSRP measurement of the PCell is below discThreshHiRemoteUE (stay); Intel: ok Qualcomm: Agree Samsung: Added else (to both 5.10.10 and 5.10.11), as well as the addition suggested by Huawei (so sections are aligned now) |  | Closed (ProSe CR) |
| E.154 | 5.10.12.2 | Wrong field names and superfluous description. Also, should be “else if” 1> if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its value is set to true; | 1 |  | Change text to 1> else if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its values is set to true; Huawei: It is fine to have or not to have a “else” here, but it is better to be consistent in the whole section for ProSe. Intel: ok Qualcomm: Agree with else if and field name change. For rest of the change (i.e. set to scheduled or ue-Selected)no strong opinion. |  | Closed (ProSe CR) |
| E.155 | 5.10.12.3 | Wrong terminology in heading #### 5.10.12.3 AS- conditions discovery transmission for remote UE | 1 |  | Change heading to #### 5.10.12.3 AS-conditions for PS relay related sidelink discovery transmission for sidelink remote UE Huawei: agree. Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| N.200 | 5.10.12.4 | it seems that Relay discovery can be done only when discovery configurations with postfix ‘PS’ are configured at the eNB while to be a remote UE you do not have to consider any resources that have ‘PS’ postfix. | 2 |  | Consider changing discConfigRelay to say discConfigRelayPS to clarify the intent? Huawei: only find discConfigRelay“ in 5.10.12.3. But fine with this proposal. Intel: ok Ericsson: We cannot find this statement in section 5.10.12.4. In general we understand Relay-related discovery to be one of two subsets of PS related discovery (the other being non-relay related discovery e.g. group member discovery). Qualcomm: OOC discovery is anyway for PS. So no need as such. Samsung: No change for now (no real need i.e. to distinguish) |  | Closed (ProSe CR) |
| N.199 | 5.10.12.4 | No such field as "reselectionInfoRemoteUE-OC“ | 1 |  | Field name should be reselectionInfoRemoteUE-OoC-r13. Huawei: agree. Intel: ok, but field name should be added without suffix “-r13”. Qualcomm: Agree |  | Closed (ProSe CR) |
| N.201 | 5.10.12.4 | Unclear "or " in conditions: “reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage); or 2> else” | 1 |  | Remove "or" or add missing level 3> statement (not sure which ist he case) Huawei: There are two alternative consitions for relay reselection. Change to 2> else if SD-RSRP of the currently selected relay is below q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage); or 2> else if upper layers indicate not to use the currently selected relay: (i.e. relay reselection): 3> select a candidate relay which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage) by minHyst; Intel: ok (if we remove “or”, “else” in the level 3> is ok? Or should it be “else if”?) Qualcomm: Agree that „or“ Should be removed Samsung: Changed by changing to or if upper layers (i.e. no additional bullet 2 |  | Closed (ProSe CR) |
| E.158 | 5.10.12.4 | Wrong terminology in heading #### 5.10.12.4 Selection and reselection of relay | 1 |  | Change heading to #### 5.10.12.4 Selection and reselection of sidelink relay UE Huawei: fine. Intel: ok, but to be more accurate, we may better say sidelink UE-to-NW relay UE. Qualcomm: Ok |  | Closed (ProSe CR) |
| E.159 | 5.10.12.4 | Wrong terminology and a strange “or else” construct. A UE capable of sidelink remote operation that is configured by upper layers to search for a sidelink relay shall: 1> if out of coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4, 11.4]; or 1> if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below discThreshHiRemoteUE: 2> search for candidate relays, in accordance with TS 36.133 [16] 2> when evaluating the one or more detected relays, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the preconfigured filterCoefficient as defined in 9.3, before using the SD-RSRP measurement results; NOTE 1: The details of the interaction with upper layers are up to UE implementation. 2> if the UE does not have a selected relay: 3> select a candidate relay which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage) by minHyst; 2> else if SD-RSRP of the currently selected relay is below q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage); or 2> else upper layers indicate not to use the currently selected relay: (i.e. relay reselection): 3> select a candidate relay which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage) by minHyst; 2> else if the UE did not detect any candidate relay which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage) by minHyst: 3> consider no relay to be selected; NOTE 2: The UE may perform relay reselection in a manner resulting in selection of the sidelink relay, amongst all candidate relays meeting higher layer criteria, that has the best radio link quality. Further details, including interaction with upper layers, are up to UE implementation. | 1 |  | Change text to A UE capable of sidelink remote UE operation that is configured by upper layers to search for a sidelink relay UE shall: 1> if out of coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4, 11.4]; or 1> if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below discThreshHiRemoteUE: 2> search for candidate sidelink relay UEs, in accordance with TS 36.133 [16] 2> when evaluating the one or more detected sidelink relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the preconfigured filterCoefficient as defined in 9.3, before using the SD-RSRP measurement results; NOTE 1: The details of the interaction with upper layers are up to UE implementation. 2> if the UE does not have a selected sidelink relay UE: 3> select a candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OoC (out of coverage) by minHyst; 2> else if SD-RSRP of the currently selected sidelink relay UE is below q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OoC (out of coverage); or if upper layers indicate not to use the currently selected sidelink relay UE: (i.e. sidelink relay UE reselection): 3> select a candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OoC (out of coverage) by minHyst; 2> else if the UE did not detect any candidate sidelink relay UE which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OoC (out of coverage) by minHyst: 3> consider no sidelink relay UE to be selected; NOTE 2: The UE may perform sidelink relay UE reselection in a manner resulting in selection of the sidelink relay UE, amongst all candidate sidelink relay UEs meeting higher layer criteria, that has the best radio link quality. Further details, including interaction with upper layers, are up to UE implementation. Huawei: Fine with the changes. We suggested another way for the change in N.201. But fine with either way. Intel: ok, but to be more accurate, we may better say sidelink UE-to-NW relay UE. Qualcomm: Ok |  | Closed (ProSe CR) |
| L.017 | 5.10.12.4 (D2D) | The IC UE uses the fiterCoefficient in SIB19. | 2 |  | 1> if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below discThreshHiRemoteUE: 2> search for candidate relays, in accordance with TS 36.133 [16] 2> when evaluating the one or more detected relays, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the fiterCoefficient in SystemInformationBlockType19 (in coverage) or the preconfigured filterCoefficient (out of coverage) as defined in 9.3, before using the SD-RSRP measurement results; Huawei: Gernerally fine with the proposal, but only fine the filterCoefficient in preconfiguration for Rel-12 reference selection. Continue to use the same filterCoefficient? Intel: ok (b.t.w. an “l” is missing in fiterCoefficient. Ericsson: Agree. Qualcomm: Agree |  | Closed (ProSe CR) |
| C.037 | #### 5.10.12.4 Selection and reselection of relay | - The full name of abbreviations SD-RSRP should be capture in section 3.2. - The parameter “reselectionInfoRemoteUE-OC”is missing in the ASN.1. | 1 |  | 5.10.12.4 Selection and reselection of relay A UE capable of sidelink remote operation that is configured by upper layers to search for a sidelink relay shall: 1> if out of coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4, 11.4]; or 1> if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below discThreshHiRemoteUE: 2> search for candidate relays, in accordance with TS 36.133 [16] 2> when evaluating the one or more detected relays, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the preconfigured filterCoefficient as defined in 9.3, before using the SD-RSRP measurement results; NOTE 1: The details of the interaction with upper layers are up to UE implementation. 2> if the UE does not have a selected relay: 3> select a candidate relay which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage) by minHyst; 2> else if SD-RSRP of the currently selected relay is below q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage); or 2> else upper layers indicate not to use the currently selected relay: (i.e. relay reselection): 3> select a candidate relay which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage) by minHyst; 2> else if the UE did not detect any candidate relay which SD-RSRP exceeds q-RxLevMin included in either reselectionInfoRemoteUE-IC (in coverage) or reselectionInfoRemoteUE-OC (out of coverage) by minHyst: 3> consider no relay to be selected; NOTE 2: The UE may perform relay reselection in a manner resulting in selection of the sidelink relay, amongst all candidate relays meeting higher layer criteria, that has the best radio link quality. Further details, including interaction with upper layers, are up to UE implementation. Huawei: fine to clarify the abbreviation of SD-RSRP. Intel: ok (for 2), should be corrected according to N.199) Qualcomm: Similar to N.199 and E.159 Samsung: Introduced abbreviation (other parts already covered by other items) |  | Closed (ProSe CR) |
| N.188 | 5.10.2 | “include discTxGapRequest and set it to indicate, for each frequency that either concerns” No such field name in ASN.1 | 1 |  | Correct the field name, should be discTxGapReq Huawei: agree. Intel: ok Qualcomm: Agree |  | Closed (ProSe CR) |
| N.194 | 5.10.2 | What does e.g. this mean – what is “non-relay PS related trasnsmission”, and where is it defined?? “if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency or, in case of non-relay PS related transmission,” | 1 |  | Is “non-relay PS related transmission” related to discovery announcements (as may indicate the context)? If so, please add this info. Huawei: Propose to change to “in case of non-relay PS related sidelink discovery announcements” Intel: not clear on the issue. Qualcomm:No need as such |  | Closed (ProSe CR) |
| H.053 | 5.10.2.1, 5.10.2.2 | "Sidelink operations" -> "sidelink operation" (several places) | 1 |  | Intel: ok |  | Closed (ProSe CR) |
| Z.022 | 5.10.2.2 | In R13, we had never discussed the non-relay related one to one sidelink communication and no agreements have been reached for the non-relay related one-to-one sidelink communication. Therefore, it is not appropriate to specify this scenario in 36.331 and it is suggested to remove all the relevant descriptions and IEs. | 3 |  | The original text is as follows which describe the non-relay related one to one sidelink communication. 2> if configured by upper layers to transmit non-relay related one to one sidelink communication: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or 3> if the last transmission of the SidelinkUEInformation message did not include commTxResourceReq121; or if the information carried by the commTxResourceReq121 has changed since the last transmission of the SidelinkUEInformation message: 4> initiate transmission of the SidelinkUEInformation message to indicate the non-relay related sidelink one to one communication transmission resources required by the UE in accordance with 5.10.2.3; Huawei: Do not agree the proposal. Non-relay one to one communication has been supported by SA2, and this was discussed a litlle bit in the 331 CR review email discussion before RAN2#92.Samsung: We think RAN2 made several agreements related to one to one SL communication (D2D BSR group index, MAC PDU format, PDCP security). In RRC we just separate relay and non-relay as for relay there are some additional aspects. I.e. we think no changes are needed Intel: further discussion is needed. Ericsson: We disagree with the problem raised by ZTE. We did agree to have e.g. unicast identifiers. Qualcomm: Current text is fine, no need to change. We discussed and agreed that One-to-one communication can be related to relay or non-relay. That is why we have separate list of destination IDs Samsung: No change as no support. If there are still concerns a contribution needs to be brought to RAN3#93 |  | Closed (ProSe CR) |
| C.025 | 5.10.2.2 | For a UE only interested in one-to-one communication, when it is handovered to a R12 eNB which broadcasts SIB18 from a R13 eNB which broadcasts SIB 18, according to the current event, the UE would not initiate the SidelinkUEInformation transmission procedure if the information in SidelinkUEInformation is not changed. Even the resource eNB has forwarded the related sidelink paramters to the target eNB during handover procedure, the target eNB can not understand the destinationInfoListUC since it’s a R12 eNB. Then if the UE is further handovered toa R13 eNB which broadcast SIB18, according to the current event, the UE would not initiate the SidelinkUEInformation transmssion procedure if the information in SidelinkUEInformation is not changed. The source eNB would not forward the destinationInfoListUC to target eNB during the handover procedure. The eNB would not provide the resources for one-to-one communication even it’s a R13 eNB During a R13 to R12 to R13 handover, the UE should initiate the SidelinkUEInformation transmission procedure to request the one-to-one communication resources after connecting to a R12 eNB. There is also similar issue for relay communication/PS discovery/gap initiated SidelinkUEInformation transmission procedure. | 3 |  | UE should be able to identify whether the eNB which broadcasts the SIB18 is a R12 or R13 eNB. Samsung: Seems good to somewhat extend the condition ‘the UE connected to a PCell not broadcasting SystemInformationBlockType18’ i.e. to cover connecting to a PCell not supporting all items the UE would included in SideLinkUEInformation Intel: the need is not clear to us. May need further discussion. Ericsson: This is an interesting problem. We think we need to add one boolean to SIB18 which indicates if the UE is allowed to request non-relay related one-to-one sidelink communication. With that, we could change the following line 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or ...to something like 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18 or broadcasting SystemInformationBlockType18 but not including ourNewParameter; or As we have a similar parameter for requesting gaps, that change would be straightforward. ZTE: looks reasonable Huawei: understand the issue raised, but the proposed change is not clear. Suggest discussing in next RAN2 meeting if needed. Qualcomm: Requires discussion Coordinator: AS release is specified only for the UE. Transmitted SIB18 maybe based on R12 or R13 syntax but the eNB does not have a Release.So if something needs to be indicated it should be the SIB version. Samsung: Assumed to be covered by latest changes on repetition following HO. If anything further is needed, a contribution is to b brought |  | Closed (ProSe CR) |
| Z.023 | 5.10.2.2 | It may happen that the discTxResources is included in SIB19 and set to acquireSI-FromCarrier. In this case, we think that the UE does not need to initiate the transmission of SidelinkUEInformation message indicate the sidelink discovery announcement resources required. So it is suggeseted that the condition should be modified as not set to noTxOnCarrier and not set to acquireSI-FromCarrier. | 4 |  | The original text is as follows: 2> if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements on the primary frequency or on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResources included and not set to noTxOnCarrier: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType19; or 3> if the last transmission of the SidelinkUEInformation message did not include discTxResourceReq; or if the sidelink discovery announcement resources required by the UE have changed (i.e. resulting in a change of discTxResourceReq) since the last transmission of the SidelinkUEInformation message: 4> initiate transmission of the SidelinkUEInformation message to indicate the sidelink discovery announcement resources required by the UE in accordance with 5.10.2.3; 2> if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency or, in case of non-relay PS related transmission, on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResourcesPS included and not set to noTxOnCarrier: It is suggested to change as follows: 2> if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements on the primary frequency or on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResources included and not set to noTxOnCarrier and not set to acquireSI-FromCarrier: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType19; or 3> if the last transmission of the SidelinkUEInformation message did not include discTxResourceReq; or if the sidelink discovery announcement resources required by the UE have changed (i.e. resulting in a change of discTxResourceReq) si nce the last transmission of the SidelinkUEInformation message: 4> initiate transmission of the SidelinkUEInformation message to indicate the sidelink discovery announcement resources required by the UE in accordance with 5.10.2.3; 2> if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency or, in case of non-relay PS related transmission, on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResourcesPS included and not set to noTxOnCarrier and not set to acquireSI-FromCarrier: Huawei: Suggest keeping the original text for the time being, and to discuss in next meeting if needed. Samsung: Note that this was discussed/ changed during e-mail review (i.e. based on a comment from Interdigital) Intel: it is for connected UE behaviour, so like Rel-12, it will send SidelinkUEInformation even though tx resource is included in SIB19, so we think the original text is correct. Or we may need further discussion. Ericsson: Agree. Samsung: Covered by R2-160013, R2-160020 (partly concluded). No change (no support). If changes are still desired, a contribution is to be brought to RAN3#93 |  | Closed (ProSe CR) |
| L.005 | 5.10.2.2 | Add ‘if included in SystemInformationBlockType19 of PCell’ | 2 |  | 4> if the UE is acting as relay; or;: 4> if the UE has a selected relay; and if SystemInformationBlockType19 is broadcast by the PCell and includes discConfigRelay; and if either discThreshHiRemoteUE is not included in SystemInformationBlockType19 or the RSRP measurement of the PCell is below discThreshHiRemoteUE if included in SystemInformationBlockType19 of PCell;: Huawei: Not so necessary. Fine to have or not to have this change.Samsung: Does not seem really needed Intel: doesn’t it already covered by the previous text “and if SystemInformationBlockType19 is broadcast by the PCell and includes discConfigRelay;”? Ericsson: Disagree. The condition already checks if discThreshHiRemoteUE is included. Qualcomm: Ok Samsung: Did some alignment of these condition i.e. to the following kind: and the RSRP measurement of the PCell is below threshHigh if included in remoteUE-Config |  | Closed (ProSe CR) |
| L.006 | 5.10.2.2 | Replace ‘:’ with ‘;’ Add ‘if included in SystemInformationBlockType19 of PCell’ In order to allow relay and/or remote UE to transmit sidelinkUEInformation without checking the threshold when they are intended to transmit non-relay related sidelink discovery announcement information, it is proposed to separate the paragraphs for relay and non-relay cases. | 2 |  | 4> if the UE is acting as relay; and if SystemInformationBlockType19 includes discConfigRelay; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19:; or 4> if the UE is selecting a relay/ has a selected relay; and if SystemInformationBlockType19 includes discConfigRelay; and if discThreshHiRemoteUE is not included or the RSRP measurement of the PCell is below discThreshHiRemoteUE if included in SystemInformationBlockType19 of PCell: Huawei: Fine with first change. Neutral with the second. Samsung: Same as for L.005 Intel: may be we can modify such as the above bullet? E.g. “and if SystemInformationBlockType19 is broadcast by the PCell and includes discConfigRelay;” Ericsson: Agree to adding the “or”. Disagree to the second addition. The condition already checks if discThreshHiRemoteUE is included. Qualcomm: Ok Samsung: See L.005 |  | Closed (ProSe CR) |
| H.054 | 5.10.2.2 | Shouldn't "since entering RRC_CONNECTED state" be "since last entering RRC_CONNECTED state" like in legacy text? (in two places) | 2 |  | Samsung: Seems good to align Intel: not sure what the issue is. In the legacy, there is “since entering RRC_CONNECTED state” Ericsson: It seems “since entering” and “since last entering” both exist in legacy text. Some aligning could be useful. We have a small preference for “since last entering” in that case. Qualcomm: No need to change, as both formulations are present in legacy. Samsung: Adopted ‘since last entering’ |  | Closed (ProSe CR) |
| H.055 | 5.10.2.2 | 2> if the UE is configured to report parameters related to Sidelink operations from system information of carriers other than the primary | 1 |  | 2> if the UE is configured to report parameters related to Sidelink operations from system information of frequenciescarriers other than the primary frequency Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| C.024 | 5.10.2.2 | A capability bit for reporting inter-frequency sidelink parameters in SIB is introduced. Therefore, only the UEs with that capability may initiate the procedure to report inter-frequency sidelink paremters in SIB | 1 |  | A UE capable of sidelink communication or discovery may initiate the procedure to request assignment of dedicated resources for the concerned sidelink communication transmission or discovery announcements and a UE capable of reporting inter-frequency Sidelinke parameters in SIB may initiate the procedure to report parameters related to Sidelink operations from system information of inter-frequency cells. Huawei: Not necessary to be so specific. Intel: ok Qualcomm: Agree |  | Closed (ProSe CR) |
| E.094 | 5.10.2.2 | The addition of “(relay)” is confusing and does not improve the understanding of the note. Relay communication is a subset of sidelink communication and is thus inlcuded in the general term “sidelink communication”. NOTE 1: A UE in RRC_IDLE that is configured to transmit sidelink (relay) communication/ discovery announcements, while SystemInformationBlockType18/ SystemInformationBlockType19 does not include the resources for transmission (in normal conditions), initiates connection establishment in accordance with 5.3.3.1a. | 1 |  | Remove “(relay)” from the note. Huawei: Fine with either way. Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| E.098 | 5.10.2.2 | The term “one to many sidelink communication” is not clearly defined. | 1 |  | Add definition or description to section 5.10.4. Sidelink communication consists of one to many and one to one sidelink communication. One to one sidelink communication consists of relay related and non-relay related one to one sidelink communication. In relay related one to one sidelink communication the communicating parties consist of one sidelink relay UE and one sidelink remote UE. This text can also be incorporated in section 3.1 Defintions. Huawei: Not clear where to put this definition and the definition has a problem. In our understanding, relay sidelink communication includes one to many sidelink communication. Qualcomm: Name itself is enough for clarity. Also the proposed definition doesnot cover eMBMS relay. Samsung: Text like this was added to 5.10.1 |  | Closed (ProSe CR) |
| E.099 | 5.10.2.2 | The term “one to many” is missing in the following statement: 4> initiate transmission of the SidelinkUEInformation message to indicate the sidelink communication transmission resources required by the UE in accordance with 5.10.2.3; | 1 |  | Change the statement to: 4> initiate transmission of the SidelinkUEInformation message to indicate the one to many sidelink communication transmission resources required by the UE in accordance with 5.10.2.3; Huawei: fine to keep existing text. There is no explicit indication that the request is for for one to many.sidelink communication. Intel: no strong view, however considering we don’t have any associated IE for 1-to-M, the original text seems ok to us. Qualcomm: No need. Samsung: No change performed (generally no such distinction) |  | Closed (ProSe CR) |
| E.100 | 5.10.2.2 | The term “one to many” is missing in the following statement: 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require sidelink communication transmission resources in accordance with 5.10.2.3; | 1 |  | Change the statement to: 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require one to many sidelink communication transmission resources in accordance with 5.10.2.3; Huawei: fine to keep existing text. There is no explicit indication that the request is for for one to many.sidelink communication. Intel: no strong view, however considering we don’t have any associated IE for 1-to-M, the original text seems ok to us. Qualcomm: No need. Samsung: No change performed (generally no such distinction) |  | Closed (ProSe CR) |
| E.101 | 5.10.2.2 | The term “non-relay related one to one sidelink communication” is not aligned in the following text. 2> if configured by upper layers to transmit non-relay related one to one sidelink communication: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or 3> if the last transmission of the SidelinkUEInformation message did not include commTxResourceReq121; or if the information carried by the commTxResourceReq121 has changed since the last transmission of the SidelinkUEInformation message: 4> initiate transmission of the SidelinkUEInformation message to indicate the non-relay related sidelink one to one communication transmission resources required by the UE in accordance with 5.10.2.3; 2> else: 3> if the last transmission of the SidelinkUEInformation message included commTxResourceReq121: 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require non-relay related sidelink one to one communication transmission resources in accordance with 5.10.2.3; | 1 |  | Change the text to the following: 2> if configured by upper layers to transmit non-relay related one to one sidelink communication: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or 3> if the last transmission of the SidelinkUEInformation message did not include commTxResourceReq121; or if the information carried by the commTxResourceReq121 has changed since the last transmission of the SidelinkUEInformation message: 4> initiate transmission of the SidelinkUEInformation message to indicate the non-relay related one to one sidelink communication transmission resources required by the UE in accordance with 5.10.2.3; 2> else: 3> if the last transmission of the SidelinkUEInformation message included commTxResourceReq121: 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require non-relay related one to one sidelink communication transmission resources in accordance with 5.10.2.3; Intel: ok Huawei: Fine to use the term “one to one sidelink communication”. However, the UE does not explicitly indicate if the request is for one to one or for one to many communication. Therefore, propose to change to 2> if configured by upper layers to transmit non-relay related one to one sidelink communication: 3> if the UE did not transmit a SidelinkUEInformation message since entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformation message the UE connected to a PCell not broadcasting SystemInformationBlockType18; or 3> if the last transmission of the SidelinkUEInformation message did not include commTxResourceReq121; or if the information carried by the commTxResourceReq121 has changed since the last transmission of the SidelinkUEInformation message: 4> initiate transmission of the SidelinkUEInformation message to indicate the non-relay related sidelink communication transmission resources required by the UE in accordance with 5.10.2.3; 2> else: 3> if the last transmission of the SidelinkUEInformation message included commTxResourceReq121: 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require non-relay related sidelink communication transmission resources in accordance with 5.10.2.3; Qualcomm: Ok Samsung: Changed as suggested by Ericsson. Separate field is used for 121, so additional changes proposed by Huawei not included |  | Closed (ProSe CR) |
| E.103 | 5.10.2.2 | The term “in case of” is typically not used like this in method descriptions. 2> if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency or, in case of non-relay PS related transmission, on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResourcesPS included and not set to noTxOnCarrier: | 1 |  | Change the text to 2> if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency; or 2> if configured by upper layers to transmit non-relay PS related transmission, on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResourcesPS included and not set to noTxOnCarrier: Huawei: Propose to change to 2> if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency or, in case of non-relay PS related transmissionsidelink discovery announcements, on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResourcesPS included and not set to noTxOnCarrier: Intel: ok Qualcomm: Ok. Samsung: Changed as suggested by Huawei (although correct single bullet was kept for alignment with other cases) |  | Closed (ProSe CR) |
| E.104 | 5.10.2.2 | The term “public safety related” is not used. 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require public safety related sidelink discovery announcement resources in accordance with 5.10.2.3; | 1 |  | Change the text to 4> initiate transmission of the SidelinkUEInformation message to indicate it does no longer require PSpublic safety related sidelink discovery announcement resources in accordance with 5.10.2.3; Huawei: Agree. Intel: ok Qualcomm: No strong opinion either way. Samsung: Changed generally |  | Closed (ProSe CR) |
| E.278 | 5.10.2.2 | 2> if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements on the primary frequency or on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResources included and not set to noTxOnCarrier: | 1 |  | Probably good to specify: if included in SystemInformationBlockType19 of the PCell or of the cell in which the UE is camping. Huawei: Agree. Samsung: We generally talk about ‘camping on’ Intel: ok, however we think it’s for connected UE to send SidelinkUEInformation, so “or of the cell in which the UE is camping” seems not needed. Qualcomm: can be considered for text improvement. |  | Closed (ProSe CR) |
| L.007 | 5.10.2.2. | editorial | 1 |  | 2> if the UE is configured to report parameters related to sSidelink operations from system information of carriers other than the primary for reporting (i.e. RRCConnectionReconfiguration with discSL-SysInfoReportConfig) and the UE supports reporting of Ssystem Iinformation for inter-frequency/PLMN discovery: 3> if the UE acquired the relevant parameters from the Ssystem Iinformation of one or more cells on those carriers and T370 is running: 4> if the UE has configured lower layers to transmit the sidelink discovery announcements on those cells;, or 4> if the UE has configured lower layers to monitor sidelink discovery announcements on those cells;: 5> initiate transmission of the SidelinkUEInformation message to report the acquired Ssystem Iinformation parameters and stop T370;. Huawei: Agree. Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| E.279 | 5.10.2.2. | if configured by upper layers to transmit PS related sidelink discovery announcements on the primary frequency or, in case of non-relay PS related transmission, on one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19, with discTxResourcesPS included and not set to noTxOnCarrier: | 1 |  | Same comment as E.278 Coordinator: This probably means that E.279 is a similar type of issue as E.278 It is not a duplicate. Huawei: Agree. Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| H.056 | 5.10.2.3 | 4> for each frequency the UE is configured to transmit non-PS related sidelink discovery announcements on that concerns the primary frequency or that is included in discInterFreqList with discTxResources included and not set to noTxOnCarrier: 5> include discTxResourceReq and set it to indicate the number of discovery messages for sidelink discovery announcement(s) for which it requests E-UTRAN to assign dedicated resources as well as the concerned frequency, if different from the primary; The number of SL discovery resources requested is corresponding to the number of SL discovery messages the UE wants to transmit. In rel-12, the UE reports the number of SL discovery messages to the eNB, and then the eNB can configure SL discovery resources according to the number of discovery messages reported. However, in rel-13, RAN2 did not agree that for each frequency the UE indicates the number of discovery messages for transmission. Alternatively, the UE can report the total number of discovery messages waiting for transmission, and the eNB configures resources accordingly. | 2 |  | 4> include discTxResourceReq and set it to indicate the number of discovery messages for sidelink discovery announcement(s) for which it requests E-UTRAN to assign dedicated resources. 4> for each frequency the UE is configured to transmit non-PS related sidelink discovery announcements on that concerns the primary frequency or that is included in discInterFreqList with discTxResources included and not set to noTxOnCarrier: 5> include discTxResourceReq and set it to indicate the number of discovery messages for sidelink discovery announcement(s) for which it requests E-UTRAN to assign dedicated resources as well as the concerned frequency, if different from the primary; Ericsson: We have made the following agreement in RAN2#91bis: - A UE in RRC connected, interested in performing discovery transmissions, can be configured via dedicated signalling with a list resources for multiple carrier frequencies in which the UE can perform discovery transmissions. The maximum number of frequencies is same as rx In order for the eNB to make this configuration, it is beneficial if it knows the requested resources per carrier. Intel: We prefer the original text. Samsung: We consider this to be new proposal and prefer to stick to the current signalling approach (see also H.066) Qualcomm: No need to change. We agreed in RAN2 to report per frequency Samsung: No change (no support). If still concerns, a paper should be brought to RAN2#93 |  | Closed (ProSe CR) |
| E.115 | 5.10.2.3 | The following text is not understandable (“on that concerns”) 4> for each frequency the UE is configured to transmit non-PS related sidelink discovery announcements on that concerns the primary frequency or that is included in discInterFreqList with discTxResources included and not set to noTxOnCarrier: | 2 |  | Discuss what the correct text should be. Maybe remove “that”? Huawei: Understand the meaning. Maybe can add “either” before “that concerns” if required. Samsung: Text seems correct, but maybe nicer to move ‘on’ i.e. for each frequency on which the UE is configured to transmit.. Intel: no strong view Qualcomm: Agree that improvement is required. |  | Closed (ProSe CR) |
| C.026 | 5.10.2.3 | The UE could be configured by upper layer to monitor and transmit discovery announcement simultaneously. It should be allowed for UE to include discRxInterest and discTxResourceReq in one SidelinkUEInformation message. | 1 |  | 2> if SystemInformationBlockType19 is broadcast by the PCell: 3> if configured by upper layers to receive sidelink discovery announcements on a serving frequency or one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19: 4> include discRxInterest; 3> else if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements: 4> for each frequency the UE is configured to transmit non-PS related sidelink discovery announcements on that concerns the primary frequency or that is included in discInterFreqList with discTxResources included and not set to noTxOnCarrier: 5> include discTxResourceReq and set it to indicate the number of discovery messages for sidelink discovery announcement(s) for which it requests E-UTRAN to assign dedicated resources as well as the concerned frequency, if different from the primary; Huawei: Tend to agree.. Intel: seems more discussion is needed (similar issue as L.008?) Samsung: Corrected (seems clear mistake) |  | Closed (ProSe CR) |
| E.108 | 5.10.2.3 | The following text is somewhat long and overly complex. 1> if the UE initiates the procedure to indicate it is (no more) interested to receive sidelink communication or discovery or to request (configuration/ release) of sidelink communication or discovery transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure): | 1 |  | Change the text to 1> if the UE initiates the procedure to release or request resources related to sidelink communication or related to sidelink discovery indicate it is (no more) interested to receive sidelink communication or discovery or to request (configuration/ release) of sidelink communication or discovery transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure): This text introduces the notion of releasing sidelink resources. For extra clarity, 5.10.2.2 could be rewritten with this in mind, but it is strictly not required. Huawei: Can consider to remove “(i.e. UE includes all concerned information, irrespective of what triggered the procedure)”, or add to note for information. Intel: we prefer the original text. In our understanding, for rx case, it’s not directly related with release or request of resources. Samsung: We think part in brackets is relevant to keep i.e. that if UE indicates resource related information, it does not signal the delta but the full Samsung; No change for now (note that Rx indication would not be covered by suggested text) |  | Closed (ProSe CR) |
| E.109 | 5.10.2.3 | The following text does not use correct terminology. 3> if configured by upper layers to transmit sidelink communication: | 1 |  | Change text to 3> if configured by upper layers to transmit one to many sidelink communication: Huawei: Agree. Intel: not sure if we agreed the upper layer will inform 1-to-M communication in separate. |  | Closed (ProSe CR) |
| E.111 | 5.10.2.3 | The following text does not use correct terminology. 3> if configured by upper layers to transmit non-relay related sidelink one to one communication: | 1 |  | Change text to 3> if configured by upper layers to transmit non-relay related one to one sidelink communication: Huawei: Agree. Intel: ok |  | Closed (ProSe CR) |
| E.114 | 5.10.2.3 | The following text is incorrect. The UE can be configured to receive discovery and to transmit, hence the use of “else” is incorrect. 3> if configured by upper layers to receive sidelink discovery announcements on a serving frequency or one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19: 4> include discRxInterest; 3> else if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements: 4> for each frequency [...] | 1 |  | Change the text to (i.e. remove “else”) 3> if configured by upper layers to receive sidelink discovery announcements on a serving frequency or one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19: 4> include discRxInterest; 3> else if the UE is configured by upper layers to transmit non-PS related sidelink discovery announcements: 4> for each frequency [...] Huawei: Agree. Intel: see the comment in C.026 (also similar to L.008?) |  | Closed (ProSe CR) |
| E.116 | 5.10.2.3 | The term “in case of” is typically not used in method desciptions like this. 3> if the frequency the UE is configured to transmit PS related sidelink discovery announcements on either concerns the primary frequency or, in case of non-relay PS related transmission, if the frequency is included in discInterFreqList with discTxResourcesPS included and not set to noTxOnCarrier: | 1 |  | Change the text to 3> if the frequency the UE is configured to transmit PS related sidelink discovery announcements on either concerns the primary frequency or, if the frequency the UE is configured to transmit non-relay PS related sidelink discovery annoucements is included in discInterFreqList with discTxResourcesPS included and not set to noTxOnCarrier: Huawei: Fine with either way. Intel: ok Qualcomm: Ok Samsung: Some changes done, but kept ‘in case of’ (as used several times) |  | Closed (ProSe CR) |
| E.117 | 5.10.2.3 | The if/else clause on level 1 is incorrect. 1> if the UE initiates the procedure to indicate it is (no more) interested to receive sidelink communication or discovery or to request (configuration/ release) of sidelink communication or discovery transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure): [...] 1> else (i.e. UE request transmission and/ or reception gaps): [...] 1> else if the UE initiates the procedure to report the system information parameters related to Sidelink operations of carriers other than the primary | 1 |  | Change to 1> if the UE initiates the procedure to indicate it is (no more) interested to receive sidelink communication or discovery or to request (configuration/ release) of sidelink communication or discovery transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure): [...] 1> else (i.e. UE request transmission and/ or reception gaps) if the UE initiates the procedure to request sidelink discovery gaps: [...] 1> else if the UE initiates the procedure to report the system information parameters related to Sidelink operations of carriers other than the primary Huawei: resource request/release and gap request can be included in one request message. Therefore, propose to remove the “else”, as follows 1> if the UE initiates the procedure to indicate it is (no more) interested to receive sidelink communication or discovery or to request (configuration/ release) of sidelink communication or discovery transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure): [...] 1> else (i.e. UE request transmission and/ or reception gaps):if the UE initiates the procedure to request sidelink discovery gaps: [...] 1> if the UE initiates the procedure to report the system information parameters related to Sidelink operations of carriers other than the primary Samsung: Agree (related to L.008) Intel: ok: Samsung: Changed as suggested by Ericsson (support of simultanous triggers requires further discussion, see L.008) |  | Closed (ProSe CR) |
| E.118 | 5.10.2.3 | The correct terminology “sidelink discovery gaps” is not used. 2> if the UE is configured with gapRequestsAllowedDedicated set to true; or 2> if the UE is not configured with gapRequestsAllowedDedicated and gapRequestsAllowedCommon is included in SystemInformationBlockType19: 3> if the UE requires gaps to monitor the sidelink discovery announcements the UE is configured to monitor by upper layers: 4> include discRxGapRequest and set it to indicate, for each frequency that either concerns the primary or is included in discInterFreqList on which the UE is configured to monitor sidelink discovery announcements and for which it requires gaps to do so, the gap pattern(s) as well as the concerned frequency, if different from the primary; 3> if the UE requires gaps to transmit the sidelink discovery announcements the UE is configured to transmit by upper layers: 4> include discTxGapRequest and set it to indicate, for each frequency that either concerns the primary or is included in discInterFreqList on which the UE is configured to transmit sidelink discovery announcements and for which it requires gaps to do so, the gap pattern(s) as well as the concerned frequency, if different from the primary; | 1 |  | Change text to 2> if the UE is configured with gapRequestsAllowedDedicated set to true; or 2> if the UE is not configured with gapRequestsAllowedDedicated and gapRequestsAllowedCommon is included in SystemInformationBlockType19: 3> if the UE requires sidelink discovery gaps to monitor the sidelink discovery announcements the UE is configured to monitor by upper layers: 4> include discRxGapRequest and set it to indicate, for each frequency that either concerns the primary frequency or is included in discInterFreqList on which the UE is configured to monitor sidelink discovery announcements and for which it requires sidelink discovery gaps to do so, the gap pattern(s) as well as the concerned frequency, if different from the primary; 3> if the UE requires sidelink discovery gaps to transmit the sidelink discovery announcements the UE is configured to transmit by upper layers: 4> include discTxGapRequest and set it to indicate, for each frequency that either concerns the primary or is included in discInterFreqList on which the UE is configured to transmit sidelink discovery announcements and for which it requires sidelink discovery gaps to do so, the gap pattern(s) as well as the concerned frequency, if different from the primary; Huawei: Regarding the terminology, can discuss if need to further use “sidelink discovery RX gap” and “sidelink discovery TX gap” here. Intel: ok |  | Closed (ProSe CR) |
| E.156 | 5.10.2.3 | Wrong terminology used. A UE capable of sidelink remote operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink discovery transmission if the following conditions are met: | 1 |  | Change text to A UE capable of sidelink remote UE operation shall inform upper layers that it is configured with radio resources that can be used for PS relay related sidelink discovery transmission if the following conditions are met: Huawei: agree. Intel: ok |  | Closed (ProSe CR) |
| E.157 | 5.10.2.3 | If statement is incorrect, should be else if, also a “while” is used. 2> if the AS-layer radio conditions specified in this section were previously met; and if discConfigRelay is included in SystemInformationBlockType19 while the RSRP measurement of the PCell is below (stay); | 1 |  | Change text to 2> else if the AS-layer radio conditions specified in this section were previously met; and if discConfigRelay is included in SystemInformationBlockType19 whileand the RSRP measurement of the PCell is below discThreshHiRemoteUE (stay); Huawei: Propose to change to 2> if discThreshHiRemoteUE is not included in SystemInformationBlockType19; or 2> if the AS-layer radio conditions specified in this section were previously not met; and if discConfigRelay is included in SystemInformationBlockType19 while the RSRP measurement of the PCell is below discThreshHiRemoteUE by discHystMaxRemoteUE (entry); or 2> if the AS-layer radio conditions specified in this section were previously met; and if discConfigRelay is included in SystemInformationBlockType19 while the RSRP measurement of the PCell is below discThreshHiRemoteUE (stay); Intel: ok Samsung: Seems to concern 5.10.11.2/ 3 |  | Closed (ProSe CR) |
| E.119 | 5.10.4 | Descriptions on the variants of sidelink communication are missing. | 2 |  | Add the following text to the start of the section. Sidelink communication consists of one to many and one to one sidelink communication. One to one sidelink communication consists of relay related and non-relay related one to one sidelink communication. In relay related one to one sidelink communication the communicating parties consist of one sidelink relay UE and one sidelink remote UE. This text can also be incorporated in section 3.1 Defintions. Huawei: Need to take into account that relay UE can transmit one to many sidelinnk communication. The description may not be needed in Stage-3, but needed in Stage-2. Qualcomm: No need |  | Closed (ProSe CR) |
| E.123 | 5.10.4 | This text is hard to understand. - Actions and conditions are in a mixed order. - Circular reference where lower layers are configured based on whether lower layers were previously configured somehow. 1> if the transmission concerns relay communication; and the UE is capable of sidelink relay or remote operation: 2> if the UE is in RRC_IDLE; and if the UE is selecting a relay/ has a selected relay: configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section, only if the following condition is met: 3> if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19; 2> configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section, only if the following condition is met: 3> if the UE configured lower layers with a pool of resources included in SystemInformationBlockType18 (i.e. commTxPoolNormalCommon or commTxPoolExceptional); and commTxAllowRelayCommon is included in SystemInformationBlockType18; or 3> if the UE configured lower layers with resources provided by dedicated signalling (i.e. commTxPoolResources set to scheduled or ue-Selected); and the UE is configured with commTxAllowRelayDedicated while its value is set to true | 2 |  | Maybe the text can be reshuffled so that conditions and actions are not mixed, i.e., first all the conditions, and then the action “configure lower layers...” Huawei: Agree with the proposal. Samsung: Was mainly done to avoid duplication. Note this relates to LG.009. May actually be solved/ superseded by the paper we plan to brin covering terminology and inter-layer modelling Intel: ok (the existing text seems hard to understand), and it would be good to see the full newly updated text first. Ericsson: Related to L.009. Qualcomm: Not sure about proposal. Text looks ok. Samsung: No change for now. If still concerns, detailed proposal may be brought to RAN2#93 |  | Closed (ProSe CR) |
| E.283 | 5.10.4 | 1> if the transmission concerns relay communication; and the UE is capable of sidelink relay or remote operation: 2> if the UE is in RRC_IDLE; and if the UE is selecting a relay/ has a selected relay: configure lower layers to transmit the sidelink control information and the corresponding data using the resources, as specified previously in this section, only if the following condition is met: | 2 |  | As mentioned above, should the UE send Sidelink control info (thereby entering connected mode) before having selected a relay?...strange. Huawei: Maybe “using the resources, as specified previously in this section,” already handled the concern. But fine to further polish the text. Intel: it would be good to see the full newly updated text first. Qualcomm: Not sure about proposal. Samsung: Removed ‘is selecting’ which is assumed to apply for discovery transmission only |  | Closed (ProSe CR) |
| E.122 | 5.10.4 | The correct terminology is not used. 1> if the transmission concerns relay communication; and the UE is capable of sidelink relay or remote operation: | 1 |  | Change the text to 1> if the transmission concerns is relay related one to one sidelink communication; and the UE is capable of sidelink relay or remote operation: Huawei: prefer original text. Intel: ok Samsung: No change (as there are concerns, and many cases) |  | Closed (ProSe CR) |
| C.027 | 5.10.5 | The UE behaviors under not configured with rx gap and configured with rx gap seem overlapped. Our suggestion is to add the constraint of not affecting normal operation for the UE not configured with rx gaps and delete the else part of the configured with rx gaps For the UE configured with rx gaps, the UE behavior is specified in another bullet. | 3 |  | A UE capable of non-PS related sidelink discovery that is configured by upper layers to monitor sidelink discovery announcements shall: 1> for each frequency the UE is configured to monitor non-PS related sidelink discovery announcements on, prioritising the frequencies included in discInterFreqList, if included in SystemInformationBlockType19: 2> if the PCell/ the cell the UE is camping on indicates the pool of resources to monitor sidelink discovery announcements on by discRxResources within discInterFreqList in SystemInformationBlockType19: 3> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxResources in SystemInformationBlockType19 without affecting normal operation i.e. receive during idle periods or by using a spare receiver, 2> else if the cell used for sidelink discovery monitoring broadcasts SystemInformationBlockType19: 3> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPool in SystemInformationBlockType19 without affecting normal operation i.e. receive during idle periods or by using a spare receiver; NOTE 1: The requirement not to affect normal UE operation also applies for the acquisition of sidelink discovery related system and synchronisation information from inter-frequency cells. NOTE 2: The UE is not required to monitor all pools simultaneously. NOTE 3: It is up to UE implementation to decide whether a cell is sufficiently good to be used to monitor sidelink discovery announcements. NOTE 4: If discRxPool includes one or more entries including rxParameters, the UE may only monitor such entries if the associated SLSSIDs are detected. When monitoring such pool(s) the UE applies the timing of the corresponding SLSS. 2> if the UE is configured with discRxGapConfig and requires gaps to monitor sidelink discovery announcements on the concerned frequency; 3> configure lower layers to monitor the concerned frequency using the gaps indicated by discRxGapConfig, 2> else: 3> configure lower layers to monitor the concerned frequency without affecting normal operation i.e. receive during idle periods or by using a spare receiver; Ericsson: We disagree with CATT and think the existing solution is fully functional. ZTE: it it not necessary to add the constraints in each possible monitoring scenarios. The original text is clear enough to only differentiate the behaviorsfor gap configured and non configured case in general. Intel: ok Huawei: agree to improve the structure. But the proposed change have not resolved the concerns well. Propose to improve the structure together with other issues raised above in this part. Qualcomm: No need to change. Samsung: No change for now. If there are still concerns, paper may be brought to RAN2#93 |  | Closed (ProSe CR) |
| C.028 | 5.10.5 | 1) Similar issue and suggestion here as C.026. 2)The condition highlighted in yellow is not related to discovery reception, which should be removed. | 3 |  | A UE capable of PS related sidelink discovery that is configured by upper layers to monitor PS related sidelink discovery announcements shall: 1> if out of coverage on the frequency, as defined in TS 36.304 [4, 11.4]: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources that were preconfigured (i.e. indicated by discRxPoolList within preconfigDisc in SL-Preconfiguration defined in 9.3); 1> else if the PCell/ the cell the UE is camping on indicates a pool of resources to monitor sidelink discovery announcements on by discRxPoolPS within discInterFreqList in SystemInformationBlockType19: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPoolPS in SystemInformationBlockType19 without affecting normal operation i.e. receive during idle periods or by using a spare receiver ; 1> else if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; and if the cell used for sidelink discovery monitoring broadcasts SystemInformationBlockType19: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPoolPS in SystemInformationBlockType19 without affecting normal operation i.e. receive during idle periods or by using a spare receiver; 1> if the UE is configured with discRxGapConfig and requires gaps to monitor sidelink discovery announcements on the concerned frequency; 2> configure lower layers to monitor the concerned frequency using the gaps indicated by discRxGapConfig, 1> else: 2> configure lower layers to monitor the concerned frequency without affecting normal operation i.e. receive during idle periods or by using a spare receiver; Huawei: Agree to improve the structure. Intel: ok Ericsson: Disagree. ZTE: it it not necessary to add the constraints in each possible monitoring scenarios. The original text is clear enough. Qualcomm: Similar to C.027. Need to check Issue 2. Samsung: Same as C.027 |  | Closed (ProSe CR) |
| L.011 | 5.10.5 | Inter-frequency relay discovery operation is not allowed. | 2 |  | A UE capable of PS related sidelink discovery that is configured by upper layers to monitor PS related sidelink discovery announcements shall: 1> if out of coverage on the frequency, as defined in TS 36.304 [4, 11.4]: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources that were preconfigured (i.e. indicated by discRxPoolList within preconfigDisc in SL-Preconfiguration defined in 9.3); 1> else if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; and if the PCell/ the cell the UE is camping on indicates a pool of resources to monitor sidelink discovery announcements on by discRxResourcePSdiscRxPoolPS within discInterFreqList in SystemInformationBlockType19: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxResourcePSdiscRxPoolPS in SystemInformationBlockType19 ; 1> else if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; and if the cell used for sidelink discovery monitoring broadcasts SystemInformationBlockType19: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPoolPS in SystemInformationBlockType19; Ericsson: We are not sure LG is correct and wonder which agreement states that inter-frequency relay discovery is not allowed. We disagree for now. Furthermore, we think the proposed solution is flawed by removing the “else”. ZTE: The modification still has problems. If the UE does not configured by upper layer to transmit discovery announcement and only want to monitor relay PS discovery announcement, no corresponding description to specify it. Intel: we assume that it is not allowed. Huawei: Fine with the motivation of changes. But not sure the change is correct. This part is discussing monitoring. Why” if configured by upper layers to transmit non-relay PS related sidelink discovery announcements”. Noticed that there is another “transmits” in the original text as highlighted. This part may need to be further updated. Samsung: Agree. Qualcomm: Agree that inter-frequency/PLMN relay operation is not supported in Rel-13. However not sure if formulation proposed by LG is correct. Samsung: Some further discussion seems required |  | Closed (ProSe CR) |
| Z.026 | 5.10.5 | A UE capable of non-PS related sidelink discovery can only monitor non-PS related sidelink discovery announcements, however the statement bellow is not so clear. A UE capable of non-PS related sidelink discovery that is configured by upper layers to monitor sidelink discovery announcements shall: | 1 |  | Add the text in hightlight: A UE capable of non-PS related sidelink discovery that is configured by upper layers to monitor non-PS related sidelink discovery announcements shall: Huawei: Seems correct, but not sure if upper layers indicate the monitoring is for PS or for non-PS. Intel: ok |  | Closed (ProSe CR) |
| E.124 | 5.10.5 | The following text can be made more precise. 3> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxResources in SystemInformationBlockType19, | 1 |  | Change the text to 3> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxResources by discRxResources within discInterFreqList in SystemInformationBlockType19, Huawei: Fine if this is aligned with the legacy manner. Intel: ok |  | Closed (ProSe CR) |
| E.125 | 5.10.5 | The terminology is not correct. 2> if the UE is configured with discRxGapConfig and requires gaps to monitor sidelink discovery announcements on the concerned frequency; 3> configure lower layers to monitor the concerned frequency using the gaps indicated by discRxGapConfig, 2> else: 3> configure lower layers to monitor the concerned frequency without affecting normal operation i.e. receive during idle periods or by using a spare receiver; | 1 |  | Change the text to 2> if the UE is configured with discRxGapConfig and requires sidelink discovery gaps to monitor sidelink discovery announcements on the concerned frequency; 3> configure lower layers to monitor the concerned frequency using the sidelink discovery gaps indicated by discRxGapConfig, 2> else: 3> configure lower layers to monitor the concerned frequency without affecting normal operation i.e. receive during idle periods or by using a spare receiver; Huawei: Fine to be more precise. Can discuss if there is a need to use “sidelink discovery TX gap” or “sidelink discovery RX gap”. Intel: ok |  | Closed (ProSe CR) |
| E.126 | 5.10.5 | The four notes are now nested in a method description giving the impression they only apply for a certain part of the method description. This is incorrect as the four notes apply to all the text. | 1 |  | Move the four notes to the end of the section. Huawei: Fine. Intel: ok |  | Closed (ProSe CR) |
| E.127 | 5.10.5 | The terminology is not correct. 1> if the UE is configured with discRxGapConfig and requires gaps to monitor sidelink discovery announcements on the concerned frequency; 2> configure lower layers to monitor the concerned frequency using the gaps indicated by discRxGapConfig, | 1 |  | Change the text to 1> if the UE is configured with discRxGapConfig and requires sidelink discovery gaps to monitor sidelink discovery announcements on the concerned frequency; 2> configure lower layers to monitor the concerned frequency using the sidelink discovery gaps indicated by discRxGapConfig, Huawei: Fine to be more precise. Can discuss if there is a need to use “sidelink discovery TX gap” or “sidelink discovery RX gap”. Intel: ok |  | Closed (ProSe CR) |
| N.029 | 5.10.5 Sidelink discovery monitoring | Usage of slash”/” is not clear : 1> else if the PCell/ the cell | 2 |  | Should it be interpreted as and or or? Avoid use of “/” Ericsson: We agree. This is used on multiple places in the spec. Intel: we assume “or”. Huawei: agree. “or” can be used here. Nokia Networks: Should use “or” instead of “/”. Qualcomm: Clear to us, but no strong opinion. Samsung: Changed several cases (although seems not ambiguous) |  | Closed (ProSe CR) |
| H.057 | 5.10.6 | 1> else if discTxPoolCommon is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: 2> select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; In our understanding, the condition should be corresponding to the case that the UE reads SIB19 autonomously from a carrier configured in discInterFreqList-v13x0 in SIB19 of the serving cell, and the discTxResourcesPS-r13 corresponding to this carrier is acquireSI-FromCarrier-r13. Need to clarify this case clearly. RAN2 did not agree that the UE can read SIB19 autonomously from another carrier which is not included in SIB19, in case SIB19 is present. Further, need another condition for the case that SIB19 is absent. | 2 |  | 1> else if the frequency used to transmit sidelink discovery on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, while discTxResources in the corresponding entry of discInterFreqList is set to acquireSI-FromCarrier, and discTxPoolCommon is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: or 1> else if SystemInformationBlockType19 of the serving cell/ PCell is absent, and discTxPoolCommon is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met, 2> select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; Samsung: We are not sure a change is really needed. In our understanding RAN2 agreed that the UE is always allowed to receive transmit on a carrier regardless of the serving cell/ PCell, as long at it does not affect ongoing communication (but only bother the eNB e.g. by requesting resources/ gaps when the concerned frequency is listed on the serving cell / PCell) Intel: Slightly prefer the original text since the proposal seems somewhat overspecified. Also we’re wondering if the proposed condition is the only case when the UE performs autonomous tx/rx. Ericsson: We are not sure a clarification is necessary, it is clear that discTxPoolCommon is read from some other cell. Furthermore, the proposed construction “or else if” should be avoided. We think “or if” is sufficient.ZTE: The modification exclude the uncoordinated scenario for announcement, In uncoordinated scenario, the serving eNB is unable to provide the frequency in discInterFreqList , Therefore , it is suggest to keep original version. Qualcomm: We agreed in RAN2 that for commercial and PS if SIB 19 is not present then UE can read SIB 19 in other frequency and perform discovery without affecting Uu. We are not sure what is motivation of Huawei proposal to create artificial restriction of not allowing UE to perform discovery on other frequency when SIB 19 is present but doesnot include the frequency. Samsung: No change for now. If there are still concerns, a paper should be brought to RAN2#93 |  | Closed (ProSe CR) |
| H.059 | 5.10.6 | 1> else if discTxPoolPS-Common is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements 3> select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; In our understanding, the condition should be corresponding to the case that the UE reads SIB19 autonomously from a carrier included in discInterFreqList-v13x0 in SIB19 of the serving cell, and the discTxResourcesPS-r13 corresponding to this carrier is acquireSI-FromCarrier-r13. Need to clarify this case clearly. RAN2 did not agree that the UE can read SIB19 autonomously from another carrier which is not included in SIB19of PCell, in case SIB19 of Pcell is present. Further, need another condition for the case that SIB19 of Pcell is absent. | 2 |  | 1> else if the frequency used to transmit sidelink discovery on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, while discTxResourcesPS in the corresponding entry of discInterFreqList is set to acquireSI-FromCarrier, and discTxPoolPS-Common is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: 1> else if SystemInformationBlockType19 of the serving cell/ PCell is absent, and discTxPoolCommonPS is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met, 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements 3> select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; Samsung: Same response as for H.057 Intel: we slightly prefer the original text since the proposal seems somewhat overspecified. Also we’re wondering if the proposed condition is the only case when the UE performs autonomous tx/rx. Qualcomm: Same as H.057 Samsung: See H.057 |  | Closed (ProSe CR) |
| Z.028 | 5.10.6 | Since non-PS related and PS related sidelink discovery are described indepently, it is better to restrict the capability of the UE in each case. A UE capable of sidelink discovery that is configured by upper layers to transmit non-PS related sidelink discovery announcements shall, for each frequency the UE is configured to transmit such announcements on: …… A UE capable of PS related sidelink discovery that is configured by upper layers to transmit PS related sidelink discovery announcements shall: …… | 1 |  | Add the text: A UE capable of non-PS related sidelink discovery that is configured by upper layers to transmit non-PS related sidelink discovery announcements shall, for each frequency the UE is configured to transmit such announcements on: Huawei: Does it mean there are two different capabilities for sidelink discovery? Can further discuss in next meeting. Intel: ok |  | Closed (ProSe CR) |
| Z.029 | 5.10.6 | 1. It is better to change “sidelink discovery” to “sidelink discovery announcements”. 2. “sidelink” is missing. | 1 |  | Add the text: 1> if the frequency used to transmit sidelink discovery announcements concerns the serving frequency (RRC_IDLE)/ primary frequency (RRC_CONNECTED): 1> else if, for the frequency used to transmit sidelink discovery announcements on, the UE is configured with dedicated resources (i.e. with discTxResources in discTxInterFreqInfo within sl-DiscConfig); and the conditions for sidelink operation as defined in 5.10.1a are met: 1> else if the frequency used to transmit sidelink discovery announcements on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, while discTxResources in the corresponding entry of discInterFreqList is set to discTxPoolCommon (i.e. serving cell/ PCell broadcasts pool of resources) and the conditions for sidelink operation as defined in 5.10.1a are met; or 1> else if discTxPoolCommon is included in SystemInformationBlockType19 acquired from cell selected on the sidelink discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: 1> if out of coverage on the frequency used to transmit PS related sidelink discovery announcements: 1> else if the frequency used to transmit sidelink discovery announcements concerns the serving frequency (RRC_IDLE)/ primary frequency (RRC_CONNECTED); and if the UE’s serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) is suitable as defined in TS 36.304 [4]: 1> else if, for the frequency used to transmit sidelink discovery announcements on, the UE is configured with dedicated resources (i.e. with discTxResourcesPS in discTxInterFreqInfo within sl-DiscConfig); and the conditions for sidelink operation as defined in 5.10.1a are met: 1> else if the frequency used to transmit sidelink discovery announcements on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, while discTxResourcesPS in the corresponding entry of discInterFreqList is set to discTxPoolCommon (i.e. serving cell/ PCell broadcasts pool of resources) and the conditions for sidelink operation as defined in 5.10.1a are met: 1> else if discTxPoolPS-Common is included in SystemInformationBlockType19 acquired from cell selected on the sidelink discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: Huawei: “transmit sidelink discovery announcements” may be a little bit strange. Propose to change to “announce sidelink discovery” or keep as it is. Intel: ok |  | Closed (ProSe CR) |
| L.013 | 5.10.6 | Wrong field name | 1 |  | 3> else if the UE is configured with discTxResourcesPSRelay set to scheduled: 4> configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResourcesPSRelay; Huawei: Agree. Intel: ok |  | Closed (ProSe CR) |
| H.058 | 5.10.6 | 1> if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig, 1> else: 2> configure lower layers to transmit on the concerned frequency without affecting normal operation i.e. receive during idle periods or by using a spare receiver; | 1 |  | 1> if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig, 1> else: 2> configure lower layers to transmit on the concerned frequency without affecting normal operation i.e. receive transmit during idle periods or by using a spare receivertransmitter; Intel: ok, or the other option would be to remove the part “i.e…xxx”. |  | Closed (ProSe CR) |
| E.132 | 5.10.6 | The following text uses “while” in a way which is typically not used. 1> else if the frequency used to transmit sidelink discovery on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, while discTxResources in the corresponding entry of discInterFreqList is set to discTxPoolCommon (i.e. serving cell/ PCell broadcasts pool of resources) and the conditions for sidelink operation as defined in 5.10.1a are met; or | 1 |  | Change to the following text. 1> else if the frequency used to transmit sidelink discovery on is included in discInterFreqList within SystemInformationBlockType19 of the serving cell/ PCell, whileand discTxResources in the corresponding entry of discInterFreqList is set to discTxPoolCommon (i.e. serving cell/ PCell broadcasts pool of resources) and the conditions for sidelink operation as defined in 5.10.1a are met; or Huawei: The comments are duplicated??? Intel: ok |  | Closed (ProSe CR) |
| E.133 | 5.10.6 | The following text does not use the correct terminology. 1> if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig, | 1 |  | Change to the following text. 1> if the UE is configured with discTxGapConfig and requires sidelink discovery gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the sidelink discovery gaps indicated by discTxGapConfig, Huawei: The comments are duplicated??? Intel: ok |  | Closed (ProSe CR) |
| E.134 | 5.10.6 | The out of coverage requirement should point to relevant section in 36.304. 1> if out of coverage on the frequency used to transmit PS related sidelink discovery announcements: | 1 |  | Change the text to 1> if out of coverage on the frequency used to transmit PS related sidelink discovery announcements as defined in TS 36.304 [4, 11.4]: Huawei: fine. Intel: ok |  | Closed (ProSe CR) |
| N.030 | 5.10.6 Sidelink discovery announcement | Usage of slash”/” is not clear : 1> if the frequency used to transmit sidelink discovery concerns the serving frequency (RRC_IDLE)/ primary frequency (RRC_CONNECTED): Also a number of other procedures in this section is defined this way. | 2 |  | Should it be interpreted as and or or? Avoid use of “/” Huawei: agree. Intel: we assume “or” Ericsson: Agree. Nokia Networks: Should use “or” instead of “/”. Qualcomm: No strong opinion either way. |  | Closed (ProSe CR) |
| C.029 | 5.10.6 Sidelink discovery announcement | When describing the non-PS related discovery transmission, transmit and transmitter should be used, not the reciever. And when describing the PS related discovery transmission, transmitter should be used, not the reciever. | 1 |  | 1> if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig, 1> else: 2> configure lower layers to transmit on the concerned frequency without affecting normal operation i.e. receive transmit during idle periods or by using a spare receivertransmitter; 1> if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig, 1> else: 2> configure lower layers to transmit on the concerned frequency without affecting normal operation i.e. transmit during idle periods or by using a spare receivertransmitter; Huawei: agree. Intel: ok, or the other option would be to remove the part “i.e…xxx”. |  | Closed (ProSe CR) |
| C.030 | 5.10.6 Sidelink discovery announcement | Since 5.10.6a is added and the RSRP based pool selection is described there, it is more appropriate to move the NOTE2 in section 5.10.6 to section 5.10.6a | 1 |  | The following Note should be moved to section 5.10.6a NOTE 2: When performing resource pool selection based on RSRP, the UE uses the latest results of the available measurements used for cell reselection evaluation in RRC_IDLE/ for measurement report triggering evaluation in RRC_CONNECTED, which are performed in accordance with the performance requirements specified in TS 36.133 [16]. Huawei: agree. Intel: ok with either way |  | Closed (ProSe CR) |
| L.012 | 5.10.6. | PCell for RRC connected UE is missing. | 2 |  | 2> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the PCell/the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19: 3> if the UE is configured with discTxPoolPS-Dedicated; or Huawei: agree. Intel: ok Ericsson: Agree. Qualcomm: Ok |  | Closed (ProSe CR) |
| Z.034 | 5.10.6a | 1> if poolSelection within poolToAddModList is set to rsrpBased: 2> select an entry of poolToAddModList for which the RSRP measurement of the reference cell as indicated by discTxRefCarrier, after applying the layer 3 filter defined by quantityConfig as specified in 5.5.3.2 , is in-between threshLow and threshHigh; The condition “if poolSelection within poolToAddModList is set to rsrpBased:”is not accurate, since it excludes the situation that the “poolSelection” is configured via SIB19. In addition, there is no such IE named as “discTxRefCarrier”. The detailed method of reference cell selection is specified in 5.10.6b. | 2 |  | 1, the sentence “if poolSelection within poolToAddModList is set to rsrpBased:” needs to be replaced by “if poolSelection within SL-DiscTxPoolList is set to rsrpBased:” 2. the sentence “select an entry of poolToAddModList for which the RSRP measurement of the reference cell as indicated by discTxRefCarrier,”needs to be replaced by “select an entry of SL-DiscTxPoolList for which the RSRP measurement of the reference cell as specified in 5.10.6b,” Huawei: Agree with the observation, but need to consider both the broadcast pools and the dedicated pools. Agree to update the IE for reference carrier. Intel: not clear on the need of the change. Isn’t SL-DiscTxPollList included into poolToAddModList? Ericsson: Agree. Same issue as L.014? Qualcomm: Agree Samsung: Removed ‘within poolToAddModList’/ replaced by a pool from the list of pools the UE is configured with |  | Closed (ProSe CR) |
| L.014 | 5.10.6a | Different fields needs to be used for RRC connected mode UE and RRC idle mode UE. | 2 |  | A UE that is is configured with a list of resource pool entries for sidelink discovery announcement transmission shall: 1> if poolSelection within poolToAddModList (RRC_CONNECTED) or is set to rsrpBased: 2> select an entry of poolToAddModList for which the RSRP measurement of the reference cell as indicated by discTxRefCarrier, after applying the layer 3 filter defined by quantityConfig as specified in 5.5.3.2 , is in-between threshLow and threshHigh; 1> else: 2> randomly select, using a uniform distribution, an entry of poolToAddModList; 1> configure lower layers to transmit the sidelink discovery announcement using the selected pool of resources; Huawei: Need to further consider broadcast pools. Intel: not clear which IE to be changed. Ericsson: We think this issue is the same as Z.034 and we prefer the solution in Z.034. ZTE: don't agree, the RSRP-based method is excluded for IDLE UE if modified. refer to our comment Z.034, Qualcomm: Similar to Z.034 Samsung: See Z.034 |  | Closed (ProSe CR) |
| Z.033 | 5.10.6a | There is an editory error in the sentence with the word “is” repeated twice: “A UE that is is configured with a list of resource pool entries for sidelink discovery announcement transmission shall:”. In addition, it’s better to clarify which IE does “a list of resource pool entries” referes to exactly. | 1 |  | The sentence could be replaced by “A UE that is configured with SL-DiscTxPoolList for sidelink discovery announcement transmission shall:” Huawei: agree. Intel: ok Samsung: Reference to IE added (as common and dedicated signalling use different fields) |  | Closed (ProSe CR) |
| E.135 | 5.10.6a | Incorrect language. A UE that is is configured with a list of resource pool entries for sidelink discovery announcement transmission shall: | 1 |  | Change text to (i.e. remove one “is”): A UE that is is configured with a list of resource pool entries for sidelink discovery announcement transmission shall: Huawei: fine. Intel: ok |  | Closed (ProSe CR) |
| E.286 | 5.10.6b | 2> else if the frequency concerns a secondary carrier: use the concerned SCell as reference; 2> else if the UE is configured with discTxRefCarrierDedicated for the frequency: use the cell indicated by this field as reference; 2> else if the UE is configured with discTxRefCarrierCommon for the frequency: use the serving cell (RRC_IDLE)/ PCell (RRC_CONNECTED) as reference; 2> else: use the DL frequency paired with the one used to transmit sidelink discovery announcements on as reference; | 2 |  | If there are more than one SCells in that carrier? The agreement says: If the UE has a configured serving cell in secondary carrier, then one of the configured serving cell is always used for DL measurements and synchronization Why in discTxRefCarrierCommon carrier PCell/SCell? Agreement says: The eNB may configure the UE with a carrier to use for DL measurements/synchronization using SIB19 for each SL carrier frequency. The reference carrier is signalled in SIB19 and via dedicated signalling. It would be beneficial to signal a reference carrier that is a serving carrier Therefore I would say that there should not be any difference between dedicated and common. ZTE: don't agree. As for Q1, generally there is only one SCell on one carrier used by the UE. As for Q2, discTxRefCarrierCommon only indicates Pcell for CONNECTED UE or serving cell for IDLE UE (note: not Scell) is used for reference. The eNB couldn't configure scell for reference for UE via discTxRefCarrierCommon in SIB19 since Scell is configured per UE not per cell Intel: to our understanding, the reference cell can be different when discTxRefCarrierDedicated is configured and discTxRefCarrierCommon is configured. The first one is the cell used to transmit discovery and the second one is PCell/serving cell. May be more discussion is needed if we don’t clearly understand the issue. Huawei: The issue is not clear to us. Needs further discussion. Samsung: We think there is no need for changes Qualcomm: Agree that instead of signaling Pcell/Scell better to signal EARFCN. Samsung: No change. If there still are concerns, a paper may be brought to RAN2#93 |  | Closed (ProSe CR) |
| E.136 | 5.10.6b | Wrong terminology 2> if the frequency concerns the primary carrier: use the PCell as reference; 2> else if the frequency concerns a secondary carrier: use the concerned SCell as reference; | 1 |  | It seems primary/secondary frequency is used. Hence change the text to 2> if the frequency concerns the primary carrier frequency: use the PCell as reference; 2> else if the frequency concerns a secondary carrier frequency: use the concerned SCell as reference; Intel: ok |  | Closed (ProSe CR) |
| E.137 | 5.10.6b | The text is somewhat compact. 2> if the frequency concerns the primary carrier: use the PCell as reference; 2> else if the frequency concerns a secondary carrier: use the concerned SCell as reference; 2> else if the UE is configured with discTxRefCarrierDedicated for the frequency: use the cell indicated by this field as reference; 2> else if the UE is configured with discTxRefCarrierCommon for the frequency: use the serving cell (RRC_IDLE)/ PCell (RRC_CONNECTED) as reference; 2> else: use the DL frequency paired with the one used to transmit sidelink discovery announcements on as reference; | 1 |  | Consider introducing new lines and change the text to 2> if the frequency concerns the primary carrier: 3> use the PCell as reference; 2> else if the frequency concerns a secondary carrier: 3> use the concerned SCell as reference; 2> else if the UE is configured with discTxRefCarrierDedicated for the frequency: 3> use the cell indicated by this field as reference; 2> else if the UE is configured with discTxRefCarrierCommon for the frequency: 3> use the serving cell (RRC_IDLE)/ PCell (RRC_CONNECTED) as reference; 2> else: 3> use the DL frequency paired with the one used to transmit sidelink discovery announcements on as reference; Huawei: fine. Intel: ok |  | Closed (ProSe CR) |
| L.015 | 5.10.7.1 | For discovery, the sync information also concerns information in MIB-SL. | 2 |  | The purpose of this procedure is to provide synchronisation information to a UE. The synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) for sidelink discovery, while it concerns an SLSS, timing information and some additional configuration parameters (i.e. the MasterInformationBlock-SL message) for sidelink communication/discovery. Ericsson: Same as L.015. We agree to the intention, but we are not sure the exact formulation suggested by ZTE is the best one. What about: “The synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) for sidelink discovery and additionally the MasterInformationBlock-SL message for PS related sidelink discovery.” ZTE: the sync information may concerns MIB-SL only for PS discovery. Refer to our comment Z.035. Intel: ok (same issue as Z.035) Huawei: slightly prefer the way proposed in Z.035. CATT: agree with the comment and change. Qualcomm: Similar to Z.035. Agree with the issue. However the proposed text doesnot capture the case for in-coverage commercial discovery MIB-SL is still not transmitted by UE. MIB-SL is transmitted only for PS UE. Samsung: See Z.035 |  | Closed (ProSe CR) |
| Z.036 | 5.10.7.2 | 3> if the sidelink discovery announcements do not concern public safety; or if syncTxPeriodic is not included: 4> transmit SLSS on the frequency used for sidelink discovery in accordance with 5.10.7.3 and TS 36.211 [21]; The condition “if the sidelink discovery announcements do not concern public safety;” could be deleted since the condition “if syncTxPeriodic is not included” is enough. | 2 |  | The sentence “if the sidelink discovery announcements do not concern public safety; or if syncTxPeriodic is not included:” could be replaced by “if syncTxPeriodic is not included:” Huawei: do not understand the motivation of this change. The behaviors for PS and non-PS SL discovery should be different. Intel: prefer the original text since it’s clearer. Ericsson: We are not sure this is a correct change. That would change the behaviour of the UE. Is that in line with the RAN1 agreements? Qualcomm: No need to change, current text is fine. Samsung: No change (no support). If there are still concerns, a paper may be brought to RAN2#93 |  | Closed (ProSe CR) |
| E.138 | 5.10.7.2 | Wrong terminology 3> if the sidelink discovery announcements do not concern public safety; or if syncTxPeriodic is not included: | 1 |  | Change the text to 3> if the sidelink discovery announcements do not concern public safetyare non-PS related; or if syncTxPeriodic is not included: Huawei: fine with either way. Intel: ok |  | Closed (ProSe CR) |
| E.139 | 5.10.7.2 | Ambiguous statement 3> else (i.e. periodic): | 1 |  | Ideally the part inside the parantheses should be removed, but we already have it on numerous places. In this case, the text inside is not complete, as the else statement also includes that case for public safety according to the preceding if-statement. Change the text to 3> else (i.e. periodic or public safety): Huawei: Not sure “periodic or public safety” is correct. Shouldn’t be “periodic and public safety”? Maybe the addition is not needed. Intel: we slightly prefer removing of “(i.e. xxx)” |  | Closed (ProSe CR) |
| E.287 | 5.10.7.2 | 3> if the sidelink discovery announcements do not concern public safety; or if syncTxPeriodic is not included: ………….. 3> if the sidelink discovery announcements concern public safety; and if syncTxPeriodic is included: | 1 |  | In other places we have used: for PS related sidelink discovery announcements. Huawei: fine with either way. Intel: prefer alignments (i.e. for PS related sidelink discovery announcements) |  | Closed (ProSe CR) |
| E.140 | 5.10.7.3 | Ambiguous language 3> if the sidelink discovery announcements concern public safety; and if syncTxPeriodic is included: 4> additionally select every subframe that is 40 subframes after the selected subframe; What we want to say is that we select the one subframe occurring peiodically 40 ms. It is possible to interpret the above text so that all subframes occurring after 40 ms are selected. | 1 |  | Change the text to 3> if the sidelink discovery announcements concern public safety; and if syncTxPeriodic is included: 4> additionally select eachevery subframe that is occurring periodically 40 subframes after the selected subframe; Huawei: fine. Can further improve if needed. Intel: ok with either way Ericsson: This is same issue as C.031 CATT: agree, same issue as C.031. |  | Closed (ProSe CR) |
| C.031 | 5.10.7.3 Transmission of SLSS | 4> additionally select every subframe that is 40 subframes after the selected subframe; may be confused. | 2 |  | 3> if the sidelink discovery announcements concern public safety; and if syncTxPeriodic is included: 4> additionally select every subframe that is 40 subframes after the selected subframeselect the subframe with a periodicity of 40 ms; Huawei: alightly prefer the sentence proposed in E.140. Intel: prefer either the original text or the Ericsson proposal (same issue as E.140) Ericsson: This is same issue as E.140. We prefer our proposed solution in E.140, but we are open to discuss a precise formulation. ZTE: don't agree. The current description is clear enough. Qualcomm: Ok |  | Closed (ProSe CR) |
| C.032 | 5.10.7.4 Transmission of MasterInformationBlock-SL message | MIB-SL triggered by sidelink discovery and in coverage shall use the IE received in SIB19. | 2 |  | The UE shall set the contents of the MasterInformationBlock-SL message as follows: 1> if triggered by sidelink communication and in coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4, 11.4]; or 1> if triggered by sidelink discovery and in coverage on the frequency used for sidelink discovery, as defined in TS 36.304 [4, 11.4]: 2> set inCoverage to TRUE; 2> set sl-Bandwidth to the value of ul-Bandwidth as included in the received SystemInformationBlockType2 of the cell chosen for sidelink communication; 2> if tdd-Config is included in the received SystemInformationBlockType1: 3> set subframeAssignmentSL to the value representing the same meaning as of subframeAssignment that is included in tdd-Config in the received SystemInformationBlockType1; 2> else: 3> set subframeAssignmentSL to none; 2> if syncInfoReserved is included in an entry of commSyncConfig from the received SystemInformationBlockType18; 3> set reserved to the value of syncInfoReserved in the received SystemInformationBlockType18; 1> if triggered by sidelink discovery and in coverage on the frequency used for sidelink discovery, as defined in TS 36.304 [4, 11.4]: 2> set inCoverage to TRUE; 2> set sl-Bandwidth to the value of ul-Bandwidth as included in the received SystemInformationBlockType2 of the cell chosen for sidelink communication; 2> if tdd-Config is included in the received SystemInformationBlockType1: 3> set subframeAssignmentSL to the value representing the same meaning as of subframeAssignment that is included in tdd-Config in the received SystemInformationBlockType1; 2> else: 3> set subframeAssignmentSL to none; 2> if syncInfoReserved is included in an entry of discSyncConfig from the received SystemInformationBlockType19; 3> set reserved to the value of syncInfoReserved in the received SystemInformationBlockType19; Huawei: Fine with the motivation, the text can be further extracted. Intel: not clear what the difference between the original text and the proposal. May need further discussion to clarify the issue. Ericsson: Agree to the problem, but perhaps the proposed solution can be “compressed” to reduce the amount of redundant statements. eg: 1> if triggered by sidelink communication and in coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4, 11.4]; or 1> if triggered by sidelink discovery and in coverage on the frequency used for sidelink discovery, as defined in TS 36.304 [4, 11.4]: 2> set inCoverage to TRUE; 2> set sl-Bandwidth to the value of ul-Bandwidth as included in the received SystemInformationBlockType2 of the cell chosen for sidelink communication; 2> if tdd-Config is included in the received SystemInformationBlockType1: 3> set subframeAssignmentSL to the value representing the same meaning as of subframeAssignment that is included in tdd-Config in the received SystemInformationBlockType1; 2> else: 3> set subframeAssignmentSL to none; 2> if triggered by sidelink communication and in coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4, 11.4] and if syncInfoReserved is included in an entry of commSyncConfig from the received SystemInformationBlockType18; 3> set reserved to the value of syncInfoReserved in the received SystemInformationBlockType18; 2> else if triggered by sidelink discovery and in coverage on the frequency used for sidelink discovery, as defined in TS 36.304 [4, 11.4] and if syncInfoReserved is included in an entry of discSyncConfig from the received SystemInformationBlockType19; 3> set reserved to the value of syncInfoReserved in the received SystemInformationBlockType19; 2> else: 3> set all bits in reserved to 0; Qualcomm: Agree. However only following part is different rest of the things can be kept as it is: 2> if syncInfoReserved is included in an entry of discSyncConfig from the received SystemInformationBlockType19; 3> set reserved to the value of syncInfoReserved in the received SystemInformationBlockType19; Samsung: Changed in accordance with suggestion from QC (i.e. keeping as much as possible common) |  | Closed (ProSe CR) |
| C.033 | 5.10.8.2 Selection and reselection of synchronisation reference UE (SyncRef UE) | According to the agreement “If the S-criteria on the PS ProSe Carrier is not met, the UE can use PS ProSe Carrier discovery resources preconfigured in the UICC or ME for out of coverage PS Discovery”, the PS discovery UE determines whether it’s out of coverage only based on PS ProSe Carrier. The current text seems to indicate the PS discovery UE has to be out of coverage on every frequency for discovery, which is as following: 1> for the frequency used for sidelink communication, if out of coverage on that frequency as defined in TS 36.304 [4, 11.4] and for each other frequency used for sidelink discovery on which the UE is out of coverage, as defined in TS 36.304 [4, 11.4]: | 2 |  | Should clarify the PS discovery UE determines whether it’s out of coverage only based on PS ProSe carrier. Huawei: Do not understand the issue. There could be multiple PS ProSe carriers for sidelinnk discovery. Intel: not clear on the issue. In the same sentence, we don’t have that for sidelink communication. Why do we need for discovery case? Ericsson: We have some understanding for this issue but we would like to clarify which carrier is the PS ProSe Carrier? Is it carrierFreq in the preconfiguration? Qualcomm: Agree. It is same as E.289 Samsung: No change, but as same as E.289 (so this copy can be closed) |  | Closed (ProSe CR) |
| Z.037 | 5.10.9.1 | The MIB-SL message is not only sent by the UE configured to receive or transmit sidelink communication but also can be sent by the UE configured to receive or transmit PS related sidelink discovery. But the description of ‘ A UE configured to receive or transmit sidelink communication shall:’ does not include the UE configured to receive or transmit PS related sidelink discovery. | 2 |  | Suggest to replace ‘ A UE configured to receive or transmit sidelink communication shall:’by ‘‘ A UE configured to receive or transmit sidelink communication or PS related sidelink discovery shall:’’ Huawei: fine. Intel: ok Ericsson: Agree. CATT: This change is necessary. Qualcomm: Agree. |  | Closed (ProSe CR) |
| N.189 | 5.10.x (D2D) | We use “if configured” in many places, could consider using “if UE is configured” instead | 2 |  | Discuss if it would be good to use “if UE is configured” instead of “if configured” in procedural text. Huawei: fine. Intel: ok with either way Ericsson: We can discuss, but to be hoest we have found this particularly confusing. CATT: In order to keep alignment of the description, we support this change. Qualcomm: No need to change Samsung: No change as used several times in REL-12. If still concerns, a proposal may be brought to RAN2#93 |  | Closed (ProSe CR) |
| Z.019 | 5.10 | In 5.10, the relay related sidelink communication is widely used. However, in some section, other terms are used to denote the relay related sidelink communication, such as relay communication, sidelink (relay) communication. In order to avoid confusion, it is suggested to unify the term usage, such as relay related sidelink communication. | 1 |  | Unify the following term: sidelink (relay) communication/ discovery announcements, relay related sidelink communication, relay communication. Huawei: Agree. Suggest using “relay related sidelink communication”. Intel: ok Ericsson: Agree, and we have a proposal in E.098. Qualcomm: Agree. Samsung: Assumed to be covered by agreements based on R2-160015 |  | Closed (ProSe CR) |
| Z.020 | 5.10.1 | The description of “The sidelink communication/ discovery/ synchronisation resource configuration applies for the frequency at which it was received/ acquired.” excludes the case of inter-frequency discovery resource configuration. | 2 |  | The sentence could be either deleted or modified. Huawei: agree. Samsung: Modification seems best (inter-frequency assignment only valid for discovery) Intel: We prefer to modify Ericsson: Agree to the problem. We prefer the sentence to be modified rather than deleted. Qualcomm: Agree Coordinator: Modification seems to be acceptable. If there is a modification proposal, the sentence can bemodified. Otherwise the issue can be closed. Samsung: Covered by agreements based on R2-160049 |  | Closed (ProSe CR) |
| Z.021 | 5.10.1 | The description of “Moreover, for a UE configured with one or more SCells, the sidelink communication/ discovery/ synchronisation resource configuration provided by dedicated signalling applies for the PCell/ the primary frequency.” excludes the case of configuring the discovery resource on Scell by dedicated signaling. | 2 |  | The sentence could be either deleted or modified. Huawei: agree. Samsung: Same remark as for Z.020 Intel: We prefer to modify Ericsson: Agree to the problem. We prefer the sentence to be modified rather than deleted. Qualcomm: Agree Coordinator: Modification seems to be acceptable. If there is a modification proposal, the sentence can bemodified. Otherwise the issue can be closed. Samsung: Covered by agreements based on R2-160049 |  | Closed (ProSe CR) |
| E.093 | 5.10.1 | The following text is unclear. The specification covers the use of UE to network sidelink relays by specifying the additional requirements that apply for a sidelink relay UE and a sidelink remote UE. I.e. for such UEs the regular sidelink UE requirements equally apply unless explicitly stated otherwise. More specifically, this text introduces - “UE to network sidelink relays” - “sidelink relay UE” - “sidelink remote UE” These are not clearly defined anywhere. | 2 |  | Discuss the purpose of this text and improve it, possibly by adding suitable definitions to section 3.2. Huawei: Suggest to clarify the sentence. Suggest to align the terminologies. “Sidelink relay UE” and “Sidelink remote UE” are preferred. Samsung: Improvements can be considered. Main intention is to say that requirements specified for a relay also apply to a relay UE i.e. that additional requirements only applicable to relay UE have been added. Intel: ok Qualcomm: Text seems ok. However, no strong opinion either way. Samsung: Assumced that no further changes are needed (i.e. given changes based on R2-160015 and R2-160049 |  | Closed (ProSe CR) |
| E.141 | 5.10.11 | Wrong terminology in title 5.10.11 Sidelink relay operation | 1 |  | Change title to 5.10.11 Sidelink relay UE operation Huawei: agree. Intel: ok, but saying sidelink UE-to-NW relay UE operation may be more accurate. Qualcomm: OK. Samsung: Changed (5.10.10 and .11) |  | Closed (ProSe CR) |
| Z.038 | 5.10.11.2 | “commTxPoolResources” should be “commTxResources”. A UE capable of sidelink relay operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met: 1> if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its value is set to true; | 1 |  | Change “commTxPoolResources” to “commTxResources”. 1> if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its value is set to true; Huawei: Agree. Intel: ok Qualcomm: Agree. Samsung: Corrected |  | Closed (ProSe CR) |
| E.148 | 5.10.11.3 | If statement is incorrect, should be else if. 2> if AS-layer conditions specified in this section were met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19 (stay); | 1 |  | Change text to 2> else if AS-layer conditions specified in this section were met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19 (stay) Huawei: There are three conditions and the UE is allowed for sidelink discovery transmission if meeting any of the conditions. Therefore, suggest changing to 2> if neither discThreshHiRelayUE nor discThreshLoRelayUE is included in SystemInformationBlockType19; or 2> else if AS-layer conditions specified in this section were not met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE by discHystMaxRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE by discHystMinRelayUE, if included in SystemInformationBlockType19 (entry); or 2> if AS-layer conditions specified in this section were met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19 (stay); Intel: ok Qualcomm: Agree |  | Closed (ProSe CR) |
| Z.039 | 5.10.12.2 | - It is suggested to remove “relay ” in title of 5.10.Y.2 to keep consistent with other titles (5.10.X.2, 5.10.X.3, 5.10.Y.3). - Out of coverage remote UE is preconfigured with preconfigComm (including SL-PreconfigCommPoolList4 or commTxPoolListExt) to perform relay communication transmission. - Missing “else” before “if in RRC_CONNECTED”. - “commTxPoolResources” should be “commTxResources”. #### 5.10.Y.2 AS- conditions relay communication transmission for remote UE A UE capable of sidelink remote operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met: 1> if the UE is out of coverage; and is preconfigured with SL-Preconfiguration including discTxPoolList and preconfigRelay; or 1> else if in RRC_IDLE; and if the UE’s serving cell is suitable as defined in TS 36.304 [4]; and if SystemInformationBlockType18 includes commTxPoolNormalCommon and commTxAllowRelayCommon; and if SystemInformationBlockType19 includes discConfigRelay: 2> if discThreshHiRemoteUE is not included in SystemInformationBlockType19; 2> else if the AS-layer radio conditions specified in this section were previously not met; and if the RSRP measurement of the PCell is below discThreshHiRemoteUE by discHystMaxRemoteUE (entry); 2> if the AS-layer radio conditions specified in this section were previously met; and the RSRP measurement of the PCell is below discThreshHiRemoteUE (stay); 1> if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its value is set to true; | 1 |  | - Remove “relay ” in the title of 5.10.Y.2 - Change “discTxPoolList” to “preconfigComm”. - Addition of “else” before “if in RRC_CONNECTED”. - Change “commTxPoolResources” to “commTxResources”. #### 5.10.Y.2 AS- conditions relay communication transmission for remote UE A UE capable of sidelink remote operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met: 1> if the UE is out of coverage; and is preconfigured with SL-Preconfiguration including discTxPoolListpreconfigComm and preconfigRelay; or 1> else if in RRC_IDLE; and if the UE’s serving cell is suitable as defined in TS 36.304 [4]; and if SystemInformationBlockType18 includes commTxPoolNormalCommon and commTxAllowRelayCommon; and if SystemInformationBlockType19 includes discConfigRelay: 2> if discThreshHiRemoteUE is not included in SystemInformationBlockType19; 2> else if the AS-layer radio conditions specified in this section were previously not met; and if the RSRP measurement of the PCell is below discThreshHiRemoteUE by discHystMaxRemoteUE (entry); 2> if the AS-layer radio conditions specified in this section were previously met; and the RSRP measurement of the PCell is below discThreshHiRemoteUE (stay); 1> else if in RRC_CONNECTED; and if the UE is configured with commTxPoolResources set to scheduled or ue-Selected; and the UE is configured with commTxAllowRelayDedicated while its value is set to true; #### Huawei: Suggest changing title to “AS- conditions for relay related sidelink communication transmission for sidelink remote UE”. Agree with other changes. Samsung: Re. 1st change: note that these conditions are about communication via relay UE, not via eNB (while still in coverage) Intel: ok Qualcomm: OK for 1, 3 and 4. On 2, No need to change as discPool is new addition and is required for relay operation. Communication pool is anyway present all the time. Samsung: Performed change 1, 3 and 4 |  | Closed (ProSe CR) |
| Z.024 | 5.10.2.2 | Instead of colon, a semicolor should be used after the condition. Also since it is not the last if condition, a “or” should be added. | 1 |  | The original text is as follows: 4> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 4> if the UE is acting as relay; and if SystemInformationBlockType19 includes discConfigRelay; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19: 4> if the UE is selecting a relay/ has a selected relay; and if SystemInformationBlockType19 includes discConfigRelay; and if discThreshHiRemoteUE is not included or the RSRP measurement of the PCell is below discThreshHiRemoteUE: 5> initiate transmission of the SidelinkUEInformation message to indicate the PS related sidelink discovery announcement resources required by the UE in accordance with 5.10.2.3; It is suggested to change as follows: 4> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 4> if the UE is acting as relay; and if SystemInformationBlockType19 includes discConfigRelay; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19; or 4> if the UE is selecting a relay/ has a selected relay; and if SystemInformationBlockType19 includes discConfigRelay; and if discThreshHiRemoteUE is not included or the RSRP measurement of the PCell is below discThreshHiRemoteUE: 5> initiate transmission of the SidelinkUEInformation message to indicate the PS related sidelink discovery announcement resources required by the UE in accordance with 5.10.2.3; Huawei: agree with the change. Intel: ok Qualcomm: Ok |  | Closed (ProSe CR) |
| Z.025 | 5.10.2.2 | A UE capable of sidelink communication or discovery may initiate the procedure to request assignment of dedicated resources for the concerned sidelink communication transmission or discovery announcements and to report parameters related to Sidelink operations from system information of inter-frequency cells. | 1 |  | Suggest to replace “inter-frequency” by “inter-frequency/PLMN”, in order to keep aligned with other sections. Huawei: Fine to have or not to have this change. Intel: ok Qualcomm: Agree |  | Closed (ProSe CR) |
| L.010 | 5.10.5 | Change ‘discRxPoolPS’ to ‘discRxResourcesPS’ | 2 |  | 1> else if the PCell/ the cell the UE is camping on indicates a pool of resources to monitor sidelink discovery announcements on by discRxPoolPSdiscRxResourcesPS within discInterFreqList in SystemInformationBlockType19: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPoolPSdiscRxResourcesPS in SystemInformationBlockType19 ; Huawei: Fine. Intel: ok Ericsson: Agree. This is same as Z.027. Qualcomm: Ok. Same as Z.027 |  | Closed (ProSe CR) |
| Z.030 | 5.10.6 | “discTxResourcesRelay” should be “discTxResourcesPS”. 1> else if the frequency used to transmit sidelink discovery concerns the serving frequency (RRC_IDLE)/ primary frequency (RRC_CONNECTED); and if the UE’s serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) is suitable as defined in TS 36.304 [4]: 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 2> if the UE is acting as relay; and if the RSRP measurement of the PCell/ the cell on which the UE camps is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19; or 2> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19: 3> if the UE is configured with discTxPoolPS-Dedicated; or 3> if the UE is in RRC_IDLE; and if discTxPoolPS-Common is included in SystemInformationBlockType19: 4> select an entry of the list of resource pool entries and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; 3> else if the UE is configured with discTxResourcesRelay set to scheduled: 4> configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResourcesRelay; | 1 |  | Change “discTxResourcesRelay” to “discTxResourcesPS”. 1> else if the frequency used to transmit sidelink discovery concerns the serving frequency (RRC_IDLE)/ primary frequency (RRC_CONNECTED); and if the UE’s serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) is suitable as defined in TS 36.304 [4]: 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements; or 2> if the UE is acting as relay; and if the RSRP measurement of the PCell/ the cell on which the UE camps is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19; or 2> if the UE is selecting a relay/ has a selected relay; and if the RSRP measurement of the cell on which the UE camps is below discThreshHiRemoteUE, if included in SystemInformationBlockType19: 3> if the UE is configured with discTxPoolPS-Dedicated; or 3> if the UE is in RRC_IDLE; and if discTxPoolPS-Common is included in SystemInformationBlockType19: 4> select an entry of the list of resource pool entries and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; 3> else if the UE is configured with discTxResourcesRelaydiscTxResourcesPS set to scheduled: 4> configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResourcesRelaydiscTxResourcesPS; Huawei: Agree. Intel: ok |  | Closed (ProSe CR) |
| Z.031 | 5.10.6 | “discTxResources” should be “discTxResourcesPS”. 1> else if, for the frequency used to transmit sidelink discovery on, the UE is configured with dedicated resources (i.e. with discTxResourcesPS in discTxInterFreqInfo within sl-DiscConfig); and the conditions for sidelink operation as defined in 5.10.1a are met: 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements: 3> if the UE is configured with discTxResourcesPS set to scheduled: 4> configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResources; | 1 |  | Change “discTxResources” to “discTxResourcesPS”. 1> else if, for the frequency used to transmit sidelink discovery on, the UE is configured with dedicated resources (i.e. with discTxResourcesPS in discTxInterFreqInfo within sl-DiscConfig); and the conditions for sidelink operation as defined in 5.10.1a are met: 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements: 3> if the UE is configured with discTxResourcesPS set to scheduled: 4> configure lower layers to transmit the sidelink discovery announcement using the assigned resources indicated by scheduled in discTxResourcesPS; Huawei: Agree. Intel: ok |  | Closed (ProSe CR) |
| Z.032 | 5.10.6 | “discTxPoolCommon” should be “discTxPoolPS-Common”. 1> else if discTxPoolPS-Common is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements 3> select an entry of the list of resource pool entries in discTxPoolCommon and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; | 1 |  | Change “discTxPoolCommon” to “discTxPoolPS-Common”. 1> else if discTxPoolPS-Common is included in SystemInformationBlockType19 acquired from cell selected on the discovery announcement frequency; and the conditions for sidelink operation as defined in 5.10.1a are met: 2> if configured by upper layers to transmit non-relay PS related sidelink discovery announcements 3> select an entry of the list of resource pool entries in discTxPoolPS-Common and configure lower layers to use it to transmit the sidelink discovery announcements as specified in 5.10.6a; Huawei: Agree. Intel: ok |  | Closed (ProSe CR) |
| Z.035 | 5.10.7.1 | The description of “The synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) for sidelink discovery, while it concerns an SLSS, timing information and some additional configuration parameters (i.e. the MasterInformationBlock-SL message) for sidelink communication.” is not accurate, since the synchronisation information may also concerns the MasterInformationBlock-SL message for PS discovery in R13. | 2 |  | The sentence “The synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) for sidelink discovery,” could be replaced by “The synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) (and may concerns the MasterInformationBlock-SL message for PS sidelink discovery) for sidelink discovery,” Huawei: agree. Intel: ok Ericsson: Same as L.015. We agree to the intention, but we are not sure the exact formulation suggested by ZTE is the best one. What about: “The synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) for sidelink discovery and additionally the MasterInformationBlock-SL message for PS related sidelink discovery.” Qualcomm: Agree that it needs update for Discovery OOC. Need to work on better text. Samsung: Changed to the following: For sidelink discovery, the synchronisation information concerns a Sidelink Synchronisation Signal (SLSS) and, in case of PS related discovery, also timing information and some additional configuration parameters (i.e. the MasterInformationBlock-SL message), while for sidelink communication it concerns an SLSS and the MasterInformationBlock-SL message |  | Closed (ProSe CR) |
| E.284 | 5.10.5 | 3> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPool in SystemInformationBlockType19; NOTE 1: The requirement not to affect normal UE operation also applies for the acquisition of sidelink discovery related system and synchronisation information from inter-frequency cells. | 1 |  | Since the sentence of not affecting normal UE operation has been deleted then also the note should be deleted and moved few lines below when we talk about gaps Huawei: Fine to improve the description. Could be like 3> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPool in SystemInformationBlockType19 without affecting normal operation i.e. receive during idle periods or sidelink discovery RX gaps, or by using a spare receiver; NOTE 1: The requirement not to affect normal UE operation also applies for the acquisition of sidelink discovery related system and synchronisation information from inter-frequency cells. Intel: we think it’s good to keep it. In our understanding, it points out acquisition of system information and synchronization should be also considered not to affect normal UE operation when gap is not configured. Samung: No change i.e. there was no deletion of the requirement not to affect normal operation (but just some move) |  | Closed (ProSe CR) |
| H.060 | 5.10.6 | 1> if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig, 1> else: 2> configure lower layers to transmit on the concerned frequency without affecting normal operation i.e. transmit during idle periods or by using a spare receiver; | 1 |  | 1> if the UE is configured with discTxGapConfig and requires gaps to transmit sidelink discovery announcements on the concerned frequency; 2> configure lower layers to transmit on the concerned frequency using the gaps indicated by discTxGapConfig, 1> else: 2> configure lower layers to transmit on the concerned frequency without affecting normal operation i.e. transmit during idle periods or by using a spare receivertransmitter; Intel: ok, or the other option would be to remove the part “i.e…xxx”. |  | Closed (ProSe CR) |
| C.036 | #### 5.10.11.3 AS- conditions discovery transmission for relay UE | “else” is missing. | 1 |  | #### 5.10.11.3 AS- conditions discovery transmission for relay UE A UE capable of sidelink relay operation shall inform upper layers that it is configured with radio resources that can be used for relay related sidelink discovery transmission if the following conditions are met: 1> if in RRC_IDLE; and if the UE’s serving cell is suitable as defined in TS 36.304 [4]; and if SystemInformationBlockType19 includes discConfigPS including discTxPoolPS-Common and discConfigRelay: 2> if neither discThreshHiRelayUE nor discThreshLoRelayUE is included in SystemInformationBlockType19; 2> else if AS-layer conditions specified in this section were not met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE by discHystMaxRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE by discHystMinRelayUE, if included in SystemInformationBlockType19 (entry); 2> else if AS-layer conditions specified in this section were met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19 (stay); 1> else if in RRC_CONNECTED; and if discTxResourcesPS is configured (i.e. set to scheduled or ue-Selected); Huawei: suggest changing to 2> if neither discThreshHiRelayUE nor discThreshLoRelayUE is included in SystemInformationBlockType19; or 2> else if AS-layer conditions specified in this section were not met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE by discHystMaxRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE by discHystMinRelayUE, if included in SystemInformationBlockType19 (entry); or 2> if AS-layer conditions specified in this section were met; and if the RSRP measurement of the PCell is below discThreshHiRelayUE, if included in SystemInformationBlockType19; and above discThreshLoRelayUE, if included in SystemInformationBlockType19 (stay); Intel: ok (same issue as E.148) Qualcomm: Agree. It is same as E.148. Coordinator: Same as E.148 |  | Closed |
| L.016 | 5.10.12.2 | Wrong field name | 1 |  | 1> if the UE is out of coverage; and is preconfigured with SL-Preconfiguration including preconfigCommdiscTxPoolList and preconfigRelay; or Huawei: agree. Intel: ok (same issue as Z.039) Qualcomm: Similar to issue 2 of Z.039. No need to change. Coordinator: Same as Z.039 |  | Closed |
| E.280 | 5.10.2.2 | initiate transmission of the SidelinkUEInformation message to indicate it does no longer require public safety related sidelink discovery announcement resources in accordance with 5.10.2.3; | 1 |  | Use abbreviation PS Huawei: Agree. Intel: ok Qualcomm: No strong opinion either way Coordinator: Same as E.104. |  | Closed |
| E.095 | 5.10.2.2 | The term “acting as relay” is ambiguous. | 1 |  | Change to “acting as a sidelink relay UE”. Multiple occurances. Huawei: fine. Samsung: Same comment sas for E.082 Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE. Qualcomm: Ok Coordinator: Same as E.082 |  | Closed |
| E.096 | 5.10.2.2 | The term “has a selected relay” is ambiguous. | 1 |  | Change to “has a selected sidelink relay UE”. Multiple occurrances. Huawei: fine. Samsung: Same comment as for E.082 Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE. Qualcomm: Ok Coordinator: Same as E.082 |  | Closed |
| E.097 | 5.10.2.2 | The term “is selecting a relay” is ambiguous. | 1 |  | Change to “is selecting a sidelink relay UE”. Multiple occurrances. Huawei: fine.Samsung: Same comment as for E.082 Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE. Qualcomm: Ok Coordinator: Same as E.082 |  | Closed |
| E.277 | 5.10.2.2 | #### Sidelink operations | 1 |  | No need to have sidelink capital Huawei: Agree. Intel: ok Qualcomm: ok Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.105 | 5.10.2.3 | The term “acting as relay” is ambiguous. | 1 |  | Change to “acting as a sidelink relay UE”. Multiple occurances. Huawei: fine.Samsung: See E.082 Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE. Qualcomm: Ok Coordinator: Same as E.082 |  | Closed |
| E.106 | 5.10.2.3 | The term “has a selected relay” is ambiguous. | 1 |  | Change to “has a selected sidelink relay UE”. Multiple occurrances. Huawei: fine. Samsung: See E.082 Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE Qualcomm: Ok Coordinator: See E.082 |  | Closed |
| E.107 | 5.10.2.3 | The term “is selecting a relay” is ambiguous. | 1 |  | Change to “is selecting a sidelink relay UE”. Multiple occurrances. Samsung: See E.082 Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE. Qualcomm: Ok Coordinator: See E.082 |  | Closed |
| E.112 | 5.10.2.3 | The term “acting as a relay” is ambiguous. | 1 |  | Change to “acting as a sidelink relay”. Two occurrances. Huawei: “acting as a sidelink relay UE” Samsung: See E.082 (will stop repeating from now on) Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE” Qualcomm: Repeated comment Coordinator: See E.082 |  | Closed |
| E.113 | 5.10.2.3 | The term “has a selected relay” is ambiguous. | 1 |  | Change to “has a selected sidelink relay”. Two occurrances. Huawei: “has a selected sidelink relay UE” Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE”.. Coordinator: See E.082 |  | Closed |
| E.282 | 5.10.2.3 | 1> else if the UE initiates the procedure to report the system information parameters related to Sidelink operations of carriers other than the primary | 1 |  | Sidelink no need to be capital Huawei: Agree. Intel: ok Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.120 | 5.10.4 | The term “has a selected relay” is ambiguous. | 1 |  | Change to “has a selected sidelink relay UE”. Multiple occurrances. Huawei: fine. Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE. Coordinator: See E.082 |  | Closed |
| E.121 | 5.10.4 | The term “is selecting a relay” is ambiguous. | 1 |  | Change to “is selecting a sidelink relay UE”. Multiple occurrances. Huawei: fine. Intel: ok but to be more accurate, we may better say sidelink UE-to-NW relay UE. Coordinator: See E.082 |  | Closed |
| Z.027 | 5.10.5 | The IE discRxPoolPS is not included in discInterFreqList. The IE represent monitoring resources for PS sidelink discovery in discInterFreqList is discRxResourcesPS. A UE capable of PS related sidelink discovery that is configured by upper layers to monitor PS related sidelink discovery announcements shall: 1> if out of coverage on the frequency, as defined in TS 36.304 [4, 11.4]: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources that were preconfigured (i.e. indicated by discRxPoolList within preconfigDisc in SL-Preconfiguration defined in 9.3); 1> else if the PCell/ the cell the UE is camping on indicates a pool of resources to monitor sidelink discovery announcements on by discRxPoolPS within discInterFreqList in SystemInformationBlockType19: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPoolPS in SystemInformationBlockType19 ; | 1 |  | Change discRxPoolPS to discRxResourcesPS. 1> else if the PCell/ the cell the UE is camping on indicates a pool of resources to monitor sidelink discovery announcements on by discRxPoolResourcesPS within discInterFreqList in SystemInformationBlockType19: 2> configure lower layers to monitor sidelink discovery announcements using the pool of resources indicated by discRxPoolResourcesPS in SystemInformationBlockType19 Huawei: Fine. Intel: ok Ericsson: Agree. This is same as L.010. Qualcomm: Ok Coordinator: Same as L.010 |  | Closed |
| E.230 | 5.10.5 Sidelink discovery monitoring 5.10.6 Sidelink discovery announcement 5.10.7.2 Initiation 5.10.7.4 Transmission of MasterInformationBlock-SL message 5.10.12.4 Selection and reselection of relay | There should be TABs between “>”and the procedure text. There should not be any SPACEs between “>” and the procedure text. | 1 |  | (If not already fixed in the v13.0.0 CR implementation) add TABs in the procedures and remove SPACEs. Huawei: fine. Intel: ok Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.128 | 5.10.6 | Descriptions of the variants of sidelink discovery are missing. | 1 |  | Add the following text to the beginning of the section. Sidelink discovery consists of Public safety related (PS related) and non-PS related sidelink discovery. PS related sidelink discovery consists of relay related and non-relay related PS related sidelink discovery. Upper layers indicate to RRC whether a particular announcement is PS related or non-PS related. Huawei: The comments are duplicated??? Intel: if we put the “PS” into abbreviations (see E.182), shouldn’t the current text be ok as it is? Coordinator: See alsoE.086, E.087 |  | Closed |
| E.129 | 5.10.6 | The term “acting as relay” is ambiguous. | 1 |  | Change to “acting as a sidelink relay UE”. Multiple occurances. Huawei: The comments are duplicated??? Intel: ok, but to be more accurate, we may better say sidelink UE-to-NW relay UE. Coordinator: See E.082 |  | Closed |
| E.130 | 5.10.6 | The term “has a selected relay” is ambiguous. | 1 |  | Change to “has a selected sidelink relay UE”. Multiple occurrances. Huawei: The comments are duplicated??? Intel: ok, but to be more accurate, we may better say sidelink UE-to-NW relay UE Coordinator: See E.082 |  | Closed |
| E.131 | 5.10.6 | The term “is selecting a relay” is ambiguous. | 1 |  | Change to “is selecting a sidelink relay UE”. Multiple occurrances. Huawei: The comments are duplicated??? Intel: ok, but to be more accurate, we may better say sidelink UE-to-NW relay UE Coordinator: See E.082 |  | Closed |
| 6.2.1 General message structure |  |  |  |  |  |  |  |
| D.016 | 6.2.1 Paging | Style error of referring to the other specification. | 1 |  | Should be corrected as follows. redistributionIndication If present: indication to trigger E-UTRAN Inter-frequency Redistribution procedure as specified in TS 36.304 [4, 5.2.4.z [4] Coordinator: Corrected in general CR |  | Closed (General CR) |
| 6.2.2 Message definitions |  |  |  |  |  |  |  |
| Z.048 | 6.2.2 | SidelinkUEInformation-v13x0-IEs ::= SEQUENCE { commTxResourceReq121-r13 SL-CommTxResourceReqUC-r13 OPTIONAL, commTxResourceInfoReqRelay-r13 SEQUENCE { commTxResourceReqRelay-r13 SL-CommTxResourceReqUC-r13, ue-Type-r13 ENUMERATED {relayUE, remoteUE} } Non-relay related one to one communication is not discussed in R13 and no conclusion is reached in RAN2 meeting. So we doubt whether it is proper to specify it now. | 2 | Non-relay related one to one communication is not discussed in R13 and no conclusion is reached in RAN2 meeting. So we doubt whether it is proper to specify it now. Huawei: non-relay one to one has been introduced in Rel-13, should be included here. Ericsson: Disagree. Qualcomm: In RAN2 we discussed and agreed for non-relay related one-to-one communication. So we disagree with this issue. Samsung: See Z.022 |  |  | Closed (ProSe CR) |
| C.042 | 6.2.2 SidelinkUEInformation message | UE should also indicate it’s remote or relay when requesting relay discovery resources for the purpse of authorization and resourse management. | 3 | SidelinkUEInformation message -- ASN1START SidelinkUEInformation-r12 ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE { sidelinkUEInformation-r12 SidelinkUEInformation-r12-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } SidelinkUEInformation-r12-IEs ::= SEQUENCE { commRxInterestedFreq-r12 ARFCN-ValueEUTRA-r9 OPTIONAL, commTxResourceReq-r12 SL-CommTxResourceReq-r12 OPTIONAL, discRxInterest-r12 ENUMERATED {true} OPTIONAL, discTxResourceReq-r12 INTEGER (1..63) OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SidelinkUEInformation-v13x0-IEs OPTIONAL } SidelinkUEInformation-v13x0-IEs ::= SEQUENCE { commTxResourceReq121-r13 SL-CommTxResourceReqUC-r13 OPTIONAL, commTxResourceInfoReqRelay-r13 SEQUENCE { commTxResourceReqRelay-r13 SL-CommTxResourceReqUC-r13, ue-Type-r13 ENUMERATED {relayUE, remoteUE} } OPTIONAL, discTxResourceReq-v13x0 SEQUENCE { carrierFreqDiscTx-r13 INTEGER (1..maxFreq), discTxResourceReqAddFreq-r13 SL-DiscTxResourceReqPerFreqList-r13 OPTIONAL ue-Type-r13 ENUMERATED {relayUE, remoteUE} } OPTIONAL, discTxResourceReqPS-r13 SL-DiscTxResourceReq-r13 OPTIONAL, discRxGapReq-r13 SL-GapRequest-r13 OPTIONAL, discTxGapReq-r13 SL-GapRequest-r13 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } Huawei: need further discussion on the necessity. Samsung: We are not aware of any RAN2 agreement on this (i.e. agreement was only for communication) Ericsson: We agree this can be beneficial. Intel: We agree with Samsung that there was no agreement in RAN2. Furthermore we wonder of the necessity for such indication in discovery case. Chair: See meeting notes (R2-160050) Samsung: Reference may be incorrect. No change for now (if change is still desired, paper to be brought to RAN2#93) |  |  | Closed (ProSe CR) (ASN.1) |
| L.053 | 6.2.2 SidelinkUEInformation-v13x0 | carrierFreqDiscTx inside the discTxResourceReq-v13x0 is not needed as carrierFreq is already included in the discTxResourceReqAddFreq? SidelinkUEInformation-v13x0-IEs ::= SEQUENCE { commTxResourceReq121-r13 SL-CommTxResourceReqUC-r13 OPTIONAL, commTxResourceInfoReqRelay-r13 SEQUENCE { commTxResourceReqRelay-r13 SL-CommTxResourceReqUC-r13, ue-Type-r13 ENUMERATED {relayUE, remoteUE} } OPTIONAL, discTxResourceReq-v13x0 SEQUENCE { carrierFreqDiscTx-r13 INTEGER (1..maxFreq), discTxResourceReqAddFreq-r13 SL-DiscTxResourceReqPerFreqList-r13 OPTIONAL } | 3 | Remove the carrierFreqDiscTx inside the discTxResourceReq-v13x0? Huawei: Tend to agree Samsung: field is for first interF carrier for which legacy fields are used to indicate resource configuration details (according to usual extension approach). May be useful to provide some clarification can be provided in field description Ericsson: See also Z.049. Qualcomm: We don’t see any issue in current ASN.1. It should be clear that carrierFreqDiscTx-r13 is just a number to indicate how many frequencies UE is going to perform discovery. Intel: We tend to agree that carrierFreqDiscTx can be removed. Chair: See meeting notes (R2-160050) Samsung: Covered by changes related to R2-160014 |  |  | Closed (ProSe CR) (ASN.1) |
| S.115 | 6.2.2 SidelinkUEInformation-v13x0 | No IE seems needed for SL-SysInfoReportList | 2 | If kept anyhow, we should to reflect in name that IE concerns discovery Chair: => Rename to SL-DiscSysInfoReportList => Can be closed when included in ProSe CR. Samsung: Renamed (to SL-DiscSysInfoReportedFreqList, while ~ToReportFreqList is used for command) |  |  | Closed (ProSe CR) (ASN.1) |
| E.160 | 6.2.2 SidelinkUEInformation | Field name can be improved. The suffix “121” is not very good. It seems “UC” is used for “unicast” in other places which is probably better. commTxResourceReq121-r13 | 1 | Change the field name to commTxResourceReq121UC-r13 Huawei: Fine. Samsung: In procedural specification the term ‘one to one’ is used a lot (to avoid confusion with regular unicast) Ericsson: Also in L.018 Coordinator: Corrected in general CR. Also refences to this field in procedures are changed Coordinator: To be included in Prose CR and removed from general CR. Samsung: Used general CR as baseline, so included in ProSe CR |  |  | Closed (ProSe CR) |
| E.162 | 6.2.2 SidelinkUEInformation | Incorrect terminology in field description commTxResourceReq121 Indicates the frequency on which the UE is interested to transmit non-relay related sidelink one to one communication as well as the sidelink communication transmission destination(s) for which the UE requests E-UTRAN to assign dedicated resources. | 1 | Change field description to commTxResourceReq121 Indicates the frequency on which the UE is interested to transmit non-relay related one to one sidelink one to one communication as well as the sidelink communication transmission destination(s) for which the UE requests E-UTRAN to assign dedicated resources. Huawei: Fine. Coordinator: Corrected in general CR. Coordinator: To be included in Prose CR and removed from general CR. Samsung: See E.160 (not repeated anymore in following cases) |  |  | Closed (ProSe CR) |
| E.235 | 6.2.2 SidelinkUEInformation | SystemInformationBlockType19 should be in Italics in carrierFreqDiscTx field description | 1 | (If not already fixed in the v13.0.0 CR implementation), change the style to Italics, i.e Value 0 corresponds to the first entry in discInterFreqList within SystemInformationBlockType19, value 1 corresponds to the second entry in this list and so on. Huawei: fine. Coordinator: Corrected in general CR. Coordinator: To be included in Prose CR and removed from general CR. |  |  | Closed (ProSe CR) |
| E.164 | 6.2.2 SidelinkUEInformation | Table of field descriptions is not in alphabetical order. | 1 | Sort the table. It seems discSysInfoReportList is out of place. Huawei: fine to sort the table. Coordinator: Corrected in general CR. Coordinator: To be included in Prose CR and removed from general CR. |  |  | Closed (ProSe CR) |
| E.163 | 6.2.2 SidelinkUEInformation | Incorrect terminology in field description commTxResourceReqRelay Indicates the relay related sidelink communication transmission destination(s) for which the relay or remote UE requests E-UTRAN to assign dedicated resources i.e. either contains relay or remote UE’s unicast destination identity. | 2 | Change the text to commTxResourceReqRelay Indicates the relay related one to one sidelink communication transmission destination(s) for which the sidelink relay UE or sidelink remote UE requests E-UTRAN to assign dedicated resources i.e. either contains relay or remote UE’s the unicast destination identity of either the sidelink remote UE or sidelink relay UE. Huawei: need further discuss definition of relay related sidelink communication. Coordinator: Discussion is needed. Changed from class 1 to class 2. Samsung: Some changes included |  |  | Closed (ProSe CR) |
| L.019 | 6.2.2 SidelinkUEInformation-v13x0 | New local IE SL-DestinationInfoListUC is needed? This new IE that is merely referring to the R12 IE? SL-CommTxResourceReqUC-r13 ::= SEQUENCE { Xxx, destinationInfoListUC-r13 SL-DestinationInfoListUC-r13 } SL-DestinationInfoListUC-r13 ::= SL-DestinationInfoList-r12 | 1 | SL-CommTxResourceReqUC-r13 ::= SEQUENCE { Xxx, destinationInfoListUC-r13 SL-DestinationInfoList-r12SL-DestinationInfoListUC-r13 } SL-DestinationInfoListUC-r13 ::= SL-DestinationInfoList-r12 Huawei: fine. Samsung: Fine, but seems like class 1 type issue Ericsson: Agree. Also found in E.161. Qualcomm: No need to change. Current formulation is fine. However, no strong opinion either way. Coordinator: Changed from class 3 to class 1. Corrected in general CR Coordinator: To be included in Prose CR and removed from general CR. |  |  | Closed (ProSe CR) |
| L.020 | 6.2.2 SidelinkUEInformation-v13x0 | Enhancement of field description for: discSysInfoReportList Indicates a list of acquired System Information from SIBs of cells on configured carrier frequencies. | 2 | discSysInfoReportList Indicates a list of acquired System system Information information relevant for sidelink discovery acquired from SIBs of cells on configured inter- carrier frequencies. Huawei: fine. Samsung: It would be good to clarify that ‘list’ relates to frequencies not to parameters, also in name of field. E.g. acquiredDiscSysInfoFreqList Ericsson: Agree. Qualcomm: Here configured word is important and shoul not be deleted. Samsung: Wording modified taking suggestions into account |  |  | Closed (Prose CR) |
| L.018 | 6.2.2 SidelinkUEInformation-v13x0-IEs | Field name for commTxResourceReq121 | 1 | Suggest to rename it to " commTxResourceReqUC" to be better aligned with other field/IE name. The same changes to the field name for the field description part SidelinkUEInformation-v13x0-IEs ::= SEQUENCE { commTxResourceReqUC121-r13 SL-CommTxResourceReqUC-r13 OPTIONAL, Huawei: fine. Samsung: See E.160 Ericsson: Agree. Also in E.161. Qualcomm: No strong opinion. Coordinator: Corrected in general CR. Changed from class 2 to class 1. Coordinator: To be included in Prose CR and removed from general CR. |  |  | Closed (ProSe CR) |
| Z.064 | 6.2.2 PhysicalConfigDedicated | “pucch-ConfigDedicated-r13” and “pusch-ConfigDedicated-r13”which are critical extensions cause the Rel-8 fields unable to be used simultaneously. | 2 | Change to non-critical extension: [[ -- excel line 13-18, 23-24 pucch-ConfigDedicated-r13v13xy PUCCH-ConfigDedicated-v13xy OPTIONAL, -- Need ON -- excel line 25-28 pusch-ConfigDedicated-r13v13xy PUSCH-ConfigDedicated-v13xy OPTIONAL, -- Need ON -- excel line 19-22 cqi-ReportConfig-r13 CQI-ReportConfig-r13 OPTIONAL -- Need ON ]] Ericsson: Agree. Chair: => Proposed change agreed => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  |  | Closed (General CR) (ASN.1) |
| E.232 | 6.2.2 RRC Connection Reconfiguration | Is it necessary to create critical extension SCellToAddMod-r13? Is it possible to extend SCellToAddMod-r10 in a non-critical manner? Non-critical extension would be preferable because then there are no problems with switching back and forth between the orignal field and the critical extension. Are there no other reasons for this extension than SCellIndex-v13xy extension? If not, it could be added as an extension to SCellIndex-r10. | 2 | Consider removing the critial SCellToAddMod-r13 and including SCellIndex-v13xy as a non-critical extension to SCellToaddMod-r10 in the following manner [[ SCellIndex-v13xy CHOICE { release NULL, setup SCellIndex-v13xy } OPTIONAL -- Need ON ]] } Samsung: Seems like class 2 issue i.e. we can just apply existing methods. There is a new field listing the additional SCell entries i.e. ListExt. Using –v13xy for SCellToAddMod would apply in case it concerned a few additional parameters for the concerned entry, but here we provide all. So, existing conventions are used, although it would be better to name the IE SCellToAddModExt-r13 (as done for e.g. PMCH-InfoListExt) Chair: => Agree to rename the field SCellToAddModExt-r13 => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  |  | Closed (General CR) (ASN.1) |
| Z.043 | 6.2.2 RRCConnectionReconfiguration | The value of "maxSCell-r10" is 4. The value range of "SCellIndex-r10" is (1..7). The value of "maxSCell-Plus1" is 5. The value range of "SCellIndex-v13xy" is (8..31). It seems we are not able to use "SCellToAddModListExt-r13" to allocate Scell index (1..7) . Then it is impossible for the network to configures 32 cells at once, as some SCell cannot have a valid SCell index by using "SCellToAddModListExt-r13". | 2 | In "SCellToAddModListExt-r13" and “SCellToReleaseListExt-r13”, change (maybe the “SCellIndex-v13xy can be removed now”): sCellIndex-v13xy SCellIndex-r13SCellIndex-v13xy, Ericsson: ZTE seem correct. It is as follows: SCellToAddModList-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SCellToAddMod-r10 SCellToAddModListExt-r13 ::= SEQUENCE (SIZE (maxSCell-Plus1..maxSCell-r13)) OF SCellToAddMod-r13 Where maxSCell-r10 is 4, maxSCell-Plus1 is 5 and maxSCell-r13 is 31. This means that if 31 SCells should be configured then the legacy SCellToAddModList-r10 type is used for SCells 1,2,3,4 and the new SCellToAddModListExt-r13 is used for SCells 5-31, but in SCellToAddModListExt-r13 there is no way to assign SCell index 5,6 or 7 Samsung:Agree, but should maybe add that ListExt is used to assign cells with index values from 5 CATT: agree with the comment and change. Chair: => The extension list will be defined to be able to indicate the full range of indexes but add a statement that the new extension list can only index above 5. One of the indexes will be removed. => Consequence that index 6 and 7 can be assigned using either legacy or new list. => Can be closed when included in the general CR. Coordinator: A field description is added in general CR to capture the statement. Closed. |  |  | Closed (General CR) (ASN.1) |
| N.036 | 6.2.2 RRCConnectionReconfiguration | The field “antennaInfoDedicatedSCell-v10i0” is missing from SCellToAddMod-r13, so “maxLayersMIMO-r10” cannot be configured for SCells 6-32 | 2 | Add the field with IE type antennaInfoDedicatedSCell-v10i0 to the SCellToAddMod-r13, i.e. add antennaInfoDedicatedSCell-v1300 AntennaInfoDedicated-v10i0 OPTIONAL -- Need ON Samsung: Some general discussion is considered beneficial on when to create –r13 versions if IEs in which all extensions are merged (rather than listing original field an bunch of extensions) Ericsson: Agree. Nokia Networks: Agree it’s good to discuss generally – this was pointed out since it is not included anywhere even though it should be. Coordinator: Not a class 1 issue because it requires general discussion. Changed from class 1 to class 2. Chair: => The field should be added to SCellToAddMod-r13 => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  |  | Closed (General CR) (ASN.1) |
| Z.049 | 6.2.2 | SidelinkUEInformation discTxResourceReq-v13x0 SEQUENCE { carrierFreqDiscTx-r13 INTEGER (1..maxFreq), discTxResourceReqAddFreq-r13 SL-DiscTxResourceReqPerFreqList-r13 OPTIONAL } Is the purpose to use carrierFreqDiscTx-r13 when UE requests resources for one single carrier? And when UE requests resources for multiple carriers, the second field discTxResourceReqAddFreq-r13 is used instead? Note that carrierFreqDiscTx-r13 is mandatory. It seems confusing by having it when UE requests resources for multiple carriers.. | 2 | The carrierFreqDiscTx-r13 could be set to optional. Huawei: Can further discuss the structure. Samsung: Indeed, the intention is that for single frequency legacy signalling is used together with carrierFreqDiscTx-r13. The carrierFreq is however only included if different from the primary, so it should be optional Ericsson: There is something strange here. The fields discTxResourceReqAddFreq and carrierFreqDiscTx are not referenced anywhere else in the spec. It must be clarified how these are set. Looking at other issues it seems it is not very clear how frequencies are signalled for discovery operation. We think this topic can benefit from a good discussion and nice contributions. This seems to be a duplicate of Z.050. Qualcomm: We don’t see any issue in current ASN.1. It should be clear that carrierFreqDiscTx-r13 is just a number to indicate how many frequencies UE is going to perform discovery. Chair: => Consequence of discussion is that carrierFreqDiscTx-r13 will be otional. => Can be closed when captured in general CR. Coordinator: Corrected in ProSe CR and removed from General CR. |  |  | Closed (General CR) (ASN.1) |
| S.050 | 6.2.2 UEInformationResponse, droppedQCI | Name and field description do not seem to reflect actual intention i.e. field merely indicates that a DRB with QCI value 1 was established when RLF was detected | 2 | Change to drbEstablishedWithQCI-1 and update field description Ericsson: Agree with the proposal Chair: => Proposal agreed. => Can be closed when included in general CR. Coordinator: Corrected in general CR. Coordinator: There is a CR that corrects this field and therefore the CR will clash with general CR. This needs to be resolved during CR implementation. |  |  | Closed (General CR) (ASN.1) |
| S.051 | 6.2.2 UEInformationResponse, inDeviceCoexDetected | Field name suggest this is an IDC idication, while it primarily indicates suspension of logging Field does not indicate different reasons for suspension, so better rephrase | 2 | Change field name to logMeasSuspendedIDC Change to ‘Indicates that measurement logging was suspended due to IDC problems’. Ericsson: Agree with proposal. The intention was just a marker to indicate reson for jump in time stamp etc. Chair: => Proposal agreed. => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  |  | Closed (General CR) (ASN.1) |
| N.203 | 6.2.2 RRCConnectionRelease | It is worth checking if cellReselectionSubPriority has correct presence assigned and whether it is aligned with the presence allocated in SIB3 and SIB5. | 2 | Define new presence, if needed. Samsung: Unrelated to SIB, but not sure if we now agreed to use need OP instead of ON for ‘one shot’ fields Coordinator: Not a class 1 issue because it requires some checking. Changed from class 1 to class 2. Coordinator: Normally Need ON is used for ‘one-shot’ fields. Can be closed. |  |  | Closed (General CR) |
| N.040 | 6.2.2 SystemInformationBlockType1 | What is the UE supposed to do if it has first received hyperSFN but then eNB stops broadcasting any value? Shall it continue to use the previously received value, or something else? | 2 | Could make field Need OP and indicate actions upon absence, or Need OR. Samsung: Need OR, as commonly used for broadcast, seems fine Nokia Networks: Fine to use OR. Coordinator: Corrected in general CR. |  |  | Closed (General CR) |
| N.041 | 6.2.2 SystemInformationBlockType1 | What is the UE supposed to do if it has first received eDRXAllowed but then eNB stops broadcasting any value? Shall it continue to use the previously received value, or something else? | 2 | Could make field Need OP and indicate actions upon absence, or Need OR. Samsung: Need OR, as commonly used for broadcast, seems fine Nokia Networks: Fine to use OR. Coordinator: Corrected in general CR. |  |  | Closed (General CR) |
| E.239 | 6.2.2 SystemInformationBlockType1 message, | It is more important for the network to know what the UE is doing rather than to know what the UE is not doing. Therefore it would be desirable to reformulate the “shall not” requirement the other way around in eDRXAllowed field description. | 2 | In eDRXAllowed field descriptionm, replace the “shall not” requirement with a requirement that indicates what the UE is required to do The UE shall not operate in stop idle mode extended DRX ifunless eDRXAllowed is set to FALSETRUE. Samsung: Field can only have value false. Anyhow, entire sentence seems not really required Coordinator: The sentence is removed. Similar kind of sentences have not been used before e.g. for Category0Allowed. Closed. Coordinator: Removed from general CR. Handled in a separate CR. |  |  | Closed (General CR) |
| Q.011 | 6.2.2 UECapabilityEnquiry - requestReducedFormat-r13 | The procedural text allows the network to request skipFallbackCombinations, but not indicate requestReducedFormat. This should not be allowed because the skipping of fallback CA combinations can only be done with the release-13 reduced format. | 2 | (There are multiple options) Add the following text in the field description of requestReducedFormat. “The E-UTRAN includes this field if skipFallbackCombinations is included in the message” BTW, the UE capability supporting only skipping of fallback CA combinations, but not supporting the release-13 reduced format seems invalid. Nokia Networks: We agree UE supporting Rel-13 additions should also always support the reduced format. Coordinator: Corrected in general CR (with a remark that all invalid but possible configurations are not used in real implementations). |  |  | Closed (General CR) |
| S.041 | 6.2.2 Paging | systemInfoModification-eDRX field description: no need to mention applicability in 1st sentence (alsready covered by 2nd) | 1 | Remove Coordinator: Corrected in general CR. |  |  | Closed (General CR) |
| N.034 | 6.2.2 Paging | Field description on redistributionIndication does not cite the specification, just reference number. | 1 | Modify as “…as specified in TS36.304 [4] section 5.2.4.z” Samsung: Section should be indicated between brackets Coordinator: Corrected in general CR. |  |  | Closed (General CR) |
| E.233 | 6.2.2 RRC Connection Reconfiguration | maxSCell-Plus1 is a REL-13 constant and therefore it should have –r13 suffix | 1 | Add -r13 suffix to maxSCell-Plus1 in RRC Connection Reconfiguration message, multiplicity and type constraints definitions, EUTRA-InterNodeDefinitions imports and SCG-ConfigInfo Intel: OK. Coordinator: Corrected in general CR. |  |  | Closed (General CR) |
| E.234 | 6.2.2 RRC Connection Reconfiguration, sCellIndeex field description | It reads that “for PSCellToAddMod, if sCellIndex-r13 is present, sCellIndex-r12 shall be ignored” but shouldn’t this reference to pSCellToAddMod field instead of PSCellToAddMod IE like other fields in this message? | 1 | Change PSCellToAddMod to pSCellToAddMod field Intel: OK. Coordinator: Corrected in general CR. |  |  | Closed (General CR) |
| N.035 | 6.2.2 RRCConnectionReconfiguration | Field description of sCellIndex should indicate that it is UE who shall ignore the index | 1 | Modify as “For PSCellToAddMod, if sCellIndex-r13 is present, UE shall ignore the sCellIndex-r12.” Intel: OK. Ericsson: Agree. Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| Z.045 | 6.2.2 SCPTMConfiguration | To align with the naming convention of IE by adding “-” between two letters of upper case. | 1 | scptm-NeighbourCellList-r13 SCPTM-NeighbourCellList-r13 OPTIONAL, Huawei: Should we change scptmNeighbourCellList-r13 or only SCPTM-NeighbourCellList-r13? Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.025 | 6.2.2 SCPTMConfiguration message | scptmNeighbourCellList-r13 SCPTMNeighbourCellList-r13 OPTIONAL, scptm-NeighbourCellList-r13 SCPTM-NeighbourCellList-r13 OPTIONAL, | 1 | Not essentially incorrect, but improves reading Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| N.039 | 6.2.2 SystemInformationBlockType1 | Missing suffix –r13 for hyperSFN and eDRXAllowed | 1 | Add missing suffixes. Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| I.025 | 6.2.2 SystemInformationBlockType1 | In SystemInformationBlockType1-v13xy-IEs the suffix “-r13” is missing for the new fields hyperSFN and eDRXAllowed. The field descriptions of eDRXAllowed and hyperSFN can be further improved. | 1 | Add the missing suffix “-r13” for the new fields hyperSFN and eDRXAllowed. Coordinator: Corrected in general CR SystemInformationBlockType1-v13xy-IEs ::= SEQUENCE { hyperSFN-r13 BIT STRING (SIZE (10)) OPTIONAL, eDRXAllowed-r13 ENUMERATED {true} OPTIONAL, cellAccessRelatedInfo-v13xy SEQUENCE { Improve field descriptions of eDRXAllowed abd hyperSFN as follows: eDRXAllowed Indicates if idle mode extended DRX is allowed in the cell. The UE shall not operate in idle mode extended DRX unless eDRXAllowed is set to TRUE. Indicates if extended DRX operation in RRC_IDLE is supported in the cell. hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. Indicates the value of H-SFN where a H-SFN value corresponds to an SFN cycle of 1024 radio frames and which is incremented at the SFN boundaries. |  |  | Closed (General CR) |
| E.236 | 6.2.2 SystemInformationBlockType1 | Optional hyperSFN field does not have a Need code. | 1 | Add Need code OR to hyperSFN field Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.184 | 6.2.2 SystemInformationBlockType1 | eDRXAllowed does not follow the naming convention | 1 | Change to eDRX-Allowed Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.237 | 6.2.2 SystemInformationBlockType1 | Optional eDRXAllowed field does not have a Need code | 1 | Add Need code OR to eDRXAllowed field field Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.241 | 6.2.2 UECapabilityEnquiry message | Optional field requestReducedFormat-r13, is missing Need code. This field should have a specified Need code because this is a downlink message. | 1 | If this field is not stored in the UE, it is probably enough to specify that there are no actions upon the absence. If so, add Need code ON to the requestReducedFormat-r13 field. Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.242 | 6.2.2 UECapabilityEnquiry message | Optional field skipFallbackCombinations-r13 is missing Need code. This field should have a specified Need code because this is a downlink message. | 1 | If this field is not stored in the UE, it is probably enough to specify that there are no actions upon the absence. If so, add Need code ON to the skipFallbackCombinations-r13field. Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.243 | 6.2.2 UECapabilityEnquiry message | Optional field requestedMaxCCsDL-r13 is missing Need code. This field should have a specified Need code because this is a downlink message. | 1 | If this field is not stored in the UE, it is probably enough to specify that there are no actions upon the absence. If so, add Need code ON to the requestedMaxCCsDL-r13 field.. Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.244 | 6.2.2 UECapabilityEnquiry message | Optional field requestedMaxCCsUL-r13 is missing Need code. This field should have a specified Need code because this is a downlink message. | 1 | If this field is not stored in the UE, it is probably enough to specify that there are no actions upon the absence. If so, add Need code ON to the requestedMaxCCsUL-r13 field. Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| E.188 | 6.2.2 UEInformationResponse | Inclear sentence: inDeviceCoexDetected Indicates the reason for measurement logging suspension due to IDC problem detection. | 1 | Clarify as: inDeviceCoexDetected Indicates that the reason for measurement logging suspension isdue to IDC problem detection. Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| Z.068 | 6.2.2 MeasResults | The suffix of rsrpResultNCell-r10 should be rel-13 | 1 | Change: rsrpResultNCell-r10r13 RSRP-Range, Coordinator: Corrected in general CR |  |  | Closed (General CR) |
| Z.066 | 6.2.2 PUCCH-Config | Some suffixes of fields are missing. | 1 | Change: PUCCH-ConfigDedicated-v13xy ::= SEQUENCE { -- Line 30 in excel spatialBundlingPUCCH-r13 BOOLEAN, -- Line 31 in excel spatialBundlingPUSCH-r13 BOOLEAN, -- Line 32 in excel harq-TimingTDD BOOLEAN, -- Line 13 in excel codebooksizeDetermination-r13 INTEGER(0..1), -- Line 25 in excel maximumPayloadCoderate-r13 INTEGER (0..7), pucch-Format-r13 CHOICE { format4-r13 SEQUENCE { -- Line 14 in excel format4-resourceConfiguration-r13 SEQUENCE (SIZE (4)) OF Format4-resource-r13, -- Line 23 in excel format4-MultiCSI-resourceConfiguration-r13 SEQUENCE (SIZE (1..2)) OF Format4-resource-r13 OPTIONAL -- Need OR }, format5-r13 SEQUENCE { -- Line 17 in excel format5-resourceConfiguration-r13 SEQUENCE (SIZE (4)) OF Format5-resource-r13, -- Line 24 in excel format5-MultiCSI-resourceConfiguration-r13 SEQUENCE (SIZE (1..2)) OF Format5-resource-r13 OPTIONAL -- Need OR } } OPTIONAL -- Need OR } Ericsson: OK. And also harq-TimingTDD. Coordinator: Corrected in general CR including harq-TimingTDD as proposed by Ericsson. |  |  | Closed (General CR) |
| Z.046 | 6.2.2 | The field “scptmNeighbourCellList” has no impact on the UE behaviors. Note that the field is not even used in the procedural text. Not sure if we need this field at all. | 3 | We could adopt the following options: 1) To remove “scptmNeighbourCellList” from SC-MCCH. 2) Capture some UE behaviors in Stage-2 3) Capture some UE behaviors in Stage-3 (e.g. general description of SC-PTM) Huawei: This field is used for sc-mtch-neighbourCell (for more compact signalling), which is the list of neighbour cells providing this MBMS service and the use of this piece of information is already captured in 36.300 section 15.4. Do we need anything more in stage 3? Chair: => Information is provided to assist the UE implementation to decide when to switch to unicast. There are no mandatory requirements related to the field. A UE implementation may choose to not use this information. => No change is needed. => Can be closed. |  |  | Closed |
| Z.063 | 6.2.2 PDCP-Config Conditional tag x | For Rlc-AM2 and Rlc-AM3, the UE behavior is not clear while the corresponding field is absent. | 2 | In the conditional tag explanation, change “OP” to “ON” for Rlc-AM2 and Rlc-AM3. CATT: the description in condition is clear, e.g. for RLC-AM2, The field is optionally present, need OP, upon setup of a PDCP entity for a radio bearer configured with RLC AM. Otherwise the field is not present. DCM: See our comment to D.025. Chair: => No change needed => Can be closed |  |  | Closed |
| Z.065 | 6.2.2 PhysicalConfigDedicated | The IE “CQI-ReportConfig-r13” is not defined. | 2 | We could also use non-critical extension. Ericsson: We are not sure what the problem is. There is an IE CQI-ReportConfig-v13xy, and that should work. We have not found any CQI-ReportConfig-r13. CATT: there is no CQI-ReportConfig-r13 in the draft CR. Samsung: Issue does not seem to exist Chair: => No change needed |  |  | Closed |
| S.042 | 6.2.2 RRCConnectionRelease, cellReselectionSubPriority | There are 2 levels of optionality i.e. one is redundant | 2 | Could remove optional at lowest level Chair: => No change needed |  |  | Closed |
| H.066 | 6.2.2 SL-DiscTxResourceReq | SL-DiscTxResourceReq-r13 ::= SEQUENCE { carrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL, discTxResourceReq-r13 INTEGER (1..63) } The number of SL discovery resources requested is corresponding to the number of SL discovery messages the UE wants to transmit. In rel-12, the UE reports the number of SL discovery messages to the eNB, and then the eNB can configure SL discovery resources according to the number of discovery messages reported. However, in rel-13, RAN2 did not agree that for each frequency the UE indicates the number of discovery messages for transmission. Alternatively, the UE can report the total number of discovery messages waiting for transmission, which has already been included in Rel-12. SidelinkUEInformation-r12-IEs ::= SEQUENCE { commRxInterestedFreq-r12 ARFCN-ValueEUTRA-r9 OPTIONAL, commTxResourceReq-r12 SL-CommTxResourceReq-r12 OPTIONAL, discRxInterest-r12 ENUMERATED {true} OPTIONAL, discTxResourceReq-r12 INTEGER (1..63) OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SidelinkUEInformation-v13x0-IEs OPTIONAL } | 2 | SL-DiscTxResourceReq-r13 ::= SEQUENCE { carrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL, discTxResourceReq-r13 INTEGER (1..63) } Samsung: We consider this to be new proposal and prefer to stick to the current signalling approach. Ericsson: We agreed that the eNB can configure the UE with discovery resources per frequency. To do that it is of course beneficial if the eNB learns the number of resources requested per frequency. CATT: it’s beneficial to indicate the number of discovery messages. Prefer to keep it. ZTE: We agree this is not officially agreed. However, with this information, eNB could make a precise discovery resource allocation to UE. Hence we can live with it. Qualcomm: We disagree with this issue. Current formulation is correct. Intel: We agree with the comments from other companies that this is a new proposal. We consider it a natural to inherit the Rel-12 approach for Rel-13 where inter-frequency discovery was introduced. Therefore, discTxResourceReq-r13 should be kept. It can be used by serving eB to allocate resources for inter-frequency discovery purposes. Chair: => No change needed => If concerns remain then they may be raised at RAN2#93. |  |  | Closed |
| N.053 | 6.2.2 UEInformationResponse::droppedQCI | The field defines only one value – do we need to add a spare value to be able to extend the field? See also N.004 | 3 | Discuss whether the field needs to be extendible. Ericsson: Do not see any reason to extend Chair: => No change needed |  |  | Closed |
| E.187 | 6.2.2 UECapabilityEnquiry | Not clear to me what "explicitly exclude" means in the field description for skipFallbackCombinations. Indicates that the UE shall explicitly exclude fallback CA band combinations in capability signalling. | 1 | Clarify the meaning of this? Coordinator: No clarifications proposed. Closed without actions. |  |  | Closed |
| Q.012 | 6.2.2 UECapabilityEnquiry, skipFallbackCombinations-r13 | skipFallbackCombinations-r13 is used both in UECapabilityEnquiry (eNB command) and UE-EUTRA-Capability (UE response). | 1 | Consider using different field names. Coordinator: No alternative field names proposed. Closed without actions. |  |  | Closed |
| Z.050 | 6.2.2 | SidelinkUEInformation discTxResourceReq-v13x0 SEQUENCE { carrierFreqDiscTx-r13 INTEGER (1..maxFreq), discTxResourceReqAddFreq-r13 SL-DiscTxResourceReqPerFreqList-r13 OPTIONAL } Is the purpose to use carrierFreqDiscTx-r13 when UE requests resources for one single carrier? And when UE requests resources for multiple carriers, the second field discTxResourceReqAddFreq-r13 is used instead? Note that carrierFreqDiscTx-r13 is mandatory. It seems confusing by having it when UE requests resources for multiple carriers.. | 2 | The carrierFreqDiscTx-r13 could be set to optional. SidelinkUEInformation discTxResourceReq-v13x0 SEQUENCE { carrierFreqDiscTx-r13 INTEGER (1..maxFreq), OPTIONAL, discTxResourceReqAddFreq-r13 SL-DiscTxResourceReqPerFreqList-r13 OPTIONAL } Huawei: Can further discuss together with other proposals, e.g., removing this IE. Samsung: Same as Z.49? Ericsson: This seems to be a duplicate of Z.049. Qualcomm: Same as Z.049. We don’t see issue. Coordinator: Same as Z.049 and hence closed. |  |  | Closed |
| E.303 | 6.2.2 Paging message | The following parameter should be udpated: Paging-v13xx-IEs. | 1 | It should be: Paging-v13xy-IEs Coordinator: This is a temporary name. No need to change anything. |  |  | Closed |
| E.161 | 6.2.2 SidelinkUEInformation | Superfluous type names. It seems SL-DestinationInfoListUC-r13 is not particularly useful as it is defined as a SL-DestinationInfoList-r12. Or is ther a good reason (e.g. future compatibility?) to have this design? SL-CommTxResourceReqUC-r13 ::= SEQUENCE { carrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL, destinationInfoListUC-r13 SL-DestinationInfoListUC-r13 } SL-DestinationInfoListUC-r13 ::= SL-DestinationInfoList-r12 | 1 | Change the text to SL-CommTxResourceReqUC-r13 ::= SEQUENCE { carrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL, destinationInfoListUC-r13 SL-DestinationInfoListUC-r13-r12 } Huawei: Fine. Ericsson: See also L.019. CATT: agree Coordinator: Same as L.019 and hence closed. |  |  | Closed |
| D.046 | 6.2.2 SystemInformationBlockType1 | A suffix, -13 is missing in the eDRX related changes. SystemInformationBlockType1-v13xy-IEs ::= SEQUENCE { hyperSFN BIT STRING (SIZE (10)) OPTIONAL, eDRXAllowed ENUMERATED {true} OPTIONAL, | 1 | Should be corrected as follows. SystemInformationBlockType1-v13xy-IEs ::= SEQUENCE { hyperSFN-r13 BIT STRING (SIZE (10)) OPTIONAL, eDRXAllowed-r13 ENUMERATED {true} OPTIONAL, Coordinator: Same as I.025 and hence closed. |  |  | Closed |
| Z.042 | 6.2.2 UECapabilityEnquiry | The need code is required for the optional field. | 2 | As this is a one-short request from the network, the need code should be “ON”: It is better to clarify the Rel-11 request as well. UECapabilityEnquiry-v1180-IEs ::= SEQUENCE { requestedFrequencyBands-r11 SEQUENCE (SIZE (1..16)) OF FreqBandIndicator-r11 OPTIONAL, -- Need ON nonCriticalExtension UECapabilityEnquiry-v13xy-IEs OPTIONAL } UECapabilityEnquiry-v13xy-IEs ::= SEQUENCE { requestReducedFormat-r13 ENUMERATED {true} OPTIONAL, -- Need ON skipFallbackCombinations-r13 ENUMERATED {true} OPTIONAL, -- Need ON requestedMaxCCsDL-r13 INTEGER (2..32) OPTIONAL, -- Need ON requestedMaxCCsUL-r13 INTEGER (2..32) OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } Nokia Networks: We haven’t used need codes for one-shot messages where there is (by definition) no delta signalling. Fine to discuss this, but we are not sure this is needed. Coordinator: requestedFrequencyBands-r11 is a legacy field and hence this is a maintenance problem. If there is an error with the field, it needs to be corrected with a REL-11 maintenance CR. Closed. |  |  | Closed |
| 6.3.1 System information blocks |  |  |  |  |  |  |  |
| Z.056 | 6.3.1 | It was not specificly agreed in RAN2#90 to enable serving eNB to provide direct discovery resouce information for other carrier. It was just agree that “For PS discovery the allowed frequency is pre-configured in the UE”. Therefore , it is not appropriate to broadcast direct PS related discvory resouce to the UE. SystemInformationBlockType19 information element … SL-CarrierFreqInfo-v13x0::= SEQUENCE { discRxResources-r13 SL-DiscRxPoolList-r12 OPTIONAL, -- Need OR discRxResourcesPS-r13 SL-DiscRxPoolList-r12 OPTIONAL, -- Need OR discTxResources-r13 SL-DiscTxResourcesInterFreq-r13 OPTIONAL, -- Need OR discTxResourcesPS-r13 SL-DiscTxResourcesInterFreq-r13 OPTIONAL, -- Need OR discTxPowerInfo-r13 SL-DiscTxPowerInfoList-r12 OPTIONAL, -- Cond Tx discTxRefCarrierCommon-r13 ENUMERATED {pCell} OPTIONAL, -- Need OR discSyncConfig-r13 SL-SyncConfigListNFreq-r12 OPTIONAL, -- Need OR cellSelectionInfo-r13 CellSelectionInfoNFreq-r13 OPTIONAL -- Need OR } … | 4 |  | Remove : discRxResourcesPS-r13 SL-DiscRxPoolList-r12 discTxResourcesPS-r13 SL-DiscTxResourcesInterFreq-r13 Huawei: not sure of the issue raised. Can further discuss if needed. Ericsson: We have agreed on Relays in IDLE, or? CATT: agree Qualcomm: Disagree. eNB can provide PS discovery info for other frequency as well (same as commercial case). It can be any PS discovery and not necessarily relay discovery. Intel: To our knowledge: -RAN2 agreed to provide inter-frequency discovery resources, but there was no further discussion to distinguish between PS- and non-PS cases. -RAN2 agreed to have separate discovery resources for PS and non-PS cases. We interpret both agreements above in that way that we need the new specific fields for PS. Chair: => To be discussed in RAN2#93. Coordinator: Can be closed. |  | Closed (ProSe CR) (ASN.1) |
| S.054 | 6.3.1 SystemInformationBlockType2, ACDC-BarringForCommon | Seems better to rename to ACDC-BarringConfig and use both as common for all PLMNs and per PLMN | 2 |  | I.e. IE per PLMN become index + acdc-BarringConfig Chair: => Samsung to send a text proposal of the change by email to others can understand the proposal. Coordinator: Can be closed. |  | Closed (General CR) (ASN.1) |
| Z.060 | 6.3.1 | SystemInformationBlockType19--> discHystMaxRelayUE-r13 discHystMinRelayUE-r13, discHystMaxRemoteUE-r13 Based on the discription in 5.10.X.3, the IE discHystMaxRelayUE-r13 and discHystMinRelayUE-r13 are optional. | 2 |  | The discHystMaxRelayUE-r13 and discHystMinRelayUE-r13 shall be set to optional. discHystMaxRelayUE-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12, dBinf}, OPTIONAL, -- Need OR discHystMinRelayUE-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12}, OPTIONAL, -- Need OR Huawei: fine. Samsung: Agree (or actually conditional, i.e. only provided if corresponding thres is signalled) Ericsson: Agree. CATT: agree Qualcomm: Agree Chair: See meeting notes Samsung: Covered by update of R2-160010 i.e. mandatory when corresponding Thresh is provided |  | Closed (ProSe CR) (ASN.1) |
| Z.061 | 6.3.1 | SystemInformationBlockType19--> discHystMaxRemoteUE-r13 Based on the discription in 5.10.Y.3, the IE discHystMaxRemoteUE-r13 is optional. | 2 |  | The discHystMaxRemoteUE-r13 shall be set to optional. discHystMaxRemoteUE-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12}, OPTIONAL, -- Need OR Huawei: fine. Ericsson: Agree. CATT:agree Qualcomm:agree Chair: See meeting notes Samsung: Covered by update of R2-160010 |  | Closed (ProSe CR) (ASN.1) |
| N.071 | 6.3.1 SystemInformationBlockType19:: SL-CarrierFreqInfo-v13x0 | The PS Rx/Tx fields could be grouped together, and the the “normal” SL resources together. | 2/3 |  | E.g the following structure could be used: SL-CarrierFreqInfo-v13x0::= SEQUENCE { sl-Discovery-r13 SL-Tx-Rx-Resources-r13 OPTIONAL, -- Need OR ps-Discovery-r13 SL-Tx-Rx-Resources-r13 OPTIONAL, -- Need OR sl-InterFreqDiscovery SL-DiscTxResourcesInterFreq-r13 OPTIONAL, -- Need OR discSyncConfig-r13 SL-SyncConfigListNFreq-r12 OPTIONAL, -- Need OR cellSelectionInfo-r13 CellSelectionInfoNFreq-r13 OPTIONAL -- Need OR } SL-DiscTxResourcesInterFreq-r13 ::= CHOICE { discoveryMode ENUMERATED {acquireSIFromCarrier, commonTxPool, requestDedicated, noTxOnCarrier}, txPoolCommon-r13 SL-DiscTxPoolList-r12 OPTIONAL, -- Cond CommonPool txPowerInfo-r13 SL-DiscTxPowerInfoList-r12 OPTIONAL, -- Cond CommonPool txRefCarrierCommon-r13 ENUMERATED {pCell} OPTIONAL -- Need OR } SL-Tx-Rx-Resources-r13 ::= SEQUENCE { rxResources-r13 SL-DiscRxPoolList-r12 OPTIONAL, -- Need OR txResources-r13 SL-DiscTxResourcesInterFreq-r13 OPTIONAL, -- Need OR } Ericsson: We are not sure this improves anything. Further more, we consider PS discovery to be one of two subsets of sidelink discovery, hence this naming can be confusing. Qualcomm: No need Chair: See meeting notes Samsung: Awaiting update of R2-160010 Samsung: Covered by update of R2-160010 |  | Closed (ProSe CR) (ASN.1) |
| N.070 | 6.3.1 SystemInformationBlockType19:: discConfigRelay-r13 | Should we group the IEs to avoid repetition in field names? | 2 |  | E.g. the following structure could be used: discConfigRelay-r13 SEQUENCE { relayUE-r13 RelayUE-Discovery-r13 OPTIONAL, -- Need OR remoteUE-r13 RemotedUE-Discovery-r13 OPTIONAL -- Need OR } OPTIONAL, -- Need OR RelayUE-Discovery-r13 ::= SEQUENCE { threshHigh-r13 RSRP-Range OPTIONAL, -- Need OR threshLow-r13 RSRP-Range OPTIONAL, -- Need OR hystMax-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12, dBinf}, hystMin-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12}, } RelayUE-Discovery-r13 ::= SEQUENCE { -- Uu threshold threshHi-r13 RSRP-Range OPTIONAL, -- Need OR hystMax-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12}, -- Uu threshold reselectionInfo-IC-r13 ReselectionInfoRelay-r13 } This could be more generic and avoid lot of repetition in names. Ericsson: We think this could improve readability. Additionally, we propose a CHOICE structure in the discConfigRelay as a UE cannot be configured to be both Remote UE and Relay UE. Qualcomm: No strong opinion. Chair: See meeting notes Samsung: Awaiting update of R2-160010 Samsung: Covered by update of R2-160010 |  | Closed (ProSe CR) (ASN.1) |
| Z.057 | 6.3.1 SystemInformationBlockType19 | Many field descriptions are absent. It is necessary to add the field description for the newly added fields , such as acquireSI-FromCarrier, requestDedicated, noTxOnCarrier | 2 |  | SL-DiscTxResourcesInterFreq-r13 ::= CHOICE { acquireSI-FromCarrier-r13 NULL, discTxPoolCommon-r13 SL-DiscTxPoolList-r12, requestDedicated-r13 NULL, noTxOnCarrier-r13 NULL } Huawei: agree to add some field descriptions. Ericsson: Agree. Qualcomm: Yes, field description can be helpful. Chair: See meeting notes (R2-160050) Samsung: See H.068 |  | Closed (ProSe CR) (ASN.1) |
| L.026 | 6.3.1 SystemInformationBlockType19 | Wonder why q-RxLevMinReselection cannot be mandatory? So, do we need to have for the field description q-RxLevMinReselection, "In case the field is absent, the UE shall apply q-RxLevMin in the field description?" CellSelectionInfoNFreq-r13 ::= SEQUENCE { -- Cell selection information as in SIB1 q-RxLevMin-r13 Q-RxLevMin, q-RxLevMinOffset INTEGER (1..8) OPTIONAL, -- Need OP -- Cell re-selection information as in SIB3 q-Hyst-r13 ENUMERATED { dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10, dB12, dB14, dB16, dB18, dB20, dB22, dB24}, q-RxLevMinReselection-r13 Q-RxLevMin OPTIONAL, -- Need OP t-ReselectionEUTRA-r13 T-Reselection } And the field descripton q-RxLevMinReselection In case the field is absent, the UE shall apply q-RxLevMin. | 3 |  | Change to: CellSelectionInfoNFreq-r13 ::= SEQUENCE { -- Cell selection information as in SIB1 q-RxLevMin-r13 Q-RxLevMin, q-RxLevMinOffset INTEGER (1..8) OPTIONAL, -- Need OP -- Cell re-selection information as in SIB3 q-Hyst-r13 ENUMERATED { dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10, dB12, dB14, dB16, dB18, dB20, dB22, dB24}, q-RxLevMinReselection-r13 Q-RxLevMin OPTIONAL, -- Need OP t-ReselectionEUTRA-r13 T-Reselection } And the field descripton q-RxLevMinReselection In case the field is absent, the UE shall apply q-RxLevMin. Huawei: Fine with existing text. Can be discussed further if needed. Ericsson: We don’t understand the argument behind making this field mandatory. Maybe LG can elaborate? Qualcomm: Agree that to make it consistent with 36.304 CR (section 11.4) this can be made mandatory and field description can be removed. Otherwise if this field is absent then current field description says that it will be same as q-RxLevMin however 36.304 says if any field is present then assume it to be 0. Coordinator: The solution proposal is difficult to understand. If the intention is to make the field mandatory present, both the optionality bit and the need code should be removed. If the intention is to keep the field optionally present, there must be a need code. After this correction there will be an optional (downlink) field without a need code looks more like an issue than a solution (?) Chair: See meeting notes (R2-160050) Samsung: Covered by agreements on R2-160049 |  | Closed (ProSe CR) (ASN.1) |
| C.091 | 6.3.1 SystemInformationBlockType18 | Related to C.025. Two bits are added to indicate whether the eNB supports one to one communication and relay communication respectively in SIB18. So UE interested in UC or relay communication could decide whether to initiate the SidelinkUEInformation transmission after connecting to a eNB which doesn’t support the UC or relay communication. | 3 |  | SystemInformationBlockType18 information element -- ASN1START SystemInformationBlockType18-r12 ::= SEQUENCE { commConfig-r12 SEQUENCE { commRxPool-r12 SL-CommRxPoolList-r12, commTxPoolNormalCommon-r12 SL-CommTxPoolList-r12 OPTIONAL, -- Need OR commTxPoolExceptional-r12 SL-CommTxPoolList-r12 OPTIONAL, -- Need OR commSyncConfig-r12 SL-SyncConfigList-r12 OPTIONAL -- Need OR } OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ commTxPoolNormalCommonExt-r13 SL-CommTxPoolListExt-r13 OPTIONAL, -- Need OR commTxAllowRelayCommon-r13 ENUMERATED {true} OPTIONAL -- Need OR commTxUCAllow-r13 ENUMERATED {true} OPTIONAL, -- Need OR commTXRelayAllow-r13 ENUMERATED {true} OPTIONAL, -- Need OR ]] } -- ASN1STOP SystemInformationBlockType18 field descriptions. commRxPool Indicates the resources by which the UE is allowed to receive sidelink communication while in RRC_IDLE and while in RRC_CONNECTED.. commSyncConfig Indicates the configuration by which the UE is allowed to receive and transmit synchronisation information. E-UTRAN configures commSyncConfig including txParameters when configuring UEs by dedicated signalling to transmit synchronisation information.. commTxPoolExceptional Indicates the resources by which the UE is allowed to transmit sidelink communication in exceptional conditions, as specified in 5.10.4.. commTxPoolNormalCommon Indicates the resources by which the UE is allowed to transmit sidelink communication while in RRC_IDLE or when in RRC_CONNECTED while transmitting sidelink via a frequency other than the primary.. commTxAllowRelayCommon Indicates whether the UE is allowed to transmit relay related data using the transmission pools included in SIB18 i.e. either via commTxPoolNormalCommon or via commTxPoolExceptional.. commTxUCAllow-r13 Indicates whether the UE is allowed to request dedicated transmission pools for non-relay related one to one data.. commTxRelayAllow-r13 Indicates whether the UE is allowed to request dedicated transmission pools for relay related data.. Coordinator: The following field descriptions are added; commTxUCAllow-r13 Indicates whether the UE is allowed to request dedicated transmission pools for non-relay related one to one data. commTxRelayAllow-r13 Indicates whether the UE is allowed to request dedicated transmission pools for relay related data. Samsung: Covered by R2-160013, R2-160020 (partly concluded). All indicatons should now have a flag by which EUTRAN can configure if the UE reports (Relay is controlled by relayConfig in SIB19 | Closed (ProSe CR) (ASN.1) |
| Z.062 | 6.3.1 | discTxRefCarrierCommon Indicates if the PCell (RRC_CONNECTED)/ serving cell (RRC_IDLE) is to be used as reference DL measurements and synchronization, instead of the frequency used to transmit sidelink discovery announcements on, see TS 36.213 [23, 14.3.1]. The description “instead of the frequency used to transmit sidelink discovery announcements on” is not accurate, since the frequency which could be used for reference should be the DL frequency paired with the frequency used to transmit sidelink discovery announcements on. | 2 |  | The sentence “instead of the frequency used to transmit sidelink discovery announcements on” need to be replaced by “instead of the DL frequency paired with the one used to transmit sidelink discovery announcements on” Huawei: fine to improve the field description. Ericsson: Agree. Qualcomm:agree Samsung: Changed based on R2-160049 |  | Closed (ProSe CR) |
| H.068 | 6.3.1 SL-DiscTxResourcesInterFreq | SL-DiscTxResourcesInterFreq-r13 ::= CHOICE { acquireSI-FromCarrier-r13 NULL, discTxPoolCommon-r13 SL-DiscTxPoolList-r12, requestDedicated-r13 NULL, noTxOnCarrier-r13 NULL } Field description is needed for this one. acquireSI-FromCarrier is not used elsewhe | 1 |  | Text proposals acquireSI-FromCarrier Indicates that the UE shall autonomously read SIB19 and other relevant SIBs on the given frequency to acquire resource configuration for sidelink discovery announcement. noTxOnCarrier Indicates that the UE is not allowed to transmit sidelink discovery announcements on the given frequency. requestDedicated Indicates that the UE shall request dedicated sidelink discovery configuration from the serving cell for sidelink discovery announcements on the given carrier. SL-CarrierFreqInfoList-v13x0 If included, the UE shall include the same number of entries, and listed in the same order, as in SL-CarrierFreqInfoList-r12. Coordinator: Corrected in general CR Coordinator: To be included in ProSe CR and removed from general CR. Samsung: Modified to be covered by one IE description (as so far we have no value descriptions (also see update of R2-160010) |  | Closed (ProSe CR) |
| L.022 | 6.3.1 SystemInformationBlockType18 | Need to clarify in the following FD the UE that can be ether of relay or remote UE, not only the relay UE commTxAllowRelayCommon Indicates whether the UE is allowed to transmit relay related data using the transmission pools included in SIB18 i.e. either via commTxPoolNormalCommon or via commTxPoolExceptional. | 2 |  | Change to "Indicate whether the UE (relay UE or remote UE)…" Huawei: not sure the problem. Ericsson: Not strictly necessary, but it can be accepted. Are there more places with the same issue asthis one? Qualcomm: Why is this category 2 issue? Anyways no need to change. Samsung: No change (if still desired, a proposal covering all cases should be provided) |  | Closed (ProSe CR) |
| L.023 | 6.3.1 SystemInformationBlockType18 | The field commTxPoolNormalCommExt can be also used to transmit relay related data. commTxAllowRelayCommon Indicates whether the UE is allowed to transmit relay related data using the transmission pools included in SIB18 i.e. either via commTxPoolNormalCommon or via commTxPoolExceptional. | 2 |  | Change to “i.e. either via commTxPoolNormalCommon/ commTxPoolNormalCommExt or via commTxPoolExceptional. Huawei: Fine to add it or not to add it. Ericsson: Ok, but the addition must be in italics. Qualcomm: Ok Samsung: Changed |  | Closed (ProSe CR) |
| E.165 | 6.3.1 SystemInformationBlockType18 | Terminology in field description is incorrect. commTxAllowRelayCommon Indicates whether the UE is allowed to transmit relay related data using the transmission pools included in SIB18 i.e. either via commTxPoolNormalCommon or via commTxPoolExceptional. | 2 |  | Change text to commTxAllowRelayCommon Indicates whether the UE is allowed to transmit relay related one to one sidelink communicationdata using the transmission pools included in SIB18 i.e. either via commTxPoolNormalCommon or via commTxPoolExceptional. Huawei: need further discuss the definition of relay sidelink communication. Coordinator: Discussions are needed. Changed from class 1 to class 2. Samsung: Not added one to one, see general issue |  | Closed (ProSe CR) |
| L.021 | 6.3.1 SystemInformationBlockType18 | Editorial | 1 |  | Change ‘SIB18’ to ‘SystemInformationBlockType18’ in field description of commTxAllowRelayCommon in SIB18 Huawei: fine. Coordinator: Corrected in general CR Coordinator: To be included in ProSe CR and removed from general CR. |  | Closed (ProSe CR) |
| E.342 | 6.3.1 SystemInformationBlockType18 | Table of field descriptions is not in alphabetical order. | 1 |  | Sort the table. Huawei: fine to sort the table. Coordinator: Corrected in general CR Coordinator: To be included in ProSe CR and removed from general CR. |  | Closed (ProSe CR) |
| N.069 | 6.3.1 SystemInformationBlockType18:: commTxAllowRelayCommon | The field description of commTxAllowRelayCommon could be improved | 1 |  | Proposal: commTxAllowRelayCommon Indicates whether the UE is allowed to transmit relay related data via using the transmission pools commTxPoolNormalCommon or commTxPoolExceptional. Coordinator: Corrected as proposed in L.021. Closed without actions. Coordinator: To be included in ProSe CR |  | Closed (ProSe CR) |
| N.193 | 6.3.1 SystemInformationBlockType18:: commTxPoolNormalCommonExt | commTxPoolNormalCommonExt is not described nor mentioned throughout the document other than in SIB18 definition. What is it used for? | 2 |  | since commConfig-r12 (that includes mandatory rx pool) is optional, maybe commTxPoolNormalCommonExt should be conditionally present only if commConfig-r12 is included otherwise the RX pools may not be configured while Tx (commTxPoolNormalCommonExt) are configured. Ericsson: A field description must be added. We understand that commTxPoolNormalCommonExt are the additional pools we added to Rel-13 when we went from 4 to 8 pools. Maybe commTxPoolNormalCommonExt is only present if commTxPoolNormalCommon is present? Qualcomm: Ok. However this was introduced to increase number of pools from 4 to 8. HV>Statement added that Ext is only configured when original field is configured (as this is assumed in procedural specification). No requirement added that original field must include 4 entries (was not agreed to add constraint during AH) |  | Closed (ProSe CR) |
| L.029 | 6.3.1 SystemInformationBlockType19 | The discovery only concerns non PS related discovery discRxPool Indicates the resources by which the UE is allowed to receive sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED. | 2 |  | Change to: discRxPool Indicates the resources by which the UE is allowed to receive non-public safety related sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED. Huawei: fine. Ericsson: Are we sure about this? In Rel-12 we have PS discovery and that is received in this pool, right? Qualcomm: No need to change. HV> According to current procedures REL-13 UE does not use legacy pool for PS. This seems fine as PS does not seem properly supported in REL-12. If companies still see issues, they are requested to bring a paper to RAN2#93 |  | Closed (ProSe CR) |
| L.028 | 6.3.1 SystemInformationBlockType19 | Better to add reference to 36.304 that has procedural section for the following field. cellSelectionInfo Parameters that may be used by the UE to select/ reselect a cell on the concerned non serving frequency. If absent, the UE acquires the information from the target cell on the concerned frequency. | 2 |  | Change to: cellSelectionInfo Parameters that may be used by the UE to select/ reselect a cell on the concerned non serving frequency. If absent, the UE acquires the information from the target cell on the concerned frequency. See 36.304 [4, 11.4]. Ericsson: Agree. Qualcomm: Agree Coordinator: Not a class 2 issue. Changed from class 2 to 1. Corrected in general CR.” TS” added before spec number. Coordinator: To be included in ProSe CR and removed from general CR. |  | Closed (ProSe CR) |
| Z.058 | 6.3.1 SystemInformationBlockType19 | There is an syntax error in the field description of discRxPoolPS. It is suggested to add the “to”. | 1 |  | The original text is as follows: discRxPoolPS Indicates the resources by which the UE is allowed receive relay and other public safety related sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED. It is suggested to change as follows: discRxPoolPS Indicates the resources by which the UE is allowed to receive relay and other public safety related sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED. Huawei: fine. Coordinator: Corrected in general CR. Coordinator: To be included in ProSe CR and removed from general CR. |  | Closed (ProSe CR) |
| L.030 | 6.3.1 SystemInformationBlockType19 | Clarify that it is ‘sidelink’ communication. discThreshHiRemoteUE Indicates when a remote UE that is in network coverage may use the broadcast PS related sidelink discovery Tx resource pool, if broadcast, or request Tx resources by dedicated signalling otherwise. The parameter equally applies for Tx resources for relay related communication | 1 |  | Change to : discThreshHiRemoteUE Indicates when a remote UE that is in network coverage may use the broadcast PS related sidelink discovery Tx resource pool, if broadcast, or request Tx resources by dedicated signalling otherwise. The parameter equally applies for Tx resources for relay related sidelink communication Huawei: fine. Coordinator: Corrected as proposed in E.167. Closed without actions. Coordinator: To be included in ProSe CR |  | Closed (ProSe CR) |
| E.166 | 6.3.1 SystemInformationBlockType19 | Terminology in field description is superfluous. discRxPoolPS Indicates the resources by which the UE is allowed receive relay and other public safety related sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED. | 1 |  | Change text to discRxPoolPS Indicates the resources by which the UE is allowed receive relay and other public safety related sidelink discovery announcements while in RRC_IDLE and while in RRC_CONNECTED. Huawei: fine. Coordinator: Corrected in general CR. Coordinator: To be included in ProSe CR and removed from general CR. |  | Closed (ProSe CR) |
| E.167 | 6.3.1 SystemInformationBlockType19 | Terminology in field description is incorrect and superfluous. According to the definition in 5.10.6 related discovery is a subset of PS-related discovery. discThreshHiRemoteUE Indicates when a remote UE that is in network coverage may use the broadcast PS related sidelink discovery Tx resource pool, if broadcast, or request Tx resources by dedicated signalling otherwise. The parameter equally applies for Tx resources for relay related communication. | 1 |  | Change text to discThreshHiRemoteUE Indicates when a sidelink remote UE that is in network coverage may use the broadcast PS related sidelink discovery Tx resource pool, if broadcast, or request Tx resources by dedicated signalling otherwise. The parameter equally applies for Tx resources for relay related communication. Coordinator: Corrected in general CR. Coordinator: To be included in ProSe CR and removed from general CR. Samsung: Kept the last sentence as it seems good to highlight that the field is used in the same way for communication |  | Closed (ProSe CR) |
| E.168 | 6.3.1 SystemInformationBlockType19 | Terminology is incorrect. And both field names can be stated in the first line. discThreshXxRelayUE Indicates when a relay UE that is in network coverage may use the broadcast PS related sidelink discovery Tx resource pool, if broadcast, or request Tx resources by dedicated signalling otherwise. | 1 |  | Change text to discThreshHiRelayUE, discThreshLoRelayUE Indicates when a sidelink relay UE that is in network coverage may use the broadcast PS related sidelink discovery Tx resource pool, if broadcast, or request Tx resources by dedicated signalling otherwise. Coordinator: Corrected in general CR. Coordinator: To be included in ProSe CR and removed from general CR. Samsung: There was a mixup in draft CR but this was corrected |  | Closed (ProSe CR) |
| E.169 | 6.3.1 SystemInformationBlockType19 | Terminology is incorrect and field name is incorrect. discTxPoolPS-Common Indicates the resources by which the UE is allowed to transmit relay and other public safety related sidelink discovery announcements while in RRC_IDLE. | 1 |  | Change text to discTxPoolPS-Common Indicates the resources by which the UE is allowed to transmit relay and other public safety related sidelink discovery announcements while in RRC_IDLE. Coordinator: Corrected in general CR. Coordinator: To be included in ProSe CR and removed from general CR. |  | Closed (ProSe CR) |
| E.170 | 6.3.1 SystemInformationBlockType19 | Terminology is incorrect. reselectionInfoRemoteUE Includes the parameters used by the UE when selecting/ reselecting a sidelink relay. | 1 |  | Change text to reselectionInfoRemoteUE Includes the parameters used by the UE when selecting/ reselecting a sidelink relay UE. Coordinator: Corrected in general CR. Coordinator: To be included in ProSe CR and removed from general CR. |  | Closed (ProSe CR) |
| E.247 | 6.3.1 SystemInformationBlockType19 | discHystMinRelayUE-r13has 6 code-points. The unused 2 code points coul be defined as spare values. | 2 |  | Add 2 spare values to discHystMinRelayUE-r13 field ZTE: Is this actually discHystMaxRemoteUE-r13? Besides, we think this IE is optional. Qualcomm: Ok Coordinator: Same as Z.059 Coordinator: To be included in ProSe CR and removed from general CR. Samsung: No change for now, see Z.059. Whether spares are usable depends on grouping i.e. is (conditionally) mandatory they are not. If optinonal, can debate if it is acceptable if UEs not comprehending spare should really assume value 0 |  | Closed (ProSe CR) |
| N.072 | 6.3.1 SystemInformationBlockType19:: SL-DiscTxResourcesInterFreq-r13 | There is no field description for any of the choices in the IE SL-DiscTxResourcesInterFreq-r13 or for any of the fields using the IE. Also, the IE could simply be enumerated and the corresponding fields could use conditions to indicate when they are included | 2 |  | Add field description to explain how the fields are used. NOTE: Also depends on how N.070 and N.071 are resolved! Ericsson: We agree that field descriptions are beneficial. Qualcomm: Ok. Coordinator: As indicated, N.072 depends on N.070 and N.071. Nokia have ASN.1 proposals there, and will also have a contribution on the topic to the AH to clarifythe proposal Samsung: See H.068 |  | Closed (ProSe CR) |
| N.185 | S6.3.1 ystemInformationBlockType19:: CellSelectionInfoNFreq-r13 | The field is defined here but also used in SL-DiscConfig – could make a named type out of this to make it easier to find? | 1 |  | Create a named type in section 6.3.8 Coordinator: It is not clear what field needs to be corrected. More information isneeded in order to make this correction. Observe that CellSelectionInfoNFreq-r13 is not a field. It is a user-defined structured ASN.1 data type. Closed without actions. A new issue can be created when it is more clear what needs to be corrected. Coordinator: To be included in ProSe CR and removed from general CR. Samsung: New top level IE introduced in 6.3.5 |  | Closed (ProSe CR) |
| N.073 | 6.3.1 SystemInformationBlockType20:: sc-mcch-ModificationPeriod-r13 | 10 values used in total, 6 spares could be possible – do we need to list them explicitly? See also N.004 | 3 |  | Discuss and decide what to do with fields that have spares possible – do we list them explicitly or not? Huawei: We cannot use any spare in broadcast signalling. ZTE: In general we should provide spares explicity, but the number of spares could increase heavily while the bits used for a field increases. Nokia Networks: This is related to general discussion – we will provide a contribution to AH meeting. Chair: See meeting notes Coordinator: Corrected in general CR. Spare values are added. See conclusions of N.004 for general approach for spare value handling Coordinator: Removed from the general CR because this is already corrected in SC-PTM CR. |  | Closed (General CR) (ASN.1) |
| E.213 | 6.3.1 SystemInformationBlockType2 | Field description acdc-BarringConfig | 2 |  | The second sentence is not aligned with procedure text in 5.5.3.2 “select the last BarringPerACDC-Category entry in the BarringPerACDC-CategoryList”), and should be deleted. acdc-BarringConfig Barring configuration for an ACDC category. If the field is absent, access to the cell is considered as not barred for the ACDC category in accordance with subclause 5.3.3.xx. Huawei: try to understand what is a misalignment? In ASN.1 changes, it is related to acdc-BarringConfig, and in procedural text, it is related to BarringPerACDC-Category, and they are not in the same level Intel: Tend to think that the referenced condition in 5.3.3.2 refers to the field descriptiom barringPerACDC-CategoryList. Coordinator: It is not clear whether there is a misalignment or not. Can be closed. If concerns remain, they can be raised during working group meetings. |  | Closed (General CR) |
| E.245 | 6.3.1 SystemInformationBlockType2 | Optional acdc-BarringConfig-r13 field does not have a Need code in BarringPerACDC-Category-r13. | 2 |  | Add Need code OR to acdc-BarringConfig-r13 field in BarringPerACDC-Category-r13 Huawei: ok with this suggestion. But wonder whether there is a need to add Need code OR to acdc-CatValue-r13 or not, because this IE is in the same level with acdc-BarringConfig-r13. Intel: Shouldn’t be Need code OP? Coordinator: There seems to be different opinions. Discussion is needed. Changed from class 1 to class 2. Coordinator: Need code OP added (similar to REL-12 acdc parameters). |  | Closed (General CR) |
| N.061 | 6.3.1 SystemInformationBlockType3 | The field cellReselectionServingFreqInfo-v13xy is Need OP but the action on when the field is not present is not indicated, so once configured, the field can never be released. | 2 |  | Indicate in the field description of cellReselectionServingFreqInfo that if the field is not present, UE shall only consider the legacy cellReselectionPriority value and release any existing received cellReselectionSubPriority value. ZTE: The field of this comment does not exist, but the issue related to this comment is clarified as follows. It is already clear to say “otherwise it is applicable for serving frequency”. No need for further description. redistributionFactorCell If redistributionFactorCell is present, this value is only applicable for the serving cell otherwise it is applicable for serving frequency Nokia Networks: This is not about otherwise-condition, but on how to release the field. Assume network first configures this field for serving cell but then wishes to remove it. The current otherwise condition then states that upon absence, UE assumes it is applicable for serving frequency. How can the network ensure UE stops treating this also for the serving frequency? Discussion is needed how to release the field. Coordinator: There seems to be different opinions. Discussion is needed. Changed from class 1 to class 2. Coordinator: 36.331 does not specify any mechanisms for “release” of broadcasted fields. Moreover, there is no solution proposal. Can be closed. If concerns remain, they can raised during REL-13 maintenance. |  | Closed (General CR) |
| S.114 | 6.3.1 SystemInformationBlockType2, acdc-CatValue | Streamline/ align naming i.e. to acdc-Category | 1 |  | Coordinator: Corrected in general CR |  | Closed (General CR) |
| S.053 | 6.3.1 SystemInformationBlockType2, acdc-HPLMNonly | Name not according to convention | 1 |  | Change to HPLMN-Only (or OnlyForHPLMN) Intel: Slight preference for acdc-OnlyForHPLMN-r13. Coordinator: Corrected in general CR as Intel proposed. |  | Closed (General CR) |
| N.055 | 6.3.1 SystemInformationBlockType3 | Field description of redistributionFactorCell: “this value” should be replaced with “redistributionFactorServing” | 1 |  | “this value” should be replaced with “redistributionFactorServing” to remove misleading description. Coordinator: Corrected in general CR |  | Closed (General CR) |
| N.065 | 6.3.1 SystemInformationBlockType5 | “physCellId” field introduced as a part of RedistributionNeighCell-r13 is not extended with “-r13” suffix. | 1 |  | Extend physCellId with “-r13” suffix. Coordinator: Corrected in general CR |  | Closed (General CR) |
| N.190 | 6.3.1 SystemInformationBlockType19:: discHystMaxRelayUE-r13, discHystMinRelayUE-r13, discHystMaxRemoteUE-r13 | discHystMaxRelayUE-r13 and discHystMinRelayUE-r13, discHystMaxRemoteUE-r13 seem to be mandatory while corresponding threshold are optional. Is this right? Should they be conditionally present if corresponding threshold are present? | 2 |  | Discuss whether to change the fields to have a condition depending on the presence of the other fielkds Ericsson: This can be beneficial to discuss. Maybe Nokia can draft a proposal? CATT: at least these fields should be optional. Nokia Networks: We will provide draft proposal to the AH. Qualcomm: Agree to the intention that hysteresis can also be made optional or made conditionaly present when corresponding threshold is present. Samsung: See Z.060 Chair: => Remove the choice structure between Relay UE and Remote UE in SIB19. Both fields can be included separately. => Add some prefix to sl-Discovery-r13 to make clear that it is no related to public safety. => Check and add clarification if needed on the support of public safety discovery in rel-12. => Condition ThreshHigh to be clarified. => Check impact on procedure caused by the grouping. => Text proposal to v13.0.0 spec to be produced. => Nokia will send text proposal by email for discussion within the ASN.1 review. Samsung: Covered by update of R2-160010 |  | Closed (ProSe CR) (ASN.1), TDoc Nokia ASN.1 AH |
| Z.052 | 6.3.1 SystemInformationBlockType20 | An extra line is required for the extention marker. | 1 |  | Add extra line for the extention marker: lateNonCriticalExtension OCTET STRING OPTIONAL, ... } Coordinator: Corrected during v1300 CR implementation. |  | Closed |
| L.024 | 6.3.1 SystemInformationBlockType19 | [[ discConfig-v13x0 SEQUENCE { discInterFreqList-v13x0 SL-CarrierFreqInfoList-v13x0 OPTIONAL, -- Need OR Is it better to rename this field (discInterFreqList) to "discInterFreqInfoList" as this field can inlcude not only freq list but also resource information? | 2 |  | [[ discConfig-v13x0 SEQUENCE { discInterFreqInfoList-v13x0 SL-CarrierFreqInfoList-v13x0 OPTIONAL, -- Need OR This renaming will affect many sections that currently refer to discInterFreqList only for the text introduced in Rel13. The Rel12 text refering to discInterFreqList does not needs to be affected. Ericsson: We are not sure this is the correct way to go. We think there is a reason that this Rel-13 field has the same name as the corresponding Rel-12 field. Qualcomm: No need Chair: => No change |  | Closed |
| Z.059 | 6.3.1 SystemInformationBlockType19, discHystMaxRelayUE-r13; discHystMinRelayUE-r13; discHystMaxRemoteUE-r13; minHyst-r13; | The number of value room shall be as 2^N, so some “spare” value shall be added at the end of the current values. There is a unneccessory blank space between “dB3” and “dB6” in the IE” discHystMaxRemoteUE-r13” | 2 |  | Add the spare value at the end of value, and delete the unneccesory blank space, as belows, discHystMaxRelayUE-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12, dBinf, spare2, spare1}, discHystMinRelayUE-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12}, -- Uu threshold …. OPTIONAL, -- Need OR discHystMaxRemoteUE-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12, spare3, spare2, spare1}, -- Uu threshold …… discHystMaxRemoteUE-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12, spare3, spare2, spare1}, … minHyst-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12, dBinf, spare2, spare1} OPTIONAL -- Need OR Ericsson: Agree. Same as E.246, E.247, E.248, and E.249. Qualcomm: Ok. Same as E.246 to E.249. Chair: See meeting notes => No spares to be added (although in system information the structure of this information and the error handling would mean that the whole IE would be ignored by legacy UEs receiving a spare). |  | Closed |
| N.063 | 6.3.1 SystemInformationBlockType5:: RedistributionNeighCell-r13 | Should the IE RedistributionNeighCell-r13 be extendible? | 2 |  | Discuss whether this field needs ellipsis or not. ZTE: SIB5 is always extendible. DCM: Using the square bracket extension here results in 3 octet overhead per carrier and so should be avoided. Chair: => No change needed |  | Closed |
| N.058 | 6.3.1 SystemInformationBlockType3 | It is worth checking if cellReselectionSubPriority has correct presence assigned and whether it is aligned with the presence allocated in RRCConnectionRelease and SIB5. | 1 |  | Define new presence, if needed. Coordinator: No solution proposal. Closed without actions. If there is still an issue, it can be corrected with a maintenance CR. |  | Closed |
| N.060 | 6.3.1 SystemInformationBlockType3 | The field cellReselectionSubPriority-r13 and IE (+ related field) CellReselectionServingFreqInfo-v13xy from R2-156957 have been left out in the merging process, and only the field description remains | 1 |  | Re-merge the field to the CR. Coordinator: Correctly implemented in v1300. Closed without actions. |  | Closed |
| N.062 | 6.3.1 SystemInformationBlockType5 | See N.062 – in SIB5, the sub-priority field also uses Need OP without explaining actions upon absence. | 1 |  | Indicate in the field description of cellReselectionSubPriority that if the field is not present, UE shall only consider the legacy cellReselectionPriority value and release any existing received cellReselectionSubPriority value. Coordinator: There is no such thing as “legacy” in a TS (or the other way around ; everything is legacy after freezing). Therefore this text cannot be cut-and-pasted to the specification as is. Closed wihtout actions. If there is still an issue, it can be corrected with a maintenance CR. |  | Closed |
| N.067 | 6.3.1 SystemInformationBlockType5 | It is worth checking if cellReselectionSubPriority has correct presence assigned and whether it is aligned with the presence allocated in SIB3 and RRCConnectionRelease. | 1 |  | Define new presence, if needed. Coordinator: No solution proposal. Closed without actions. If there is still an issue, it can be corrected with a maintenance CR. |  | Closed |
| L.025 | 6.3.1 SystemInformationBlockType19 | discThreshHiRelayUE-r13 RSRP-Range OPTIONAL -- Need OR discThreshLoRelayUE-r13 RSRP-Range OPTIONAL -- Need OR Missing comma at the end of the both field | 2 |  | discThreshHiRelayUE-r13 RSRP-Range OPTIONAL, -- Need OR discThreshLoRelayUE-r13 RSRP-Range OPTIONAL, -- Need OR Ericsson: Agree. Qualcomm: Agree; but why is it marked category 2? Coordinator: Syntax errors are already corrected during v1300 CR implementation. |  | Closed |
| L.027 | 6.3.1 SystemInformationBlockType19 | SL-DiscTxResourcesInterFreq-r13 ::= CHOICE { acquireSI-FromCarrier-r13 NULL, discTxPoolCommon-r13 SL-DiscTxPoolList-r12, requestDedicated-r13 NULL noTxOnCarrier-r13 NULL } Missing comma | 2 |  | SL-DiscTxResourcesInterFreq-r13 ::= CHOICE { acquireSI-FromCarrier-r13 NULL, discTxPoolCommon-r13 SL-DiscTxPoolList-r12, requestDedicated-r13 NULL, noTxOnCarrier-r13 NULL } Huawei: fine. Ericsson: Agree. Qualcomm: Ok Coordinator: Syntax errors are already corrected during v1300 CR implementation. |  | Closed |
| E.246 | 6.3.1 SystemInformationBlockType19 | discHystMaxRelayUE-r13 has 6 code-points. The unused 2 code points coul be defined as spare values. | 2 |  | Add 2 spare values to discHystMaxRelayUE-r13 field Qualcomm: Ok Coordinator: Same as Z.059 |  | Closed |
| E.248 | 6.3.1 SystemInformationBlockType19 | discHystMinRelayUE-r13has 5 code-points. The unused 3 code points coul be defined as spare values. | 2 |  | Add 3 spare values to discHystMinRelayUE-r13 field Qualcomm: Ok Coordinator: Same as Z.059 |  | Closed |
| E.249 | 6.3.1 SystemInformationBlockType19 | minHyst-r13 has 6 code-points. The unused 2 code points coul be defined as spare values. | 2 |  | Add 2 spare values to minHyst-r13 field Qualcomm: Ok Coordinator: Same as Z.059 |  | Closed |
| Z.054 | 6.3.1 SystemInformationBlockType20 | Extra line in the field description should be removed. | 1 |  | Extra line in the field description should be removed for “sc-mcch-ModificationPeriod” and “sc-mcch-Offset”. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| Z.055 | 6.3.1 SystemInformationBlockType19 | Missing “,”. SL-DiscTxResourcesInterFreq-r13 ::= CHOICE { acquireSI-FromCarrier-r13 NULL, discTxPoolCommon-r13 SL-DiscTxPoolList-r12, requestDedicated-r13 NULL noTxOnCarrier-r13 NULL } | 1 |  | Addition of “,”. SL-DiscTxResourcesInterFreq-r13 ::= CHOICE { acquireSI-FromCarrier-r13 NULL, discTxPoolCommon-r13 SL-DiscTxPoolList-r12, requestDedicated-r13 NULL, noTxOnCarrier-r13 NULL } Samsung: Is already present (version was syntactically correct)’ Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.308 | 6.3.1 System information blocks | The field description for cellReselectionSubPriorityis supposed to be above the description for freqBandInfo. | 1 |  | Change the order. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| I.032 | 6.3.1 SystemInformationBlockType2 | In the field description of acdc-BarringConfig there is some text in red font color that should corrected to black font color. | 1 |  | Change red font color to black font color: acdc-BarringConfig Barring configuration for an ACDC category. If the field is absent, access to the cell is considered as not barred for the ACDC category in accordance with subclause 5.3.3.13. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| H.067 | 6.3.1 SystemInformationBlockType20 | “ ...} ” “}” shall be put in the next row; | 1 |  | Huawei: Agree. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.056 | 6.3.1 SystemInformationBlockType3 | Field description of redistrOnPagingOnly: Space missing between TS number and reference ID. | 1 |  | Add missing space. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.057 | 6.3.1 SystemInformationBlockType3 | Field description of redistributionFactorServing: Unnecessary space/indentation before the beginning of description. Space missing between TS number and reference ID. | 1 |  | Remove indentation, add space. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.059 | 6.3.1 SystemInformationBlockType3 | “t-360” field description contains redundant hyphen and “dots” at the end of the description. | 1 |  | Correct the field name to be “t360” and remove those dots. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.066 | 6.3.1 SystemInformationBlockType5 | Field description of redistributionFactorCell: Space missing between TS number and reference ID. | 1 |  | Add missing space. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.068 | 6.3.1 SystemInformationBlockType5 | Probably redundant comma after “OPTIONAL” in cellReselectionSubPriority-r13 CellReselectionSubPriority-r13 | 1 |  | Discard the comma. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| C.043 | 6.3.1 SystemInformationBlockType5 | Editorial correction in the field description of interFreqCarrierFreqListExt : interFreqCarrierFreqListExt List of additional neighbouring inter-frequencies, i.e. extending the size of the inter-frequency carrier list using the general principles specified in 5.1.2. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the E-ARFCN used to indicate this. EUTRAN may include interFreqCarrierFreqListExt even if interFreqCarrierFreqList (i.e without suffix) does not include maxFreq entries. If E-UTRAN includes interFreqCarrierFreqListExt-r12 and/or InterFreqCarrierFreqListExt-v13xy it includes the same number of entries, and listed in the same order, as in interFreqCarrierFreqList Ext-r12 | 1 |  | The space in IE interFreqCarrierFreqListExt-r12 is deleted: interFreqCarrierFreqListExt List of additional neighbouring inter-frequencies, i.e. extending the size of the inter-frequency carrier list using the general principles specified in 5.1.2. E-UTRAN does not configure more than one entry for the same physical frequency regardless of the E-ARFCN used to indicate this. EUTRAN may include interFreqCarrierFreqListExt even if interFreqCarrierFreqList (i.e without suffix) does not include maxFreq entries. If E-UTRAN includes interFreqCarrierFreqListExt-r12 and/or InterFreqCarrierFreqListExt-v13xy it includes the same number of entries, and listed in the same order, as in interFreqCarrierFreqListExt-r12 Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.064 | 6.3.1 SystemInformationBlockType5 | The extension of SystemInformationBlockType5-v8h0-IEs differs from Rel-12 version: Rel-12: SystemInformationBlockType5-v8h0-IEs ::= SEQUENCE { interFreqCarrierFreqList-v8h0 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v8h0 OPTIONAL, -- Need OP nonCriticalExtension SystemInformationBlockType5-v9e0-IEs OPTIONAL } SystemInformationBlockType5-v9e0-IEs ::= SEQUENCE { interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v9e0 OPTIONAL, -- Need OR nonCriticalExtension # SEQUENCE {} OPTIONAL } Rel-13: SystemInformationBlockType5-v8h0-IEs ::= SEQUENCE { interFreqCarrierFreqList-v8h0 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v8h0 OPTIONAL, -- Need OP nonCriticalExtension SystemInformationBlockType5-v10j0-IEs OPTIONAL } SystemInformationBlockType5-v10j0-IEs ::= SEQUENCE { interFreqCarrierFreqList-v10j0 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v10j0 OPTIONAL, -- Need OR nonCriticalExtension SystemInformationBlockType5-v9e0-IEs OPTIONAL } SystemInformationBlockType5-v9e0-IEs ::= SEQUENCE { interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v9e0 OPTIONAL, -- Need OR nonCriticalExtension SEQUENCE {} OPTIONAL | 1/2 |  | Correct the extension so we have legacy-compatible specification. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| 6.3.2 Radio resource control information elements |  |  |  |  |  |  |  |
| N.078 | 6.3.2 aperiodicCSI-Trigger-v13xy and aperiodicCSI-Trigger2-r13 | triggerX (extension of R10) and triggerX-SubframeSetIndicator (extension of R12) can be now configured both but in reality only one of those is configured. It should be noted that R12 signaling allows configuration of both type of triggers. | 2 |  | Consider having aperiodicCSI-Trigger-v13xy and aperiodicCSI-Trigger2-r13 in CHOICE structure. TBC – how R13 versions relate to R10 and R12. Should they replace old or be additional triggers. Chair: => To be discussed in RAN2#93 Coordinator: Can be closed |  | Closed (General CR) (ASN.1) |
| N.080 | 6.3.2 CQI-ReportConfig:: CQI-ReportAperiodic-v13xy, cqi-ReportModeAperiodic-r13, aperiodicCSI-Trigger2-r13 | Why are these field OR instead of ON with setup-release? | 2 |  | Is there a reason why these fields are not using setup-release structure and Need ON? Chair: => Can be discussed again in RAN2#93 Coordinator: Can be closed |  | Closed (General CR) (ASN.1) |
| N.079 | 6.3.2 CQI-ReportConfig::CQI-ReportConfigSCell-v13x0, PhysicalConfigDedicated, cqi-ReportConfigSCell-v13x0 | The IE cqi-ReportConfigSCell-v13x0 can never be released once configured. The field doesn’t support delta signalling – with release-setup structure that could be supported | 2 |  | Discuss whether and how to release this field – the intent seems delta signalling since upper leve field is Need ON, so release should be possible, e.g. change to use delta signalling with release-setup structure. Ericsson: This can be considered. CATT: agree. Chair: => cqi-ReportConfigSCell-v13x0 is never references in v13.0.0 spec. Needs to be checked whether a reference is needed or whether the IE can be deleted.. To be added to the list as a separate issue. => Revisit when new issue (E.343) identified above has been resolved. Coordinator: cqi-ReportConfigSCell-v13x0 field does not exist but there is cqi-ReportConfig-v13xy field. It is possible to add release/setup structure in CQI-ReportConfig-v13xy IE and hence the field can be released. This solution is captured in general CR but needs to be confirmed. See also E.343. Coordinator: The setup release is removed from CQI-ReportConfig-v13xy because there is already setup/release structure at lower level. No problems to release this field. |  | Closed (General CR) (ASN.1) |
| E.343 | 6.3.2 | CQI-ReportConfigSCell-v13x0 is never references in v13.0.0 spec. | 3 |  | Needs to be checked whether a reference is needed or whether the IE can be deleted. Related to N.079. Coordinator: CQI-ReportConfigSCell-v13x0 IEis removed because it is never referenced. cqi-ReportConfigPCell-v13xy field is removed according to ad hoc meeting agreements. |  | Closed (General CR) (ASN.1) |
| E.073 | 6.3.2 CSI-process | RAN1 LS states R13-Codebook-Subset-Restriction-1; Only applies when CSIReportingType = Class A. R13-Codebook-Subset-Restriction-2; Only applies when CSIReportingType = Class A. R13-Codebook-Subset-Restriction-3; Only applies when CSIReportingType = Class B (K=1) PMI-Config = 1. Current RRC does not comply with this P-C-AndCBSR-r13 ::= SEQUENCE { legacySet SEQUENCE { p-C-r11 INTEGER (-8..15), codebookSubsetRestriction1-r13 BIT STRING } OPTIONAL, -- Cond BeamformedKna codebookSubsetRestriction2-r13 BIT STRING OPTIONAL, -- Cond NonPreCoded codebookSubsetRestriction3-r13 BIT STRING OPTIONAL -- Cond BeamformedK1a } P-C-AndCBSR-PerResourceConfig-r13 ::= SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r13 Both CBSR1 and CBSR2 should apply to Cond NonPrecoded LG proposal in the email discusion has been to edit as follows: P-C-AndCBSR-r13 ::= SEQUENCE { legacySet SEQUENCE { p-C-r11 INTEGER (-8..15), codebookSubsetRestriction-r11 BIT STRING } OPTIONAL, -- Cond BeamformedKna newSet SEQUENCE { codebookSubsetRestriction1-r13 BIT STRING codebookSubsetRestriction2-r13 BIT STRING } OPTIONAL, -- Cond NonPreCoded codebookSubsetRestriction3-r13 BIT STRING OPTIONAL -- Cond BeamformedK1a } Not sure of the structure but in 213 there is CBSR, CBSR1, CBSR2, CBSR3 defined separately. According to 213: For a UE configured in transmission mode 9 or 10, and for a CSI process and UE configured with higher layer parameter CSI-Reporting-Type, and CSI-Reporting-Type is set to ‘CLASS B’, and more than one CSI-RS resource configured, the bitmap parameter codebookSubsetRestriction is configured for each CSI-RS resource of the CSI process and each subframe sets (if subframe sets and are configured by higher layers) by higher layer signaling. The LG edit seems correct. The length of the Rel 13 codebook for class A is N1O1N1O2 For the below table it is cleat that the List should be deleted and there should be these four entries as currently the table is missing CBSR1 alltogether. I did not find any reference why the deleted sentence shold or should not be there for CBSR2. For CBSR3 213 says: For a UE configured in transmission mode 9 or 10, and for a CSI process and UE configured with higher layer parameter CSI-Reporting-Type, and CSI-Reporting-Type is set to ‘CLASS B’, and one CSI-RS resource configured, and higher layer parameter PMI-Config is set to ‘1’, the bitmap parameter codebookSubsetRestriction-3 is configured for the CSI process and each subframe sets (if subframe sets and are configured by higher layers) by higher layer signaling. In RAN2 spec there is alternativeCodebookEnabledBeamformed correspond to PMI-Config in RAN1 and CBSR3 applies when PMI-Config=1 which means thw W2 feedback. W2 feedback is enabled when alternativeCodebookEnabledBeamformed is configured. codebookSubsetRestriction Parameter: codebookSubsetRestriction, see TS 36.213 [23] and TS 36.211 [21]. The number of bits in the codebookSubsetRestriction for applicable transmission modes is defined in TS 36.213 [23].. codebookSubsetRestriction1 Parameter: codebookSubsetRestriction1, see TS 36.213 [23] and TS 36.211 [21]. The number of bits in the codebookSubsetRestriction1 for applicable transmission modes is defined in TS 36.213 [23].. codebookSubsetRestriction2List Parameter: codebookSubsetRestriction2, see TS 36.213 [23] and TS 36.211 [21]. The number of bits in the codebookSubsetRestriction2 for applicable transmission modes is defined in TS 36.213 [23].If p-C-AndCBSRList-r13 is included, E-UTRAN includes 2 entries otherwise 1 (i.e. E-UTRAN configures the same number of entries for the original and the CBSR2 field.. codebookSubsetRestriction3 Parameter: codebookSubsetRestriction3, see TS 36.213 [23] and TS 36.211 [21]. The number of bits in the codebookSubsetRestriction3 for applicable transmission modes is defined in TS 36.213 [23]. E-UTRAN configures the field only if neither p-C-AndCBSRListExt csi-RS-ConfigNZPIdListExt is not configured, but nor alternativeCodebookEnabledBeamformed is not configured.. Same comment for BeamformedK1a as for CBSR3 above. Conditional presence; Explanation. BeamformedK1a; The field is optionally present, need OR, if included in CSI-RS-InfoBeamformed while neither csi-RS-ConfigNZPIdListExt is not configured, but nor alternativeCodebookEnabledBeamformed is configured. Otherwise the field is not present. BeamformedKna; The field is optionally present, need OR, if included in CSI-RS-InfoBeamformed while csi-RS-ConfigNZPIdListExt is configured but alternativeCodebookEnabledBeamformed is not. Otherwise the field is not present. NonPreCoded; The field is optionally present, need OR, if included in CSI-RS-InfoNonPrecoded. Otherwise the field is not present. Thus, I would second LG’s comments but not sure of the RRC code structure. | 2 |  | Discuss whether the RRC specification is aligned with RAN1 specs. CATT: related to C.053, C.054, C.055, propose to discuss the structure to reflect the RAN1 agreement clearly. Samsung: We intend to draft a contribution on this Chair: => Remove field CBSR3 from IE P-C-AndCBSR-r13 (not needed) but add description to indicate what is used when RAN1 spec refers to field CBSR3 => Consider how to use the conditions for 'BeamformedKna' and 'NonPreCoded' => p-C-AndCBSRList-r13 reference the existing type instead of repeating the defintion. => Continue to progress the remaining issues via email as part of ASN.1 review. Coordinator: Handled in a separate CR. Can be closed. |  | Closed (General CR) (ASN.1) |
| N.085 | 6.3.2 CSI-Process::cqi-ReportAperiodicProc-r13, CSI-Process::cqi-ReportAperiodicProc2-r13 | Critically extended field (original is Rel-11) but no mention whether UE should ignore Rel-11 version if the newer one is present. Also, proc1 is just need ON whereas proc2 uses release-setup – why is that, now proc1 cannot be released at all? | 3 |  | Should this use NCE instead, or do we just add a field description? And should we unify the release-setup for both fields to allow release of the proc1 field as well? Or use Need OR? Ericsson: This is an interesting question and we think it should be discussed and resolved properly. Chair: => To be discussed in RAN2#93 Coordinator: Can be closed after IE grouping and restructuring. |  | Closed (General CR) (ASN.1) |
| I.039 | 6.3.2 CSI-RS-InfoNonPrecoded | Currently, no CSI-RS configuration information is present in the field CSI-RS-InfoNonPrecoded-r13. Consequently, Transmission Mode 9 (TM9) with antenna ports 12 and 16 cannot be configured for a sering cell with current ASN.1 structure. | 3 |  | Discss how to add the CSI-RS resource configuration for antenna ports 12 and 16 for a serving cell configured with TM9. One possible way is to add the field eMIMO-Info-r13 and make it optional. Ericsson: This should be discussed. CATT: need to check the support it on TM9 further. Intel: Support of eMIMO in TM9 was discussed in RAN1 and there was support from some operators. As result it was agreed that TM9 should be also supported for eMIMO. This was communicated to RAN2 per LS in R2-156884/R1-157694 (received in RAN2#92), see below. “Additionally, RAN1 also agreed that TM9 supports Rel. 13 FD-MIMO; the set of features supported in TM9 is to be concluded in UE capability discussions, and in the meantime RAN1 recommends RAN2 to work to support TM9 in Rel-13 FD-MIMO.“ Chair: => As requested by RAN1 we intent to support TM9 in FD-MIMO. Details of which fields are needed for this to be discussed. => Revisit in RAN2#93. Coordinator: Handled in a separate CR. Can be closed. |  | Closed (General CR) (ASN.1) |
| C.059 | 6.3.2 interferenceMeasRestriction-r13 | For beamformed type, interferenceMeasRestriction-r13 is only configured in CSI-IM-ConfigIdInfo-r13, which is the extended configuraiton and the max size is 7.So for the legacy csi-IM-ConfigId-r11 in CSI-Process-r11, it also need the corresponding interferenceMeasRestriction-r13. | 2 |  | CSI-Process-r11 ::= SEQUENCE { csi-ProcessId-r11 CSI-ProcessId-r11, csi-RS-ConfigNZPId-r11 CSI-RS-ConfigNZPId-r11, csi-IM-ConfigId-r11 CSI-IM-ConfigId-r11, p-C-AndCBSRList-r11 SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r11, cqi-ReportBothProc-r11 CQI-ReportBothProc-r11 OPTIONAL, -- Need OR cqi-ReportPeriodicProcId-r11 INTEGER (0..maxCQI-ProcExt-r11) OPTIONAL, -- Need OR cqi-ReportAperiodicProc-r11 CQI-ReportAperiodicProc-r11 OPTIONAL, -- Need OR ..., [[ alternativeCodebookEnabledFor4TXProc-r12 ENUMERATED {true} OPTIONAL, -- Need ON csi-IM-ConfigIdList-r12 CHOICE { release NULL, setup SEQUENCE (SIZE (1..2)) OF CSI-IM-ConfigId-r12 } OPTIONAL, -- Need ON cqi-ReportAperiodicProc2-r12 CHOICE { release NULL, setup CQI-ReportAperiodicProc-r11 } OPTIONAL -- Need ON ]], [[ cqi-ReportAperiodicProc-r13 CQI-ReportAperiodicProc-r13 OPTIONAL, -- Need ON cqi-ReportAperiodicProc2-r13 CHOICE { release NULL, setup CQI-ReportAperiodicProc-r13 } OPTIONAL, -- Need ON eMIMO-Type-r13 CHOICE { release NULL, setup CHOICE { nonPrecoded-r13 CSI-RS-InfoNonPrecoded-r13, beamformed-r13 CSI-RS-InfoBeamformed-r13 } } interferenceMeasRestriction-r13 ENUMERATED {on} OPTIONAL -- Need OR OPTIONAL -- Need ON ]] } Chair: => More time needed to investigate. => Revisit in RAN2#93 Samsung: We plan a contribution on this to RAN2#93 Coordinator: Handled in a separate CR. Can be closed. |  | Closed (General CR) (ASN.1) |
| N.119 | 6.3.2 PhysicalConfigDedicated:: pucch-ConfigDedicatedExt{1,2,3,4} | The fields can never be released via delta signalling once configured | 2 |  | Do we need to be able to release these fields? Ericsson: Yes, it should be possible to release this field. CATT: according to RAN2 agreements, the change between PUCCH SCell and SCell can only be implemented by release+addition, so it is unnecessary to support delta configuration for PUCCH SCell to SCell change. Samsung: Note also that for PCell signalling of these extensions is the same (i.e. optional, need ON) Chair: See meeting notes Coordinator: Relates to grouping disucssion. See S.112 for ad hoc meeting conclusions. Coordinator: Can be closed after IE grouping and restructuring. |  | Closed (General CR) (ASN.1) |
| N.121 | 6.3.2 PhysicalConfigDedicated:: uplinkPowerControlDedicated-r13, ::uplinkPowerControlDedicatedExt-r13 | Why do we need the Ext field here – wouldn’t it be better to group these together in one IE? Also, the fields cannot be released once configured. | 2 |  | Discuss how to treat Ext-fields in Rel-13 for cases when we need to repeat existing fields from earlier releases. Do we need to be able to release these fields? (At least the Rel-8 version has only mandatory content, but the Rel-10 version contents are Need OR.) Samsung: We think some further discussion would be beneficial regarding this i.e. also considering creating a proper REL-13/ merged version Chair: => See meeting notes. Coordinator: Relates to grouping discussion. See S.112 for ad hoc meeting conclusions. Coordinator: Can be closed after IE grouping and restructuring. |  | Closed (General CR) (ASN.1) |
| N.124 | 6.3.2 PhysicalConfigDedicatedSCell-r10 | pucch-ConfigDedicated-r13 PUCCH-ConfigDedicated OPTIONAL, -- Need ON pucch-ConfigDedicatedExt1-r13 PUCCH-ConfigDedicated-v1020 OPTIONAL, -- Need ON pucch-ConfigDedicatedExt2-r13 PUCCH-ConfigDedicated-v1130 OPTIONAL, -- Need ON pucch-ConfigDedicatedExt3-r13 PUCCH-ConfigDedicated-v1250 OPTIONAL, -- Cond eimta pucch-ConfigDedicatedExt4-r13 schedulingRequestConfig-r13 SchedulingRequestConfig OPTIONAL, -- Need ON schedulingRequestConfigExt-r13 SchedulingRequestConfig-v1020 OPTIONAL, -- Need ON pusch-ConfigDedicated-r13 PUSCH-ConfigDedicated OPTIONAL, -- Cond PUSCH-SCell pusch-ConfigDedicatedExt-r13 PUSCH-ConfigDedicated-v1020 OPTIONAL, -- Cond PUSCH-SCell pusch-ConfigDedicatedExt2-r13 PUSCH-ConfigDedicated-v13xy OPTIONAL, -- Cond PUSCH-SCell uplinkPowerControlDedicated-r13 UplinkPowerControlDedicated OPTIONAL, -- Cond UL-Power-SCell uplinkPowerControlDedicatedExt-r13 UplinkPowerControlDedicated-v1020 OPTIONAL, -- Cond UL-Power-SCell Consider making these conditionally present only for PUCCH SCell as they should only be configured if the cell is PUCCH SCell. Maybe one should also group PUCCH SCell related parameters under one IE? | 3 |  | One alternative could be choice structure with options release/setup and PUCCH SCell parameters (listed in description of this “error”) then listed under SEQUENCE for setup. As PUCCH SCell change was agreed to follow release/setup of SCell then need codes could be ON. See also N.116 and N.131. Ericsson: As indicated, we would be fine with having an "umbrella IE" and this should have condition on PUCCH SCell. CATT: support to group all the related parameters in to one IE, it will make the structure clear and readible DCM: Support the proposal. Chair: => See meeting notes Coordinator: Relates to grouping discussion. See conlusion of S.112. Coordinator: Can be closed after IE grouping and restructuring. |  | Closed (General CR) (ASN.1) |
| N.125 | 6.3.2 PhysicalConfigDedicatedSCell-r10 | Now we have many Ext versions of pucch-configDedicated and PUSCHCOnfigDedicated and uplinkPowerControlDedicated. Should we introduce new IE combining these? | 3 |  | Should we introduce r13 specific versions of IEs to avoid multiple Ext-versions? CATT: agree. DCM: agree. Chair: => See meeting notes Coordinator: Relates to grouping discussion. See the conclusions of S.112. Coordinator: Can be closed after IE grouping and restructuring. |  | Closed (General CR) (ASN.1) |
| S.073 | 6.3.2 RadioResourceConfigDedicated, cond CRSIM | The intention seems to be that the REL-13 version is not configured when the REL-11 version is configured. However, this does not relate to presence in the same message (actually REL-11 field may be present and set to release), so we should clarify this in field description | 2 |  | Clarify that E-UTRAN configures the REL-13 version only if the REL-11 version is not configured (and vice versa). Chair: => More investigation needed to consider general principle and other cases where this situation occurs. => Can be revisisted in RAN2#93 Coordinator: Can be closed after RAN2#93. |  | Closed (General CR) (ASN.1) |
| E.251 | 6.3.2 CSI-Process | P-C-AndCBSR-r13 has a field called “legacySet”. It is a little bit unfortunate name (in a specification) because aft er REL-13 freezing everything is legacy. | 2 |  | The field could be either renamed or removed completely. It is probably better to remove because it does not have any semantics. In that way, both p-C-r11 and codebookSubsetRestriction1-r13 fields should have optionality bits and conditioning to BeamformedKna CATT: same comment as E.073. Chair: See meeting notes Coordinator: See conclusion of E.073. Samsung: Will bring a proposal to RAN2#93 Coordinator: Handled in a separate CR. Can be closed. |  | Closed (General CR) (ASN.1) |
| N.120 | 6.3.2 PhysicalConfigDedicated:: pucch-ConfigDedicatedExt3-r13 | Missing condition definition for “eimta” | 2 |  | Add condition definition – the intent seems to be to add it when eIMTA is configured? Chair: => Remove condition and include the eNB restriction in the field description. => Further investigation needed to determine the conditions. Coordinator: Can be closed after RAN2#93. |  | Closed (General CR) (ASN.1) |
| D.023 | 6.3.2 CQI-ReportConfig | Suffix problem: CQI-ReportAperiodic-v13xy | 2 |  | should be CQI-ReportAperiodic-r13 because it is replacing CQI-ReportAperiodic-r10 and includes the fields in CQI-ReportAperiodic-v1250 ? Ericsson: It doesn’t seem to us as if CQI-ReportAperiodic-v13xy is replacing the fields in –r10 and –v1250. However, it seems to us that N.078 is about something similar, so it might be good to have a discussion about this. Samsung: Some discussion seems beneficial, preferrably by a paper summarising which aperiodic CSI reporting trigger options can work together, and discussing whether non-critical extension approach is possible and if so, how (i.e. by fields of different releases e.g. v1250 and v13xy) Coordinator: It seems that discussion is needed. Changed from class 1 to class 2. Coordinator: Awaiting for conclusion of N.078 Coordinator: Can be closed after RAN2#93. |  | Closed (General CR) (ASN.1) |
| D.029 | 6.3.2 PhysicalConfigDedicated | -r13 Suffix problem schedulingRequestConfig-r13 SchedulingRequestConfig OPTIONAL, -- Need ON .. pusch-ConfigDedicated-r13 PUSCH-ConfigDedicated OPTIONAL, -- Cond PUSCH-SCell ... uplinkPowerControlDedicated-r13 UplinkPowerControlDedicated OPTIONAL, -- Cond UL-Power-SCell | 1 |  | Suffix –r13 is not needed Coordinator: It is not completely clear why the suffix needs to be removed for the field indentifiers. These are REL-13 extension fieds. (They are surely not needed in the type references but there –r13 is not used). This needs to be confirmed. |  | Closed (General CR) |
| D.030 | 6.3.2 PUSCH Config | -r13 suffix problem betaOffsetMC-r13 SEQUENCE { betaOffset-ACK-Index-MC-r13 INTEGER (0..15), betaOffset2-ACK-Index-MC-r13 INTEGER (0..15) } | 2 |  | Change to –v13xy betaOffsetMC-r13v13xy SEQUENCE { betaOffset-ACK-Index-MC-r13v13xy INTEGER (0..15), betaOffset2-ACK-Index-MC-r13 INTEGER (0..15) } Samsung: Some discussion seems beneficial, preferrably by a paper discussing which fields may be signalled together, alike for D.023/ N.078 Coordinator: Needs discussion.Changed from class 1 to class 2. Chair: => More investigation of RAN1 requirements is needed. Coordinator: Can be closed after RAN2#93. |  | Closed (General CR) (ASN.1) |
| Z.071 | 6.3.2 | In the IE “CSI-RS-Info”, 3 spare values are missing for the definition of new IE “CSI-RS-InfoNonPrecoded-r13”. Furthermore, in field description for “codebookOverSamplingRateConfig-Ox, the value “na” should be clarified. ” | 1 |  | CSI-RS-InfoNonPrecoded-r13 ::= SEQUENCE { p-C-AndCBSRList-r13 SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r13 OPTIONAL, -- Need OR, codebookConfigN1-r13 ENUMERATED {an1, an2, an3, an4, an8, spare3, spare2, spare1}, codebookConfigN2-r13 ENUMERATED {an1, an2, an3, an4, an8, spare3, spare2, spare1}, codebookOverSamplingRateConfig-O1-r13 ENUMERATED {na,an4,an8, spare1}, codebookOverSamplingRateConfig-O2-r13 ENUMERATED {na,an4,an8, spare1}, codebookConfig-r13 ENUMERATED {an1, an2, an3, an4}, interferenceMeasRestriction-r13 ENUMERATED {on} OPTIONAL -- Need OR } codebookOverSamplingRateConfig-Ox Indicates the spatial over-sampling rate in dimension x as used for transmission of CSI reference signals. Value an1 corresponds to 1 antenna port, an2 to 2 antenna ports and so on, see TS 36.213 [23]. Huawei: OK Ericsson: Agree. Coordinator: The use of spare values will be disucssed during the review meeting and therefore this is not corrected in general CR. It is not clear what should be corrected in the field desription. If something needs to be clarified, there should be a text proposal. Chair: => Spare values not needed based on general discussion. => Field description to be revised to specify 'na' (detail to be checked with RAN1) and avoid reference to 'na1', 'na2' when these values are not possible. => Can be closed when included in general CR. Coordinator: Can be closed after RAN2#93. |  | Closed (General CR) (ASN.1) |
| C.047 | 6.3.2 CRI-ReportConfig-r13 | or value range of The cri-ConfigIndex-r13, current maximum value is N32, which is the number of RI period. But according to current 36.213 FD-MIMO CR, section 7.2.2 as follow, in case of no RI it is also be the number of CSI period. Since CSI period is smaller than RI period, the maximum number of CRI configIndex should extended, i.e. N128. - In case CRI reporting is configured, - if the number of antenna ports in each configured CSI-RS resource is one, - the reporting interval of the CRI reporting is an integer multiple of period (in subframes) … - otherwise - the reporting interval of the CRI reporting is an integer multiple of period (in subframes). … | 2 |  | CRI-ReportConfig-r13 ::= CHOICE { release NULL, setup SEQUENCE { cri-ConfigIndex-r13 ENUMERATED {n1, n2, n4, n8, n16, n32，n64，n128}, cri-ConfigIndex2-r13 ENUMERATED {n1, n2, n4, n8, n16, n32，n64，n128} OPTIONAL -- Need OR } } Huawei: Wait for RAN1 decision Ericsson: This seems reasonable. That also handles the question on adding spare values. Chair: => Any additional values should be indicated by RAN1. (The code points are available to use in future if RAN1 conclude even after ASN.1 freeze). |  | Closed (General CR) (ASN.1) |
| C.048 | 6.3.2 CrossCarrierSchedulingConfig-r13 | In case of no cross carrier scheduling, if cif-Presence-r13 is true, but SCellIndex is higher than 7, it is unclear which value should be used for CIF value. Currently there is no any description in RAN1 and RAN2. | 2 |  | There are two options: Option 1:Configure cif-InSchedulingCell-r13 in case of own-r13; Option 2: specify the value used for CIF in this case in field description, e.g. 0. We need to choose one of them . Ericsson: Agree that cif-InSchedulingCell-r13 should be indexed with 0. Samsung: We agree with Ericsson Chair: => Option 2 agreed as proposed in R2-160023. => Some offline checking and companies may come back. Coordinator: Can be closed after RAN2#93 discussion. |  | Closed (General CR) (ASN.1) |
| N.113 | 6.3.2 PhysicalConfigDedicated:: soundingRS-UL-ConfigDedicatedUpPTsExt-r13, :: soundingRS-UL-ConfigDedicatedAperiodic-v13x0 | Is the Ext-marking needed at the end – there doesn’t seem to be existing list to which these would be appended (as per the general rule for using XXXExt naming)? | 2 |  | Discuss whether the Ext-suffix can be removed from the name Chair: => Will be handled in general restructuring. => Can be closed when covered in general CR. Coordinator: Ext removed from the identifier name. |  | Closed (General CR) (ASN.1) |
| N.086 | 6.3.2 CSI-Process::, CSI-RS-Config::eMIMO-Type-r13 | IE used in at least two places Create a new named type? | 2 |  | Create a new named type for the structure and use it consistently. Ericsson: This can be beneficial. Chair: => Agreed => Can be closed when included in the general CR. Samsung: Seems best to specify as part of CSI-RS-Config (with slight renaming) Coordinator: Corrected in general CR according to the proposal in I.060 whicih is similar to that of Samsung’s proposal |  | Closed (General CR) (ASN.1) |
| H.073 | 6.3.2 PhysicalConfigDedicated | pucch-ConfigDedicated-r13 PUCCH-ConfigDedicated OPTIONAL, -- Need ON pucch-ConfigDedicatedExt1-r13 PUCCH-ConfigDedicated-v1020 OPTIONAL, -- Need ON pucch-ConfigDedicatedExt2-r13 PUCCH-ConfigDedicated-v1130 OPTIONAL, -- Need ON pucch-ConfigDedicatedExt3-r13 PUCCH-ConfigDedicated-v1250 OPTIONAL, -- Cond eimta pucch-ConfigDedicatedExt4-r13 PUCCH-ConfigDedicated-v13xy OPTIONAL, -- Need ON Should reflet the agreement in RAN2 91 bis with Conditional presence. => Configure PUCCHConfigCommonSCell and PUCCHConfigDedicatedSCell are provided only for a PUCCH SCell. | 2 |  | Add condition to make it clear. Ericsson: Agree, condition PUCCH-SCell seems needed here. Samsung: Will condition become circular i.e. PUCCH SCell is cell configured with these fields. Anyhow, relates to more general issue of how to group functionally related extensions Chair: See meeting notes Coordinator: Relates to grouping discussion. See the conclusions of S.112. => Conditions may be used in case of a condition related to the contents of a given message (rather than related to the configuration as set by previous messages) and when it is not feasible to reflect this by grouping the fields. No change needed. If concerns remain, they can be addressed in a maintenance CR. |  | Closed (General CR) (ASN.1) |
| E.258 | 6.3.2 PhysicalConfigDedicated | The conditioning PUCCH-SCell specifies when the field is optionally present but it does not specify field handling upon the absence. | 2 |  | Add Need code ON to the conditioning, i.e. The field is optionally present, Need ON, for PUCCH SCell. Otherwise it is not present. CATT: agree. DCM: agree Chair: => Agreed => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| N.114 | 6.3.2 PhysicalConfigDedicated:: cqi-ReportConfig-v13xy , PhysicalConfigDedicated::cqi-ReportConfigPCell-v13x0 | There are two different fields with the same type - are both needed? What happens if the UE receives two conflicting configurations? | 2 |  | Discuss if both fields are needed. CATT: propose to delete one of them. And same problem is in the config on SCell DCM: Agree with CATT. Samsung: Agree with CATT Chair: See discussion of N.079. => Agree that one should be removed (cqi-ReportConfigPCell-v13x0) => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| E.191 | 6.3.2 PUCCH-Config | codebooksizeDetermination has type INTEGER(0..1). But a ENUMERATED - OPTIONAL style would be more suitable. | 2 |  | Discuss whether to have ENUMERATED {DAI}. The relevant RAN1 agreement is as follows: Agreements: • eNodeB can configure by RRC signaling an eCA UE to determine HARQ-ACK codebook size according to either (a) or (b) as follows: – Solution (a): • DAI based solution is supported for dynamic HARQ-ACK codebook: – Solution (b): • An eCA UE determines HARQ-ACK codebook size according to the number of configured CCs (i.e. as in Rel-12) – Both solution (a) and solution (b) are mandatory feature as UE capability from RAN1 recommendation point of views for UEs supporting more than 5 CCs DCM: Agree to use ENUMERATED instead of INTEGER (0..1). Given that there are two choices, should it be ENUMERATED {DAI, CC} for instance? Otherwise, it is not sure how the UE assumes that eNB configured the CC based solution (b). Chair: => Agreed => Can be closed when included in the general CR. => Need to check if any change is needed in RAN1 specs. Corrected: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| N.131 | 6.3.2 PUCCH-Config - PUCCH-ConfigDedicated-v13xy | It is not possible to release codebooksizeDetermination-r13, maximumPayloadCoderate-r13 and pucch-NumRepetitionCE-r13. | 2 |  | Make it possible to release these. See also N.116 and N.124. Ericsson: OK DCM: Agree. Coordinator: This is not class 1. It needs some discussion because there are several different possible solutions. Changed from class 1 to class 2. Chair: => Make all the fields Need OR => Can be closed when included in the general CR (first 2) and MTC CR (last one) Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| H.074 | 6.3.2 RadioResourceConfigCommon (?) | pucch-ConfigCommon-r13 PUCCH-ConfigCommon OPTIONAL, -- Cond UL Should reflet the agreement in RAN2 91 bis with Conditional presence. => Configure PUCCHConfigCommonSCell and PUCCHConfigDedicatedSCell are provided only for a PUCCH SCell. | 2 |  | Add condition to make it clear. Chair: See meeting notes Coordinator: Relates to grouping discussion. See the conclusions of S.112. => Conditions may be used in case of a condition related to the contents of a given message (rather than related to the configuration as set by previous messages) and when it is not feasible to reflect this by grouping the fields. No change needed. If concerns remain, they can be addressed in a maintenance CR. |  | Closed (General CR) (ASN.1) |
| N.144 | 6.3.2 SoundingRS-UL-Config | SRS-ConfigAp-r13: there seems to be a lot of confusion inside. This is defined as a new Release-13 SEQUENCE but predominantly contains field denoted by “-r10”. Usually in such cases extensions were used. Moreover, “-r10” suffix seems to be not correctly used in some cases (e.g. transmissionCombAp-r10 is actually a new field with a new range so it should be labeled as “-r13”) | 2 |  | Verify If SRS-ConfigAp should be introduced as a new field/structure or as an extension of the previously defined. If the former then it is worth checking “-rXY” suffixes. Huawei: should be changed to -r13 Samsung: All subfields of SRS-ConfigAp-r13 should have extension –r13 (even if value range is same as in REL-10) Chair: => Field names within SRS-ConfigAp-r13 should be -r13 => Can be closed when included in the general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| I.060 | 6.3.2. CSI-RS-Info | IE CSI-RS-Info is defined but not referenced. Actually, it is used to refer to the IEs CSI-RS-InfoNonPrecoded-r13, CSI-RS-InfoBeamformed-r13 and CSI-IM-ConfigIdInfo-r13. Therefore, it is better to define the IEs CSI-RS-InfoNonPrecoded-r13, CSI-RS-InfoBeamformed-r13 and CSI-IM-ConfigIdInfo-r13 separately. | 2 |  | Remove IE CSI-RS-Info and define the IEs CSI-RS-InfoNonPrecoded-r13, CSI-RS-InfoBeamformed-r13 and CSI-IM-ConfigIdInfo-r13 separately. Chair: => Move the choice (between beamformed and precoded) from eMIMO-Type-r13 into an IE CSI-RS-Info. => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| E.252 | 6.3.2 CSI-Process | codebookSubsetRestriction3 field description uses two forms of negation in the same sentence and hence there is a risk for misconceptions. | 2 |  | Remove the redundant “not, i.e. revise the field desription as E-UTRAN configures the field only if neither p-C-AndCBSRListExt nor alternativeCodebookEnabledBeamformed is not configured. CATT: we think the condition is not correct. Related to C.053 on the condition BeamformedK1a. Chair: See meeting notes Coordinator: See conclusion of E.073. Samsung: Issue can be closed (field will disappear) |  | Closed (General CR) (ASN.1) |
| C.071 | 6.3.2 PDCCH-Config | It should be configured on any serving cell, but currently it only included in PDCCH-ConfigSCell-r13, the configuration on PCell is lack. | 2 |  | PhysicalConfigDedicated ::= SEQUENCE { … [[ csi-RS-Config-v13x0 CSI-RS-Config-v13x0 OPTIONAL -- Need ON pdcch-candidateReductions-r13 PDCCH-CandidateReductions-r13 OPTIONAL -- Need OR ]] } Chair: => Move PDCCH-CandidateReductions-r13 to PhysicalConfigDedicated with a setup/release structure and Need ON. Coordinator: See R2-160025. Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| N.082 | 6.3.2 CQI-ReportConfig:: triggerN-SubframeSetIndicator | References to correct RAN1 specs section and CSI request fields are missing. Also the field descriptions are very ugly like this with a lot of repetition | 2 |  | Correct the references (once they are known) and discuss if these field descriptions can be simplified to avoid repetition. Ericsson: Agree. CATT: support. Only one field description for all trigger3-SubframeSetIndicatorN is simple and sufficient. Coordinator: Corrected in general CR. See conclusion of N.007. |  | Closed (General CR) |
| C.057 | 6.3.2 CSI-RS-ConfigNZP-r11 | resourceConfig-r11 should be ignored if eMIMO-Info-r13 is configured. | 2 |  | Field description resourceConfig Parameter: CSI reference signal configuration, see TS 36.211 [21, table 6.10.5.2-1 and 6.10.5.2-2]. resourceConfig-r11 should be ignored if eMIMO-Info-r13 is configured. Chair: => Agree with proposal with a 'shall' instead of 'should' => Can be closed when included in the general CR. Samsung: Prefer to adopt non-critical extension approach i.e. re-using legacy field and adding just 1..7 additional resource configurations. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| C.060 | 6.3.2 CSI-RS-Info | If csi-RS-ConfigNZPIdListExt-r13 is configured, it has the same size of csi-IM-ConfigIdListExt-r13 and p-C-AndCBSR-PerResourceConfigList-r13. It is proposed to capture in the field description of CSI-RS-InfoBeamformed | 2 |  | Add in the field description. CSI-RS-InfoBeamformed E-UTRAN configures field csi-IM-ConfigIdListExt only if the IE is included in CSI-Process (i.e. when TM10 is configured for the serving cell). If csi-RS-ConfigNZPIdListExt-r13 is configured, it has the same size of csi-IM-ConfigIdListExt-r13 and p-C-AndCBSR-PerResourceConfigList-r13. Samsung: Prefer to slightly modify as E-UTRAN responsibility i.e. E-UTRAN configures Coordinator: Firstly, CSI-RS-InfoBeamformed is not a field. It is an IE. Secondly, CSI-RS-InfoBeamformed IE is removed because of agreed IE restructuring. See conclusion of I.060. Can be closed. |  | Closed (General CR) |
| H.072 | 6.3.2 PDCP-Config | Rlc-AM2: The field is optionally present, need OP, upon setup of a PDCP entity for a radio bearer configured with RLC AM. Otherwise the field is not present. | 2 |  | Should be “The field is optionally present, need OP, upon setup of a PDCP entity for a radio bearer configured with RLC AM, if pdcp-SN-Size-v1300 is absent. Otherwise the field is not present.” Intel: The change might not be correct. The intention in CR R2-157135 is to say that if 15 bit PDCP extension is not configured, then 18 bit PDCP extension can be configured. Ericsson: Agree. DCM: See our comment to D.025 Coordinator: See conclusions of D.025. |  | Closed (General CR) |
| D.028 | 6.3.2 PhysicalConfigDedicated | Cond PUCCH SCell for pucchSCell-r13 Cond PUCCH SCell is defined as “The field is optionally present for PUCCH SCell. Otherwise it is not present.”, however the field pucchSCell-r13 is also needs set in all SCell which is not PUCCH SCell (but sends the PUCCH via PUCCH SCell). | 2 |  | Remove the Condition since the field description is clear enough Ericsson: I think we can keep the condition but the field description could be imroved though as the logic is difficult to understand (nested if's). Field description could be changed as This field indicates whether PUCCH feedback is set on on PCell/PSCell or on the PUCCH SCell. If the PUCCH SCell is used to send PUCCH feedback for this SCell then this field is present,else if the PCell/PSCell is used to send PUCCH feedback of this SCell the field is absent." The condition seems also to be wrong, should it not be "This field is optionally present if a PUCCH SCell is configured. Otherwise it is not present." Coordinator: D.028 proposes to remove the Condition PUCCH SCell for pucchSCell-r13 but there is no such field as pucchSCell-r13 in PhysicalConfigDedicated. There are fields tpc-PDCCH-ConfigPUCCH-SCell-r13 and pucch-Cell-r13 and both of them have condition PUCCH SCell. So, it seems that the intention of this correction is to remove either this, tpc-PDCCH-ConfigPUCCH-SCell-r13 TPC-PDCCH-ConfigSCell OPTIONAL, -- Cond PUCCH-SCell or this pucch-Cell-r13 ENUMERATED {true} OPTIONAL, -- Cond PUCCH_SCell It needs to be clarified which one of the fields should be corrected. It is not completely clear either what field description Ericsson would like to correct but probably it is pucch-Cell because pucch-Cell field description specifies PUCCH feedback handling. So, maybe Ericsson’s comment proposes the following change in the field description, pucch-Cell This field indicates whether PUCCH feedback is set on on PCell/PSCell or on the PUCCH SCell. If the PUCCH SCell is used to send PUCCH feedback for this SCell then this field is present,else if the PCell/PSCell is used to send PUCCH feedback of this SCell the field is absent. if PUCCH feedback of this SCell is not sent on PCell or PSCell, this field is present if SCell is used to send PUCCH feedback of this SCell (or cell). In addition, Ericsson’s comment on the condition seems to require the following change The field is optionally present forif a PUCCH SCell is configured. Otherwise it is not present. Also these comments on the field description and condition should be clarified. Otherwise it is difficult to know what exactly is a subject for correction. DCM: Our intention was 1) to improve the field description of pucch-“Cell”-r13 and 2) to remove the conditional presence as to PUCCH SCell. On 1), Ericsson’ proposal looks good to us with minor improvement as shown above. On 2), We’re fine to keep the condition with the clarification suggested by Ericsson above, i.e., to clarify that the optional presence is if a PUCCH SCell is configured. Furthermore, we think the field name, “pucch-Cell” is somewhat misleading and so wonder if it could be renamed appropriately, e.g., UCI-PUCCH-SCell-r13. Coordinator: Changed from class 1 to class 2. Coordinator: Field description is improved and the condition is changed. Corrected in general CR. Closed. |  | Closed (General CR) |
| E.250 | 6.3.2 CQI-ReportConfig | cri-ReportConfig-r13 field is specified as Optional No (ON) action both in CQI-ReportPeriod.v13xy and in CQI-ReportPeriodicProcExt-r11. It seems that this field cannot be released and a relase/setup contstruct would be useful. | 2 |  | Consider adding release/setup construct to cri-ReportConfig-r13 field. CATT: no need, because current structure of cri-ReportConfig-r13 includes release and setup branch. Samsung: Agree with CATT i.e. no need for change (although one could argue delta signalling is not essential as CRI-ReportConfig is <=7b). Propose to close without change. Coordinator: This correction is not deemed necessary. Closed without actions. |  | Closed (General CR) (ASN.1) |
| D.020 | 6.3.2 CSI-RS-ConfigNZP | Field description of nzp-resourceConfigList is missing. | 2 |  | Coud be added as follows. nzp-resourceConfigList Indicate a list of non-zero power transmission CSI-RS resources using parameter resourceConfig. Ericsson: Agree. CATT: all the new parameters of FD MIMO should be added in the field description. It is a general comments. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| N.101 | 6.3.2 PDCCH-Config | The name of the IE is PDCCH-ConfigSCell | 1 |  | Fix the name of the IE in title. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| C.072 | 6.3.2 PhysicalConfigDedicatedSCell | All the PUCCH SCell related configuration should be in the condition of PUCCH SCell. It is better to understand the relationship to the PUCC SCell or normal SCell. | 2 |  | Following parameter should in condition of PUCCH-SCell. pucch-ConfigDedicated-r13 pucch-ConfigDedicatedExt1-r13 pucch-ConfigDedicatedExt2-r13 pucch-ConfigDedicatedExt3-r13 pucch-ConfigDedicatedExt4-r13 schedulingRequestConfig-r13 schedulingRequestConfigExt-r13 cqi-ReportConfig-r13 Ericsson: OK. CATT: the intention of this comment is to group all the PUCCH SCell related configuration togeter. Samsung: See N.121 Coordinator: See R2-160026. Already covered by general discussion on grouping. Closed. |  | Closed (General CR) |
| E.192 | 6.3.2 PUSCH-Config | Field description missing for betaOffset2-ACK-Index | 2 |  | Update field descriptions as follows: betaOffset-ACK-Index, betaOffset-ACK2-Index, betaOffset-ACK-Index-MC, betaOffset2-ACK-Index-MC Parameter: , see TS 36.213 [23, Table 8.6.3-1]. betaOffset-ACK-Index and betaOffset-ACK2-Index are used for single-codeword and betaOffset-ACK-Index-MC and betaOffset2-ACK-Index-MC are used for multiple-codeword. If betaOffset-ACK2-Index is configured; betaOffset-ACK-Index is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset-ACK2-Index is used. If betaOffset-ACK2-Index-MC is configured; betaOffset-ACK-Index-MC is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset-ACK2-Index-MC is used. One value applies for all serving cells with an uplink and not configured with uplink power control subframe sets. The same value also applies for subframe set 1 of all serving cells with an uplink and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell). Coordinator: It would be useful to indicate what has been updated. It seems that the following correction is proposed betaOffset-ACK-Index, betaOffset-ACK2-Index, betaOffset-ACK-Index-MC, betaOffset2-ACK-Index-MC Parameter: , see TS 36.213 [23, Table 8.6.3-1]. betaOffset-ACK-Index and betaOffset-ACK2-Index are used for single-codeword and betaOffset-ACK-Index-MC and betaOffset2-ACK-Index-MC are used for multiple-codeword. If betaOffset-ACK2-Index is configured; betaOffset-ACK-Index is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset-ACK2-Index is used. If betaOffset-ACK2-Index-MC is configured; betaOffset-ACK-Index-MC is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset-ACK2-Index-MC is used. One value applies for all serving cells with an uplink and not configured with uplink power control subframe sets. The same value also applies for subframe set 1 of all serving cells with The same value also applies for subframe set 1 of all serving cells with an uplink and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell). This needs to be confirmed. Is the intention also to merge these descriptions into one field description? Coordinator: Corrected in general CR. |  | Closed (General CR) |
| E.193 | 6.3.2 PUSCH-Config | Field description missing for betaOffset2-ACK-Index-SubframeSet2 and betaOffset2-ACK-Index-MC-SubframeSet2. | 2 |  | Update field descriptions as follows: betaOffset-ACK-Index-SubframeSet2, betaOffset2-ACK-Index-SubframeSet2, betaOffset-ACK-Index-MC-SubframeSet2, betaOffset2-ACK-Index-MC-SubframeSet2 Parameter: , see TS 36.213 [23, Table 8.6.3-1]. betaOffset-ACK-Index-SubframeSet2 and betaOffset2-ACK-Index-SubframeSet2 are used for single-codeword, betaOffset-ACK-Index-MC-SubframeSet2, betaOffset2-ACK-Index-MC-SubframeSet2 are used for multiple-codeword. If betaOffset2-ACK-Index-SubframeSet2 is configured; betaOffset-ACK-Index-SubframeSet2 is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index-SubframeSet2 is used. If betaOffset2-ACK-Index-MC-SubframeSet2 is configured; betaOffset-ACK-Index-MC-SubframeSet2 is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index-MC-SubframeSet2 is used. One value applies for subframe set 2 of all serving cells with an uplink and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell configured with uplink power control subframe sets). Coordinator: It would be useful to indicate what has been updated. It seems that the following correction is proposed Update field descriptions as follows: betaOffset-ACK-Index-SubframeSet2, betaOffset2-ACK-Index-SubframeSet2, betaOffset-ACK-Index-MC-SubframeSet2, betaOffset2-ACK-Index-MC-SubframeSet2 Parameter: , see TS 36.213 [23, Table 8.6.3-1]. betaOffset-ACK-Index-SubframeSet2 and betaOffset2-ACK-Index-SubframeSet2 are used for single-codeword, betaOffset-ACK-Index-MC-SubframeSet2, betaOffset2-ACK-Index-MC-SubframeSet2 are used for multiple-codeword. If betaOffset2-ACK-Index-SubframeSet2 is configured; betaOffset-ACK-Index-SubframeSet2 is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index-SubframeSet2 is used. If betaOffset2-ACK-Index-MC-SubframeSet2 is configured; betaOffset-ACK-Index-MC-SubframeSet2 is used when up to 22 HARQ-ACK bits are transmitted otherwise betaOffset2-ACK-Index-MC-SubframeSet2 is used. One value applies for subframe set 2 of all serving cells with an uplink and configured with uplink power control subframe sets (the associated functionality is common i.e. not performed independently for each cell configured with uplink power control subframe sets). This needs to be confirmed. Is the intention also to merge these descriptions into one field description? Coordinator: Corrected in general CR. |  | Closed (General CR) |
| C.052 | 6.3.2 codebookConfigN1 | Field description part, codebookConfigN1 should be codebookConfigNx | 1 |  | codebookConfigNx Indicates the number of antenna ports per polarization in dimension x as used for transmission of CSI reference signals. Value an1 corresponds to 1 antenna port, an2 to 2 antenna ports and so on, see TS 36.213 [23]. Huawei: OK Ericsson: Agree. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| C.051 | 6.3.2 codebookConfig-r13 | - The config is not related to antenna ports. - Lack of field description | 1 |  | 1) codebookConfig-r13 ENUMERATED {an1, an2, an3, an4} Coordinator: No comments on the field description below but the ASN.1 standard does not allow enumerated type to have integer values as proposed above. So either there should some characters (e.g. an) before the integer values or the ASN.1 type should be changed from ENUMERATED to INTEGER. - field description: codebookConfig Indicate a sub-set of the codebook entry, see 36.213[23] Huawei: OK Ericsson: Agree with Coordinator. But a field description should also be added. CATT: Propose to change from ENUMERATED to INTEGER, since ‘an’ is inappropriate to add here Coordinator: Corrected in general CR. ASN.1 data type changed from ENUMERATED to INTEGER and “TS” is added before spec number. |  | Closed (General CR) |
| I.035 | 6.3.2 CQI-ReportConfig | In the field description of cri-ConfigIndex2 the reference [23] to TS 36.213 is missing. Furthermore, the reference to parameter CRI Config Index ICRI should be corrected to cri-ConfigIndex ICRI to be aligned with RAN1. | 1 |  | Add the missing reference [23] to TS 36.213 and correct the reference to parameter CRI Config Index ICRI to cri-ConfigIndex ICRI. cri-ConfigIndex2 Parameter: CRI Config Index ICRI, see TS 36.213 []. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets. E-UTRAN configures cri-ConfigIndex2 only if cri-ConfigIndex is configured. Ericsson: Agree. Coordinator: The following correction is proposed ConfigIndex2 Parameter: cri-ConfigIndex ICRICRI Config Index ICRI, see TS 36.213 [23]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets. E-UTRAN configures cri-ConfigIndex2 only if cri-ConfigIndex is configured. Corre cted in general CR. |  | Closed (General CR) |
| H.069 | 6.3.2 CQI-ReportConfig | csi-ProcessToAddModList For a serving frequency E-UTRAN configures one or more CSI-Process only when transmission mode 10 is configured for the serving cell on this carrier frequency. | 1 |  | Transmission mode 9 should also be supported. Ericsson: “CSI-process” is supported for tm9 according to RAN1 specs and for tm9 only one CSI-process can be configured. This should be made clear if the field description is updated. Coordinator: Corrected in general CR as “mode 9 or 10 is …” Hopefully this addresses the concern. |  | Closed (General CR) |
| D.018 | 6.3.2 CQI-ReportConfig | Field description of cri-configIndex (without index) is missing. | 1 |  | Could be added as follows. cri-ConfigIndex Parameter: CRI Config Index ICRI, see TS 36.213 [23, tables X.Y.Z]. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet1 or corresponding to the CSI subframe set 1 indicated by csi-MeasSubframeSets-r12. Ericsson: Agree, but also look at issue I.035. Coordinator: Corrected in general CR as proposed in I.035. |  | Closed (General CR) |
| C.061 | 6.3.2 CQI-ReportConfig | In field description, the reference section is 7.3. | 1 |  | In field description see TS 36.213 [23, 7.3x.y]. Coordinator: Corrected in general CR for simultaneousAckNackAndCQI-Format4-Format5 field. |  | Closed (General CR) |
| D.024 | 6.3.2 cri-ReportConfig | Needs “” for the following “beamformed”: …eMIMO-Type set to beamformed… | 1 |  | …eMIMO-Type set to “beamformed”… Coordinator: Corrected in general CR. |  | Closed (General CR) |
| N.084 | 6.3.2 CrossCarrierSchedulingCOnfig field descriptions | The CIF value indicated in cif-inSchedluingCell | 1 |  | CIF abbrevation is not introduced in the specification, It can be deduced from the reference RAN1 specification 36.212, but maybe worthwhile to add the CIF abbreviation Ericsson: OK. Coordinator: So what is the outcome of the deduction? At least it does not seem to be captured in the list of abbreviations. Closed for now (since there is no solution). Can be added later if needed. |  | Closed (General CR) |
| I.036 | 6.3.2 CSI-Process | There are some isues in ASN.1 and field descriptions: - In ASN.1 of P-C-AndCBSR-r13 the suffix for the field “p-C-r11” should be corrected to “p-C-r13”. - In ASN.1 the text in red font color should be corrected in black font color. - Field description of codebookSubsetRestriction1 is missing. We think that the same definition as for codebookSubsetRestriction applies. - There is no corresponding field codebookSubsetRestriction2List defined in ASN.1. We assume it should refer to codebookSubsetRestriction2. - In the field description of codebookSubsetRestriction2List the reference to “original field” and “CBSR2 field” is ambiguous. Therefore, it should be replaced by codebookSubsetRestriction and codebookSubsetRestriction2. - In the field description of codebookSubsetRestriction3 there is a reference to p-C-AndCBSRListExt but there is no such field defined in ASN.1. We assume it should refer to csi-RS-ConfigNZPIdListExt. | 1 |  | Fix the issues in ASN.1 and field descriptions. P-C-AndCBSR-r13 ::= SEQUENCE { legacySet SEQUENCE { p-C-r1113 INTEGER (-8..15), codebookSubsetRestriction1-r13 BIT STRING } OPTIONAL, -- Cond BeamformedKna codebookSubsetRestriction2-r13 BIT STRING OPTIONAL, -- Cond NonPreCoded codebookSubsetRestriction3-r13 BIT STRING OPTIONAL -- Cond BeamformedK1a } P-C-AndCBSR-PerResourceConfig-r13 ::= SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r13 codebookSubsetRestriction, codebookSubsetRestriction1 Parameter: codebookSubsetRestriction, see TS 36.213 [23] and TS 36.211 [21]. The number of bits in the codebookSubsetRestriction for applicable transmission modes is defined in TS 36.213 [23]. codebookSubsetRestriction2List If p-C-AndCBSRList-r13 is included, E-UTRAN includes 2 entries otherwise 1 (i.e. E-UTRAN configures the same number of entries for the originalcodebookSubsetRestriction and the CBSR2codebookSubsetRestriction2 field. codebookSubsetRestriction3 E-UTRAN configures the field only if neither p-C-AndCBSRListExtcsi-RS-ConfigNZPIdListExt nor alternativeCodebookEnabledBeamformed is not configured. Ericsson: This is same issue as N.087 and Z.070. This proposed solution seems correct, but there seems to be a double negation in the field description for codebookSubsetRestriction3 which should be improved. CATT: related to C.054 and C.055. Cooordinator: Font colour issues are corrected during v1300 CR implementation. The rest is corrected in general CR. |  | Closed (General CR) |
| D.019 | 6.3.2 CSI-RS-ConfigNZP | In the field description of cdmType, the meaning of values, cdm2, cdm4 are not explained. | 1 |  | Could be added as follows. cdmType Parameter: CDMType, see TS 36.211 [21, 6.10.5.2]. EUTRAN configures this field only for CSI processes that include eMIMO-Type set to nonPrecoded. Ericsson: Agree. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| I.037 | 6.3.2 CSI-RS-ConfigNZP | In IE eMIMO-Info-r13 the suffix “-r13“ is missing for field cdmType. | 1 |  | Add missing suffix “-r13“for field cdmType. [[ eMIMO-Info-r13 CHOICE { release NULL, setup SEQUENCE { nzp-resourceConfigList-r13 SEQUENCE (SIZE (2..8)) OF ResourceConfig-r13, cdmType-r13 ENUMERATED {cdm2, cdm4} OPTIONAL -- Need OR } } OPTIONAL -- Need ON ]] Ericsson: Agree Coordinator: Corrected in general CR. |  | Closed (General CR) |
| C.056 | 6.3.2 CSI-RS-ConfigNZP-r11 | For the configuration of eMIMO-Info-r13, not only nonPrecoded but also all the config is applicable for class A (i.e. non precoded). | 2 |  | Field description cdmType eMIMO-Info EUTRAN configures this field only for CSI processes that include eMIMO-Type set to nonPrecoded. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| E.074 | 6.3.2 CSI-RS-Info | Additional comment to BeamformedKna: This part “but alternativeCodebookEnabledBeamformed is not.” could be deleted as according to RAN1 LS, PMI-Config applies only the CLASS B and K=1 and thus cannot be anyway configured with csi-RS-ConfigNZPIdListExt as highlited below. alternativeCodebookEnabledBeamformed If included in CSI-Process (i.e. TM10), the field indicates whether code book in TS 36.213 [23] Table 7.2.4-X to Table 7.2.4-Z is being used for deriving CSI feedback and reporting for a CSI process. EUTRAN configures the field only for a process referring to a single RS configuration using non-zero power transmission (i.e a process for which csi-RS-ConfigNZPIdListExt is not configured). | 1 |  | Update the field description accordingly. Coordinator: Update the BearmformedKna conditional presence as follows, The field is optionally present, need OR, if included in CSI-RS-InfoBeamformed while csi-RS-ConfigNZPIdListExt is configured but alternativeCodebookEnabledBeamformed is not. Otherwise the field is not present Coordinator: Corrected in general CR. |  | Closed (General CR) |
| E.076 | 6.3.2 CSI-RS-Info | Should this fiel be Nx instead of N1? codebookConfigN1 Indicates the number of antenna ports per polarization in dimension x as used for transmission of CSI reference signals. Value an1 corresponds to 1 antenna port, an2 to 2 antenna ports and so on, see TS 36.213 [23]. similar to: codebookOverSamplingRateConfig-Ox Indicates the spatial over-sampling rate in dimension x as used for transmission of CSI reference signals. Value an1 corresponds to 1 antenna port, an2 to 2 antenna ports and so on, see TS 36.213 [23]. | 1 |  | Consider updating the field name. Coordinator: Update the field name as follows, codebookConfigN1Nx Indicates the number of antenna ports per polarization in dimension x as used for transmission of CSI reference signals. Value an1 corresponds to 1 antenna port, an2 to 2 antenna ports and so on, see TS 36.213 [23]. Same as H.071, N.090. Corrected in general CR. |  | Closed (General CR) |
| N.092 | 6.3.2 DMRS-Config | DMRS-Config field descriptions, weird phrase in dmrs-tableAlt: “…indicates whether or not…” | 1 |  | Remove “or not” part from the underlined part. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| E.254 | 6.3.2 MAC-MainConfig | It reads in sCellDeactivationTimer field description that the field does not apply for the PUCCH SCell but the intention is probably ot specify that it does not apply for the UE that is configured with PUCCH SCell. | 1 |  | Add the missing word “configuration”, i.e. Field sCellDeactivationTimer does not apply for the PUCCH SCell configuration. Coordinator: Corrected in general CR. Removed from the General CR based on 36.331 rapporteur’s comments. |  | Closed (General CR) |
| N.100 | 6.3.2 MAC-MainConfig:: extendedPHR2-r13 | The field cannot be released once it’s configured. | 1 |  | Use release-setup or Need OR to allow release of the field. Ericsson: Agree. Either setup release or OR would work. Samsung: Boolean with need ON seems better (and is often used in such cases) Coordinator: Corrected in general CR. The data type is changed from ENUMERATED to BOOLEAN and the Need code is maintained as ON. |  | Closed (General CR) |
| C.053 | 6.3.2 P-C-AndCBSR-r13 | Condition (BeamformedK1a) is incorrect. According to the description in RAN1 parameter list, i.e. “Only applies when CSIReportingType = Class B (K=1) PMI-Config = 1”, alternativeCodebookEnabledBeamformed should be configured. | 1 |  | The field is optionally present, need OR, if included in CSI-RS-InfoBeamformed while neither csi-RS-ConfigNZPIdListExt noris not configured but alternativeCodebookEnabledBeamformed is configured. Otherwise the field is not present Huawei: OK Coordinator: Corrected in general CR. |  | Closed (General CR) |
| C.055 | 6.3.2 P-C-AndCBSR-r13 | if codebookSubsetRestriction3-r13 is configured then the UE shall ignore the legacy field codebookSubsetRestriction-r11 (for TM10) or codebookSubsetRestriction-r10 (for TM9). The relationship should be specified in the field description. | 1 |  | Add in Field description codebookSubsetRestriction3 UE shall ignore codebookSubsetRestriction-r11 or codebookSubsetRestriction-r10 if codebookSubsetRestriction3-r13 is configured. Huawei: OK Coordinator: It is proposed to addsome text in the field description, i.e. codebookSubsetRestriction3 E-UTRAN configures the field only if neither p-C-AndCBSRListExt nor alternativeCodebookEnabledBeamformed is not configured. UE shall ignore codebookSubsetRestriction-r11 or codebookSubsetRestriction-r10 if codebookSubsetRestriction3-r13 is configured. Usually all RAN2 requirements are formulated as “The UE shall …”. So perhaps the added text should be reformulated as The UE shall ignore codebookSubsetRestriction-r11…. Corrected in general CR. |  | Closed (General CR) |
| I.044 | 6.3.2 PhysicalConfigDedicated | In IE LAA-SCellConfiguration-r13 the suffix “-r13” is missing for the new fields. Furthermore, in the concerned field descriptions some editorial corrections need to be made: | 1 |  | Add missing suffices “-r13” for the new fields and fix the editorials in the field descriptions. LAA-SCellConfiguration-r13 ::= SEQUENCE { subframeStartPosition-r13 ENUMERATED {s0, s07}, laa-SCellSubframeConfig-r13 BIT STRING (SIZE(8)) } laa-SCellSubframeConfig A bit-map indicating laaLAA SCell subframe configuration, "1" denotes that the corresponding subframe is allocated as MBSFN subframe. The bitmap is interpreted as follows: Starting from the first/leftmost bit in the bitmap, the allocation applies to subframes #1, #2, #3 , #4, #6, #7, #8,and #9. subframeStartPosition Indicates possible starting positions of transmission in the first subframe of the DL transmission burst, see TS 36.211 [21].Value s0 means the starting position is subframe boundary, s07 means the starting position is either subfarmesubframe boundary or slot boundary. Ericsson: Agree Coordinator: Corrected in general CR. |  | Closed (General CR) |
| D.021 | 6.3.2 PhysicalConfigDedicated | Suffix “-r13” is missing for subframeStartPosition and laa-SCellSubframeConfig in LAA-SCellConfiguration-r13. | 1 |  | LAA-SCellConfiguration-r13 ::= SEQUENCE { subframeStartPosition-r13 ENUMERATED {s0, s07}, laa-SCellSubframeConfig-r13 BIT STRING (SIZE(8)) } Ericsson: Agree Coordinator: Corrected in general CR |  | Closed (General CR) |
| D.027 | 6.3.2 PhysicalConfigDedicated | pucch-Cell field description can be improved: “”if PUCCH feedback of this SCell is not sent on PCell or PSCell, this field is present if SCell is used to send PUCCH feedback of this SCell (or cell). | 1 |  | Proposed improvement “this field is present if PUCCH feedback is sent on PUCCH SCell of this SCell, and not setn on PCell or PSCell” Ericsson: Agree (the typo in the proposed improvement should be fixed though). Coordinator: The intention of this proposal seems to be to improve the field description as follows, if PUCCH feedback of this SCell is not sent on PCell or PSCell, this field is present if PUCCH feedback is sent on PUCCH SCell of this SCell, and not sent on PCell or PSCellSCell is used to send PUCCH feedback of this SCell (or cell). This needs to be confirmed. Otherwise it is difficult to make the correction. Samsung: If the field is configured for a PUCCH cell, the field description could be simple i.e. as follows: “If present, PUCCH feedback is sent on the PUCCH cell. If absent PUCCH feedback is sent on PCell or PSCel. The field is however not really needed for the PUCCH SCell, so we may also consider the following: “If present, PUCCH feedback is sent on the PUCCH cell. If absent PUCCH feedback is sent on PCell or PSCel, or if the cell concerns the PUCCH SCell, on the concerned cell. Coordinator: Corrected in general CR according to Samsungs second proposal. |  | Closed (General CR) |
| E.259 | 6.3.2 PhysicalConfigDedicated | TPC-PDCCH-ConfigSCell is a REL-13 information element and therefore both its definition and type reference should use –r13 suffix. | 1 |  | Add –r13 suffix to TPC-PDCCH-ConfigSCell type definition and to the type refence in PhysicalConfigDedicated Coordinator: Add –r13 suffix to TPC-PDCCH-ConfigSCell-r13 ::= CHOICE { release NULL, setup SEQUENCE { tpc-Index-PUCCH-SCell-r13 TPC-Index } } and tpc-PDCCH-ConfigPUCCH-SCell-r13 TPC-PDCCH-ConfigSCell-r13 OPTIONAL, -- Cond PUCCH-SCell Coordinator: Same as D.031. Corrected in general CR. |  | Closed (General CR) |
| C.074 | 6.3.2 PUCCH-Config | In field description, the reference section is 7.2. | 1 |  | In field description numberOfPRB-format4, startingPRB-format4 Parameter see TS 36.213 [23,x.y]. PUCCH resource(s) of PUCCH format 4. numberOfPRB-format5, cdm-index-format5 Parameter see TS 36.213 [23,x.y]. PUCCH resource(s) of PUCCH format 5. Ericsson: OK Coordinator: Correctd in general CR. |  | Closed (General CR) |
| I.040 | 6.3.2 SoundingRS-UL-Config | There are some issues in ASN.1 and field description of srs-UpPtsAdd: - ASN.1: the text in red font color should be corrected to black font color. - ASN.1: Suffix “-r13” is missing for the field srs-UpPtsAdd (2x). - ASN.1: In IE SRS-ConfigAp-r13 all fields with suffix “-r10” should be corrected to “-r13. - Field description: a “shall” is normally not used for E-UTRAN behavior, so the description should be better rephrased to e.g. “If E-UTRAN configures both soundingRS-UL-ConfigDedicatedUpPTsExt and soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt, srs-UpPtsAdd in both fields is set to the same value.” | 1 |  | Fix the issues in ASN.1 and field description of srs-UpPtsAdd. srs-UpPtsAdd The field only applies for TDD. If E-UTRAN configures both soundingRS-UL-ConfigDedicatedUpPTsExt and soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt, it shall set srs-UpPtsAdd to in both fields to the same value srs-UpPtsAdd in both fields is set to the same value. Huawei: OK Coordinator: Apart from the field reformulation the following ASN.1 corrections are proposed SoundingRS-UL-ConfigDedicatedUpPTsExt-r13 ::= CHOICE{ release NULL, setup SEQUENCE { srs-UpPtsAdd-r13 ENUMERATED {sym2, sym4}, srs-Bandwidth-r13 ENUMERATED {bw0, bw1, bw2, bw3}, and SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 ::= CHOICE{ release NULL, setup SEQUENCE { srs-UpPtsAdd-r13 ENUMERATED {sym2, sym4}, srs-ConfigIndexAp-r13 INTEGER (0..31), and SRS-ConfigAp-r13 ::= SEQUENCE { srs-AntennaPortAp-r10-r13 SRS-AntennaPort, srs-BandwidthAp-r10-r13 ENUMERATED {bw0, bw1, bw2, bw3}, freqDomainPositionAp-r10-r13 INTEGER (0..23), transmissionCombAp-r10-r13 INTEGER (0..3), cyclicShiftAp-r10-r13 ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7, cs8, cs9, cs10, cs11}, transmissionCombNum-r13 ENUMERATED {n2, n4} } Coordinator: Corrected in general CR. |  | Closed (General CR) |
| N.143 | 6.3.2 SoundingRS-UL-Config | SoundingRS-UL-Config field descriptions: unnecessary/misleading “to” in the description of srs-UpPtsAdd | 1 |  | Remove redundant “to” or specify to which value those should be set. Coordinator: Corrected in general CR |  | Closed (General CR) |
| D.032 | 6.3.2 UplinkPowerControl | Lack of –r13 suffix: deltaF-PUCCH-Format4 deltaF-PUCCH-Format5 | 1 |  | Add –r13 suffix Coordinator: Corrected in general CR. |  | Closed (General CR) |
| N.081 | 6.3.2 CRI-ReportConfig-r13:: cri-ConfigIndex-r13, cri-ConfigIndex2-r13 | Could use a new type value for these since they have exactly the same value range. See also N.004 | 2 |  | Define a new IE for the type and use that for both fields. Ericsson: Agree. Samsung: Agree (also to generally define IEs if used multiple times) Chair: => Agree to proposal => Can be closed when added to general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| N.130 | 6.3.2 PUCCH-ConfigDedicated-v13xy | codebooksizeDetermination-r13 INTEGER(0..1), maximumPayloadCoderate-r13 INTEGER (0..7), These should be only configured if format4/5 is used. | 2 |  | Consider making these OPTIONAL with COND indicating “The field is mandatory present if the pucch-Format is format4 or format5.” CATT: according to current structure of PUCCH-ConfigDedicated-v13xy, it can only configured with format 4/5. DCM: According to the L1 parameter list, this parameter can be configured for UEs configured with more than 5 CCs regardless of the PUCCH format. It does not have to be restricted for PUCCH format 4/5. In that sense, optional presence with Need ON seems better. Chair: => Covered by N.131. Coordinator: can be closed since already covered by another issue, |  | Closed (ASN.1) |
| Z.070 | 6.3.2 | In the IE “CSI-Process”, for the definition of new IE “P-C-AndCBSR-r13”, there is no exact Bit string size for “codebookSubsetRestriction1-r13” “codebookSubsetRestriction2-r13” “codebookSubsetRestriction3-r13” Also in field description, there are some inconsistency with above two parameters. | 2 |  | P-C-AndCBSR-r13 ::= SEQUENCE { legacySet SEQUENCE { p-C-r11 INTEGER (-8..15), codebookSubsetRestriction1-r13 BIT STRING } OPTIONAL, -- Cond BeamformedKna codebookSubsetRestriction2-r13 BIT STRING OPTIONAL, -- Cond NonPreCoded codebookSubsetRestriction3-r13 BIT STRING OPTIONAL -- Cond BeamformedK1a } codebookSubsetRestriction1(?) Parameter: codebookSubsetRestriction, see TS 36.213 [23] and TS 36.211 [21]. The number of bits in the codebookSubsetRestriction for applicable transmission modes is defined in TS 36.213 [23]. codebookSubsetRestriction2(?) If p-C-AndCBSRList-r13 is included, E-UTRAN includes 2 entries otherwise 1 (i.e. E-UTRAN configures the same number of entries for the original and the CBSR2 field. Huawei: the size has been captured in 213. Ericsson: Starting in Rel-10 it seems that codebookSubsetRestriction is defined as a BIT STRING without explicit length indicator. We think it is fine to do so now as well. Furthermore we agree that the field descriptions are confusing and should be improved but we are not sure how to resolve it. This issue is similar to N.087 CATT: in.054, we propose to remove the field description for codebooksubsetRestriction2List, since the condition is clear. For codebookSubsetRestriction we think it is incorrect to change it to codebookSubsetRestriction1, but we think we can include all the restriction in the field description, i.e. codebookSubsetRestriction, codebookSubsetRestriction1, codebookSubsetRestriction2, codebookSubsetRestriction3, because the bit string length is defined in RAN1 spec. Coordinator: Packed encoding rules can handle unconstraint bit strings. In that case, a length determinant is appended. So, technically nothing is incorrect but it should be noted that the encoding would be more compact if the bit string had a constraint size. There seems to be different opinions and therefore discussion si needed. Changed from class 1 to class 2. Chair: => No change needed |  | Closed |
| N.077 | 6.3.2 CQI-ReportPeriodic-v13xy | Should CRI-ReportConfig-R13 be need OR instead of ON in order to be able to remove it? | 2 |  | Change OR to ON – NOTE two occasions! Ericsson: This can be considered. CATT: release+setup structure is used for CRI-ReportConfig-R13 inside. so maybe ON is more appropriate than OR. Samsung: See E.250 Chair: => No change needed |  | Closed |
| I.038 | 6.3.2 CSI-RS-ConfigNZP | In eMIMO-Info-r13 the smallest number of ResourceConfig-r13 in nzp-resourceConfigList-r13 is 2. Since resourceConfig-r11 is always present, how to jointly use the field resourceConfig-r11 and nzp-resourceConfigList-r13 needs to be clarified for NonPrecoded CSI-RS. For instance, it is not clear how to configure NZP CSI-RS configuation for antenna port 16. [[ eMIMO-Info-r13 CHOICE { release NULL, setup SEQUENCE { nzp-resourceConfigList-r13 SEQUENCE (SIZE (2..8)) OF ResourceConfig-r13, cdmType ENUMERATED {cdm2, cdm4} OPTIONAL -- Need OR } } OPTIONAL -- Need ON ]] | 3 |  | Discuss how to jointly use the field resourceConfig-r11 and nzp-resourceConfigList-r13 for NonPrecoded CSI-RS. Options are: - Clarify that UE omits the resourceConfig-r11 on the condtion that nzp-resourceConfigList-r13 is present, or - change the value range of IE nzp-resourceConfigList-r13 from (Size (2..8)) to (Size (1..7)) Huawei: slightly prefer option 1 Ericsson: We have a slight preference for the first option, but let’s discuss. CATT: same comment as C.057. our proposal is same as the option 1, e.g. clarification on ignoring the legacy config. Intel: we are fine to go with option 1. Chair: => Duplicate of C.057. See conclusion of C.057. |  | Closed |
| N.088 | 6.3.2 CSI-RS-ConfigNZP:: ResourceConfig-r13 | Type could also be used for Rel-11 field resourceConfig-r11 since it corresponds to the same type? | 2 |  | Discuss if we use the same type for both IEs from Rel-13 onwards. Ericsson: Technically agree, but we wonder if it would be confusing if a Rel-11 IE would suddenly use a Rel-13 type? Or do we then simply name the IE ResourceConfig-r11? Samsung: Agree. Note that there are more cases of an rel-N field referring to an rel M IE (with M> N) i.e. we consider this to be appropriate Chair: => No change |  | Closed |
| I.041 | 6.3.2 MAC-MainConfig | eDRX-Config-CycleStartOffset-r13 is placed after the extension marker but it should be better added as Rel-13 NCE of longDRX-CycleStartOffset-v13xy within DRX-Config. Furthermore, the value ranges for sf5120 and sf10240 are incorrect. eDRX-Config-CycleStartOffset-r13 CHOICE { release NULL, setup CHOICE { sf5120 INTEGER(0..1), sf10240 INTEGER(0..3) } } OPTIONAL, -- Need ON, As result, the field description eDRX-Config-CycleStartOffset and its reference in longDRX-CycleStartOffset can be removed as well, and a reference to longDRX-CycleStartOffset-v13xy needs to be added in longDRX-CycleStartOffset. | 2 |  | Add the extended values sf5120 and sf10240 in ASN.1 and update the field description of longDRX-CycleStartOffset as shown below: drx-Config-v13xy DRX-Config-v13xy OPTIONAL -- Need ON DRX-Config-v13xy ::= SEQUENCE { longDRX-CycleStartOffset-v13xy CHOICE { sf5120-v13xy INTEGER(0..5019), sf10240-v13xy INTEGER(0..10239) } } longDRX-CycleStartOffset longDRX-Cycle and drxStartOffset in TS 36.321 [6] unless eDRX-Config-CycleStartOffset is configured. The value of longDRX-Cycle is in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. If shortDRX-Cycle is configured, the value of longDRX-Cycle shall be a multiple of the shortDRX-Cycle value. The value of drxStartOffset value is in number of sub-frames. In case longDRX-CycleStartOffset-v1130 or longDRX-CycleStartOffset-v13xy is signalled, the UE shall ignore longDRX-CycleStartOffset (i.e. without suffix). Coordinator: Something needs to be adjusted because otherwise this correction cannot be made. The DRX-Config-v13xy IE is defined as DRX-Config-v13xy ::= SEQUENCE { onDuration-r13 ENUMERATED {psf300, psf400, psf500, psf600, psf800, psf1000, psf1200, psf1600}, drx-RetransmissionTimer-r13 ENUMERATED {psf40, psf64, psf80, psf96, psf112, psf128, psf160, psf320} } but it does not remind the proposed solution whereas the following extension field of MAC-MainConfig IE is mentioned in the issue description, eDRX-Config-CycleStartOffset-r13 CHOICE { release NULL, setup CHOICE { sf5120 INTEGER(0..1), sf10240 INTEGER(0..3) } } OPTIONAL, -- Need ON, So it seems that the following ASN.1 correction is proposed eDRX-Config-CycleStartOffset-r13 CHOICE { release NULL, setup CHOICE { sf5120-v13xy INTEGER(0..50190..1), sf10240-v13xy INTEGER(0..102390..3) } } i OPTIONAL, -- Need ON, Intel will come back to the proposed correction by coordinator Chair: => No change needed => May be revisited if an issue is found. |  | Closed |
| D.025 | 6.3.2 PDCP-Config | Condition for pdcp-SN-Size-v13xx is Rlc-AM3 which indicates “if pdcp-SN-Size-v1130 is absent”. Does it means that if if pdcp-SN-Size-v1130 is NOT absent, then pdcp-SN-Size-v13xx can not be configured? Or was the intention to replace pdcp-SN-Size-v1130? | 3 |  | Clarification of intention. Intel: See comment for H.072. Ericsson: Is it not clear as it is now? I.e. pdcp-SN-Size-v13xx can only be configured if pdcp-SN-Size-v1130 is not present? And now it says that if pdcp-SN-Size-v1130 is present then the field is not present. So is this not correct? Coordinator: Changed from class 1 to class 3 because this needs discussion. DCM: Our comment was to clarify that both pdcp-SN-Size-v1130 and pdcp-SN-Size-v13xx cannot be configured and either of them should be configured. From comments from Ericsson and Intel, it was clarified and so we’re fine with the current text. BTW, what is the UE behaviour if this field is not present as need OP? Is it to fallback 12 bit SN? There seems no field description or procedure text. This is also the same as the condition of 15 bit SN. Chair: => No change needed |  | Closed |
| N.109 | 6.3.2 PDCP-Config | The field pdcp-SN-Size-v13xx cannot be released once configured. The condition also states it is Need OP but semantics for UE when the field is missing are not indicated in the field description | 2 |  | Allow field to be released and indicate UE actions when the field is missing. Intel: The handling of pdcp-SN-Size-v13xx is same as pdcp-SN-Size-v1130. Not sure whether anything additional needs to be done. Ericsson: Agree. DCM: See our comment to D.025 Chair: => No change needed |  | Closed |
| S.060 | 6.3.2 PDCP-Config, ul-DataSplitThreshold | Clarification may be needed that UE ignores field ul-DataSplitDRB-ViaSCG when ul-DataSplitThreshold is configured | 2 |  | Ericsson: Disagree. The UE does not ignore ul-DataSplitDRB-ViaSCG when ul-DataSplitThreshold is configured. If the threshold is configured but not met the UE steers traffic according to ul-DataSplitDRB-ViaSCG. Intel: Disagree. Even if ul-DataSplitThreshold is configured, ul-DataSplitDRB-ViaSCG is still used. See section 4.2.2 of PDCP CR in R2-156330. DCM: Agree with Ericsson Chair: => No change needed. => Can check whether field description of ul-DataSplitDRB-ViaSCG needs to be updated. |  | Closed |
| N.111 | 6.3.2 PDSCH-Config:: PDSCH-ConfigDedicated-v13x0 | The field inside the PDSCH-ConfigDedicated-v13x0 is need ON, but the upper level fields (inside PhysicalConfigDedicated) are also Need ON. Hence, this IE can never be released. | 2 |  | Discuss whether to use Need OR or setup-release-structure for the inner level IE to allow releasing the field. Chair: => No change needed |  | Closed |
| N.122 | 6.3.2 PhysicalConfigDedicated:: crossCarrierSchedulingConfig-r13 | This field can never be released via delta signalling once configured | 2 |  | Do we need to be able to release this field? Ericsson: Yes, it should be possible to release this field. CATT: If CrossCarrierSchedulingConfig-r10 is configured, it can not be released too. Same way can be applied on the CrossCarrierSchedulingConfig-r13. Samsung: Agree with CATT i.e. not clear which scenario requires release (assuming reverting to REL-10 version is done by fullConfig) Chair: => No change needed |  | Closed |
| N.117 | 6.3.2 PhysicalConfigDedicated::cond PUCCH-SCell | None of the fields defined with this condition can be released – there are no actions if the field is not present. | 2 |  | Discuss if it needs to be ensured fields can be released properly. Ericsson: Yes, it should be possible to release this field. CATT: the action if the filed is not present should be added. Chair: => No change needed |  | Closed |
| N.118 | 6.3.2 PhysicalConfigDedicated::laa-SCellConfiguration-r13 | This field can never be released via delta signalling once configured | 2 |  | Do we need to be able to release this field? (The content fields have mandatory presence, so the only way to get rid of this is to release the whole configuration.) Ericsson: We would be fine with allowing this field to be released. CATT: same comments as N.119, since no delta configuration is supported on PUCCH SCell and SCell change, i.e. only releas+addition, it is unnecessary to release only this field. Samsung: See S.061. Comment CATT seems misplaced (issue concerns LAA). Top level should remain optional, need ON. If these fields are optional for LAA cell, release option may need be introduced (but note that delta signalling does not extend to lowest levels) Chair: => No change needed |  | Closed |
| N.115 | 6.3.2 PhysicalConfigDedicated::pucch-ConfigDedicated-r13 (PCell) | The upper level configuration cannot be released | 2 |  | Discuss whether setup-release mechanism is needed CATT: it should be support to release the configuration, and setup+release mechanism should be supported. DCM: Agree to use the release-setup structure like the legacy extensions, PUCCH-ConfigDedicated-vXYZ Samsung: Not sure as delta signalling does not extend to lowest levels. Should remain consistent with other similar extensions (introduced earlier). Note that suffix of field seems incorrect i.e. should be –v13xy Chair: See meeting notes |  | Closed |
| N.116 | 6.3.2 PhysicalConfigDedicated::pucch-ConfigDedicated-r13 (SCell) | The upper level configuration cannot be released, and there are multiple fields with the same prefix (pucchConfigDedicated) | 2 |  | Discuss whether setup-release mechanism is needed. Could also group the IEs under one umbrella IE. See also N.124 and N.131 Ericsson: It would be good to have an "umbrella IE ". CATT: setup-release mechanism shoud be supported. DCM: Agree to use the release-setup structure like the legacy extensions, PUCCH-ConfigDedicated-vXYZ Chair: See meeting notes |  | Closed |
| S.061 | 6.3.2 PhysicalConfigDedicatedSCell, laa-SCellConfiguration | Are both fields always configured for an LAA SCell? Isn’t there a need to release these parameters (while maintaning delta signalling possibility)? | 2 |  | Ericsson: We would be fine to have a release-setup possibility for laa-SCellConfiguration-r13. CATT: we donot see the need to change LAA SCell and normal SCell via delta configuration, so it is OK to always configured the parameters for LAA SCell. Chair: => Same issue as N.118. See conclusion of N.118. |  | Closed |
| E.262 | 6.3.2 RACH-ConfigCommon | MAC-ContentionResolutionTimer-v13xy is defined as an enumerated type with 6 code points. The unused codepoints could be defined as spare values. | 2 |  | Add 2 spare values to MAC-ContentionResolutionTimer-v13xy Chair: => No change need as difficult to use the spare values in a future release. |  | Closed |
| S.068 | 6.3.2 RadioResourceConfigCommonSIB, prach-Config-v13xy | Parameter is missing | 2 |  | CR? Chair => See S.064 |  | Closed |
| S.069 | 6.3.2 RadioResourceConfigCommonSIB, pucch-ConfigCommon-v13xy | Parameter is missing | 2 |  | CR? Chair => See S.064 |  | Closed |
| S.072 | 6.3.2 RadioResourceConfigDedicated, mbsfn-SubframeConfigList in neighCellsCRS-Info-r13 | A new IE is created (now used for SCells also), with the only difference being that mbsfn-SubframeConfigList is optional. No constraints are however defined regarding when this is/ is not configured e.g. only for PCell as only there CRE is supported? | 3 |  | Add clarification, unless no constraints apply Chair: => No change needed |  | Closed |
| N.137 | 6.3.2 SoundingRS-UL-Config:: SoundingRS-UL-ConfigDedicated{UpPTsExt-r13, -v13x0} | Do these IEs need to be extendible (like the corresponding aperiodic IEs are)? | 2 |  | Discuss whether to add ellipsis. Huawei: do not see the need to extend. Samsung: Agree with Huawei Chair: => No change needed |  | Closed |
| Z.067 | 6.2.2 RLC-Config | Value TRUE should be italic in the field description. | 1 |  | Change: Value TRUE means that 16 bit SN length and 16 bit SO length shall be used Ericsson: OK. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| Z.069 | 6.3.2 | In the IE “CQI-ReportConfig”, 2 spare values are missing for the definition of new IE “CRI-ReportConfig-r13”. The field description for “cri-ConfigIndex2” is there, but the field description for “cri-ConfigIndex” is missig! | 1 |  | CRI-ReportConfig-r13 ::= CHOICE { release NULL, setup SEQUENCE { cri-ConfigIndex-r13 ENUMERATED {n1, n2, n4, n8, n16, n32, spare2, spare1}, cri-ConfigIndex2-r13 ENUMERATED {n1, n2, n4, n8, n16, n32, spare2, spare1} OPTIONAL -- Need OR } } cri-ConfigIndex ……. (to be added?) cri-ConfigIndex2 Parameter: CRI Config Index ICRI, see TS 36.213 []. The parameter applies to the subframe pattern corresponding to csi-MeasSubframeSet2 or corresponding to the CSI subframe set 2 indicated by csi-MeasSubframeSets. E-UTRAN configures cri-ConfigIndex2 only if cri-ConfigIndex is configured. Huawei: OK Ericsson: Agree. Adding spare values and field description is beneficial. Coordinator: The handling of spare values will be discussed separately, see e.g . N.004. No proposal for the missing field description. Closed. |  | Closed |
| Z.072 | 6.3.2 | In the IE “SoundingRS-UL-Config”, for the IE definition of “SoundingRS-UL-ConfigDedicatedUpPTsExt-r13” and “SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13” The field “srs-UpPtsAdd” should be appended with –r13, since there was no legacy“srs-UpPtsAdd before | 1 |  | SoundingRS-UL-ConfigDedicatedUpPTsExt-r13 ::= CHOICE{ release NULL, setup SEQUENCE { srs-UpPtsAdd-r13 ENUMERATED {sym2, sym4}, srs-Bandwidth-r13 ENUMERATED {bw0, bw1, bw2, bw3}, srs-HoppingBandwidth-r13 ENUMERATED {hbw0, hbw1, hbw2, hbw3}, freqDomainPosition-r13 INTEGER (0..23), duration-r13 BOOLEAN, srs-ConfigIndex-r13 INTEGER (0..1023), transmissionComb-r13 INTEGER (0..3), cyclicShift-r13 ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7, cs8, cs9, cs10, cs11}, srs-AntennaPort-r13 SRS-AntennaPort, transmissionCombNum-r13 ENUMERATED {n2, n4} } } SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 ::= CHOICE{ release NULL, setup SEQUENCE { srs-UpPtsAdd-r13 ENUMERATED {sym2, sym4}, srs-ConfigIndexAp-r13 INTEGER (0..31), srs-ConfigApDCI-Format4-r13 SEQUENCE (SIZE (1..3)) OF SRS-ConfigAp-r13 OPTIONAL,--Need ON srs-ActivateAp-r13 CHOICE { release NULL, setup SEQUENCE { srs-ConfigApDCI-Format0-r13 SRS-ConfigAp-r13, srs-ConfigApDCI-Format1a2b2c-r13 SRS-ConfigAp-r13, ... } } OPTIONAL -- Need ON } } Huawei: OK Ericsson: Agree. Coordinator: Same as I.040. Closed. |  | Closed |
| N.076 | 6.3.2 CQI-ReportConfig | Description of cri-ReportConfig: Shouldn’t it be introduced as a new IE? | 1 |  | Consider if this is sufficient to have just cri-ReportConfig field description. Coordinator: There is cri-ReportConfig –r13 field and there is also CRI-ReportConfig-r13 information element. There does not seem to be any issue. Closed without actions. If there is still an issue, a new issue can be created (or the problem can be solved with a maintenance CR). |  | Closed |
| N.075 | 6.3.2 CQI-ReportConfig | Missing description for cri-ConfigIndex | 1 |  | Add description Huawei: OK Coordinator: Same as D.018. Since D.018 has a field description proposal, this issue can be closed. |  | Closed |
| I.034 | 6.3.2 CQI-ReportConfig | In the field description of cri-ReportConfig the references eMIMO-Type and beamformed should be set in italics. | 1 |  | Set the references eMIMO-Type and beamformed in italics. cri-ReportConfig E-UTRAN configures the field only if the UE is configured with eMIMO-Type set to beamformed and if multiple references to RS configuration using non-zero power transmission are configured (i.e. if csi-RS-ConfigNZPIdListExt is configured). Huawei: OK Ericsson: Agree. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.310 | 6.3.2 CQI-ReportConfig | The field description for cri-ReportConfig is supposed to be after the description for cri-ConfigIndex2. | 1 |  | Update the order Coordinator: Corrected during v1300 CR implementation |  | Closed |
| C.049 | 6.3.2 CrossCarrierSchedulingConfig-r13 | In field description, the reference section is 5.3.3.1 | 1 |  | cif- InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate this cell, see TS 36.212 [22, 5.x.x.x5.3.3.1]. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.083 | 6.3.2 CrossCarrierSchedulingConfig-r13:: cif-InSchedulingCell-r13 | Field is underlined and field description has placeholder section number | 1 |  | Remove underlining and correct placeholder number Ericsson: OK. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.087 | 6.3.2 CSI-Process | Field descriptions of codebookSubsetRestrictionX do not match the content of CSI-Process IE. There should be 3 separate codebookSubsetRestriction fields. It is unclear why there is something called “codebookSubsetRestriction2List” (not reflected in IE content) | 1 |  | Clean the situation regarding codebokSubsetRestriction parameters. Ericsson: Agree. This seems similar to Z.070 and I.036. Coordinator: Covered in Z.070 and I.036.Closed without actions. |  | Closed |
| E.072 | 6.3.2 CSI-process, CSI-RS-Config, CSI-RS-Info | The eMIMO-Type-r13 in the RAN2 spec could adopt the name from 36.213 CSI-reporting-Type. Also, in 36.213 the “nonprecoded” is CLASS A and “beamformed” is CLASS B but not sure which is better, maybe the RAN2 is more informative. [[ eMIMO-Type-r13 CHOICE { release NULL, setup CHOICE { nonPrecoded-r13 CSI-RS-InfoNonPrecoded-r13, beamformed-r13 CSI-RS-InfoBeamformed-r13 } OPTIONAL } OPTIONAL -- Need ON ]] CSI-RS-Config-v13x0 ::= SEQUENCE { eMIMO-Type-r13 CHOICE { release NULL, setup CHOICE { nonPrecoded-r13 CSI-RS-InfoNonPrecoded-r13, beamformed-r13 CSI-RS-InfoBeamformed-r13 } } OPTIONAL -- Need ON } | 1 |  | Consider changing names to align with RAN1 specs. CATT: support. same comment as C.050. Coordinator: No comments on the naming issue but this ASN.1 coding does not seem to be exactly the same as the one in the draft specification, e.g. there is an optionality bit for the setup entry in eMIMO-Type-r13 which is not correct ASN.1 syntax and that error cannot be found in the CR. [[ eMIMO-Type-r13 CHOICE { release NULL, setup CHOICE { nonPrecoded-r13 CSI-RS-InfoNonPrecoded-r13, beamformed-r13 CSI-RS-InfoBeamformed-r13 } OPTIONAL } OPTIONAL -- Need ON ]] It needs to be confirmed where this coding comes from and if there is an issue with the CR. Same as C.050. Closed Samsung: Incorrect optionality seems resolved/ not present |  | Closed |
| E.075 | 6.3.2 CSI-RS-Info | For the alternativeCodebookEnabledBeamformed, should there be “EUTRAN configures only when eMIMO-Type-r13 configured as Beamformed” added as this applies only to CLASS B? I think the limitation currently talks about K=1 which is the case for NonPrecoded as well. | 1 |  | Consider updating the field descriptions accordingly. Coordinator: It is not clear where this sentence shoul dbe added. Closed wihtout actions. If there is still an issue, a new issue can be created or the problem can be solved with a maintenance CR. |  | Closed |
| N.089 | 6.3.2 CSI-RS-Info | The name of the column for field descriptions is incorrectly named: “CSI-RS-ConfigNZP field descriptions” | 1 |  | Correct column’s name. Huawei: what’s the real problem? Coordinator: Corrected during v1300 CR implementation. |  | Closed |
| N.090 | 6.3.2 CSI-RS-Info | Field description for “codebookConfigN1” was probably supposed to be named “codebookConfig-Nx”, according to the subsequent text. | 1 |  | Change the field name. Coordinator: Same as H.071 and E.076 |  | Closed |
| H.070 | 6.3.2 CSI-RS-Info | CSI-RS-InfoNonPrecoded-r13 ::= SEQUENCE { p-C-AndCBSRList-r13 SEQUENCE (SIZE (1..2)) OF P-C-AndCBSR-r13 OPTIONAL, -- Need OR, | 1 |  | Delete the “,” behind “Need OR” Coordinator: Corrected during v1300 CR implementation |  | Closed |
| H.071 | 6.3.2 CSI-RS-Info | codebookConfigN1 Indicates the number of antenna ports per polarization in dimension x as used for transmission of CSI reference signals. Value an1 corresponds to 1 antenna port, an2 to 2 antenna ports and so on, see TS 36.213 [23]. | 1 |  | codebookConfigN1should be changed to codebookConfigNx Coordinator: Same as N.090 and E.076 |  | Closed |
| N.091 | 6.3.2 DMRS-Config | “OPTIONAL-- Need OR” is missing space between OPTIONAL and --. | 1 |  | Introduce the missing space. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| C.050 | 6.3.2 eMIMO-Type-r13 | Name should be aligned with 36.213. eMIMO-Type-r13 change to CSI-Reporting-Type nonPrecoded-r13 change to CLASS A beamformed-r13 change to CLASS B | 1 |  | CSI-RS-Config-v13x0 ::= SEQUENCE { eMIMO-Type-r13 CHOICE { release NULL, setup CHOICE { nonPrecoded-r13ClassA-r13 CSI-RS-InfoNonPrecoded-r13, beamformed-r13ClassB-r13 CSI-RS-InfoBeamformed-r13 } } OPTIONAL -- Need ON } CSI-Process-r11 ::= SEQUENCE { … … eMIMO-Type-r13 CHOICE { release NULL, setup CHOICE { nonPrecoded-r13ClassA-r13 CSI-RS-InfoNonPrecoded-r13, beamformed-r13ClassB-r13 CSI-RS-InfoBeamformed-r13 } } OPTIONAL -- Need ON ]] } Ericsson: Agree. Coordinator: No comment on the naming issue but if the names are changed they should start with lower case letters because they are field identifiers, e.g. nonPrecoded-r13classA-r13 CSI-RS-InfoNonPrecoded-r13, Samsung: RAN2 generally tends to use more meaningful names so we see no need for changes CATT: Propose to change the name, and confirm coordinator's correction on the name. Coordinator: This does not seem to be agreeable. No change for now. Closed without actions. If there is still interest to change something, it can be discussed in a maintenance CR. |  | Closed |
| C.063 | 6.3.2 MAC-MainConfig | Field description, editorial correction | 1 |  | Value in number of SR period(s) of shortest SR period of any serving cell with PUCCH. Ericsson: We think the original text is fine and correct. Coordinator: This does not seem to be agreeable. No change for now. Closed without actions. If there is still interest to change something, it can be discussed in a maintenance CR. |  | Closed |
| C.054 | 6.3.2 P-C-AndCBSR-r13 | codebookSubsetRestriction2List codebookSubsetRestriction3 the parameters can be removed in field description because the description is not updated with current structure, and it seems not so useful. | 1 |  | Remove codebookSubsetRestriction2List and codebookSubsetRestriction3 in field description. Huawei: prefer to keep it. Coordinator: This does not seem to be agreeable. No change for now. Closed without actions. If there is still interest to change something, it can be discussed in a maintenance CR. |  | Closed |
| E.190 | 6.3.2 PDCCH-Config | The field descriptions for pdcch-candidateReductionAL1/2/3/4/5 is named " pdcch-candidateReductionALx ". I don’t know if this is OK | 1 |  | If not OK to name the field description like this then fix this. Coordinator: ASN.1 standard allows such an identifier name but it is, of course, up to the working group to decide what kind of field names are valid. If there are no other proposals, the field name is probably OK. Closed without actions Samsung: No change needed i.e. this approach is used several times |  | Closed |
| C.069 | 6.3.2 PDCCH-Config | In field description, the reference section is 9.1. | 1 |  | In field descripton skipMonitoringDCI-format0-1A Indicates the whether UE is configured to omit monitoring DCI fromat 0/1A, see TS 36.213 [23, 9.1x.y]. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| C.070 | 6.3.2 PDCCH-Config | In field description, the reference section is 9.1. | 1 |  | In field descripton pdcch-candidateReductionALx Indicates reduced (E)PDCCH monitoring requirements on user specific search space of the x-th aggregation level, see TS 36.213 [23, 9.1x.y]. n0 corresponds to value 0%, n33 corresponds to 33% and so on. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.314 | 6.3.2 PDCP-Config | The field description is supposed to be before the description statusPDU-TypeForPeriodic. | 1 |  | Update the order Coordinator: There is no such field as statusPDU-TypeForPeriodic. Closed without actions.. |  | Closed |
| C.068 | 6.3.2 PDCP-Config | UE and PDCP based SR have not been fully agreed at stage 2 yet. | 3 |  | Remove all UE and PDCP based SR contents and related IEs: statusFeedbackRequired-r13 statusPDU-TypeForPolling-r13 statusPDU-TypeForPeriodic-r13 statusPDU-Periodicity-r13 Ericsson: This is a duplicate. Qualcomm: Duplicate entry Coordinate: Closed because duplicated |  | Closed |
| D.022 | 6.3.2 PhysicalConfigDedicated | Editorial correction of the field description of laa-SCellSubframeConfig and pucch-Cell. | 1 |  | laa-SCellSubframeConfig A bit-map indicating laaLAA SCell subframe configuration pucch-Cell ifIf PUCCH feedback of this SCell Ericsson: Agree. CATT: PUCCH feedback is proposed to change to HARQ feedback. Coordinator. The proposed editorials are corrected during v1300 CR implementation. The name change proposal (CATT) is left open. DCM: See our comment to D.028 Coordinator: Corrected during v1300 CR implementation |  | Closed |
| E.316 | 6.3.2 PhysicalConfigDedicated | Typo in the following description: “subframeStartPosition Indicates possible starting positions of transmission in the first subframe of the DL transmission burst, see TS 36.211 [21].Value s0 means the starting position is subframe boundary, s07 means the starting position is either subfarme boundary or slot boundary.” | 1 |  | Coordinator: Same as I.044. |  | Closed |
| N.123 | 6.3.2 PhysicalConfigDedicatedSCell-r10 | pucch-Cell-r13 presence condition incorrect. It should be only present for non PUCCH SCell. | 2 |  | Change condition to “The field is optionally present for SCell not being PUCCH SCell, Need OR. Otherwise it is not present” Coordinator: Same as N.117. |  | Closed |
| C.073 | 6.3.2 PUCCH-Config | In field description, the reference section is 7.3. | 1 |  | In field description, codebooksizeDetermination Parameter indicates whether HARQ codebook size is determined with downlink assignment indicator based solution or number of configured CCs, see TS 36.213 [23, 7.3x.y]. Ericsson: OK. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| C.075 | 6.3.2 PUCCH-Config | In field description, the reference section is 7.2. And the description is updated according to the RAN1 LS. | 1 |  | In field description, maximumPayloadCoderate Parameter xx see TS 36.213 [23, 7.2x.y]. Maximum payload or code rate for multi P-CSI and P-CSI + HARQ-ACK + SR (if any).on each PUCCH resource Coordinator: Corrected during v1300 CR implementation |  | Closed |
| C.076 | 6.3.2 PUCCH-Config | In field description, the reference section is 7.3. | 1 |  | In field description spatialBundlingPUCCH Parameter indicates whether spatial bundling is enabled or not for PUCCH, see TS 36.213 [23, 7.3x.y] spatialBundlingPUSCH Parameter indicates whether spatial bundling is enabled or not for PUSCH, see TS 36.213 [23, 7.3x.y] Ericsson: OK Coordinator: Corrected durng v1300 CR implementation |  | Closed |
| C.077 | 6.3.2 PUCCH-Config | In field description, the reference section is 10.2. | 1 |  | In field description, harq-TimingTDD Parameter indicates for a TDD SCell when aggregated with a TDD PCell of different UL/DL configurations whether deriving the HARQ timing for such a cell is done in the same way as the DL HARQ timing of an FDD SCell with a TDD PCell, see TS 36.213 [23, x.y]. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.138 | 6.3.2 SoundingRS-UL-Config | Missing comma after “OPTIONAL” in case of transmissionComb-v13x0 | 1 |  | Add missing comma. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.139 | 6.3.2 SoundingRS-UL-Config | transmissionCombNum-r13: “OPTIONAL-- Need OR” should be fixed (space introduced). | 1 |  | Introduce the missing space. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.140 | 6.3.2 SoundingRS-UL-Config | srs-ConfigApDCI-Format1a2b2c- v13x0 within SoundingRS-UL-ConfigDedicatedAperiodic-v13x0 is written with unnecessary space between the hyphen and “v13x0” | 1 |  | Remove unnecessary space. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.141 | 6.3.2 SoundingRS-UL-Config | In SRS-ConfigAp-v13x10 (transmissionCombAp-v13x0) Redundant comma: OPTIONAL, -- Need OR, | 1 |  | Remove unnecessary comma Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.142 | 6.3.2 SoundingRS-UL-Config | In SRS-ConfigAp-v13x10 (cyclicShiftAp-v13x0) Redundant comma: OPTIONAL, -- Need OR, | 1 |  | Remove unnecessary comma Coordinator: Corrected during v1300 CR implementation |  | Closed |
| D.031 | 6.3.2 TPC-PDCCH-Config | Lack of –r13 suffix TPC-PDCCH-ConfigSCell | 1 |  | Add –r13 suffix Coordinator: Same as E.259 |  | Closed |
| N.145 | 6.3.2 UplinkPowerControl:: UplinkPowerControlCommon-v13xy, UplinkPowerControlPSCell-v13xy | Missing blank between two values: “deltaFX2,deltaFX3” | 1 |  | Add blank, i.e. “deltaFX2,deltaFX3” “deltaFX2, deltaFX3” Coordinator: Corrected during v1300 CR implementation |  | Closed |
| 6.3.3 Security control information elements |  |  |  |  |  |  |  |
| 6.3.4 Mobility control information elements |  |  |  |  |  |  |  |
| N.148 | 6.3.4 SCellIndex, ServCellIndex | Naming of SCellIndex-r13 differs from ServCellIndexFull-r13 | 2 |  | Discuss if we align the names: Either we 1) use the suffix “Full” for both, or 2) no suffix for both, or 3) leave them as they are. Ericsson: Would be fine to use "Full". Nokia Networks: We are also fine with “Full” for both. Coordinator: See also D.033. Samsung: Suffix –r13 already implies it is a revised version which of course should include all values so we no need to use full (and would be confusing to do so, as there are several similar cases) Chair: => Remove the 'full' from the name ServCellIndexFull-r13 => Can be closed when included in the general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| S.111 | 6.3.4 CellReselectionSubPriority | Text can be simplified/ clarified | 1 |  | Rephrase to: The IE CellReselectionSubPriority indicates a value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency/ set of frequencies (GERAN)/ bandclass (CDMA2000). Coordinator: It seems that the following correction is proposed, The IE CellReselectionSubPriority indicates a value to be added to the value of cellReselectionPriority to obtain the absolute priority of the concerned carrier frequency/ set of frequencies (GERAN)/ bandclass (CDMA2000). an input parameter to derive the ultimate priority of the concerned carrier frequency for E-UTRAN to be used in Cell Reselection process, as defined in TS 36.304 [4]. The value of CellReselectionSubPriority is added to the value of cellReselectionPriority. Value oDot2 corresponds to 0.2, oDot4 corresponds to 0.4 and so on. Samsung: The last sentence should be kept i.e. clarifying the values. Coordinator: Corrected in general CR. The last sentence is kept. |  | Closed (General CR) |
| D.033 | 6.3.4 SCellIndex and ServCellIndex | For SCellIndex, SCellIndex-r13 is defined to cover the full range INTEGER (1..31), For ServCellIndex, ServCellIndexFull-r13 is defined to cover the full range. Probably good to align the name. | 1 |  | Change “ServCelIIndexFull-r13” to “ServCellIndex-r13” Ericsson: Agree. Coordinator: See also N.148. |  | Closed |
| 6.3.5 Measurement information elements |  |  |  |  |  |  |  |
| N.162 | 6.3.5 MeasResults:: ul-PDPC-DelayResult-r13 | For now the reporting of the UL PDCP delay is totally up to UE implementation, the result can be available but ReportCOnfig does not cover measurement reporting configuration | 4 |  | Contribution to the next meeting to discuss further. Ericsson: Agree that this needs further discussion, handling the meas. Object and the meas. reporting is not clear currently. Coordinator: Handled in a separate CR in R2-161918 CR 2040R1 |  | Closed (General CR) TDoc Nokia |
| C.078 | 6.3.5 MeasConfig | For measurment reporting, measurement id is mandatory present. However, the UL delay configuration and report cannot be associated with a measurement ID with the current extension. For example, if eNB configures multiple measurement IDs via measObjectToAddModList or measIdToAddModListExt-r12, and UL delay configuration as well in the same MeasConfig, the UE would be confused to associate which measurment ID with UL delay measurment. | 3 |  | In order to solve this issue, two alternative can be considered: Alt1: Reserve a measurement ID in specification for UL delay configuration and report. But this method is not flexible and not future-extension. Alt2: Non-critical extend the current measurement and report configuration structure to include the UL delay measurement. Possible non-critical extensions include measurement object extension (e.g. measObjectULDelay-r13) and report configuration extension (e.g. ReportConfigToAddMod-r13 ). The extended measurement object may be PDCP Layer or a specific QCI. And the extended report configuration includes the delay threshold and the report interval. Chair: - ul-DelayConfig-r13 moved to be included in the ReportConfigEUTRA - MeasObject value is ignored for a MeasId that points to a ReportConfigEUTRA that contains ul-DelayConfig-r13 (eNB can set MeasObject to any value). Other parameters may also need to be ignored (details to be determined). => Nokia will draft a text proposal based on bullets above and discussion to continue via email discussion as part of ASN.1 review. Coordinator: Handled in a separate CR. Can be closed. |  | Closed (General CR) (ASN.1) |
| S.077 | 6.3.5 MeasObjectEUTRA, MeasRSSI-Config | Field description should be clarified that this field defines the RMTC, and how that is defined in terms of its subfields i.e. for start, duration and periodicity | 2 |  | Improve field description and simplify description of subfields. Also consider renaming to RMTC Coordinator: A text proposal would be useful. Ericsson: Agree. Chair: => Samsung to distribute a text proposal by email to show the field description. => Rename MeasRSSI-Config as RMTC-Config. Coordinator: The IE name is changed. The field descriptions needs to be added. Coordinator: Can be closed after RAN2#93. Handled in a separate CR. |  | Closed (General CR) (ASN.1) |
| S.092 | 6.3.5 ReportConfigEUTRA, cond SINR | As we have delta signalling, values may be signalled in different messages. In such cases it is convention to use statement in field descrition. Also no need for UE autonomous release (i.e. E-UTRAN can easily release) | 2 |  | Include statement in field description indicating that E-UTRAN configures aN-ThresholdSINR only if triggerQuantity is set to SINR DCM: aN-ThresholdSINR is defined as conditional presence if triggerQuantity is set to SINR. Samsung: The point is that the two fields may not always appear together in the same message (due to delta signalling), in which case we clarify this constraint in the field descriptions Chair: => Agreed to change field description as proposed => Can be closed when included in general CR. Intel: May require further discussion how to use condition SINR and to update the field description of aN-ThresholdSINR due to the agreed restructuring acc. to N.164. Coordinator: Can be closed after RAN2#93. |  | Closed (General CR) (ASN.1) |
| N.160 | 6.3.5 MeasResults:: measResultPCell-v13xy, :: measResultSCell-v13xy, :: measResultBestNeighCell-v13xy | Is there a need to have the SEQUENCE wrapper since there is only one IE inside it? It does indicate these are NCEs of existing fields, but ASN.1 looks more complicated. Also, the measurements are not taken into account in procedural text – that should be added (if necessary). | 2 |  | Discuss if we should remove the SEQUENCE and rename the field as (e.g.) rs-SinrResultPCell-r13 and rs-SinrResultSCell-r13? Needs discussion how to add the procedural text that fills the measurements DCM: Should be discussed in ASN.1 review adhoc with detailed proposed changes. Coordinator: A detailed proposal is needed. Nokia still thinking about this but likely will not bring a contribution on this (i.e. this can stay as it is). Samsung: Without structure there is a need to clarify for which cell the result applies, so prefer to stick to current approach (more according to common conventions) Coordinator: Can be closed |  | Closed (General CR) |
| S.100 | 6.3.5 UL-DelayConfig, delayReportInterval | These values can already be signalled by the reportInterval within reportConfig i.e. no need for further signalling | 2 |  | Remove (somewhat related to general discussion about placement of the configuration) CATT: How to configure UL delay measurements needs further be discussed. DCM: The actual measurement and reporting is in and by PDCP layer and is not visible in RRC (not per MeasObject – ReportConfig) and therefore the configuration as such is needed. One alternative is to include hits UL-DelayConfig into “Other Config”. Chair: => Nokia to include in proposal for handling UL delay reporting. => Agree to attempt to use the existing field if feasible. Coordinator: Handled in a separate CR. Can be closed. |  | Closed (General CR) (ASN.1) |
| E.269 | 6.3.5 ReportConfigEUTRA | ThresholdEUTRA “field” description has a FFS for RS-SINR based .threhold for event evaluation There should not be any FFS in a frozen specification. Can it be confirmed that there is no agreed value yet? | 4 |  | Either determine a value or remove the threshold. DCM: it couldn’t be decided at RAN2#92 since RAN4 was still discussing the measured quantity value. After the meeting, RAN2 received an LS from RAN4 (R4-158389) to fix this FFS part. In that case, would it better to discuss at RAN2#93? Coordinator: Corrected in general CR based on the ad hoc meeting notes => For RS-SINR: RS-SINR based threshold for event evaluation. The actual value is (IE value – 46)/2 dB. See R2-160003. A small remark; it should be “field value” instead of “IE value” but this is a more general issue that can be fixed in a separate CR. |  | Closed (General CR) (ASN.1) TDoc DCM |
| S.098 | 6.3.5 ReportConfigEUTRA field, useWhiteList | Field description is confusing i.e. suggests the applicable cell definition in 5.5.4.1only applies when the bit is set | 2 |  | Refer to the whitelist e.g. ‘Indicates whether only the cells included in the whitelist of the associated measObject are applicable, as defined in xx Chair: => Proposed field description is agreed. => Can be closed when included in the general CR. Intel: Corrected in general CR |  | Closed (General CR) (ASN.1) |
| S.099 | 6.3.5 ReportConfigEUTRA field, useWhiteList | Constraints indicated on cover page are not specified i.e. that EUTRAN does not configure the field for events A1, A2, C1, C2 | 2 |  | Refer to the whitelist e.g. ‘Indicates whether only the cells included in the whitelist of the associated measObject are applicable, as defined in xx Samsung: Suggestion is actually to add a statement that E-UTRAN does not configure the field for events A1, A2, C1, C2 Chair: => Add a condition that it can only be configured for the relevent events. => Can be closed when included in the general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| S.096 | 6.3.5 ReportConfigEUTRA, | It is not clear if any restrictions apply regarding legacy fields and whether the UE shall ignore some fields | 3 |  | Clarify that the RSSI measurements applies for periodical (no restrictions regarding reportAmount). Clarify that UE ignores triggerQuantity, reportQuantity and maxReportCells Clarify how to handle purpose e.g. ignore or set to particular value? Ericsson: Agree, RSSI measurements are periodical. CATT: Support. DCM: Agree to clarify in the field description. Coordinator: The following correction is proposed measRSSI-ReportConfig If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy measurement reporting and ignore the fields triggerQuantity, reportQuantity and maxReportCells. E-UTRAN only includes this field when setting triggerType to periodical. Chair: => Proposal is agreed. => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| S.094 | 6.3.5 ReportConfigEUTRA, measRSSI-ReportConfig a.o. | Delta signalling is normally supported (given extension marker overhead) | 2 |  | Introduce choice release/ setup DCM: Agree. Chair: => Change measRSSI-ReportConfig-r13 to need ON and add OPTIONAL need OR one level lower (i.e. at channelOccupancyThreshold-r13) => Can be closed when included in general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| S.088 | 6.3.5 ReportConfigEUTRA, RS-SINR | Within aN-ThresholdSINR, the thresholds are replacement so we should not use ThresholdEUTRA-v13xy. I.e. can use IE RS-SINR-Range directly | 2 |  | DCM: Similar thing was already performed for ThresholdEUTRA-v1250. Need further discussion. Samsung: Lets do better/ not make the same mistake as in REL-12.. Chair: => Remove the IE level ThresholdEUTRA-v13xy => Can be closed when included in general CR. Intel: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| D.036 | 6.3.5 RSRQ-Range | RSRQ-Range-v13xx is introduced to use the full range value without using the original field (w/o suffix). Should it be clarified in the spec for readability? In addition, given that the –v13xx field covers the original range and extended range in Rel-12, should it be considered to replace the legacy fields and so should be –r13? | 2 |  | Samsung: Suffix –r13 seems appropriate (and sufficient) Intel: We prefer to remove RSRQ-Range-v13xx and replace its references by RSRQ-Range-v1250, see also C.080 Chair: => Change the suffix to -r13 and add text to explain the usage of the -r13 variant compared to the -v1250 variant. => Can be closed when included in the general CR. Coordinator: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| Z.074 | 6.3.5 | In the IE “MeasObjectEUTRA”, 3 spare values are missing for the definition of new IE “MeasRSSI-Config-r13”. In the field description for “rmtc-SubframeOffset”, the behavior for “intra-frequency case” is not decribed.” | 1 |  | MeasRSSI-Config-r13 ::= CHOICE { release NULL, setup SEQUENCE { rmtc-Period-r13 ENUMERATED {ms40, ms80, ms160, ms320, ms640, spare3, spare2, spare1}, rmtc-SubframeOffset-r13 INTEGER(0..639) OPTIONAL, -- Need ON measDuration-r13 ENUMERATED {sym1, sym14, sym28, sym42, sym70, spare3, spare2, spare1} } } rmtc-SubframeOffset Indicates the RSSI and channel occupancy measurement timing configuration (RMTC) subframe offset for this frequency, see TS 36.214 [48]. For inter-frequency measurements, if this field is not configured, the UE chooses a random starting offset for measDuration. For intra-frequency measurment, this field shall be configured while measRSSI-Config is configured. Ericsson: Agree. Coordinator: Spare valuers are only specified for broadcasting. See e.g. N.004. Closed without actions,. |  | Closed (General CR) |
| Z.075 | 6.3.5 | In the IE “MeasResults”, for the definition of new IE “MeasResultForRSSI-r13”, there is corresponding field description for “channelOccupancy”, but no field description for “rssi-Result”. | 1 |  | MeasResultForRSSI-r13 ::= SEQUENCE { rssi-Result-r13 RSSI-Range-r13, channelOccupancy-r13 INTEGER (0..100) } rssi-Result Measured LAA RSSI result in dBm. (Note: mimic to rssiWLAN Measured WLAN RSSI result in dBm.) Ericsson: We should not call it "LAA RSSI" since this seems to limit the use of the RSSI measurements. We could just skip the "LAA" part. Preferably we should align with RAN1/4 specifications and there I assume they don’t say "LAA RSSI"? DCM: Agree with Ericsson view. Intel: Corrected in general CR. Followed Ericsson proposal to skip “LAA” part. |  | Closed (General CR) |
| S.074 | 6.3.5 MeasConfig, ul-DelayConfig (eMDT) | Parameters specifying when the UE should report what are normally part of the report configuration | 3 |  | Needs to be discussed during R2#93 (As this is not frequency specific measurement, we should consider how to handle the measObject i.e. either have no object linked or another type of object e.g. related to DRBs) Ericsson: We agree this should be discussed Coordinator: A discussion paper would be helpful. Samsung: We intend to cover this in a paper Chair: See meeting notes => General issue of measurements extensibility can be progressed in R14 under TEI14. Coordinator: Closed without actions. See also S.007, C.078, and S.093. |  | Closed (General CR) |
| I.048 | 6.3.5 MeasDS-Config | In the field description of ds-OccasionDuration it is said: The field ds-OccasionDuration is set to values 1 for an LAA SCell. However, the added sentence is misleading and might lead to confusion in case durationTDD-r12 is configured by ds-OccasionDuration where the min value is 2. Therefore, it is better to clarify that ds OccasionDuration-r13 should be ignored by the UE for an LAA SCell and the UE shall apply a value 1 instead. ds-OccasionDuration-r12 CHOICE { durationFDD-r12 INTEGER(1..maxDS-Duration-r12), durationTDD-r12 INTEGER(2..maxDS-Duration-r12) }, Furthermore, in the field description of subframeOffset there is a typo (remove “s” in the word “values”). | 2 |  | Clarify field description of ds-OccasionDuration and fix the typo in the field description of subframeOffset:as shown below: ds-OccasionDuration Indicates the duration of discovery signal occasion for this frequency. Discovery signal occasion duration is common for all cells transmitting discovery signals on one frequency. The UE shall ignore the field ds-OccasionDuration is set to values 1 for an LAA SCell and apply a value 1 instead. subframeOffset Indicates the subframe offset between SSS of the cell indicated by physCellId and the CSI-RS resource in a discovery signal occasion. The field subframeOffset is set to values 0 for an LAA SCell. Coordinator: Corrected in general CR. See also N.150. |  | Closed (General CR) |
| S.075 | 6.3.5 MeasDS-Config field descriptions, LAA | Restrictions regarding values should be formulated as E-UTRAN constraints | 1 |  | Change to: ‘For an LAA SCell, E-UTRAN configures/ does not configure Ericsson: Agree. Coordinator: Corrected in general CR. |  | Closed (General CR) |
| N.150 | 6.3.5 MeasDS-Config:: ds-OccasionDuration | The added field description is slightly confusing since the DS configuration is for a frequency, not for an Scell. See also N151 and N.152. | 1 |  | Change the added field description to: “The field ds-OccasionDuration is set to values 1 for a carrier frequency with a configured LAA SCell.” Ericsson: Agree. Coordinator: The following correction is proposed, ds-OccasionDuration Indicates the duration of discovery signal occasion for this frequency. Discovery signal occasion duration is common for all cells transmitting discovery signals on one frequency. The field ds-OccasionDuration is set to values 1 for a carrier frequency with a configured an LAA SCell. Coordinator: Corrected in general CR. See also I.048. . |  | Closed (General CR) |
| N.151 | 6.3.5 MeasDS-Config:: resourceConfig | The added field description is slightly confusing since the DS configuration is for a frequency, not for an Scell. See N.150 and N.152. | 1 |  | Change the added field description to: “The values {0, 4, 5, 9, 10, 11, 18, 19} are not applicable for a carrier frequency with a configured LAA SCell.” Ericsson: Agree. Coordinator: The following correction is proposed, resourceConfig Parameter: CSI reference signal configuration, see TS 36.211 [21, table 6.10.5.2-1 and 6.10.5.2-2]. The values {0, 4, 5, 9, 10, 11, 18, 19} are not applicable for a carrier frequency with a configured an LAA SCell. This needs tobe confirmed. Coordinator: Same as N.150. Closed. |  | Closed (General CR) |
| N.152 | 6.3.5 MeasDS-Config:: subframeOffset | The added field description is slightly confusing since the DS configuration is for a frequency, not for an Scell. See N.150 and N151. | 1 |  | Change the added field description to: “The field subframeOffset is set to values 0 for a carrier frequency with a configured LAA SCell.” Ericsson: Agree. Coordinator: The following correction is proposed, subframeOffset Indicates the subframe offset between SSS of the cell indicated by physCellId and the CSI-RS resource in a discovery signal occasion. The values {0, 4, 5, 9, 10, 11, 18, 19} are not applicable for a carrier frequency with a configured an LAA SCell. Coordinator: Same as N.150 and N.151. Closed. |  | Closed (General CR) |
| N.153 | 6.3.5 MeasObjectEUTRA:: rmtc-Period-r13, ::measDuration-r13 | Do we need to define spare values for these IEs? See also N.004. | 1 |  | See N.004 for discussion. Ericsson: Agree, applies to more fields for the RMTC. Coordinator: Spare values are only defined for broadcasting. No change. Closed. |  | Closed (General CR) |
| H.077 | 6.3.5 MeasResults | measResult Measured result of an E-UTRA cell; Measured result of a UTRA cell; Measured result of a GERAN cell or frequency; Measured result of a CDMA2000 cell; Measured result of a WLAN. Measured result of UE Rx–Tx time difference; or Measured result of UE SFN and subframe timing difference. In the above last one bullet, radio frame timing difference is absent. | 1 |  | measResult Measured result of an E-UTRA cell; Measured result of a UTRA cell; Measured result of a GERAN cell or frequency; Measured result of a CDMA2000 cell; Measured result of a WLAN. Measured result of UE Rx–Tx time difference; or Measured result of UE SFN, radio frame and subframe timing difference. Ericsson: Agree. DCM: Agree. Intel: Corrected in general CR. |  | Closed (General CR) |
| C.079 | 6.3.5 MeasResults | rsrpResultNCell-r10 should be rsrpResultNCell-r1 in MeasResultServFreq-r133. | 1 |  | measResultBestNeighCell-r13 SEQUENCE { physCellId-r13 PhysCellId, rsrpResultNCell-r13 RSRP-Range, rsrqResultNCell-r13 RSRQ-Range-v13xx, rs-sinrResult-r13 RS-SINR-Range-r13 OPTIONAL } OPTIONAL, Coordinator: Perhaps there is a misconception but there is no rsrpResultNCell-r10 field in MeasResultServFreq-r13 IE. There is a rsrpResultNcell-r13 field and it is named as proposed in the issue description. So there does not seem to be any issue and hence this can be closed. CATT: Following is the structure of MeasResultServFreq-r13 in -v015 CR. And we think rsrpResultNCell-r10 (in yellow) should be rsrpResultNCell-r13. MeasResultServFreq-r13 ::= SEQUENCE { servFreqId-r13 ServCellIndexFull-r13, measResultSCell-r13 SEQUENCE { rsrpResultSCell-r13 RSRP-Range, rsrqResultSCell-r13 RSRQ-Range-v13xx, rs-sinrResult-r13 RS-SINR-Range-r13 OPTIONAL } OPTIONAL, measResultBestNeighCell-r13 SEQUENCE { physCellId-r13 PhysCellId, rsrpResultNCell-r10 RSRP-Range, rsrqResultNCell-r13 RSRQ-Range-v13xx, rs-sinrResult-r13 RS-SINR-Range-r13 OPTIONAL } OPTIONAL, ... } Intel: The suffix for rsrpResultNCell-r10 was already corrected in general CR. |  | Closed (General CR) |
| C.080 | 6.3.5 MeasResults | Range is same as RSRQ-Range-v1250, i.e. INTEGER(-30..46). So it is no need to introce new IE. | 1 |  | MeasResultServFreq-r13 ::= SEQUENCE { servFreqId-r13 ServCellIndexFull-r13, measResultSCell-r13 SEQUENCE { rsrpResultSCell-r13 RSRP-Range, rsrqResultSCell-r13 RSRQ-Range-v125013xx, rs-sinrResult-r13 RS-SINR-Range-r13 OPTIONAL } OPTIONAL, measResultBestNeighCell-r13 SEQUENCE { physCellId-r13 PhysCellId, rsrpResultNCell-r10 RSRP-Range, rsrqResultNCell-r13 RSRQ-Range-v125013xx, rs-sinrResult-r13 RS-SINR-Range-r13 OPTIONAL } OPTIONAL, ... } RSRQ-Range information element RSRQ-Range-v13xx ::= INTEGER(-30..46) Intel: Agree Coordinator: Even though the new identifier has exactly the same range, it ihas different semantics. It suggests to not making any changes. Closed. |  | Closed (General CR) |
| C.081 | 6.3.5 MeasResults | RSSI and channel occupancy measurment is missing in the field description of measResult. | 1 |  | Add the missing measurement descriptions as follows: measResult Measured result of an E-UTRA cell; Measured result of a UTRA cell; Measured result of a GERAN cell or frequency; Measured result of a CDMA2000 cell; Measured result of a WLAN. Measured result of UE Rx–Tx time difference; or Measured result of UE SFN and subframe timing difference.; or Measured result of RSSI and channel occupancy. Ericsson: Agree. Qualcomm: Agree; also better to add “WLAN” before “RSSI”. Intel: Corrected in general CR, but w/o adding “WLAN” before “RSSI” to keep it generic and aligned with procedural text. |  | Closed (General CR) |
| D.035 | 6.3.5 MeasResults | A hyphen is missing in the filed name of rs-sinrResult. In addition, rsrpResult and rerqResult should also be corrected for the Re-13 extension although the error has been there since Rel-8? | 1 |  | rs-sinrResult-r13 should be rs-sinr-Result-r13. rsrpResultSCell-r13 -> rsrp-ResultSCell-r10 rsrqResultSCell-r13 -> rsrq-ResultSCell-r13 rsrpResultNCell-r10 -> rsrp-ResultNCell-r10 rsrqResultNCell-r13 -> rsrq-ResultNCell-r13 Intel: Corrected only the issue related to rs-sinrResult-r13 in general CR. Made also the corresponding corrections in the SCG-ConfigInfo message (10.2.2) and IE RRM-Config (10.3) + an additional correction of the suffix “-v13xy” to “-r13” in IE RRM-Config. Did not make the corrections to rsrpResultX and rsrqResultX as it would affect legacy. |  | Closed (General CR) |
| H.076 | 6.3.5 MeasResultsSTD | #### MeasResultSSTD The IE MeasResultSSTD consists of SFN, radio frame and subframe boundary difference between the PCell and the PSCell as specified in TS 36.214 [48]. In the above sentence, one reference is absent, because “subframeBoundaryOffsetResult” is according to the mapping table in TS 36.133 [16]. | 1 |  | #### MeasResultSSTD The IE MeasResultSSTD consists of SFN, radio frame and subframe boundary difference between the PCell and the PSCell as specified in TS 36.214 [48] and TS 36.133 [16]. DCM: OK Intel: OK Ericsson: OK. Intel: Corrected in general CR. |  | Closed (General CR) |
| N.163 | 6.3.5 QuantityConfig:: filterCoefficientRS-SINR-r13 | This field can never be released once configured | 1 |  | Change to Need OR like with CSI-RSRP L3 filter coefficient. DCM: Agree with Nokia. It is also applied for filterCoefficient defined for WLAN RSSI. Samsung: Seems preferrable to maintain delta signalling i.e. optional, need ON. So far filter coefficients are never released (but should be kept, even if hanging) Coordinator: Keep ON as explained by Samsung. Closed. |  | Closed (General CR) |
| S.095 | 6.3.5 ReportConfigEUTRA, measRSSI-ReportConfig a.o. | Wording field description can be improved | 1 |  | Change to: If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy Ericsson: OK DCM: OK. Coordinator: The following correction is proposed measRSSI-ReportConfig If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy measurement reporting. Intel: Corrected in general CR. |  | Closed (General CR) |
| N.166 | 6.3.5 ReportConfigEUTRA:: aN-Threshold2-r13 | Would be better to rename this as a5-Threshold-r13 since this is only used for event A5. | 1 |  | Rename the field and consider changing the field description as well. DCM: Agree if N.164 is adopted. Chair: => Align witrh R12. Add an-Threshold1 and a5-Threahold2 in R13, => Can be closed when included in general CR. Intel: Corrected in general CR. |  | Closed (General CR) |
| N.164 | 6.3.5 ReportConfigEUTRA:: triggerQuantity-v13xy, ::aN-ThresholdSINR-r13, ::reportQuantity-v13xy | Since these are all related to RS SINR, they could be put under the same setup-release structure – either they are all used or none are used | 1 |  | Move all fields under the same release-setup structure, e.g. like this: rs-Sinr-Config-r13 CHOICE { release NULL, setup SEQUENCE { triggerQuantity-v13xy ENUMERATED {sinr} OPTIONAL, -- Need ON aN-Threshold1-r13 ThresholdEUTRA-v13xy OPTIONAL, -- Need ON aN-Threshold2-r13 ThresholdEUTRA-v13xy OPTIONAL, -- Need ON reportQuantity-v13xy ENUMERATED {rsrpANDsinr, rsrqANDsinr, all} } } DCM: Agree. Coordinator: The following correction is proposed, rs-Sinr-Config-r13 CHOICE { release NULL setup SEQUENCE { triggerQuantity-v13xy CHOICE { release NULL, setup ENUMERATED {sinr} } OPTIONAL, -- Need ON aN-ThresholdSINR-r13 CHOICE { release NULL, setup SEQUENCE { threshold1-r13 ThresholdEUTRA-v13xy OPTIONAL, -- Need ON threshold2-r13 ThresholdEUTRA-v13xy OPTIONAL -- Need ON } } OPTIONAL, -- Cond SINR reportQuantity-v13xy CHOICE { release NULL, setup ENUMERATED {rsrpANDsinr, rsrqANDsinr, all} } } OPTIONAL, -- Need ON Samsung: With the new structure it is not possible to release an field anymore (i.e. entire SINR config needs to be released and added again), but maybe this is not a relevant scenario.. Note that abbreviation SINR should be all upper or lower case. Chair: => Agreed => Can be closed when included in the general CR Intel: Corrected in general CR. |  | Closed (General CR) |
| N.170 | 6.3.6 RF-Parameters-v13x0::supportedMIMOCapabilityDL | This seems like an error – we never agreed to have per UE MIMO capabilities signalled. | 1 |  | Remove the capability. Coordinator: The following correction is proposed, RF-Parameters-v13x0 ::= SEQUENCE { requestedMaximumCCs ENUMERATED {supported, includedUL, includedDL, includedBoth} OPTIONAL, skipFallbackCombinations ENUMERATED {supported, included} OPTIONAL, supportedBandCombinations-r13 SupportedBandCombinations-r13 OPTIONAL, supportedMIMO-CapabilityDL-r13 MIMO-CapabilityDL-r10 OPTIONAL } Intel: Corrected in general CR. Issue was put in the wrong subclause 6.3.5, it should be 6.3.6. Same as D.040. |  | Closed (General CR) |
| N.171 | 6.3.6 UE-EUTRA-Capability::supportOfLAA | Field description should state the LAA support is for DL (and for SCell) | 1 |  | Update field description: “Indicates whether the UE supports SCell LAA operation on DL.” Ericsson: In this spec it is only for SCells and for DL. I guess current wording is OK. But it would be fine to do as Nokia suggests if companies prefer that. Coordinator: The following correction is proposed, supportOfLAA Indicates whether the UE supports SCell LAA operation on DL. Intel: Corrected in general CR. Issue was put in the wrong subclause 6.3.5, it should be 6.3.6. |  | Closed (General CR) |
| C.084 | 6.3.5 ReportConfigEUTRA | According to descriptions of 5.5.3.1, if the purpose is set to reportCGI, the RSSI and channel occpancy measurments, SSTD measurments and periodical CSI-RS measurements would not be performed. Therefore, for measRSSI-ReportConfig, reportSSTD-Meas and reportStrongestCSI-RSs, the corresponding triggerType and purpose should be restricted to be periodical and reportStrongestCells respectively. | 2 |  | Add correponding descriptions in the filed description of measRSSI-ReportConfig, reportSSTD-Meas and reportStrongestCSI-RSs. DCM: For reportSSTD-Meas, it was already described in the field description which was missed in the initial draft version of 36.331. Samsung: Issue does not exist in MCC provided version. Coordinator: Corrected in general CR. |  | Closed (ASN.1) |
| S.087 | 6.3.5 MeasResults, LAA | Field channelOccupancy: wording can be improved | 1 |  | Change to: Indicates the percentage of samples when the RSSI was above channelOccupancyThreshold as configured within the associated reportConfig Ericsson: No strong view. The existing wording is very similar and equally easy to understand. Intel: We see no stringent need to update the field description. Coordinator: Closed without actions because thec current worindg is deemed sufficiently clear. |  | Closed |
| N.161 | 6.3.5 MeasResults:: measResultForRSSI-r13, :: rssi-Result-r13 | The name doesn’t tell these are RSSI measurements for LAA. There is also no field description. | 1 |  | Either rename as measResultForLAA-RSSI-r13 to differentiate LAA RSSI from WLAN RSSI measurements or add field description that says this is LAA RSSI. Ericsson: This was discussed a bit in Anaheim and we should not tie a certain measurement to a certain feature, i.e. we should not say that it is "LAA RSSI". Intel: same as Z.075. Corrected in general CR acc. to Z.075. |  | Closed |
| N.156 | 6.3.5 MeasObjectEUTRA | WhiteCellsToAddMod-r13 contains cellINdex-r13 | 2 |  | Is there a need to define cellIndex withr13 suffix? If there is no difference with cellIndex-r12? DCM: Even for cellIndex-r12, there was a original field without suffix… Perhaps, the same error existed from Rel-12? In general, the original field introduced from Rel-8 should have been reused? Samsung: Every subfield of a new REL-13 IE uses –r13 suffix, even if it refers to an REL-8 IE. So no change needed. It however would be appropriate to introduce an IE CellIndex for INTEGER (1..maxCellMeas), as this is used several times. Chair: => No change needed |  | Closed |
| S.076 | 6.3.5 MeasObjectEUTRA, LAA | It should be possible to release field rmtc-SubframeOffset. | 2 |  | As delta signalling does not seem essential, it seems easiest to just change to need OR Ericsson: Agree. CATT: Delta signalling would not be applied according to descriptions in 5.5.2.5. Chair: => No change needed |  | Closed |
| S.078 | 6.3.5 MeasObjectEUTRA, whitelist | Clarify that E-UTRA does not include a PCI in both the whitelist and the blacklist | 2 |  | (Assumption is that both lists may appear together, as black listed cells apply for all measurements using the object while whitelisted cells only apply for some). Add the following sentence to the whiteCellsToAddModList field description: If E-UTRAN configures white-listed cells, it only includes physical cell identities that are not configured as part of the black-listed cells. Chair: => Not considered a realistic network configuration => No change needed |  | Closed |
| S.089 | 6.3.5 ReportConfigEUTRA, aN-ThresholdSINR | As the subfields use the same names as the original one, It is not clear which fields the UE applies and which ones it ignores | 2 |  | Clarify by adding that the UE only considers the thresholds include in aN-ThresholdSINR (and ignores any other corresponding values) DCM: In the field description, there is a statement that When configured, UE ignores aN-ThresholdM. Although it seems enough, further clarification could be considered. Samsung: Ok, no change seems needed Chair: => No Change needed |  | Closed |
| S.090 | 6.3.5 ReportConfigEUTRA, aN-ThresholdSINR | No need for IE ThresholdEUTRA-v13x0 | 2 |  | Directly refer to RS-SINR-Range DCM: See our comment to S.088 Chair => See S.088 |  | Closed |
| S.091 | 6.3.5 ReportConfigEUTRA, eventId | Seems better to state in trigger- and reportQuantity that E-UTRAN does not configure value SINR for events C1 and C2 | 2 |  | Move DCM: No strong opinion but slightly prefer the current place instead of duplicating the same text to triggerQuantity and reportQuantity. Chair: => No change needed |  | Closed |
| S.097 | 6.3.5 ReportConfigEUTRA, LAA | It is not clear if any restrictions apply regarding legacy fields and whether the UE shall ignore some fields | 3 |  | Clarify that the RSSI measurements applies for periodical Coordinator: See also S.096 Chair => See conclusion of S.096 |  | Closed |
| S.093 | 6.3.5 ReportConfigEUTRA, measRSSI-ReportConfig a.o. | The way new measurements are introduces does not seem very aligned (also regarding field names) | 3 |  | Try to align between the different new measurements that are introduced e.g. reportSTTD Chair: => No change needed Coordinator: It is agreed that general issue of measurements extensibility can be progressed in R14 under TEI14. Closed without actions. See also S.007, C.078 and S.074. |  | Closed |
| N.165 | 6.3.5 ReportConfigEUTRA:: MeasRSSI-ReportConfig-r13 :: channelOccupancyThreshold-r13 | This configuration is used to trigger RSSI measurements, but did we agree to different channel occupancy thresholds per event? | 3 |  | Discuss if it was agreed that we have different channel occupancy thresholds in each event. If not, then this could be moved to MeasRSSI-Config-r13 in MeasObjectEUTRA Ericsson: I think we should support to have separate thresholds per event. Or does Nokia see a problem with this? CATT: Periodical reporting was agreed for RSSI measurements. The same threshold for RSSI measurements on different frequencies may be applied. There is no need to change the existing configuration. DCM: It seems straightforward to configure the channel occupancy threshold as reportConfig rather than measObject. In that sense, threshold per event seems reasonable as well. Chair: => No change needed |  | Closed |
| E.198 | 6.3.5 DelayConfig | Missing line break before " delayReportInterval " | 1 |  | Add line break Coordinator: Corrected during v1300 CR implementation |  | Closed |
| D.034 | 6.3.5 MeasObjectEUTRA | Style error on the field description of measDuration. | 1 |  | Style should be corrected accordingly. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.155 | 6.3.5 MeasObjectEUTRA:: whiteCellsToAddModList, ::whiteCellsToRemoveList | The font size is 10 instead of 9 | 1 |  | Use font size 9. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| N.154 | 6.3.5 MeasObjectEUTRA::measDuration | Font of the field description is wrong and sym1 / sym14 are not italicized. | 1 |  | Correct to use Arial and add italics. Ericsson: Agree Coordinator: Corrected during v1300 CR implementation |  | Closed |
| C.085 | 6.3.5 ReportConfigEUTRA | In ReportConfigEUTRA field descriptions, the description for reportSSTD-Meas is missing | 1 |  | According to R2-156904,the following description should be added into specification. reportSSTD-Meas If this field is set to true, the UE shall measure SSTD between the PCell and the PSCell as specified in TS 36.214 [48] and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN only sets this field to true when setting triggerType to periodical and purpose to reportStrongestCells. Intel: Was a CR implementation issue. CR R2-156904 contains the field description. Ericson: Agree DCM: Agree. Coordinator: CR implementation error. Can be closed. |  | Closed |
| E.071 | 6.3.5 UL-DelayConfig | Unnecessary spaces in the following: UL-DelayConfig information element -- ASN1START UL-DelayConfig-r13 ::= CHOICE { release NULL, setup SEQUENCE { delayThreshold-r13 ENUMERATED { ms30, ms40, ms50, ms60, ms70, ms80, ms90,ms100, ms150, ms300, ms500, ms750, spare4, ms1024, ms2048, ms5120, ms10240} } } -- ASN1STOP | 1 |  | Remove unnecessary spaces. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| 6.3.6 Other information elements |  |  |  |  |  |  |  |
| E.172 | 6.3.6 UE-EUTRA-Capability | The field description can be cleaned up a bit. discSysInfoReporting Indicates whether the UE supports reporting of System Information for inter-frequency/PLMN discovery - | 1 |  | Change the text to discSysInfoReporting Indicates whether the UE supports reporting of System Information for inter-frequency/PLMN sidelink discovery - Coordinator: Already corrected. Closed. |  | Closed (ProSe CR) |
| E.171 | 6.3.6 UE-EUTRA-Capability | Capabilities for sidelink Rel-13 are probably missing. They have not been discussed so it is hard to tell if anything is missing. Strictly speaking, as they have not been agreed, there are no more capabilities for SL than what is in there now. | 4 |  | The capabilities for sidelink Rel-13 must be discussed. Coordinator: These capabilities should probably be discussed and agreed during the RAN2#93 meeting. Coordinator: Capabilities will not be added in general CR. Closed. |  | Closed (General CR) (ASN.1) |
| D.038 | 6.3.6 UE-EUTRA-Capability | As for the convention of defining the capability signalling, RAN2 at the #81 meeting agreed as follows. 4 Remove the optional at group level for both optimisations (i.e. regardless of how few capabilities are contained) and modify rf-Parameters i.e. mandatory at group level and optional at capability level. Nevertheless, it was not followed from Rel-12 and onwards… DCM would like to understand if the above agreement in Rel-11 is no longer valid and the group level capability is defined as optional as done from Rel-12. | 3 |  | Coordinator: It is generally desirable to be consistent with earlier agreements. Maybe this should be changed from class 2 to class 3. Samsung: Agree to continue earlier agreement (good that someone remembers) Chair: => Not add the OPTIONAL at the group level if there is only one field in the definition of the lower level IE. If more than one field in the definition of the lower level IE, then include the OPTIONAL at the group level. => Capture this rule in the spec from R13. Coordinator: Can be captured in v13.2.0 CR. Closed. |  | Closed (General CR) (ASN.1) |
| E.209 | 6.3.6 UE-EUTRA-Capability | BandCombinationList-r13 is not variable size. | 2 |  | BandCombinationList-r13 should be variable size. BandCombination-r13 is not needed, the additionalFallbackSupported should be inserted into BandCombinationParametersCommon-r13 (which should be renamed to BandCombinationParameters-r13. BandCombinationList-r13 ::= SEQUENCE (SIZE (maxBandComb-r13)) OF BandCombination-r13 BandCombination-r13 ::= SEQUENCE { additionalFallbackSupported-r13 ENUMERATED {true} OPTIONAL, bandCombinationParameters-r13 BandCombinationParametersCommon-r13 } DCM: Proposal seems to be agreeable but need to look at the detail proposed changes. Should be discussed in ASN.1 review adhoc. Samsung: Detailed proposal would indeed be appreciated Chair: => Variable size list is covered by Z.082 => Ericsson to provide text proposal on second aspect. Coordinator: Band combinations are handled in a separate discussion/CR |  | Closed (General CR) (ASN.1) |
| H.081 | #### 6.3.6 UE-EUTRA-Capability | drb-TypeSCG, drb-TypeSplit The drb-TypeSplit indicates whether the UE supports split bearer except for PDCP data transfer in UL. The drb-TypeSCG indicates whether the UE supports SCG bearer. The above drb-TypeSCG, drb-TypeSplit is R12 IE, therefore it is better to show that drb-TypeSplit indicates whether the UE supports split bearer in DL. | 1 |  | drb-TypeSCG, drb-TypeSplit The drb-TypeSplit indicates whether the UE supports split bearer in DL except for PDCP data transfer in UL. The drb-TypeSCG indicates whether the UE supports SCG bearer. Maybe another change option can be (i.e., to cancle except for PDCP data transfer in UL) : drb-TypeSCG, drb-TypeSplit The drb-TypeSplit indicates whether the UE supports split bearer in DL. The drb-TypeSCG indicates whether the UE supports SCG bearer. Intel: Original text seems OK. No strong view. Ericsson: Agree, these capability bits could be having separate entries in the field description table as proposed by someone else above. DCM: In the initial draft CR, it was written as such. However, it was modified according to the comment received during the email discussion of [91bis#22] as shown below. “Are the statements of “The drb-TypeSplit indicates whether the UE supports split bearer in DL. “ and “The drb-TypeSplitUL indicates whether the UE supports split bearer in UL “ accurate? Split bearer will be configured for both DL and UL, while the difference is only that PDCP data transmission is split or not on UL.” From this history, DCM prefers to keep the current text unless there is a serious concern. Samsung: We agree with Intel and prefer to change to: indicates whether the UE supports split bearer for PDCP data transfer in DL, see S.105. Intel: May require further discussion, so class should be changed to 2. Therefore, the only change we can do is to separate the field descriptions what was done in I.053. Coordinator: Can be closed after RAN2#93. |  | Closed (General CR) |
| S.105 | 6.3.6 UE-EUTRA-Capability, drb-TypeSplit field description | Formulation need not refer to UL i.e. can just talk about split in DL | 1 |  | Rephrase (indicates whether the UE supports split bearer for PDCP data transfer in DL) Ericsson: Agree. DCM: Please refer our comment to H.081. Intel: May require further discussion, see H.081. Coordinator: Can be closed. |  | Closed (General CR) |
| Z.081 | 6.3.6 UE-EUTRA-Capability | The value of “supported” in skipFallbackCombinations is not clear. If the value is not required at all, we may set it as spare. Note that the presence of skipFallbackCombinations itself has already been enough to indicate whether UE supports reception of requestedMaximumCCsDL and requestedMaximumCCsUL. | 2 |  | The value of “supported” in skipFallbackCombinations may only be to indicate the support of reception of skipFallbackCombinations. We could clarify this in the field description: skipFallbackCombinations Value supported Indicates indicates whether UE supports receiving reception of skipFallbackCombinations that requests UE to exclude fallback band combinations from capability signalling. DCM: The value of “included” should also be explained if we do this. On the other hand, it may not be needed if it is clear from the procedure text. Intel: We think that both values “supported” and “included” should be clarified in the field description of skipFallbackCombinations. For the value “included” the relevant subclause should be added where it’s use is described. Chair: See meeting notes Coordinator: See conclusion of E.208. No changes. Closed. Intel: Tend to think that this issue can be closed after discussion and agreements made for E.208 in R2-160028, i.e. field skipFallbackCombinations contains only the value “supported” so that we can keep existing field description. |  | Closed (General CR) (ASN.1) |
| Z.082 | 6.3.6 UE-EUTRA-Capability | The list of BandCombinationList-r13 should be variable size. | 2 |  | Change: BandCombinationList-r13 ::= SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombination-r13 DCM: Agree with the proposal. Same with E.209 Chair => Agreed => Can be closed when included in general CR. Intel: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| I.052 | 6.3.6 UE-EUTRA-Capability | The order of the new capabilities in UE-EUTRA-Capability-v13x0-IEs should be aligned with the order in previous releases and TS 36.306. The suffix for rf-Parameters should be corrected to “-v13x0”. Regarding the new capability for SL, is there a need to add it as part of new sl-Parameters-r13 or should it be better added to the existing group sl-Parameters-r12 as Rel-13 NCE? | 2 |  | Change the order of the new capabilities as follows: pdcp-Parameters-v13x0 rlc-Parameters-v13x0 phyLayerParameters-v13x0 rf-Parameters-v13x0 ue-CategoryDL-v13x0 ue-CategoryUL-v13x0 measParameters-v13xy mac-Parameters-v13x0 dc-Parameters-v13x0 scptm-Parameters-v1300 sl-Parameters-r13 wlan-LWA-Parameters-r13 supportOfLAA-r13 UE-EUTRA-Capability-v13x0-IEs ::= SEQUENCE { dc-Parameters-v13x0 DC-Parameters-v13x0 OPTIONAL, measParameters-v13xy MeasParameters-v13xy OPTIONAL, supportOfLAA-r13 ENUMERATED {supported} OPTIONAL, mac-Parameters-v13x0 MAC-Parameters-v13x0 OPTIONAL, pdcp-Parameters-v13x0 PDCP-Parameters-v13x0 OPTIONAL, phyLayerParameters-v13x0 PhyLayerParameters-v13x0 OPTIONAL, rf-Parameters-v13x RF-Parameters-v13x0 OPTIONAL, rlc-Parameters-v13x0 RLC-Parameters-v13x0 OPTIONAL, ue-CategoryDL-v13x0 INTEGER (17) OPTIONAL, ue-CategoryUL-v13x0 INTEGER (14) OPTIONAL, scptm-Parameters-v1300 SCPTM-Parameters-v1300 OPTIONAL, sl-Parameters-r13 SL-Parameters-r13 OPTIONAL, wlan-LWA-Parameters-r13 WLAN-LWA-Parameters-r13 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } Ericsson: Regarding the capabilities for sidelink; We don’t think we have really discussed that. At the moment there is only one capability, but we suspect there might be an interest to have more. We would like to settle that discussion first before deciding how to encode it. See E.171. Chair: => Reorder fields to align with previous releases => sl-Parameters to be discussed at RAN2#93 => Define a group IE for LAA parameters. Intel: Corrected in general CR. |  | Closed (General CR) (ASN.1) |
| S.103 | UE-EUTRA-Capability, measParameters (SINR) | There is a double optional – not needed as there are no further measurement related capabilities | 2 |  | Remove one (lower one seems most commonly used candidate) DCM: There is also the other measurement related capability, i.e., whiteCellList. This is also related to how the capability signalling is defined as raised by D.038. Chair: See meeting notes Coordinator: See conclusion of D.038. Intel: Can be closed acc. to the agreement made for D.038, i.e. we keep OPTIONAL at group level if there is more than one field at lower level. |  | Closed (General CR) (ASN.1) |
| Z.080 | 6.3.6 UE-EUTRA-Capability | The value of “supported” in requestedMaximumCCs is not clear. If the value is not required at all, we may set it as spare. Note that the presence of requestedMaximumCCs itself has already been enough to indicate whether UE supports reception of requestedMaximumCCsDL and requestedMaximumCCsUL. | 2 |  | The value of “supported” in requestedMaximumCCs may only be to indicate the support of reception of requestedMaximumCCsDL and requestedMaximumCCsUL. We could clarify this in the field description: requestedMaximumCCs Value supported Indicates indicates whether UE supports reception of requestedMaximumCCsDL and requestedMaximumCCsUL. DCM: Similar to Z.080. It may not be needed if it is clear from the procedure text. Intel: Tend to think that this issue can be closed after discussion and agreements made for E.208 in R2-160028, i.e. field requestedMaximumCCs contains only the value “supported” so that we can keep existing field description. Coordinator: Same as Z.081. No changes needed. Closed. |  | Closed (General CR) |
| Z.083 | 6.3.6 UE-EUTRA-Capability | To align with the naming convention | 1 |  | Change: fdd-HarqHARQ-TimingTDD-r13 ENUMERATED {supported} OPTIONAL, Intel: Corrected in general CR. Made also the change in the corresponding field descripiton. |  | Closed (General CR) |
| Z.084 | 6.3.6 UE-EUTRA-Capability | Missing suffix | 1 |  | Change: RF-Parameters-v1300 ::= SEQUENCE { requestedMaximumCCs-r13 ENUMERATED {supported, includedUL, includedDL, includedBoth} OPTIONAL, skipFallbackCombinations-r13 ENUMERATED {supported, included} OPTIONAL, supportedBandCombinations-v13 SupportedBandCombinations-r13 OPTIONAL, supportedMIMO-CapabilityDL-r13 MIMO-CapabilityDL-r10 OPTIONAL } Intel: Already fixed in v13.0.0, so no further changes need to be made in general CR. |  | Closed (General CR) |
| I.050 | 6.3.6 UE-EUTRA-Capability | Just editorial: for “more than two but less than 6” in field description supportedCellGrouping, it might be better to align the usage of “two” and “6”. supportedCellGrouping This field indicates for which mapping of serving cells to cell groups (i.e. MCG or SCG) the UE supports asynchronous DC. This field is only present for a band combination with more than two but less than 6 band entries where the UE supports asynchronous DC. | 1 |  | Change “6” to “six”. Ericsson: OK. Intel: Corrected in general CR. |  | Closed (General CR) |
| I.051 | 6.3.6 UE-EUTRA-Capability | The introduction of "supportOfLAA-r13" in UE-EUTRA-Capability-v13x0-IEs is not aligned with the LAA 36.306 CR R2-156918 where the capability is grouped under LAA parameters. | 1 |  | Change the introduction of "supportOfLAA-r13" in UE-EUTRA-Capability-v13x0-IEs as follows: UE-EUTRA-Capability-v13x0-IEs ::= SEQUENCE { dc-Parameters-v13x0 DC-Parameters-v13x0 OPTIONAL, measParameters-v13xy MeasParameters-v13xy OPTIONAL, supportOfLAA-r13 ENUMERATED {supported} OPTIONAL, mac-Parameters-v13x0 MAC-Parameters-v13x0 OPTIONAL, pdcp-Parameters-v13x0 PDCP-Parameters-v13x0 OPTIONAL, phyLayerParameters-v13x0 PhyLayerParameters-v13x0 OPTIONAL, rf-Parameters-v13x RF-Parameters-v13x0 OPTIONAL, rlc-Parameters-v13x0 RLC-Parameters-v13x0 OPTIONAL, ue-CategoryDL-v13x0 INTEGER (17) OPTIONAL, ue-CategoryUL-v13x0 INTEGER (14) OPTIONAL, scptm-Parameters-v1300 SCPTM-Parameters-v1300 OPTIONAL, sl-Parameters-r13 SL-Parameters-r13 OPTIONAL, laa-Parameters-r13 LAA-Parameters-r13 OPTIONAL, wlan-LWA-Parameters-r13 WLAN-LWA-Parameters-r13 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } LAA-Parameters-r13 ::= SEQUENCE { supportOfLAA-r13 ENUMERATED {supported} OPTIONAL } Ericsson: OK. Intel: Corrected in general CR. |  | Closed (General CR) |
| I.053 | 6.3.6 UE-EUTRA-Capability | The field descriptions for drb-TypeSCG, drb-TypeSplit are combined. However, as they are two different capabilities it is better to separate the field descriptions. drb-TypeSCG, drb-TypeSplit The drb-TypeSplit indicates whether the UE supports split bearer except for PDCP data transfer in UL. The drb-TypeSCG indicates whether the UE supports SCG bearer. | 1 |  | Separate the descriptions for drb-TypeSCG, drb-TypeSplit. Intel: same as S.104 Ericsson: Agree. DCM: Fine to separate. Coordinator: The following corrections are proposed drb-TypeSCG, drb-TypeSplit The drb-TypeSplit indicates whether the UE supports split bearer except for PDCP data transfer in UL. The drb-TypeSCG indicates whether the UE supports SCG bearer. and drb-TypeSplit The drb-TypeSplit indicates whether the UE supports split bearer except for PDCP data transfer in UL. Intel: Corrected in general CR. |  | Closed (General CR) |
| D.037 | 6.3.6 UE-EUTRA-Capability | Suffix is not correct for some of capabilities in UE-EUTRA-Capability-v13x0-IEs and RF-Parameters-v13x0. | 1 |  | scptm-Parameters-v1300 should be scptm-Parameters-r13 as this is a new parameter. sl-Parameters-r13 should be sl-Paarameters-v13xy as there is an original field in Rel-12. rf-Parameters-v13x shold be rf-Parameters-v13xy requestedMaximumCCs and skipFallbackCombinations should be requestedMaximumCCs-r13 and skipFallbackCombinations-r13, respectively. Huawei: Agree. Intel: see also I.052 Cooordinator: All names with x and y in the suffix are temporary names and they removed during CR implementation. There are reasons to correct temporary names. Intel: The suffices to scptm-Parameters-v13x0 and sl-Parameters-r13 were corrected in general CR. Other issues were already fixed in v13.0.0. |  | Closed (General CR) |
| D.040 | 6.3.6 UE-EUTRA-Capability | supportedMIMO-CapabilityDL-r13 is included in RF-Parameters-v13x0. However, it does not have to be included here. | 1 |  | Remove supportedMIMO-CapabilityDL-r13 from RF-Parameters-v13x0. Intel: Corrected in general CR, see N.170. |  | Closed (General CR) |
| D.041 | 6.3.6 UE-EUTRA-Capability | A redundant space. whiteCellList Indicates whether the UE supports whiteCellList-r13 to limit the set of cells applicable for measurements. | 1 |  | whiteCellList Indicates whether the UE supports whiteCellList-r13 to limit the set of cells applicable for measurements. Samsung: Fine (but suffix –r13 seems not needed in field description – acktually it is better to refer to functionality i.e. ‘EUTRA white cell listing’ rather than to this field as there is also useWhiteCellList) Intel: Corrected in general CR. |  | Closed (General CR) |
| D.042 | 6.3.6 UE-EUTRA-Capability | Is the “SEQUENCE OF” needed (and correct)? BandParametersUL-r13 ::= SEQUENCE OF CA-MIMO-ParametersUL-r10 BandParametersDL-r13 ::= SEQUENCE OF CA-MIMO-ParametersDL-r13 DCM understands that this change was due to the recent agreement that UE shall indicate parameters for only one CA uplink or downlink bandwidth class in a single band entry for one band combination entry. In that sense, SEQUENCE is not needed. | 1 |  | Should be corrected as follows. BandParametersUL-r13 ::= SEQUENCE OF CA-MIMO-ParametersUL-r10 BandParametersDL-r13 ::= SEQUENCE OF CA-MIMO-ParametersDL-r13 Coordinator: Corrected in general CR. |  | Closed (General CR) |
| H.082 | 6.3.6 UE-EUTRA-Capability | ue-SSTD-Meas Indicates whether the UE supports SSTD measurements between the PCell and the PSCell as specified in TS 36.214 [48]. “subframeBoundaryOffsetResult” should be according to the mapping table in TS 36.133 [16]. So one reference is absent. | 1 |  | ue-SSTD-Meas Indicates whether the UE supports SSTD measurements between the PCell and the PSCell as specified in TS 36.214 [48] and TS 36.133 [16]. Intel: Ericsson: OK. DCM: Could be OK. But if 36.133 is added, the field description should mention about the reporting, e.g., “UE suppports SSTD measurements and reporting between…” Intel: Corrected in general CR by only adding the reference to TS 36.133. We don’t see the need to add “reporting” in the field description of the capability. |  | Closed (General CR) |
| I.054 | 6.3.6 UE-EUTRA-Capability | Field description of extendedLongDRX can be improved. | 1 |  | Improve field description of extendedLongDRX as per below:. extendedLongDRX Indicates whether the UE supporstsupports extended long DRX cycles values of 5.12s and 10.24s in connected modeRRC_CONNECTED. DCM: Agree. Coordinator: See also E.214 Intel: Corrected in general CR. |  | Closed (General CR) |
| Z.085 | 6.3.6 | In the IE “UE-EUTRA-Capability”, “supportOfLAA-r13” is introduced as per UE capability, namely agnostic to CA band combinations. We are wondering whether this is sufficient for LAA, and wanna RAN4 further studies. | 2 |  | UE-EUTRA-Capability-v13x0-IEs ::= SEQUENCE { dc-Parameters-v13x0 DC-Parameters-v13x0 OPTIONAL, measParameters-v13xy MeasParameters-v13xy OPTIONAL, supportOfLAA-r13 ENUMERATED {supported} OPTIONAL, mac-Parameters-v13x0 MAC-Parameters-v13x0 OPTIONAL, pdcp-Parameters-v13x0 PDCP-Parameters-v13x0 OPTIONAL, phyLayerParameters-v13x0 PhyLayerParameters-v13x0 OPTIONAL, rf-Parameters-v13x RF-Parameters-v13x0 OPTIONAL, rlc-Parameters-v13x0 RLC-Parameters-v13x0 OPTIONAL, ue-CategoryDL-v13x0 INTEGER (17) OPTIONAL, ue-CategoryUL-v13x0 INTEGER (14) OPTIONAL, scptm-Parameters-v1300 SCPTM-Parameters-v1300 OPTIONAL, sl-Parameters-r13 SL-Parameters-r13 OPTIONAL, wlan-LWA-Parameters-r13 WLAN-LWA-Parameters-r13 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } Ericsson: We believe it is sufficient to have this capability bit in combination with the supported band combinations. I.e. a UE setting this bit and indicates support of LAA frequencies support LAA. Nothing more is needed. This has already been discussed and we prefer to keep it like this. DCM: Agree with Ericsson. It could be specified as a new parameter of LAA parameter as suggested by D.039 and I.051 Chair: => No change needed. |  | Closed |
| D.039 | 6.3.6 UE-EUTRA-Capability | supportOfLAA-r13 is not categorised into the existing group. The supported LAA feaure is somewhat variant and so it would not fit into the exiting category. Nevertheless, DCM thinks that LAA specific grouping is beneficial even from Rel-13 given that enhanced LAA feaures are likely to be added in the later release. | 2 |  | To introduce a LAA specific grouping, e.g., laa-Parameters-r13. supportOfLAA-r13 is introduced as a field in laa-Parameters-r13. Ericsson: Agree (this is the same as I.051) Chair: See I.051 |  | Closed |
| E.214 | 6.3.6 UE-EUTRA-Capability | Field description extendedLongDRX | 1 |  | Change to RRC_CONNECTED. Change to “support”(misspelled) extendedLongDRX Indicates whether the UE supporst extended long DRX cycles of 5.12s and 10.24s in connected mode Coordinator: Same as I.054. |  | Closed |
| E.324 | 6.3.6 UE-EUTRA-Capability | The following notation needs to be checked: MAC-Parameters-r12 ::= SEQUENCE { logicalChannelSR-ProhibitTimer-r12 ENUMERATED {supported} OPTIONAL, longDRX-Command-r12 ENUMERATED {supported} OPTIONAL } MAC-Parameters-v13x0 ::= SEQUENCE { extendedMAC-LengthField-r13 ENUMERATED {supported} OPTIONAL, extendedLongDRX-r13 ENUMERATED {supported} OPTIONAL } | 1 |  | Coordinator: It is a notation for temporary identifier names. The final name is inserted during CR implementation. Closed without actions. |  | Closed |
| E.325 | 6.3.6 UE-EUTRA-Capability | The field description of additionalFallbackSupported is supposed to be after the description for accessStratumRelease.” | 1 |  | Update the order. Coordinator: Corrected during v1300 CR implementation |  | Closed |
| S.104 | 6.3.6 UE-EUTRA-Capability, field descriptions | Strange to combine field descriptions of drb-TypeSCG and drb-TypeSplit | 1 |  | Separate Intel: same as I.053. Ericsson: Agree. Coordinator: Same as I.053 |  | Closed |
| 6.3.7 MBMS information elements |  |  |  |  |  |  |  |
| Z.092 | 6.3.7a SC-MTCH-InfoList | The size of the bit string sc-mtch-NeighbourCell-r13 should be variable from 1 to maxNeighCellSCPTM-r13. Otherwise the field is absent. | 2 |  |  | sc-mtch-nNeighbourCell-r13 BIT STRING (SIZE(1..maxNeighCellSCPTM-r13)) OPTIONAL, Huawei: Prefer fixed size because if there are many services, it seems likely that there are 5 or more neighbours then it is better to not waste 3 bits/service because of variable size. Chair => No change needed | Closed |
| 6.3.8 ProSe information elements |  |  |  |  |  |  |  |
| N.175 | 6.3.8 SL-DiscConfig:: SL-DiscTxRefCarrierDedicated-r13 | Does not take into account the CA extensions in Rel-13 since only Rel-10 SCell indexes are allowed. | 4 |  |  | Discuss how to fix: One choice would be to use ServCellIndexFull without the choice, where 0 refers to PCell. Or then use SCellIndex-r13 Ericsson: This is a very interesting problem. We should look closer at this. Nokia Networks: Will submit a paper to AH or February meeting. Qualcomm: Ok Coordinator: Can be closed. | Closed (ProSe CR) (ASN.1), TDoc Nokia RAN2#93 |
| L.050 | 6.3.8 SL-DiscConfig | Better to place the yellow condition in the procedural text as normally we specify timer handling in procedural text. discSysInfoReportConfig Indicates the request to start a SidelinkUEInformation procedure for reporting system information acquired during an inter-frequency discovery procedure. When receving a discSysInfoReportConfig setup, the UE shall start timer T3xx with the timer value set to [60] seconds. When receiving a discSysInfoReportConfig release, the UE shall stop timer T3xx. |  |  |  | If agree to move them to procedural part, we wil propose text for the procedural text Ericsson: We have no strong opinion. CATT: should introduce procedural text. Qualcomm: Wait for LG’s proposal Coordinator: It would be useful to see a proposal what precedure is impacted and where this text should be move. Otherwise it is difficult to make this correction. Samsung: Agree this should normally be in procedural specification Chair: => Move to the procedure text. => LG will provide a text proposal by email to show how this can be done in he procedure text. Samsung: Section 5.3.10.15 seems appropriate place but is so far quite general. Awaiting TP Coordinator: Can be closed. | Closed (ProSe CR) (ASN.1) |
| Q.034 | 6.3.8 SL-SysInfoReport | #### Inside it is incorrect to just provide discSyncConfig-r13. Actually whatever UE reads from SIB19 of other frequency cell with respect to Synch config that should be provided. It should be noted that cell in other frequency can be Rel-12 or Rel-13. So following should be replaced. discSyncConfig-r13 SEQUENCE { asyncParameters-r13 SEQUENCE { syncCP-Len-r13 SL-CP-Len-r12, syncOffsetIndicator-r13 SL-OffsetIndicatorSync-r12, slssid-r13 SLSSID-r12 } OPTIONAL, txParameters-r13 SEQUENCE { syncTxParameters-r13 SL-TxParameters-r12, syncTxThreshIC-r13 RSRP-RangeSL-r12, syncInfoReserved-r13 BIT STRING (SIZE (19)) OPTIONAL, syncTxPeriodic-r13 ENUMERATED {true} OPTIONAL } OPTIONAL } OPTIONAL, | 4 |  |  | The mentioned IE can be replaced by : SL-SyncConfigList-r12 ::= SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfig-r12 SL-SyncConfig-r12 ::= SEQUENCE { syncCP-Len-r12 SL-CP-Len-r12, syncOffsetIndicator-r12 SL-OffsetIndicatorSync-r12, slssid-r12 SLSSID-r12, txParameters-r12 SEQUENCE { syncTxParameters-r12 SL-TxParameters-r12, syncTxThreshIC-r12 RSRP-RangeSL-r12, syncInfoReserved-r12 BIT STRING (SIZE (19)) OPTIONAL -- Need OR } OPTIONAL, -- Need OR rxParamsNCell-r12 SEQUENCE { physCellId-r12 PhysCellId, discSyncWindow-r12 ENUMERATED {w1, w2} } OPTIONAL, -- Need OR ..., [[ syncTxPeriodic-r13 ENUMERATED {true} OPTIONAL -- Need OR ]] } SL-SyncConfigListNFreq-r12 ::= SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfigNFreq-r13 SL-SyncConfigNFreq-r13 ::= SEQUENCE { asyncParameters-r13 SEQUENCE { syncCP-Len-r13 SL-CP-Len-r12, syncOffsetIndicator-r13 SL-OffsetIndicatorSync-r12, slssid-r13 SLSSID-r12 } OPTIONAL, -- Need OR txParameters-r13 SEQUENCE { syncTxParameters-r13 SL-TxParameters-r12, syncTxThreshIC-r13 RSRP-RangeSL-r12, syncInfoReserved-r13 BIT STRING (SIZE (19)) OPTIONAL, -- Need OR syncTxPeriodic-r13 ENUMERATED {true} OPTIONAL -- Need OR } OPTIONAL, -- Need OR rxParameters-r1s SEQUENCE { discSyncWindow-r13 ENUMERATED {w1, w2} } OPTIONAL, -- Need OR ... } Coordinator: First of all, discSyncConfig-r13 is a field, i.e. it is not an information element. So how message content could it be replaced with a type definition? It is dificult to see what is changed in the ASN.1. It is also hard to understand if the intention is to changed also REL-12 syntax. Qualcomm will look into if this is a problem or not. Samsung: Further discussion seems required and a more detailed proposal would be appreciated. Chair: => To be discussed at RAN2#93 Coordinator: | Closed (ProSe CR) (ASN.1) |
| N.178 | 6.3.8 SL-DiscResourcePool:: txParamsAddNeighFreq-r13 | Field can never be released once configured | 2 |  |  | Add possibility of release. Coordinator: This is a fair comment but how would you like to release the field? Do you want a release/setup choice ro to change the need code to Need OR, or something else? This needs to be clarified. Otherwise it is not possible to make this correction. Samsung: No real concerns, but wondering for which scenario this is required Samsung: Included in updated CR | Closed (ProSe CR) (ASN.1) |
| N.177 | 6.3.8 SL-DiscResourcePool:: rxParamsAddNeighFreq-r13 | Field can never be released once configured | 2 |  |  | Add possibility of release. Coordinator: This is a fair comment but how would you like to release the field? See also N.178. Samsung: No real concerns, but wondering for which scenario this is required Samsung: Included in updated CR | Closed (ProSe CR) (ASN.1) |
| L.037 | 6.3.8 SL-CommConfig | Need to add description on the order of the LCG ID for which associated priorities are configured. logicalChGroupInfoList Indicates for each logical channel group the list of associated priorities, used as specified in TS 36.321 [6]. | 3 |  |  | Not yet suggest any specific proposals (checking 36.321)… Ericsson: We suggest to associate it to the destination list, i.e the same order. Would that work? CATT: seems not necessary. Coordinator: It would be desirable to present some suggestions (and preferably in writing) because otherwise it is difficult to make this correction. Qualcomm: Agree that order in LogicalChGroupInfoList corrsponds to LCG ID. Chair: => Check if required after looking at Qcom CR in R2-160049 | Closed (ProSe CR) (ASN.1) |
| L.042 | 6.3.8 SL-DiscConfig | Wonder if this field is needed, i.e. not redundant, because TX carrier info is already in the entry of the discTxInfoInterFreqListExt? discTxInterFreqInfo-r13 CHOICE { release NULL, setup SEQUENCE { discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL -- Need OR discTxRefCarrierDedicated-r13 SL-DiscTxRefCarrierDedicated-r13 OPTIONAL -- Need OR discTxInfoInterFreqListExt-r13 SL-DiscTxInfoInterFreqListExt-r13 OPTIONAL -- Need ON } } OPTIONAL, -- Need ON | 3 |  |  | discTxInterFreqInfo-r13 CHOICE { release NULL, setup SEQUENCE { discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL -- Need OR discTxRefCarrierDedicated-r13 SL-DiscTxRefCarrierDedicated-r13 OPTIONAL -- Need OR discTxInfoInterFreqListExt-r13 SL-DiscTxInfoInterFreqListExt-r13 OPTIONAL -- Need ON } } OPTIONAL, -- Need ON Ericsson: There seems to be several open issues related to how and which frequencies can be configured for discovery. Perhaps we could use a clarifying discussion and good contributions to settle this? Qualcomm: Agree that discTxCarrierFreq-r13 is also present inside discTxInfoInterFreqListExt-r13. Hence redundant here and can be removed. Samsung: Intend to provide some discussion/ clarification Intel: Agree that discTxCarrierFreq-r13 can be removed Chair: See meeting notes (R2-160050) Samsung: Covered by agreements on R2-160045 | Closed (ProSe CR) (ASN.1) |
| L.043 | 6.3.8 SL-DiscConfig | Wonder if this field is needed, i.e. not redundant, as discTxRefCarrierDedicated is anyway included in discTxInfoInterFreqListExt? discTxInterFreqInfo-r13 CHOICE { release NULL, setup SEQUENCE { discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL -- Need OR discTxRefCarrierDedicated-r13 SL-DiscTxRefCarrierDedicated-r13 OPTIONAL -- Need OR discTxInfoInterFreqListExt-r13 SL-DiscTxInfoInterFreqListExt-r13 OPTIONAL -- Need ON } } OPTIONAL, -- Need ON | 3 |  |  | discTxInterFreqInfo-r13 CHOICE { release NULL, setup SEQUENCE { discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL -- Need OR discTxRefCarrierDedicated-r13 SL-DiscTxRefCarrierDedicated-r13 OPTIONAL -- Need OR discTxInfoInterFreqListExt-r13 SL-DiscTxInfoInterFreqListExt-r13 OPTIONAL -- Need ON } } OPTIONAL, -- Need ON Ericsson: There seems to be several open issues related to how and which frequencies can be configured for discovery. Perhaps we could use a clarifying discussion and good contributions to settle this? Qualcomm: Agree that discTxRefCarrierDedicated-r13 is also present inside discTxInfoInterFreqListExt-r13. Hence redundant here and can be removed. Intel: Agree that discTxRefCarrierDedicated-r13 can be removed Chair: See meeting notes (R2-160050) Samsung: See L.042 | Closed (ProSe CR) (ASN.1) |
| L.044 | 6.3.8 SL-DiscConfig | Better to have procedural text to handle the list below: SL-DiscTxInfoInterFreqListExt-r13 ::= SEQUENCE { discTxFreqToAddModList-r13 SEQUENCE (SIZE {1..maxFreq)) OF SL-DiscTxResourceInfoPerFreq-r13 OPTIONAL, -- Need ON discTxFreqToReleaseList-r13 SEQUENCE (SIZE {1..maxFreq)) OF ARFCN-ValueEUTRA-r9 OPTIONAL -- Need ON } | 3 |  |  | Will propose text proposal if agreed to introduce some procedural text. Ericsson: We look forward to a proposal. CATT: should introduce procedural text Qualcomm: What is the proposal? Coordinator: Maybe it is more successful to do the other way around, i.e. first present a text proposal and then discuss the correction. Chair: => LG wil distribute a text proposa by email for discussion as part of ASN.1 review. Samsung: Section 5.3.10.15 seems appropriate place but is so far quite general. No details are provided for the similar fields in discTxResources for the case of scheduled resources (i.e. poolToAddModList-r12) so there does not seem a real need. Hence no change done | Closed (ProSe CR) (ASN.1) |
| L.046 | 6.3.8 SL-DiscConfig | Can we simply refer to the IE SL-DiscTxConfigScheduled-r13 for this field, thus not listing the yellow fields? (related to optionality of discHoppingConfig in DiscTxConfigScheduled-r13) SL-DiscTxResource-r13 ::= CHOICE { release NULL, setup CHOICE { scheduled-r13 SEQUENCE { discTxConfig-r13 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r13 SL-TF-IndexPairList-r12b OPTIONAL, -- Need ON discHoppingConfig-r13 SL-HoppingConfigDisc-r12 OPTIONAL -- Need ON }, ue-Selected-r13 SEQUENCE { discTxPoolDedicated-r13 SEQUENCE { poolToReleaseList-r13 SL-TxPoolToReleaseList-r12 OPTIONAL, -- Need ON poolToAddModList-r13 SL-DiscTxPoolToAddModList-r12 OPTIONAL -- Need ON } OPTIONAL -- Need ON } | 3 |  |  | Ericsson: We are not sure what the proposal is. Maybe LG can clarify? Coordinator: This solution proposal should be clarified because based on this text it is difficult to understand what exactly needs to be corrected. Qualcomm: Current form is also right. However Agree that highlighted portion in LG proposal can be replaced by DiscTxConfigScheduled-r13. One more thing inside DiscTxConfigScheduled-r13 we must ensure that we use TF-IndexPairList-r12b instead of TF-IndexPairList-r12 Samsung: Understand that IE SL-DiscTxConfigScheduled-r13 covers the same fields, in which case it seems indeed be good to use (assuming we agree that hpping config should be optional in that IE). Note however that there are also differences regarding SL-TF-IndexPairList i.e. –r12 vs –r12b that requires some checking - -r12b is the corrected version that should be used from now on (i.e. seems to require change of IE SL-DiscTxConfigScheduled-r13). Chair: => Agree to refer to SL-DiscTxConfigScheduled-r13 instead of yellow fields. => Can be closed when included in the ProSe CR | Closed (ProSe CR) (ASN.1) |
| L.047 | 6.3.8 SL-DiscConfig | Can we simply refer to the IE SL-DiscTxPoolDedicated-r13thus not listing the yellow fields? SL-DiscTxResource-r13 ::= CHOICE { release NULL, setup CHOICE { scheduled-r13 SEQUENCE { discTxConfig-r13 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r13 SL-TF-IndexPairList-r12b OPTIONAL, -- Need ON discHoppingConfig-r13 SL-HoppingConfigDisc-r12 OPTIONAL -- Need ON }, ue-Selected-r13 SEQUENCE { discTxPoolDedicated-r13 SEQUENCE { poolToReleaseList-r13 SL-TxPoolToReleaseList-r12 OPTIONAL, -- Need ON poolToAddModList-r13 SL-DiscTxPoolToAddModList-r12 OPTIONAL -- Need ON } OPTIONAL -- Need ON } | 3 |  |  | Ericsson: We are not sure what the proposal is. Maybe LG can clarify? Qualcomm: Ok to keep current form. Coordinator: This solution proposal should be clarified because based on this text it is difficult to understand what exactly needs to be corrected. Samsung: Agree to re-use the IE (note that suffixes of fields contain errors i.e. should be –r12, also in the IE) Chair => Agree to refer to IE SL-DiscTxPoolDedicated-r13 instead of yellow fields. => Can be closed when included in the ProSe CR Samsung:: seems no need to still have an optinal field discTxPoolDedicated-r13. Hence choice value ue-Selected was changed to directly refer to IE SL-DiscTxPoolDedicated-r13 (similar as for choice scheduled, see L.046) | Closed (ProSe CR) (ASN.1) |
| L.049 | 6.3.8 SL-DiscConfig | Wonder why this field is not OPTIONAL, NEED ON like other occurrences for the same fields? SL-DiscTxConfigScheduled-r13 ::= SEQUENCE { discTxConfig-r12 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r12 SL-TF-IndexPairList-r12 OPTIONAL, -- Need ON discHoppingConfig-r12 SL-HoppingConfigDisc-r12 | 3 |  |  | SL-DiscTxConfigScheduled-r13 ::= SEQUENCE { discTxConfig-r12 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r12 SL-TF-IndexPairList-r12 OPTIONAL, -- Need ON discHoppingConfig-r12 SL-HoppingConfigDisc-r12 OPTIONAL -- Need ON Ericsson: We think we can make this field optional. Qualcomm: Agree Samsung: Agree. Some more changes seem needed i.e. discTF-IndexList should refer to –r12b version (see L.046) Chair: => Agreed => Can be closed when included in ProSe CR | Closed (ProSe CR) (ASN.1) |
| L.045 | 6.3.8 SL-DiscConfig | Need to add field description that would be the same as the one in SIB19? SL-DiscTxResourceInfoPerFreq-r13 ::= SEQUENCE { discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9, discTxResources-r13 SL-DiscTxResource-r13 OPTIONAL, -- Need OR discTxResourcesPS-r13 SL-DiscTxResource-r13 OPTIONAL, -- Need OR discTxRefCarrierDedicated-r13 SL-DiscTxRefCarrierDedicated-r13 OPTIONAL, -- Need OR cellSelectionInfo-r13 CellSelectionInfoNFreq-r13 OPTIONAL -- Need OR } | 2 |  |  | Introduce the field description that is the same as in the SIB19. Ericsson: We think a field description can be beneficial. Qualcomm: Ok Coordinator: Please present a text proposal. Otherwise it is difficult to make this correction. Chair: => To be discussed at RAN2#93 as potentially impacts 36.304 as well. Samsung: There seems no real issue i.e. there already was a description for the corresponding field in SIB19, which was simply copied. | Closed (ProSe CR) (ASN.1) |
| L.048 | 6.3.8 SL-DiscConfig | Need to check whethe it should be SL-TF-IndexPairList-r12b. SL-DiscTxConfigScheduled-r13 ::= SEQUENCE { discTxConfig-r12 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r12 SL-TF-IndexPairList-r12 OPTIONAL, -- Need ON discHoppingConfig-r12 SL-HoppingConfigDisc-r12 | 2 |  |  | SL-DiscTxConfigScheduled-r13 ::= SEQUENCE { discTxConfig-r12 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r12 SL-TF-IndexPairList-r12b OPTIONAL, -- Need ON discHoppingConfig-r12 SL-HoppingConfigDisc-r12 Ericsson: It must be “-r12b” as “-r12” is flawed. Maybe that can be added somewhere, that “-r12” is not to be used? CATT: should be r12b. Qualcomm: Agree that it should be r12b. Coordinator: See also Q.023. Chair: => Agree with proposal. => Can be closed when included in ProSe CR | Closed (ProSe CR) (ASN.1) |
| E.180 | 6.3.8 SL-GapConfig | Field descriptions for gapPeriod and gapOffset are missing. | 2 |  |  | Add descriptions, but this should probably be discussed first. - Is the length of the subframe bitmap same as the gap period? - How is the offset calculated (e.g. to SFN 0 of the PCell)? Qualcomm: Ok to add field description. However it is clear from field description of gapSubframeBitmap that size is upto 10*gapPeriod. Offset should be from SFN 0 of Pcell. Coordinator: A text proposal is needed. Ericsson will think about this and try to come up with something for the ASN.1 review meeting. Chair: See meeting notes (R2-160050) Samsung: Covered by R2-160017 | Closed (ProSe CR) (ASN.1) |
| Z.095 | 6.3.8 SL-SyncConfig SL-SyncConfigListNFreq-r12, SystemInformationBlockType19, SL-SyncConfigListNFreq-r12 | This IE” SyncConfigListNFreq-r12” is only used in Rel-13 IE, and never used in Rel-12 before. | 2 |  |  | Change the IE “SL-SyncConfigListNFreq-r12” into “SL-SyncConfigListNFreq-r13”, the relevant modification is in “SystemInformationBlockType19” and “SL-SyncConfig”. Ericsson: Agree. And same as N192. Qualcomm: Agree Coordinator: It seems that the following correction is proposed, SL-SyncConfigListNFreq-r12r13 ::= SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfigNFreq-r13 SL-CarrierFreqInfo-v13x0::= SEQUENCE { discRxResources-r13 SL-DiscRxPoolList-r12 OPTIONAL, -- Need OR discRxResourcesPS-r13 SL-DiscRxPoolList-r12 OPTIONAL, -- Need OR discTxResources-r13 SL-DiscTxResourcesInterFreq-r13 OPTIONAL, -- Need OR discTxResourcesPS-r13 SL-DiscTxResourcesInterFreq-r13 OPTIONAL, -- Need OR discTxPowerInfo-r13 SL-DiscTxPowerInfoList-r12 OPTIONAL, -- Cond Tx discTxRefCarrierCommon-r13 ENUMERATED {pCell} OPTIONAL, -- Need OR discSyncConfig-r13 SL-SyncConfigListNFreq- r12r13 OPTIONAL, -- Need OR cellSelectionInfo-r13 CellSelectionInfoNFreq-r13 OPTIONAL -- Need OR } This needs to be confirmed. Chair: => Agreed => Can be closed when included in the ProSe CR. | Closed (ProSe CR) (ASN.1) |
| Q.033 | 6.3.8 SL-SysInfoReport | #### SL-SysInfoReport Second q-RxLevMin-r13 used for reselection should be renamed to q-RxLevMinReselection-r13 which is similar to what we have used in CellSelectionInfoNFreq-r13. Also add -- Cell selection information as in SIB1 And -- Cell re-selection information as in SIB3 | 2 |  |  | cellSelectionInfo-r13 SEQUENCE { -- Cell selection information as in SIB1 q-RxLevMin-r13 Q-RxLevMin, q-RxLevMinOffset-r13 INTEGER (1..8) OPTIONAL } OPTIONAL, -- Cell re-selection information as in SIB3 q-Hyst-r13 ENUMERATED { dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10, dB12, dB14, dB16, dB18, dB20, dB22, dB24} OPTIONAL, q-RxLevMinReselection-r13 q-RxLevMin-r13 Q-RxLevMin OPTIONAL, t-ReselectionEUTRA-r13 T-Reselection OPTIONAL, tdd-Config-r13 TDD-Config OPTIONAL, freqInfo-r13 SEQUENCE { ul-CarrierFreq-r13 ARFCN-ValueEUTRA OPTIONAL, ul-Bandwidth-r13 ENUMERATED {n6, n15, n25, n50, n75, n100} OPTIONAL, additionalSpectrumEmission-r13 AdditionalSpectrumEmission OPTIONAL } OPTIONAL, Coordinator: It seems that the following correction is proposed cellSelectionInfo-r13 SEQUENCE { q-RxLevMin-r13 Q-RxLevMin, q-RxLevMinOffset-r13 INTEGER (1..8) OPTIONAL } OPTIONAL, q-Hyst-r13 ENUMERATED { dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10, dB12, dB14, dB16, dB18, dB20, dB22, dB24} OPTIONAL, q-RxLevMinReselection-r13 q-RxLevMin-r13 Q-RxLevMin OPTIONAL, t-ReselectionEUTRA-r13 T-Reselection OPTIONAL, tdd-Config-r13 TDD-Config OPTIONAL, freqInfo-r13 SEQUENCE { ul-CarrierFreq-r13 ARFCN-ValueEUTRA OPTIONAL, ul-Bandwidth-r13 ENUMERATED {n6, n15, n25, n50, n75, n100} OPTIONAL, additionalSpectrumEmission-r13 AdditionalSpectrumEmission OPTIONAL } OPTIONAL, Samsung: May be clearer to have a sequency for the cellReselectionInfo (as for cellSelectionInfo) Samsung: Introduced optonal sequence for reselection info, changing all fields to mandatory (as always present in SIB3) | Closed (ProSe CR) (ASN.1) |
| L.039 | 6.3.8 SL-CommConfig | Not clear which value indicates support: multipleTxAllowed Indicates whether or not multiple transmissions in one SC period is allowed. | 3 |  |  | Option1) Suggest to explicitly indicate in the FD which value (e.g.TRUE) correponds to that the multiple TX is allowed. multipleTxAllowed Indicates whether or not multiple transmissions in one SC period is allowed. Value ‘true” indicate that multiple transmission is allowed. Option2: we could have this field as ENUMERATE with "allowed, notAllowed" Ericsson: As we suggest a name change in L.038, if we go for a name like multipleTxConfigured, then it becomes apparent that “true” means it is configured and “false” means it is not. Qualcomm: It is boolean so true means allowed and false means not allowed. However fine to clarify this in field description. Coordinator: No opinion on the different options but clearly both cannot be used. So one of them should be selected. Are there any preferences? Samsung: Fine to add the proposed clarification, could be done by modifying existing sentence. Seems minor issue i.e. class 1/2. Ericsson suggestion should be discussed separately Samsung: Clarification added that TRUE means allowed. Did not change field name to ‘configured’ as this seems to imply UE is required to perform multiple transmissions, which also requires eNB does to know UE support. Field name and description was further changed i.e. removal of notion of allowed but whith ref to MAC specification for detailed requirements | Closed (ProSe CR) |
| N.179 | 6.3.8 SL-GapConfig::gapPeriod-r13 | Discuss spare handling. See N.004. | 1 |  |  | Define according to resolution of N.004. Samsung: All spares in dedicated signalling are to be removed. General review of all cases seems desirable (including which ones may be added on broadcast) | Closed (ProSe CR) |
| L.052 | 6.3.8 SL-SysInfoReport | Enhancement to IE description | 2 |  |  | The IE SL-SysInfoReport contains the parameters related to sSidelink operations acquired from system information of inter-frequency cells Ericsson: Agree. Qualcomm: Ok | Closed (ProSe CR) |
| Q.030 | 6.3.8 | Inside txParamsAddNeighFreq-r13, physCellId-r13 should be made list of cell IDs as it is possible that same Tx resource pool is used in all neighboring cells. As captured in Chair’s notes (RAN2#92): - The cell id of the cells associated with a transmission configuration should also be provided when the serving cell configures resources via dedicated signaling | 4 |  |  | Qualcomm: Changed the category from 1 to 2. As it is important change for ASN.1. Samsung: Considered to be outside scope of ASN.1 review i.e. requires separate paper for RAN2#93 | Closed (ProSe CR) |
| Z.097 | 6.3.8 | discTxRefCarrierDedicated Indicates if the PCell or an SCell is to be used as reference DL measurements and synchronization, instead of the frequency used to transmit sidelink discovery announcements on, see TS 36.213 [23, 14.3.1]. The description “instead of the frequency used to transmit sidelink discovery announcements on” is not accurate, since the frequency which could be used for reference should be the DL frequency paired with the frequency used to transmit sidelink discovery announcements on. | 2 |  |  | The sentence “instead of the frequency used to transmit sidelink discovery announcements on” need to be replaced by “instead of the DL frequency paired with the one used to transmit sidelink discovery announcements on” Ericsson: Agree. Qualcomm: Agree Coordinator: The following correction is proposed, discTxRefCarrierDedicated Indicates if the PCell or an SCell is to be used as reference DL measurements and synchronization, instead of the DL frequency paired with the one used to transmit sidelink discovery announcements on instead of the frequency used to transmit sidelink discovery announcements on, see TS 36.213 [23, 14.3.1]. Samsung: Corrected | Closed (ProSe CR) |
| Z.099 | 6.3.8 | syncTxThreshIC Indicates the threshold used while in coverage. In case the RSRP measurement of the cell chosen for transmission of sidelink communication/ discovery announcements, is below the level indicated by this field, the UE may transmit SLSS (i.e. become synchronisation reference) when performing the corresponding sidelink transmission.. The description “In case the RSRP measurement of the cell chosen for transmission of sidelink communication/ discovery announcements,” is not correct, since another cell other than the cell chosen for transmission of sidelink discovery announcements could be configured for DL measurement in the inter-frequency discovery scenario. | 2 |  |  | The sentence “In case the RSRP measurement of the cell chosen for transmission of sidelink communication/ discovery announcements,” needs to be replaced by “In case the RSRP measurement of the cell chosen for transmission of sidelink communication or the RSRP measurement of the reference cell used for transmission of sidelink discovery announcements, ” Ericsson: Agree. Qualcomm: Agree Coordinator: The following is proposed In case the RSRP measurement of the cell chosen for transmission of sidelink communication or the RSRP measurement of the reference cell used for transmission of sidelink discovery announcements or the RSRP measurement of the reference cell used for transmission of sidelink discovery announcements syncTxThreshIC is a REL-12 field. It is surely possible to add some new text but removal of legacy text is something that should be addressed with a separate CR Samsung: Agree, and as this does not affect REL-12 operation/ specs, we see no real need for a separate CR Samsung: Changed | Closed (ProSe CR) |
| Z.098 | 6.3.8 | syncTxPeriodic Indicates whether in each discovery period in which UE transmits discovery, the UE transmits SLSS once or perodically (i.e. every 40ms). There is an wording error in the word “perodically”. | 1 |  |  | The word “perodically” need to be replaced by “periodically” | Closed (ProSe CR) |
| Q.021 | 6.3.8 | Field description of “commTxAllowRelayDedicated” is confusing as it is written in negative format. | 1 |  |  | Field Description can be modified to: commTxAllowRelayDedicated Indicates for whether the UE is not allowed to transmit relay related data using the configured dedicated transmission resources i.e. neither via scheduled nor via UE selected resources. | Closed (ProSe CR) |
| Q.022 | 6.3.8 | Field description of “multipleTxAllowed” can be improved based on chairman’s notes in RAN2 #92; Multiple transmissions (to different destinations) for Mode 1 can be supported by the transmissions of multiple grants within a SC period. For Mode 2 (to different destinations), the UE randomly picks resources for a particular destination within a pool (similar to Rel-12) and if collision occurs between different destinations it is up to UE implementations how to handle it. | 1 |  |  | Field description can be enhanced in following manner (as captured in the Chairman’s notes in RAN2#92): multipleTxAllowed Indicates whether or not multiple transmissions to different destinations in one SC period is allowed. | Closed (ProSe CR) |
| Q.025 | 6.3.8 | Usage of name cellSelectionInfo in SIB19 causes confussion in 36.304 | 1 |  |  | cellSelectionInfo name can be changed to something like disccellSelectionInfo Coordinator: No opinon on the issue but if this is corrected, it probably needs to be named as discCcellSelectionInfo. | Closed (ProSe CR) |
| Q.026 | 6.3.8 | SL-DiscResourcePool it is better to add 160ms discovery period in the extenssion | 1 |  |  | Delete rf16 from discPeriod-r12 and put it in r-13 as this is also new value which got added in r-13. discPeriod-r12 ENUMERATED {rf32, rf64, rf128, rf256, rf512, rf1024, rf16-v13x0, spare}, and put: discPeriod-v13x0 CHOICE { release NULL, setup ENUMERATED {rf4, rf7, rf8, rf14, rf28, rf16 spare3, spare2, spare} spare3, spare2, spare} } OPTIONAL, -- Need ON Samsung: This does not seem needed i.e. there is no functional difference – this is just less efficient Samsung: No change done | Closed (ProSe CR) |
| Q.027 | 6.3.8 | Inside txParamsAddNeighFreq-r13, release-11 extension of tdd-Config is present. tdd-Config-r13 TDD-Config OPTIONAL, -- Need OR tdd-Config-v1130 TDD-Config-v1130 OPTIONAL, -- Cond TDD-OR | 2 |  |  | The need of tdd-Config-v1130 should be confirmed HV> QC initially assume that it would be sufficient to just refer to IE TDD-Config, as used for the Rel-12 Rx pool. However, REL-12 Tx pool was actually also referring to TDD-Config-v1130. So it seems fine to also reference this for neighbouring frequencies. I.e. no change (i.e. both fields Cond TDD-OR) | Closed (ProSe CR) (ASN.1) |
| Q.029 | 6.3.8 | Make all fields inside txParamsAddNeighFreq optional | 2 |  |  | Samsung: Text proposal would be appreciated Samsung: Understand that the suggestion was to make additionalSpectrumEmission & referenceSignalPower inside txParamsAddNeighFreq optional, need OR. After further consideration, QC suggested to keep fields mandatory to avoid having to specify UE behaviour for the case of absence. I.e. no change | Closed (ProSe CR) (ASN.1) |
| Q.031 | 6.3.8 | Description of syncConfigIndex can be updated to handle syncConfigIndex-r13, as it is for Tx case as well and points to different synch config (i.e. inter frequency synch config). | 1 |  |  | HV> Added yellow text as shown below: syncConfigIndex Indicates the synchronisation configuration that is associated with a reception or transmission pool, by means of an index to the corresponding entry of discSyncConfig in SystemInformationBlockType19. | Closed (ProSe CR) |
| L.038 | 6.3.8 SL-CommConfig | multipleTxAllowed Indicates whether or not multiple transmissions in one SC period is allowed. Currently this field seems to be placed under the field scheduled. Our understanding is that for mode1, eNB needs to indicate this to enabler intended UE behaviors, but for mode2 multiple transmissons are allowed by default. If this is a correct understanding, do we need to clarify this in the field description? (i.e to prevent the misunderstanding that multipleTX is not allowed at all for mode2?) | 3 |  |  | multipleTxAllowed Indicates whether or not multiple transmissions in one SC period is allowed for eNB scheduled transmission. Ericsson: It can be good to clarify this. Actually, the word “allowed” may be misleading. As we are talking about Mode 1, the UE follows the eNB, hence it is not “allowed” for the UE (as that indicates some level of optionality). Maybe we can think about renaming this field to e.g. multipleTxConfigured or something? CATT: mulitple transmission for mode2 is allowed if the UE can. No clarification is needed in this field description. May be some clarification in stage 2. Qualcomm: Agree HV> Clarification added that this concerns scheduled transmissions as suggested. Allowed was removed from name as REL-13 UEs are to perform multiple transmissions if the field is set (QC clarified that all Rel-13 UEs support this, only the fixed number of HARQ process is to be decided). | Closed (ProSe CR) |
| Q.034 | 6.3.8 SL-CommConfig | commTxResources The REL-13 extension does not have the same information structure as the legacy field (i.e. no choice between scheduled and ue-Selected resources, but separate extensions with their own release/ setup choice) commTxResources-r12 CHOICE { release NULL, setup CHOICE { scheduled-r12 SEQUENCE { [[ scheduled-r13x0 CHOICE { release NULL, setup SEQUENCE { logicalChGroupInfoList-r13 LogicalChGroupInfoList-r13, multipleTxAllowed-r13 BOOLEAN } } OPTIONAL, -- Need ON ue-Selected-r13x0 SEQUENCE { commTxPoolNormalDedicatedExt-r13 SEQUENCE { | 2 |  |  | Adopt the same structure as in REL-12 Samsung: It seems good to modify the REL-13 information structure, as it introduces some potential ambiguity: Suppose EUTRAN configured scheduled resources (with the REL-13 extension) and now wants to switch to UE selected resources in one step. With the REL-12 signalling it is clear that there is no explicit release of the parameters configured for scheduled operation. In the REL-13 extension it is not so clear whether EUTRAN should release the REL-13 scheduled resource parameters when switching to UE selected resources. So maybe it indeed is better to follow the REL-12 structure for the REL-13 extension also | Closed (ProSe CR) |
| E.173 | 6.3.8 SL-CommConfig | Terminology and language can be improved in the following. commTxAllowRelayDedicated Indicates for whether the UE is not allowed to transmit relay related data using the configured dedicated transmission resources i.e. neither via scheduled nor via UE selected resources. | 1 |  |  | Change text to commTxAllowRelayDedicated Indicates for whether the UE is not allowed to transmit relay related dataone to one sidelink communication using the configured dedicated transmission resources i.e. neither via scheduled nor via UE selected resources. | Closed (ProSe CR) |
| E.174 | 6.3.8 SL-DiscConfig | Wrong type, why not use BOOLEAN instead? gapRequestsAllowedDedicated-r13 ENUMERATED {false, true} OPTIONAL, -- Need ON | 1 |  |  | Change text to gapRequestsAllowedDedicated-r13 ENUMERATED {false, true}BOOLEAN OPTIONAL, -- Need ON | Closed (ProSe CR) |
| E.175 | 6.3.8 SL-DiscConfig | Table of field descriptions is not alphabetical order | 1 |  |  | Sort the table. Coordinator: This issue is already in the frozen specification. It should be corrected with a separate CR if deemed necessary. Samsung: It should be no problem to just correct in REL-13, so no need for separate CR | Closed (ProSe CR) |
| E.176 | 6.3.8 SL-DiscConfig | The conditional DiscTxSetup is not used. | 1 |  |  | Remove the conditional. | Closed (ProSe CR) |
| N.187 | 6.3.8 SL-DiscConfig | The field DiscTxSetup is described but there is no such a field in the ASN.1 definition | 1 |  |  | Either remove the field description or correct it to the right field name! HV> Removed | Closed (ProSe CR) |
| Z.094 | 6.3.8 SL-DiscConfig scheduled-r13x0 ue-Selected-r13x0 | The “-r13x0” should be changed into “v13x0’ | 1 |  |  | Change the ‘’-r13x0” into “v13x0”, as below scheduled-r13x0--> scheduled-v13x0 ue-Selected-r13x0--> ue-Selected-v13x0 | Closed (ProSe CR) |
| N.174 | 6.3.8 SL-DiscConfig:: discTxInfoInterFreqListExt-r13 | Why is there Ext in this field/IE name? How can we extend the upper level field via included field in the same release?? | 1 |  |  | Remove Ext from the name if it’s not extending another list. Samsung: Current specification assumes that field is not used for the first Tx frequency covered by legacy fields. See L.042 | Closed (ProSe CR) |
| E.177 | 6.3.8 SL-DiscResourcePool | The applicability of the following text is ambiguous (a Note in ASN.1 text). -- Note: Extended values apply for PS discovery (not only for relay). E-UTRAN sets original -- field to spare value to ensure legacy UEs ignore the concerned entry | 1 |  |  | Integrate the text in the field description for discPeriod instead. Coordinator: Specification drafting rules (TR 21.801) gives the following guidance for notes integrated in the text; Notes and examples should preferably be placed at the end of the clause or subclause, or after the paragraph, to which they refer. This could be understood such that the note can be either 1) at the end of the information element if the information element definition could be seens as a subclause or after the field definition to which it refers if the ASN.1 segment can be considered as a paragraph. Samsung: Moved to field description | Closed (ProSe CR) |
| E.178 | 6.3.8 SL-DiscResourcePool | The following text should be optionally present with a contional. tdd-Config-r13 TDD-Config OPTIONAL, -- Need OR | 1 |  |  | Change the text to tdd-Config-r13 TDD-Config OPTIONAL, -- Need ORCond TDD-OR | Closed (ProSe CR) |
| N.176 | 6.3.8 SL-DiscResourcePool::discPeriod-v13x0 | If legacy UEs are supposed to ignore this field, why not name it completely differently? | 2 |  |  | Rename the field and not use it as a NCE Qualcomm: No need to change Samsung: Not aware of any problem (i.e. original field is set to spare, causing UE to ignore the pool entry). As we just introduce additional values for an existing field, there seems no reason to change Samsung: No change | Closed (ProSe CR) |
| L.033 | 6.3.8 SL-GapConfig | The meaning of the value for each bit needs to be clarified in the field description of gapSubframeBitmap. gapSubframeBitmap Indicates the subframes of one or more individual gaps, not only covering the subframes of the associated discovery resources but also including e.g. re-tuning delays. The UE and E-UTRAN signal bit strings of valid sizes only i.e. sizes up to (10* gapPeriod). | 2 |  |  | gapSubframeBitmap Indicates the subframes of one or more individual gaps, not only covering the subframes of the associated discovery resources but also including e.g. re-tuning delays. The UE and E-UTRAN signal bit strings of valid sizes only i.e. sizes up to (10* gapPeriod). Value 1 indicates that the UE is allowed to use for sidelink discovery for the subframe. Ericsson: We agree in principle. The added text can be improved, change to “...UE is allowed to use the subframe for sidelink discovery.” Qualcomm: Ok Samsung: Covered by R2-160017 | Closed (ProSe CR) |
| E.276 | 6.3.8 SL-GapConfig | #### SL-GapConfig The IE SL-GapConfig indicates the gaps assigned by E-UTRAN to enable the UE to receive or transmit sidelink discovery, intra or inter frequecny. ……. gapSubframeBitmap Indicates the subframes of one or more individual gaps, not only covering the subframes of the associated discovery resources but also including e.g. re-tuning delays. The UE and E-UTRAN signal bit strings of valid sizes only i.e. sizes up to (10* gapPeriod). | 1 |  |  | This IE is used also by the gap request. Therefore “gaps assigned by E-UTRAN” might be misleading. frequecny -> frequency To make it clearer, I would add also other delays, e.g. also including other overheads needed to perform sidelink operations in the discovery frequency, e.g. re-tuning delays, synchronization overhead, Coordinator: It needs to be clarified exactly what needs to be added. Otherwise it isnot possible to make this correction. Samsung: Fine to remove ‘assigned by E-UTRAN’ and to add ‘synchronisation’ i.e. ‘e.g. re-tuning and synchronisation delays | Closed (ProSe CR) |
| Q.032 | 6.3.8 SL-GapConfig | Update SL-GapConfig to indicate that it can be for same or different PLMN as well. #### – SL-GapConfig The IE SL-GapConfig indicates the gaps assigned by E-UTRAN to enable the UE to receive or transmit sidelink discovery, intra or inter frequecny. #### – SL-GapRequest The IE SL-GapRequest indicates the gaps requested by the UE to receive or transmit sidelink discovery, intra or inter frequecny. #### – SL-SysInfoReport The IE SL-SysInfoReport contains the parameters related to Sidelink operations from system information of inter-frequency cells | 1 |  |  | #### – SL-GapConfig The IE SL-GapConfig indicates the gaps assigned by E-UTRAN to enable the UE to receive or transmit sidelink discovery, intra or inter frequency of same or different PLMN. #### – SL-GapRequest The IE SL-GapRequest indicates the gaps requested by the UE to receive or transmit sidelink discovery, intra or inter frequency of same or different PLMN. #### – SL-SysInfoReport The IE SL-SysInfoReport contains the parameters related to Sidelink operations from system information of inter-frequency cells of same or different PLMN. | Closed (ProSe CR) |
| L.032 | #### 6.3.8 SL-GapRequest | Modification of the explanation for clarity. | 1 |  |  | #### – SL-GapRequest The IE SL-GapRequest indicates the gaps requested by the UE to receive or transmit sidelink discovery, on intra or inter frequencny. Samsung: No change | Closed (ProSe CR) |
| L.034 | 6.3.8 SL-Priority | Wrong reference | 1 |  |  | #### SL-Priority The IE SL-Priority indicates the priority of resource pool used for sidelink communication, see TS 36.3211 [621]. Ericsson: Agree. Qualcomm: Ok Coordinator: Changed from class 2 to class 1. It seems that following correction is proposed #### SL-Priority The IE SL-Priority indicates the priority of resource pool used for sidelink communication, see TS 36.321 [6]36.211 [21]. This needs to be confirmed. Samsung: Changed as suggested | Closed (ProSe CR) |
| L.051 | 6.3.8 SL-SyncConfig | Editorial: SL-SyncConfigListNFreq-r12 ::= SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfigNFreq-r13 SL-SyncConfigNFreq-r13 ::= … OPTIONAL, -- Need OR txParameters-r13 SEQUENCE { syncTxParameters-r13 SL-TxParameters-r12, syncTxThreshIC-r13 RSRP-RangeSL-r12, syncInfoReserved-r13 BIT STRING (SIZE (19)) OPTIONAL, -- Need OR syncTxPeriodic-r13 ENUMERATED {true} OPTIONAL -- Need OR } OPTIONAL, -- Need OR rxParameters-r1s SEQUENCE { discSyncWindow-r13 ENUMERATED {w1, w2} } OPTIONAL, -- Need OR ... } | 1 |  |  | SL-SyncConfigListNFreq-r12 ::= SEQUENCE (SIZE (1..maxSL-SyncConfig-r12)) OF SL-SyncConfigNFreq-r13 SL-SyncConfigNFreq-r13 ::= … OPTIONAL, -- Need OR txParameters-r13 SEQUENCE { syncTxParameters-r13 SL-TxParameters-r12, syncTxThreshIC-r13 RSRP-RangeSL-r12, syncInfoReserved-r13 BIT STRING (SIZE (19)) OPTIONAL, -- Need OR syncTxPeriodic-r13 ENUMERATED {true} OPTIONAL -- Need OR } OPTIONAL, -- Need OR rxParameters-r13s SEQUENCE { discSyncWindow-r13 ENUMERATED {w1, w2} } OPTIONAL, -- Need OR ... } Ericsson: Agree. Same as E.181. Qualcomm: Ok Coordinator: Changed from class 2 to class 1. | Closed (ProSe CR) |
| N.186 | 6.3.8 SL-SysInfoReport:: SL-CarrierFreqInfo | SL-CarrierFreqInfo described in SL-SysInfoReport information element section is not part of SL-SysInfoReport. | 1 |  |  | Field description should be for carrierFreqInfo-r13? Coordinator: It seems so. Also the field descriptions should be ordered. | Closed (ProSe CR) |
| Z.096 | 6.3.8 | SL-DiscTxRefCarrierDedicated-r13 ::= CHOICE { pCell NULL, sCell SCellIndex-r10 } Only pcell or scell of the UE could be configured to be reference cell other than the paired frequency with the one to transmitt discovery message on, which excluded that the eNB may configure a cell other than the pcell/scell as reference cell for the UE. | 2 |  |  | Why it is excluded that the eNB may configure a cell other than the pcell/scell as reference cell for the UE? Ericsson: We do not think there are any agreements suggesting that the eNB may configure a non-serving cell as reference cell. These are the relevant agreements: - The eNB may configure the UE with a carrier to use for DL measurements/synchronization using SIB19 for each SL carrier frequency. The reference carrier is signalled in SIB19 and via dedicated signalling. It would be beneficial to signal a reference carrier that is a serving carrier. - The UE shall use the DL carrier paired to the SL carrier frequency for DL measurements/synchronization if no carrier for DL measurements/synchronization is indicated in SIB19 and dedicated signalling. - The UE shall use the PCell for DL measurements and synchronization when the UE performs ProSe Discovery on the primary carrier. - If the UE has a configured serving cell in secondary carrier, then one of the configured serving cell is always used for DL measurements and synchronization Qualcomm: We have agreement in RAN2 to use pCell and Scell(s). Intel: We agree that there was no agreement to configure a non-serving cell as reference cell. Therefore, we consider ZTE comment as a new proposal what is out-of-scope of ASN.1 review. Chair: => No change needed | Closed |
| S.106 | 6.3.8 SC-MTCH-InfoList, sc-mtch-neighbourCell | Should the size of the bit string always be fixed to max number of cells? | 2 |  |  | Huawei: Yes (see Z.092) Chair: See conclusion of Z.092 | Closed |
| L.035 | 6.3.8 SL-CommConfig | SL-CommConfig-r12 ::= SEQUENCE { … [[ scheduled-r13x0 CHOICE { release NULL, setup SEQUENCE { logicalChGroupInfoList-r13 LogicalChGroupInfoList-r13, multipleTxAllowed-r13 BOOLEAN } OPTIONAL, -- Need ON ue-Selected-r13x0 SEQUENCE { scheduled and its subfields are misplaced. | 3 |  |  | release NULL, setup CHOICE { scheduled-r13 SEQUENCE { logicalChGroupInfoList-r13 LogicalChGroupInfoList-r13, multipleTxAllowed-r13 BOOLEAN Ericsson: Agree. CATT: agree Qualcomm: Ok Coordinator: This solution proposal should be clarified because based on this text it is difficult to understand what exactly needs to be corrected. In addition, there is no such field as scheduled-r13 but there is a field scheduled-r13x0 (which seems to have a type, so either x0 needs to be removed or ‘r’ changed to ‘v’). Samsung: We are not aware of any problem and do not understand what is proposed in detail. There are two fields, either of which should be possible to be configured, omitted (no change) or released. Chair: => No change needed | Closed |
| H.083 | 6.3.8 SL-CommConfig | SL-CommConfig-r12 ::= SEQUENCE { commTxResources-r12 CHOICE { release NULL, setup CHOICE { scheduled-r12 SEQUENCE { sl-RNTI-r12 C-RNTI, mac-MainConfig-r12 MAC-MainConfigSL-r12, sc-CommTxConfig-r12 SL-CommResourcePool-r12, mcs-r12 INTEGER (0..28) OPTIONAL -- Need OP }, ue-Selected-r12 SEQUENCE { -- Pool for normal usage commTxPoolNormalDedicated-r12 SEQUENCE { poolToReleaseList-r12 SL-TxPoolToReleaseList-r12 OPTIONAL, -- Need ON poolToAddModList-r12 SL-CommTxPoolToAddModList-r12 OPTIONAL -- Need ON } } } } OPTIONAL, -- Need ON ..., [[ scheduled-r13x0 CHOICE { release NULL, setup SEQUENCE { logicalChGroupInfoList-r13 LogicalChGroupInfoList-r13, multipleTxAllowed-r13 BOOLEAN } } OPTIONAL, -- Need ON ue-Selected-r13x0 SEQUENCE { commTxPoolNormalDedicatedExt-r13 SEQUENCE { poolToReleaseListExt-r13 SL-TxPoolToReleaseListExt-r13 OPTIONAL, -- Need ON poolToAddModListExt-r13 SL-CommTxPoolToAddModListExt-r13 OPTIONAL -- Need ON } }, commTxAllowRelayDedicated-r13 BOOLEAN OPTIONAL -- Need ON ]] } Needs to first discuss if Multiple transmission during a sidelink period is manatory in rel-13. If it is manatory, scheduled-r13x0 itself can indicate the eNB allowing multiple transmission. multipleTxAllowed is not necessary. | 2 |  |  | SL-CommConfig-r12 ::= SEQUENCE { commTxResources-r12 CHOICE { release NULL, setup CHOICE { scheduled-r12 SEQUENCE { sl-RNTI-r12 C-RNTI, mac-MainConfig-r12 MAC-MainConfigSL-r12, sc-CommTxConfig-r12 SL-CommResourcePool-r12, mcs-r12 INTEGER (0..28) OPTIONAL -- Need OP }, ue-Selected-r12 SEQUENCE { -- Pool for normal usage commTxPoolNormalDedicated-r12 SEQUENCE { poolToReleaseList-r12 SL-TxPoolToReleaseList-r12 OPTIONAL, -- Need ON poolToAddModList-r12 SL-CommTxPoolToAddModList-r12 OPTIONAL -- Need ON } } } } OPTIONAL, -- Need ON ..., [[ scheduled-r13x0 CHOICE { release NULL, setup SEQUENCE { logicalChGroupInfoList-r13 LogicalChGroupInfoList-r13, multipleTxAllowed-r13 BOOLEAN } } OPTIONAL, -- Need ON ue-Selected-r13x0 SEQUENCE { commTxPoolNormalDedicatedExt-r13 SEQUENCE { poolToReleaseListExt-r13 SL-TxPoolToReleaseListExt-r13 OPTIONAL, -- Need ON poolToAddModListExt-r13 SL-CommTxPoolToAddModListExt-r13 OPTIONAL -- Need ON } }, commTxAllowRelayDedicated-r13 BOOLEAN OPTIONAL -- Need ON ]] } Ericsson: Agree to discuss, but we think we can leave it in ASN.1 for now. This should somehow be related to UE capabiblities then? CATT: prefer not mandatory for eNB Qualcomm: We donot have agreement in RAN2 that Multiple transmission is mandatory in Rel-13. We would like to keep the ASN.1 as it is. Samsung: Agree with Ericssion that we should not introduce changes now. Change to an explicit indication definitely requires more discussion. Intel: RAN2 agreed to allow eNB to indicate whether multiple Tx is allowed or not. Therefore, ASN.1 is aligned with the RAN2 agreement. But from UE pov the support of multiple TX should be discussed as part of the UE capability discussion. Chair: => No change needed | Closed |
| H.084 | 6.3.8 SL-DiscConfig | SL-DiscConfig-r12 ::= SEQUENCE { discTxResources-r12 CHOICE { release NULL, setup CHOICE { scheduled-r12 SEQUENCE { discTxConfig-r12 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r12 SL-TF-IndexPairList-r12 OPTIONAL, -- Need ON discHoppingConfig-r12 SL-HoppingConfigDisc-r12 OPTIONAL -- Need ON }, ue-Selected-r12 SEQUENCE { discTxPoolDedicated-r12 SEQUENCE { poolToReleaseList-r12 SL-TxPoolToReleaseList-r12 OPTIONAL, -- Need ON poolToAddModList-r12 SL-DiscTxPoolToAddModList-r12 OPTIONAL -- Need ON } OPTIONAL -- Need ON } } } OPTIONAL, -- Need ON ..., [[ discTF-IndexList-v1260 CHOICE { release NULL, setup SEQUENCE { discTF-IndexList-r12b SL-TF-IndexPairList-r12b } } OPTIONAL -- Need ON ]], [[ discTxResourcesPS-r13 CHOICE { release NULL, setup CHOICE { scheduled-r13 SL-DiscTxConfigScheduled-r13, ue-Selected-r13 SEQUENCE { discTxPoolPS-Dedicated-r13 SL-DiscTxPoolDedicated-r13 } } } OPTIONAL, -- Need ON discTxInterFreqInfo-r13 CHOICE { release NULL, setup SEQUENCE { discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL, -- Need OR discTxRefCarrierDedicated-r13 SL-DiscTxRefCarrierDedicated-r13 OPTIONAL, -- Need OR discTxInfoInterFreqListExt-r13 SL-DiscTxInfoInterFreqListExt-r13 OPTIONAL -- Need ON } } OPTIONAL, -- Need ON gapRequestsAllowedDedicated-r13 ENUMERATED {false, true} OPTIONAL, -- Need ON discRxGapConfig-r13 CHOICE { release NULL, setup SL-GapConfig-r13 } OPTIONAL, -- Need ON discTxGapConfig-r13 CHOICE { release NULL, setup SL-GapConfig-r13 } OPTIONAL, -- Need ON discSysInfoReportConfig-r13 CHOICE { release NULL, setup SL-DiscSysInfoReportFreqList-r13 } OPTIONAL -- Need ON ]] } It seems more clear if the legacy fields are only used for Pcell and discTxInterFreqInfo-r13 is used for other frequencies. Therefore these two parameters are not needed. discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL, -- Need OR discTxRefCarrierDedicated-r13 | 3 |  |  | SL-DiscConfig-r12 ::= SEQUENCE { discTxResources-r12 CHOICE { release NULL, setup CHOICE { scheduled-r12 SEQUENCE { discTxConfig-r12 SL-DiscResourcePool-r12 OPTIONAL, -- Need ON discTF-IndexList-r12 SL-TF-IndexPairList-r12 OPTIONAL, -- Need ON discHoppingConfig-r12 SL-HoppingConfigDisc-r12 OPTIONAL -- Need ON }, ue-Selected-r12 SEQUENCE { discTxPoolDedicated-r12 SEQUENCE { poolToReleaseList-r12 SL-TxPoolToReleaseList-r12 OPTIONAL, -- Need ON poolToAddModList-r12 SL-DiscTxPoolToAddModList-r12 OPTIONAL -- Need ON } OPTIONAL -- Need ON } } } OPTIONAL, -- Need ON ..., [[ discTF-IndexList-v1260 CHOICE { release NULL, setup SEQUENCE { discTF-IndexList-r12b SL-TF-IndexPairList-r12b } } OPTIONAL -- Need ON ]], [[ discTxResourcesPS-r13 CHOICE { release NULL, setup CHOICE { scheduled-r13 SL-DiscTxConfigScheduled-r13, ue-Selected-r13 SEQUENCE { discTxPoolPS-Dedicated-r13 SL-DiscTxPoolDedicated-r13 } } } OPTIONAL, -- Need ON discTxInterFreqInfo-r13 CHOICE { release NULL, setup SEQUENCE { discTxCarrierFreq-r13 ARFCN-ValueEUTRA-r9 OPTIONAL, -- Need OR discTxRefCarrierDedicated-r13 SL-DiscTxRefCarrierDedicated-r13 OPTIONAL, -- Need OR discTxInfoInterFreqListExt-r13 SL-DiscTxInfoInterFreqListExt-r13 OPTIONAL -- Need ON } } OPTIONAL, -- Need ON gapRequestsAllowedDedicated-r13 ENUMERATED {false, true} OPTIONAL, -- Need ON discRxGapConfig-r13 CHOICE { release NULL, setup SL-GapConfig-r13 } OPTIONAL, -- Need ON discTxGapConfig-r13 CHOICE { release NULL, setup SL-GapConfig-r13 } OPTIONAL, -- Need ON discSysInfoReportConfig-r13 CHOICE { release NULL, setup SL-DiscSysInfoReportFreqList-r13 } OPTIONAL -- Need ON ]] } Ericsson: We don’t understand the reason behind this change. Maybe Huawei can clarify? There seems to be several open issues related to how and which frequencies can be configured for discovery. Perhaps we could use a clarifying discussion and good contributions to settle this? Qualcomm: Not sure about the proposal. Current formulation is fine. Can be discussed in separate discussion document. Samsung: See/ same as L.042/3. Chair: Covered by L.042/3 | Closed |
| L.040 | 6.3.8 SL-DiscConfig | [[ discTxResourcesPS-r13 CHOICE { release NULL, setup CHOICE { scheduled-r13 SL-DiscTxConfigScheduled-r13, ue-Selected-r13 SEQUENCE { discTxPoolPS-Dedicated-r13 SL-DiscTxPoolDedicated-r13 } } } OPTIONAL, -- Need ON Editorial correction: hyphen is not needed | 2 |  |  | [[ discTxResourcesPS-r13 CHOICE { release NULL, setup CHOICE { scheduled-r13 SL-DiscTxConfigScheduled-r13, ue-Selected-r13 SEQUENCE { discTxPoolPS-Dedicated-r13 SL-DiscTxPoolDedicated-r13 } } } OPTIONAL, -- Need ON Ericsson: Agree. Qualcomm: Ok Samsung: Do not agree i.e. needed after abbreviation (PS) Chair: => No change needed | Closed |
| L.041 | 6.3.8 SL-DiscConfig | Suggest to rename "discTxInterFreqInfo" to discTxInterFreqResources" to be aligned with discTxResources-r12 or discTxResourcesPS-r13 discTxInterFreqInfo-r13 CHOICE { release NULL, setup SEQUENCE { | 2 |  |  | discTxInterFreqResourcesInfo-r13 CHOICE { release NULL, setup SEQUENCE { Ericsson: Not sure of the benefit of this. Qualcomm: No need to change Chair: => No change needed | Closed |
| Q.023 | 6.3.8 | R12 is used for discTF-IndexList-r12 inside SL-DiscTxConfigScheduled-r13. It should be discTF-IndexList-r12b | 1 |  |  | Use the field name discTF-IndexList-r12b and the definition SL-TF-IndexPairList-r12b CATT: agree Coordinator: Same as L.048 | Closed |
| L.036 | 6.3.8 SL-CommConfig | Editorial: commTxAllowRelayDedicated Indicates for whether the UE is not allowed to transmit relay related data using the configured dedicated transmission resources i.e. neither via scheduled nor via UE selected resources. | 1 |  |  | commTxAllowRelayDedicated Indicates for whether the UE is not allowed to transmit relay related data using the configured dedicated transmission resources i.e. neither via scheduled nor via UE selected resources. Coordinator: See also E.173 | Closed |
| E.327 | 6.3.8 SL-CommConfig, | commTxAllowRelayDedicated field description needs to be before the description for commTxPoolNormalDedicated. | 1 |  |  | Update the order Coordinator: Correceted during v1300 CR implementation | Closed |
| E.179 | 6.3.8 SL-GapConfig | Superfluous space character in the following. gapSubframeBitmap-r13 BIT STRING (SIZE (1.. 10240)) | 1 |  |  | Change text to gapSubframeBitmap-r13 BIT STRING (SIZE (1..10240)) Coordinator: Corrected during v1300 CR implementation | Closed |
| E.271 | 6.3.8 SL-GapRequest | Spelling error; frequecny should be frequency | 1 |  |  | Correct the spelling error The IE SL-GapRequest indicates the gaps requested by the UE to receive or transmit sidelink discovery, intra or inter frequencyfrequecny. Coordinator: Correceted during v1300 CR implementation. | Closed |
| E.181 | 6.3.8 SL-SyncConfig | Typo. rxParameters-r1s SEQUENCE { | 1 |  |  | Change text to rxParameters-r1s3 SEQUENCE { Ericsson: Same as L.051. Coordinator: Same as L.051 and hence closed. | Closed |
| N.192 | 6.3.8 SL-SyncConfig | SL-SyncConfigListNFreq-r12 is newly defined but suffix -r12 | 1 |  |  | Correct to use suffix –r13. Ericsson: Agree and same as Z.095. Coordinator: Same as Z.095 and hence closed. | Closed |
| 6.4 RRC multiplicity and type constraints values |  |  |  |  |  |  |  |
| Q.035 | 6.4 Multiplicity and type constraint values | maxSL-DiscRxPoolPreconf-r13 INTEGER ::= 4 -- Maximum number of preconfigured sidelink -- discovery Rx resource pool entries Preconfig Discovery Rx pool shoul dbe 16 (similar to SIB19) | 4 |  |  | maxSL-DiscRxPoolPreconf-r13 INTEGER ::= 416 -- Maximum number of preconfigured sidelink Qualcomm: raising the category Samsung: We think the intention was to align with communication in REL-12 preconfiguration. Probably requires further discussion Samsung: Paper for RAN2#93 seems desirable Coordinator: Can be closed. | Closed (ProSe CR) |
| Q.036 | 6.4 Multiplicity and type constraint values | maxSL-DiscTxPoolPreconf-r13 INTEGER ::= 1 -- Maximum number of preconfigured sidelink -- discovery Tx resource pool entries Preconfig discovery Tx pool should be 4 (similar to SIB19) | 4 |  |  | maxSL-DiscTxPoolPreconf-r13 INTEGER ::= 14 -- Maximum number of preconfigured sidelink -- discovery Tx resource pool entries Qualcomm: raising the category Samsung: We think the intention was to align with communication in REL-12 preconfiguration. Probably requires further discussion Samsung: Same as Q.035 Coordinator: Can be closed. | Closed (ProSe CR) |
| Z.100 | 6.4 | To align the naming convention by adding “-” between two letters of upper case: | 1 |  |  | To align the naming convention: maxMBMS-ServiceListPerUE-r13 Huawei: Agree. Intel: Corrected in general CR. Made also the corresponding correction in IE MBMS-ServiceList in section 6.3.7. | Closed (General CR) |
| 9.1 Specified configurations |  |  |  |  |  |  |  |
| Z.101 | 9.1.1.4 | A spelling error of a field in which the first letter “s” should be lower case. | 1 |  |  | Snsn-FieldLength Intel: Agree, but it is a legacy issue. Coordinator: Minor editorial anyway. Corrected in general CR. Corrected also for the same error in 9.1.1.6. | Closed (General CR) |
| Z.102 | 9.1.1.7 | A spelling error of a field in which the first letter should be lower case. | 1 |  |  | Snsn-FieldLength Intel: Agree Coordinator: Corrected in general CR. | Closed (General CR) |
| 9.2 Default radio configurations |  |  |  |  |  |  |  |
| 9.3 ProSe pre-defined configurations |  |  |  |  |  |  |  |
| Z.103 | 9.3.2 | “SL-PreconfigCommPoolListExt-r13” should be “SL-PreconfigCommTxPoolListExt-r13”. SL-Preconfiguration-r12 ::= SEQUENCE { preconfigGeneral-r12 SL-PreconfigGeneral-r12, preconfigSync-r12 SL-PreconfigSync-r12, preconfigComm-r12 SL-PreconfigCommPoolList4-r12, ..., [[ preconfigComm-v13x0 SEQUENCE { SEQUENCE { commRxPoolListExt-r13 SL-PreconfigCommRxPoolListExt-r13, commTxPoolListExt-r13 SL-PreconfigCommPoolListExt-r13 OPTIONAL } OPTIONAL, preconfigDisc-r13 SEQUENCE { discRxPoolList-r13 SL-PreconfigDiscRxPoolList-r12, discTxPoolList-r13 SL-PreconfigDiscTxPoolList-r12 OPTIONAL } OPTIONAL preconfigRelay-r13 SL-PreconfigRelay-r13 OPTIONAL ]] } | 1 |  |  | Change “SL-PreconfigCommPoolListExt-r13” to “SL-PreconfigCommTxPoolListExt-r13”. SL-Preconfiguration-r12 ::= SEQUENCE { preconfigGeneral-r12 SL-PreconfigGeneral-r12, preconfigSync-r12 SL-PreconfigSync-r12, preconfigComm-r12 SL-PreconfigCommPoolList4-r12, ..., [[ preconfigComm-v13x0 SEQUENCE { SEQUENCE { commRxPoolListExt-r13 SL-PreconfigCommRxPoolListExt-r13, commTxPoolListExt-r13 SL-PreconfigCommTxPoolListExt-r13 OPTIONAL } OPTIONAL, preconfigDisc-r13 SEQUENCE { discRxPoolList-r13 SL-PreconfigDiscRxPoolList-r12, discTxPoolList-r13 SL-PreconfigDiscTxPoolList-r12 OPTIONAL } OPTIONAL preconfigRelay-r13 SL-PreconfigRelay-r13 OPTIONAL ]] } Samsung: Was already correct in official spec verion (only difference is number of pool entries) | Closed (ProSe CR) |
| E.340 | 9.3.2 - SL-PreconfigGeneral-r12 | The field carrierFreq denoted the “ProSe carrier” in Rel-12, i.e. the carrier on which the UE performs sidelink communication when out of coverage. The corresponding field description only mentions “sidelink operation”. We think this can be improved. carrierFreq Indicates the carrier frequency for sidelink operation. In case of FDD it is uplink carrier frequency and the corresponding downlink frequency can be determined from the default TX-RX frequency separation defined in TS 36.101 [42, table 5.7.3-1]. | 2 |  |  | Change the field description to: carrierFreq Indicates the carrier frequency for sidelink communication and sidelink discovery out of coverage. In case of FDD it is uplink carrier frequency and the corresponding downlink frequency can be determined from the default TX-RX frequency separation defined in TS 36.101 [42, table 5.7.3-1]. Coordinator: It seems that the following correction is proposed, carrierFreq Indicates the carrier frequency for sidelink operationcommunication and sidelink discovery out of coverage. In case of FDD it is uplink carrier frequency and the corresponding downlink frequency can be determined from the default TX-RX frequency separation defined in TS 36.101 [42, table 5.7.3-1]. | Closed (ProSe CR) |
| E.272 | 9.3.2 Pre-configurable parameters, SL-Preconfiguration | Optional SL extension field syncTxPeriodic-r13 has a Need code OR but need codes are defined for DL only. | 2 |  |  | Remove the Need code syncTxPeriodic-r13 ENUMERATED {true} OPTIONAL -- Need OR Qualcomm: Agree Chair: => Agree to remove Need code. => Can be closed when included in the ProSe CR. | Closed (ProSe CR) (ASN.1) |
| N.191 | 9.3.2 SL-Preconfiguration:: commRxPoolListExt-r13, commTxPoolListExt-r13 | What are these extending – there are no existing IEs with the same name but without Ext-suffix? Also, why is the commRxPoolListExt-r13 mandatory, just like SL-PreconfigCommPoolList4-r12? | 2 |  |  | Samsung: We should not just consider naming. I.e. the existing pool is named as common for Tx and Rx, but only first entry is used for Tx i.e. so functionally there is seperation Ericsson: We have some sympathy for this proposal. We think this can be resolved by renaming the lists. Chair: => Remove the -Ext and explain in field description that the the list is an extension of the list of receive pools and transmit pools respectively. => Can be closed when included in the ProSe CR. | Closed (ProSe CR) (ASN.1) |
| 10.1 General |  |  |  |  |  |  |  |
| 10.2 RRC messages transferred across network nodes |  |  |  |  |  |  |  |
| S.107 | 10.2.2 SCG-ConfigInfo, measResultSSTD | Prefer not use uncommon abbreviation in field description | 1 |  |  | Change to SFN and Subframe Timing Difference Nokia: Corrected in the general CR. | Closed (General CR) |
| N.181 | 10.2.2 | SCG-ConfigInfo message, missing space in: “OPTIONAL-- Cond SCellAdd2" | 1 |  |  | Add missing space. Coordinator: Corrected during v1300 CR implementation | Closed |
| 10.3 IE definition |  |  |  |  |  |  |  |
| 10.4 RRC multiplicity and type constraints values |  |  |  |  |  |  |  |
| 10.5 Mandatory information in AS-Configuration |  |  |  |  |  |  |  |

# Sections not part of the review (for information)

| No | Clause(s) | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- | --- |
| 4.1 Introduction |  |  |  |  |  |
| 4.2 Architecture |  |  |  |  |  |
| 4.3 Services |  |  |  |  |  |
| 4.4 Functions |  |  |  |  |  |
| 5.1 General |  |  |  |  |  |
| 5.7 Generic error handling |  |  |  |  |  |
| 6.1 General |  |  |  |  |  |
| 7.1 UE variables |  |  |  |  |  |
| H.085 | 5.5.2.1 7.1 | 5.5.2.1 1> if the received measConfig includes the ul-DelayConfig: 2> set the parameter ul-DelayConfig within VarMeasConfig to the received value of ul-DelayConfig; 2> configure the PDCP layer to apply ul-DelayConfig; ul-DelayConfig is missing in VarMeasConfig | 1/2 | 7.1 VarMeasConfig ::= SEQUENCE { -- Measurement identities measIdList MeasIdToAddModList OPTIONAL, measIdListExt-r12 MeasIdToAddModListExt-r12 OPTIONAL, -- Measurement objects measObjectList MeasObjectToAddModList OPTIONAL, measObjectList-v9i0 MeasObjectToAddModList-v9e0 OPTIONAL, -- Reporting configurations reportConfigList ReportConfigToAddModList OPTIONAL, -- Other parameters quantityConfig QuantityConfig OPTIONAL, measScaleFactor-r12 MeasScaleFactor-r12 OPTIONAL, s-Measure INTEGER (-140..-44) OPTIONAL, speedStatePars CHOICE { release NULL, setup SEQUENCE { mobilityStateParameters MobilityStateParameters, timeToTrigger-SF SpeedStateScaleFactors } } OPTIONAL, allowInterruptions-r11 BOOLEAN OPTIONAL, ul-DelayConfig OPTIONAL } Ericsson: Agree, but from the Huawei proposal above it looks like ul-DelayConfig has no type which is incorrect. It should be something like: ul-DelayConfig-r13 UL-DelayConfig-r13 OPTIONAL DCM: Agree with Ericsson. Chair: => Needs to be revisited after the configuration structure for ul delay config is concluded. Coordinator: Can be fixed after freezing. Coordinator: The procedures are changed such that there are no requirements to store this field and hence there are no reasons to change the variable. Closed without actions. | Closed (General CR) (ASN.1) |
| 7.2 Counters |  |  |  |  |  |
| 7.3 Timers |  |  |  |  |  |
| E.027 | 7.3 | Description of timer T360 is missing | 2 | Add description of timer T360. Coordinator: A text proposal would be useful Samsung: Already provided in S.109 (same issue) Coordinator: Can be closed because already covered by another issue. | Closed |
| S.109 | T360 | Add the missing row | 2 | Start: Upon performing redistribution target selection as specified in TS 36.304 [4]. Stop: Upon entering RRC_CONNECTED, upon receiving a Paging message including redistributionIndication and upon reselecting a cell not belonging to the redistribution target. Expiry: Stop considering a frequency or cell to be redistribution target. Coordinator: Corrected in general CR. | Closed (General CR) |
| 7.4 Constants |  |  |  |  |  |
| 8.1 Genera |  |  |  |  |  |
| 8.2 Structure of encoded RRC messages |  |  |  |  |  |
| 8.3 Basic production |  |  |  |  |  |
| 8.4 Extension |  |  |  |  |  |
| 11.1 UE capability related constraints |  |  |  |  |  |
| 11.2 Processing delay requirements for RRC procedures |  |  |  |  |  |
| Annex A (informative): Guidelines, mainly on use of ASN.1 |  |  |  |  |  |
| D.044 |  | Some of guidelines seem obsoleted. If time allows, it would be better to update the guideline. | 3 |  |  |
| Annex B (normative): Release 8 and 9 AS feature handling |  |  |  |  |  |
| I.058 | B.1 Feature group indicators | New reference [73] refers to TS 23.179 but spec number is missing. | 1 | Add missing spec number TS 23.179 before reference [73]: In Table B.1-1, a 'VoLTE capable UE' corresponds to a UE which is IMS voice capable and a ‘MCPTT capable UE’ corresponds to a UE which supports MCPTT voice application as defined in TS 23.179 [73]. Coordinator: Corrected in general CR. | Closed (General CR) |
| Annex C (normative): Release 10 AS AS feature handling |  |  |  |  |  |
| Annex D (informative): Descriptive background information |  |  |  |  |  |

# Next issue list

| Company | Next issue |
| --- | --- |
| CATT | C.092 |
| DOCOMO | D.047 |
| Ericsson | E.344 |
| Huawei | H.088 |
| Intel | I.061 |
| LG Electronics Inc | L.054 |
| Nokia | N.205 |
| Qualcomm | Q.037 |
| Samsung | S.116 |
| ZTE | Z.106 |

# Revisions

| TDoc # | Title | Source | Agenda item # | Agenda item description | Reservation date |
| --- | --- | --- | --- | --- | --- |
| R2-160004 | Review issue list for ASN.1 freeze | Ericsson, Samsung | 3.1 | LTE ASN.1 review: Organisational | 2016-01-08 09:55:58 |
| R2-160048 | Review issue list for ASN.1 freeze | Ericsson, Samsung | 3.1 | LTE ASN.1 review: Organisational | 2016-01-11 14:51:58 |
| R2-161083 | Review issue list for ASN.1 freeze | Ericsson, Samsung | 7.19.1 | LTE Rel-13: LTE ASN.1 review: Organisational | 2016-01-29 15:30:30 |