3GPP TSG-RAN2#95 meeting	 Tdoc R2-165199

Gothenburg, Sweden, 22 - 26 August 2016

Agenda Item:	7.6.1

Souce:	MediaTek Inc.

Title:	Periodic WLAN measurement reporting

Document for:		Discussion and decision

# 1 WLAN measurement reporting

In LTE, the triggering mechanism for sending a measurement report can be either event driven or periodic. In Rel-13, RAN2 defined the new measurement events W1, W2, and W3 [1] to support the LWA, RCLWI, and LWIP features. During RAN2 discussions in Release 13, there was no discussion of periodic WLAN measurement reporting. However, there is no normative text in either stage 2 or stage 3 specifications that specifically prohibits the eNB from configuring periodic WLAN measurements.

Observation 1: Release 13 specifications appear to support periodic WLAN measurements even though the topic was never discussed.

In the interest of clarifying the intended behaviour, we suggest that RAN2 confirms that periodic WLAN measurements are not supported for the Release 13 LWA, LWIP, and RCLWI features. We note that if companies feel that periodic WLAN measurements are needed, then the same can be introduced as part of Release 14 LWA WI.

Proposal 1: RAN2 is requested to confirm that periodic WLAN measurements are not supported for Release 13 LWA, LWIP, and RCLWI features.

We do not think any change to Release 13 RAN2 specifications is required, but will be glad to provide CRs if other companies feel a need for such clarification.

We would also like to point out that 3GPP TS 36.133 [2] indicates support of periodic WLAN measurements (See clause 8.1.2.4.19.2.3), which may need to be rectified.

Proposal 2: RAN2 is requested to inform RAN4 that periodic WLAN measurements are not supported for Release 13.

If Proposal 2 is agreed, then MediaTek would be happy to draft a suitable LS to RAN4.

# 2 Conclusion

In this contribution, we discuss the issue of measurement reporting for WLAN. Our proposals and observations are summarized below.

Observation 1: Release 13 specifications appear to support periodic WLAN measurements even though the topic was never discussed.

Proposal 1: RAN2 is requested to confirm that periodic WLAN measurements are not supported for Release 13 LWA, LWIP, and RCLWI features.

Proposal 2: RAN2 is requested to inform RAN4 that periodic WLAN measurements are not supported for Release 13.

# 4 References

[1]. 3GPP TS 36.331, RRC specification, Release 13

[2]. 3GPP TS 36.133, Requirements for support of radio resource management, V14.0.0