3GPP TSG-RAN2 Meeting #95 	R2-165348

# Gothernburg, Sweden, 22nd – 26th August 2016

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | CRNum | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for skipped padding transmission (and?) short SPS intervals |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LATRED_L2 |  |  |  |  |  |  | Date: |  | 2016-08-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Capability introduction for: - L2 enhancement to SPS to allow for periodic UL grants every TTI (Fast UL) providing means to reduce the latency of the first UL transmission. - UEs to skip UL transmissions on configured and dynamic uplink grants if no data is available for transmission in the UE buffer. With frequent UL grants, decreased padding transmission may decrease UL interference and improve UE battery efficiency with reduced UP latency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New MAC fields for capability are introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scheduling enhancements in L2 does not facilitate capability signalling for the use shorter SPS intervals and dynamic scheduling with decreased UL interference and improved UE battery efficiency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.19x, 4.3.19y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### --- Sections Omitted ---

### 4.3.19	MAC parameters

#### 4.3.19.1	longDRX-Command-r12

This field defines whether the UE supports Long DRX Command MAC Control Element as specified in TS 36.321 [4]. It is mandatory for UEs of this release of the specification.

#### 4.3.19.2	logicalChannelSR-ProhibitTimer-r12

This field defines whether the UE supports the logicalChannelSR-ProhibitTimer as specified in TS 36.321 [4]. It is mandatory for UEs of any ue-Category-NB to support this feature.

#### 4.3.19.3	extendedMAC-LengthField-r13

This field defines whether the UE supports 16 bit length of MAC L field as specified in TS 36.321 [4].

#### 4.3.19.4	extendedLongDRX-r13

This field defines whether the UE supports the longDRX-Cycle values of 5120 and 10240 subframes as specified in TS 36.321 [4].

4.3.19.x		skipUplinkSPS-r14

This field defines whether the UE supports skipping UL transmissions for configured uplink grants as specified in TS 36.321 [4]. A UE that supports skipUplinkSPS-r14 shall also support configured Semi-persistent scheduling intervals shorter than 10 sub-frames.

4.3.19.y		skipUplinkDynamic-r14

This field defines whether the UE supports skipping PDCCH indicated uplink grants as specified in TS 36.321 [4].

### --- End change ---