3GPP TSG-RAN WG2 Meeting #95bis															 R2-166554

Kaohsiung, 10th – 14th October 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR |  | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the usage of exceptional pool |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Core |  |  |  |  |  |  | Date: |  | 2016-10-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For the RLF scenario, it is stated that the UE uses the exceptional pool of the re-establiment cell based on sensing in the approved CR “RP-161746”. However, the cell on which the UE initiates connection re-establishment may be the pcell at which the UE detected radio link failure, or be the target cell in the case of handover. As agreed in RAN2#95 meeting, pre-sensing on the exceptional pool is used when exceptional pool is provided by serving cell, and random selection is used when the exceptional pool is provided by the target cell. As a result, it is reasonable that the UE uses the exceptional pool based on random selection when T301 is running if the re-establishment cell is the target cell, no matter whether the UE has already obtained the target cell’s exceptional pool by HO command message or not before the initiation of the re-establishment procedure. In addition, it is agreed in RAN2#95 that pre-sensing on the exceptional pool is used for exceptional pool provided by serving cell in the following three scenarios: idle-to-connected, RLF cases, RRC reconfig. So it is appropriate to move the Note 1 to the bottom of the section in order to cover all the three scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. It is clarified that pre-sensing on the exceptional pool when T301 is running is used only if the cell on which the UE initiates connection re-establishment is the Pcell of the UE. if the re-establishement cell is not the Pcell of the UE(e.g. the target cell in case of handover), the UE uses the exceptional cell based on random selection. 2. Move Note 1 to the bottom of the section in order to cover all the three scenarios (i.e. idle-to-connected, RLF cases, RRC reconfig). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of exceptional pool is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.y.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.10.y	V2X sidelink communication transmission

5.10.y.1	Transmission of V2X sidelink communication

A UE capable of V2X sidelink communication that is configured by upper layers to transmit V2X sidelink communication and has related data to be transmitted shall:

1>	if the conditions for sidelink operation as defined in 5.10.1d are met:

2>	if in coverage on the frequency used for V2X sidelink communication as defined in TS 36.304 [4, 11.4]; or

2>	if the frequency used to transmit V2X sidelink communication is included in v2x-InterFreqInfoList in RRCConnectionReconfiguration or in v2x-InterFreqInfoList within SystemInformationBlockType21:

3>	if the UE is in RRC_CONNECTED:

4>	if the UE is configured, by the current PCell with commTxResources set to scheduled:

5>	if T310 or T311 is running; and if the PCell at which the UE detected physical layer problems or radio link failure broadcasts SystemInformationBlockType21 including v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon; or

5>	if T301 is running and the cell on which the UE initiated connection re-establishment is the Pcell of the UE and broadcasts SystemInformationBlockType21 including v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon; or

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6] and TS 36.213 [23];

NOTE 1:	The UE is assumed to perform sensing on the resources of the pool (as defined in TS 36.213 [23]) indicated by v2x-CommTxPoolExceptional within SystemInformationBlockType21, before it transmits using the pool of resources.

5>	if T301 is running and the cell on which the UE initiated connection re-establishment is not the Pcell of the UE and broadcasts SystemInformationBlockType21 including v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon;

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on random selection using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6] and TS 36.213 [23];

5>	if T304 is running and the UE is configured with v2x-CommTxPoolExceptional included in mobilityControlInfoV2X in RRCConnectionReconfiguration:

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on random selection using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6];

5>	else:

6>	configure lower layers to request E-UTRAN to assign transmission resources for V2X sidelink communication;

4>	else if the UE is configured with v2x-commTxPoolNormalDedicated in RRCConnectionReconfiguration:

5>	if the time required by the physical layer in TS 36.213 [23] for sensing on the resources configured in v2x-CommTxPoolNormalDedicated is not met;

6>	if v2x-CommTxPoolExceptional is included in mobilityControlInfoV2X in RRCConnectionReconfiguration (i.e., handover case);

7>	configure lower layers to transmit the sidelink control information and the corresponding data based on random selection using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6];

6>	else if the Pcell broadcasts SystemInformationBlockType21 including v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon;

7>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing using the pool of resources indicated by v2x-CommTxPoolExceptional as defined in TS 36.321 [6] and TS 36.213 [23];

5>	else:

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using one of the resource pools indicated by v2x-commTxPoolNormalDedicated, which is selected according to 5.10.y.2;

3>	else (i.e. V2X sidelink communication in RRC_IDLE or on cell other than PCell in RRC_CONNECTED):

4>	if the cell chosen for V2X sidelink communication transmission broadcasts SystemInformationBlockType21:

5>	if SystemInformationBlockType21 includes v2x-CommTxPoolNormalCommon in sl-V2X-ConfigCommon:

6>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using one of the resource pools indicated by v2x-CommTxPoolNormalCommon, which is selected according to 5.10.y.2;

5>	else if SystemInformationBlockType21 includes v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon:

6>	from the moment the UE initiates connection establishment until receiving an RRCConnectionReconfiguration including sl-V2X-ConfigDedicated or until receiving an RRCConnectionRelease or an RRCConnectionReject;

7>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using the pool of resources indicated in v2x-CommTxPoolExceptional;

2>	else (i.e. out of coverage on sidelink carrier):

3>	configure lower layers to transmit the sidelink control information and the corresponding data based on sensing (as defined in TS 36.321 [6] and TS 36.213 [23]) using one of the resource pools indicated by v2x-CommTxPoolList in SL-V2X-Preconfiguration, which is selected according to 5.10.y.2, and in accordance with the timing of the selected reference as defined in 5.10.8;

NOTE 1:	The UE is assumed to perform sensing on the resources of the pool (as defined in TS 36.213 [23]) indicated by v2x-CommTxPoolExceptional within SystemInformationBlockType21, before it transmits using the pool of resources.

CHANGE END