3GPP TSG-RAN WG2 Meeting #96	R2-167710

Reno, USA, 14 - 18 November 2016

Agenda item:	9.2.2.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	RRC_INACTIVE: Some UP considerations

WID/SID:	FS_NR_newRAT - Release 14

Document for:	Discussion and Decision

# 1	Introduction

In the scope of NR, RRC_INACTIVE state is being studied in RAN2. Some basic characteristics have been agreed during RAN2#95bis meeting [1].

In this contribution, we discuss some user place and DRB aspects related to the RRC_INACTIVE state.

# 2	Interaction with multi-connectivity

## 2.1	Multi-connectivity in NR

In NR, we assume that there will some kind of Multi-connectivity feature as agreed in RAN2#94 meeting [1]:

Agreements

1	 As in LTE, NR shall study lower layer aggregation (e.g. CA-like) and upper layer aggregation (e.g. DC-like)

In this document, we focus on the “DC-like” multi-connectivity. In this configuration, it is safe assume that UE in NR can be configured with three kind of DRB [2]:

-	MCG Bearers

-	SCG Bearers

-	Split Bearers

With the possible addition of “SCG split bearer”.

In the following section, we analyse the UE behaviour in RRC_INACTIVE, and its interaction with multi-connectivity.

## 2.1	Mobility in RRC_INACTIVE

When in RRC_INACTIVE, it is assumed, that for power saving reasons, the UE will perform Cell Selection or other Mobility scheme, but it will do it in one frequency at a time. Indeed this power saving state should not mandate extra UE power consumption

Observation 1: When UE is in RRC_INACTIVE, it will most probably not simultaneously “camp” on more than one frequency layer.

When the UE comes back to RRC_CONNECTED state, it will do it using via the communication to a single cell at least for the first message exchange.

Even if the UE can monitor a second layer during RRC_INACTIVE state, that would be used to establish a second connection (SCG), when the UE is reconfigured to RRC_CONNECTED, this would need some (re-)configuration on network side because the secondary PCell cell may have changed during the time UE is in RRC_INACTIVE.

Observation 2: When UE is reconfigured from RRC_INACTIVE to RRC_CONNECTED state, it will probably be connected to one cell only for the first message exchange.

According to the analysis above, the UE will not use Multi-Connectivity during at the moment of its reconfiguration to RRC_CONNECTED. It could be configured again at a later stage but will require the configuration of the secondary Link in “SgNB”.

As a consequence, when moved to RRC_INACTIVE UE should be de-configured from Multi-connectivity. This mean that the split and SCG bearer would need to be reconfigured to MCG bearer. This could take some time and delay the reconfiguration of UE to RRC_INACTIVE. Furthermore, the merging of the two procedures (transition to RRC_INACTIVE and reconfiguration of split/SCG bearers) would introduce more complexity in the specifications. For the sake of simplicity, we propose to decouple those issues and assume that UE is not configured with multi-connectivity when send to RRC_INACTIVE. It would be up to the network to prepare the UE for the state transition.

Proposal 1: In the first version of the specification, UE should not be configured with multi-connectivity when sent to RRC_INACTIVE.

# 3	Buffer

The second aspect that is discussed in this contribution is the state of the Layer 2 buffers, when UE is sent to RRC_INACTIVE.

As a first assumption, UE will not communicate with the network during RRC_INACTIVE state. In this case if some data is present in the buffer of the UE, it will request a transition back to RRC_CONNECTED to send the data. Thus, it does not make sense to send the UE in RRC_INACTIVE when its buffer are not empty.

But it could be possible that UE is capable of sending and receiving data in RRC_INACTIVE (in a similar way to CELL_FACH in UMTS for example). In this case, UE could still transmit data, but most probably, the efficiency will be lower than in RRC_CONNECTED and only small amount could be transferred. Consequently, if the UE buffer contains a large amount of data, it would be not wise to transfer it to RRC_INACTIVE.

Observation 3: It is not wise to send UE to RRC_INACTIVE if UE has data in its PDCP buffer.

# 4	Conclusion

In this contribution, we have discusses some UP aspects of RRC_INACTIVE state and we have maded the following proposals and observations:

Observation 1: When UE is in RRC_INACTIVE, it will most probably not simultaneously “camp” on more than one frequency layer.

Observation 2: When UE is reconfigured from RRC_INACTIVE to RRC_CONNECTED state, it will probably be connected to one cell only for the first message exchange.

Proposal 1: In the first version of the specification, UE should not be configured with multi-connectivity when sent to RRC_INACTIVE.

Observation 3: It is not wise to send UE to RRC_INACTIVE if UE has data in its PDCP buffer.

# References

- Draft Report of RAN2#95bis meeting

- R2-164670, Report of RAN2#94 meeting

- R2-167046, Upper layer aggregation for NR-NR multi-connectivity, Nokia, Alcatel-Lucent Shanghai Bell