3GPP TSG-RAN WG2 Meeting #97																R2-1700778

Athens, Greece, February 13 - 17, 2017

Title: 	The performance evaluation of non-orthogonal MA in eV2X

Source: 	ZTE

Agenda item:	8.13.9

Document for:	Discussion

Introduction

According to the LTE based V2X WID [1], the core part WI is planned to be 100% complete in March 2017. After that, the study of eV2X will be started followed. Currently, the requirement study of eV2X has been almost completed in SA1. According to outputted TR 22.886 [2], 25 use cases and corresponding service requirements are identified to support enhanced V2X services, which are featured by low latency, high reliability, high data rate and variable transmission range. There are two ways to support these advanced V2X applications, one way is eV2X enhancement based on LTE, another way is eV2X evolution based on NR.

Considering the technologies studied in NR, non-orthogonal multiple access (non-orthogonal MA) is an attractive technology which may be used in eV2X both based on NR or LTE. In this paper, we firstly analyze the benefits of application of non-orthogonal MA to eV2X to fulfill the requirement of eV2X and then provide some system level simulation results.

Enhanced eV2X communication with non-orthogonal MA

Technical considerations

In TR 22.886, high density of congested traffic eV2X use case is considered [2] as follows:

[PR-5.5-010] The 3GPP system shall support high connection density for congested traffic. Example estimate is for worst case US Freeway scenario that does not include arterial roads (i.e. onramps): 5 lanes in each direction or 10 lanes total per highway, for up to 3 highways intersecting = 15,840 cars per mile.

In the high density use case, resource collision is the main cause of performance degradation when UEs select resource autonomously. In Rel-14 LTE V2X, sensing based resource selection is introduced in Mode 4 to solve this problem, but it leads to more time delay and complexity, and the resource collision may still exist.

Meanwhile, the low latency use cases are also considered in TR 22.886 [2]:

[PR.5.6-004] The 3GPP system shall be able to support [3ms] end-to-end latency and [99.999%] reliability for message transfer among a group of UEs supporting V2X application within [200m] communication range. [PR.5.6-005] The 3GPP system shall be able to support [10ms] end-to-end latency and [99.99%] reliability for message transfer among a group of UEs supporting V2X application within [500m] communication range. [PR.5.6-006] The 3GPP system shall be able to support [50ms] end-to-end latency and [99%] reliability for message transfer among a group of UEs supporting V2X application within [1000m] communication range.

With regard to the high density and low latency requirement in eV2X, grand-Free non-orthogonal MA (Spreading-based non-orthogonal multiple access) can be considered to increase the multiplexing number of users at the same time and avoid resource collision as used in massive machine-type communication (mMTC) and ultra-reliable and low latency communications (URLLC). The summary of simulation results for NR MA is listed in [3]. Non-orthogonal MA and multi user detection (MUD) e.g., SIC-based multiple user detection may be used to meet these extremely high requirements in eV2X.

The benefit from non-orthogonal sequence is more UEs can be multiplexed on the same resources. The system capacity will increase significantly with such simultaneous transmissions. Although the same resource is used, the Tx UEs can be distinguished by the code domain, power domain, and spatial domain signatures, which is helpful for the resource selection in Mode 4 (autonomous resource selection by UE self). Due to that, it is no longer needed to wait for the transmission occasion compared to sensing based resource selection in Rel-14 V2X. As we can see, the latency can be significantly reduced if non-orthogonal MA is used to enhance the sidelink autonomous resource selection in eV2X.

Another notable feature of non-orthogonal MA is the SIC based MUD algorithm. It is very suitable for the V2X scenarios in which many UEs broadcast at the same time, so the receiver needs to receive the data from multiple transmit sources. Normally these transmit sources have different distances to the receiver and the Near-Far Effect is no longer a disadvantage to the SIC receiver.  Even if more than one UE are collided on the same resources, there is a great probability to correctly decode these UEs data. In other words, it is robust to resource collision using non-orthogonal MA.

Some system level evaluation results are present in section 2.2. Taking MUSA (Multiple User Shared Access) as an example, the performance of legacy LTE V2X and non-orthogonal MA enhanced eV2X is compared. From the evaluation results we can see that the packet reception ratio (PRR) and the latency performance of non-orthogonal MA enhanced eV2X are significantly higher than that of legacy LTE based V2X.

Observation 1: Non-orthogonal MA can be beneficial in increasing the packet reception rate and reducing the latency for eV2X.

Simulation and evaluation

In this section, taking MUSA as an example, two schemes are compared by system level simulation:

Scheme 1: LTE based V2X with sensing.

Scheme 2: non-orthogonal MA enhanced eV2X.

The simulation assumptions are shown in Table A1 in the appendix, others are described in [4]. For all cases, the message packet is assumed as 190 bytes and the transmission number is fixed to 2. For scheme 1, the UE performs the sensing based autonomous resource selection and uses one sub-channel for each packet transmission. For scheme 2, the non-orthogonal sequence length is 4, (i.e. there are 64 sequences) and the UE randomly selects one among them. The UE uses four sub-channels for each packet transmission. The MMSE-SIC receiver is used for non-orthogonal MA and ideal channel estimation is assumed. So if one UE is detected successfully, the interference caused by this UE could be cancelled perfectly.

The following figures (Figure 1, Figure 2 and Figure 3) are the performance of PRR in three V2X scenarios. From the three figures, we can see that the performance of non-orthogonal MA enhanced V2X is significantly higher than that of legacy LTE based V2X in both in freeway and urban scenarios. By comparing Figure 1 and Figure 2, we can see that the denser the vehicles, the higher gains are obtained though non-orthogonal MA. For these interference limited scenarios in V2X, by Spreading-based non-orthogonal MA and SIC-based MUD, while one UE is detected successfully, the interference caused by this UE probably can be eliminated. Spreading-based non-orthogonal MA and SIC-based MUD are beneficial for the high density V2X scenarios and could bring great performance gains.

Observation 2: The PRR performance of non-orthogonal MA enhanced V2X is significant better than that of LTE based V2X, especially in high density scenario.

Figure 1: PRR of freeway scenario at 140km/h absolute speed

Figure 2: PRR of freeway scenario at 70km/h absolute speed

Figure 3: PRR of urban scenario at 60km/h absolute speed

In addition, the latency is evaluated. As shown in Figure 4, two schemes are compared. In scheme 1, it is assumed that the UE selects resources in 100ms selecting window after sensing in 1000ms sensing window. In scheme 2, to randomized the interference and mitigate the limitation of half duplex, the UE sends the initial transmission immediately when the packet is generated and randomly chooses another subframe for the second transmission in the range of [1-5]. The delay is calculated from the time that the packet is generated to the time the packet is received by Rx-UE successfully.

Figure 4: The latency CDF of two schemes

From Figure 4, we can see that the latency of non-orthogonal MA enhanced eV2X is decreased significantly while the PRR is still better than legacy LTE based V2X.

Observation 3: The latency of non-orthogonal MA enhanced eV2X is decreased significantly while the PRR is still better than that of LTE based V2X.

In a sum, according to the simulation results, the scheme of non-orthogonal MA enhanced eV2X can be a good choice for eV2X study. Considering that the non-orthogonal MA may be studied in NR in the future, the non-orthogonal MA enhanced eV2X may be further studied in NR based eV2X.

Proposal 1: non-orthogonal MA should be considered in eV2X study.

Conclusion

In this paper, the application of non-orthogonal MA in eV2X is analysed considering the requirement of eV2X and some system level simulation results were also presented. The following observations and proposal are given:

Observation 1: Non-orthogonal MA can be beneficial in increasing the packet reception rate and reducing the latency for eV2X.

Observation 2: The PRR performance of non-orthogonal MA enhanced V2X is significant better than that of LTE based V2X, especially in high density scenario.

Observation 3: The latency of non-orthogonal MA enhanced eV2X is decreased significantly while the PRR is still better than that of LTE based V2X.

Proposal 1: non-orthogonal MA should be considered in eV2X study.

References

[1] 3GPP RP-162191, V2X SR to RAN74, RAN#74, Vienna, December 5 - 8, 2016.

[2] 3GPP TR 22.886 V1.0.0, Sep. 2016.

[3]  3GPP R1-1611251, Summary of simulation results for NR MA, RAN1#87，Huawei, HiSilicon.

[4] 3GPP TR 36.885 V2.0.0, Jun. 2016.

- Appendix

The detail simulation assumptions are provided in Table A1:

Table A1: Simulation parameters

| Parameter | Value |
| --- | --- |
| System bandwidth | 10MHz |
| Carrier Frequency | 5.9GHz |
| Tx power | 23 dBm |
| Noise figure | 9 dB |
| Vehicle Antenna Number | 1 TX and 2 RX |
| IBE model | {3,6,3,3} |
| Thermal noise level | –174 dBm/Hz |
| Modulation | 16QAM |
| Sub-channel size | 10 RB |
| Sub-channel number | 4 |