3GPP TSG-RAN WG2 #97	Tdoc R2-1700841

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.1.1

Source:	Ericsson

Title:	RLC transmission modes for Upper layer aggregation

Document for:	Discussion, Decision

# Introduction

RAN2#94 has agreed that both lower layer aggregation (e.g., CA-like) and upper layer aggregation (e.g., DC-like) shall be studied for the aggregation of NR carriers. RAN2#94 has also agreed that only DC approach for LTE-NR aggregation will be studied. In the RAN2#95bis, it was agreed to support ARQ in RLC for NR. In the RAN#74 plenary it was also decided that no dedicated meeting time in the WGs will be allocated until March 2017 for

  - Multi-connectivity constituted with more than two nodes (gNBs and/or eNBs)

    - I.e., Dual Connectivity is continued to study in the SI phase.

In this paper, we discuss RLC transmission modes in NR and DC-like aggregation for multi-connectivity. We also provide a text proposal for TR 38.804 to conclude the RLC transmission modes and upper layer aggregation topics.

# Discussion

## RLC transmission modes in NR

In LTE, RLC supports different transmission modes. Among these transmission modes, RLC AM (acknowledgement mode) provides correction to the residual failures from lower layer, i.e., HARQ. The RLC AM PDU includes sequence number and supports status reports to achieve reliable transmission. RLC AM is suitable for carrying TCP traffic. RLC UM (unacknowledged mode) does not guarantee the delivery, but can be beneficial for latency reduction and is therefore more suitable for delay-sensitive services. RLC TM (transparent mode) is transparent to the packet data units delivered by higher layer. It does not add any process or packet overhead. RLC TM is suitable to transmit RRC messages such as broadcast System Information (SI) messages, paging messages, and RRC messages sent via SRB0.

It has been agreed that NR supports ARQ in RLC. For the same reasoning as that in LTE, it is natural to support all three modes for RLC in NR (as suggested in [1]).

NR RLC supports three transmission modes, i.e., AM, UM and TM.

## RLC transmission modes for DC-like MC in NR

For LTE DC, only RLC AM mode is supported for split bearers. As PDCP reordering is always enabled in NR (i.e. even in non-DC case), RLC UM can be used for Multi-Connectivity split bearers (MCG/SCG split bearers) in NR.

RLC AM ARQ improves the reliability by correcting residual HARQ errors, but this results in additional retransmission delays and increased overhead due to the use of RLC status reports. With duplicate transmissions, this retransmission delay can be avoided (to the expense of lower resource efficiency) by using RLC UM.

In general, the use of RLC UM must be complemented with additional means to provide a high transmission reliability; especially for services that requires this. The usage of duplicated transmission at PDCP is one example, in terms of a balance between the reduction of UP latency and the fulfilment of the transmission robustness requirements.

For DC-like MC split bearers (MCG/SCG split bearers), the usage of RLC UM is beneficial to reduce UP latency but may need other complemental means to ensure the transmission reliability.

The DC-like MC split bearers (MCG/SCG split bearers) support RLC UM mode besides RLC AM mode.

## Text proposal to TR 38.804 on RLC Sublayer

Below texts should be captured by TR 38.804, in a suitable section.

Adopt the text proposal on “RLC Sublayer” to TR 38.804

5.4.2 RLC Sublayer This subclause provides an overview on services, functions and PDU structure provided by the RLC sublayer in NR. Note that: - The reliability of RLC is configurable: some radio bearers may tolerate rare losses (e.g. TCP traffic); - Radio Bearers are not characterized by a fixed sized data unit (e.g. a fixed sized RLC PDU). ### 5.4.2.1 Services and Functions The main services and functions of the RLC sublayer in NR include: - Transfer of upper layer PDUs; - Supports three transmission modes, i.e., AM, UM and TM - Error Correction through ARQ (only for AM data transfer); - segmentation and reassembly of RLC SDUs (only for UM and AM data transfer); - Re-segmentation of RLC data PDUs (only for AM data transfer); - Reordering of RLC data PDUs (only for UM and AM data transfer); - Duplicate detection (only for UM and AM data transfer); - Protocol error detection (only for AM data transfer); - RLC SDU discard (only for UM and AM data transfer); As PDCP reordering is always enabled in NR (i.e. even in non-DC case), RLC UM can be used for DC-like MC with split bearers (MCG/SCG split bearers) in NR besides RLC AM.

# Conclusion

In section 0 we made the following observations:

Observation 1	For DC-like MC split bearers (MCG/SCG split bearers), the usage of RLC UM is beneficial to reduce UP latency but may need other complemental means to ensure the transmission reliability.

Based on the discussion in section 0 we propose the following:

Proposal 1	NR RLC supports three transmission modes, i.e., AM, UM and TM.

Proposal 2	The DC-like MC split bearers (MCG/SCG split bearers) support RLC UM mode besides RLC AM mode.

Proposal 3	Adopt the text proposal on “RLC Sublayer” to TR 38.804

# Reference

R2-1700498, “RLC modes for NR”, Samsung