3GPP TSG-RAN WG2 #97	 R2-1700998

Athens, Greece, Feb. 13th- Feb. 17th, 2017

Agenda Item:	10.4

Source: 	Xiaomi

Title:  	PRACH partitioning, access and congestion control Consideration for Network Slicing

Document for:	Discussion and Decision

Introduction

During last RAN2 meeting, RAN2 discussed PRACH partitioning, access and overload control mechanisms to support network slicing, but people had different opinions on the motivation of supporting per network slice configuration, especially for access barring. Thus, no conclusion was made:

FFS whether it is possible to provide different PRACH, access barring and congestion control information for different slices.

In this contribution, we will give our view on the motivation and feasibility of supporting different PRACH, access and congestion control information for different network slices.

Discussion

PRACH partitioning

In LTE, PRACH resources (time, frequency, preamble) are common configured for idle mode UE, and configured through dedicated signalling for SCell, handover. For network slicing, PRACH resources partitioning will help avoid impact of access overload between different slices. RAN can indicate PRACH-config/RACH-config per network slice in SIB and dedicated signalling. Upon RRC connection establishment, to enable UE initiate random access using PRACH resources of a specific network slice, UE NAS needs to inform AS about the network slice that trigger the RRC connection establishment. For connected/inactive state UE, DRBs are mapped to network slices. Random access triggered by DRBs of a specific network slice may also use PRACH resources of that network slice. How to map random access triggered by SRBs to PRACH resources of network slices may need further consideration.

Observation: To support PRACH partitioning, NAS needs to inform AS about about the network slice that trigger the RRC connection establishment.

Proposal 1: PRACH partitioning is supported for network slicing for idle, inactive state and connected UE.

Access Barring

Access barring is used in LTE to prevent certain UEs from accessing the network to solve the load condition. Access barring mechanisms introduced in LTE include ACB, EAB, SSAC, ACDC, UDT, which are used to address different service requirements, e.g. MTC, NB-IoT, MMTEL voice, MMTEL video, emergency call, etc. In last RAN2 meeting, RAN2 agreed to specify one unified access barring mechanism for NR that can address all the use cases and scenarios defined in LTE, and applicable for all RRC states in NR. The unified access barring mechanism may need to have the granularity of service type and can control the access of each service type separately. For network slicing, SA2 has agreed that different network slices may be used to transmit the same service but to different group of UEs [1]. That is, the granularity of service type can not set apart the same services from different network slices. Since the overload situation in different network slices may be different, different access barring parameters may be applied. For this purpose, access barring should be able to configure different access barring parameters for different network slices.

Proposal 2: Access barring should be able to support network slice specific configuration.

Congestion control

The congestion control mechanisms include random access backoff indicator, RRC connection release, RRC connection reject. In LTE, backoff indicator is included in RAR. If NR follows this method and adopt PRACH partitioning for network slicing, network can know which network slice triggers the random access according to the PRACH resources the UE uses. Then the network can set the backoff indicator accordingly.

RRC connection reject bases on the establishment cause value contained in RRC connection request message. Establishment cause value includes emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess, mo-VoiceCall. Which type of services will be rejected depends on the load condition. In some cases, maybe only delayTolerantAccess will be rejected. In some other cases, even mo-data can be rejected. For network slicing, since different network slices are isolated, their load condition may be different. It is possible that different network slices will reject different sets of services. Therefore, network needs to know which network slice initiate the RRC connection establishment. PRACH partitioning is one way to provide this information.

Proposal 3: Backoff indicator and RRC connection reject can base on PRACH partitioning for the handling of different network slices. There is no need for slice specific backoff indicator and network slice indication in the establishment cause.

RRC connection release is used to release the established connection when congestion occurs. It is the last step. Before this, network may use RRC connection reconfiguration to release some low priority DRBs in a congested network slice or move them to another slice. In release message, network can tell UE the time it needs to wait before initiating a connection establishment request. LTE introduces the extended wait time for delay tolerant access. For NR, such mechanism can also be used. Whether it can be used for non delay tolerant access is unclear. If configuring the wait timer per service is necessary, there is no need to differentiate services from different network slices, since there will not be different network slices handle the same service in one UE.

Proposal 4: The use case of different RRC connection release information for different network slices is unclear.

Conclusions

Based on the discussion, we have the following observations and proposals:

Observation: To support PRACH partitioning, NAS needs to inform AS about about the network slice that trigger the RRC connection establishment.

Proposal 1: PRACH partitioning is supported for network slicing for idle, inactive state and connected UE.

Proposal 2: Access barring should be able to support network slice specific configuration.

Proposal 3: Backoff indicator and RRC connection reject can base on PRACH partitioning for the handling of different network slices. There is no need for slice specific backoff indicator and network slice indication in the establishment cause.

Proposal 4: The use case of different RRC connection release information for different network slices is unclear.

References

[1] TR23.799 Study on Architecture for Next Generation System.