3GPP TSG RAN WG2 #97	R2-1701992

Athens, Greece, 13th to 17th February 2017 	                Resubmission of R2-1700601

Agenda Item:		10.2.1.2

Souce:		Samsung

Title:		Logical Channel Prioritization Considering Multiple

Numerologies and/or TTIs

Document for:	Discussion/Decision

Introduction

In the RAN2 NR ad-hoc meeting on January 2017, the following agreements about supporting multiple numerologies were made.

Agreements - A single logical channel can be mapped to one or more numerology/TTI duration. - ARQ can be performed on any numerologies/TTI lengths that the LCH is mapped to. - The RLC configuration is per logical channel without dependency on numerology/TTI length. - Logical channel to numerology/TTI length mapping can be reconfigured via RRC reconfiguration. - RAN2 will leave RAN1 to decide whether HARQ retransmission can be performed across different numerologies and/or TTI durations. - Wait for more details from RAN1 to decide whether HARQ configuration, if any, needs to be numerology/TTI duration specific. - A single MAC entity can support one or more numerology/TTI durations. - LCP takes into account the mapping of logical channel to one or more numerology/TTI duration. Details of LCP will be discussed in the WI phase.

In this contribution, we will investigate an impact of multiple numerologies and/or TTIs on LCP (logical channel prioritization).

Motivation of LCP Enhancement

For easy discussion, we have the following assumptions throughout this contribution.

- A UE is simultaneously using multiple services denoted by S1 and S2. - The services, S1 and S2, mainly use different numerologies, N1 and N2, respectively. - This does not preclude that S1 (S2) can be transmitted on N2 (N1). - S1 and S2 share common radio resources (i.e., time and frequency resources). - The TTI length of N2 is shorter than that of N1 (i.e., TTI1 > TTI2). - S2 requires shorter latency than S1. - The UE currently has UL traffic of S1 and S2 in its buffer. - Each service corresponds to a set of logical channels.

If we met the similar situation in LTE, the UE would perform UL scheduling based on the current LCP procedure [1]. However, it is not designed by considering multiple numerologies and/or TTIs with variable duration. To overcome this limitation, we need to study how the LCP in LTE should be enhanced for NR.

For this purpose, we consider different HARQ timelines according to different numerologies/TTIs. If we apply our assumptions to the situation in Fig. 1, the following two operations of the UE can be considered.

Figure 1 Example of using different numerologies/TTIs at a UE

Case 1

At T1, the UL resource with TTI1 is allocated to the UE.

- The UE puts all of the UL traffic of S1 and S2 into the allocated resource with TTI1.

  - We herein assume that the size of the allocated resource is enough.

Case 2

At T1, the UL resource with TTI1 is allocated to the UE.

- The UE only puts the UL traffic of S1 into the allocated resource with TTI1.

  - The UE does not put the UL traffic of S2 into the allocated resource with TTI1.

At T2, the UL resource with TTI2 is allocated to the UE.

- The UE puts the UL traffic of S2 into the allocated resource with TTI2.

  - Note that Case 2 is the same as the case where S1 and S2 use separate radio resources.

We now compare these two cases from the latency perspective. Fig. 1 shows that the resource with a longer HARQ timeline is allocated earlier than that with a shorter one. Then, how does the UE use these resources to transmit the UL traffic of S1 and S2 that have different latency requirements?

If the UE does not know the fact that the resource with a shorter HARQ timeline will be allocated within a certain deadline, it is the best choice to put all of the UL traffic of S1 and S2 into the currently available resource as early as possible.

On the other hand, if the UE knows the fact that the resource with a shorter HARQ timeline will be allocated within a certain deadline, it is the best choice to put the UL traffic of each service into the resource that is mainly used for the service. Accordingly, the UL traffic of S2 can be delivered earlier than that of S1 although it is transmitted later. This phenomenon is illustrated in Fig. 1.

However, it is difficult for a UE to know future scheduling information in advance (e.g., knowing at T1 whether the resource with a shorter HARQ timeline will be scheduled within a certain deadline or not). As a result, it can be a reasonable approach that a pre-defined LCP rule reflects not only the priority among logical channels but also characteristics of the allocated resources, such as numerologies, TTIs or HARQ timelines.

Observation 1: When multiple numerologies and/or TTIs are simultaneously used by a UE, the following situation can happen: the UL resource with a longer HARQ timeline is allocated earlier than that with a shorter HARQ timeline. The UE should have means of how to use these resources to transmit its UL traffic.

Possible Options for LCP Enhancement

To handle the situation explained in Section 2, we investigate the following two options.

Option 1

Different priorities among logical channels are applied to the resources with different numerologies and/or TTIs. Fig. 2 shows an example of this option.

- If the UE is assigned to the resource with TTI type A, the logical channels associated with service S1 has a higher priority than those associated with service S2.

  - That is, the logical channel priority of 1 > 2 > 3 > 4 is applied.

- If the UE is assigned to the resource with TTI type B, the logical channels associated with service S2 has a higher priority than those associated with service S1.

  - That is, the logical channel priority of 3 > 4 > 1 > 2 is applied.

Figure 2 Different logical channel priorities for each numerology/TTI

Option 2

This option allows the gNB to dynamically control the priority among logical channels, if needed.

- The UE has a default priority among logical channels, for instance, 1 > 2 > 3 > 4.

- When sending UL grant to the UE, the gNB indicates the logical channel that has the highest priority for this UL grant.

- The UE adjusts the default priority according to the indication from the gNB. Fig. 3 shows an example of this operation.

Figure 3 Dynamic indication of logical channel priority from gNB

The pros and cons of these options are summarized in Table 1.

Table 1

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | Semi-static rules configured by RRC are used so that complexity is not significantly increased compared to LTE. | Multiple sets of priorities among logical channels are needed to be configured for each numerology and/or TTI. |
| Option 2 | The gNB can control the priority among logical channels to satisfy QoS of the UE. | The dynamic indication via UL grant increases the control channel overhead. |

Proposal 1: The LCP in LTE should be considered as a baseline of LCP discussion in NR.

Proposal 2: The LCP in LTE should be enhanced to support multiple numerologies and/or TTIs in NR.

Proposal 3: The LCP in NR should be able to apply different logical channel priorities to different numerologies and/or TTIs.

Proposal 4: A UE should be able to adapt logical channel priorities based on an indication of priority information from a gNB.

Conclusions

Observation 1: When multiple numerologies and/or TTIs are simultaneously used by a UE, the following situation can happen: the UL resource with a longer HARQ timeline is allocated earlier than that with a shorter HARQ timeline. The UE should have means of how to use these resources to transmit its UL traffic.

Proposal 1: The LCP in LTE should be considered as a baseline of LCP discussion in NR.

Proposal 2: The LCP in LTE should be enhanced to support multiple numerologies and/or TTIs in NR.

Proposal 3: The LCP in NR should be able to apply different logical channel priorities to different numerologies and/or TTIs.

Proposal 4: A UE should be able to adapt logical channel priorities based on an indication of priority information from a gNB.

Reference

[1]	3GPP TS 36.321 V13.3.0, “MAC protocol specification (Release 13),” September 2016