3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702740

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.2.3

Source:	Ericsson

Title:	RLC STATUS report format

Document for:	Discussion, Decision

# Introduction

Status report is sent by RLC AM to provide positive or negative acknowledgement of the PDUs. It consists of control PDU header and STATUS PDU payload.

In this contribution we discuss some of the stage-3 aspects of the RLC status report format. Particularly, about the status report byte alignment and if range of non-acknowledged Sequence Numbers should be added to the format.

# Discussion

In LTE, RLC AM status report format is bit-aligned. This reduces the overhead but may have negative impact on memory access efficiency. Having byte alignment rather than bit alignment is more efficient in terms of memory access, which is especially important in high data rate scenarios for both, UE and gNB.

Byte-aligned format has some benefits in processing

Number of reserved bits used in the RLC status report is depending if the bit or byte alignment is used. Additionally, it is depending on the used Sequence Numbers. Some companies have proposed to reduce the SN size options [1]. We also think that reducing SN size options would be a beneficial [2][3] Having less SN size options results fewer header options for transmitter and receiver, which can be optimized to reduce the number of Reserved-bits in the header. Having fewer options for supported SN sizes also reduces the implementation complexity.

Figure 1 shows RLC status report format with examples of acknowledged and non-acknowledged header formats. When only the cumulative ACK SN is added (1a, 1c) is used, the overhead caused by the byte-alignment is one bit. When status report has a list of “NACK_SN:s” or “NACK_SN:s with segmentation information”, the overhead is 3 bits (1b, 1d). We think that the overall overhead caused by byte-alignment in the RLC status report is not significant, when considering that the longer format with NACK_SN:s is sent only when one or more SN:s are missing.

Figure 1: Bit aligned RLC status report formats including NACK_SN_RANGE

Overhead due to byte alignment is not significant compared to the size of the status report and taking into account that status reports are rare compared to the number of data PDUs.

RLC status report format is byte-aligned

Bit-shifting of fields (NACK_CN, ACK_SN and SO) should be minimized in the RLC status PDU.

Each status PDU sub-block should use only one bit-shifting pattern which is achieved by byte-aligning each status sub-block (ACK_SN, NACK_SN, NACK_SN+SO).

In RAN2 #97-AH, the working assumption of removal RLC concatenation was agreed. As a consequence, there are as many RLC SN:s in flight as there are PDCP PDU:s (unless DC is used). If one MAC PDU including multiple RLC SN:s gets lost, the RLC status report size is increased compared to LTE since many NACKED SN:s are bundled to the status report. To overcome this limitation, we suggest introducing the possibility to report a range of SNs as being lost (NACK_SN_RANGE). The NACK_SN_RANGE includes information about the adjacent non-acknowledged Sequence Numbers. When many consecutive SN:s have been missed, the receiver may request those SN:s by adding the SN range to the status report. As can be expected, using NACK_SN_RANGE would also reduce the over-the air overhead since less SN:s with length of 12-bit or 18-bit  are needed in the header.

Using the NACK SN range reduces the RLC PDU status report size.

Introduce a “NACK SN range” as additional status report format.

Study further optimizations and the use of fields in RLC status report.

# Conclusion

In section 2 we made the following observations:

Observation 1	Byte-aligned format has some benefits in processing

Observation 2	Overhead due to byte alignment is not significant compared to the size of the status report and taking into account that status reports are rare compared to the number of data PDUs.

Observation 3	Using the NACK SN range reduces the RLC PDU status report size.

Based on the discussion in section 2 we propose the following:

Proposal 1	RLC status report format is byte-aligned

Proposal 2	Bit-shifting of fields (NACK_CN, ACK_SN and SO) should be minimized in the RLC status PDU.

Proposal 3	Each status PDU sub-block should use only one bit-shifting pattern which is achieved by byte-aligning each status sub-block (ACK_SN, NACK_SN, NACK_SN+SO).

Proposal 4	Introduce a “NACK SN range” as additional status report format.

Proposal 5	Study further optimizations and the use of fields in RLC status report.

# References

R2-1701987. PDCP/RLC Sequence Number Size for NR. Samsung

R2-1702741. PDCP SN. Ericsson

R2-1702739. RLC PDU design. Ericsson