3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702793

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.4.1

Source:	Ericsson

Title:	New AS layer PDU design

Document for:	Discussion, Decision

# Introduction

In RAN2#96, the following agreements were made:

1: In DL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping (confirmation of SA2 agreement status).

2. DL packets over Uu are marked in band with QOS-flow-id for the purposes of reflective QoS

3. For reflective QoS, the UE determines QoS Flow ID to DRB mapping in the uplink based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

4. The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly.

The RAN2#97-AH agreed the following:

Agreements

1: A new user plane AS protocol layer (e.g. PDAP) above PDCP should be introduced to accommodate all the functions introduced in AS for the new QoS framework, including:

-	QOS flow->DRB routing;

-	QoS-flow-id marking in DL packets;

-	QoS-flow-id marking in UL packets;

2	The new protocol layer is applicable for all cases connecting to the 5G-CN

3:	Single protocol entity is configured for each individual PDU session.

# Discussion

RAN2-97-AH agreed to have new AS layer above PDCP layer. Purpose of the new layer is to accommodate QoS functions, for instance, the flow ID field in the reflective QoS concept. As the new layer is above the PDCP layer, therefore, we think that the correct position of the new AS layer header would be after the PDCP header. In other words, the new AS layer SDU is PDCP PDU.

Figure 1: New AS layer header placement

New AS layer PDU is PDCP SDU

RAN2-96 discussed whether the flow_id field should always be present in UL and DL direction or whether the eNB should configure their presence explicitly. In our view, the flow id field should not be mandatory. There are cases when the Flow ID does not need to be conveyed:  these are, for instance, when the eNB configures a dedicated DRB for a Flow (1:1 mapping) and if it configures the Flow to be mapped to the DRB (explicitly configured UL “AS filter”), neither the UE nor the eNB need to include the flow ID. Since this may be a typical case for IMS Voice as well as for latency critical services where the relative overhead due to the Flow-ID is significant, the eNB should have means to configure for each DRB whether the Flow-ID is conveyed in the header.

Flow_id in the new AS header is optional and may not always be configured

AS layer header may include Flow ID depending on network configuration

As the new layer is introduced instead of using PDCP header to convey the flow_id information, the header format should be byte-aligned. Having byte-alignment improves memory access and may, especially in the high data rate scenarios, provide additional performance gain.

AS layer header is byte-aligned

# Conclusion

In section 2 we made the following observations:

Observation 1	Flow_id in the new AS header is optional and may not always be present

Based on the discussion in section 2 we propose the following:

Proposal 1	New AS layer PDU is PDCP SDU

Proposal 2	AS layer header may include Flow ID depending on network configuration

Proposal 3	AS layer header is byte-aligned

# References

R2-1702756 QoS Stage-2 aspects, Ericsson

R2-1700842. QoS framework for NR, Ericsson