3GPP TSG-RAN WG2 NR #97b Meeting                                                    R2-1703455

Spokane, USA 3rd-7th April 2017

Agenda item:	10.4.1.3

Source:	Samsung

Title:	RRM measurement bandwidth in NR

Document for:	Discussion & Decision

# 1. Introduction

RAN#71 in March approved a 5G SID [1] on New Radio (NR) access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

During SI phase in NR, the following agreements were achieved regarding bandwidth considerations for NR.

RAN1 Agreements (#86bis)

- At least for single carrier operation, NR should allow a UE to operate in a way where it receives at least downlink control information in a first RF bandwidth and where the UE is not expected to receive in a second RF bandwidth that is larger than the first RF bandwidth within less than X µs (FFS: value of X)

  - FFS the first RF bandwidth is within the second RF bandwidth

    - FFS the first RF bandwidth is at the center of the second RF bandwidth

  - FFS the maximal ratio of the first RF bandwidth over the second RF bandwidth

  - FFS detailed mechanism

- FFS RF bandwidth adaptation for RRM measurement

For the frequency-domain aspects:

- A UE monitors for downlink control information in one or more “control subband”

  - This does not preclude that UE may receive additional control information elsewhere within or outside the control subband in the same or different OFDM symbol(s)

- A “control subband” is smaller than or equal to the carrier bandwidth (up to a certain limit)

- A “control subband” consists of an integer number of RBs/PRBs in the frequency domain

RAN1 Agreements (#87)

- NR should support adaptation and network indication of the valid time and frequency resources which may be used for inter-/intra-frequency RRM measurements and reports for ‘CONNECTED’ mode UEs

  - FFS: UE-specific and/or cell-specific indication

  - FFS: dynamic adaptation/configuration

  - FFS: idle and possibly new state modes

  - FFS: whether to support aperiodic

- It is up to RAN4 for determining requirements regarding the extent the UE can restrict its measurement to a subband of the configured bandwidth

RAN1 Agreements (#87)

- NR strives for efficient support of dynamic resource allocation of different numerologies in FDM/TDM fashion.

  - Potential specification impact includes but is not limited to:

    - FFS:CSI-RS measurement

  - FFS: the time and frequency granularity of dynamic resource allocation

    - FFS: If spectrum confinement (filtering, windowing, …) can be dynamically varied or not

RAN1 Agreements (AH#1)

- From RAN1 specification perspective, maximum channel bandwidth per NR carrier is [400, 800, 1000] MHz in Rel-15

  - RAN1 recommends RAN4 to consider at least 100 MHz maximum channel bandwidth per NR carrier in Rel-15 considering carrier frequency bands

    - RAN1 asks the feasibility of at least followings

      - For sub-6 GHz, 100 MHz is considered and for above-6 GHz, wider than 100 MHz is considered

      - Other cases can be considered by RAN4, e.g., 40 MHz, 200 MHz

  - Note that RAN1 will specify all details for channel bandwidth at least up to 100 MHz per NR carrier in Rel-15

  - Also note that RAN1 will consider scalable design(s) for up to maximum channel bandwidth per NR carrier

- From RAN1 specification perspective, the maximum number of NR carriers for CA and DC is [8, 16, 32]

- The maximum FFT size is not larger than [8192, 4096, 2048]

Agreements:

- For CONNECTED and IDLE mode UEs, NR should support network indication of SS burst set periodicity and information to derive measurement timing/duration (e.g., time window for NR-SS detection)

  - Network provides one SS burst set periodicity information per frequency carrier to UE and information to derive measurement timing/duration if possible

    - In case that one SS burst set periodicity and one information regarding timing/duration are indicated, UE assumes the periodicity and timing/duration for all cells on the same carrier

    - RAN1 recommends shorter measurement duration than configured periodicity e.g., 1, 5 or 10 ms

      - Note that L1/L3 filtering across multiple periods is still allowed

    - FFS more than one periodicity/timing/duration indication

  - If the network does not provide indication of SS burst set periodicity and information to derive measurement timing/duration the UE should assume 5 ms as the SS burst set periodicity

  - NR should support set of SS burst set periodicity values for adaptation and network indication

    - Candidate periodicity values to be evaluated are [5, 10, 20, 40, 80 and 160 ms]

RAN2 Agreements (#97)

Agreements

1	An RRC_CONNECTED UE should be able to perform RRM measurements on always on IDLE RS (e.g. synchronization signal).

2	An RRC_CONNECTED UEs should be able to perform RRM measurements on additional RS (e.g. CSI-RS, MRS, etc.).

3	Network should be able to configure RRM measurements via dedicated signalling to be performed on additional RS and/or IDLE RS

Above agreements 2 and 3 are based on the assumption that RAN1 will define additional RS or connected mobility purposes which is not yet concluded in RAN1.

This contribution investigates design considerations to support wide BW in NR; especially about RRM measurement for idle and connected state.

# 2. 	Design Considerations for Wider BW Supports in NR

  - Key Design Issues to Support Mobility in Wider BW

Monitoring DL signal in partial BW

For IDLE mode UE, fixed or semi-static configuration of common BW for monitoring at least NR-PSS/SSS and SI is preferred from the UE complexity and power consumption perspectives. The size of common BW, to achieve its purpose, is less than that of full operating BW of the system. Meanwhile for CONNECTED mode UE, as identified in RAN1, monitoring full BW has a significant impact to UE power consumption. Thus, control resource set (CS) for monitoring DL control information was agreed during NR SI phase to compensate UE power consumption. The size of CS is smaller than or equal to the carrier bandwidth. The CS is a set of resource element groups (REGs) within which the UE attempts to blindly decode downlink control information. The REGs may or may not be frequency contiguous. A UE may have one or more CSs. Even though details of CS are not decided yet, the companion contribution [2] can be referred for more understanding about the issue on restricted monitoring BW at UE.

Observation 1: Monitoring DL signal at UE may be restricted in frequency to compensate higher power consumption for operations in wider bandwidth.

BW adaptation at UE

In RAN1, issues on BW adaptation was discussed and agreed on a design consideration to handle the latency for transition from narrow to wide BW. Since buffering for full BW is not available before retuning RF, dynamic change of monitoring BW is limited due to the latency value. In a case of IDLE to CONNECTED state, the latency does not affect worse. However in other cases frequent BW adaptation from narrow to wide may incur additional latency or power consumption at UE.

Observation 2: Frequent BW adaptation may incur additional latency or power consumption for CONNECTED mode UE.

In long time scale, time portion for IDLE state is much longer than that for CONNECTED state. Thus it is beneficial to minimize power consumption for IDLE mode UE. Static BW configuration for initial access, paging or cell (re)selection will helps minimizing impact from BW adaptation. Definitely common BW across cells is reasonable for IDLE mode operation since reconfiguration to IDLE mode UE is not feasible. In RAN1, NR-xSS, and NR-PBCH are designed for initial access and mobility support. RAN1 also discussed about the type of RS for RRM measurement. At least NR-SSS is used for DL based RRM measurement for L3 mobility in IDLE mode. For CONNECTED mode RRM measurement for L3 mobility, CSI-RS can be used, in addition to IDLE mode RS. Detection of neighbor cell for measurement is based on NR-xSS. The NR-xSS and NR-PBCH is typically located at center frequency in a component carrier. So we can assume that for IDLE mode UE, common resource including NR-xSS and NR-PBCH is configured in frequency across cells. Meanwhile, RAN1 decided network indication of SS burst periodicity for Connected and Idle UE which implies a possible different monitoring timing of the UE.

Proposal 1: At least for mobility of IDLE mode UE, common monitoring resource in frequency to transmit NR-xSS and NR-PBCH is configured across cells in a carrier.

  - RRM Measurement in Wide BW

To achieve optimal performance for handover, monitoring full bandwidth at UE is beneficial to compensate the radio aspects of frequency selectivity or shadowing. In higher frequency bands, blockage of beamforming transmission is a considerable factor for design when different beams are configured in different frequency segments. In unlicensed bands, LBT (Listen-before-talk) failure needs to be considered as well. In those senses, monitoring partial bandwidth for RRM measurement is generally not preferred.

In dense small cell deployments, however monitoring full bandwidth is critical for UE’s battery lifetime. Since UE needs to monitor signals from neighbouring cells frequently, time portion consumed for RRM measurement may be dominant compared to other operations. Even though control subband with limited BW is configured in serving cell, whole power consumption gain may be limited due to monitoring full bandwidth and higher power drain for RRM measurement.

Observation 3: If RRM measurement for neighbouring cell is performed in full bandwidth, power saving gain from restricted bandwidth for operations in serving cell may be negligible from the overall performance perspective, compared to higher power drain for RRM measurement.

If configured BW for both control/data channel (i.e. control/data resource set) in serving cell and RRM measurement for neighbouring cell is restricted for higher power consumption gain, the handover performance could be degraded due to inaccurate measurement quantity as stated here. But difficulty to handle this trade-off issue comes from that serving cell monitoring for control/data channel and RRM measurement is managed by different layers. Thus analysis or discussion on impacts to power consumption is required for further study. Based on discussion, whether or not two operations can be handled separately could be decided.

Proposal 2: For CONNECTED mode UE, RAN2 should discuss the bandwidth adaptation toward RRM measurement for CONNECTED mode UE, considering restricted operating BW of UE.

The operating BW for both control/data channel in serving cell may be configured by RRC or L1 signalling. To support latency constrained service such as URLLC, L1 signalling is preferred as indication of BW reconfiguration for UE. However there is a potential fluctuation of measurement quantity due to different size of monitoring BW. Since the difference of monitoring BW is not known to RRC layer, unreliable decision for handover would be made. Having said that, it is beneficial that monitoring BW for RRM measurement is configured by RRC signalling no matter what operating BW is configured by L1 signalling. If gNB and UE do not have the same notion for the BW, it may impact to handover performance. Thus potential conflict with L1 operation relating to adaptive BW should be avoided.

Proposal 3: For CONNECTED mode UE, monitoring BW for RRM measurement is configured by RRC signalling. RAN1/2 needs to study how to avoid potential conflict with L1 operation.

RRM measurement resource

The resource for monitoring may be at least carrier-specific, that is common across cells, like measurement object in current LTE. Since the monitoring BW for IDLE mode UE can be used for CONNECTED mode UE as well, it could be a baseline operation that the UE measures and reports for NR-SSS and NR-PBCH for CONNECTED mobility. Meanwhile, RAN1 agreed on the network indication of time/frequency resource within the carrier for RRM measurement. Even though the common BW is given or configured, time resource may be differently configured by configurable SS burst periodicity which is introduced to support NR Non-stand-alone (NSA) or LTE-NR coexistence. Note that SS burst comprises of SS blocks which is composed of NR-PSS/SSS and NR-PBCH.

Proposal 4: UE assumes at least a carrier-specific RRM monitoring BW which is derived from detection of NR-xSS and NR-PBCH, and time resource in the monitoring BW could be configured by network.

If more enhanced operation is needed, the cell- or UE-specific RRM resource may be indicated. One expected gain comes from the flexible configuration for various RRM scenarios, e.g. including per-TRP/beam/cell measurement or supporting of multiple numerologies [3]. On the other hand, CSI-RS may be used for RRM measurement in wider band to compensate mismatch of measurement due to higher frequency selectivity. For the above 6 GHz, CSI-RS may provide more accuracy to narrow beam which is used for actual data transmission of CONNECTED mode UE. The fine granularity of RRM resource in terms of time/frequency/space could be helpful to enable accurate and fast measurement performance. However possible location in 3 dimensions of the RRM resource should be restricted for less signalling overhead. In that sense, amongst two approaches, the cell-specific RRM resource is preferred to relax the network overhead and UE complexity. For example, only additional BW which is different from the common BW for IDLE mode UE, could be indicated to CONNECTED mode UE.

Proposal 5: If needed for fine RRM measurement, RAN2 considers additional carrier-specific RRM resource in higher priority. RAN2 holds over the discussion on cell/UE-specific RRM resource, until the reasonable feasibility be shown.

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

Observation 1: Monitoring DL signal at UE may be restricted in frequency to compensate higher power consumption for operations in wider bandwidth.

Observation 2: Frequent BW adaptation may incur additional latency or power consumption for CONNECTED mode UE.

Observation 3: If RRM measurement for neighbouring cell is performed in full bandwidth, power saving gain from restricted bandwidth for operations in serving cell may be negligible from the overall performance perspective, compared to higher power drain for RRM measurement.

Proposal 1: At least for mobility of IDLE mode UE, common monitoring resource in frequency to transmit NR-xSS and NR-PBCH is configured across cells in a carrier.

Proposal 2: For CONNECTED mode UE, RAN2 should discuss the bandwidth adaptation toward RRM measurement for CONNECTED mode UE, considering restricted operating BW of UE.

Proposal 3: For CONNECTED mode UE, monitoring BW for RRM measurement is configured by RRC signalling. RAN1/2 needs to study how to avoid potential conflict with L1 operation.

Proposal 4: UE assumes at least a carrier-specific RRM monitoring BW which is derived from detection of NR-xSS and NR-PBCH, and time resource in the monitoring BW could be configured by network.

Proposal 5: If needed for fine RRM measurement, RAN2 considers additional carrier-specific RRM resource in higher priority. RAN2 holds over the discussion on cell/UE-specific RRM resource, until the reasonable feasibility be shown.

References

[1] R2-160671, New SID Proposal: Study on NR New Radio Access Technology

[2] R2-1703452, “RAN1 issues to support wider band operations”, Samsung, April 2017

[3] R2-1703457, “NR RRM measurement for multiple numerologies”, Samsung, April 2017