3GPP TSG-RAN WG2 Meeting #98	R2-1704349

Hangzhou, China, 15 - 19 May 2017

Agenda item:	10.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Comparing RRC reconfiguration latency for SCG SRB versus RRC containers via MCG SRB

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the performance gain arising from using the SCG SRB in the intra-SgNB mobility scenario. We compare the intra-SgNB handover latency using direct NR RRC signalling versus RRC containers conveyed via the MeNB.

# 2	Background and assumptions

For comparing the reconfiguration latency with direct SRB versus use of RRC containers via MCG SRB, we present a simple illustrative calculation as follows.

The overall procedure duration is measured from the point the UE is ready to send the RRC Measurement Report to the point when the network receives the RRC Connection Reconfiguration Complete; in other words, from the identification of the need for a handover to its completion.

The following assumptions  are used:

UE scheduling delays

Scheduling delays are estimated to be half of the minimum time between two scheduling opportunities.

In LTE, the minimum time between two opportunities to send a Scheduling Request is assumed 5 ms; this leads to an average scheduling delay of 2.5ms.

In NR, the minimum time between two opportunities to send a Scheduling Request is being defined. We will assume it is 1ms.

Uu transmission delays

Uu transmission delays are set to one TTI, meaning 1ms for LTE. For NR we assume a 0.125ms TTI.

SgNB processing and scheduling delays

This corresponds to the time required to process an UL scheduling request from the UE or a DL transmission request and to wait for the right TTI. As this is dependent on UL/DL split, it is proposed to use a fixed processing time of 1ms plus two TTIs, i.e. 1.25ms.

Procedure delays and interruption times

These haven’t yet been defined for NR, but are expected to be lower than for LTE. One way to proceed is to apply a  factor of performance improvement over LTE (3x or 5x).

In LTE, the maximum procedure delay for the UE handling of the RRC Connection Reconfiguration is given by TS36.331 section 11.2 (15ms for intra-LTE mobility). The maximum interruption time is given by TS36.133 section 5.1.2.1.2 (20ms, by assuming the target cell is already known and there is no delay until first PRACH occasion). The total is 35ms.

For NR, we assume for the rest of this discussion that a modest performance gain of 4x leading to a total procedure delay and interruption time of 8ms (rounding up 35/4).

NR RACH procedure

TR38.804 section E.2 gives a RACH minimum latency of 4ms.

Handover preparation

This is implementation-dependent and therefore cannot be estimated fairly. However, this delay is the same for an intra-gNB handover using RRC containers via the MeNB and using direct RRC signalling, so it is only significant if we look at the relative gain compared to the total duration. One way to address this is too look at two metrics:

- The relative gain compared to the total duration, in which the handover preparation is included and set to an arbitrarily fixed number; we propose 25ms.

- Consider the relative gains of handover preparation without including the handover preparation gain.

# 3	Heading 1

Based on the background and assumptions provided in the previous section, the tables provide the results of the simple calculations:

Table 1: Intra-SgNB mobility using the MCG SRB involving the MeNB

Table 2: Intra-SgNB mobility using the direct SCG SRB (not involving the MeNB)

Observation 1: There is a clear 50% latency gain in RRC reconfiguration using the direct SCG SRB for intra-SgNB mobility.

Observation 2: When the HO preparation time is additionally included, there still remains a large gain of close to 30% in procedural execution time for RRC reconfiguration when using the direct SCG SRB for intra-SgNB mobility.

# 4	Conclusion

In this contribution, we have provided the calculations to prove the gains coming from using the SgNB RRC. Given the clear gains that can be achieved with direct NR RRC signalling it is proposed to make it mandatory for the UE to support it.

Observation 1: There is a clear 50% latency gain in RRC reconfiguration using the direct SCG SRB for intra-SgNB mobility.

Observation 2: With the HO preparation time is included, there is still a large gain of close to 30% gain in procedural execution time for RRC reconfiguration using the direct SCG SRB for intra-SgNB mobility.

Proposal 1: The SCG SRB is configured by default during the initial SgNB addition; given the clear gains in latency and RRC reconfiguration procedure time.

# References

- R2-1700055: RRC aspects for direct signalling path from secondary node to the UE, Nokia, Alcatel Lucent Shanghai Bell