3GPP TSG-RAN WG2 NR Ad Hoc                                                                   R2-1700155

Spokane, USA, 17th – 19th January 2017

Source:	ZTE, ZTE Microelectronics

Title:	Consideration on the two-step RACH in NR

Agenda Item:	3.2.1.4

Document for:	Discussion and Decision

Introduction

In RAN2#96 meeting, several agreements are achieved for two-step RACH [1] as follows:

Agreements

If 2 step RACH is supported:

1 The 2-step RACH resources are optionally configurable by the NW

FFS whether it can be configured by broadcast and/or by dedicated signalling.

2: NW can configure/restrict the usage of the 2-step RACH for certain cases ( e.g. procedures/services/radio condition, etc) (FFS for which cases for which it is possible to configure/restrict the usage)

3	RAN2 expects a benefit in latency for the 2 step RACH procedure

4	From RAN2 point of view, the 2-step RACH procedure is not restricted to be used with certain UE ID size.

5	Can provide RAN1 with the different size of message size and UE ID size for the different scenarios in LTE. Indicate to RAN1 that for some use cases the UE ID only would not be sufficient. For NR we are still studying.

In this document, we will discuss the remaining issues on two-step RACH based on the agreement above.

Discussion

Use cases

Use case analysis

Regarding the use cases, as proposed in the last meeting, the two-step RACH is applicable for the following cases shown in table 1.

Table 1: Use cases of two-step RACH

|  | cases | Applicable UE state | Pros of two-step RACH |
| --- | --- | --- | --- |
| 1) | Attach/TAU[8] | RRC_IDLE RRC_INACTIVE RRC_CONNECTED | It’s good for delay reduction when attach request or TAU request is sent with msg1. |
| 2) | Area update for RAN paging[8] | RRC_INACTIVE | It’s good for delay reduction when area update request is sent with msg1. |
| 3) | Other SI requirement(FFS) | RRC_IDLE RRC_INACTIVE RRC_CONNECTED | It’s an available way for other SI requirement. The detail should be refered to section 2.3.4. |
| 4) | RRC Connection establishment[2][8] | RRC_IDLE | It’s good for delay reduction for initial access when RRC msg is sent with msg1. Scheduled should be done quickly if BSR is sent in msg1. |
| 5) | RRC Connection re-establishment[2] | RRC_ CONNECTED | It’s good for delay reduction for RRC re-establishment when RRC msg is sent with msg1. Scheduled should be done quickly if BSR is sent in msg1. |
| 6) | Resume[8] | RRC_INACTIVE | It’s good for delay reduction for RRC resume when RRC msg is sent with msg1. Scheduled should be done quickly if BSR in sent in msg1. |
| 7) | UL data arrival[5] [6][13][20] | RRC_CONNECTED RRC_INACTIVE RRC_IDLE(FFS) | It’s good for delay reduction. Scheduled should be done quickly if BSR in sent in msg1. |
| 8) | Handover/SCG change | RRC_CONNECTED | It’s good for delay reduction for the interruption of HO/SCG change. Scheduled should be done quickly if BSR in sent in msg1. |
| 9) | DL data arrival | RRC_CONNECTED | It’s good for delay reduction. |
| 10) | positioning purpose during RRC_CONNECTED | RRC_CONNECTED | It’s good for delay reduction. |

Proposal 1: RAN2 is kindly requested to confirm the use cases of two-step RACH listed in Table 1.

For Cases 9/10, although these cases are not requiring the UL data transmission, two-step RACH which only requires the PUSCH transmission can also be triggered by the network, if the preamble resources is not sufficient.

Observation 1: Two-step RACH can be triggered with/without UL data arrival.

For Case 1/3/5/7/8/9/10, the two-step RACH which is beneficial for the fast data transmission (especially for the UL data arrival) can be applied to the UE in RRC_CONNECTED.

For Case 1/2/3/4/6/7, the two-step RACH which is beneficial for reducing the state transition delay can be applied to the UE in RRC_IDLE and RRC_INACTIVE.

Observation 2: Two-step RACH can be applicable for UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

Trigger conditions

According to the LTE MAC specification, the legacy RACH procedure can be triggered by the following conditions:

- Option 1): PDCCH order (e.g. UL de-sync upon DL data arrival)

- Option 2): MAC itself (e.g. BSR transmission without pre-allocated SR resources)

- Option 3): RRC message (e.g. SCG change)

According to the use case analysis given above, the trigger conditions listed as the legacy 4-step RACH are also applicable for two-step RACH.

Observation 3: Two-step RACH can be triggered for the following conditions:

- PDCCH order

- MAC itself

- RRC message

RAN2 specification Impacts

If the above use cases of two-step RACH are agreed, we could have the major impacts (with related RRC STATE(s)) of two-step RACHs as given in the following table:

Table 2: RAN2 specification impacts of two-step RACH

| Issue List | Descriptions | IDLE | INACTIVE | CONN |
| --- | --- | --- | --- | --- |
| 1) | How does two-step RACH co-exist with four-step RACH? | Yes | Yes | Yes |
| 2) | How does the contention resolved? | Yes | Yes | Yes |
| 3) | How to handle the failure of two-step RACH? | Yes | Yes | Yes |
| 4) | How/when to trigger (i.e. use cases like on-demand SI) the two-step RACH? | Yes | Yes | No |
| 5) | How to handle the context (e.g. AS/NAS) of the UE? | Yes | Yes | No |
| 6) | How/whether to support HARQ/ARQ? | Yes | Yes | No |
| 7) | How to support ACB for two-step RACH? | Yes | Yes | No |

Issue 1): How does two-step RACH co-exist with four-step RACH?

On the one hand, same/different preamble groups/RACH resources can be used for four-step and two-step RACH. On the other hand, four-step or two-step RA could be triggered by UE and controlled by NW. The rules can be configured by broadcast and/or by dedicated signaling. Thus, two step RACH may/may be triggered together with four-step RACH.

Issue 2): How does the contention resolved?

In order for contention, appropriated UE ID should be included in msg1 which is shown in table 3. Different UE ID(s) at different layer could be used for two-step RACH for contention resolution.

Table 3

|  | cases | appropriated UE ID in msg1 | Other information should be included in msg1 except for UE ID | Possible Payload size(byte) |
| --- | --- | --- | --- | --- |
| 1) | Attach/TAU[8] | Random Value for the distinguish of ACKNACK | Attach request/TAU request | Random value: FFS Attach request: 29+n[25] TAU request:15[25] |
| 2) | Area update for RAN paging[8] | Random Value for the distinguish of ACKNACK | Area update request | FFS |
| 3) | Other SI requirement(FFS) | No UE ID if other SI is broadcasted. UE ID such as S-TMSI is needed if other SI is point-to-point or multi-casted. | The information of which other SI is required. | S-TMSI:5[26] Other SI required:2(FFS) |
| 4) | RRC Connection establishment[2][8] | S-TMSI or random value | RRC connection request | RRC connection request: 6[26] |
| 5) | RRC Connection re-establishment[2] | S-TMSI or random value | RRC Connection re-establishment request | RRC Connection re-establishment request: 7[26] |
| 6) | Resume[8] | Resume ID | RRC resume request, BSR | RRC resume request:6[26] |
| 7) | UL data arrival[5] [6][13][20] | C-RNTI for UE in RRC_CONNECTED S-TMSI for other cases | UL data | UL data size:FFS |
| 8) | Handover/SCG change | C-RNTI | RRC reconfiguration complete | RRC reconfiguration complete:1[26] |
| 9) | DL data arrival | C-RNTI | NULL |  |
| 10) | positioning purpose during RRC_CONNECTED | C-RNTI | NULL |  |

Issue 3): How to handle the failure of two-step RACH?

Both contention-based and contention-free RA procedure should be supported in NR. When two-step RA is failed a fall back to legacy RA such as four-step RA would be possible [4][20]. Other ways can be considered also. More study is required on how to handle/reduce/avoid the failure of two-step RACH, as we could have several different failure causes.

Issue 4): How/when to trigger (i.e. use cases like on-demand SI) the two-step RACH?

Two-step RACH can also be used in other SI requirement. Many pending issues exist in this case. Except the information in msg1 described in section 2.3.2, whether RAR exists and how it looks like depends on how the other SI is transmitted to UE, broadcast, multi-cast or point-to-point? More study is required on how the other SI request can be sent via two-step RACH.

Issue 5): How to handle the context (e.g. AS/NAS) of the UE?

In order to transmit data in INACTIVE/IDLE, the UE context should be kept at both the UE and the network. Then how to maintain these context (e.g. AS/NAS status/configuration) should be studied further not just by RAN2, but also by RAN3 and SA2/3. More study is required on how/whether to maintain the AS/NAS context of the UE.

Issue 6): How/whether to support HARQ/ARQ?

To support HARQ/ARQ, we should firstly determine the target block/packet error rate for the PUSCH transmission of 2-step RACH. Then simulation/evaluation is required on the potential solutions of supporting HARQ/ARQ.

Issue 7): How to support ACB for two-step RACH?

According to the LTE light connection discussion, ACB which does not support IDLE is not supported. If two-step RACH is supported for INACTIVE/IDLE, we should consider how to alleviate the congestion issue of the two-step RACH to improve the resource efficiency and so on. More simulation/evaluation is required to evaluate how/in which case to support two-step RACH, so as to improve the performance while collision occurs.

Based on the specification analysis given above, we consider that the 2-step RACH may not be able to be completed in the first phase in NR. We kindly request RAN2 to consider the 2-step RACN in a study item of NR.

Observation 4: Considering that 2-step RACH has many remaining issues as listed in table 2, the 2-step RACH may not be able to complete in the first phase of NR.

Proposal 2: To de-prioritize the two-step RACH in the NR study.

Conclusion

According to the analysis above, we have the following Observations and proposals:

Observation 1: Two-step RACH can be triggered with/without UL data arrival.

Observation 2: Two-step RACH can be applicable for UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

Observation 3: Two-step RACH can be triggered for the following conditions:

- PDCCH order

- MAC itself

- RRC message

Observation 4: Considering that 2-step RACH has many remaining issues as listed in table 2, the 2-step RACH may not be able to complete in the first phase of NR.

Proposal 1: RAN2 is kindly requested to confirm the use cases of two-step RACH listed in Table 1.

Proposal 2: To de-prioritize the two-step RACH in the NR study.

References

[1]  RAN2-96-Reno-Chairman-Notes-2016-11-18-eom.doc

[2]  R2-167568	Random Access Procedure in NR	Samsung Electronics Co., Ltd	discussion

[3]  R2-168254	Random access procedure in NR	Huawei, HiSilicon	discussion

[4]  R2-168013	Considerations on NR RACH procedures	Nokia, Alcatel-Lucent Shanghai Bell	discussion

[5]  R2-168035 	RAN2 aspects on random access procedure for NR	NTT DOCOMO, INC.	discussion

[6]  R2-168521	Considerations of Random access procedure	Intel Corporation	discussion

[7]  R2-167588	   Discussion on 2-step RACH procedure	Huawei, HiSilicon	discussion

[8]  R2-168520	   2-step Random Access in NR	Intel Corporation	discussion

[9]  R2-167569	   Random Access in NR – Flexible UE Bandwidth Aspects	Samsung Electronics Co., Ltd	discussion

[10]  R2-167831	Random Access in NR	ZTE, ZTE Microelectronics	discussion

[11]  R2-167850	Considerations on the Random Access Procedure in Massive MIMO NR	National Instruments Corp.	discussion

[12]  R2-167951	Random access procedure in NR	CATT	discussion

[13]  R2-168039	Random access procedure related to RAN2	ETRI	discussion

[14]  R2-168215	Random access procedure in NR	ASUSTEK COMPUTER (SHANGHAI)	discussion

[15]  R2-168241	Discussion on latency of random access in NR	ASUSTEK COMPUTER (SHANGHAI)	discussion

[16]  R2-168375	RA enhancement for New RAT	LG Electronics Inc.	discussion

[17]  R2-168376	Potential impact of beam sweeping on RA	LG Electronics Inc.	discussion

[18]  R2-168465	Random access procedure in NR	InterDigital Communications	discussion

[19]  R2-168607	NR beam refinement in RA	Qualcomm Incorporated	discussion

[20]  R2-168665	Random Access enhancements	Ericsson	discussion

[21]  R2-168668	Text Proposal to TR 38.804 on Random Access Procedure	Ericsson	discussion

[22]  R2-168835	Flexible RACH	MediaTek Inc.	discussion

[23]  Chairman's Notes RAN1_87_v029.doc

[24]  36822-b00.doc

[25]  24301-e20.doc

[26]  36331-e00.doc