3GPP TSG-RAN WG2 Meeting NR ad-hoc	Tdoc R2-1700429

# Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.1 - Overall user plane architecture

Source:	Ericsson

Title:	Further design aspects of the NR User Plane stack

Document for:	Discussion, Decision

# Introduction

In previous RAN2 meetings, segmentation and concatenation functionality had been discussed and lead to the following agreements and working assumptions:

RAN2#95bis

Agreements

1: In NR, the segmentation function is only placed in the RLC layer as in LTE.

Agreement

=>	 SO-based segmentation can be considered for both segmentation and resegmentation as a baseline in NR user plane to support high data rate. (Does not imply anything about location of concatenation). At least overhead for the low data rate case should be analysed further.

RAN2#96

Working assumption:

-	Support the No concatenation in RLC solution (R2-169092)

=>	Aim is to confirm, or otherwise, the working assumption at the January ad hoc

Agreements

1	PDCP supports the re-ordering functionality (T-reordering)

2	RLC AM supports T-reordering like functionality for the purposes of determining the content of the RLC status report.

FFS whether RLC UM needs to support T-reordering like functionality for the purposes moving the lower edge of the receive window, or for other purposes. Could be discussed in stage 3

3	RLC reassembles RLC SDU and delivers them to upper layers in the order they are received (no need to mention reordering with respect to this functionality)

FFS whether in-order delivery for a DRB can be disabled via RRC signalling. This only affects PDCP operation. Could be discussed in stage 3Working assumption:

-	Support the No concatenation in RLC solution (R2-169092)

=>	Aim is to confirm, or otherwise, the working assumption at the January ad hoc

In this contribution, we discuss the implications of these agreements and assumptions and propose further aspects that must be resolved before confirming the previous working assumptions.

# Discussion

## RLC and PDCP sequence numbers

With the current working assumption of not supporting concatenation in RLC, each PDCP PDU will be mapped to an RLC PDU and may be concatenated (multiplexed) with other RLC PDUs in MAC. As in LTE, RLC AM performs ARQ and both RLC AM and UM perform segmentation and reassembly. Unlike LTE, the RLC receiver is assumed not to ensure in-sequence delivery towards PDCP.

Some companies suggested that the 1:1 mapping would allow PDCP and RLC to use a common sequence number and thereby to save signalling overhead.

However, as discussed in previous meetings, an RLC transmitter entity does not necessarily obtain PDCP PDUs with continuously increasing sequence numbers. The most prominent example is dual connectivity with a split bearer where separate RLC entities exist in MCG and SCG. But there may also be cases where PDCP PDUs are dropped or lost (e.g. in case of a centralized PDCP entity) resulting in a gap of the observed PDCP SN at the RLC transmitter. One could certainly introduce means to fill these gaps or to inform the RLC receiver to ignore a gap in the received sequence of RLC PDUs. However, the expected gains do not justify the additional complexity, processing and protocol interdependencies.

An RLC transmitter entity does not necessarily obtain a PDCP PDUs with continuously increasing sequence numbers (e.g. DC, PDCP drops/losses).

Furthermore, there are RLC SDUs which don’t have a PDCP sequence number: PDCP control PDUs do not carry a PDCP SN since they provide information about the reverse direction (ROHC, status report). The lack of a PDCP SN is an important property as it allows the PDCP transmitter to inject these control PDUs as soon as they become available without affecting the possibly pre-generated sequence numbers of PDCP data PDUs in the same logical channel.

PDCP control PDUs do intentionally not carry a PDCP SNs but they require a sequence number on RLC level for ARQ and segmentation/reassembly.

Based on the observations above, we propose not to reuse PDCP SNs in RLC.

RLC and PDCP operate with separate and independent sequence numbers.

While we believe that PDCP and RLC should use their own sequence numbers, the fact that an RLC transmitter maps each PDCP PDU into one RLC PDU suggests that the sequence number spaces of the two protocols should be equivalent.

As baseline, RLC and PDCP should support the same sequence number space (e.g. 18 bit).

## RLC segmentation and header length

In the following we review the working assumption on not supporting concatenation in RLC and the assumption that SO-based re-segmentation can be reused also for segmentation. Figure 1 illustrates how the MAC PDU is constructed, which is also further discussed below. It is highlighted in green, which header and payload can be pre-processed and stored in memory. It is shown in red, which headers must be created or modified on the fly i.e. are dependent on TB size.

Figure 1: MAC PDU construction.

Per our understanding, MAC sub-headers would be interleaved in the solution without RLC concatenation. This way, the MAC sub-headers as well as RLC and PDCP headers could be pre-generated.

Confirm that MAC sub-headers are interleaved with RLC PDUs.

If RLC does no longer support concatenation of RLC SDUs, the variable length train of L-fields in RLC is not needed anymore. The MAC L field points to the end of the RLC PDU and thereby to the beginning of the next MAC sub-header.

The RLC header must indicate whether a complete AMD PDU or just an AMD PDU Segment follows. If it is a segment, the Segment Offset (SO) field should indicate the position of the first payload octet in the SDU. When the RLC transmitter pre-generates RLC headers, it will typically assume that all RLC PDUs can be sent without segmentation and will hence not include the SO field. When the RLC transmitter is unable to send the entire RLC PDU due to limited space in the MAC PDU, it must at least indicate that the last payload octet of the RLC PDU is not the last octet of the RLC SDU. But since the first octet of the RLC PDU payload contains the first octet of the RLC SDU, the segment offset is 0 and should preferably be omitted. This avoids having to update the RLC header size for the first segment of an PDU.

The header of the RLC AMD PDU indicates with one bit whether the last octet of the RLC PDU payload is also the last octet of the RLC SDU.

Any subsequent segment (not starting with the first octet of the RLC SDU) must carry the additional SO field. To inform the RLC receiver whether the SO field follows the RLC “base header”, an additional bit is needed in the AMD PDU header:

The header of the RLC AMD PDU indicates with one bit whether the first octet of the RLC PDU payload is also the first octet of the RLC SDU.

If the above-mentioned header field indicates that the first octet of the RLC PDU is not the first octet of the RLC SDU the SO field is present in the header.

With Proposal 4 and Proposal 5 these two bit follow largely the definition of the FI field in the LTE RLC header. Of course, the exact placement of the header fields can be discussed in the work item phase.

## Multiple transport blocks and LCP

The UE may need to create multiple transport blocks (TB) within the same TTI when carrier aggregation or MIMO is used. The TBs may be filled from the same logical channel and thus, the implementation must be fast enough in this case to produce all the TBs in time. That means, the UE must determine which (parts of) which RLC PDUs to transmit in which transport block (MAC PDU). If an RLC PDU does not fit entirely into a transport block, it must be segmented, i.e., a bit must be set in the header to indicate that the last octet of the RLC PDU is not the last octet of the RLC SDU (see Proposal 4). Due to the definition of the two bits (Proposal 4 and Proposal 5) the SO field does not need to be added to the RLC header in this transport block. However, also the remainder of this segmented RLC PDU should be transmitted in another transport block of the same TTI.

At RAN2-96 some companies suggested that the remainder of a segmented PDU could be postponed to a subsequent TTI while the UE could fill a second transport block with another RLC PDU. However, this would have the undesirable consequence that, when the sum of all TBs suffices for the number of transmittable octets, the UE transmits padding rather than draining the buffer. In fact, this is likely that this happens at the end of ever data burst. To avoid resource inefficiency and additional transmission delay at the end of a data burst we suggest the following:

The UE shall transmit the remainder of segmented RLC PDU in another transport block of the same subframe if such is available for the UE.

This is also depicted in Figure 2 where the remainder of the last PDCP PDU in TB1 is transmitted in the beginning of TB2.

Figure 2: RLC PDUs with sequential sequence number within a transport block 
and across transport blocks of the same subframe.

Furthermore, it was suggested that the UE may fill a transport block with RLC PDUs that are not in continuously increasing sequence number order. For example, transport block A could carry RLC PDUs with sequence numbers 1, 3, 5 and a segment of PDU 7. Another transport block B in the same subframe could carry RLC PDUs 2, 4, 6 and a segment of PDU 8. It was argued that this could allow the transmitter to start sending RLC PDUs towards L1 before determining how many RLC PDUs this transport block could serve in total. However, it contradicts the original argumentation that the removal of concatenation allows building a sequential stream of MAC/RLC/PDCP PDUs and feeding that stream quickly to L1. More importantly, it further increases the processing load on the receiver side.

When the UE builds multiple TBs per TTI (DC, CA and MIMO) spectral efficiency and efficient receiver side processing must be ensured.

Since a receiver must be dimensioned for the worst allowed transmitter behaviour the allowed transmitter behaviour must be specified properly. Therefore, we propose the following:

Within one transport block the RLC data PDUs of one logical channel shall be in continuously increasing sequence number order (except for RLC retransmissions).

Figure 2 illustrates the behaviour in accordance with Proposal 7 and Proposal 8. It is highlighted in green, which blocks of pre-processed PDUs are sent unmodified, and highlighted in red, which headers needed to be updated on the fly.

## Placement of MAC Control Elements

RAN2-96 discussed briefly the order of MAC control elements within the transport block. Many companies pointed out that it would be desirable to place in particular the BSR and the PHR the at the beginning of the MAC PDU so that the receiver can feed this information quickly towards the uplink scheduler. It was discussed that this could be too demanding for the UE to compute in the short time available prior to the beginning of the UL data symbols. However, since the UE should anyway determine which (segments of which) RLC PDUs to transmit in which MAC PDU (see Proposal 7 and Proposal 8), it also knows the number of data bytes that will be left after completing the current subframe. Hence, the BSR could also be included in the front of the MAC PDU. Similarly, the UL TX power must be computed prior to the UL transmission. Hence, also the power headroom should be known beforehand.

The UE shall include the BSR and the PHR MAC CEs in the beginning of the PDU, i.e., prior to the MAC SDUs.

# Conclusion

In section 2 we made the following observations:

Observation 1	An RLC transmitter entity does not necessarily obtain a PDCP PDUs with continuously increasing sequence numbers (e.g. DC, PDCP drops/losses).

Observation 2	PDCP control PDUs do intentionally not carry a PDCP SNs but they require a sequence number on RLC level for ARQ and segmentation/reassembly.

Observation 3	When the UE builds multiple TBs per TTI (DC, CA and MIMO) spectral efficiency and efficient receiver side processing must be ensured.

Based on the discussion in section 2 we propose the following:

Proposal 1	RLC and PDCP operate with separate and independent sequence numbers.

Proposal 2	As baseline, RLC and PDCP should support the same sequence number space (e.g. 18 bit).

Proposal 3	Confirm that MAC sub-headers are interleaved with RLC PDUs.

Proposal 4	The header of the RLC AMD PDU indicates with one bit whether the last octet of the RLC PDU payload is also the last octet of the RLC SDU.

Proposal 5	The header of the RLC AMD PDU indicates with one bit whether the first octet of the RLC PDU payload is also the first octet of the RLC SDU.

Proposal 6	If the above-mentioned header field indicates that the first octet of the RLC PDU is not the first octet of the RLC SDU the SO field is present in the header.

Proposal 7	The UE shall transmit the remainder of segmented RLC PDU in another transport block of the same subframe if such is available for the UE.

Proposal 8	Within one transport block the RLC data PDUs of one logical channel shall be in continuously increasing sequence number order (except for RLC retransmissions).

Proposal 9	The UE shall include the BSR and the PHR MAC CEs in the beginning of the PDU, i.e., prior to the MAC SDUs.