3GPP TSG-RAN WG2 NR Ad Hoc	Tdoc R2-1700437

Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.1

Source:	Ericsson

Title:	Upper layer aggregation (DC like) for multi-connectivity

Document for:	Discussion, Decision

# Introduction

RAN2#94 has agreed that both lower layer aggregation (e.g., CA-like) and upper layer aggregation (e.g., DC-like) shall be studied for the aggregation of NR carriers. RAN2#94 has also agreed that only DC approach for LTE-NR aggregation will be studied. In the RAN#74 plenary it was also decided that no dedicated meeting time in the WGs will be allocated until March 2017 for

  - Multi-connectivity constituted with more than two nodes (gNBs and/or eNBs)

    - I.e., Dual Connectivity is continued to study in the SI phase.

In this paper, we discuss DC-like aggregation for multi-connectivity.

# Discussion

## Duplicated transmission

In [1], RRC diversity has been analyzed and it is found that mobility robustness can be improved in different overlay scenarios including both NR-only deployments and LTE-NR deployments. For ultra-reliable and low latency (URLLC) like services, duplicated transmissions for UP data is expected to offer similar gain as that given by RRC diversity for CP. In the contribution [2] several aspects of duplicated transmissions for URLLC scenarios are discussed.

Duplicated packets at higher layer are sent across multiple links and are transmitted in parallel. Hence, the UE has a higher probability to recover from transmission failures at HARQ. To ensure both a high transmission reliability and very low latency for URLLC like services, a backhaul link with short latency between MeNB and SeNBs may need to be supported together with duplicated transmissions.

For DC-like MC, a backhaul link with short latency between MeNB and SeNBs may be required together with duplicated transmissions to fulfill the service requirements for URLLC like services.

The network should have means to control when to trigger duplicated transmission. Typical scenarios when duplicated transmission should be enabled is when there is a risk of transmission failure at lower layers. Using duplicated transmission also has a cost in wasted radio resources which is why it should be disabled if the transmission at lower layers become stable.

DC-like MC support duplicated UP data transmission.

The NW configures a UE when to enable and disable duplicated data transmission.

For LTE DC, only RLC AM mode is supported for split bearers. As PDCP reordering is always enabled in NR (i.e. even in non-DC case), RLC UM can be used for Multi-Connectivity in NR.

RLC AM ARQ improves the reliability by correcting residual HARQ errors, but this results in additional retransmission delays and increased overhead due to the use of RLC status reports. With duplicate transmissions, this retransmission delay can be avoided (to the expense of lower resource efficiency) by using RLC UM.

In general, the usage of RLC UM must be complemented with additional means to provide a high transmission reliability; especially for services that requires this. The usage of duplicated transmission at PDCP is one example, in terms of a balance between the reduction of UP latency and the fulfilment of the transmission robustness requirements.

For DC-like MC, the usage of RLC UM is beneficial to reduce UP latency but may need other complemental means to ensure the transmission reliability.

The DC-like MC support RLC UM mode besides RLC AM mode.

# Conclusion

In section 0 we made the following observations:

Observation 1	For DC-like MC, a backhaul link with short latency between MeNB and SeNBs may be required together with duplicated transmissions to fulfill the service requirements for URLLC like services.

Observation 2	For DC-like MC, the usage of RLC UM is beneficial to reduce UP latency but may need other complemental means to ensure the transmission reliability.

Based on the discussion in section 0 we propose the following:

Proposal 1	DC-like MC support duplicated UP data transmission.

Proposal 2	The NW configures a UE when to enable and disable duplicated data transmission.

Proposal 3	The DC-like MC support RLC UM mode besides RLC AM mode.

# Reference

R2-166776 “RRC diversity”

R2-1700428 “Data duplication and link selection for URLLC in NR”