3GPP TSG-RAN WG2 NR Ad Hoc	Tdoc R2-1700484

Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.2.2.5

Source:	Ericsson

Title:	Direct SRB between secondary node and UE

Document for:	Discussion, Decision

# Introduction

In RAN2#95bis, the following agreements were made:

Agreements:

- Agree the following principle: the master node and the secondary node only need to use own RAT UE capabilities (which will include some other RAT capabilities relating to the interworking). At least for the initial configuration of interworking case these are provided on the master node RAT or from core network

- Allow gNB to format NR RRC PDUs for the UE configuration.

As understood from the agreements, gNB can send an RRC message, yet it is still open whether the RRC messages generated by NR node would be the final RRC messages or not e.g., whether an NR RRC PDU can be transmitted over NR SRB when gNB is the secondary node.

In this contribution, we will be seeking answers to this open issue; as well as present and propose how RRC configuration should be handled overall for LTE-NR tight-interworking.

# Discussion

As the general principle, RRC procedures which need coordination between master node and secondary node does not benefit from direct SRB between secondary node and UE. This is because when coordination is needed, the RRC messages from secondary node anyway need to be sent to master node to check if that RRC message is appropriate from master node perspective, then there will be no difference from the solution where final RRC message is sent from master node to UE.

Based on the principle above, let us check the RRC procedures between secondary node and UE one by one to see which one needs coordination, which one does not. Those RRC procedures are listed as below

1)     Measurement configuration

Measurement configuration specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT measurements as well as configuration of measurement gaps. When there is no secondary node, of course, master node need to configure UE inter-RAT related measurement so that UE can detect secondary node and report to RAN. Once there is secondary node, it is reasonable for secondary node to configure UE to measure the frequencies bundled to RAT of secondary node. Due to three reasons, the measurement configuration still needs coordination between master node and secondary node. First, each measurement object is linked to a unique measurement ID. When there is no coordination, the measurement ID selected by secondary node could be the one selected by master node already, this will result in a conflict. Second, secondary node need to know the measurement object configured by master node on the frequency of secondary RAT, otherwise, same measurement object (frequency) could be configured twice by both maser node and secondary node which cause confusion at UE as well. Third, measurement gap configuration may require coordination as it depends on UE capabilities.

Of course, some solutions can be used to avoid coordination between master and secondary node for each measurement configuration. For example, master node can inform secondary node the measurement ID for the frequency it configured on secondary RAT and the range of measurement ID that it will used. Then secondary node can know the measurement ID for the frequency configured by master node and the remaining ID can be used by secondary node. No coordination is needed for later measurement configuration.

2)   Measurement report

The measurement report related to secondary RAT from UE can be sent toward secondary node directly once secondary node is there. We do not see any coordination requirement for such message.

3)     Secondary node release

Secondary node release procedure is used to release the connection between secondary node and UE. This signaling procedure can be carried out between secondary node and UE directly. We do not need any coordination needed between secondary node and master node. However, master node need be informed immediately once connection between secondary node and UE is released, e.g. master node need to inform CN (core network) not to send data to secondary node anymore if there is SCG bearer.

In other word, although secondary node release procedure itself can benefit from direct SRB between secondary node and UE, the follow up procedure which requires involvement from master node immediately makes the gain of this procedure not apparent.

4)     Secondary node modification

SeNB modification is used to perform configuration changes of the SCG within the same SeNB, e.g. to trigger the addition, release of SCG SCell(s) (other than PSCell), addition, modification, release of SCG bearer(s) and the SCG part of split bearer(s), and to trigger PSCell change.

For release of SCG SCell, SCG bearer, these procedures could be suitable for direct SRB, as it can be done without coordination between master node and secondary node. However, it would be good that MeNB is informed, since in the event of SCG bearer release, the MeNB could want to reconfigure the bearer to a MCG bearer instead. Also for some UE capabilities may be released, which could be taken in to use in the MCG configuration. Similar to secondary node release, after release of SCG bearer, secondary node need to inform master node immediately so that master node will not continue to deliver packets to secondary node anymore.

For addition of SCG SCell, or SCG bearer or SCG part of split bearer, it could require coordination with MeNB, as an addition of SCG SCell (SCG bearer) means to add a new frequency, and more L2 buffer are needed from secondary node, which again may require capability coordination between MeNB and SeNB.

5)     Secondary node change

Secondary node change is used to change the secondary node from one to another. When the new secondary node and the old secondary node work on the same frequency, this may not require coordination with master node, therefore it can benefit from direct SRB. If new secondary node works on a different frequency from old secondary node, coordination with master node maybe required as new frequency may means new band combination and thus coordination is required.

Whatever the secondary node change is intra or inter-frequency change, when secondary node is changed, the data path need be updated, either to CN for SCG bearer, or to MeNB for SCG part split bearer. Therefore, master node need be informed of secondary node change immediately. Similar to secondary node release, the gain from direct communication between secondary node and UE for such procedure is not very apparent.

The RRC procedures which can benefit from direct SRB between secondary node and UE are quite limited.

Within those RRC procedures which benefit from direct SRB, some of them need be followed with immediate communication with master node which make the gain of direct SRB not apparent

As not all RRC procedures between secondary node and UE are suitable to use direct SRB, the support of direct SRB between secondary node and UE also introduced complexity into system. Secondary node need to check the RRC procedure one by one and then decide whether to transmit the RRC message via direct SRB between secondary node and UE or transmit the RRC message over X2’ to master node. Similarly, UE need also check which RRC message it need to deliver to master node, which message it can deliver to secondary node.

The support of direct SRB between secondary node and UE also make both NW and UE implementation complicated.

Direct SRB between secondary node and UE is not supported due to no apparent gain identified and the complexity introduced

One concern with the LTE DC like solution, i.e. SRB communication only between master node and UE, is the longer delay for RRC procedures between secondary node and UE. For LTE-NR tight interworking, this can be solved by deployment. Either deploy NR gNB and LTE eNB co-located, or connect NR gNB and LTE eNB with fast backhaul.

Furthermore, RRC messages can be transmitted to UE via SRB diversity, similar to split bearer for DRB. That is, the PDCP entity at the transmitter side will deliver each PDCP PDU to the UE via MCG and/or SCG. PDCP PDU duplication detection and removal at receiver side is done by PDCP entity and thus RRC is not affected at all. In this way, RRC messages can benefit from the low L1 latency from secondary node if it is NR gNB. Besides, as the robustness of RRC message is improved, the latency of RRC procedure is reduced as well when RRC message transmitted over SCG fails.

Detailed discussion on SRB diversity is in [1].

SRB diversity is supported for LTE-NR tight interworking

# Conclusion

Based on the discussion in section 2 we have the following observations and proposals:

Observation 1	The RRC procedures which can benefit from direct SRB between secondary node and UE are quite limited.

Observation 2	Within those RRC procedures which benefit from direct SRB, some of them need be followed with immediate communication with master node which make the gain of direct SRB not apparent

Observation 3	The support of direct SRB between secondary node and UE also make both NW and UE implementation complicated.

Proposal 1	Direct SRB between secondary node and UE is not supported due to no apparent gain identified and the complexity introduced

Proposal 2	SRB diversity is supported for LTE-NR tight interworking

# References

R2-1700485, “RRC diversity”, Ericsson.