3GPP TSG RAN WG2 NR Ad-hoc	R2-1700605

Spokane, USA, 17th to 19th January 2017

Agenda Item:		3.3.1.1.1

Souce:		Samsung

Title:		RRM Measurement Model in NR

Document for:	Discussion/Decision

Introduction

In the last RAN2 #96 meeting, we discussed the mobility with RRC involvement and made some agreements as follows.

Agreements for connected active - The scope of the RRM measurement should mainly be to facilitate the RRC driven ‘cell’ level mobility. - RAN2 working assumption to be confirmed by RAN1: - Connected active mode RRM measurement and reporting based on at least the signals used by idle mode RRM measurement should be supported in the NR. - Additional RS may need to be used for RRM measurement in the connected active mode besides the signals used by idle mode RRM measurement, which is dependent on RAN1’s decision. - The RRM measurement framework (measurement object, measurement id, reporting config) in LTE as a baseline in NR.

Agreements for connected active - RRM measurement for cell level mobility should be performed based on a common framework regardless of network beam configurations (e.g., number of beams) and the UE beam configuration. FFS: Which beams the UE selects from the detected beams in order to derive a cell level quality. Options to be studied: - best beam - N-best beams - all detected beams - beams above a threshold Other options are not precluded.

Agreements - At least events like LTE A1-A6 and periodic will be supported for NR (modification to the events may be considered). FFS: other events may be studied. - Measurement report will contain cell measurements. FFS: whether the measurement report contains beams measurements.

To make progress of the discussion on the mobility with RRC involvement, we will discuss how to enhance the RRM measurement model in LTE for supporting gNBs and UEs that are equipped with multiple beams. In this context, we will tackle the following issues.

- Input and output of the RRM measurement model in NR

- L1 and L3 filtering: beam level or cell level?

Discussion

In LTE, cell-level mobility such as handover, cell addition and release is supported by the RRM measurement model specified in [1], as illustrated in Fig. 1.

Figure 1 RRM measurement model in LTE

In this model, the point “A” indicates the measurements (samples) internal to the physical layer of LTE. To generate “A”, a UE typically measures CRS (Cell-specific Reference Signal), which is a non-UE specific and non-beamformed signal. If we consider beamformed Tx and Rx of a reference signal using a single or multiple beams at a gNB and a UE in NR, the point “A” in Fig. 1 can be seen as follows.

Figure 2 RRM measurement considering multiple beams at gNB and UE

Fig. 2 shows that a UE measures a reference signal transmitted by a gNB. More specifically, the RSRP between gNB beam i and UE beam j, which is denoted by Rij, is measured by the UE. It means that the RSRP is actually measured per beam pair. However, since RRM measurement is used to support cell-level mobility of the UE, the gNB does not care about the UE beam-specific RSRP. On the other hand, the gNB beam-specific RSRP should be managed by the UE since it can be used to derive a cell quality. In this context, we propose the following to develop the RRM measurement model in NR.

Proposal 1: The RRM measurement model in LTE should be considered as a baseline in NR.

Proposal 2: The input of the RRM measurement model in NR should be gNB beam-specific RSRP or RSRQ.

Proposal 3: The output of the RRM measurement model in NR, which is used to trigger measurement report, should be cell-specific RSRP or RSRQ (i.e., one per cell).

In addition to the input and output of the RRM measurement model in NR, we should discuss how L1 and L3 filters work. It is known that they are used to alleviate large fluctuation of measured RSRP and have an impact on mobility performance in terms of the probability of handover failure and the frequency of ping-pong handover. The operation of L1 and L3 filters is illustrated in Fig. 3.

Figure 3 Operation of L1 and L3 filters

Remind that we propose that the input and output of the RRM measurement model in NR should be the gNB beam-specific RSRP and the cell-specific RSRP, respectively. In order to derive the cell-specific RSRP from the gNB beam-specific RSRP, we need to study how L1 and L3 filters work during this derivation procedure. More specifically, it is required to define the input and output of the L1 and L3 filters. The possible options are listed in Table 1.

Table 1 Options for L1 and L3 filters: per beam or per cell?

|  | Input | L1 filter | L3 filter | Output |
| --- | --- | --- | --- | --- |
| 1 | gNB beam-specific RSRPs | Per cell | Per cell | Cell-specific RSRP |
| 2 |  | Per gNB beam | Per cell |  |
| 3 |  | Per gNB beam | Per gNB beam |  |

Furthermore, each option in Table 1 is explained as follows.

Option 1 is illustrated in Fig. 4, in which the UE derives a cell-level value before performing L1 filtering. Then, this value goes through the L1 and L3 filters. This option is as simple as the RRM measurement model in LTE. However, beam-related information, for instance, how frequently gNB beams are changing within a cell, may be ignored when the UE triggers measurement report.

Figure 4 L1 filtering per cell and L3 filtering per cell (Option 1)

Option 2 is illustrated in Fig. 5. In this option, the UE performs L1 filtering for each gNB beam separately and then derives a cell-level value based on the outputs of the L1 filters. Then, this value is used for L3 filtering. The UE complexity of this option is increased compared to Option 1, however, the layer 3 can utilize the gNB beam-specific RSRP after L1 filtering.

Figure 5 L1 filtering per gNB beam and L3 filtering per cell (Option 2)

Option 3 is illustrated in Fig. 6. If this option is used, the UE performs L1 and L3 filtering for each gNB beam separately and then derives a cell-level value based on the outputs of the L3 filters. This option allows the UE to observe each gNB beam in more detail by virtue of L3 filtering, however, the UE complexity for doing this should be increased.

Figure 6 L1 filtering per gNB beam and L3 filtering per gNB beam (Option 3)

Through Options 1, 2 and 3 above, we have investigated how L1 and L3 filters work in the RRM measurement model in NR. As a result, we have the following proposal.

Proposal 4: RAN2 should investigate how L1 and L3 filters operate for the RRM measurement model in NR. The following options should be considered for this discussion.

- Option 1) L1 and L3 filtering per cell

- Option 2) L1 filtering per gNB beam and L3 filtering per cell

- Option 3) L1 and L3 filtering per gNB beam

Conclusions

Proposal 1: The RRM measurement model in LTE should be considered as a baseline in NR.

Proposal 2: The input of the RRM measurement model in NR should be gNB beam-specific RSRP or RSRQ.

Proposal 3: The output of the RRM measurement model in NR, which is used to trigger measurement report, should be cell-specific RSRP or RSRQ (i.e., one per cell).

Proposal 4: RAN2 should investigate how L1 and L3 filters operate for the RRM measurement model in NR. The following options should be considered for this discussion.

- Option 1) L1 and L3 filtering per cell

- Option 2) L1 filtering per gNB beam and L3 filtering per cell

- Option 3) L1 and L3 filtering per gNB beam

Reference

[1]	3GPP TS 36.331 V13.1.0, “Radio Resource Control (RRC) Protocol Specification,” April 2016