3GPP TSG-RAN WG2 #NR AH1801	Tdoc R2-1800327

# Vancouver, Canada, 22nd – 26th January, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of support for IMS Emergency calls in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-01-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | SA2 has progressed Emergency Services in 3GPP TS 23.501 and, among other things, described that, “… For emergency services, other than eCall over IMS, the UEs in limited service state determine that the cell supports emergency services over NG-RAN from a broadcast indicator in AS. Emergency calls for eCall Over IMS are only performed if the UE has a UICC.” and “…A serving network shall provide an Access Stratum broadcast indication to UEs as to whether eCall Over IMS is supported. A UE that is not in limited service state determines that the cell supports eCall Over IMS using the broadcast indicator for eCall over IMS”. This CR introduce these broadcast indicators in 38.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of IMS Emergency and eCall over IMS broadcast indicator descriptions in 38.300 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 RAN IMS emergency description missing/not alinged with SA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

## 16.x	IMS Emergency

### 16.x.1	IMS Emergency call

IMS emergency calls are supported in this release of the specification and UE may initiate an IMS emergency call if the network supports it. An IMS Emergency call support indication is provided to inform the UE that emergency bearer services are supported. In normal service state the UE is informed if the PLMN supports emergency services through an Emergency Service Support indicator in the Attach and TAU procedures [ref. to 23.501]. In limited service state and for emergency services other than eCall over IMS, a UE is informed about if a cell supports emergency services over NG-RAN from a broadcast indication (ims-Emergency). The broadcast indicator is set to “support” if any AMF in a non-shared environment or at least one of the PLMN’s in a shared environment supports IMS emergency bearer services.

### 16.x.2	eCall over IMS

NG-RAN broadcast an indication to indicate support of eCall over IMS (eCallOverIMS). UE’s that are in limited service state need to consider both eCallOverIMS and ims-Emergency to determine if eCall over IMS is possible.

End of changes