3GPP TSG-RAN WG2 NR Ad hoc 1801	R2-1800535

Vancouver, Canada, 22 – 26 January 2018

Agenda item:	10.2.1

Source:	Rapporteur (Nokia)

Title:	NR Stage 2 Open Issues

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This contribution keeps track of all the FFSs which were removed from 38.300 before RAN#78. It takes R2-1712357 as starting point and relies of track changes.

# 2	Open Issues

The list of Stage 2 open issues with corresponding subclauses is given below.

Inactive State

- FFS if data transmission is possible (7.2)

- FFS if PLMN selection is supported (7.2)

- FFS whether initial access from RRC_INACTIVE also relies on the default numerology (7.3)

- The relation between the I-RNTI and the network identifier used to locate the UE context is FFS (8.1)

- How to use the gNB ID in the context of mobility in RRC_INACTIVE and the corresponding identification of the UE Context (I-RNTI) has to be discussed (8.1)

- Cell reselection in general (9.2.2.2)

- alternatives selected for RNA configuration (9.2.2.3)

Access Control

- Whether the unified access barring mechanism is actually applicable for all RRC states in NR (RRC_IDLE, RRC_CONNECTED and RRC_INACTIVE) (7.4)

- FFS what NAS does for RRC_INACTIVE and FFS for RRC_IDLE whether the information is directly provided by NAS, derived from the access category.... (7.4)

Transport of NAS Messages

- Transport of NAS messages (7.6), in particular if separate NAS messages are used for PDU session establishment (Annex A)

Identities

- Network Identities: How to use the gNB ID in the context of mobility in RRC_INACTIVE and the corresponding identification of the UE Context (I-RNTI) has to be discussed. This also contains discussions on possible size limitations of the I-RNTI (8.2)

- Network Identities: It is FFS and subject to SA2 whether TAIs in the NG-RAN have the same format (or share same ID space) as TAIs in E-UTRAN (8.2)

- Network Identities: The format of the S-NSSAI is FFS subject to SA2 (8.2)

Idle

- Initial Access: FFS whether initial access from RRC_IDLE relies on the default numerology (9.2.1.1).

- Idle: FFS for which services the service specific frequency prioritisation is applied and how it applies for the case of network slices (9.2.1.2).

Mobility

- Exact message names FFS (9.2.3.1)

- FFS whether QoS flow can be remapped at handover and, if supported, whether the handover is lossless in this case (9.2.3.1)

- FFS whether there may be cases for which intra-cell mobility needs to be handled by RRC (9.2.3.1)

- FFS whether the AS configuration includes the QoS flow to DRB mapping that was not configured to the UE via RRC signalling (9.2.3.1)

- FFS whether there is a need to provide the system information from source equivalent to LTE’s MIB, SIB-1 and SIB-2 (9.2.3.1)

- FFS if there could be exceptions where the Handover Command is entirely generated by the target gNB - e.g. any enhancements similar to MBB enhancement in LTE, if supported (9.2.3.2.1)

- FFS for which cell(s) beam measurement information can be included e.g. only candidate target cell (9.2.3.2.1) → solved with the agreed CandidateCellInfoList

- U-Plane handling for handover (9.2.3.2.2)

Measurements

- Possible difference between serving and non-serving could be the value of “N” i.e. N best beams - FFS (9.2.4) → solved with the agreement that “Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s).”

- FFS whether cell and beam measurements need to be consistent (9.2.4) → solved with the agreed reportQuantityRsIndexes

Miscellaneous

- Random Access: FFS whether the gNB can be provided with more information (compared to LTE) from the UE on the Msg.3 to provide. (9.2.6)

- Downlink rate control (10.5.1)

- QoS: Notification Control (which when included indicates that notification is requested when the QoS of some flows can no longer be met) is pending clarifications from SA2 (12)

- UE Capabilities: RAN2 aim not to duplicate band combinations to indicate the combination of DL bands and UL bands and RAN2 aim to introduce per UE baseband capabilities separated from the band combinations (e.g. FD-MIMO capabilities, number of CSI processes, etc.) and minimise per band combination signalling (14)

- Self-Configuration and Self-Optimisation (15)