3GPP TSG-RAN WG2 NR Ad hoc 0118	R2-1801348

Vancouver, Canada, 22nd January – 26th January 2018

Agenda Item:	11.1

Source: 	ITRI

Title:  	Discussion on the topology and protocol stack of IAB

Document for:	Discussion

# 1	Introduction

Study item, Study on Integrated Access and Backhaul for NR, was approved and will be discussed in RAN2 NR Ah in January. Based on the SI proposal [1], we discussed the topology management issue in this paper.

# 2	Discussion

LTE relaying was proposed to enhance performance at the cell edge for a comparatively low cost. In LTE Rel. 10, because of the features of improved received SINR due to noise elimination, ease of coordinating standard specifications, and ease of implementing the technology, layer 3 relay technology was agreed to standardize specifications. Figure 1 is the overall E-UTRAN architecture supporting RNs (Relay Nodes). An RN acts as an UE to the DeNB (Donor eNB) so an Un interface is specified. An RN acts as an eNB to the UE, so S1 and X2 interfaces are specified.

Figure 1 Overall E-UTRAN architecture supporting RNs

However in NR, due to the expected larger bandwidth available compared to LTE, an opportunity to develop and deploy IAB (integrated access and backhaul) links with different challenges is created. For example, since mmWave is used, a fast RAN-based mechanism for switching between rTRPs (relay transmission/reception point) is need to solve the severe short-term blocking problem. That implies that the rTRP in IAB network may not have the S1 interface to communicate with MME, instead an rTRP needs to realize the autonomous configuration and adaptation with the anchor node and other rTRPs. Detailed objectives of IAB are listed in [1], and we focus on the topology management issue in this paper. Since the IAB topology considers operation of multiple relay hops between the anchor node and UE, we propose two kinds of topologies in Figure 2.

- Tree topology

- Mesh topology

Figure 2 Overall IAB architecture supporting rTRPs

The rTRP in Figure 2 has two interfaces, one is Xn for communicating with Anchor node and other rTRPs, and the other is NR to access the UE. Based on the down-stream direction from the anchor node, one rTRP has one-input interface, multiple-output interfaces in a tree topology; and one rTRP has multiple-input/multiple-output interfaces in a mesh topology. The following table shows a quick comparison between the tree and mesh. We suggest that the two kinds of IAB topologies, tree and mesh, are both included in TR 38.874. However, for simplicity, we prefer to focus on the tree architecture first.

|  | Backhaul management | Resource allocation | Interference avoidance/mitigation | Routing optimization |
| --- | --- | --- | --- | --- |
| Tree | the backhaul establishment and maintenance may be simpler due to the simple tree topology. | An rTRP receives the grants from only one-input interface, so resource management is simpler. | The synchronized-link interference is most, and the cross-link interference may be mild. | RAN-based mechanisms may be centralized. For the requirement of redundant connectivity, a dynamic tree may be designed. |
| Mesh | the number of backhaul links is larger, and the backhaul relationship between rTRPs may be more complicated. | An rTRP receives multiple grants from multiple-input interfaces, so a negotiation between rTRPs is needed. | Both synchronized-link and cross-link interferences need well resolved. | RAN-based mechanisms may be distributed, and inherently with good redundant connectivity. |

Figure 3 shows the functionalities of the anchor and rTRPs. We assume that there are two kinds of rTRPs, one with higher capabilities is called rTRP-H, and the other called rTRP-L only has autonomous configuration search and receiving, MAC and PHY functionalities. According to this assumption, we propose two possible protocol stacks in Figure 4. Option 1 is similar to the protocol stack in LTE relay, and provides the link guarantee. Option 2 has the end-to-end guarantee between rTRP-H and UE, and may have benefits for the packet-loss resolution in a multi-hop IAB.

Figure 3 functionalities of the anchor and rTRPs

- Option 1

- Option 2

Figure 4 Protocol Stack of IAB

The protocol stack and RAN-based mechanisms, such as scheduling and routing, are highly dependent on the IAB topology, thus we propose that RAN2 should clarify the network architecture and protocol stack with RAN3, then RAN2 may focus on the backhaul and access link scheduling for autonomous configuration and adaptation.

Proposal 1: RAN2 should clarify the network architecture and protocol stack with RAN3, then RAN2 may focus on the backhaul and access link scheduling for autonomous configuration, backhaul establishment, route selection and migration, etc.

# 3 	Conclusions

Based on the discussion, we have the following proposal:

Proposal 1: RAN2 should clarify the network architecture and protocol stack with RAN3, then RAN2 may focus on the backhaul and access link scheduling for autonomous configuration, backhaul establishment, route selection and migration, etc.

# 4 	References

[1] RP-171880 New SID Proposal: Study on Integrated Access and backhaul for NR, AT&T, Qualcomm, Samsung, RAN Meeting #77