3GPP TSG-RAN WG2 NR Ad Hoc 	R2-1801540

# Vancouver, Canada, 22–26 January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PHR MAC CE in EN-DC in TS38.321 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-01-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Considering it was agreed in RAN2#100 meeting, for EN-DC, only Multiple Entry PHR MAC CE with 4 octets CC bitmap will be used.This should be clarified in MAC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that for EN-DC, Multiple Entry PHR MAC CE will only use four octets CC bitmap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For EN-DC, Multiple Entry PHR MAC CE with single octets CC bitmap may be used by mistake. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR ... CR ... TS38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.3.9	Multiple Entry PHR MAC CE

The Multiple Entry PHR MAC CE is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-2. It includes the bitmap, a Type 2 PH field and an octet containing the associated PCMAX,c field (if reported) for the PCell, a Type 2 PH field and an octet containing the associated PCMAX,c field (if reported) for either PSCell or PUCCH SCell, a Type 1 PH field and an octet containing the associated PCMAX,c field (if reported) for the PCell. It further includes, in ascending order based on the ServCellIndex, one or multiple of Type 1 PH fields and octets containing the associated PCMAX,c fields (if reported) for SCells indicated in the bitmap.

The presence of Type 2 PH field for PCell is configured by phr-Type2PCell, and the presence of Type 2 PH field for either PSCell or for PUCCH SCell is configured by phr-Type2OtherCell.

A single octet bitmap is used for indicating the presence of PH per SCell when the highest SCellIndex of SCell with configured uplink is less than 8, otherwise four octets are used. For EN-DC, four octets are always used.

UE determines whether PH value for an activated Serving Cell is based on real transmission or a reference format by considering the downlink control information which has been received until and including the PDCCH occasion in which the first UL grant is received since a PHR has been triggered.

The PHR MAC CEs are defined as follows:

-	Ci: this field indicates the presence of a PH field for the SCell with SCellIndex i as specified in TS 38.331 [5]. The Ci field set to "1" indicates that a PH field for the SCell with SCellIndex i is reported. The Ci field set to "0" indicates that a PH field for the SCell with SCellIndex i is not reported;

-	R: reserved bit, set to "0";

-	V: this field indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, V=0 indicates real transmission on PUSCH and V=1 indicates that a PUSCH reference format is used. For Type 2 PH, V=0 indicates real transmission on PUCCH and V=1 indicates that a PUCCH reference format is used. For Type 3 PH, V=0 indicates real transmission on SRS and V=1 indicates that an SRS reference format is used. Furthermore, for Type 1, Type 2, and Type 3 PH, V=0 indicates the presence of the octet containing the associated PCMAX,c field, and V=1 indicates that the octet containing the associated PCMAX,c field is omitted;

-	Power Headroom (PH): this field indicates the power headroom level. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1 (the corresponding measured values in dB for the NR serving cell are specified in TS 38.133 [11] while the corresponding measured values in dB for the LTE serving cell are specified in TS 36.133 [12]);

-	P: this field indicates whether the MAC entity applies power backoff due to power management. The MAC entity shall set P=1 if the corresponding PCMAX,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,c: if present, this field indicates the PCMAX,c or P̃CMAX,c (as specified in TS 38.213 [6]) used for calculation of the preceding PH field. The reported PCMAX,c and the corresponding nominal UE transmit power levels are shown in Table 6.1.3.8-2 (the corresponding measured values in dB for the NR serving cell are specified in TS 38.133 [11] while the corresponding measured values in dB for the LTE serving cell are specified in TS 36.133 [12]).

Figure 6.1.3.9-1: Multiple Entry PHR MAC CE with the highest SCellIndex of SCell with configured uplink is less than 8

Figure 6.1.3.9-2: Multiple Entry PHR MAC CE with the highest SCellIndex of SCell with configured uplink is equal to or higher than 8