3GPP TSG-RAN WG2 NR Ad hoc 1801		R2-1801637

Vancouver, Canada, 22 – 26 January, 2017

Agenda Item:	10.4.2.3

Source: 		Intel Corporation

Title: 		Offline discussion #42: PDCP version change for SRBs

Document for:			Discussion and Decision

# Discussion

# SRB:

## NR PDCP to LTE PDCP

Proposal #1: PDCP version change is done using Release and add the PDCP entity for this SRB.  Use a bit in LTE SRB addmodlist to configure this change:

### 36.331 TP

SRB-ToAddModList ::=				SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=	SEQUENCE {

srb-Identity						INTEGER (1..2),

rlc-Config							CHOICE {

explicitValue						RLC-Config,

defaultValue						NULL

}		OPTIONAL,																-- Cond Setup

logicalChannelConfig				CHOICE {

explicitValue						LogicalChannelConfig,

defaultValue						NULL

}		OPTIONAL,																-- Cond Setup

...,

[[	pdcp-verChange-r15				ENUMERATED {true}	OPTIONAL			-- Cond NR-PDCP

]]

}

RadioResourceConfigDedicated field descriptions

pdcp-verChange Indicates that the PDCP version of the SRB is changed from NR PDCP to LTE PDCP. Network only configures this version change for during handover, Resume and first reconfiguration after re-establishment.

| Conditional presence | Explanation |
| --- | --- |
| NR-PDCP | The field is optional present, Need ON, when the SRB is configured with NR-PDCP prior to reception of this reconfiguration message. Otherwise it is not present. |

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration (SRB reconfiguration):

2>	if the pdcp-verChange is received (i.e, NR PDCP to E-UTRA PDCP change):

3>	establish  PDCP entity and configure it with the current (MCG) security configuration;

3> associate the RLC bearer of this SRB with the established PDCP entity;

3>	release the NR PDCP entity of this SRB;

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2>	reconfigure the DCCH logical channel in accordance with the received logicalChannelConfig;

## LTE PDCP to NR PDCP:

Proposal #2: PDCP version change from LTE to NR PDCP is done using Release and add the PDCP entity for this SRB.   Use SRB-toaddmodlist in RBConfig to reconfigure this change.

Already covered by existing spec.  No further changes needed.

### Current spec as FYI

##### 5.3.5.6.3	SRB addition/ modification

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration or configured with pdcp-Config (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity and configure it with the security algorithms according to securityConfig and apply the keys (KUPenc) associated with the KeNB/S-KgNB as indicated in keyToUse, if applicable;

2>	if the current UE configuration as specified in TS 36.331 includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC and DCCH entities of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config.

2>	else:

3>	configure the PDCP entity in accordance with the specified configuration defined in 9.1 for the corresponding SRB;