3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810146

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.4.1.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Provision of NSSAI by CN for overload control

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

SA2 agreed that slice based overload control with possible impacts to RAN. However, the complete is pending RAN2 and RAN3 insight.

# 2	Discussion

## 2.1	NSSAI check in RRC setup

The AMF supports mechanisms for avoiding and handling overload situations. For this purpose the AMF may include the S-NSSAI(s) in N2 overload control message sent to 5G-AN node(s) to indicate the congestion of the Network Slice(s) at the CN part. Given the indication in gNB, N2 overload control can result in RRC reject or unified access barring [2].

SA2 agreed that the NG-RAN performs slice based overload control during RRC connection establishment procedure on request from the AMF, by checking Requested NSSAI included in the RRC connection establishment. However, there was no agreements in SA2 whether that is needed during Service Request [1].

RAN2 receives in the SA2 LS [1] query to answer:

1) Does RAN need Requested NSSAI during RRC Connection establishment procedure triggered by the NAS procedure other than Registration procedure (e.g. Service Request procedure)? If the RAN needs it then how does the RAN use Requested NSSAI in RRC Connection establishment procedure for purposes different than AMF routing?

TS 23.501[2] defines:

Requested NSSAI: NSSAI provided by the UE to the Serving PLMN during registration.

Checking Requested NSSAI is needed to filter admission in NG-RAN under high network load conditions. In case NG-RAN has been notified that AMF suffers from overload for some slices, the slices should be identified and limited in access attempts.

The RRC signaling has not intentionally distinguished different triggers (e.g. Registration or Service Request) for NSSAI provision. The RRC signaling capability needs to ensure encoding of NSSAI (full assistance information) anyway:  RAN2 agreed that MSG5 contains NSSAI as provided by upper layers. According to TS38.300 [3] and pending implementation of the stage 3 detailed signaling [4], the RRC Setup Complete message does not condition format or purpose of NSSAI. I.e. NSSAI support should be mandatory based on provision by NAS.

Is it needed in RAN?

Let’s assume the following scenario:

- the UE supporting two slices (IoT, eMBB)

- there is load condition for IoT (maybe local to RAN, or maybe triggered by Overlaod control from N2.

The current assumption is that the UE provides full NSSAI (IoT, eMBB) in RRCSetupComplete message during Registration.

There might be different methods of handling the Service Request and algorithms that decide on the connection establishment:

Let’s suppose the trigger is a SR triggered by NAS signalling related to IoT:

1) RRC connection establishment with NSSAI that includes only IoT gets rejected. To then send signalling for eMBB a second attempt including eMBB needs to be issued. Threafter IoT signalling also can be sent on same RRC connection.

2)RRC connection establishment with  NSSAI that includes IoTand eMBB does not get rejected as eMBB is allowed slice. Then UE can send NAS singalling for both IoT and eMBB on same RRC connection.

The differnce between 1 and 2 is an extra RRC connection establishment in case one is needed. i.e. if we include a NSSAI that has a smaller set of sliced than the allowed NSSAI we may overall increase the # of RRC connection establishment attempt and still we are unable to avoid the fact that some NAS signalling for slices that are subject to Access stratum level load management can use connections established thanks to the fact the UE idicates in NSSAI slices that are not subject to load management.

A method to block slice specific request may be  unified access control, but then this would become a routine tool for load management also on a short term basis and based on control plane logic (whereas today access control it is more CN or OAM controlled) Effectively this would cause the RAN to block specific NAS layer  signalling. If this was not done then we can also establish RRC connection for slices not subject to UAC  and then send NAS signalling for any slice over the established RRC connection. In general, routine use of UAC is not advisable also because it would require frequent UAC Categories configuration in UE and frequent SIB updates to all UEs and these are slow and resource consuming procedures.

Observation: There is no benefit to limit NSSAI provision to Registration and its content to just a subset of slices (actually this will on average increase RRC connection establishment related traffic).

Proposal 1: Allowed NSSAI is included also for SR.

## 2.2	NSSAI impacts to Paging

The other question to answer to SA2 LS [1] is:

2) Would the RAN benefit from receiving an Allowed NSSAI from CN during CN initiated paging procedure?

AMF under congestion, can filter the delivery of some paging messages itself. Hence, we assume the slice specific extensions to Paging would help the NG-RAN to filter some paging attempts in case it suffers from overload of some slices. This would serve optimized paging in RAN domain.

Would the RAN benefit from slice specific paging?

Paging attempts corresponding to the slice that is under NW load control may generate useless RRC connection establishments which in turn will get disallowed (by UAC) or released (as per paragraph above) thus further increasing uselessly the NG-RAN load. Due to these reasons, there may be need to consider slicing during Paging. We note PagingPriority in N2 paging message is the global priority. Each slice is a network specific setup and Paging priority can be scoped to the slice. The slicing information if considered to help the NG-RAN node to filter out paging, could be correlated with the priorities. This requires RAN3 insight and decision.

However, Paging content over RRC should be very carefully thought out. The newtork may address multiple UEs within Paging message. When the UE reads the Paging Channel and finds its paging group, but does not find its unique and precise UE identity(I-RNTI), it leads to unnecessary waste of resources and introduces complexity to conformance tests.

Proposal 2: Slicing information does not impact RRC Paging message. However it is beneficial for the RAN to know what slices a paging message relates to.

# 3	Conclusion

Observation: There is no benefit to limit NSSAI to just a subset of slices (actually this will on average increase RRC connection establishment related traffic).

Proposal 1: Allowed NSSAI is included also for SR.

Proposal 2: Slicing information does not impact RRC Paging message. However it is beneficial for the RAN to know what slices a paging message relates to.

Proposal 3: Reply to SA2 that provision of NSSAI during RRC Connection establishment procedure is not limited to Registration procedure, but providing different NSSAI for Registration and SR should be avoided. For Paging handling, certain benefits are seen if the RAN is limiting/not providing service for certain slices (realization and impacts need to be assessed by RAN3).

Corresponding LS in [5].

# References

- S2-186222, LS on inclusion of NSSAI in RRC connection establishment procedure

- TS23.501, System Architecture for the 5G System, SA2

- TS38.300, NR, Stage 2, RAN2

- R2-1807172, NR RRC signalling for slice assistance information – Nokia, Nokia Shanghai Bell

- R2-1810147, Draft Reply LS on inclusion of NSSAI in RRC connection establishment procedure, Nokia