3GPP TSG-RAN WG2 Meeting #AH 1807 	R2-1810190

# Montreal, Canada, 2nd July 2018 – 6th July 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0253 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BWP switching for BFR RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-06-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When BFR RACH is initiated, and if the active UL BWP is configured with PRACH occasions (i.e., BeamFailureRecoveryConfig), the UE will switch the active DL BWP to the DL BWP with the same BWP ID as the active UL BWP. However, the BeamFailureRecoveryConfig has already included the search space ID, and the UE will monitor the PDCCH according to the configured search space ID. It should be noted that the search space ID is unique among the BWPs of a serving cell according the TS 38.331, which means the search space ID is associated with a specific DL BWP. So, if the UE switches to the DL BWP with the same index as active UL BWP but there is no such search space ID configured on the DL BWP, the UE can not monitor the PDCCH according to the beam failure recovery configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When BFR RACH is initiated, the DL BWP should be switched to the DL BWP on which the recoverySearchSpaceId is configured. Impact analysis Impacted functionality: BWP operation. Inter-operability: - If the network is implemented according to the CR and the UE is not there are no inter-operability problems. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE can not monitor the PDCCH according to the beam failure recovery configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification 1>

### 5.4.5	Buffer Status Reporting

[Omit]

Upon initiation of the Random Access procedure on a Serving Cell, the MAC entity shall for this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is a SpCell:

3>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is a SpCell:

3>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17) and if the active DL BWP does not configure the recoverySearchSpaceId:

4>	switch the active DL BWP to the DL BWP configured with the recoverySearchSpaceId.

3>	else if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

[Omit]

<End of modification 1>