3GPP TSG-RAN WG3 Meeting #101bis	     R3-186260

Chengdu, P. R. China, 08-12 October 2018

Agenda Item:	24.1.2.1

Source:	Ericsson

Title:	TP to TR 38.874: Comparison of CP Alternatives of Architecture 1a

Document for:	pCR to TR 38.874

# Introduction

The online discussion about CP alternatives for IAB architecture option 1a has been assigned the following comeback:

CB: # 77_IAB_CP_alts_comparison

-  merge from online disc, including 5731 (6021 had apparently no support?)

- check details

- aim is to compare focusing on RAN3 aspects (e.g. F1AP impact, logical node complexity, …)

- recipient is also RAN2

- company coordination: ideally RAN2 discussion should not be stalled because of this

# TP to TR 38.874

### 8.3.4 CP alternatives for architecture 1a

<<skipped text>>

The comparison analysis of the five CP alternatives are provided in the Table 8.3.4-x. More comparison aspects are not excluded.

Table 8.3.4-1. Comparison of the five CP alternatives of architecture 1a

| Comparison aspects |  | Alt 1 | Alt 2 | Alt 3 | Alt 4 | Alt 5 | Comparison analysis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transport for CP signalling on wireless plane | UE/IAB-MT’s RRC | SRB in access link, SRB over RLC channel in backhaul links | Same as alt 1 | Same as alt 1 | Same as alt 1 | Same as alt 1 | SRB is recommended to carry UE/IAB-MT’s RRC signaling in all the alternatives. Use of SRB to carry IAB-DU’s F1AP may require definition of new SRB(s). Use of DRB to carry IAB-DU’s F1AP configures the DRB with high priority. |
|  | IAB-DU’s F1AP | SRB of collocated MT | Same as alt 1 | Same as alt 1 | RLC channel | DRB |  |
| Encapsulation on the wireless plane | UE/IAB-MT’s RRC | Within PDCP but without encapsulation in F1-AP of serving IAB node | Within PDCP and F1-AP of serving IAB node | Same as alt 1 | Same as alt 2 | Same with alt 1 | The case without encapsulating RRC message into F1-AP message has an impact since the F1 (i.e. non-RRC) information need to be transferred in some other way to the IAB node. |
|  | IAB-DU’s F1AP | Within RRC of collocated MT | Within PDCP of collocated MT | Same as Alt 2 | Within DTLS/SCTP/IP above RLC channel | Within PDCP of collocated MT | All options has some protocol impacts compared to normal Uu stack carrying IP packets over PDCP. |
| Encapsulation (over the wired donor CU-DU link) | UE/IAB-MT’s RRC | Native F1-C to donor DU | Within end to end F1-AP/PDCP and Native F1-C to donor DU | Same as Alt1 | Native end to end F1-C | Same as Alt1 | On alternative 1, 3, 5 the F1-C is terminated in the Donor-DU meaning that the F1-C information related to RRC message need to be transferred in some other way to IAB node. In alternative 2 the donor DU has some new F1-C transparent tunnelling mechanism where PDCP packets are forwarded to RLC channels. In alternative 4 the donor DU forward IP packets destined for the IAB node to the RLC channel. |
|  | IAB-DU’s F1AP | Within end to end RRC/PDCP and Native F1-C to donor DU | Within end to end PDCP and Native F1-C to donor DU | Same as Alt2 | Native end to end IP | Within end to end PDCP and GTP-U to the donor | In all solutions the F1-AP protocol is terminated in IAB node. |
| Security of F1AP |  | Protected by PDCP | Same as alt 1 | Same as alt 1 | Protected by DTLS | Same as alt 1 | Using PDCP for F1-AP security incurs less overhead. The total overhead of F1-AP signalling may be insignificant compared to F1-U traffic. |
| Routing of control plane PDUs |  | Adaptation layer is responsible for routing | Same as alt 1 | Same as alt 1 | Same as alt 1 | Same as alt 1 | In all alternatives, the adaptation layer is used for routing. |
| Impact to IAB donor CU |  | Support of using donor F1-AP to directly transfer RRC messages to UE or IAB MT Support of using donor F1-AP to transfer end to end F1-AP to IAB DU. | Support of using donor F1-AP to transfer F1-AP messages to the IAB node | Same as alt 1 for RRC messages to UE or IAB MT Same as alt2 for F1-AP to IAB DU | Support for DTLS | Same as alt 1 for RRC messages to UE or IAB MT Support for transport of F1-AP messages over E1 |  |
| Impact to IAB donor DU |  | Support for mapping F1-AP messages to the proper adaptation layer address | Same as alt1 | Same as alt1 | Support for IP routing function (the same functionality as in UP option e) | Same as alt1 for RRC messages to UEs and IAB’s MT Support for mapping GTP-U tunnels to the proper adaptation layer address (for F1-AP to IAB DU) |  |
| The transport of RRC-related and non-RRC-related F1-AP messages. |  | No | Yes | No | Yes | No | Solution 1, 3, 5 transmit F1-AP and RRC messages in different ways, thereby not being able to transport F1-AP messages to the IAB node that have embedded RRC message to the UE. |