3GPP TSG-RAN WG3 Meeting #103	R3-190693

# Athens, Greece, 25th, February – 1st March 2019

Agenda Item:	9.2

Source:	Ericsson

Title: 	Intra eNB CP Relocation for NB-IoT

Document for:	Discussion and Approval

# 1	Discussion

For NB-IOT UEs using Control Plane CIoT EPS optimizations, after UE has initiated a RRC Re-establishment procedure, eNB uses the eNB CP Relocation Indication procedure to request authentication and establishment of the S1 connection. The specification has assumed that the eNB receiving the RRC Re-establishment is the new eNB, which has not serviced the UE before.

For example in TS 36.300, it says:

When the new eNB receives the RRCConnectionReestablishmentRequest message, it triggers the eNB CP Relocation Indication procedure including NAS-level security information received from the UE.

..

# Interactions with the MME CP Relocation and UE Context Release procedures:

In case of successful UE authentication, the MME initiates the UE Context Release procedure to release the UE's S1-connection in the old eNB. The MME may initiate the MME CP Relocation procedure before the release procedure in order to trigger the old eNB to return non-delivered NAS PDUs to the MME.

In the case of Intra eNB CP relocation, the above stage 2 text becomes inaccurate.

Proposal 1:   The stage 2 procedure text in eNB CP Relocation is correct to cover the intra eNB case.

# 2	Proposal

Proposal 1:   The stage 2 procedure text in eNB CP Relocation is correct to cover the intra eNB case.

# 3	Reference

[1]	R3-190694	Correction on eNB CP Relocation, Ericsson