3GPP TSG-RAN3 Meeting #104	R3-193023

Reno, U.S., 13th – 17th May 2019

Agenda Item:	14.3

Source: 	Ericsson

Title:  	Support of extended DRX in CM-IDLE

Document for:	Discussion

# Introduction

New Rel-16 WID revisions for eMTC [1] and NB-IoT [2] have been proposed in RAN#83 with the following objective: ​

Connection to 5GC: - Specify support for the following features [RAN2, RAN3] - Support of extended DRX in CM-IDLE - Support of extended DRX in CM-CONNECTED with RRC_INACTIVE (support of sleep cycles up to the NAS and SMS retransmission timers) - Support of EDT for Data over NAS and UP solution (see Note) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC Note: Based on the outcome of RAN2/SA2 liaison exchanges, UP solution to be supported for connection to 5G-CN may be later updated.

This contribution presents our view on the changes needed in RAN3 specifications to support extended DRX in CM-IDLE.

# Discussion

In previous RAN3 meeting, the topic of support of extended DRX in CM-IDLE when connecting CIoT UEs to 5GC was briefly discussed. It was observed by some companies that extended DRX for UEs in CM-IDLE would be similar to Rel-13 eDRX in LTE-M and NB-IoT; at least from a RAN perspective.

In EPC, the MME provides the eNB with the extended DRX parameters (Paging eDRX cycle, PTW, etc.) via the S1 Paging message. From RAN3 point of view, in order to limit the specification impacts, we suggest to include the same eDRX related parameters (the Paging eDRX Information and NB-IoT Paging eDRX Information IEs) in NGAP Paging. In this way, when the CIoT UE is connected to 5GC, the 5GC can provide eDRX parameters over NGAP PAGING message to the NG-RAN node. The latter can hence use, similarly to when the CIoT UE is connected to EPC, the same mechanism to configure eDRX to UEs in CM-IDLE via paging.

Include the Paging eDRX Information and NB-IoT Paging eDRX Information IEs in NGAP PAGING message

We provide the corresponding CR in [3].

# Conclusion

In the previous section, we discussed how to support e DRX in CM-IDLE when connecting to 5GC and proposed the following:

- Include the Paging eDRX Information and NB-IoT Paging eDRX Information IEs in NGAP PAGING message

# Reference

- RP-190770, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, RAN#83

- RP-190757, “WID revision: Additional enhancements for NB-IoT”, Huawei, RAN#83

- R3-193023, “Support of extended DRX in CM-IDLE for NB-IoT/LTE-M connected to 5GC”, Ericsson, RAN3#104