3GPP TSG-RAN WG3 Meeting #105	R3-194134

# Ljubljana, Slovenien, 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0412 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing alternative QoS profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA2 has agreed in S2-1908223 to introduce Alternative QoS profiles. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Alternative QoS profiles for GBR QoS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to send Alternative QoS Profiles. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.26, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0196 TS 38.413 CR 0219 TS 38.473 CR 0121 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.27 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.29 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows only. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and shall be ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. | - |  |
| Paging Priority Indicator (PPI) | O |  | 9.3.1.55 |  | - |  |
| RDI | O |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. | - |  |
| Alternative QoS Profile List | 0..<maxnoofAlternativeQoSProfiles> |  |  | This IE may be present for GBR QoS flows only. It is the Alternative QoS Profile as specified in TS 23.501 [20]. | YES | ignore |
| > GBR QoS Flow Information | M |  | 9.3.1.30 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAlternativeQoSProfiles | Maximum no. of Alternative QoS Profiles for the GBR QoS flows that are under Notification Control. Value is 8. |

-------------------------

Skip to the next change

-------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-AlternativeQoSProfileList,

maxnoofAlternativeQoSProfiles,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI

-------------------------

Skip to the next change

-------------------------

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

ActivityInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ActivityNotificationLevel	::=	ENUMERATED	{

drb,

pdu-session,

ue,

...

}

AveragingWindow  ::= INTEGER (0..4095, ...)

AlternativeQoSProfileList ::= SEQUENCE (SIZE(1..maxnoofAlternativeQoSProfiles)) OF GBR-QoSFlowInformation

-------------------------

Skip to the next change

-------------------------

#### -- Q

QCI ::= INTEGER (0..255)

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				Non-Dynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer	{{QoS-Characteristics-ExtIEs}}

}

QoS-Characteristics-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

QoS-Flow-Identifier	::=	INTEGER (0..63)

QoS-Flow-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Item

QoS-Flow-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoS-Flow-Mapping-Indication		PRESENCE optional},

...

}

QoS-Flow-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Failed-Item

QoS-Flow-Failed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Mapping-Item

QoS-Flow-Mapping-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowMappingIndication						QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Mapping-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Mapping-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-Indication ::= ENUMERATED {ul, dl, ...}

QoS-Parameters-Support-List	::= SEQUENCE {

eUTRAN-QoS-Support-List			EUTRAN-QoS-Support-List			OPTIONAL,

nG-RAN-QoS-Support-List			NG-RAN-QoS-Support-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { QoS-Parameters-Support-List-ItemExtIEs} } OPTIONAL,

...

}

QoS-Parameters-Support-List-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSPriorityLevel ::= INTEGER (0..127, ...)

QoS-Flow-QoS-Parameter-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-QoS-Parameter-Item

QoS-Flow-QoS-Parameter-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-QoS-Parameter-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-QoS-Parameter-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-AlternativeQoSProfileList	CRITICALITY ignore	EXTENSION AlternativeQoSProfileList	PRESENCE optional},

...

}

-------------------------

Skip to the next change

-------------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

-------------------------

Skip to the next change

-------------------------

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAlternativeQoSProfiles				INTEGER ::= 8

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

-------------------------

Skip to the next change

-------------------------

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID													ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance											ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication									ProtocolIE-ID ::= 80

id-AlternativeQoSProfileList								ProtocolIE-ID ::= 1xy

END

-- ASN1STOP