3GPP TSG-RAN3 Meeting #105	R3-194189

Ljubljana, Slovenia, 26th August – 30th August 2019

Agenda Item:	31.3.02

Source: 	Ericsson

Title:  	Discussion on the need of signalling of SPID and ARPI over E1

Document for:	Discussion

# Introduction

At the last RAN3 meeting a CR in R3-193211 was drafted and cosigned by a number of companies, proposing to signal the Subscriber Profile ID for RAT/Frequency priority IE and the Additional RRM Policy Index IE parameters over the E1 interface. The discussion was marked as “to be continued” and it was proposed that a discussion document is provided to summarise the arguments given in support of the previously submitted CR. This document resumes the main reasons for proposing signaling of SPID and ARPI over E1.

# Discussion

SPID and ARPI are indexes that can point at different RRM policies. RRM policies may consist of a number of behaviours aimed at differentiating the treatment of traffic for a given UE.  As an example, an RRM policy could consist of a scheduling policy, by which a UE is treated in a better (e.g. faster, over-provisioned) way with respect to other UEs.

The introduction of the ARPI value was agreed in SA2 and reflected in RAN specifications to allow for the use of different RRM policies for UEs supporting the same services. Namely, the purpose is to enable a differentiated user plane treatment for two UEs that are potentially using a service with the same QoS parameters.

Observation 1: SPID and ARPI enable user plane policy differentiation between UEs that could potentially support bearers with the same QoS Parameters

When enabling a policy for user plane traffic, the node hosting PDCP, i.e. the gNB-CU-UP, is a key player. For example, the gNB-CU-UP needs to allocate sufficient processing resources to serve a UE with a given traffic policy. In the example of two UEs using a single bearer with the same QoS parameters, if one of these UEs is given a priorirtised policy via SPID/ARPI, then the gNB-CU-UP would have to increase the processing resources used for serving the UE in question, otherwise UP traffic may be bottlenecked at gNB-CU-UP, which defeats the whole purpose of policy differentiation.

As a second example, the gNB-CU-UP may implement pre-scheduling mechanisms to packets before they enter PDCP. This is purely up to implementation, but it may constitute a smart way to improve performance. Traffic for a UE with a prioritised UP policy might be placed ahead of traffic for lower priority UEs, before entering PDCP. This would facilitate prioritisation of traffic and it would be beneficial especially if the amount of traffic reaching the gNB-CU-UP (from UPF) is so large to require buffering at gNB-CU-UP.

Therefore, it seems logical and beneficial to make the gNB-CU-UP aware of SPID and ARPI, as this would enable the node to recognise the UP policy to be given to the UE. Being the gNB-CU-UP the node that handles UP traffic transmission to the gNB-DU, policies for an optimised transmission to the gNB-DU can be enabled based on SPID and ARPI knowledge.

Conclusion: Signalling of SPID and ARPI to the gNB-CU-UP enables adoption of optimised traffic handling policies by the gNB-CU-UP when transmitting traffic to nodes hosting lower layers.

# Conclusion

This paper has provided an overview of the reasons to signal SPID and ARPI to gNB-CU-UPs. The paper made the following observation and conclusion:

Observation 1: SPID and ARPI enable user plane policy differentiation between UEs that could potentially support bearers with the same QoS Parameters

Conclusion: Signalling of SPID and ARPI to the gNB-CU-UP enables adoption of optimised traffic handling policies by the gNB-CU-UP when transmitting traffic to nodes hosting lower layers.

It is proposed to agree to the signalling of SPID and ARPI over the E1 interface, as per CR in R3-194120