3GPP TSG-RAN3 Meeting #105	R3-194273

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0428 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1 support for positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2019-07-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to support NR positioning, new procedures should be defined over F1 interferface. Current version does not support postioning function in F1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new procedures for supporting Positioning. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because this only changes the Semantics description of the old gNB-DU UE F1AP ID IE in the tabular of DL RRC MESSAGE TRANSFER message. This CR has an impact under functional point of view. The impact can be considered isolated because the change affects DL RRC MESSAGE TRANSFER message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Positioning funciton cannot be supported in F1AP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| GNB-DU RESOURCE COORDINATION | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| Downlink Positioning Information Exchange | DOWNLINK POSITIONING INFORMATION REQUEST | DOWNLINK POSITIONING INFORMATION RESPONSE | DOWNLINK POSITIONING INFORMATION FAILURE |
| Uplink Positioning Information Exchange | UPLINK POSITIONING INFORMATION REQUEST | UPLINK POSITIONING INFORMATION RESPONSE | UPLINK POSITIONING INFORMATION FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| UL Positioning inforamtion Update | UPLINK POSITIONING INFORMATION UPDATE |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Measurement Terminate | POSITIONING MEASUREMENT TERMINATE |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.x	Positioning message transfer procedures

### 8.x.1	Downlink Positioning Information Exchange

#### 8.x.1.1	General

The purpose of the Downlink Positioning Information Exchange procedure is to allow the gNB-CU to request the gNB-DU node to transfer Downlink Positioning Information to the gNB-CU.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Downlink Positioning Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending an DOWNLINK POSITIONING INFORMATION REQUEST message. The cell ID is included in the message to indicate in which cell the positioning information is requested. The gNB-DU node responds with DOWNLINK POSITIONING INFORMATION RESPONSE message that contains the available downlink positioning information applicable to the relevant cells/TPs.

#### 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1: Downlink Positioning Information Exchange procedure, unsuccessful operation

If the gNB-DU node does not have any downlink positioning information to report, the gNB-DU node shall respond with an DOWNLINK POSITIONING INFORMATION FAILURE message.

### 8.x.2	Uplink Positioning Information Exchange

#### 8.x.2.1	General

The Uplink Positioning Information Exchange procedure is initiated by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals and to retrieve the SRS configuration from the eNB.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Uplink Positioning Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a UPLINK POSITIONING INFORMATION REQUEST message to the gNB-DU. This message may contain the SRS configuration and number of SRS transmissions desired. If the gNB-CU requests a number of SRS transmissions, the gNB-DU may take this information into account when configuring SRS transmissions for the UE. The gNB-DU shall reply with the UPLINK POSITIONING INFORMATION RESPONSE message.

#### 8.x.2.3	Unsuccessful Operation

Figure 8.x.2.3-1: Uplink Positioning Information Exchange procedure, unsuccessful operation

If the gNB-DU is unable to configure any SRS transmissions for the UE, the gNB-DU shall respond with a UPLINK POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the eNB shall send a UPLINK POSITIONING INFORMATION FAILURE message with an appropriate cause value.

### 8.x.3	Uplink Positioning Information Update

#### 8.x.3.1	General

The Uplink Positioning Information Update procedure is sent by the gNB-DU to indicate to the gNB-CU that a change has occurred in the SRS configuration due to a change in SRS configuration parameters in one or more cells.

#### 8.x.3.2	Successful Operation

Figure 8.x.3.2-1: Uplink Positioning Information Update procedure, successful operation

The gNB-DU initiates the procedure by sending a UPLINK POSITIONING INFORMATION UPDATE message to the gNB-CU. This message contains the SRS configuration information for all cells with SRS configured.

#### 8.x.3.3	Unsuccessful Operation

Not Applicable.

### 8.x.4	Positioning Measurement

#### 8.x.4.1	General

The purpose of the Positioning Measurement procedure is to allow the gNB-CU to request the gNB-DU to perform and report positioning measurements for the purpose of UL-TDOA, UL-AOA, E-CID and multi-RTT positioning.

#### 8.x.4.2	Successful Operation

Figure 8.x.3.2-1: Positioning Measurement procedure. Successful operation.

The gNB-CU initiates the procedure by sending a POSITIONING MEASUREMENT REQUEST message to the gNB-DU. If the requested positioning measurement has been successful, the gNB-DU shall reply with a POSITIONING MEASUREMENT RESPONSE message.

#### 8.x.4.3	Unsuccessful Operation

Figure 8.x.3.3-1: Positioning Measurement procedure. Unsuccessful operation.

If the gNB-DU cannot successfully measure the required measurements for the target UE, it shall respond with a POSITIONING MEASUREMENT FAILURE message indicating the cause of the failure.

### 8.x.5	Positioning Measurement Update

#### 8.x.5.1	General

The purpose of the Positioning Measurement Update Procedure is to notify the gNB-DU of a change in the previously signalled measurement configuration for one or more cells for the UE being positioned.

#### 8.x.5.2	Successful Operation

# Figure 8.X.4.2-1: Positioning Measurement Update: Successful Operation.

The gNB-CU initiates the procedure by sending a POSITIONING MEASUREMENT UPDATE message.

#### 8.x.5.3	Unsuccessful Operation

Not applicable.

### 8.x.6	Positioning Measurement Terminate

#### 8.x.6.1	General

The purpose of the Measurement Terminate Procedure is to enable the gNB-CU to terminate an on-going measurement identified by the LMF Measurement ID for UL-TDOA, UL-AOA, and multi-RTT positioning; or to terminate periodical E-CID measurements performed by gNB-DU.

#### 8.x.6.2	Successful Operation

# Figure 8.x.5.2-1: Positioning Measurement Terminate Procedure: Successful Operation.

The gNB-CU initiates the procedure by sending a POSITIONING MEASUREMENT TERMINATE message.

#### 8.x.6.3	Unsuccessful Operation

Not applicable.

### 8.x.7	Positioning Measurement Report

#### 8.x.7.1	General

The purpose of Positioning Measurement Report procedure is for the gNB-DU node to provide the positioning measurements for the UE to the gNB-CU.

#### 8.x.7.2	Successful Operation

Figure 8.x.6.2-1: Positioning Measurement Report procedure, successful operation

The gNB-DU node initiates the procedure by sending an POSITIONING MEASUREMENT REPORT message.

#### 8.x.7.3	Unsuccessful Operation

Not applicable.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.x	Positioning message transfer messages

#### 9.2.x.1	DOWNLINK POSITIONING INFORMATION REQUEST

This message is sent by gNB-CU to request DL positioning information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL Positioning Information | M |  |  |  |  |  |
| >NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| >NR PCI | O |  | INTEGER(0..1007) |  |  |  |
| >DL Information | M |  | ENUMERATED (NR-arfcn, cpLength, noDlFrames, noAntennaPorts, sFNInitTime, nG-RANAccessPointPosition, tpid, tpType, crsCPlength, dlBandwidth, …, tddConfig) |  | YES | reject |

#### 9.2.x.2	DOWNLINK POSITIONING INFORMATION RESPONSE

This message is sent by gNB-DU to provide DL positioning information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL Cell Information | M |  | 9.3.1.a |  | YES | reject |

#### 9.2.x.3	DOWNLINK POSITIONING INFORMATION FAILURE

This message is sent by gNB-DU to indicate that the DL positioning information cannot be provided.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	UPLINK POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.b |  | YES | ignore |

#### 9.2.x.5	UPLINK POSITIONING INFORMATION RESPONSE

This message is sent by the gNB-DU to provide the configured SRS information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UL Configuration | M |  | 9.3.1.c |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.6	UPLINK POSITIONING INFORMATION FAILURE

This message is sent by the gNB-DU to indicate that no SRS transmissions could be configured for the UE for uplink positioning measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.7			UPLINK POSITIONING INFORMATION UPDATE

This message is sent by the gNB-DU to indicate that the SRS configuration for the UE, for one or more cells, has changed.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UL Configuration | O |  | 9.3.1.c |  | YES | ignore |

### 9.2.x.8			POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to make positioning measurements.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Measurement ID | M |  | INTEGER (FFS) |  | YES | reject |
| UL Measurement Configuration | M |  | 9.3.1.d |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measuremrent Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Relative Time of Arrival, Time Difference of Rx and Tx, RSRP, RSRQ,…) |  |  |  |
| >Report Characteristics | O |  | ENUMERATED(OnDemand, Perodic,…) |  |  |  |
| >Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is FFS. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

### 9.2.x.9			POSITIONING MEASUREMENT RESPONSE

This message is sent by the gNB-DU to report UL positioning measurements for the target UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Measurement ID | M |  | INTEGER (FFS) |  | YES | reject |
| Positioning measurements | M |  | 9.3.1.e |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### 9.2.x.10			POSITIONING MEASUREMENT FAILURE

This message is sent by the gNB-DU to report measurement failure.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Measurement ID | M |  | INTEGER (FFS) |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### 9.2.x.11			POSITIONING MEASUREMENT UPDATE

This message is sent by the gNB-CU to indicate to the gNB-DU that the previously signalled measurement configuration for the target UE has changed for one or more cells, and to convey the SRS configuration for all cells with SRS configured for the target UE.

Direction: gNB-CU   gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Measurement ID | M |  | INTEGER (FFS) |  | YES | reject |
| UL Measurement Configuration | M |  | 9.3.1.d |  | YES | reject |

### 9.2.x.12			POSITIONING MEASUREMENT TERMINATE

This message is sent by the gNB-CU to request the gNB-DU to terminate the measurement identified by the Measurement ID.

Direction: gNB-CU   gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | - |  |
| Measurement ID | M |  | INTEGER (FFS) |  | YES | reject |

#### 9.2.x.13	POSITIONING MEASUREMENT REPORT

This message is sent by gNB-DU to report the results of the requested positioning measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | - |  |
| Measurement ID | M |  | INTEGER (FFS) |  | YES | reject |
| Positioning Measurements | M |  | 9.3.1.e |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.1	Radio Network Layer Related IEs

### 9.3.1.a	DL Cell Information

This IE contains DL positioning related cell information of a cell/TP.

Editor’s Note: Details are FFS pending RAN1 progress.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  |
| NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID |
| NR-ARFCN | M |  | INTEGER (0..3279165) |  |
| NG-RAN Access Point Position | M |  | 9.3.1.f | The configured estimated geographical position of the antenna of the cell/TP. |
| gNB-DU Type | M |  | ENUMERATED (pos-only, …) | A gNB-DU with positioning function only. |

### 9.3.1.b	Requested SRS Transmission Characteristics

The purpose of the Requested SRS Transmissions Characteristics information element is to inform the gNB-DU of the number and bandwidth of SRS transmissions requested for the UE for the purpose of UL positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number Of Transmissions | M |  | INTEGER (FFS) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. |
| Bandwidth | M |  | INTEGER (FFS) | The requested bandwidth of the SRS transmissions, the value of which corresponds to the number of resource blocks requested to be allocated. |

### 9.3.1.c	UL Configuration

The purpose of the Uplink Configuration information element is to inform the gNB-CU of the uplink configuration parameters.

Editor’s Note: Details are FFS pending RAN1 progress.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Configuration | M | 1 ..<maxServCell> |  | Configuration of SRS for corresponding serving cells. |
| >NR PCI | M |  | INTEGER (0.. 1007) |  |
| >UL NR-ARFCN | M |  | INTEGER (0.. 3279165) | The uplink NR-UTRA carrier frequency |
| >Number of Transmissions | M |  | INTEGER (FFS) | The number of periodic SRS transmissions. The value of ‘0’ represents an infinite number of SRS transmissions. |
| >UL-bandwidth | M |  | ENUMERATED(FFS) | Cell transmission bandwidth configuration in uplink |
| >srs-BandwidthConfig | M |  | ENUMERATED (FFS) |  |

| Range bound | Explanation |
| --- | --- |
| maxServCell | Maximum number of serving cells with SRS configuration. Value is FFS. |

### 9.3.1.d	UL Measurement Configuration

The purpose of the UL Measurement Configuration IE is to convey the UL measurement parameters to the gNB-DU.

Editor’s Note: Details are FFS pending RAN1 progress.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Search Window Parameters | O |  | FFS |  |
| Number of Transmissions | M |  | INTEGER (FFS) | The number of periodic SRS transmissions. The value of ‘0’ represents an infinite number of SRS transmissions. |
| SRS Configuration | M | <1.. maxServCell> |  | Configuration of SRS for corresponding serving cells. |
| >NR PCI | M |  | INTEGER (0.. 1007) |  |
| >UL NR-ARFCN | M |  | INTEGER (0.. 3279165) |  |
| >UL-bandwidth | M |  | ENUMERATED (FFS) | Cell transmission bandwidth configuration in uplink |
| >srs-BandwidthConfig | M |  | ENUMERATED (FFS) | Cell-specific SRS bandwidth configuration |

| Range bound | Explanation |
| --- | --- |
| maxServCell | Maximum number of serving cells with SRS configuration. Value is FFS. |

### 9.3.1.e	Positioning Measurements

The purpose of the Positioning Measurements IE is to signal positioning measurement results to the gNB-CU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NG-RAN Access Point Position | O |  | 9.3.1.f | The configured estimated geographical position of the antenna of the cell. |
| Measurement Results |  | 0 .. <maxnoMeas> |  |  |
| >CHOICE Measurement Value | M |  |  |  |
| >>RTOAMeasurements | M | <0..maxNoRTOA> |  |  |
| >>>UL EARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>> Value RTOA | M |  | INTEGER (FFS) | Relative time of arrival measruement. |
| >>AOAMeasurements |  | <0..maxNoAOA> |  |  |
| >>>UL EARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>> Value RTOA | M |  | INTEGER (FFS) | Angle of arrival measruement. |
| >>TDRTMeasurements |  | <0..maxNoTDRT> |  |  |
| >>>UL EARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>>Value TDRT | M |  | INTEGER (FFS) | Time difference between Rx and Tx Measruement. |
| >>CellID |  |  |  |  |
| >>>Serving Cell ID | M |  | NG-RAN CGI 9.3.1.126 | NG-RAN Cell Identifier of the serving cell |
| >>>Serving Cell TAC | M |  | 5GS TAC 9.2.1.29 | Tracking Area Code of the serving cell |
| >>RSRPMeasurements |  | <0.. maxCellReport > |  |  |
| >>>NR CGI | O |  | 9.3.1.12 |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>UL EARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>> Value RSRP | M |  | INTEGER (FFS) | RSRP Measruement. |
| >>RSRQMeasurements |  | <0.. maxCellReport > |  |  |
| >>>NR CGI | O |  | 9.3.1.12 |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>UL EARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>> Value RSRQ | M |  | INTEGER (FFS) | RSRQ Measruement. |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is FFS. |
| maxNoRTOA | Maximum no. of UL RTOA measurements per UE is FFS , one measurement per carrier. |
| maxNoAOA | Maximum no. of UL AOA measurements per UE is FFS , one measurement per carrier. |
| maxNoTDRT | Maximum no. of UL TDRT measurements per UE is FFS , one measurement per carrier. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is FFS. |

### 9.3.1.f	NG-RAN Access Point Position

The NG-RAN Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| Uncertainty semi-major | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Uncertainty semi-minor | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Orientation of major axis | M |  | INTEGER (0..179) |  |
| Uncertainty Altitude | M |  | INTEGER (0..127) | The uncertainty altitude "h" expressed in metres is derived from the "uncertainty code" k, by: h=45x(1.025k-1). |
| Confidence | M |  | INTEGER (0..100) | In percentage |

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>