3GPP TSG-RAN WG3 Meeting #106	R3-196616

# Reno, USA, November 18-22 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0100 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of some DRBs to be released |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-11-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For scenarios involving a reconfiguration, the existing Stage 3 specifications unecesarily mandate the RRC Reconfiguration Complete Indicator IE to be signaled in an additional modification procedure whenever the gNB-DU has included CellGroupConfig IE. It also mandates not to apply the new configuration at gNB-DU until it has been explicitly indicated via a separate modification procedure. The following is defined in 38.473: “If the ongoing reconfiguration procedure involves changes of the L1/L2 configuration at the gNB-DU signalled to the gNB-CU via the CellGroupConfig IE, the gNB-CU shall include the RRC Reconfiguration Complete Indicator IE in the UE CONTEXT MODIFICATION REQUEST message to inform the gNB-DU that the ongoing reconfiguration procedure, including CellGroupConfig IE, has been successfully or unsuccesfully performed. In the case that the ongoing reconfiguration procedure has failed, the gNB-DU shall continue to use the old UE configuration.” This additional requirement conflicts existing Stage 2 specifications for bearer removal scenarios, as TS 38.401 (section 8.9.3.1) does not mandate the use of two bearer context modification procedures in order to release some of the DRBs for a UE. Thus, there is a misalignment between Stage 2 and Stage 3 specifications as follows. - Stage 2 flow: - CU indicates DU the DRB to remove (Bearer Context Modification procedure). DU can remove the DRB. - CU indicates UE the DRB to remove (RRC Reconfiguration) - That is, only one modification procedure is needed between CU and DU. - Stage 3 flow: - CU indicates DU the DRB to remove (Bearer Context Modification procedure). DU cannot remove the DRB but is forced to wait until an additional indication is received. - CU indicates the UE the DRB to remove (RRC Reconfiguration) - CU indicates DU the result of the reconfiguration (Bearer Context Modification procedure). DU changes to the new configuration at this point. - That is, two modification procedures are needed between CU and DU. This inconsistency is also applicable for the case when RACH procedure is required during reconfiguration and incurs conflicts in multi-vendor scenarios as behavior will not be aligned. Thus, to resolve this conflict, the scenario for removal of some DRBs and the case RACH is performed should be excluded from mandating use of an additional modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedural text changes to clearly define the overall procedure on DRB removal scenarios and the case where RACH is performed. Impact assessment towards the previous version of the specification (same release): This CR has an impact under functional point of view The impact can be considered isolated because the change is limited to DRB removal scenarios and the case where RACH is performed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Behaviour misaligned between Stage 2 and Stage 3, as issues in multi-vendor scenarios due to different behaviour implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.473 CR0448 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

Beginning of Text Proposal to TS 38.401

### 8.9.3	Bearer context release over F1-U

#### 8.9.3.1	gNB-CU-CP initiated bearer context release

Figure 8.9.3.1-1 shows the procedure used to release the bearer context in the gNB-CU-UP initiated by the gNB-CU-CP.

Figure 8.9.3.1-1: Bearer context release over F1-U – gNB-CU-CP initiated

0.	Bearer context release (e.g., following an SGNB RELEASE REQUEST message from the MeNB) is triggered in gNB-CU-CP.

1.	The gNB-CU-CP sends a BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP.

2.	The gNB-CU-UP responds with a BEARER CONTEXT MODIFICATION RESPONSE carrying the PDCP UL/DL status.

3.	F1 UE context modification procedure is performed to stop the data transmission for the UE. It is up to gNB-DU implementation when to stop the UE scheduling.

NOTE: step 1-3 are performed only if the PDCP status of the bearer(s) needs to be preserved e.g., for bearer type change.

4.	The gNB-CU-CP may receive the UE CONTEXT RELEASE message from the MeNB in EN-DC operation as described in Section 8.4.2.1.

5.	and 7. Bearer context release procedure is performed.

6.	F1 UE context release procedure is performed to release the UE context in the gNB-DU.

#### 8.9.3.2	gNB-CU-UP initiated bearer context release

Figure 8.9.3.2-1 shows the procedure used to release the bearer context in the gNB-CU-UP initiated by the gNB-CU-UP.

Figure 8.9.3.2-1: Bearer context release over F1-U – gNB-CU-UP initiated

0.	Bearer context release is triggered in gNB-CU-UP e.g., due to local failure.

1.	The gNB-CU-UP sends a BEARER CONTEXT RELEASE REQUEST message to request the release of the bearer context in the gNB-CU-UP. This message may contain the PDCP status.

2.- 5. If the PDCP status needs to be preserved, the E1 Bearer Context Modification and the F1 UE Context Modification procedures are performed. The E1 Bearer Context Modification procedure is used to convey data forwarding information to the gNB-CU-UP. The gNB-CU-CP may receive the UE Context Release from the MeNB.

6. The gNB-CU-CP sends a BEARER CONTEXT RELEASE COMMAND message to release the bearer context in the gNB-CU-UP.

7.	The gNB-CU-UP responds with a BEARER CONTEXT RELEASE COMPLETE to confirm the release of the bearer context including also data forwarding information.

8.	F1 UE context release procedure may be performed to release the UE context in the gNB-DU.

#### 8.9.3.x	Release of some DRBs

Figure 8.9.3.x-1 shows the procedure used to release some of the DRBs in the gNB-CU-UP and gNB-DU.

Figure 8.9.3.x-1: Release of some DRBs

1.	Release of some of the DRBs is triggered.

2.	The gNB-CU-CP sends UE CONTEXT MODIFICATION REQUEST message indicating the DRBs to be released and prepares the CellGroupConfig IE for the RLC release for these DRBs.

3.	The gNB-DU replies with UE CONTEXT MODIFICATION RESPONSE message including CellGroupConfig IE for the DRBs to be released. The gNB-DU removes the DRBs.

4.	The gNB-CU generates the RRCReconfiguration message indicating the UE to release the DRBs and encapsulates it in the DL RRC MESSAGE TRANSFER message.

5.	The gNB-DU sends RRCReconfiguration message to the UE.

6.   The UE sends RRCReconfigurationComplete message to the gNB-DU.

7.	The gNB-DU encapsulates the RRCReconfigurationComplete in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

8.	The gNB-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message indicating the DRBs to be released to the gNB-CU-UP. The gNB-CU-UP removes the DRBs.

NOTE: step 2 and 7 may be performed in parallel.

9.	The gNB-CU-UP replies with BEARER CONTEXT MODIFICATION RESPONSE message.

End of Text Proposal to TS 38.401