3GPP TSG-RAN WG3 Meeting #106	R3-196675

Reno, USA, 18 – 22 November 2019

Title: 	Consideration on NB-IoT SON

Source: 	Huawei

Agenda item:	14.4.1

Document for:	discussion

1. Introduction

In Rel-16 Additional enhancements for NB-IoT WI, there is one objective about network management tool enhancement:

Network management tool enhancement:

- SON support for reporting of [RAN2, RAN3]

  - Cell Global Identity and strongest measured cell(s) (ANR)

  - Random access performance

  - Radio link failure (RLF), if needed

In this contribution, we discuss the how to support RLF report exchange, NPRACH configuration exchange and ANR Report exchange for NB-IoT.

2. Discussion

## 2.1 RLF report exchange

Some discussions took place in the previous RAN2 meetings and the following agreements were made:

RAN2#105 agreements: - RLF report: The UE informs network about having an RLF report available via rlf-InfoAvailable indication during the initial RRC connection re-establishment process after RLF. The rlf-InfoAvailable parameter is included in Msg5 of the RRC connection re-establishment process. It is up to the network to decide when to request for the RLF report, using UEInformationRequest/UEInformationResponse procedure. In case of no/failed RRC connection re-establishment, the UE may store the RLF report for indicating rlf-InfoAvailable in Msg5 of later connection establishment. The following legacy parameters are supported for RLF reporting in NB-IoT: - Failed cell ECGI - Last Serving Cell RSRP/RSRQ (Whether OK for CP-solution is FFS) - Absolute Time Stamp (optional, if available) - Location Info (optional, if available) - FFS: further information. RLF reporting does not trigger RRC connection establishment/resume, including EDT. Neither rlf-InfoAvailable indication nor RLF report is included in the Msg3 of the EDT procedure to indicate to the network about an available RLF report.

Based on these RAN2 agreements especially the different content of the RLF reporting, based on the RAN2 running CR (R2-1911592 endorsed after RAN2#107), the RLF-Report-NB-r16 will be introduced.

In LTE, RLF Report is transferred over X2 and S1 interfaces, to assist the network to detect and fix the potential issue of the network deployment and then promote the UE experience.

In TS 36.423 and TS 36.413, the UE RLF Report Container IE is included in the X2AP: RLF INDICATION message, X2AP: HANDOVER REPORT message, S1AP: MME CONFIGURATION TRANSFER message, and the eNB CONFIGURATION TRANSFER message.

Considering of the new RAN2 container, RAN3 needs to update the definition of the UE RLF Report Container IE like below:

X2AP:

| UE RLF Report Container | O |  | OCTET STRING | RLF -Report-r9 IE contained in the UEInformationResponse message, or RLF-Report-NB-r16 IE contained in the UEInformationResponse-NB message (TS 36.331 [9]) | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

S1AP:

| UE RLF Report Container | M |  | OCTET STRING | rlf-Report-r9 contained in UEInformationResponse message, or rlf-Report-NB-r16 contained in UEInformationResponse-NB message as defined in TS 36.331 [16]. |
| --- | --- | --- | --- | --- |

Proposal 1: update the the definition of the UE RLF Report Container IE in S1AP and X2AP specifications.

Note: the corresponding Xn and NG part will be considered after NR progress.

## 2.2 NPRACH configuration exchange

RAN2 made some progresses on the RACH report as it is mainly related to the information to be reported to the eNB from the UE, there is no RAN3 impacts foreseen.

RAN2#103bis agreements: Support RACH report for NB-IoT Will consider whether information in LTE RACH report is extended RAN2#104 agreements: - RACH report: In addition to legacy parameters for RACH reporting, the first selected resource pool (E.g. CE level, EDT) is included in the RACH report. This agreement is also applicable for eMTC. RAN2#105 agreements: - RACH report: For NB-IoT RACH report, range of numberOfPreamblesSent is INTEGER (1..64). contentionDetected is Boolean. Initial CEL is included in NB-IoT RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. Take the ASN.1 structure provided in the report (proposal 5) as baseline for NB-IoT running CR. Initial CEL is included in eMTC RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. Take the ASN.1 structure provided in the report (proposal 8) as baseline for eMTC running CR. RAN2 aims on standardizing a single method of sending NB-IoT RACH report, i.e., avoid multiple solutions. Proposed solutions should be discussed considering signalling overhead, UE power consumption, security aspects and other reports being considered in other email discussions. NB-IoT RACH report includes the information from the latest successful RACH procedure. NB-IoT RACH report information is discarded when the UE goes to IDLE. FFS how to report. RAN2#105bis agreements: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. At least RACH report for NB-IoT based on UE information request/response procedure is supported. It is up to the network to decide when to request for the RACH report. RAN2#106 agreements: - RACH report: Not support EDT RACH report in EDT Msg3.

Based on TS 36.300, RACH optimization is supported by UE reported information and by PRACH parameters exchange between eNBs. The PRACH parameters exchange between eNBs [2] can minimize the operator planning, but the NPRACH parameters were not exchanged between eNB yet.

In order to support RACH optimization for NB-IoT, to voide potential random access interference among NB-IoT Cells, it is also needed to exchange the NPRACH configurations of the anchor carrier and non-anchor carriers over X2 interface.

Note: Based on TS36.331, there are quite a lot of different NPRACH configurations with similar sub IEs but different value code points, it is better to define the corresponding NPRACH Configuration IE in X2/Xn AP including the full set of these code points.

Proposal 2: exchange the NPRACH configurations of the anchor carrier and non-anchor carriers over X2 and Xn interface, to support RACH optimization.

## 2.3 ANR Report exchange

Some discussions took place in the previous RAN2 meetings and the following agreements were made:

RAN2#103bis agreements: ANR reporting for NB-IoT only uses idle-mode measurements (i.e. we won’t introduce connected mode measurements) RAN2#104 agreements: RAN2 understanding is that the purpose of SON/ANR reporting in NB-IoT is network optimisation rather than immediately updating neighbour relations like with LTE ANR, and is therefore not time critical. SON reporting does not trigger RRC connection establishment/resume - FFS whether this includes EDT. SON information can be reported along with EDT, FFS what and how. RAN2#105 agreements: Solution direction based on option a: - Single set of measurements only. - No new measurement requirements. - ANR measurement reporting using the UE Information Request / Response framework is supported. Other methods FFS. - ANR reporting for the CP solution is not supported in Rel-16. RAN2#106 agreements: ANR report does not trigger EDT. ANR report is not carried in EDT Msg3. anr-InfoAvailable indication can be reported in EDT Msg3. ANR procedure steps: - 1) UE receives “measurement” configuration in dedicated signalling. - 2) When the UE is released to idle: - The UE performs the instructed measurements according to existing RAN4 cell reselection measurement performance requirements. - CGI reading is limited to cells being above a certain threshold. Threshold details FFS. 3) Reporting takes place, upon network request, next time UE is connected mode again. RAN2#107 agreements: Configuration: Dedicated ANR measurement configuration is provided in RRCConnectionRelease message only. The list of frequencies where to perform the measurements can be optionally provided in dedicated signalling, as a subset of the frequencies signalled in system information. Per default, the list frequencies signalled in system information is used. Maximum of 8 carriers can be configured. A blacklist of cells can be optionally signalled per carrier. Measurements: ANR measurements are only performed while UE remains camped on the same cell from which the ANR measurement configuration was received. (i.e. When the UE changes cell the ANR measurements are stopped). Neighbour cell relaxed monitoring criteria and Ssearch criteria don’t apply while performing an ANR measurement. ANR measurement doesn’t affect DRX / eDRX operation. CGI-reading applies to strongest cell on each frequency, if above the RSRP threshold. The RSRP threshold is an absolute threshold that is common to all frequencies. Reporting: anr-InfoAvailable indication is reported in and only in RRCConnectionSetupComplete, RRCConnectionResumeComplete, RRCConnectionReestablishmentComplete and RRCConnectionResumeRequest for EDT. The ANR report might be empty for a given carrier, if a measurement is performed and no strong cell is found. UE only indicates the availability of the report if 1) a report is available and 2) if the RPLMN is included in the list of EPLMNs stored by the UE when the ANR measurement was configured. The ANR report is discarded after: - Timer expiry. Value FFS. - Power off or detach. - Reporting anr-InfoAvailable and returning to idle. UE Information Request / Response is the only reporting procedure for ANR measurement. For a measured carrier the following information is included: - Carrier frequency. - Strongest cell PCI, RSRP/RSRQ. - Strongest cell CGI-info if RSRP above the threshold. Serving cell RSRQ/RSRP and CGI are included in the ANR report. ANR reporting is optional for NB-IoT UEs with capability signaling. FFS: ANR applicability to 5GC.

Based on these RAN2 agreements, it could be understand that the corresponding procedure could be performed as below:

Cell A

Phy

-

CID=3

Global

-

CID =17

Cell B

Phy

-

CID=5

Global

-

CID =19

1) ANR measurement configuration

& RRC connection release

2

)

Read BC

C

H

3) RRCConnectionSetupComplete, RRCConnectionResumeComplete, RRCConnectionReestablishmentComplete or RRCConnectionResumeRequest for EDT

with ANR report availability flag

4) Request ANR report

5) Report strongest cell (Phy-CID =5, signal level), Global-CID = 19

As shown in Figure above, the eNB1 (Cell A) configures the UP solution UE in connected mode via RRCConnectionRelease message, and after RRC connection release, the UE performs the neighbour cell (Cell B) measurement in idle mode, and the UE performs ANR reporting when next time UE is connected mode again.

Considering the power consumption of the UE to perform ANR measurement, and the potential benefit for the network to get such report, when the UE moves to connected mode in a new eNB2 rather than the old eNB1, it is helpful for the new eNB2 to forward the UE’s ANR report to the old eNB1.

Observation: it is beneficial to forward the UE’s ANR report from new eNB to old eNB, when the UE access to a new eNB with anr-InfoAvailable indication.

As agreed by RAN2, the anr-InfoAvailable indication is reported in and only in:

- RRCConnectionSetupComplete

- RRCConnectionResumeComplete

- RRCConnectionReestablishmentComplete

- RRCConnectionResumeRequest for EDT

For 2), 3), and 4), as there will be an UE Context Retrieve procedure between the new and the old eNBs, it is possible for the new eNB to provide the ANR report to the old eNB via an UE associated procedure, e.g. include the ANR report in the X2AP: UE CONTEXT RELEASE message.

But this solution does not fly in case of 1) RRCConnectionSetupComplete, there is no UE association exists between the old and the new eNB. Considering of the RAN2 agreement “Serving cell RSRQ/RSRP and CGI are included in the ANR report”, note that the serving cell is the cell where the ANR measurement configuration was received, and it is understood that the serving cell RSRQ/RSRP and CGI are included only if the current serving cell is different than the old serving cell. With the last serving cell ECGI, when the UE moves to connected mode in a new eNB2, for all the cases 1) 2) 3) and 4), it is feasible for the new eNB2 to forward the UE’s ANR report to the old eNB based on the last serving cell ECGI. Therefore, it is preferred to also introduce a new non UE associated X2AP procedure to transmit the ANR report from the new eNB to the old eNB, to be used in case of 1), and it is up to the eNB implementation whether to use it in case of 2), 3) and 4).

Proposal 3: include the ANR report in the X2AP: UE CONTEXT RELEASE message.

Proposal 4: introduce a new non UE associated X2AP procedure to transmit the ANR report from the new eNB to the old eNB.

3. Conclusion

- In this contribution, we analyse how to support RLF report exchange, NPRACH configuration exchange and ANR Report exchange, get the following proposals:

- Proposal 1: update the the definition of the UE RLF Report Container IE in S1AP and X2AP specifications.

- The corresponding S1AP CR and X2AP CR are provided in [1] and [2].

Proposal 2: exchange the NPRACH configurations of the anchor carrier and non-anchor carriers over X2 and Xn interface, to support RACH optimization.

- The corresponding X2AP CR and XnAP CR are provided in [3] and [4].

Proposal 3: include the ANR report in the X2AP: UE CONTEXT RELEASE message.

Proposal 4: introduce a new non UE associated X2AP procedure to transmit the ANR report from the new eNB to the old eNB.

- The corresponding X2AP CR is provided in [5].

4. Reference

- R3-196676	Support of RLF in NB-IoT	Huawei	Rel-16	36.413	15.7.1	NB_IOTenh3-Core	CR#1729

- R3-196677	Support of RLF in NB-IoT	Huawei	Rel-16	36.423	15.7.0	NB_IOTenh3-Core	CR#1405

- R3-196678	NPRACH configuration exchange	Huawei	Rel-16	36.423	15.7.0	NB_IOTenh3-Core	CR#1406

- R3-196679	NPRACH configuration exchange	Huawei	Rel-16	38.423	15.5.0	NB_IOTenh3-Core	CR#0271

- R3-196680	Introduction of NB-IoT ANR report	Huawei	Rel-16	36.423	15.7.0	NB_IOTenh3-Core	CR#1407