3GPP TSG-RAN WG3 Meeting #106	R3-197692

# Reno, USA, 18 – 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0495 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Positioning support over F1AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2019-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of support for positioning |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the Assistance Information Control and Assistance Information Feedback procedures (both Class 2) Introduction of positioning measurement procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for positioning over F1AP |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR . TBD.. |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[3]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[4]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[5]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[6]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[7]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[9]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[10]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[11]	3GPP TS 23.203: "Policy and charging control architecture".

[12]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[13]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[14]	3GPP TR 25.921: (version.7.0.0): "Guidelines and principles for protocol description and error".

[15]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[16]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[17]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[18]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U) ".

[19]	3GPP TS 38.414: "NG-RAN; NG data transport".

[20]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[21]	3GPP TS 23.501: "System Architecture for the 5G System".

[22]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.304: "NR;  User Equipment (UE) procedures in Idle mode and RRC Inactive state ".

[25]	3GPP TS 36.104: "Base Station (BS) radio transmission and reception".

[26]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[27]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[28]	3GPP TS 38.423: "NG-RAN; Xn application protocol (XnAP)".

[x]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[y]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

>>>> NEXT CHANGE <<<<

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

CN	Core Network

CG	Cell Group

CGI	Cell Global Identifier

CP	Control Plane

DL	Downlink

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IMEISV 	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

NSSAI	Network Slice Selection Assistance Information

RANAC	RAN Area Code

RRC	Radio Resource Control

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

>>>> NEXT CHANGE <<<<

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

[Editor’s Notes: procedures and associated functions require further checking]

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| GNB-DU RESOURCE COORDINATION | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Termination | POSITIONING MEASUREMENT TERMINATION COMMAND |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Modification | POSITIONING MEASUREMENT MODIFICATION |

>>>> NEXT CHANGE <<<<

#### 8.3.4.2	Successful Operation

Editor’s Note: whether the SRS Transmission request is sent via this procedure, or via a positioning specific procedure, is FFS.

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

>>> skip unchanged text <<<

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the Requested SRS Transmission Characteristics IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may take it into account when configuring SRS transmission for the UE.

>>>> NEXT CHANGE <<<<

## 8.x	Positioning Procedures

### 8.x.1	Positioning Assistance Information Control

#### 8.x.1.1	General

The purpose of the Positioning Assistance Information Control procedure is to allow the gNB-CU to signal positioning assistance information to the gNB-DU for positioning assistance information broadcasting.

#### 8.x.1.2	Successful Operation

# Figure 8.x.1.2-1: Positioning Assistance Information Control procedure

The gNB-CU initiates the procedure by sending an POSITIONING ASSISTANCE INFORMATION CONTROL message.

If the Assistance Information IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message, the gNB-DU shall replace any previously stored assistance information and use the received information to configure assistance information broadcasting.

[Editor’s note: the contents of the Assistance Information IE are FFS – depending on NRPPa definition first and then whether F1AP refers to NRPPa, or defines corresponding IEs]

If the Broadcast Priority IE is included in the Assistance Information IE, the gNB-DU may take it into account when configuring broadcasting for the relevant information. Assistance information having the same Broadcast Priority value should receive the same treatment (i.e. broadcast by the gNB-DU or not broadcast).

If the Broadcast IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message and set to "start", the gNB-DU may start broadcasting the assistance information. If the Broadcast IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message and set to "stop", the gNB-DU may stop broadcasting the assistance information.

#### 8.x.1.3	Abnormal Conditions

If the Broadcast IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message and set to "start", and no assistance information is available, the gNB-DU shall consider the procedure as failed.

If neither the Assistance Information IE nor the Broadcast IE are included in the POSITIONING ASSISTANCE INFORMATION CONTROL message, the gNB-DU shall consider the procedure as failed.

### 8.x.2	Positioning Assistance Information Feedback

#### 8.x.2.1	General

The purpose of the Positioning Assistance Information Feedback procedure is to allow the gNB-DU to give feedback to the gNB-CU on positioning assistance information broadcasting.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Positioning Assistance Information Feedback procedure

If the Assistance Information Failure List IE is included in the POSITIONING ASSISTANCE INFORMATION FEEDBACK message, the gNB-CU shall consider that positioning assistance information broadcasting could not be configured for the relevant information.

[Editor’s note: the contents of the Assistance Information Failure List IE are FFS – depending on NRPPa definition first and then whether F1AP refers to NRPPa, or defines corresponding IEs]

#### 8.x.2.3	Abnormal Conditions

Void.

### 8.x.3	Positioning Information Exchange [FFS]

#### 8.x.3.1	General

The purpose of the Positioning Information Exchange procedure is to exchange positioning information. The procedure uses UE-associated signalling when used to request positioning measurements for a UE.

Editor’s Note: It is FFS whether a single procedure is used for UE associated measurements and non-UE associated information exchange, or whether these are separated into two procedures. As currently the procedure describes UE measurements, and is UE-associated,  if it stays as such, then it should be renamed to Positioning Measurement procedure.

#### 8.x.3.2	Successful Operation

Figure 8.x.3.2-1: Positioning Information Exchange procedure: successful operation

The gNB-CU initiates the procedure by sending a POSITIONING INFORMATION REQUEST message. If the gNB-DU is able to provide the requested information, it shall reply with the POSITIONING INFORMATION RESPONSE message.

If the Measurement Quantities IE is included in the POSITIONING INFORMATION REQUEST message:

-	For each Report Characteristics IE set to "OnDemand", the gNB-DU shall return the corresponding measurement results in the Measurement Result List IE in the POSITIONING INFORMATION RESPONSE message, and the gNB-CU shall consider that this measurement has been terminated by the gNB-DU.

-	For each Report Characteristics IE set to "Periodic", the gNB-DU shall initiate the corresponding measurement, and it shall reply with the POSITIONING INFORMATION RESPONSE message without including this measurement in the message. The gNB-DU shall then periodically initiate the Positioning Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

-	If the gNB-DU is not able to initiate one or more requested measurements, it may include the Measurement Failure List IE in the POSITIONING INFORMATION RESPONSE message with the corresponding LMF UE Measurement ID(s).

Editor’s note: further details are FFS pending RAN1 and RAN2 progress

#### 8.x.3.3	Unsuccessful Operation

Figure 8.x.3.3-1: Positioning Information Exchange procedure: unsuccessful operation

If the gNB-DU is unable to provide the requested positioning information, it shall respond with a POSITIONING INFORMATION FAILURE message.

#### 8.x.3.4	Abnormal Conditions

If the gNB-DU receives a POSITIONING INFORMATION REQUEST message containing an LMF UE Measurement ID corresponding to an ongoing positioning measurement, it shall consider the procedure as failed and initiate local error handling.

### 8.x.4	Positioning Measurement Report [FFS]

#### 8.x.4.1	General

The purpose of the Positioning Measurement Report procedure is for the gNB-DU to report positioning measurements for the UE to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.x.4.2	Successful Operation

Figure 8.x.4.2-1: Positioning Measurement Report procedure: successful operation

The gNB-DU initiates the procedure by sending a POSITIONING MEASUREMENT REPORT message. The POSITIONING MEASUREMENT REPORT message contains the positioning measurement results according to the measurement configuration in the respective POSITIONING INFORMATION REQUEST message.

Editor’s note: further details are FFS pending RAN1 and RAN2 progress

#### 8.x.4.3	Unsuccessful Operation

Not applicable.

#### 8.x.4.4	Abnormal Conditions

Not applicable.

### 8.x.5	Positioning Measurement Termination [FFS]

#### 8.x.5.1	General

The purpose of the Positioning Measurement Termination procedure is to terminate one or more periodic positioning measurements for the UE performed by the gNB-DU. The procedure uses UE-associated signalling.

#### 8.x.5.2	Successful Operation

Figure 8.x.5.2-1: Positioning Measurement Termination procedure: successful operation

The gNB-CU initiates the procedure by generating a POSITIONING MEASUREMENT TERMINATION COMMAND message.

#### 8.x.5.3	Unsuccessful Operation

Not applicable.

#### 8.x.5.4	Abnormal Conditions

Not applicable.

### 8.x.6	Positioning Measurement Failure Indication [FFS]

#### 8.x.6.1	General

The purpose of the Positioning Measurement Failure Indication procedure is for the gNB-DU to notify the gNB-CU that the positioning measurements previously requested with the Positioning Information Exchange procedure can no longer be reported. This procedure uses UE-associated signalling.

#### 8.x.6.2	Successful Operation

Figure 8.x.4.2-1: Positioning Measurement Failure Indication procedure: successful operation

Upon reception of the POSITIONING MEASUREMENT FAILURE INDICATION message, the gNB-CU shall consider that the positioning measurements for the UE have been terminated by the gNB-DU.

#### 8.x.6.3	Unsuccessful Operation

Not applicable.

#### 8.x.6.4	Abnormal Conditions

Not applicable.

### 8.x.7	Positioning Measurement Modification [FFS]

#### 8.x.7.1	General

The purpose of the Positioning Measurement Modification procedure is to modify one or more periodic positioning measurements for the UE performed by the gNB-DU. The procedure uses UE-associated signalling.

[Editor’s note: It is FFS if this procedure is required vs e.g. stop/start]

#### 8.x.7.2	Successful Operation

Figure 8.x.7.2-1: Positioning Measurement Modification procedure: successful operation

The gNB-CU initiates the procedure by generating a POSITIONING MEASUREMENT MODIFICATION message. Upon receiving the message, the gNB-DU shall overwrite the previously received measurement configuration for the corresponding measurements.

#### 8.x.7.3	Unsuccessful Operation

Not applicable.

#### 8.x.7.4	Abnormal Conditions

If the gNB-DU cannot identify the given positioning measurements, it shall regard the procedure as failed and initiate local error handling.

>>>> NEXT CHANGE <<<<

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Editor’s Note: whether the SRS Transmission request is sent via this procedure, or via a positioning specific procedure, is FFS.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the RRCConnectionReconfiguration message as defined in TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) |  | - |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | - |  |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >> UL UP TNL Information to be setup List |  | 0..1 |  |  | - |  |
| >>> UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >> RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication | YES | reject |
| >> DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.d | For positioning purposes | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |

>>>> NEXT CHANGE <<<<

### 9.2.x	Messages for Positioning Procedures

#### 9.2.x.1	POSITIONING ASSISTANCE INFORMATION CONTROL

This message is sent by the gNB-CU to transfer assistance information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Positioning Assistance Information | O |  | It is FFS if this IE is referenced to 38.455, or coded explicilty |  | YES | reject |
| Broadcast | O |  | ENUMERATED (start, stop, ...) |  | YES | reject |
| Positioning Broadcast Target Cell [FFS if this should be a cell list] | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| Routing ID (FFS) | M |  | OCTET STRING |  | YES | reject |

#### 9.2.x.2	POSITIONING ASSISTANCE INFORMATION FEEDBACK

This message is sent by the gNB-DU to give feedback on assistance information broadcasting.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Positioning Assistance Information Failure List | O |  | It is FFS if this IE is referenced to 38.455, or coded explicilty |  | YES | reject |
| Positioning Broadcast Target Cell [FFS if this should be a cell list] | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| Routing ID (FFS) | M |  | OCTET STRING |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.3	POSITIONING INFORMATION REQUEST [FFS]

This message is sent by the gNB-CU to request positioning information.

Direction: gNB-CU  gNB-DU.

Editor’s note: message contents are FFS pending RAN1 and RAN2 progress; TRP-specific IEs can be added to the message. It is FFS if the Measurement Quantities IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Quantities |  | 0..1 |  |  | YES | reject |
| >Measurement Quantities Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | - |
| >>Measurement Type | M |  | ENUMERATED (Cell-ID, UL RTOA, UL AoA, gNB RX-TX,…) [FFS] |  |  | - |
| >>Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | - |
| >>Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) [FFS] |  | YES | - |
| >>UL SRS Measurement Configuration | C-ifULRTOA |  | [FFS] |  | YES | - |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Condition | Explanation |
| --- | --- |
| ifULRTOA | This IE shall be present if the Measurement Type IE is set to the value "UL RTOA". |

#### 9.2.x.4	POSITIONING INFORMATION RESPONSE [FFS]

This message is sent by the gNB-DU to signal the requested positioning information.

Direction: gNB-DU  gNB-CU.

Editor’s note: message contents are FFS pending RAN1 and RAN2 progress; TRP-specific IEs can be added to the message. . It is FFS if the Measurement Result List IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Result List |  | 0..1 |  |  | YES | reject |
| >Measurement Result List Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | - | - |
| >>Measurement Result | M |  | 9.3.1.a |  | - | - |
| Measurement Failure List |  | 0..1 |  |  | YES | ignore |
| >Measurement Failure List Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | - |
| >>Cause | M |  | 9.3.1.2 |  | YES | - |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

#### 9.2.x.5	POSITIONING INFORMATION FAILURE [FFS]

This message is sent by the gNB-DU to indicate that the requested positioning information cannot be signaled.

Direction: gNB-DU  gNB-CU.

Editor’s note: Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measurements that can be reported with one message. Value is 64. |

#### 9.2.x.6	POSITIONING MEASUREMENT REPORT [FFS]

This message is sent by the gNB-DU to report the results of the requested positioning measurements.

Direction: gNB-DU  gNB-CU.

Editor’s note: message contents are FFS pending RAN1 and RAN2 progress. TRP-specific IEs can be added to the message. . It is FFS if the Measurement Result List IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Result List |  | 1 |  |  | YES | reject |
| >Measurement Result List Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | - | - |
| >>Measurement Result | M |  | 9.3.1.a |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measurements that can be reported with one message. Value is 64. |

#### 9.2.x.7	POSITIONING MEASUREMENT TERMINATION COMMAND [FFS]

This message is sent by the gNB-CU to terminate the requested positioning measurement.

Editor’s Note: It is FFS if the Measurement List IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement List |  | 1..<maxnoMeas> |  |  | EACH | reject |
| >LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | - |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measurements that can be reported with one message. Value is 64. |

#### 9.2.x.8	POSITIONING MEASUREMENT FAILURE INDICATION [FFS]

This message is sent by the gNB-DU to indicate that the previously requested positioning measurements can no longer be reported.

Editor’s note: TRP-specific IEs can be added to the message. . It is FFS if the Measurement Failure List IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Failure List |  | 1 |  |  | YES | ignore |
| >Measurement Failure List Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | - |
| >>Cause | M |  | 9.3.1.2 |  | YES | - |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measurements that can be reported with one message. Value is 64. |

#### 9.2.x.9	POSITIONING MEASUREMENT MODIFICATION [FFS]

This message is sent by the gNB-CU to modify ongoing positioning measurements.

Direction: gNB-CU  gNB-DU.

Editor’s note: message contents are FFS pending RAN1 and RAN2 progress; TRP-specific IEs can be added to the messageIt is FFS if the Measurement Quantities to be Modified IE is coded explicitly in F1AP or defined as e.g. an octet string / container. Transaction ID is missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Quantities To Be Modified |  | 0..1 |  |  | YES | reject |
| >Measurement Quantities To Be Modified Item |  | 1..<maxnoMeas> |  |  | EACH |  |
| >>LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | - |
| >>UL SRS Measurement Configuration | O |  | [FFS] |  | YES | - |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

NEXT CHANGE

### 9.3.1.a	Measurement Result [FFS]

The purpose of this information element is to provide the measurement result.

Editor’s note: This IE currently contains only the necessary information for NR Cell ID with cell portions (supported in NRPPa Rel-15); all other IE contents are FFS pending RAN1 and RAN2 progress.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR CGI | O |  | 9.3.1.12 |  |
| 5GS TAC | O |  | 9.3.1.29 |  |
| Access Point Position | O |  | 9.3.1.b | The configured estimated geographical position of the antenna of the cell. |
| Cell Portion ID | O |  | 9.3.1.c |  |
| UL RTOA Measurement | O |  | [FFS] |  |

NEXT CHANGE

### 9.3.1.b	Access Point Position

This IE is used to identify the geographical position of an NR Access Point / TRP / TRP Antenna Reference Points --- details FFS.. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [x].

[Editor’s Note: IE contents are FFS pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| Uncertainty semi-major | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Uncertainty semi-minor | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Orientation of major axis | M |  | INTEGER (0..179) |  |
| Uncertainty Altitude | M |  | INTEGER (0..127) | The uncertainty altitude "h" expressed in metres is derived from the "uncertainty code" k, by: h=45x(1.025k-1). |
| Confidence | M |  | INTEGER (0..100) | In percentage |

NEXT CHANGE

### 9.3.1.c	Cell Portion ID

This IE gives the current Cell Portion associated with the target UE. The Cell Portion ID is the unique identifier for a cell portion within a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Portion ID | M |  | INTEGER (0..4095) |  |

NEXT CHANGE

### 9.3.1.d	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE for positioning purposes.

[Editor’s Note: IE contents are FFS pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number Of Transmissions | M |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. |
| Bandwidth | M |  | INTEGER (1..100,...) | The requested bandwidth of the SRS transmissions, the value of which corresponds to the number of resource blocks requested to be allocated. |