3GPP TSG-RAN WG3 Meeting #106	R3-197823

# Reno, NV, USA, 18 – 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 00xx | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Non Public Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN-Core |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Stage 2 for Non Public Networks (NPN) has been agreed by SA2 in section 5.30.3 of TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Non Public Networks are introduced. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the Non Public Networks function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non Public Networks are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.x, 16.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | Rev0: Includes agreements from RAN3#105bis (R3-196265) Rev1: Includes agreements from RAN3#106 (R3-197589, R3-197777, R3-197598) |  |  |  |  |  |  |  |  |

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MU-MIMO	Multi User MIMO

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NR	NR Radio Access

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN Cell	Public Network Cell

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

SNPN ID	Stand-alone Non-Public Network Identity

S-NSSAI	Single Network Slice Selection Assistance Information

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

UCI	Uplink Control Information

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

Cell-Defining SSB: an SSB with an RMSI associated.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

MSG1: preamble transmission of the random access procedure.

MSG3: first scheduled transmission of the random access procedure.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

SNPN Identity: the identity of Stand-alone NPN defined by the pair (PLMN ID, NID).

Xn: network interface between NG-RAN nodes.

## Next change

## 16.x	Stand-Alone NPN

### 16.x.x1	Self-Configuration of S-NPN functions

Self-configuration of NG interface builds on existing functionality including functions described in section 15. An NG-RAN node and an AMF exchange information to associate an NG interface instance with the appropriate S-NPN ID(s).

Editor’s note: it is FFS if an NG interface instance can be associated with multiple S-NPN ID(s).

Self-configuration of Xn interface builds on existing functionality including functions described in section 15. NG-RAN nodes exchange information to associate an Xn interface instance with the appropriate S-NPN ID(s).

Editor’s note: it is FFS if an Xn interface instance can be associated with multiple S-NPN ID(s).

### 16.x.x2	Access Control

During the establishment of the UE-associated logical NG-connection towards the 5GC, the AMF checks that the NG interface instance over which the establishment is requested corresponds to an SNPN which is allowed for the UE.

If the check is successful, the AMF sets up the UE-associated logical NG-connection and provides the NG-RAN node with the mobility restrictions applicable for the SNPN.

Editor’s Note: reference to TS 23.501 to be added.

### 16.x.2	Mobility

#### 16.x.2.1	Mobility Principles

- Editor’s note: describe the general principles of mobility for Stand-Alone NPN. This should cover, intra-frequency, inter-frequency and inter-RAT [RAN2].

#### 16.x.2.2	Idle Mode

- Editor’s note: describe inbound and outbound mobility in IDLE [RAN2].

#### 16.x.2.3	Inactive Mode

- Editor’s note: describe inbound and outbound mobility in INACTIVE [RAN2].

#### 16.x.2.4	Connected Mode

- Editor’s note: describe inbound and outbound mobility in CONNECTED [RAN2]. Impacts to Xn/NG procedures including data forwarding [RAN3].

The NG-RAN node has received the mobility restrictions of a given UE from the AMF at the time of UE context creation. As part of these mobility restrictions it has received the serving SNPN ID for this UE.

The NG-RAN node is also aware of the list of SNPN ID(s) supported by the candidate target cells.

At the time of handover, the source NG-RAN node determines the target cell by selecting among the candidate target cells one cell for which the list of supported SNPN ID(s) matches the serving SNPN ID.

The target NG-RAN node performs access control. In case it cannot accept the handover for the serving SNPN the target NG-RAN node fails the handover including the appropriate cause value.

### 16.x.x3	Paging

When receiving a paging message over an NG interface instance corresponding to an SNPN, the NG-RAN node may do some paging optimization by avoiding to page in cells which are not compatible with this SNPN.

For UEs in RRC_INACTIVE state, the NG-RAN node may page a neighbour NG-RAN node including the serving SNPN ID so that the neighbour NG-RAN node can do some paging optimization by avoiding to page in cells which are not compatible with the received SNPN ID.

## Next change

## 16.y	Public Network Integrated NPN

### 16.y.y1	Self-Configuration of PNI NPN functions

Self-configuration of NG interface builds on existing functionality including functions described in section 15.

Editor’s note: Exchange of list of CAG ID(s) over NG is FFS.

Self-configuration of Xn interface builds on existing functionality including functions described in section 15.

Each NG-RAN node informs the connected neighbour NG-RAN nodes of the list of supported CAG ID(s) per cell in the appropriate Xn interface management procedures.

### 16.y.y2	Access Control

During the establishment of the UE-associated logical NG-connection towards the 5GC originating from a PLMN cell, the AMF checks that the UE is allowed to access non-CAG cells.

During the establishment of the UE-associated logical NG-connection towards the 5GC originating from a CAG cell, the AMF checks that the UE is allowed to access at least one of the CAGs which is signalled as supported by the CAG cell.

If the check is successful, the AMF sets up the UE-associated logical NG-connection and provides the NG-RAN node with the list of CAGs allowed for the UE and, whether the UE is allowed to access non-CAG cells. This information is used by the NG-RAN for access control of subsequent mobility.

Editor’s Note: reference to TS 23.501 to be added.

### 16.y.2	Mobility

#### 16.y.2.1	Mobility Principles

- Editor’s note: describe the general principles of mobility for Public Network Integrated NPN. This should cover, intra-frequency, inter-frequency and inter-RAT [RAN2].

#### 16.y.2.2	Idle Mode

- Editor’s note: describe inbound and outbound mobility in IDLE [RAN2].

#### 16.y.2.3	Inactive Mode

- Editor’s note: describe inbound and outbound mobility in INACTIVE [RAN2].

#### 16.y.2.4	Connected Mode

- Editor’s note: describe inbound and outbound mobility in CONNECTED [RAN2]. Impacts to Xn/NG procedures including data forwarding [RAN3].

The source NG-RAN node has received the PNI-NPN mobility restrictions of a given UE from the AMF at the time of UE context creation. As part of these mobility restrictions it has received the CAGs allowed for the UE and whether the UE can also access PLMN cells.

The source NG-RAN node is also aware of the list of CAG IDs supported by the candidate target cells which are CAG cells.

At the time of handover, the source NG-RAN node determines a target cell among the candidates which is compatible with the received PNI-NPN restrictions.

At incoming handover, the target NG-RAN node receives the PNI-NPN mobility restrictions from the AMF and checks that the selected target cell is compatible with the received mobility restrictions.

### 16.y.y3	Paging

The NG-RAN node may receive a paging message including the list of CAGs allowed for the UE, and whether the UE is allowed to access PLMN cells. The NG-RAN node may use this information to do some paging optimization by avoiding to page in cells on which the UE is not allowed to camp.

For UEs in RRC_INACTIVE state, the NG-RAN node may page a neighbour NG-RAN node including the list of CAGs allowed for the UE, and whether the UE is allowed to access PLMN cells. The neighbour NG-RAN node may use this information to do some paging optimization by avoiding to page in cells on which the UE is not allowed to camp.