3GPP TSG-RAN3 Meeting #107-e	R3-201253

E-Meeting, 24 February – 6 March, 2020

Title: 	(TP for WWC BL CR for TS 29.413) Support for interfacing trusted non-3GPP access networks to the 5GC

Source: 	Huawei, Telecom Italia, BT, Broadcom

Agenda item:	21.2.2

Document for:	Other

1. Introduction

CB: # 38_Email038-WWC_TPs

-  Check TPs 0390,0391,0392,0393 (from both AIs 21.2.2 and 21.2.3)

- revise if needed; check details

- go for agreement

(HW)

Summary of offline disc R3-201197

0390 rev in R3-201253

0391 rev in R3-201254

0392 rev in R3-201255

0393 rev in R3-201256

5. Annex – TP

1st Change

# 3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G-RG	5G Residential Gateway

FN-RG	Fixed Network Residential Gateway

N3IWF	Non-3GPP InterWorking Function

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

W-AGF	Wireline Access Gateway Function

2nd Change

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

/**Skip the unrevelant**/

DOWNLINK NAS TRANSPORT message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	MobilityRestriction List IE

-	Index to RAT/Frequency Selection Priority IE

UPLINK NAS TRANSPORT message:

-	W-AGF Identity List IE: the information given within this IE between the W-AGF and the AMF contains a list of identifiers of N3 terminations at W-AGF as specified in TS 23.316 [x].

-	TNGF Identity Information IE: the information given within this IE between the TNGF and the AMF contains a list of identifiers of N3 terminations at TNGF as specified in TS 23.502 [4].

-	TWIF Identity Information IE: the information given within this IE between the TWIF and the AMF contains a list of identifiers of N3 terminations at TWIF as specified in TS 23.502 [4].

/**Skip the unrevelant**/

The User Location Information IE in the applicable NGAP messages between the N3IWF and the AMF includes the IP address and port number as specified in TS 38.413 [2].

The User Location Information IE in the applicable NGAP messages between the TNGF and the AMF includes the TNGF User Location Information IE as specified in TS 38.413 [2].

The User Location Information IE in the applicable NGAP messages between the TWIF and the AMF includes the TWIF User Location Information IE as specified in TS 38.413 [2].

The User Location Information IE in the applicable NGAP messages between the W-AGF and the AMF includes the Global Line Identifier or the Global Cable Identifier as specified in TS 38.413 [2].