3GPP TSG-RAN WG3 Meeting #107e	R3-201174

- Online, 24th – 28th February 2020

Agenda Item:	16.2.7

Source:	Ericsson

Title:	Summary of offline discussion for CB: # 80_Email080-NPN_E1

Document for:	Discussions & Approval

# 1	Introduction

This is the summary document for the offline discussion on come back number eighty.

Chair’s summary:

CB: # 80_Email080-NPN_E1

-  Over E1, send the list of CAG IDs supported by gNB-CU-UP to gNB-CU-CP; maximum number of CAG IDs per PLMN that can be signaled over E1 shall match the maximum number of cells that can be hosted in a gNB (i.e. 16384)? (Nok), (ZTE)

- use CHOICE? Other details? (HW, CT)

- A new NPN Support Information IE able to carry NIDs and CAGs associated with the PLMN Identity in the Supported PLMNs IE is included in the Supported PLMNs IE in the respective E1 interface management messages; If CAG-Identifiers are included in the new NPN Support Information IE, the gNB-CU-UP provides resources only for UEs registered in such PNI-NPNs within the respective CAG cells; The Bearer Context management related messages carry PNI-NPN related information only if the gNB-CU-UP has indicated support for those CAGs. It is up to the gNB-CU-CP to select another gNB-CU-CP, if the UE moves into a public cell or into another PNI-NPN not supported by the gNB-CU-UP? (E///)

- if agreeable, revise/merge as needed

(E///)

Summary of offline disc R3-201174

# 2	Discussion

## 2.1	E1 Interface management signalling related

Proposal 1:	PNI-NPN and SNPN support information organised as a CHOICE, included on a per PLMN basis [2][4][6][7].

Comments on proposal 1:

1.1: (CB rapporteur): It was assumed that this approach is acceptable for proponents of [1].

Proposal 2: 	The maximum number of CAG IDs per PLMN that can be signalled over E1 shall match the maximum number of cells that can be hosted in a gNB (i.e. 16384) [1].

Comments on proposal 2:

2.1: (CB rapporteur): this is related to another discussion, proposal to not discuss this in this CB

Proposal 3:	It is proposed to follow [7] as baseline for E1 interface management signalling related E1AP changes.

Comments on proposal 3:

3.1: (CB rapporteur): I wouldn’t see too many issues with that, please comment on any details spotted.

Proposal 4	Include basic information into TS 38.460, as shown in [6]

Comments on proposal 4:

4.1: (CB rapporteur): proposal to go with [6], as it is more general and the wording “NID and CAG information” might be misleading, if we follow proposal 3.

## 2.2	E1 UE Context signalling related

Proposal 5	Introduce serving NID in E1 UE Context signalling, as shown in [4], and (probably implicitly intended) in [3] and [7].

Comments on proposal 5:

Proposal 6	Introduce PNI-NPN information in E1 UE Context signalling , as shown in [3] and [7].

Comments on proposal 6:

Proposal 7	Include basic information into TS 38.460, following agreements in 5 and 6

Comments on proposal 7:

# 3	Conclusion and Proposals

It is proposed to follow possible agreements in section 2 in terms of final TPs for 38.463 and 38.460.

# 4	References

[1]	R3-200239, (TP for NPN BL CR for TS 38.463): NPN for E1 (Nokia, Nokia Shanghai Bell), other

[2]	R3-200341, TP to capture agreements for NPN support to TS38.463 (ZTE Corporation), other

[3]	R3-200342, TP for NPN support to TS38.463 (ZTE Corporation), other

[4]	R3-200722, (TP for NPN BL CR for TS 38.463): Further support of NPN over E1 (Huawei, China Telecommunication), other

[5]	R3-200723, CR to TS 38.460 on support of NPN (Huawei, China Telecommunication), CR0029r1, TS 38.460 v16.0.0, Rel-16, Cat. B

[6]	R3-200973, NPN – E1 Aspects (Ericsson), other

[7]	R3-200974, [TP for NPN BL CR 38.463] Introduction of further NPN E1AP content (Ericsson), other