3GPP TSG-RAN WG3 #107-e	R3-201411

24 February-6 March 2020 E-Meeting

Title:	LS on removal of Management Based MDT Allowed IE for NR

- Release:	Rel-16

Work Item:	NR_SON_MDT-Core

Source:	RAN3

To:	RAN2

CC:	SA5

- Contact Person:

- Name:	Shankar Krishnan

E-mail Address:	shakrish@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- 1. Overall Description:

RAN3 has reached an agreement to not include Management Based MDT Allowed IE in NGAP and XnAP specifications and instead reuse Management Based MDT PLMN List IE which signals the list of PLMNs supported for Management based MDT as an indicator for signalling user consent as well for Management Based MDT.

To give a background, Management Based MDT Allowed IE was introduced in LTE Release-10 for signalling user consent and Management Based MDT PLMN List IE in a later LTE release to signal the list of PLMNs supported for management-based MDT. To optimize the signalling behaviour, it was agreed in RAN3 to reuse the Management Based MDT PLMN List IE as an indirect indicator for user consent as well and remove the duplicate Management Based MDT Allowed IE for NR MDT.

Please note that the Management Based MDT Allowed IE is still needed for legacy MDT (LTE/UMTS) and the agreement to remove this IE applies only to MDT behaviour for NR. Considering the above agreement, RAN2 stage 2 specifications (TS 37.320) need to be updated to clarify that Management Based MDT Allowed IE is not needed for NR MDT.

- 2. Actions:

ACTION:

RAN3 respectfully ask RAN2 to consider the above agreement to update the corresponding clauses in stage2 specifications

RAN3 respectfully ask SA5 to consider the above agreement before adding clauses for user consent information for NR MDT.

- 3. Date of Next TSG-RAN WG3 Meetings:

TSG RAN WG2 Meeting #109bis			20-24 April 2020	Sapporo, Japan