3GPP TSG-RAN WG3 Meeting #107bis-e	R3-201832

E-meeting, 20 – 30 April, 2020

Agenda item:	10.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Load reporting metric per slice for improved interoperability

Document for:	Text Proposal

# 1	Introduction

For the purpose of improved interoperability RAN3 has agreed to introduce detailed load metric e.g. for radio load on top of the composite available capacity metric. In the current version of the baseline CRs, these additional metrics have been agreed on cell level, and on beam level for radio load. However further slice level information is crucial on the F1 interface, where it would help the CU-CP to anticipate on access control outcome in the gNB-DU and help align access control functionality (and speed) between split and non-split architecture.

TP for BL CR to TS 38.473 [1] is included in annex of this paper.

# References

[1]		R3-201618, BL CR to TS 38.473

# Annex	- TP for BL CR to TR 38.473

<<< start of changes >>>

#### 9.3.1.x7 Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources per network slice that are available relative to the total gNB-DU resources. The Slice Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Available Capacity List |  | 1 |  |  |
| Slice Available Capacity Item |  | 1..< maxnoofSliceItems > |  |  |
| >PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN |
| >S-NSSAI Available Capacity List |  | 1 |  |  |
| >>S-NSSAI Available Capacity Item | M | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI |  |  | 9.3.1.38 |  |
| >>>Slice Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Slice Capacity Value should be measured on a linear scale. |
| >>>TNL Capacity Load Indicator | O |  | 9.2.1.X1 |  |
| >>>Slice Radio Resource Status | O |  | 9.3.1.xy |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems [FFS] | Maximum no. of signalled slice support items. Value is 1024. [FFS] |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

<<< next change >>>

#### 9.3.1.xy	Slice Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs within a slice for all traffic in downlink and uplink and the usage of PDCCH CCEs for downlink and uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL GBR PRB usage | M |  | INTEGER (0..100) | DL GBR PRB usage |
| UL GBR PRB usage | M |  | INTEGER (0..100) | UL GBR PRB usage |
| DL non-GBR PRB usage | M |  | INTEGER (0..100) | DL non-GBR PRB usage |
| UL non-GBR PRB usage | M |  | INTEGER (0..100) | UL non-GBR PRB usage |
| DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

<<< end of changes >>>