3GPP TSG-RAN3 Meeting #107bis-e	R3-201932

20 – 30 April 2020 E-Meeting

Title: 	(TP for SON BL CR for TS 38.413)Addition of RAReport and ConnEstFailReport transfer

Source: 	CATT

Agenda item:	10.2.1.1

Document Type:	other

# 1. Introduction

This contribution is the TP for NGAP BL CR R3-201612 with Addition of inter-node transfer of RAReport and ConnEstFailReport.

# 2. Discussion

Addition of RAReport and ConnEstFailReport in SON Information IE and Inter-system SON Information IE in UPLINK/ DOWNLINK RAN CONFIGURATION TRANSFER message.

# 3. Annex – TP

Start of the first change

9.3.3.Y3	Failure Indication

This IE contains the failure indication to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE RLF Report Container | O |  | 9.3.3.Z1 | FFS |
| UE RA Report Container | O |  | OCTET STRING | RA-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [18]. |
| UE ConnEstFail Report Container | O |  | OCTET STRING | ConnEstFailReport-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [18]. |

### 9.4.5	Information Element Definitions

#### -- F

FailureIndication ::= SEQUENCE {

uERLFReportContainer 	UERLFReportContainer,

uERAReportContainer     OCTET STRING,

uECEFReportContainer    OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { FailureIndication-ExtIEs} }	OPTIONAL

}

End of the last change