3GPP TSG-RAN WG3 Meeting #107e-bis	R3-202563

e-meeting, 20 - 24 April 2020				revision from R3-200362, 2016

Title:	[DRAFT] Response LS on the “LS out on Location of UEs and associated key issues”

Response to:	S2-1912560

Release:	Rel-17

Work Item:	FS_5GSAT_ARCH

Source:	Thales [To be RAN3]

To:	SA2, RAN2, SA3-LI

Cc:

Contact Person:

Name:	Nicolas Chuberre

Tel. Number:	+33680948432

E-mail Address:	nicolas.chuberre@thalesaleniaspace.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall Description:

RAN3 would like to thank SA2 for their LS in S2-1912560 on Location of UEs and associated key issues.

RAN3 would like to clarify that it has completed its work in the scope of the rel-16 NTN Study Item (“Study on solutions for NR to support non-terrestrial networks (NTN)”, FS_NR_NTN_solutions). The architectures and the solutions considered by RAN3 are captured in TR 38.821. No further study on NTN is expected in Rel-16 by RAN3.

However RAN3 would like to provide the following feedbacks on the solutions #1 & #12 described in TR 23.737.

## Solution #1: Position-based and fixed TA Satellite Access

During the study phase, RAN3 discussed the Earth fixed tracking areas, and the location based paging (Section 8.2.1 of TR38.821). But RAN3 did not discuss some aspects of Solution 1, e.g. “On the basis of the knowledge of the position of UE, the corresponding cell / TA /RA to which the UE belongs is determined by the RAN”; “the NG-RAN is assumed to handle all aspects with respect to the motion of the satellite (including beam steering with the motion of non-geostationary satellites).”, etc.

For beam size larger than maximum cell size supported by NR (e.g. 100 km diameter), Solution #1 is feasible. In other cases, Solution #1 may be possible, further study is needed on the location of UE by the network which is planned as part of the Rel-17 NR-NTN WI.

The applicability of the solution may differ according to the NTN scenarios identified in “Table 4.2-1: Reference scenarios” of TR 38.821.

| Principles of the solution #1 of TR 23.737 | NGSO with Earth moving beam | GEO or NGSO with Earth Fixed beam |
| --- | --- | --- |
| A - Principle of the fixed Tracking Areas (TAs) | Supported by NG-RAN assuming (Note 1) and (Note 2) | Supported by NG-RAN assuming (Note 1) |
| B - Principle for the mobility registration update | It can be supported by NG-RAN thanks to detection of TAU/RAU | For beam size larger than maximum cell size supported by NR, a procedure to locate UE by the NG-RAN is needed (Note 3), else the existing procedures apply |
| C - Principle for CN Paging | Paging of UE within the allocated RA | Paging of UE within the allocated RA |

Note 1: For all scenarios, tracking areas can be defined as Earth fixed geographical zones (with vertices for instances) bounded with country borders. Network operators can define the tracking areas each assigned to a policy class.  For beam size larger than typical cell size, the TAs may be defined smaller than the satellite beam coverage.

Note 2: The NG-RAN can update the list of TAs to be broadcasted at a given beam in real-time according to the ephemeris and confirm the broadcast TAC is associated with the geographical area covered by the satellite beam at a given time. When a UE does not receive one of the TAs associated to its assigned RA, it has to initiate a Registration Area Update. This allows the NG-RAN to detect that the UE crosses a border without impacting cell (re)selection and hand-over procedures.

Note 3: In order to detect when a UE crosses country borders NG-RAN can rely on the UE location reported by the UE or it can locate the UE. The later feature is planned to be defined as part of the Rel-17 NR-NTN WI.

## Solution #12: Satellite cells for 5G satellite access

During the SI phase, RAN3 did not discuss Solution #12 or like solutions.

Adoption, of Solution #12 would require the addition in RAN specifications of new concepts such as virtual cells or geographical zones for defining areas with specific policy requirements. This may add complexity (e.g. signaling impact) which should be avoided if simpler solutions can be identified.

Solution #12 does not align with some RAN3 agreements during the SI, for example, RAN3 assumed that Earth stations are transport nodes out of NG-RAN scope.

# Actions:

To SA2, RAN2, SA3-LI working group

RAN WG3 respectfully requests SA2, RAN2 and SA3-LI to take the above information into account.

# Date of Next TSG-RAN3 Meetings:

3GPPRAN3#108-e	01 - 12 June 2020   	e-meeting

3GPPRAN3#109	24 - 28 August 2020   	Toulouse, France

3GPPRAN3#109bis	12 - 16 October 2020   	Sophia Antipolis, France