3GPP TSG-RAN WG3#107bis-e                                                                           R3-202859

20 - 30 April 2020

Online

Title:	LS on cell-specific signals/channel configurations

Response to:

Release:	Rel-16

Work Item:	IAB_NR

Source:	RAN3

To:	RAN1

Cc:

Contact Person:

Name:	Xueying Diao

E-mail Address:	diao.xueying@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

During RAN3#107e meeting, an LS on updated Rel-16 LTE and NR parameter lists [R3-200081] was sent from RAN1 and updated Rel-16 NR PHY parameter list for IAB is provided in the LS. According to the LS, a parent IAB node/donor can be provided with cell-specific signal/channel configurations of each child IAB-DU where these cell-specific signal/channel configurations include resources for periodic CSI-RS transmission and SR reception. The corresponding RAN1 agreement is copied below.

However, RAN3 would like to point out that CSI-RS and SR resources are UE-specific. Since each child IAB-DU may serve many UEs and IAB-MTs, and since these UE-specific parameters are likely to change often, providing such dynamic UE-specific configuration via F1AP may cause a signaling storm between IAB donor and IAB nodes. We believe that the explicit configuration of these resources as Not Available on the parent node provide the same functionality but without causing such signaling storm. Please take these considerations into account in further discussions.

RAN1 #99 Agreements: A parent IAB node/donor can be provided with cell-specific signals/channels configurations of each child IAB-DU. How/whether to use the information to handle any potential conflict at the parent IAB node/donor is left to network implementation The list of cell-specific signals/channels includes: - resources for SSB transmission at DU, including both CD-SSB and non-CD-SSB; - configured RACH occasions for receiving at the DU - periodic CSI-RS transmission at the DU - scheduled resource for receiving SR at DU

2. Actions:

To RAN1 group:

ACTION: 	RAN3 kindly ask RAN1 to take RAN3’s considerations into account in further discussion. .

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP RAN3#108-e          1 – 12 June 2020			Online

3GPP RAN3#109             24 – 28 August 2020                                          Toulouse, France