3GPP TSG-RAN WG3 #109-e																		R3-204635

17-28 August 2020 Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0520 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of direct data forwarding for inter-system HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, CATT,Nokia , Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Direct_data_fw_NR-Core |  |  |  |  |  | Date: |  |  | 2020-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For inter-system HO,direct data forwarding is not supported in case the source node or target node is CP/UP separation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | One new optional IE i.e. E-RAB ID, is added into the QoS Flow Item IE in order to inform the gNB-CU-UP of the E-RAB-to-QoS-flow mapping for inter-system direct data forwarding. One new IE i.e. Data Forwarding Request E-RAB List is added into the following two E1AP structures respectively: the PDU Session Resource To Setup Item and the PDU Session Resource To Setup Modification Item. One new IE i.e.Data Forwarding Response E-RAB List, is added into the following two E1AP structures respectively: the PDU Session Resource Setup Item and PDU Session Resource Setup Modification Item One new IE i.e. Data Forwarding E-RAB List is added into PDU Session Resource To Modify Item IE Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the inter-system handover function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Direct data forwarding could not be supported in case the source node or target node is CP/UP separation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2,8.3.2.2,9.3.12.25, 9.3.1.x, 9.3.3.2, 9.3.3.5, 9.3.3.10, 9.3.3.11, 9.3.3.17, 9.3.3.y, 9.3.3.z, 9.4.4,9.4.5,9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.413 CR 1738 TS/TR 37.340 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////start of change ////////////////////////////////////////////////////////////////////////

8.3.1	Bearer Context Setup

8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT SETUP RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	For each established PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each established PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each established DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each established DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Existing Allocated S1 DL UP Transport Layer Information IE or the Existing Allocated NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may re-use the indicated resources already allocated for this bearer context. If the gNB-CU-UP decides to re-use the indicated resources, it shall include the S1 DL UP Unchanged IE or the NG DL UP Unchanged IE in the BEARER CONTEXT SETUP RESPONSE message.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [20].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE, Data Forwarding Request E-RAB List IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE, Data Forwarding Response E-RAB List IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT SETUP RESPONSE message.

If the DL UP Parameters IE is contained in the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall configure the corresponding information.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE of the BEARER CONTEXT SETUP RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane ciphering for the concerned PDU session.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall use this value when enforcing the maximum integrity protected data rate for the UE.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

For each requested DRB, if the PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-UP shall include two UP Transport Layer Information IEs in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of the two UP Transport Layer Information IEs is for the primary path.

If the PDCP SN Status Information IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the QoS Flow Mapping Indication IE is contained in the QoS Flows Information To Be Setup IE within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take it into account as specified in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT SETUP RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT SETUP REQUEST message.

If the Trace Activation IE is included in the BEARER CONTEXT SETUP REQUEST message the gNB-CU-UP shall, if supported, initiate the requested trace function as described in TS 32.422 [24].

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the UE CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the UE CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20].

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT MODIFICATION RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	A list of PDU Session Resources which are successfully modified shall be included in the PDU Session Resource Modified List IE;

-	A list of PDU Session Resources which failed to be modified shall be included in the PDU Session Resource Failed To Modify List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Security Information IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Aggregate Maximum Bit Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE, Data Forwarding Request E-RAB List IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE , Data Forwarding Response E-RAB List IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information, except for the PDCP SN UL Size IE, the PDCP SN DL Size IE and the RLC mode IE which shall be ignored.

If the E-UTRAN QoS IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the PDCP SN Status Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the UL COUNT Value IE and the DL COUNT Value IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP SN Status Information IE is contained in the DRB To Setup List IE or the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the DL UP Parameters IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Cell Group To Add IE or the Cell Group To Modify IE or the Cell Group To Remove IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall add or modify or remove the corresponding cell group.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501 [20].

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the SDAP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Flow Mapping Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

For each requested DRB, if the PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-CP shall include two UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION REQUEST message, and the gNB-CU-UP shall also include two UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of the two UP Transport Layer Information IEs is for the primary path.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the Bearer Context Modification (gNB-CU-CP initiated) procedure, i.e. only one UP Transport Layer Information per Cell Group ID is present in DL UP Parameters IE for the concerned DRB, then the gNB-CU-UP shall consider that PDCP duplication is deconfigured for this DRB. If such Bearer Context Modification (gNB-CU-CP initiated) procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the New UL TNL Information Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the new UP Transport Layer Information in the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE of the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List of the BEARER CONTEXT MODIFICATION REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane ciphering for the concerned PDU session.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

If the QoS Flow Mapping Indication IE is contained in the QoS Flow QoS Parameters List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Data Discard Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to “Required”, the gNB-CU-UP shall consider that a RAN Paging Failure occurred for that UE. The gNB-CU-UP shall discard the user plane data for that UE and consider that the bearer context is still suspended.

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store the corresponding information and replace any existing information.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take it into account for each DRB, as specified in TS 28.552 [22].

If the DRB QoS IE is contained within the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account for each DRB, as specifed in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message including Activity Notification Level IE and its value does not match the current bearer context, the gNB-CU-UP shall ignore the Activity Notification Level IE and also the requested modification of inactivity timer.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT MODIFICATION RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT MODIFICATION REQUEST message.

If the Old QoS Flow List - UL End Marker expected IE is included in the PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message for a DRB to be modified, the gNB-CU-UP shall consider that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. The gNB-CU-UP shall consider that the Old QoS Flow List - UL End Marker expected  IE only contains UL QoS flow information for QoS flows for which no SDAP end marker has been yet received on the source side.

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

If there is at least one DRB removed by the gNB-CU-UP, the gNB-CU-UP shall, if supported, include the Retainability Measurements Information IE in the BEARER CONTEXT MODIFICATION RESPONSE message, providing information on the removed DRB(s) for retainability measurements in the gNB-CU-CP, as described in TS 32.425 [26] and TS 28.552 [22].

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20].

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.1.25	QoS Flow QoS Parameters List

This IE contains a list of QoS Flows including the QoS Flow parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow List |  | 1 |  |  | - | - |
| >QoS Flow Item |  | 1..<maxnoofQoSFlows> |  |  | - | - |
| >>QoS Flow Identifier | M |  | 9.3.1.24 |  | - | - |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.26 |  | - | - |
| >>QoS Flow Mapping Indication | O |  | 9.3.1.60 | Indicates that only the uplink or downlink QoS flow is mapped to the DRB | - | - |
| >>E-RAB ID | O |  | 9.3.1.X | Used for inter-system handovers from 5GS to EPS. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.1.X	E-RAB ID

This IE is the identifier of the LTE E-RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-RAB ID | M |  | INTEGER (0..15, …) |  |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.3.2	PDU Session Resource To Setup List

This IE contains PDU session resource related information used at Bearer Context Setup Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when at least one Non-GBR QoS Flows is being setup. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Existing Allocated NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >Network Instance | O |  | 9.3.1.62 | This IE is ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Contains the PDCP SN Status at setup after Resume. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB. | YES | ignore |
| >Data Forwarding Request E-RAB List | O |  | 9.3.3.y | For inter-system handovers from EPS to 5GS. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.3.5	PDU Session Resource Setup List

This IE contains setup PDU session resource related information used at Bearer Context Setup Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |  |  |
| >Security Result | O |  | 9.3.1.52 |  |  |  |
| >NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |  |  |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. |  |  |
| >NG DL UP Unchanged | O |  | ENUMERATED (True, …) |  |  |  |
| >DRB Setup List |  | 1 |  |  |  |  |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. |  |  |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  |  |  |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |  |  |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |  |  |
| >DRB Failed List |  | 0.. 1 |  |  |  |  |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>Cause | M |  | 9.3.1.2 |  |  |  |
| >Data Forwarding Response E-RAB List | O |  | Data Forwarding E-RAB List 9.3.3.z | For inter-system handovers from EPS to 5GS. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.3.10	PDU Session Resource To Setup Modification List

This IE contains PDU session resource to setup related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when Non-GBR QoS Flows are setting up. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 |  | - | - |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >Data Forwarding Request E-RAB List | O |  | 9.3.3.y | For inter-system handovers from EPS to 5GS. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Indication | O |  | 9.3.1.23 | This IE is not used in this release. | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | - | - |
| >NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >PDU Session Data Forwarding Information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 | This IE is ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flow Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>DRB Data forwarding information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | Indicates that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. | YES | reject |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >S-NSSAI | O |  | 9.3.1.9 |  | YES | reject |
| >Data Forwarding Request E-RAB List | O |  | 9.3.3.z | For inter-system handovers from EPS to 5GS. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.3.17	PDU Session Resource Setup Modification List

This IE contains setup PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resource Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >Security Result | O |  | 9.3.1.52 |  |
| >NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. |
| >DRB Setup List |  | 1 |  |  |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >DRB Failed List |  | 0.. 1 |  |  |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |
| >>>Cause | M |  | 9.3.1.2 |  |
| >Data Forwarding E-RAB List | O |  | 9.3.3.z | For inter-system handovers from EPS to 5GS. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.3.y	Data Forwarding Request E-RAB List

This IE is used during a handover between EPS and 5GS to request data forwarding address information, if direct data forwarding is applied.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Forwarding Request E-RAB Item |  | 1..<maxnoofERABs> |  |  |
| >E-RAB ID | M |  | 9.3.1.x |  |
| >E-RAB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.3.z	Data Forwarding Response E-RAB List

This IE is used during a handover between EPS and 5GS to provide data forwarding address information, if direct data forwarding is applied.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Forwarding E-RAB Item |  | 1..<maxnoofERABs> |  |  |
| >E-RAB ID | M |  | 9.3.1.x |  |
| >E-RAB Data forwarding information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info either from the target gNB-CU-UP or to the source gNB-CU-UP. |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID,

TraceID,

TraceActivation,

SubscriberProfileIDforRFP,

AdditionalRRMPriorityIndex,

RetainabilityMeasurementsInfo,

Transport-Layer-Address-Info,

E-RABID,

Data-Forwarding-Request-E-RAB-List,

Data-Forwarding-Response-E-RAB-List

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-TraceID,

id-TraceActivation,

id-SubscriberProfileIDforRFP,

id-AdditionalRRMPriorityIndex,

id-RetainabilityMeasurementsInfo,

id-Transport-Layer-Address-Info,

id-E-RABID,

id-Data-Forwarding-Request-E-RAB-List,

id-Data-Forwarding-Response-E-RAB-List

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

ActivityInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ActivityNotificationLevel	::=	ENUMERATED	{

drb,

pdu-session,

ue,

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AveragingWindow  ::= INTEGER (0..4095, ...)

#### -- B

BearerContextStatusChange	::= 	ENUMERATED {

suspend,

resume,

...

}

BitRate ::= INTEGER (0..4000000000000,...)

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer	{{Cause-ExtIEs}}

}

Cause-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

unknown-or-already-allocated-gnb-cu-cp-ue-e1ap-id,

unknown-or-already-allocated-gnb-cu-up-ue-e1ap-id,

unknown-or-inconsistent-pair-of-ue-e1ap-id,

interaction-with-other-procedure,

pPDCP-Count-wrap-around,

not-supported-QCI-value,

not-supported-5QI-value,

encryption-algorithms-not-supported,

integrity-protection-algorithms-not-supported,

uP-integrity-protection-not-possible,

uP-confidentiality-protection-not-possible,

multiple-PDU-Session-ID-Instances,

unknown-PDU-Session-ID,

multiple-QoS-Flow-ID-Instances,

unknown-QoS-Flow-ID,

multiple-DRB-ID-Instances,

unknown-DRB-ID,

invalid-QoS-combination,

procedure-cancelled,

normal-release,

no-radio-resources-available,

action-desirable-for-radio-reasons,

resources-not-available-for-the-slice,

pDCP-configuration-not-supported,

...,

ue-dl-max-IP-data-rate-reason,

uP-integrity-protection-failure,

release-due-to-pre-emption

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...

}

Cell-Group-Information	::= SEQUENCE (SIZE(1.. maxnoofCellGroups)) OF Cell-Group-Information-Item

Cell-Group-Information-Item	::=	SEQUENCE {

cell-Group-ID							Cell-Group-ID,

uL-Configuration						UL-Configuration		OPTIONAL,

dL-TX-Stop								DL-TX-Stop				OPTIONAL,

rAT-Type								RAT-Type				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Cell-Group-Information-Item-ExtIEs } }	OPTIONAL,

...

}

Cell-Group-Information-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Cell-Group-ID	::=		INTEGER (0..3, ...)

CipheringAlgorithm	::=	ENUMERATED {

nEA0,

c-128-NEA1,

c-128-NEA2,

c-128-NEA3,

...

}

CNSupport ::= ENUMERATED {

c-epc,

c-5gc,

both,

...

}

CommonNetworkInstance ::= OCTET STRING

ConfidentialityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

ConfidentialityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

CP-TNL-Information		::= 	CHOICE {

endpoint-IP-Address		TransportLayerAddress,

choice-extension		ProtocolIE-SingleContainer	{{CP-TNL-Information-ExtIEs}}

}

CP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

{ ID id-endpoint-IP-Address-and-Port	CRITICALITY reject	TYPE Endpoint-IP-address-and-port 	PRESENCE mandatory},

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

transactionID					TransactionID					OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxnoofErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request,

qoS-Flows-Forwarded-On-Fwd-Tunnels	QoS-Flow-Mapping-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

Data-Usage-per-PDU-Session-Report ::= SEQUENCE {

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

pDU-session-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-PDU-Session-Report-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-PDU-Session-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-QoS-Flow-List	::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF Data-Usage-per-QoS-Flow-Item

Data-Usage-per-QoS-Flow-Item ::= SEQUENCE {

qoS-Flow-Identifier		QoS-Flow-Identifier,

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

qoS-Flow-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-QoS-Flow-Item-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-QoS-Flow-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-Report-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF Data-Usage-Report-Item

Data-Usage-Report-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

rAT-Type					RAT-Type,

dRB-Usage-Report-List		DRB-Usage-Report-List,

iE-Extensions	ProtocolExtensionContainer { { Data-Usage-Report-ItemExtIEs } }	OPTIONAL,

...

}

Data-Usage-Report-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DefaultDRB	::=	ENUMERATED	{

true,

false,

...

}

DiscardTimer	::=	ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity}

DL-TX-Stop	::=	ENUMERATED	{

stop,

resume,

...

}

DRB-Activity	::= ENUMERATED {

active,

not-active,

...

}

DRB-Activity-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Activity-Item

DRB-Activity-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

dRB-Activity				DRB-Activity,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-EUTRAN

DRB-Confirm-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-NG-RAN

DRB-Confirm-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-EUTRAN

DRB-Failed-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-EUTRAN

DRB-Failed-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-NG-RAN

DRB-Failed-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-NG-RAN

DRB-Failed-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-EUTRAN

DRB-Failed-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-NG-RAN

DRB-Failed-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-ID	::=	INTEGER (1..32, ...)

DRB-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-EUTRAN

DRB-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-NG-RAN

DRB-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-UP-Transport-Parameters					UP-Parameters							OPTIONAL,

pDCP-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

flow-Setup-List								QoS-Flow-List							OPTIONAL,

flow-Failed-List							QoS-Flow-Failed-List					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Removed-Item	::=	SEQUENCE {

dRB-ID									DRB-ID,

dRB-Released-In-Session					ENUMERATED {released-in-session, not-released-in-session, ...}		OPTIONAL,

dRB-Accumulated-Session-Time			OCTET STRING (SIZE(5))													OPTIONAL,

qoS-Flow-Removed-List					SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Removed-Item		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Removed-Item-ExtIEs } }			OPTIONAL,

...

}

DRB-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-EUTRAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-EUTRAN

DRB-Required-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information							OPTIONAL,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-NG-RAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-NG-RAN

DRB-Required-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

flow-To-Remove							QoS-Flow-List								OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-EUTRAN

DRB-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

s1-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-EUTRAN

DRB-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-NG-RAN

DRB-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-NG-RAN

DRB-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Status-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

pDCP-DL-Count				PDCP-Count		OPTIONAL,

pDCP-UL-Count				PDCP-Count		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Status-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Status-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-EUTRAN

DRBs-Subject-To-Counter-Check-Item-EUTRAN	::=	SEQUENCE {

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions		ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-NG-RAN

DRBs-Subject-To-Counter-Check-Item-NG-RAN	::=	SEQUENCE {

pDU-Session-ID				PDU-Session-ID,

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions			ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-EUTRAN

DRB-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration						OPTIONAL,

eUTRAN-QoS								EUTRAN-QoS								OPTIONAL,

s1-UL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request					PDCP-SN-Status-Request						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

cell-Group-To-Add						Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify					Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove					Cell-Group-Information					OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-NG-RAN

DRB-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration						OPTIONAL,

pDCP-Configuration							PDCP-Configuration						OPTIONAL,

dRB-Data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request							PDCP-SN-Status-Request						OPTIONAL,

pdcp-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

cell-Group-To-Add							Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify						Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove						Cell-Group-Information					OPTIONAL,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List				OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject	EXTENSION QoS-Flow-List	PRESENCE optional}|

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-EUTRAN

DRB-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-EUTRAN

DRB-Required-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-NG-RAN

DRB-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-NG-RAN

DRB-Required-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-EUTRAN

DRB-To-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration,

eUTRAN-QoS								EUTRAN-QoS,

s1-UL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information					Cell-Group-Information,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

existing-Allocated-S1-DL-UP-TNL-Info	UP-TNL-Information						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-EUTRAN

DRB-To-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

pDCP-Configuration							PDCP-Configuration,

eUTRAN-QoS									EUTRAN-QoS,

s1-UL-UP-TNL-Information					UP-TNL-Information,

data-Forwarding-Information-Request			Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information						Cell-Group-Information,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

qos-flow-Information-To-Be-Setup					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-SN-Status-Information								PDCP-SN-Status-Information								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-To-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-NG-RAN

DRB-To-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer					OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional},

...

}

DRB-Usage-Report-List ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF DRB-Usage-Report-Item

DRB-Usage-Report-Item	::= SEQUENCE {

startTimeStamp					OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions					ProtocolExtensionContainer { { DRB-Usage-Report-Item-ExtIEs} } OPTIONAL,

...

}

DRB-Usage-Report-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Duplication-Activation	::=		ENUMERATED {

active,

inactive,

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel					QoSPriorityLevel,

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

fiveQI								INTEGER (0..255, ...)								OPTIONAL,

delayCritical						ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

averagingWindow 					AveragingWindow										OPTIONAL,

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

DataDiscardRequired	::= 	ENUMERATED {

required,

...

}

Data-Forwarding-Request-E-RABList	::= SEQUENCE (SIZE(1.. maxnoofERABs)) OF Data-Forwarding-Request-E-RAB-Item

Data-Forwarding-Request-E-RAB-Item	::= SEQUENCE {

e-RAB-ID										E-RAB-ID,

e-RAB-Data-Forwarding-Information-Request		E-RAB-Data-Forwarding-Information-Request,

iE-Extensions					ProtocolExtensionContainer { { Data-Forwarding-Request-E-RAB-Item-ExtIEs} } OPTIONAL,

...

}

Data-Forwarding-Request-E-RAB-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Response-E-RAB-List	::= SEQUENCE (SIZE(1.. maxnoofERABs)) OF Data-Forwarding-Response-E-RAB-Item

Data-Forwarding-Response-E-RAB-Item	::= SEQUENCE {

e-RAB-ID										E-RAB-ID,

e-RAB-Data-Forwarding-Information				E-RAB-Data-Forwarding-Information,

iE-Extensions					ProtocolExtensionContainer { { Data-Forwarding-Response-E-RAB-Item-ExtIEs} } OPTIONAL,

...

}

Data-Forwarding-Response-E-RAB-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

EncryptionKey	::=	OCTET STRING

Endpoint-IP-address-and-port::= SEQUENCE {

endpoint-IP-Address			TransportLayerAddress,

portNumber					PortNumber,

iE-Extensions							ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL

}

Endpoint-IP-address-and-port-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {EUTRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

EUTRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofEUTRANQOSParameters)) OF EUTRAN-QoS-Support-Item

EUTRAN-QoS-Support-Item ::= SEQUENCE {

eUTRAN-QoS	EUTRAN-QoS,

iE-Extensions			ProtocolExtensionContainer { { EUTRAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

EUTRAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS	::= SEQUENCE {

qCI										QCI,

eUTRANallocationAndRetentionPriority	EUTRANAllocationAndRetentionPriority,

gbrQosInformation						GBR-QosInformation										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { EUTRAN-QoS-ExtIEs } }	OPTIONAL,

...

}

EUTRAN-QoS-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

E-RAB-ID				:: INTEGER (0..15, …)

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PDCP-Configuration	::=	SEQUENCE {

pDCP-SN-Size-UL							PDCP-SN-Size,

pDCP-SN-Size-DL							PDCP-SN-Size,

rLC-Mode								RLC-Mode,

rOHC-Parameters							ROHC-Parameters			OPTIONAL,

t-ReorderingTimer						T-ReorderingTimer		OPTIONAL,

discardTimer							DiscardTimer			OPTIONAL,

uLDataSplitThreshold					ULDataSplitThreshold	OPTIONAL,

pDCP-Duplication						PDCP-Duplication		OPTIONAL,

pDCP-Reestablishment					PDCP-Reestablishment	OPTIONAL,

pDCP-DataRecovery						PDCP-DataRecovery		OPTIONAL,

duplication-Activation				Duplication-Activation		OPTIONAL,

outOfOrderDelivery						OutOfOrderDelivery		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Configuration-ExtIEs } }	OPTIONAL,

...

}

PDCP-Configuration-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-Count	::=	SEQUENCE {

pDCP-SN				PDCP-SN,

hFN					HFN,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Count-ExtIEs } }	OPTIONAL,

...

}

PDCP-Count-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN-Status-Request	::=		ENUMERATED {

requested,

...

}

PDCP-DataRecovery	::=	ENUMERATED	{

true,

...

}

PDCP-Duplication	::=	ENUMERATED	{

true,

...

}

PDCP-Reestablishment	::=	ENUMERATED	{

true,

...

}

PDU-Session-Resource-Data-Usage-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Data-Usage-Item

PDU-Session-Resource-Data-Usage-Item	::= SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

mRDC-Usage-Information						MRDC-Usage-Information,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Data-Usage-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Data-Usage-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN		::=		INTEGER	(0..262143)

PDCP-SN-Size	::=	ENUMERATED	{

s-12,

s-18,

...

}

PDCP-SN-Status-Information ::= SEQUENCE {

pdcpStatusTransfer-UL	DRBBStatusTransfer,

pdcpStatusTransfer-DL	PDCP-Count,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransfer-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBBStatusTransfer ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..131072))										OPTIONAL,

countValue				PDCP-Count,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-ID	::=	INTEGER (0..255)

PDU-Session-Resource-Activity	::= ENUMERATED {

active,

not-active,

...

}

PDU-Session-Resource-Activity-List ::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Activity-Item

PDU-Session-Resource-Activity-Item	::= SEQUENCE {

pDU-Session-ID									PDU-Session-ID,

pDU-Session-Resource-Activity					PDU-Session-Resource-Activity,

iE-Extensions	ProtocolExtensionContainer 		{ { PDU-Session-Resource-Activity-ItemExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Confirm-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Confirm-Modified-Item

PDU-Session-Resource-Confirm-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

dRB-Confirm-Modified-List-NG-RAN		DRB-Confirm-Modified-List-NG-RAN	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Confirm-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Confirm-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Item

PDU-Session-Resource-Failed-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Mod-Item

PDU-Session-Resource-Failed-Mod-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-To-Modify-Item

PDU-Session-Resource-Failed-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Modified-Item

PDU-Session-Resource-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

securityResult							SecurityResult							OPTIONAL,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN					OPTIONAL,

dRB-Failed-List-NG-RAN				DRB-Failed-List-NG-RAN				OPTIONAL,

dRB-Modified-List-NG-RAN				DRB-Modified-List-NG-RAN				OPTIONAL,

dRB-Failed-To-Modify-List-NG-RAN		DRB-Failed-To-Modify-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Required-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Required-To-Modify-Item

PDU-Session-Resource-Required-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

dRB-Required-To-Modify-List-NG-RAN		DRB-Required-To-Modify-List-NG-RAN		OPTIONAL,

dRB-Required-To-Remove-List-NG-RAN				DRB-Required-To-Remove-List-NG-RAN				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Required-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Required-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Item

PDU-Session-Resource-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

securityResult							SecurityResult				OPTIONAL,

nG-DL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

nG-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN,

dRB-Failed-List-NG-RAN					DRB-Failed-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Data-Forwarding-Response-E-RAB-List		CRITICALITY ignore	EXTENSION Data-Forwarding- Response-E-RAB-List		PRESENCE optional 	}

...

}

PDU-Session-Resource-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Mod-Item

PDU-Session-Resource-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID											PDU-Session-ID,

securityResult											SecurityResult								OPTIONAL,

nG-DL-UP-TNL-Information								UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-Mod-List-NG-RAN								DRB-Setup-Mod-List-NG-RAN,

dRB-Failed-Mod-List-NG-RAN								DRB-Failed-Mod-List-NG-RAN					OPTIONAL,

iE-Extensions											ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Data-Forwarding-Response-E-RAB-List		CRITICALITY ignore	EXTENSION Data-Forwarding-Response-E-RAB-List		PRESENCE optional 	}

...

}

PDU-Session-Resource-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Modify-Item

PDU-Session-Resource-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

securityIndication									SecurityIndication						OPTIONAL,

pDU-Session-Resource-DL-AMBR						BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information						OPTIONAL,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Data-Forwarding-Information	Data-Forwarding-Information	OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

networkInstance										NetworkInstance							OPTIONAL,

dRB-To-Setup-List-NG-RAN							DRB-To-Setup-List-NG-RAN				OPTIONAL,

dRB-To-Modify-List-NG-RAN							DRB-To-Modify-List-NG-RAN				OPTIONAL,

dRB-To-Remove-List-NG-RAN					DRB-To-Remove-List-NG-RAN		OPTIONAL,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SNSSAI		CRITICALITY reject	EXTENSION SNSSAI		PRESENCE optional}|

{ ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

{ID id-Data-Forwarding-Request-E-RAB-List		CRITICALITY ignore	EXTENSION Data-Forwarding- Request-E-RAB-List		PRESENCE optional 	},

...

}

PDU-Session-Resource-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Remove-Item

PDU-Session-Resource-To-Remove-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Remove-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Remove-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Cause		CRITICALITY ignore	EXTENSION Cause		PRESENCE optional},

...

}

PDU-Session-Resource-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Item

PDU-Session-Resource-To-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

pDU-Session-Type						PDU-Session-Type,

sNSSAI									SNSSAI,

securityIndication						SecurityIndication,

pDU-Session-Resource-DL-AMBR			BitRate						OPTIONAL,

nG-UL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer			Inactivity-Timer	OPTIONAL,

existing-Allocated-NG-DL-UP-TNL-Info	UP-TNL-Information		OPTIONAL,

networkInstance							NetworkInstance		OPTIONAL,

dRB-To-Setup-List-NG-RAN				DRB-To-Setup-List-NG-RAN,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

{ID id-Data-Forwarding-Request-E-RAB-List		CRITICALITY ignore	EXTENSION Data-Forwarding- Request-E-RAB-List		PRESENCE optional 	},

...

}

PDU-Session-Resource-To-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Mod-Item

PDU-Session-Resource-To-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

pDU-Session-Type									PDU-Session-Type,

sNSSAI												SNSSAI,

securityIndication									SecurityIndication,

pDU-Session-Resource-AMBR							BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

dRB-To-Setup-Mod-List-NG-RAN						DRB-To-Setup-Mod-List-NG-RAN,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-NetworkInstance		CRITICALITY ignore	EXTENSION NetworkInstance		PRESENCE optional}|

{ID id-CommonNetworkInstance	CRITICALITY ignore	EXTENSION CommonNetworkInstance	PRESENCE optional}|

{ID id-Data-Forwarding-Request-E-RAB-List		CRITICALITY ignore	EXTENSION Data-Forwarding- Request-E-RAB-List		PRESENCE optional 	},

...

}

PDU-Session-To-Notify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-To-Notify-Item

PDU-Session-To-Notify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

qoS-Flow-List							QoS-Flow-List,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-To-Notify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-To-Notify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Type ::= ENUMERATED {

ipv4,

ipv6,

ipv4v6,

ethernet,

unstructured,

...

}

PLMN-Identity ::= OCTET STRING (SIZE(3))

PortNumber ::= BIT STRING (SIZE(16))

PPI ::= INTEGER (0..7, ...)

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

#### -- Q

QCI ::= INTEGER (0..255)

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				Non-Dynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer	{{QoS-Characteristics-ExtIEs}}

}

QoS-Characteristics-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

QoS-Flow-Identifier	::=	INTEGER (0..63)

QoS-Flow-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Item

QoS-Flow-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoS-Flow-Mapping-Indication		PRESENCE optional},

...

}

QoS-Flow-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Failed-Item

QoS-Flow-Failed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Mapping-Item

QoS-Flow-Mapping-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowMappingIndication						QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Mapping-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Mapping-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-Indication ::= ENUMERATED {ul, dl, ...}

QoS-Parameters-Support-List	::= SEQUENCE {

eUTRAN-QoS-Support-List			EUTRAN-QoS-Support-List			OPTIONAL,

nG-RAN-QoS-Support-List			NG-RAN-QoS-Support-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { QoS-Parameters-Support-List-ItemExtIEs} } OPTIONAL,

...

}

QoS-Parameters-Support-List-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSPriorityLevel ::= INTEGER (0..127, ...)

QoS-Flow-QoS-Parameter-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-QoS-Parameter-Item

QoS-Flow-QoS-Parameter-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-QoS-Parameter-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-QoS-Parameter-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-E-RABID ignore		CRITICALITY ignore	EXTENSION E-RAB-ID	PRESENCE optional},	...

}

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QosMonitoringRequest	CRITICALITY ignore	EXTENSION QosMonitoringRequest	PRESENCE optional},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QoS-Flow-Removed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoS-Flow-Released-In-Session			ENUMERATED {released-in-session, not-released-in-session, ...}			OPTIONAL,

qoS-Flow-Accumulated-Session-Time		OCTET STRING (SIZE(5))														OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flow-Removed-Item-ExtIEs } }		OPTIONAL,

...

}

QoS-Flow-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info								ProtocolIE-ID ::= 86

id-QoSMonitoringRequest										ProtocolIE-ID ::= 87

id-E-RABID													ProtocolIE-ID ::= xx

id-Data-Forwarding-Request-E-RAB-List						ProtocolIE-ID ::= yy

id-Data-Forwarding-Response-E-RAB-List						ProtocolIE-ID ::= zz

END

-- ASN1STOP

////////////////////////////////////////////////////////////////////////end of change////////////////////////////////////////////////////////////////////////