3GPP TSG-RAN3 Meeting #109-e	R3-205209

E-meeting, 17 - 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0459 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Non-Monotonic Alternative QoS Parameter values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN 3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL, eV2XARC |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.501 clause 5.7.1.2a documents that: There is no requirement that the GFBR monotonically decreases, nor that the PDB or PER monotonically increase as the Alternative QoS Profiles become less preferred. This is important information for RAN implementations and should be captured in TS 38.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PDU Session Setup and Modification procedures are updated to include this information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN design may be incompatible with HPLMN Core Network/Application Function implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | ? | Other core specifications |  |  |  | ………….Xn-AP, F1-AP? |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node.

The PDU SESSION RESOURCE SETUP REQUEST message shall contain the information required by the NG-RAN node to setup the PDU session related NG-RAN configuration consisting of at least one PDU session resource and include each PDU session resource to setup in the PDU Session Resource Setup Request List IE.

Upon reception of the PDU SESSION RESOURCE SETUP REQUEST message, if resources are available for the requested configuration, the NG-RAN node shall execute the requested NG-RAN configuration and allocate associated resources over NG and over Uu for each PDU session listed in the PDU Session Resource Setup Request List IE.

If the RAN Paging Priority IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

For each requested PDU session, if resources are available for the requested configuration, the NG-RAN node shall establish at least one DRB and associate each accepted QoS flow of the PDU session to a DRB established.

For each PDU session successfully established the NG-RAN node shall pass to the UE the PDU Session NAS-PDU IE, if included, and the value contained in the PDU Session ID IE received for the PDU session. The NG-RAN node shall not send to the UE the PDU Session NAS PDUs associated to the failed PDU sessions.

If the NAS-PDU IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall pass it to the UE.

For each PDU session the NG-RAN node shall store the UL NG-U UP TNL Information IE included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and use it as the uplink termination point for the user plane data for this PDU session.

For each PDU session, if the Additional UL NG-U UP TNL Information IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may allocate for this split PDU session resources for an additional NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Setup Request List IE and it shall indicate these QoS flows in the Additional DL QoS Flow per TNL Information IE in the PDU Session Resource Setup Response Transfer IE. In case the Additional DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional UL NG-U UP TNL information as available again.

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and the Common Network Instance IE is not present, the NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [9].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [9].

For each PDU session, if the Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for this PDU session for the redundant transmission and it shall include the Redundant QoS Flow per TNL Information IE in the PDU Session Resource Setup Response Transfer IE as described in TS 23.501 [9].

For each PDU session, if the Additional Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may allocate for this split PDU session resources for an additional redundant NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Setup Request List IE and it shall indicate these QoS flows in the Additional Redundant DL QoS Flow per TNL Information IE in the PDU Session Resource Setup Response Transfer IE. In case the Additional Redundant DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional Redundant UL NG-U UP TNL information as available again.

For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [9].

For each PDU session, if the TSC Traffic Characteristics IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, store it and use it as specified in TS 23.501 [9].

For each PDU session, if the PDU Session Type IE included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message is set to "ethernet", the NG-RAN node may perform appropriate header compression for the concerned PDU session, or if it is set to "unstructured", the NG-RAN node shall not perform header compression for the concerned PDU session.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the NG-RAN node shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU session. If the NG-RAN node cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU session resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required".

For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message.

For each PDU session for which the Maximum Integrity Protected Data Rate Downlink IE or the Maximum Integrity Protected Data Rate Uplink IE are included in the Security Indication IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store the respective information and, if integrity protection is to be performed for the PDU session, it shall enforce the traffic limits corresponding to the received values, for the concerned PDU session and concerned UE, as specified in TS 23.501 [9].

For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the NG-RAN node shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the NG-RAN node shall not perform user plane ciphering for the concerned PDU session.

For each PDU session for which the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store the received value in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9].

For each PDU session in the PDU SESSION RESOURCE SETUP REQUEST message, if the Additional QoS Flow Information IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may consider it for the DRB allocation process. It is up to NG-RAN node implementation to decide whether and how to use it.

For each PDU session in the PDU SESSION RESOURCE SETUP REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may accept the setup of the QoS flow when notification control has been enabled if the requested QoS parameters or at least one of the alternative QoS parameters sets can be fulfilled at the time of setup. In case the NG-RAN node accepts the setup fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it currently fulfils in the Current QoS Parameters Set Index IE within the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message. The Guaranteed Flow Bit Rate(s) need not monotonically decrease, and the Packet Delay Budget or Packet Error Rate need not monotonically increase as the Alternative QoS Profiles become less preferred in the Alternative QoS Parameters Set List IE.

For each QoS flow which has been successfully established, the NG-RAN node shall store the Redundant QoS Flow Indicator IE if included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and consider it for the redundant transmission as specified in TS 23.501 [9].

For each QoS flow which has been successfully established, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [9].

For each QoS flow requested to be setup the NG-RAN node shall take into account the received QoS Flow Level QoS Parameters IE. For each QoS flow the NG-RAN node shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows:

-	The NG-RAN node shall consider the priority level of the requested QoS flow, when deciding on the resource allocation.

-	The priority levels and the pre-emption indicators may (individually or in combination) be used to determine whether the QoS flow setup has to be performed unconditionally and immediately. If the requested QoS flow is marked as "may trigger pre-emption" and the resource situation requires so, the NG-RAN node may trigger the pre-emption procedure which may then cause the forced release of a lower priority QoS flow which is marked as "pre-emptable". Whilst the process and the extent of the pre-emption procedure are operator-dependent, the pre-emption indicators shall be treated as follows:

1.	The values of the last received Pre-emption Vulnerability IE and Priority Level IE shall prevail.

2.	If the Pre-emption Capability IE is set to "may trigger pre-emption", then this allocation request may trigger the pre-emption procedure.

3.	If the Pre-emption Capability IE is set to "shall not trigger pre-emption", then this allocation request shall not trigger the pre-emption procedure.

4.	If the Pre-emption Vulnerability IE is set to "pre-emptable", then this QoS flow shall be included in the pre-emption process.

5.	If the Pre-emption Vulnerability IE is set to "not pre-emptable", then this QoS flow shall not be included in the pre-emption process.

-	The NG-RAN node pre-emption process shall keep the following rules:

1.	The NG-RAN node shall only pre-empt QoS flows with lower priority, in ascending order of priority.

2.	The pre-emption may be done for QoS flows belonging to the same UE or to other UEs.

For each QoS flow which has been successfully established, the NG-RAN node shall store the mapped E-RAB ID if included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and use it as specified in TS 38.300 [8].

For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, store the received information in the UE context and setup the redundant user plane for the redundant PDU session as specified in TS38.300 [8] and TS 23.501 [9]. If the PDU Session Type IE is set to “ethernet” and the redundancy requirement is fulfilled using a secondary NG-RAN node, the NG-RAN node shall, if supported, include the Global RAN Node ID of Secondary NG-RAN Node IE in the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message.

The NG-RAN node shall report to the AMF in the PDU SESSION RESOURCE SETUP RESPONSE message the result for each PDU session resource requested to be setup:

-	For each PDU session resource successfully setup, the PDU Session Resource Setup Response Transfer IE shall be included containing:

1.	The NG-U UP transport layer information to be used for the PDU session and associated list of QoS flows which have been successfully established, in the QoS Flow per TNL Information IE.

2.	The list of QoS flows which failed to be established, if any, in the QoS Flow Failed to Setup List IE. When the NG-RAN node reports unsuccessful establishment of a QoS flow, the cause value should be precise enough to enable the SMF to know the reason for the unsuccessful establishment.

-	For each PDU session resource which failed to be setup, the PDU Session Resource Setup Unsuccessful Transfer IE shall be included containing a cause value that should be precise enough to enable the SMF to know the reason for the unsuccessful establishment.

Upon reception of the PDU SESSION RESOURCE SETUP RESPONSE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Setup Response Transfer IE or PDU Session Resource Setup Unsuccessful Transfer IE to the SMF associated with the concerned PDU session.

Upon reception of the PDU SESSION RESOURCE SETUP REQUEST message to setup a QoS flow for IMS voice, if the NG-RAN node is not able to support IMS voice, the NG-RAN node shall initiate EPS fallback or RAT fallback for IMS voice procedure as specified in TS 23.501 [9] and report unsuccessful establishment of the QoS flow in the PDU Session Resource Setup Response Transfer IE or in the PDU Session Resource Setup Unsuccessful Transfer IE with cause value "IMS voice EPS fallback or RAT fallback triggered".

For each PDU session for which the Global RAN Node ID of Secondary NG-RAN Node IE is included in the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message, the SMF shall, if supported, handle this information as specified in TS 23.501 [9].

The UE Aggregate Maximum Bit Rate IE should be sent to the NG-RAN node if the AMF has not sent it previously. If it is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9].

Interactions with Handover Preparation procedure:

If a handover becomes necessary during the PDU Session Resource Setup procedure, the NG-RAN node may interrupt the ongoing PDU Session Resource Setup procedure and initiate the Handover Preparation procedure as follows:

1.	The NG-RAN node shall send the PDU SESSION RESOURCE SETUP RESPONSE message in which the NG-RAN node shall indicate, if necessary, all the PDU session resources which failed to be setup with an appropriate cause value, e.g. "NG intra-system handover triggered", "NG inter-system handover triggered" or "Xn handover triggered".

2.	The NG-RAN node shall trigger the handover procedure.

#### **************** next changes *******************************

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE MODIFY REQUEST message to the NG-RAN node.

The PDU SESSION RESOURCE MODIFY REQUEST message shall contain the information required by the NG-RAN node, which may trigger the NG-RAN configuration modification for the existing PDU sessions listed in the PDU Session Resource Modify Request List IE.

Upon reception of the PDU SESSION RESOURCE MODIFY REQUEST message, if the NG-RAN configuration is triggered to be modified and if resources are available for the modified NG-RAN configuration, the NG-RAN node shall execute the configuration modification for the requested PDU session.

If the RAN Paging Priority IE is included in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

For each PDU session, if the S-NSSAI IE is included in the PDU Session Resource Modify Request Item IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall replace the previously provided S-NSSAI by the received S-NSSAI for the concerned PDU session and use it as specified in TS 23.502 [10].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message and the Common Network Instance IE is not present, the NG-RAN node shall, if supported, use it as specified in TS 23.501 [9].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, use it as specified in TS 23.501 [9].

For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, use it for the redundant transmission as specified in TS 23.501 [9].

For each PDU session, if the TSC Traffic Characteristics IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, store it and use it as specified in TS 23.501 [9].

For each PDU session, if the Redundant QoS Flow Indicator IE is included and set to “false” for all QoS flows, the NG-RAN node shall, if supported, stop the redundant transmission and release the redundant tunnel for the concerned PDU session as specified in TS 23.501 [9].

For each PDU session in the PDU SESSION RESOURCE MODIFY REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resource Modify Request Transfer IE of the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node may accept the setup of the QoS flow when notification control has been enabled if the requested QoS parameters or at least one of the alternative QoS parameters sets can be fulfilled at the time of setup. In case the NG-RAN node accepts the setup fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it currently fulfils in the Current QoS Parameters Set Index IE within the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE MODIFY RESPONSE message. The Guaranteed Flow Bit Rate(s) need not monotonically decrease, and the Packet Delay Budget or Packet Error Rate need not monotonically increase as the Alternative QoS Profiles become less preferred in the Alternative QoS Parameters Set List IE.

For each PDU session included in the PDU Session Resource Modify Request List IE:

-	For each QoS flow included in the QoS Flow Add or Modify Request List IE, based on the QoS Flow Level QoS Parameters IE, the NG-RAN node may establish, modify or release the DRB configuration and may change allocation of resources on NG or Uu accordingly. The NG-RAN node shall associate each QoS flow accepted to setup or modify with a DRB of the PDU session. The associated DRB for the QoS flow accepted to modify may not change.

-	For each QoS flow, if the Redundant QoS Flow Indicator IE is included, the NG-RAN node shall, if supported, store it and consider it for the redundant transmission as specified in TS 23.501 [9].

-	For each QoS flow included in the QoS Flow Add or Modify Request List IE, if the QoS Flow Add or Modify Request Item IE is included for an existing QoS Flow Identifier IE, the NG-RAN node shall overwrite the content of the full QoS Flow Add or Modify Request Item IE.

-	For each QoS flow included in the QoS Flow to Release List IE, the NG-RAN node shall de-associate the QoS flow with the previously associated DRB.

-	The NG-RAN node shall pass the NAS-PDU IE received for the PDU session to the UE when modifying the PDU session configuration. The NG-RAN node does not send the NAS PDUs associated to the failed PDU sessions to the UE.

-	The NG-RAN node may change allocation of resources on NG according to the requested target configuration.

-	If the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall store and use the received PDU Session Aggregate Maximum Bit Rate value when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9].

-	If the UL NG-U UP TNL Modify List IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall update the transport layer information for the uplink data accordingly for the concerned transport bearers identified by the DL NG-U UP TNL Information IE included in the PDU Session Resource Modify Request Transfer IE for the concerned PDU session.

-	If the Additional UL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node may allocate resources for an additional NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Add or Modify Request List IE and it shall indicate these QoS flows in the Additional DL QoS Flow per TNL Information IE in the PDU Session Resource Modify Response Transfer IE. In case the Additional DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional UL NG-U UP TNL information as available again.

-	In case more than one NG-U transport bearers have been set up for the PDU session, if all the QoS flows associated to one existing NG-U transport bearer are included in the QoS Flow to Release List IE in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node and 5GC shall consider that the concerned NG-U transport bearer is removed for the PDU session, and both NG-RAN node and 5GC shall therefore consider the related NG-U UP TNL information as available again.

-	If the Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall update the transport layer information for the uplink data accordingly for the concerned transport bearer identified by the Redundant DL NG-U UP TNL Information IE included in the PDU Session Resource Modify Request Transfer IE for the concerned PDU session.

-	If the Additional Redundant UL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node may allocate resources for an additional redundant NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Add or Modify Request List IE and it shall indicate these QoS flows in the Additional Redundant DL QoS Flow per TNL Information IE in the PDU Session Resource Modify Response Transfer IE. In case the Additional Redundant DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional Redundant UL NG-U UP TNL information as available again.

For each QoS flow which has been successfully added or modified, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [9].

The NG-RAN node shall report to the AMF, in the PDU SESSION RESOURCE MODIFY RESPONSE message, the result for each PDU session requested to be modified listed in the PDU SESSION RESOURCE MODIFY REQUEST message:

-	For each PDU session which is successfully modified, the PDU Session Resource Modify Response Transfer IE shall be included containing:

1.	The list of QoS flows which have been successfully setup or modified, if any, in the QoS Flow Add or Modify Response List IE in case the PDU Session Resource Modify procedure is triggered by QoS flow setup or modification.

2.	The list of QoS flows which have failed to be setup or modified, if any, in the QoS Flow Failed to Add or Modify List IE in case the PDU Session Resource Modify procedure is triggered by QoS flow setup or modification.

-	For each PDU session which failed to be modified, the PDU Session Resource Modify Unsuccessful Transfer IE shall be included containing the failure cause.

-	For each PDU session, if the DL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Response Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message, it shall be considered by the SMF as the new DL transport layer address for the PDU session. The NG-RAN also may indicate the mapping between each new DL transport layer address and the corresponding UL transport layer address assigned by the 5GC.

-	For each PDU session, if the Additional NG-U UP TNL Information IE is included in the PDU Session Resource Modify Response Transfer IE in the PDU SESSION RESOURCE MODIFY RESPONSE message, it shall be considered by the SMF as the new DL transport layer address(es) for the PDU session. The NG-RAN also may indicate the mapping between each new DL transport layer address and the corresponding UL transport layer address assigned by the 5GC.

Upon reception of the PDU SESSION RESOURCE MODIFY RESPONSE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Modify Response Transfer IE or PDU Session Resource Modify Unsuccessful Transfer IE to each SMF associated with the concerned PDU session.

The NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE MODIFY RESPONSE message location information of the UE in the User Location Information IE.

For a PDU session or a QoS flow which failed to be modified, the NG-RAN node shall fall back to the configuration of the PDU session or the QoS flow as it was configured prior to the reception of the PDU SESSION RESOURCE MODIFY REQUEST message.

Upon reception of the PDU SESSION RESOURCE MODIFY REQUEST message to setup a QoS flow for IMS voice, if the NG-RAN node is not able to support IMS voice, the NG-RAN node shall initiate EPS fallback or RAT fallback for IMS voice procedure as specified in TS 23.501 [9] and report unsuccessful establishment of the QoS flow in the PDU Session Resource Modify Response Transfer IE or in the PDU Session Resource Modify Unsuccessful Transfer IE with cause value "IMS voice EPS fallback or RAT fallback triggered".

If the User Location Information IE is included in the PDU SESSION RESOURCE MODIFY RESPONSE message, the AMF shall handle this information as specified in TS 23.501 [9].

Interactions with Handover Preparation procedure:

If a handover becomes necessary during the PDU Session Resource Modify procedure, the NG-RAN node may interrupt the ongoing PDU Session Resource Modify procedure and initiate the Handover Preparation procedure as follows:

1.	The NG-RAN node shall send the PDU SESSION RESOURCE MODIFY RESPONSE message in which the NG-RAN node shall indicate, if necessary, all the PDU sessions failed with an appropriate cause value, e.g. "NG intra-system handover triggered", "NG inter-system handover triggered" or "Xn handover triggered".

2.	The NG-RAN node shall trigger the handover procedure.

************* end of changes ******************