3GPP TSG-RAN WG3 #109-e	[draft]R3-205585

Online, 17-27 Aug 2020

Agenda Item:	9.3.2

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on the Overlapping Band Handling over F1

Document for:	Approval

# Introduction

CB: # 64_OverlappingBandsF1

- Scenario: single connectivity, UE connected to a cell belonging to overlapping bands; whether network should change band selected by UE? Which procedure to use?

- common understanding that SpCell or SCell selected by the gNB-CU cannot be changed by the gNB-DU; which node selects the band?

- Need to notify CU of the band selected by the DU?

- whether band list also applies to non-DC scenarios?

- possible compromise: Rel-16? Rel-17?

(ZTE - moderator)

Summary of offline disc R3-205585

# For the Chairman’s Notes

Propose the following:

# Discussion

After online discussion, this CB mainly focuses on the scenarios and solutions.

- Scenario1: Single connectivity, PCell supports multiple overlapping bands

As described in [6], there is a cell with multiple overlapping bands, let’s assume 2 overlapping bands, which overlap in full. A typical example of cells that can adopt such configuration is where band n2 and n25 are used at the same time in the same cell. Also, let’s assume there are UEs that can access both such bands. A UE selects the first band in the frequencyBandList (broadcast in SIB1) that it supports, according to supportedBandListNR reported in UE-NR-Capability.

What RAN2 was discussed before is that if RAN side decides to perform DC for this UE, while the current serving band selected by UE does not support DC, therefore, intra-Cell HO will be triggered.

It seems that there is no other case that band-switch of PCell is necessary.

Meanwhile, some companies think that band selection is some kind of PHY resource allocation, which is allowed to be performed in the gNB-DU side.

Possible solutions:

gNB-CU performs the band selection:

- The gNB-CU konws the characteristic of bands, and usually the DC operation decision is made on the gNB-CU.

The gNB-CU should trigger RRC reconfiguration towards the UE, and a UE Context Modification procedure towards the gNB-DU in order to provide the correct frequency band in CellGroupConfig IE (see below) and measurement configuration for gap configuration purposes (if MeasConfig IE has changed and included) .

TS38.331:

CellGroupConfig->spCellConfig->reconfigurationWithSync ->spCellConfigCommon(ServingCellConfigCommon)->downlinkConfigCommon->frequencyInfoDL->frequencyBandList

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

gNB-DU performs the band selection:

- gNB-DU selects the best band combination according to the UE capability and informs gNB-CU of the chosen bands via a new IE describing the band selection for each serving cell performed by the gNB-DU for the UE in DU to CU RRC Information IE in UE Context Setup Response.

The gNB-CU should trigger RRC reconfiguration towards the UE and a UE Context Modification Request towards the gNB-DU in order to provide the correct frequency band in CellGroupConfig IE and measurement configuration for gap configuration purposes (if MeasConfig IE has changed and included) .

The details can be seen in [7][8], adding the new Band Selection IE in the DU to CU RRC Information over F1.

#### 9.3.1.x	Band Selection

The Band Selection indicates the band per cell as chosen by the gNB-DU.

| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | - | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| FreqBandIndicatorNR | O |  | INTEGER (1,..1024,…) |  | - |  |
| SCell Band Selection List |  | 0..1 |  |  | YES | ignore |
| >SCell Band Selection Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>FreqBandIndicatorNR | O |  | INTEGER (1,..1024,…) |  | - |  |

- The gNB-DU selects the best band combination according to the UE capability and informs gNB-CU of the chosen bands via the requested BandCombinationIndex in DU to CU RRC Information IE in UE Context Setup Response.

The gNB-CU should trigger RRC reconfiguration towards the UE and a UE Context Modification Request towards the gNB-DU in order to provide the correct frequency band in CellGroupConfig IE and measurement configuration for gap configuration purposes (if MeasConfig IE has changed and included) .

The BandCombinationIndex means the position of a band combination in the supportedBandCombinationList IE supported by the UE, while after double check with RAN2, only if all the single bands supported by Cell are listed in the supportedBandCombinationList IE, then the BandCombinationIndex IE can be reused to indicate a single band.

Considering that PCell band switch is an unusual case, SlutionC) is acceptable with above limitation usage.

Extend the scope of IE Selected BandCombinationIndex to include single PCell scenario is needed, see below:

| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For SA, (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Question 1: Do companies think the Scenario1) is valid?

| Company | Comment |
| --- | --- |
| ZTE | For PCell band switch case, it seems that CU selection is enough, then SolutionA) is enough which has already been supported in current spec. If majority companies are prefer to let DU perform the band selection, then SolutionC) is acceptable for us with minimum standard impact. |
| CATT | The scenario seems reasonable. As to which node decides the band switch, for MR-DC, we have agreed that it is DU which decides the band combination,we think it is reasonable that it also applied to CA and non-CA case for SA scenario. Then for solutions, we think current Selected BandCombinationIndex IE could resolve the issue completely. It is ever doubted that this information only applied to DC and CA scenario based on what described in 38.331.However,it is an error in 38.331.The common understanding in RAN2 is that it would also applied to single cell.Now,there is an email discussion ongoing in RAN2 which tried to fix it(refer to RAN2 discussion Summary of offline 010 Rel-15 UE cap Clarifications).All companies agree that the error in 38.331 should be fixed. With that, we think the problem could be resolved. |

Question 2:Is this Scenario1 and the F1 solution for R15 and R16, or only R16? or leave it for R17?

| Company | Comment |
| --- | --- |
| ZTE | SolutionA) has no impact on spec. While for SolutionC), CRs for both R15 and R16 are preferred, while only R16 CR is also acceptable. |
| CATT | Solution C is preferred and it is OK to have both Rel-15/Rel-16 CR |

- Scenario2: In CA and DC cases,  the UE may support different band combinations with overlapping bands, and the network shall select the band combination with highest throughput to provide the best performance for the UE.

In this case, the gNB-DU shall transmit the newly selected band according to the UE capability in form of band combination to the gNB-CU. It allows the gNB-CU to have a proper mapping of measurement objects and frequency bands to which the measurement object is associated.

For DC case, it has already been supported in current spec with the Selected BandCombinationIndex IE in DU to CU RRC Information over F1, see below:

| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

However, the semantic description of this IE does not cover the CA case yet. Extend the scope of IE Selected BandCombinationIndex to include CA scenario is needed, and the corresponding CRs are provided in [2][3] as below:

| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For NR CA, (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Question 3: Do companies think the CA scenario is valid?

| Company | Comment |
| --- | --- |
| ZTE | Yes, it helps to make the CA function complete in the CU-DU split case. |
| CATT | Yes |

Question 4: Is this Scenario2 and the F1 correction for R15 and R16, or only R16? or leave it for R17?

| Company | Comment |
| --- | --- |
| ZTE | Considering that it is a small correction, CRs for both R15 and R16 are preferred, while only R16 CR is also acceptable. |
| CATT | Similar view as ZTE. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-204893 Discussion on Overlapping Band Handling over F1 (ZTE)

- R3-204894 Correction on Overlapping Band Handling over F1 (ZTE) CR0631r, TS 38.473 v15.10.0, Rel-15, Cat. F

- R3-205533 Correction on Overlapping Band Handling over F1 (ZTE) CR0632r, TS 38.473 v16.2.0, Rel-16, Cat. A

- R3-205274 Further discussions on overlapping band handling (Huawei)

- R3-204874 Discussion on Band and Cell Selection at DU (Nokia, Nokia Shanghai Bell)

- R3-204776 Overlapping bands handling (Ericsson)

- R3-204777 Overlapping band handling CR 38.473 (Ericsson) CR0584r1, TS 38.473 v15.10.0, Rel-15, Cat. F

- R3-204778 Overlapping band handling CR 38.473 (Ericsson) CR0585r1, TS 38.473 v16.2.0, Rel-16, Cat. A

- R3-205465 Response to R3-205274 and R3-204874 (Ericsson)