3GPP TSG-RAN WG3 #109-e																		R3-205603

17-28 August 2020 Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0137 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCTP association change when current SCTP association is failed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telcom, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification only describes how the TNL binding update is performed for F1 interface and E1 interface, but not for Xn interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description for TNL binding update for Xn interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear behaviour on Xn TNL binding update, which may cause IOT issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Updated … |  |  |  |  |  |  |  |  |

Start of Change

## 11.1	Multiple TNLAs for Xn-C

In the following, the procedure for managing multiple TNLAs for Xn-C is described.

Figure 11.1-1: Managing multiple TNLAs for Xn-C.

1.	The NG-RAN node1 establishes the first TNLA with the NG-RAN node2 using a configured TNL address.

NOTE: 	The NG-RAN node1 may use different source and/or destination IP end point(s) if the TNL establishment towards one IP end point fails. How the NG-RAN node1 gets the remote IP end point(s) and its own IP address are outside the scope of this specification.

2-3.	Once the TNLA has been established, the NG-RAN node1 initiates the Xn Setup procedure to exchange application level configuration data

4-6.	The NG-RAN node2 may add additional TNL Endpoint(s) to be used for Xn-C signalling between the NG-RAN node1 and the NG-RAN node2 pair using the NG-RAN node Configuration Update procedure. NG-RAN node Configuration Update procedure also allows the NG-RAN node2 to request the NG-RAN node1 to modify or release TNLA(s).

7-9.	The NG-RAN node1 may add additional TNL Endpoint(s) to be used for Xn-C signalling between the NG-RAN node1 and the NG-RAN node2 pair using the NG-RAN node Configuration Update procedure. NG-RAN node Configuration Update procedure also allows the NG-RAN node1 to request the NG-RAN node2 to modify or release TNLA(s).

The XnAP UE TNLA binding is a binding between a XnAP UE association and a specific TNL association for a given UE. After the XnAP UE TNLA binding is created, the NG-RAN node1 or the NG-RAN node2 can update the UE TNLA binding by sending the XnAP message for the UE to the peer NG-RAN node via a different TNLA. The peer NG-RAN node shall update the XnAP UE TNLA binding with the new TNLA.

End of Change