| TSG-RAN Working Group 3 Meeting #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | R3-000965 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sphia Antipolis, France, 28 Feb - 3 Mar 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 25.423 |  |  |  |  |  | CR |  |  | 16r2 |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  | 3.0.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | TSG RAN #7 |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  | X | Core Network |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | RAN3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 29 February 2000 |  |  |  |  |  |  |
| Subject: |  |  | Rearrangement of Neighbouring Cell Information group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |
| Reason for change: |  |  | Currently the Neighbouring Cell Information contains some redundant contents. This CR proposes to rearrange the Neighbouring Cell Information group in order to reduce the number of octet occupied by this group. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 9.1.4 RADIO LINK SETUP RESPONSE 9.1.5 RADIO LINK SETUP FAILURE 9.1.7 RADIO LINK ADDITION RESPONSE 9.1.8 RADIO LINK ADDITION FAILURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 9.1.4	RADIO LINK SETUP RESPONSE

#### 9.1.4.1	FDD Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |
| Transaction ID | M |  |  |  |
| D-RNTI | O |  |  |  |
| CN PS Domain Identifier | O |  |  |  |
| CN CS Domain Identifier | O |  |  |  |
| RL Information Response |  | 1..<maxnoofRLs> |  |  |
| RL ID | M |  |  |  |
| SAI | M |  |  |  |
| UL Interference Level | M |  |  |  |
| DL Code Information |  | 1.. <maxnoofDLCodes |  |  |
| DL Scrambling Code | M |  |  |  |
| FDD DL Channelisation Code Number | M |  |  |  |
| Diversity Indication | C-NotFirstRL |  |  |  |
| CHOICE diversity Indication |  |  |  |  |
| Combining |  |  |  |  |
| RL ID | M |  |  | Reference RL ID for the combining |
| Non Combining or IE not present |  |  |  | "IE not present" is equivalent to "First RL". |
| DCH Information Response |  | 0..<maxnoofDCHs> |  | Only one DCH per set of co-ordinated DCHs shall be included |
| DCH ID | M |  |  |  |
| Binding ID | M |  |  |  |
| Transport Layer Address | M |  |  |  |
| SSDT Support Indicator | M |  |  |  |
| Maximum Uplink Eb/No | M |  | Uplink Eb/No |  |
| Minimum Uplink Eb/No | M |  | Uplink Eb/No |  |
| Neighbouring Cell Information |  | 0..<maxnoofneighbouringRNCs> |  |  |
| RNC-Id | M |  |  |  |
| CN PS Domain Identifier | O |  |  |  |
| CN CS Domain Identifier | O |  |  |  |
| Per FDD Cell Information |  | 0..<maxnoofFDDneighbours> |  |  |
| C-Id | M |  |  |  |
| UARFCN | M |  |  |  |
| Frame Offset | O |  |  |  |
| Primary Scrambling Code | M |  |  |  |
| Primary CPICH Power | O |  |  |  |
| Per TDD Cell Information |  | 0..<maxnoofTDDneighbours> |  |  |
| C-Id | M |  |  |  |
| UARFCN | M |  |  |  |
| Frame Offset | O |  |  |  |
| Cell Parameter ID | M |  |  |  |
| Sync Case | M |  |  |  |
| Time Slot | C-Case1 |  |  |  |
| PSCH Time Slot | C-Case2&3 |  |  |  |
| Uplink Eb/No Target | O |  | Uplink Eb/No |  |
| Downlink Eb/No Target | O |  |  |  |
| Criticality Diagnostics | O |  |  |  |

| Condition | Explanation |
| --- | --- |
| IfComb | This IE is present if the 'Diversity Indication' IE indicates 'combining' in the Node B. |
| IfNotComb | This IE is present if the 'Diversity Indication' IE indicates 'non combining' in the Node B. |
| NotFirstRL | The IE is present only if the RL is not the first RL in the RL Information |
| Case1 | This IE is present only if Sync Case = Case1. |
| Case2&3 | This IE is present only if Sync Case = Case2 or Case3. |

| Range bound | Explanation |
| --- | --- |
| MaxnoofRLs | Maximum no. of RLs for one UE. |
| MaxnoofDCHs | Maximum no. of DCHs for one UE. |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |
| MaxnoofFDDneighbours | Maximum number of neighbouring FDD cell for one cell. |
| MaxnoofTDDneighbours | Maximum number of neighbouring TDD cell for one cell. |

#### 9.1.4.2	TDD Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |
| Transaction ID | M |  |  |  |
| D-RNTI | O |  |  |  |
| CN PS Domain Identifier | O |  |  |  |
| CN CS Domain Identifier | O |  |  |  |
| RL Information Response |  | 1 |  |  |
| RL ID | M |  |  |  |
| SAI | M |  |  |  |
| UL Interference Level | M |  |  |  |
| Maximum Uplink Eb/No | M |  | Uplink Eb/No |  |
| Minimum Uplink Eb/No | M |  | Uplink Eb/No |  |
| Uplink Eb/No Target | O |  | Uplink Eb/No |  |
| Downlink Eb/No Target | O |  |  |  |
| UL CCTrCH Information |  | 1..<maxnoofCCTrCHs> |  |  |
| CCTrCH ID | M |  |  |  |
| UL DPCH Information |  | 1..<MaxnoofDPCHs> |  |  |
| DPCH ID | M |  |  |  |
| TDD Channelisation Code | M |  |  |  |
| Burst Type | M |  |  |  |
| Midamble Shift | M |  |  |  |
| Time Slot | M |  |  |  |
| TDD Physical Channel Offset | M |  |  |  |
| Repetition Period | M |  |  |  |
| Repetition Length | M |  |  |  |
| TFCI Presence | M |  |  |  |
| DL CCTrCH Information |  | 1..<maxnoofCCTrCHs> |  |  |
| CCTrCH ID | M |  |  |  |
| DL DPCH Information |  | 1..<MaxnoofDPCHs> |  |  |
| DPCH ID | M |  |  |  |
| TDD Channelisation Code | M |  |  |  |
| Burst Type | M |  |  |  |
| Midamble Shift | M |  |  |  |
| Time Slot | M |  |  |  |
| TDD Physical Channel Offset | M |  |  |  |
| Repetition Period | M |  |  |  |
| Repetition Length | M |  |  |  |
| TFCI Presence | M |  |  |  |
| DCH Information Response |  | 1..<maxnoofDCHs> |  | Only one DCH per set of co-ordinated DCHs shall be included. |
| DCH ID | M |  |  |  |
| Binding ID | M |  |  |  |
| Transport Layer Address | M |  |  |  |
| Neighbouring Cell Information | O | 0..<maxnoofneighbouringRNCs> |  |  |
| RNC-Id | M |  |  |  |
| CN PS Domain Identifier | O |  |  |  |
| CN CS Domain Identifier | O |  |  |  |
| Per FDD Cell Information |  | 0..<maxnoofFDDneighbours> |  |  |
| C-Id | M |  |  |  |
| UARFCN | M |  |  |  |
| Frame Offset | O |  |  |  |
| Primary Scrambling Code | M |  |  |  |
| Primary CPICH Power | O |  |  |  |
| Per TDD Cell Information |  | 0..<maxnoofTDDneighbours> |  |  |
| C-Id | M |  |  |  |
| UARFCN | M |  |  |  |
| Frame Offset | O |  |  |  |
| Cell Parameter ID | M |  |  |  |
| Sync Case | M |  |  |  |
| Time Slot | C-Case1 |  |  |  |
| PSCH Time Slot | C-Case2&3 |  |  |  |
| Criticality Diagnostics | O |  |  |  |

| Condition | Explanation |
| --- | --- |
| Case1 | This IE is present only if Sync Case = Case1. |
| Case2&3 | This IE is present only if Sync Case = Case2 or Case3. |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDPCHs | Maximum no. of DPCHs for one CCTrCH. |
| MaxnoofDCHs | Maximum no. of DCHs for one UE. |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |
| MaxnoofFDDneighbours | Maximum number of neighbouring FDD cell for one cell |
| MaxnoofTDDneighbours | Maximum number of neighbouring TDD cell for one cell |
| MaxnoofCCTrCHs | Maximum no. of CCTrCH for one UE. |

### 9.1.5	RADIO LINK SETUP FAILURE

#### 9.1.5.1	FDD Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |
| Transaction ID | M |  |  |  |
| D-RNTI | O |  |  |  |
| CN PS Domain Identifier | O |  |  |  |
| CN CS Domain Identifier | O |  |  |  |
| Unsuccessful RL Information Response |  | 1...<maxnoofRLs> |  |  |
| RL ID | M |  |  |  |
| Cause | M |  |  |  |
| Successful RL Information Response |  | 0..<maxnoofRLs-1> |  |  |
| RL ID | M |  |  |  |
| SAI | M |  |  |  |
| UL Interference Level | M |  |  |  |
| DL Code Information |  | 1.. <maxnoofDLCodes |  |  |
| DL Scrambling Code | M |  |  |  |
| FDD DL Channelisation Code Number | M |  |  |  |
| Diversity Indication | M |  |  |  |
| CHOICE diversity Indication |  |  |  |  |
| Combining |  |  |  |  |
| RL ID | M |  |  | Reference RL ID for the combining |
| Non Combining or IE not present |  |  |  | "IE not present" is equivalent to "First RL". |
| DCH Information Response |  | 0..<maxnoofDCHs> |  | Only one DCH per set of co-ordinated DCHs shall be included. |
| DCH ID | M |  |  |  |
| Binding ID | M |  |  |  |
| Transport Layer Address | M |  |  |  |
| SSDT Support Indicator | M |  |  |  |
| Neighbouring Cell Information | O | 0..<maxnoofneighbouringRNCs> |  |  |
| RNC-Id | M |  |  |  |
| CN PS Domain Identifier | O |  |  |  |
| CN CS Domain Identifier | O |  |  |  |
| Per FDD Cell Information |  | 0..<maxnoofFDDneighbours> |  |  |
| C-Id | M |  |  |  |
| UARFCN | M |  |  |  |
| Frame Offset | O |  |  |  |
| Primary Scrambling Code | M |  |  |  |
| Primary CPICH Power | O |  |  |  |
| Per TDD Cell Information |  | 0..<maxnoofTDDneighbours> |  |  |
| C-Id | M |  |  |  |
| UARFCN | M |  |  |  |
| Frame Offset | O |  |  |  |
| Cell Parameter ID | M |  |  |  |
| Sync Case | M |  |  |  |
| Time Slot | C-Case3 |  |  |  |
| PSCH Time Slot | C-Case2&3 |  |  |  |
| Uplink Eb/No Target | O |  | Uplink Eb/No |  |
| Maximum Uplink Eb/No | M |  | Uplink Eb/No |  |
| Minimum Uplink Eb/No | M |  | Uplink Eb/No |  |
| Downlink Eb/No Target | O |  |  |  |
| Criticality Diagnostics | O |  |  |  |

| Condition | Explanation |
| --- | --- |
| IfComb | This IE is present if the 'Diversity Indication' IE indicates 'combining' in the Node B. |
| IfNotComb | This IE is present if the 'Diversity Indication' IE indicates 'non combining' in the Node B. |
| NotFirstRL | The IE is present only if the RL is not the first RL in the RL Information |
| Case1 | This IE is present only if Sync Case = Case1. |
| Case2&3 | This IE is present only if Sync Case = Case2 or Case3. |

| Range bound | Explanation |
| --- | --- |
| MaxnoofRLs | Maximum no. of RLs for one UE. |
| MaxnoofDCHs | Maximum no. of DCHs for one UE. |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |
| MaxnoofFDDneighbours | Maximum number of neighbouring FDD cell for one cell |
| MaxnoofTDDneighbours | Maximum number of neighbouring TDD cell for one cell |

#### 9.1.5.2	TDD Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |
| Transaction ID | M |  |  |  |
| Unsuccessful RL Information Response |  | 1 |  |  |
| RL ID | M |  |  |  |
| Cause | M |  |  |  |
| Criticality Diagnostics | O |  |  |  |

### 9.1.7	RADIO LINK ADDITION RESPONSE

#### 9.1.7.1	FDD Message

| IE/Group Name | Presence |  | Range |  | IE type and reference |  | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |  |  |  |  |
| Transaction ID | M |  |  |  |  |  |  |  |
| RL Information Response |  |  | 1..<maxnoofRLs-1> |  |  |  |  |  |
| RL ID | M |  |  |  |  |  |  |  |
| SAI | M |  |  |  |  |  |  |  |
| UL Interference Level | M |  |  |  |  |  |  |  |
| DL Code Information |  |  | 1..<maxnoofDLCodes> |  |  |  |  |  |
| DL Scrambling Code | M |  |  |  |  |  |  |  |
| DL Channelisation Code | M |  |  |  |  |  |  |  |
| Diversity Indication | M |  |  |  |  |  |  |  |
| CHOICE diversity indication |  |  |  |  |  |  |  |  |
| Combining |  |  |  |  |  |  |  |  |
| RL ID | M |  |  |  |  |  | Reference RL-Id |  |
| Non combining |  |  |  |  |  |  |  |  |
| DCH Information Response |  |  | 1..<maxnoofDCHs> |  |  |  | Only one DCH per set of co-ordinated DCHs shall be included. |  |
| DCH ID | M |  |  |  |  |  |  |  |
| Binding ID | M |  |  |  |  |  |  |  |
| Transport Layer Address | M |  |  |  |  |  |  |  |
| SSDT Support Indicator | M |  |  |  |  |  |  |  |
| Minimum Uplink Eb/No | M |  |  |  | Uplink Eb/No |  |  |  |
| Maximum Uplink Eb/No | M |  |  |  | Uplink Eb/No |  |  |  |
| Neighbouring Cell Information |  |  |  | 0..<maxnoofneighbouringRNCs> |  |  |  |  |
| RNC-Id |  | M |  |  |  |  |  |  |
| CN PS Domain Identifier |  | O |  |  |  |  |  |  |
| CN CS Domain Identifier |  | O |  |  |  |  |  |  |
| Per FDD Cell Information |  |  |  | 0..<maxnoofFDDneighbours> |  |  |  |  |
| C-Id |  | M |  |  |  |  |  |  |
| UARFCN |  | M |  |  |  |  |  |  |
| Frame Offset |  | O |  |  |  |  |  |  |
| Primary Scrambling Code |  | M |  |  |  |  |  |  |
| Primary CPICH Power |  | O |  |  |  |  |  |  |
| Per TDD Cell Information |  |  |  | 0..<maxnoofTDDneighbours> |  |  |  |  |
| C-Id |  | M |  |  |  |  |  |  |
| UARFCN |  | M |  |  |  |  |  |  |
| Frame Offset |  | O |  |  |  |  |  |  |
| Cell Parameter ID |  | M |  |  |  |  |  |  |
| Sync Case |  | M |  |  |  |  |  |  |
| Time Slot |  | C-Case1 |  |  |  |  |  |  |
| PSCH Time Slot |  | C-Case2&3 |  |  |  |  |  |  |
| Criticality Diagnostics |  | O |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Case1 | This IE is present only if Sync Case = Case1. |
| Case2&3 | This IE is present only if Sync Case = Case2 or Case3. |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDCHs | Maximum number of dedicated channels on one RL |
| MaxnoofRLs | Maximum number of radio links for one UE |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |
| MaxnoofFDDneighbours | Maximum number of neighbouring FDD cell for one cell |
| MaxnoofTDDneighbours | Maximum number of neighbouring TDD cell for one cell |
| MaxnoofDLCodes | Maximum number of DL code information |

#### 9.1.7.2	TDD Message

| IE/Group Name | Presence |  | Range |  | IE type and reference |  | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |  |  |  |  |
| Transaction ID | M |  |  |  |  |  |  |  |
| RL Information Response |  |  | 1 |  |  |  |  |  |
| RL ID | M |  |  |  |  |  |  |  |
| SAI | M |  |  |  |  |  |  |  |
| UL Interference Level | M |  |  |  |  |  |  |  |
| UL CCTrCH Information |  |  | 1..<maxnoof CCTrCHs> |  |  |  |  |  |
| CCTrCH ID | M |  |  |  |  |  |  |  |
| UL DPCH Information |  |  | 1..<maxnoOfDPCHs> |  |  |  |  |  |
| DPCH ID | M |  |  |  |  |  |  |  |
| TDD Channelisation Code | M |  |  |  |  |  |  |  |
| Burst Type | M |  |  |  |  |  |  |  |
| Midamble Shift | M |  |  |  |  |  |  |  |
| Time Slot | M |  |  |  |  |  |  |  |
| TDD Physical Channel Offset | M |  |  |  |  |  |  |  |
| Repetition Period | M |  |  |  |  |  |  |  |
| Repetition Length | M |  |  |  |  |  |  |  |
| TFCI Presence | M |  |  |  |  |  |  |  |
| DL CCTrCH Information |  |  | 1..<maxnoof CCTrCHs> |  |  |  |  |  |
| CCTrCH ID | M |  |  |  |  |  |  |  |
| DL DPCH information |  |  | 1..<maxnoOfDPCHs> |  |  |  |  |  |
| DPCH ID | M |  |  |  |  |  |  |  |
| TDD Channelisation Code | M |  |  |  |  |  |  |  |
| Burst Type | M |  |  |  |  |  |  |  |
| Midamble Shift | M |  |  |  |  |  |  |  |
| Time Slot | M |  |  |  |  |  |  |  |
| TDD Physical Channel Offset | M |  |  |  |  |  |  |  |
| Repetition Period | M |  |  |  |  |  |  |  |
| Repetition Length | M |  |  |  |  |  |  |  |
| TFCI Presence | M |  |  |  |  |  |  |  |
| Diversity Indication | M |  |  |  |  |  |  |  |
| CHOICE diversity indication |  |  |  |  |  |  |  |  |
| Combining |  |  |  |  |  |  |  |  |
| RL ID | M |  |  |  |  |  | Reference RL |  |
| Non combining |  |  |  |  |  |  |  |  |
| DCH Information Response |  |  | 1..<maxnoofDCHs> |  |  |  | Only one DCH per set of co-ordinated DCHs shall be included. |  |
| DCH ID | M |  |  |  |  |  |  |  |
| Binding ID | M |  |  |  |  |  |  |  |
| Transport Layer Address | M |  |  |  |  |  |  |  |
| Minimum Uplink Eb/No | M |  |  |  | Uplink Eb/No |  |  |  |
| Maximum Uplink Eb/No | M |  |  |  | Uplink Eb/No |  |  |  |
| Neighbouring Cell Information |  |  |  | 0..<maxnoofneighbouringRNCs> |  |  |  |  |
| RNC-Id |  | M |  |  |  |  |  |  |
| CN PS Domain Identifier |  | O |  |  |  |  |  |  |
| CN CS Domain Identifier |  | O |  |  |  |  |  |  |
| Per FDD Cell Information |  |  |  | 0..<maxnoofFDDneighbours> |  |  |  |  |
| C-Id |  | M |  |  |  |  |  |  |
| UARFCN |  | M |  |  |  |  |  |  |
| Frame Offset |  | O |  |  |  |  |  |  |
| Primary Scrambling Code |  | M |  |  |  |  |  |  |
| Primary CPICH Power |  | O |  |  |  |  |  |  |
| Per TDD Cell Information |  |  |  | 0..<maxnoofTDDneighbours> |  |  |  |  |
| C-Id |  | M |  |  |  |  |  |  |
| UARFCN |  | M |  |  |  |  |  |  |
| Frame Offset |  | O |  |  |  |  |  |  |
| Cell Parameter ID |  | M |  |  |  |  |  |  |
| Sync Case |  | M |  |  |  |  |  |  |
| Time Slot |  | C-Case1 |  |  |  |  |  |  |
| PSCH Time Slot |  | C-Case2&3 |  |  |  |  |  |  |
| Criticality Diagnostics |  | O |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Case1 | This IE is present only if Sync Case = Case1 |
| Case2&3 | This IE is present only if Sync Case = Case2 or Case3. |

| Range Bound | Explanation |  |
| --- | --- | --- |
| MaxnoofDCHs | Maximum number of dedicated channels on one RL |  |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |  |
| MaxnoofFDDneighbours | Maximum number of neighbouring FDD cell for one cell |  |
| MaxnoofTDDneighbours | Maximum number of neighbouring TDD cell for one cell |  |
| MaxnoofDLCodes | Maximum number of DL code information |  |
| MaxnoOfDPCHs |  | Maximum number of DPCH in one CCTrCH |
| MaxnoofCCTrCHs |  | no. of CCTrCH for one UE. |

### 9.1.8	RADIO LINK ADDITION FAILURE

#### 9.1.8.1	FDD Message

| IE/Group Name | Presence |  | Range |  | IE type and reference |  | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |  |  |  |  |
| Transaction ID | M |  |  |  |  |  |  |  |
| Unsuccessful RL Information Response |  |  | 1..<maxnoofRLs-1> |  |  |  |  |  |
| RL ID | M |  |  |  |  |  |  |  |
| Cause | M |  |  |  |  |  |  |  |
| Succesfull RL Information Response |  |  | 1..<maxnoofRLs-2> |  |  |  |  |  |
| RL ID | M |  |  |  |  |  |  |  |
| SAI | M |  |  |  |  |  |  |  |
| UL Interference Level | M |  |  |  |  |  |  |  |
| DL Code Information |  |  | 1..<maxnoofDLCodes> |  |  |  |  |  |
| DL scrambling code | M |  |  |  |  |  |  |  |
| DL channelisation code | M |  |  |  |  |  |  |  |
| Diversity Indication | M |  |  |  |  |  |  |  |
| CHOICE diversity indication |  |  |  |  |  |  |  |  |
| Combining |  |  |  |  |  |  |  |  |
| RL ID | M |  |  |  |  |  | Reference RL-Id |  |
| Non combining |  |  |  |  |  |  |  |  |
| DCH Information Response |  |  | 1..<maxnoofDCHs> |  |  |  | Only one DCH per set of co-ordinated DCHs shall be included. |  |
| DCH ID | M |  |  |  |  |  |  |  |
| Binding ID | M |  |  |  |  |  |  |  |
| Transport Layer Address | M |  |  |  |  |  |  |  |
| SSDT Support Indicator | M |  |  |  |  |  |  |  |
| Minimum Uplink Eb/No | M |  |  |  | Uplink Eb/No |  |  |  |
| Maximum Uplink Eb/No | M |  |  |  | Uplink Eb/No |  |  |  |
| Neighbouring Cell Information |  |  |  | 0..<maxnoofneighbouringRNCs> |  |  |  |  |
| RNC-Id |  | M |  |  |  |  |  |  |
| CN PS Domain Identifier |  | O |  |  |  |  |  |  |
| CN CS Domain Identifier |  | O |  |  |  |  |  |  |
| Per FDD Cell Information |  |  |  | 0..<maxnoofFDDneighbours> |  |  |  |  |
| C-Id |  | M |  |  |  |  |  |  |
| UARFCN |  | M |  |  |  |  |  |  |
| Frame Offset |  | O |  |  |  |  |  |  |
| Primary Scrambling Code |  | M |  |  |  |  |  |  |
| Primary CPICH Power |  | O |  |  |  |  |  |  |
| Per TDD Cell Information |  |  |  | 0..<maxnoofTDDneighbours> |  |  |  |  |
| C-Id |  | M |  |  |  |  |  |  |
| UARFCN |  | M |  |  |  |  |  |  |
| Frame Offset |  | O |  |  |  |  |  |  |
| Cell Parameter ID |  | M |  |  |  |  |  |  |
| Sync Case |  | M |  |  |  |  |  |  |
| Time Slot |  | C-Case1 |  |  |  |  |  |  |
| PSCH Time Slot |  | C-Case2&3 |  |  |  |  |  |  |
| Criticality Diagnostics |  | O |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Case1 | This IE is present only if Sync Case = Case1. |
| Case2&3 | This IE is present only if Sync Case = Case2 or Case3. |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDCHs | Maximum number of dedicated channels on one RL |
| MaxnoofRLs | Maximum number of radio links for one UE |
| MaxnoofneighbouringRNCs | Maximum number of neighbouring RNCs |
| MaxnoofFDDneighbours | Maximum number of neighbouring FDD cell for one cell |
| MaxnoofTDDneighbours | Maximum number of neighbouring TDD cell for one cell |
| MaxnoofDLCodes | Maximum number of DL code information |

#### 9.1.8.2	TDD Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  |  |  |
| Transaction ID | M |  |  |  |
| Unsuccessful RL Information Response |  | 1 |  |  |
| RL ID | M |  |  |  |
| Cause | M |  |  |  |
| Criticality Diagnostics | O |  |  |  |

    - PDU Definitions to be corrected!!

....

maxNoOfDL-Codes,

maxNrOfCCTrCHs,

maxNrOfDCHs,

maxNrOfDL-Codes,

maxNrOfDPCHs,

maxNrOfFACH-FD-Size,

maxNrOfMACcSDU-Length,

maxNrOfRLs,

maxNrOfSCCPCHs,

maxRNCinURA,

maxnoofneighbouringRNCs

maxnoofFDDneighboursperRNC

maxnoofTDDneighboursperRNC

FROM RNSAP-Constants;

-- **************************************************************

--

-- RADIO LINK SETUP RESPONSE FDD

--

-- **************************************************************

- RadioLinkSetupResponseFDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkSetupResponseFDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkSetupResponseFDD-Extensions}}                   OPTIONAL,

...

}

- RadioLinkSetupResponseFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-D-RNTI					CRITICALITY ignore	TYPE D-RNTI 					PRESENCE optional	} |

{ ID id-CN-PS-DomainIdentifier			CRITICALITY ignore	TYPE CN-PS-DomainIdentifier			PRESENCE optional	} |

{ ID id-CN-CS-DomainIdentifier			CRITICALITY ignore	TYPE CN-CS-DomainIdentifier			PRESENCE optional	} |

{ ID id-RL-InformationResponseList-RL-SetupRspFDD

CRITICALITY ignore	TYPE RL-InformationResponseList-RL-SetupRspFDD

PRESENCE mandatory	} |

{ ID id-UL-EbNoTarget				CRITICALITY ignore	TYPE UL-EbNoTarget 				PRESENCE optional	} |

{ ID id-DL-EbNoTarget				CRITICALITY ignore	TYPE DL-EbNoTarget 				PRESENCE optional	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

RL-InformationResponseList-RL-SetupRspFDD 		::= RL-IE-ContainerList { {RL-InformationResponseItemIEs-RL-SetupRspFDD} }

- RL-InformationResponseItemIEs-RL-SetupRspFDD RNSAP-PROTOCOL-IES ::= {

{ ID id-RL-InformationResponseItem-RL-SetupRspFDD

CRITICALITY ignore	TYPE RL-InformationResponseItem-RL-SetupRspFDD 	PRESENCE mandatory	},

...

}

- RL-InformationResponseItem-RL-SetupRspFDD ::= SEQUENCE {

rL-ID						RL-ID,

sAI						SAI,

ul-InterferenceLevel				ScaledUL-InterferenceLevel,

dl-CodeInformation				DL-CodeInformationList-RL-SetupRspFDD,

sSDT-SupportIndicator				SSDT-SupportIndicator,

maxUL-EbNo					UL-EbNo,

minUL-EbNo					UL-EbNo,

neighbouringFDD-CellInformation			NeighbouringFDD-CellInformationList-RL-SetupRsp OPTIONAL,

neighbouringTDD-CellInformation			NeighbouringTDD-CellInformationList-RL-SetupRsp OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RL-InformationResponseItem-RL-SetupRspFDD-ExtIEs} } OPTIONAL,

...

}

- RL-InformationResponseItem-RL-SetupRspFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-CodeInformationList-RL-SetupRspFDD ::= SEQUENCE (SIZE (1..maxNoOfDL-Codes)) OF DL-CodeInformationItem-RL-SetupRspFDD

- DL-CodeInformationItem-RL-SetupRspFDD ::= SEQUENCE {

dl-ScramblingCode				DL-ScramblingCode,

fDD-DL-ChannelisationCodeNumber			FDD-DL-ChannelisationCodeNumber,

-- ** NOTE: How many alternatives are there, 2 or 3? **

diversityIndication				CHOICE {

combining					SEQUENCE {

rL-ID						RL-ID

},

nonCombiningOrIENotPresent			SEQUENCE {

dCH-InformationResponse-RL-SetupRspFDD		DCH-InformationResponseList-RL-SetupRspFDD	OPTIONAL

}

}										OPTIONAL

-- This IE is present only if the RL is not the first on in the RL Information -- ,

iE-Extensions					ProtocolExtensionContainer { {DL-CodeInformationItem-RL-SetupRspFDD-ExtIEs} } OPTIONAL,

...

}

- DL-CodeInformationItem-RL-SetupRspFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DCH-InformationResponseList-RL-SetupRspFDD ::= SEQUENCE (SIZE (1..maxNrOfDCHs)) OF DCH-InformationResponseItem-RL-SetupRspFDD

- DCH-InformationResponseItem-RL-SetupRspFDD ::= SEQUENCE {

dCH-ID						DCH-ID,

bindingID					BindingID,

transportLayerAddress				TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { {DCH-InformationResponseItem-RL-SetupRspFDD-ExtIEs} } OPTIONAL,

...

}

- DCH-InformationResponseItem-RL-SetupRspFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Both FDD and TDD messages use these definitions **

- NeighbouringFDD-CellInformationList-RL-SetupRsp ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringFDD-CellInformationItem-RL-SetupRsp

- NeighbouringFDD-CellInformationItem-RL-SetupRsp ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-FDD-Cell-Information			Per-FDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringFDD-CellInformationItem-RL-SetupRsp-ExtIEs} } OPTIONAL,

...

}

- NeighbouringFDD-CellInformationItem-RL-SetupRsp-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-FDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofFDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset			OPTIONAL,

primaryScramblingCode			PrimaryScramblingCode,

primaryCPICH-Power				PrimaryCPICH-Power			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-FDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-FDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- NeighbouringTDD-CellInformationList-RL-SetupRsp ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringTDD-CellInformationItem-RL-SetupRsp

- NeighbouringTDD-CellInformationItem-RL-SetupRsp ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-TDD-Cell-Information			Per-TDD-Cell-Information,

ul-EbNo						UL-EbNo				OPTIONAL,

dl-EbNo						DL-EbNo				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringTDD-CellInformationItem-RL-SetupRsp-ExtIEs} } OPTIONAL,

...

}

- NeighbouringTDD-CellInformationItem-RL-SetupRsp-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-TDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofTDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset		OPTIONAL,

cellParameterID				CellParameterID,

syncCase						SyncCase,

timeSlot						TimeSlot			OPTIONAL,

-- This IE is present only if SyncCase is Case1 -- ,

pSCH-TimeSlot					PSCH-TimeSlot	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-TDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-TDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- RadioLinkSetupResponseFDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- RADIO LINK SETUP RESPONSE TDD

--

-- **************************************************************

RadioLinkSetupResponseTDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkSetupResponseTDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkSetupResponseTDD-Extensions}}                   OPTIONAL,

...

}

RadioLinkSetupResponseTDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-D-RNTI					CRITICALITY ignore	TYPE D-RNTI 					PRESENCE optional	} |

{ ID id-CN-PS-DomainIdentifier			CRITICALITY ignore	TYPE CN-PS-DomainIdentifier			PRESENCE optional	} |

{ ID id-CN-CS-DomainIdentifier			CRITICALITY ignore	TYPE CN-CS-DomainIdentifier			PRESENCE optional	} |

{ ID id-RL-InformationResponse-RL-SetupRspTDD	CRITICALITY ignore	TYPE RL-InformationResponse-RL-SetupRspTDD 	PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

RL-InformationResponse-RL-SetupRspTDD ::= SEQUENCE {

rL-ID						RL-ID,

sAI						SAI,

ul-InterferenceLevel				ScaledUL-InterferenceLevel,

maxUL-EbNo					UL-EbNo,

minUL-EbNo					UL-EbNo,

ul-EbNoTarget					UL-EbNo						OPTIONAL,

dl-EbNoTarget					DL-EbNo						OPTIONAL,

ul-CCTrCHInformation				UL-CCTrCHInformationList-RL-SetupRspTDD,

dl-CCTrCHInformation				DL-CCTrCHInformationList-RL-SetupRspTDD,

dCH-InformationResponse				DCH-InformationResponseList-RL-SetupRspTDD,

neighbouringFDD-CellInformation			NeighbouringFDD-CellInformationList-RL-SetupRsp OPTIONAL,

-- refer to “NeighbouringFDD-CellInformationList-RL-SetupRsp” in the “RL Seup Response FDD”

neighbouringTDD-CellInformation			NeighbouringTDD-CellInformationList-RL-SetupRsp	OPTIONAL,

-- refer to “NeighbouringFDD-CellInformationList-RL-SetupRsp” in the “RL Seup Response FDD”

iE-Extensions					ProtocolExtensionContainer { {RL-InformationResponse-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

RL-InformationResponse-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

UL-CCTrCHInformationList-RL-SetupRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF UL-CCTrCHInformationItem-RL-SetupRspTDD

UL-CCTrCHInformationItem-RL-SetupRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

ul-DPCH-Information				UL-DPCH-InformationList-RL-SetupRspTDD,

iE-Extensions					ProtocolExtensionContainer { {UL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

UL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

UL-DPCH-InformationList-RL-SetupRspTDD ::= SEQUENCE (SIZE (1..maxNrOfDPCHs)) OF UL-DPCH-InformationItem-RL-SetupRspTDD

-- **NOTE: UL-DPCH-InformationItem-RL-SetupRspTDD and DL-DPCH-InformationItem-RL-SetupRspTDD

--         are currently similar. Combine them? **

UL-DPCH-InformationItem-RL-SetupRspTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode,

burstType					BurstType,

midambleShift					MidambleShift,

timeSlot					TimeSlot,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {UL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

UL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-CCTrCHInformationList-RL-SetupRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF DL-CCTrCHInformationItem-RL-SetupRspTDD

DL-CCTrCHInformationItem-RL-SetupRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

dl-DPCH-Information				DL-DPCH-InformationList-RL-SetupRspTDD,

iE-Extensions					ProtocolExtensionContainer { {DL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

DL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-DPCH-InformationList-RL-SetupRspTDD ::= SEQUENCE (SIZE (1..maxNrOfDPCHs)) OF DL-DPCH-InformationItem-RL-SetupRspTDD

DL-DPCH-InformationItem-RL-SetupRspTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode,

burstType					BurstType,

midambleShift					MidambleShift,

timeSlot					TimeSlot,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {DL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

DL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DCH-InformationResponseList-RL-SetupRspTDD ::= SEQUENCE (SIZE (1..maxNrOfDCHs)) OF DCH-InformationResponseItem-RL-SetupRspTDD

DCH-InformationResponseItem-RL-SetupRspTDD ::= SEQUENCE {

dCH-ID						DCH-ID,

bindingID					BindingID,

transportLayerAddress				TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { {DCH-InformationResponseItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

DCH-InformationResponseItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

RadioLinkSetupResponseTDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- RADIO LINK SETUP FAILURE FDD

--

-- **************************************************************

RadioLinkSetupFailureFDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkSetupFailureFDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkSetupFailureFDD-Extensions}}                   OPTIONAL,

...

}

RadioLinkSetupFailureFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-D-RNTI					CRITICALITY ignore	TYPE D-RNTI					PRESENCE mandatory	} |

{ ID id-CN-PS-DomainIdentifier			CRITICALITY ignore	TYPE CN-PS-DomainIdentifier			PRESENCE mandatory	} |

{ ID id-CN-CS-DomainIdentifier			CRITICALITY ignore	TYPE CN-CS-DomainIdentifier			PRESENCE mandatory	} |

{ ID id-UnsuccessfulRL-InformationResponseList-RL-SetupFailureFDD

CRITICALITY ignore	TYPE UnsuccessfulRL-InformationResponseList-RL-SetupFailureFDD

PRESENCE mandatory	} |

{ ID id-SuccessfulRL-InformationResponseList-RL-SetupFailureFDD

CRITICALITY ignore	TYPE SuccessfulRL-InformationResponseList-RL-SetupFailureFDD

PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UnsuccessfulRL-InformationResponseList-RL-SetupFailureFDD ::= RL-IE-ContainerList { {UnsuccessfulRL-InformationResponse-RL-SetupFailureFDD-IEs} }

UnsuccessfulRL-InformationResponse-RL-SetupFailureFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-UnsuccessfulRL-InformationResponse-RL-SetupFailureFDD

CRITICALITY ignore	TYPE UnsuccessfulRL-InformationResponse-RL-SetupFailureFDD

PRESENCE mandatory	},

...

}

UnsuccessfulRL-InformationResponse-RL-SetupFailureFDD ::= SEQUENCE {

rL-ID						RL-ID,

cause						Cause,

iE-Extensions					ProtocolExtensionContainer { {UnsuccessfulRL-InformationResponse-RL-SetupFailureFDD-ExtIEs} } OPTIONAL,

...

}

UnsuccessfulRL-InformationResponse-RL-SetupFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

SuccessfulRL-InformationResponseList-RL-SetupFailureFDD ::= RL-IE-ContainerList { {SuccessfulRL-InformationResponse-RL-SetupFailureFDD-IEs} }

SuccessfulRL-InformationResponse-RL-SetupFailureFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-SuccessfulRL-InformationResponse-RL-SetupFailureFDD

CRITICALITY ignore	TYPE SuccessfulRL-InformationResponse-RL-SetupFailureFDD

PRESENCE mandatory	},

...

}

SuccessfulRL-InformationResponse-RL-SetupFailureFDD ::= SEQUENCE {

rL-ID						RL-ID,

sAI						SAI,

ul-InterferenceLevel				ScaledUL-InterferenceLevel,

dl-CodeInformation				DL-CodeInformationList-RL-SetupFailureFDD,

sSDT-SupportIndicator				SSDT-SupportIndicator,

neighbouringFDD-CellInformation			NeighbouringFDD-CellInformationList-RL-SetupFailureFDD OPTIONAL,

neighbouringTDD-CellInformation			NeighbouringTDD-CellInformationList-RL-SetupFailureFDD OPTIONAL,

ul-EbNoTarget					UL-EbNo,

maxUL-EbNo					UL-EbNo,

minUL-EbNo					UL-EbNo,

dl-EbNoTarget					DL-EbNo,

iE-Extensions					ProtocolExtensionContainer { {SuccessfulRL-InformationResponse-RL-SetupFailureFDD-ExtIEs} } OPTIONAL,

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-CodeInformationList-RL-SetupFailureFDD ::= SEQUENCE (SIZE (1..maxNoOfDL-Codes)) OF DL-CodeInformationItem-RL-SetupFailureFDD

SuccessfulRL-InformationResponse-RL-SetupFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-CodeInformationItem-RL-SetupFailureFDD ::= SEQUENCE {

dl-ScramblingCode				DL-ScramblingCode,

fDD-DL-ChannelisationCodeNumber			FDD-DL-ChannelisationCodeNumber,

-- ** NOTE: How many alternatives are there, 2 or 3? **

diversityIndication				CHOICE {

combining					SEQUENCE {

rL-ID						RL-ID

},

nonCombiningOrIENotPresent			SEQUENCE {

dCH-InformationResponse-RL-SetupFailureFDD		DCH-InformationResponseList-RL-SetupFailureFDD	OPTIONAL

}

}										OPTIONAL

-- This IE is present only if the RL is not the first on in the RL Information -- ,

iE-Extensions					ProtocolExtensionContainer { {DL-CodeInformationItem-RL-SetupFailureFDD-ExtIEs} } OPTIONAL,

...

}

DL-CodeInformationItem-RL-SetupFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DCH-InformationResponseList-RL-SetupFailureFDD ::= SEQUENCE (SIZE (1..maxNrOfDCHs)) OF DCH-InformationResponseItem-RL-SetupFailureFDD

DCH-InformationResponseItem-RL-SetupFailureFDD ::= SEQUENCE {

dCH-ID						DCH-ID,

bindingID					BindingID,

transportLayerAddress				TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { {DCH-InformationResponseItem-RL-SetupFailureFDD-ExtIEs} } OPTIONAL,

...

}

DCH-InformationResponseItem-RL-SetupFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbouringFDD-CellInformationList-RL-SetupFailureFDD ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringFDD-CellInformationItem-RL-SetupFailureFDD

NeighbouringFDD-CellInformationItem-RL-SetupFailureFDD ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-FDD-Cell-Information			Per-FDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringFDD-CellInformationItem-RL-SetupFailureFDD-ExtIEs} } OPTIONAL,

...

}

NeighbouringFDD-CellInformationItem-RL-SetupFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-FDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofFDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset			OPTIONAL,

primaryScramblingCode			PrimaryScramblingCode,

primaryCPICH-Power				PrimaryCPICH-Power			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-FDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-FDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

NeighbouringTDD-CellInformationList-RL-SetupFailureFDD ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringTDD-CellInformationItem-RL-SetupFailureFDD

NeighbouringTDD-CellInformationItem-RL-SetupFailureFDD ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-TDD-Cell-Information			Per-TDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringTDD-CellInformationItem-RL-SetupFailureFDD-ExtIEs} } OPTIONAL,

...

}

NeighbouringTDD-CellInformationItem-RL-SetupFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-TDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofTDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset		OPTIONAL,

cellParameterID				CellParameterID,

syncCase						SyncCase,

timeSlot						TimeSlot			OPTIONAL,

-- This IE is present only if SyncCase is Case1 -- ,

pSCH-TimeSlot					PSCH-TimeSlot	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-TDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-TDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

RadioLinkSetupFailureFDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

------------------partly omitted----------------------

-- **************************************************************

--

-- RADIO LINK ADDITION RESPONSE FDD

--

-- **************************************************************

- RadioLinkAdditionResponseFDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkAdditionResponseFDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkAdditionResponseFDD-Extensions}}                   OPTIONAL,

...

}

- RadioLinkAdditionResponseFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-D-RNTI					CRITICALITY ignore	TYPE D-RNTI 					PRESENCE optional	} |

- { ID id-RL-InformationResponseList-RL-AdditionRspFDD

CRITICALITY ignore	TYPE RL-InformationResponseList-RL-AdditionRspFDD

PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

RL-InformationResponseList-RL-AdditionRspFDD 		::= RL-IE-ContainerList { {RL-InformationResponseItemIEs-RL-AdditionRspFDD} }

- RL-InformationResponseItemIEs-RL-AdditionRspFDD RNSAP-PROTOCOL-IES ::= {

{ ID id-RL-InformationResponseItem-RL-AdditionRspFDD

CRITICALITY ignore	TYPE RL-InformationResponseItem-RL-AdditionRspFDD 	PRESENCE mandatory	},

...

}

- RL-InformationResponseItem-RL-AdditionRspFDD ::= SEQUENCE {

rL-ID						RL-ID,

sAI						SAI,

ul-InterferenceLevel				ScaledUL-InterferenceLevel,

dl-CodeInformation				DL-CodeInformationList-RL-AdditionRspFDD,

sSDT-SupportIndicator				SSDT-SupportIndicator,

maxUL-EbNo					UL-EbNo,

minUL-EbNo					UL-EbNo,

neighbouringFDD-CellInformation			NeighbouringFDD-CellInformationList-RL-SetupRsp OPTIONAL,

neighbouringTDD-CellInformation			NeighbouringTDD-CellInformationList-RL-SetupRsp OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RL-InformationResponseItem-RL-AdditionRspFDD-ExtIEs} } OPTIONAL,

...

}

- RL-InformationResponseItem-RL-AdditionRspFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-CodeInformationList-RL-AdditionRspFDD ::= SEQUENCE (SIZE (1..maxNoOfDL-Codes)) OF DL-CodeInformationItem-RL-AdditionRspFDD

- DL-CodeInformationItem-RL-AdditionRspFDD ::= SEQUENCE {

dl-ScramblingCode				DL-ScramblingCode,

fDD-DL-ChannelisationCodeNumber			FDD-DL-ChannelisationCodeNumber,

-- ** NOTE: How many alternatives are there, 2 or 3? **

diversityIndication				CHOICE {

combining					SEQUENCE {

rL-ID						RL-ID

},

nonCombiningOrIENotPresent			SEQUENCE {

dCH-InformationResponse-RL-AdditionRspFDD		DCH-InformationResponseList-RL-AdditionRspFDD	OPTIONAL

}

}										OPTIONAL

-- This IE is present only if the RL is not the first on in the RL Information -- ,

iE-Extensions					ProtocolExtensionContainer { {DL-CodeInformationItem-RL-AdditionRspFDD-ExtIEs} } OPTIONAL,

...

}

- DL-CodeInformationItem-RL-AdditionRspFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DCH-InformationResponseList-RL-AdditionRspFDD ::= SEQUENCE (SIZE (1..maxNrOfDCHs)) OF DCH-InformationResponseItem-RL-AdditionRspFDD

- DCH-InformationResponseItem-RL-AdditionRspFDD ::= SEQUENCE {

dCH-ID						DCH-ID,

bindingID					BindingID,

transportLayerAddress				TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { {DCH-InformationResponseItem-RL-AdditionRspFDD-ExtIEs} } OPTIONAL,

...

}

- DCH-InformationResponseItem-RL-AdditionRspFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Both FDD and TDD messages use these definitions **

- NeighbouringFDD-CellInformationList-RL-AdditionRsp ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringFDD-CellInformationItem-RL-AdditionRsp

- NeighbouringFDD-CellInformationItem-RL-AdditionRsp ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-FDD-Cell-Information			Per-FDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringFDD-CellInformationItem-RL-AdditionRsp-ExtIEs} } OPTIONAL,

...

}

- NeighbouringFDD-CellInformationItem-RL-AdditionRsp-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-FDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofFDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset			OPTIONAL,

primaryScramblingCode			PrimaryScramblingCode,

primaryCPICH-Power				PrimaryCPICH-Power			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-FDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-FDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- NeighbouringTDD-CellInformationList-RL-AdditionRsp ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringTDD-CellInformationItem-RL-AdditionRsp

- NeighbouringTDD-CellInformationItem-RL-AdditionRsp ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-TDD-Cell-Information			Per-TDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringTDD-CellInformationItem-RL-AdditionRsp-ExtIEs} } OPTIONAL,

...

}

- NeighbouringTDD-CellInformationItem-RL-AdditionRsp-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-TDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofTDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset		OPTIONAL,

cellParameterID				CellParameterID,

syncCase						SyncCase,

timeSlot						TimeSlot			OPTIONAL,

-- This IE is present only if SyncCase is Case1 -- ,

pSCH-TimeSlot					PSCH-TimeSlot	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-TDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-TDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- RadioLinkAdditionResponseFDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- RADIO LINK ADDITION RESPONSE TDD

--

-- **************************************************************

- RadioLinkAdditionResponseTDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkAdditionResponseTDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkAdditionResponseTDD-Extensions}}                   OPTIONAL,

...

}

- RadioLinkAdditionResponseTDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-D-RNTI					CRITICALITY ignore	TYPE D-RNTI 					PRESENCE optional	} |

- { ID id-RL-InformationResponse-RL-AdditionRspTDD

CRITICALITY ignore	TYPE RL-InformationResponse-RL-AdditionRspTDD 	PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

- RL-InformationResponse-RL-AdditionRspTDD ::= SEQUENCE {

rL-ID						RL-ID,

sAI						SAI,

ul-InterferenceLevel				ScaledUL-InterferenceLevel,

ul-CCTrCHInformation				UL-CCTrCHInformationList-RL-AdditionRspTDD,

dl-CCTrCHInformation				DL-CCTrCHInformationList-RL-AdditionRspTDD,

diversityIndication				CHOICE {

combining					SEQUENCE {

rL-ID						RL-ID

},

nonCombiningOrIENotPresent			SEQUENCE {

dCH-InformationResponse-RL-AdditionRspFDD		DCH-InformationResponseList-RL-AdditionRspFDD	OPTIONAL

}

}										OPTIONAL,

maxUL-EbNo					UL-EbNo,

minUL-EbNo					UL-EbNo,

neighbouringFDD-CellInformation			NeighbouringFDD-CellInformationList-RL-AdditionRspTDD 	OPTIONAL,

neighbouringTDD-CellInformation			NeighbouringTDD-CellInformationList-RL-AdditionRspTDD	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RL-InformationResponse-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

- RL-InformationResponse-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

UL-CCTrCHInformationList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF UL-CCTrCHInformationItem-RL-AdditionRspTDD

- UL-CCTrCHInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

ul-DPCH-Information				UL-DPCH-InformationList-RL-AdditionRspTDD,

iE-Extensions					ProtocolExtensionContainer { {UL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

- UL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

UL-DPCH-InformationList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfDPCHs)) OF UL-DPCH-InformationItem-RL-AdditionRspTDD

- UL-DPCH-InformationItem-RL-AdditionRspTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode,

burstType					BurstType,

midambleShift					MidambleShift,

timeSlot					TimeSlot,

offset						Offset,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {UL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

- UL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-CCTrCHInformationList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF DL-CCTrCHInformationItem-RL-AdditionRspTDD

- DL-CCTrCHInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

dl-DPCH-Information				DL-DPCH-InformationList-RL-AdditionRspTDD,

iE-Extensions					ProtocolExtensionContainer { {DL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

- DL-CCTrCHInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-DPCH-InformationList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (1..maxNrOfDPCHs)) OF DL-DPCH-InformationItem-RL-AdditionRspTDD

- DL-DPCH-InformationItem-RL-AdditionRspTDD ::= SEQUENCE {

dPCH-ID						DPCH-ID,

tDD-ChannelisationCode				TDD-ChannelisationCode,

burstType					BurstType,

midambleShift					MidambleShift,

timeSlot					TimeSlot,

tDD-PhysicalChannelOffset			TDD-PhysicalChannelOffset,

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tFCI-Presence					TFCI-Presence,

iE-Extensions					ProtocolExtensionContainer { {DL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

- DL-DPCH-InformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- NeighbouringFDD-CellInformationList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringFDD-CellInformationItem-RL-AdditionRspTDD

- NeighbouringFDD-CellInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-FDD-Cell-Information			Per-FDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringFDD-CellInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

- NeighbouringFDD-CellInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-FDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofFDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset			OPTIONAL,

primaryScramblingCode			PrimaryScramblingCode,

primaryCPICH-Power				PrimaryCPICH-Power			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-FDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-FDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- NeighbouringTDD-CellInformationList-RL-AdditionRspTDD ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringTDD-CellInformationItem-RL-AdditionRspTDD

- NeighbouringTDD-CellInformationItem-RL-AdditionRspTDD ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-TDD-Cell-Information			Per-TDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringTDD-CellInformationItem-RL-AdditionRspTDD-ExtIEs} } OPTIONAL,

...

}

- NeighbouringTDD-CellInformationItem-RL-AdditionRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-TDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofTDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset		OPTIONAL,

cellParameterID				CellParameterID,

syncCase						SyncCase,

timeSlot						TimeSlot			OPTIONAL,

-- This IE is present only if SyncCase is Case1 -- ,

pSCH-TimeSlot					PSCH-TimeSlot	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-TDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-TDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- RadioLinkAdditionResponseTDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- RADIO LINK ADDITION FAILURE FDD

--

-- **************************************************************

- RadioLinkAdditionFailureFDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkAdditionFailureFDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkAdditionFailureFDD-Extensions}}                   OPTIONAL,

...

}

- RadioLinkAdditionFailureFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-UnsuccessfulRL-InformationResponseList-RL-AdditionFailureFDD

CRITICALITY ignore	TYPE UnsuccessfulRL-InformationResponseList-RL-AdditionFailureFDD

PRESENCE mandatory	} |

{ ID id-SuccessfulRL-InformationResponseList-RL-AdditionFailureFDD

CRITICALITY ignore	TYPE SuccessfulRL-InformationResponseList-RL-AdditionFailureFDD

PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

- UnsuccessfulRL-InformationResponseList-RL-AdditionFailureFDD ::= RL-IE-ContainerList { {UnsuccessfulRL-InformationResponse-RL-AdditionFailureFDD-IEs} }

- UnsuccessfulRL-InformationResponse-RL-AdditionFailureFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-UnsuccessfulRL-InformationResponse-RL-AdditionFailureFDD

CRITICALITY ignore	TYPE UnsuccessfulRL-InformationResponse-RL-AdditionFailureFDD

PRESENCE mandatory	},

...

}

- UnsuccessfulRL-InformationResponse-RL-AdditionFailureFDD ::= SEQUENCE {

rL-ID						RL-ID,

cause						Cause,

iE-Extensions					ProtocolExtensionContainer { {UnsuccessfulRL-InformationResponse-RL-AdditionFailureFDD-ExtIEs} } OPTIONAL,

...

}

- UnsuccessfulRL-InformationResponse-RL-AdditionFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- SuccessfulRL-InformationResponseList-RL-AdditionFailureFDD ::= RL-IE-ContainerList { {SuccessfulRL-InformationResponse-RL-AdditionFailureFDD-IEs} }

- SuccessfulRL-InformationResponse-RL-AdditionFailureFDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-SuccessfulRL-InformationResponse-RL-AdditionFailureFDD

CRITICALITY ignore	TYPE SuccessfulRL-InformationResponse-RL-AdditionFailureFDD

PRESENCE mandatory	},

...

}

- SuccessfulRL-InformationResponse-RL-AdditionFailureFDD ::= SEQUENCE {

rL-ID						RL-ID,

sAI						SAI,

ul-InterferenceLevel				ScaledUL-InterferenceLevel,

dl-CodeInformation				DL-CodeInformationList-RL-AdditionFailureFDD,

sSDT-SupportIndicator				SSDT-SupportIndicator,

maxUL-EbNo					UL-EbNo,

minUL-EbNo					UL-EbNo,

neighbouringFDD-CellInformation			NeighbouringFDD-CellInformationList-RL-AdditionFailureFDD OPTIONAL,

neighbouringTDD-CellInformation			NeighbouringTDD-CellInformationList-RL-AdditionFailureFDD OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {SuccessfulRL-InformationResponse-RL-AdditionFailureFDD-ExtIEs} } OPTIONAL,

...

}

- SuccessfulRL-InformationResponse-RL-AdditionFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DL-CodeInformationList-RL-AdditionFailureFDD ::= SEQUENCE (SIZE (1..maxNoOfDL-Codes)) OF DL-CodeInformationItem-RL-AdditionFailureFDD

- DL-CodeInformationItem-RL-AdditionFailureFDD ::= SEQUENCE {

dl-ScramblingCode				DL-ScramblingCode,

dl-ChannelisationCode				DL-ChannelisationCode,

diversityIndication				CHOICE {

combining					SEQUENCE {

rL-ID						RL-ID

},

nonCombiningOrIENotPresent			SEQUENCE {

dCH-InformationResponse-RL-AdditionFailureFDD		DCH-InformationResponseList-RL-AdditionFailureFDD	OPTIONAL

}

}										OPTIONAL

-- This IE is present only if the RL is not the first on in the RL Information -- ,

iE-Extensions					ProtocolExtensionContainer { {DL-CodeInformationItem-RL-AdditionFailureFDD-ExtIEs} } OPTIONAL,

...

}

- DL-CodeInformationItem-RL-AdditionFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

-- ** NOTE: Shall this be made as an IE container? **

DCH-InformationResponseList-RL-AdditionFailureFDD ::= SEQUENCE (SIZE (1..maxNrOfDCHs)) OF DCH-InformationResponseItem-RL-AdditionFailureFDD

- DCH-InformationResponseItem-RL-AdditionFailureFDD ::= SEQUENCE {

dCH-ID						DCH-ID,

bindingID					BindingID,

transportLayerAddress				TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { {DCH-InformationResponseItem-RL-AdditionFailureFDD-ExtIEs} } OPTIONAL,

...

}

- DCH-InformationResponseItem-RL-AdditionFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- NeighbouringFDD-CellInformationList-RL-AdditionFailureFDD ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringFDD-CellInformationItem-RL-AdditionFailureFDD

- NeighbouringFDD-CellInformationItem-RL-AdditionFailureFDD ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-FDD-Cell-Information			Per-FDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringFDD-CellInformationItem-RL-AdditionFailureFDD-ExtIEs} } OPTIONAL,

...

}

- NeighbouringFDD-CellInformationItem-RL-AdditionFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-FDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofFDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset			OPTIONAL,

primaryScramblingCode			PrimaryScramblingCode,

primaryCPICH-Power				PrimaryCPICH-Power			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-FDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-FDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- NeighbouringTDD-CellInformationList-RL-AdditionFailureFDD ::= SEQUENCE (SIZE (0..maxnoofneighbouringRNCs)) OF

NeighbouringTDD-CellInformationItem-RL-AdditionFailureFDD

- NeighbouringTDD-CellInformationItem-RL-AdditionFailureFDD ::= SEQUENCE {

rNC-ID						RNC-ID,

cN-PS-DomainIdentifier				CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier				CN-CS-DomainIdentifier		OPTIONAL,

per-TDD-Cell-Information			Per-TDD-Cell-Information,

iE-Extensions					ProtocolExtensionContainer { {NeighbouringTDD-CellInformationItem-RL-AdditionFailureFDD-ExtIEs} } OPTIONAL,

...

}

- NeighbouringTDD-CellInformationItem-RL-AdditionFailureFDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Per-TDD-Cell-Information ::= SEQUENCE ( SIZE (1..maxnoofTDDneighboursperRNC)) OF

SEQUENCE {

c-Id							C-Id,

uARFCN							UARFCN,

frameOffset					FrameOffset		OPTIONAL,

cellParameterID				CellParameterID,

syncCase						SyncCase,

timeSlot						TimeSlot			OPTIONAL,

-- This IE is present only if SyncCase is Case1 -- ,

pSCH-TimeSlot					PSCH-TimeSlot	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Per-TDD-Cell-Information -ExtIEs} } OPTIONAL,

...

}

- Per-TDD-Cell-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

- RadioLinkAdditionFailureFDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxRateMatching								INTEGER ::= 10

maxNrOfTFCs								INTEGER ::= 10

maxNrOfTFs								INTEGER ::= 10

maxNoOfDL-Codes								INTEGER ::= 10

maxNrOfCCTrCHs								INTEGER ::= 10

maxNrOfDCHs								INTEGER ::= 10

maxNrOfDL-Codes								INTEGER ::= 10

maxNrOfDPCHs								INTEGER ::= 10

maxNrOfErrors								INTEGER ::= 10

maxNrOfFACH-FD-Size							INTEGER ::= 10

maxNrOfMACcSDU-Length							INTEGER ::= 10

maxNrOfRLs								INTEGER ::= 10

maxNrOfSCCPCHs								INTEGER ::= 10

maxRNCinURA								INTEGER ::= 10

maxTTI-Count								INTEGER ::= 10

maxnoofneighbouringRNCs							INTEGER::= 10

maxnoofFDDneighboursperRNC						INTEGER::= 10

maxnoofTDDneighboursperRNC						INTEGER::= 10