3GPP TSG-RAN WG3 #108-e	R3-206861

Online, 1-11 June 2020

Agenda Item:	20.2.1

Source:	Ericsson, Qualcomm (moderators)

Title:	CB: # 18_NTN_IDhandling - Summary of email discussion

Document for:	Approval

# Introduction

Chair’s reflections copied below

CB: # 18_NTN_IDhandling

- note LS; discuss possible reply and impact to RAN3 protocols

FH 5940

- the list of network identities encompasses gNB ID, NCGI, NCI, NPCI, TAC, TAI, NID, and CAG ID.

- The network identities TAC and TAI have been studied and two new solutions, i.e. multiple TAI and periodic registration area update have been introduced.

- decide whether the previously introduced solutions for TAC/TAI of multiple TAIs and/or periodic registration area update are valid solutions or need further study in coordination with RAN2.

CATT 6275

- Network Identifiers can be decoupled from strictly fixed (constant and exclusive mapping) on geographical areas, although they can be associated to them.

- CGI for a given PLMN should keep constant when a NTN cell is moving over the earth, unless there is any topological shift on the link, e.g. switching to another satellite, another NTN gateway, or another gNB.

- One cell broadcasts only one TAI per PLMN, and the boundary of TAs can float in transparent LEO with earth moving cell scenario.

E/// 6381

- Strive for RAN2 confirming the assumptions concerning SIB content in moving satellite cells, i.e. the necessity of broadcast all Tracking Areas corresponding to the momentary coverage area of a moving satellite beam, to cover Tracking area borders.

- Agree that it is not necessary to inform the 5GC or neighbor gNBs about the fact that Tracking Areas reported to be served by a gNB are not continuously served by that gNB.

- Agree from a RAN3 point of view, that a Cell Global Identity corresponds to a fixed geographical area. This is applicable to fixed as well to earth moving cells. Inform RAN2 and SA2 and ask for confirmation.

- Strive for RAN2 confirming the assumptions that SIB content in moving satellite cells need to correspond to the momentary coverage area of a moving satellite beam with respect to fixed geographical area definition of a Cell ID.

- Agree that in case several Cell IDs would correspond to the same geographical (cell) area, it can be assumed that neighbor gNBs and the 5GC is able to cope with such ambiguity.

- Agree that in case several Cell IDs would correspond to the same geographical (cell) area, neighbor gNBs do not need to be constantly updated by the currently applicable set of Cell-IDs.

Intel 6400

- NTN enhancements standardized in RAN3 should strive to minimize 5GC (and by extension – NG-AP) impacts.

- Rule out solution 4 (CN receives “cell ID” and geographical coordinates).

- NTN does not require changes to the “User Location Information” NG-AP IE.

Chair: Consensus that “autonomous” periodic reconfiguration (OAM configures once and for all, then RAN nodes periodically applies config changes) seems better than signaling-assisted (e.g. periodic Xn signaling exchange)? If so, attempt st2 TP

(E///,QC - moderator)

Summary of offline disc R3-206861

# For the Chairman’s Notes [to be confirmed after discussions]

Propose to capture the following:

Issue 1 - Cell ID maps to a fixed geographical area:

It is agreed that a Cell ID corresponds to a fixed geographical area.

Issue 2 - minimizing 5GC/NGAP impact:

WA: RAN3 strives for minimizing NGAP impact for NTN. [probably with additions along discussions below]

Issue 3 - consequences of network identifiers mapped to fixed geographical areas:

to be added.

Issue 4 - avoid periodic reconfiguration: agree on the following

If solutions to support fixed geographical areas for TAI/Cell ID result in TAIs/Cell IDs being served only part-time by a gNB, this should not result in periodic configuration update on NG/Xn. gNBs are able to apply “autonomous” reconfiguration by means of OAM.

# Discussion

## Cell ID maps to a fixed geographical area (applying the same decision as for TA)

Given the SA2 LS in R3-206842 and input papers to RAN3#110-e, it appears to be straight forward  to confirm SA2’s assumption. Please provide your view on that approach.

| Company | Comment |
| --- | --- |

## 5GC impact: strive for minimizing 5GC/NGAP impact

This topic has several aspects: impact on ULI, additional provision of more granular location information and maybe others. Please provide your view, and, if applicable, add aspects and your view.

| Company | Comment |
| --- | --- |

## Consequences of network identifiers mapped to fixed geographical areas.

In order to operate an NTN, basic network identifiers (PLMN ID, TAI, Cell ID) as used on network interfaces (e.g. served cells on Xn, served TAs on NG), Uu (e.g. PLMN index in msg5), in broadcast content and in UE procedures (e.g. ULI on NG) need to have the same characteristics and follow the same assumption, especially their relation to fixed geographically represent.

a)	One possible approach would be to assume that SIB content corresponds to momentary coverage area of a beam related to the geographically fixed areas of PLMNs/TAs/Cells - irrespective of whether the beam is fixed or moving. We could ask RAN2 to confirm this assumption.

b) Another possible approach would be discuss de-coupling (but associating) network identifiers from its fixed geographical relation (6275, CATT).

Please provide your view.

| Company | Comment |
| --- | --- |

## Impact on Xn/NG management procedures - avoid periodic re-configuration.

Solutions for fixed geographical areas for TAI/Cell ID may result in gNBs not always serving the same, constant set of TAIs/Cells at all time. In that case Xn and NG exchange of configuration information (Setup/Configuration Update) shall not reflect the momentary served set of Identities but rather the super-set of identities, avoiding periodic re-configuration. This would follow the assumption that gNBs are able to apply “autonomous” reconfiguration by means of OAM.

Please provide your view.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-206842 Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (SA WG2) LS in

- R3-205940 NR-NTN: Network Identities Handling (Fraunhofer IIS, Fraunhofer HHI) discussion

- R3-206275 Discussion on identifier handling in NTN (CATT) discussion

- R3-206381 Network Identifier Handling for moving cells (Ericsson) discussion

- R3-206400 On Cell id in User Location Information (Intel Corporation) discussion

- R3-206261 Handling of cell identity signalling towards the CN (Qualcomm Incorporated) discussion

- R3-206262 [DRAFT] Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (Qualcomm Incorporated) LS out

- R3-206263 Support of NTN RAT identification and NTN RAT restrictions (Qualcomm Incorporated) CR0490r, TS 38.413 v16.3.0, Rel-17, Cat. B

- R3-206264 Support of NTN RAT identification and NTN RAT restrictions (Qualcomm Incorporated) CR0488r, TS 38.423 v16.3.0, Rel-17, Cat. B