3GPP TSG-RAN WG3 Meeting #111-e	R3-210658

E-meeting, 25 January – 4 February, 2021

Agenda item:	15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	QoE measurement collection continuity vs. reporting continuity

Document for:	Text Proposal

# 1	Introduction

Several open points for NR QMC (QoE Measurement Collection) framework were captured in RAN3#110-e chairman's minutes and in TR 38.890. In this paper we discuss some of the open points relating to reporting aspects, and in particular the need for the RRC layer to provide QoE measurement reporting continuity for the life cycle of the application session.

In section 2.3 of this paper we also look further into a possible need for clarification or correction of the signalling-based QMC deactivation procedure.

# 2	Discussion

## 2.1	Current requirement status

TR 38.890 v0.2.0 clause 6.6 "Support of mobility" contains the following editor's notes which all have a link with QoE measurement reporting continuity (we have added numbers used for identification further down in this text):

Editor's NOTE 1: Management-based activation to be further checked.

Editor's NOTE 2: the solutions enabling the fulfilment of the SA4 QoE requirements are FFS.

Editor's NOTE 3: FFS whether inter-RAT and/or inter-system mobility for QoE measurements should be supported.

Editor's NOTE 4: FFS whether, and under which conditions, the target node may decide the subsequent handling of management based QoE configuration.

In order to handle these open points, we aim to further analyse whether and how to ensure measurement reporting continuity, taking into account that QMC is a feature that will only provide measurement reports to the network if the UE's application layer and RRC layer are aligned. So far, the discussion in RAN3 only took connected mode handover scenarios into account, but RRC state transitions, in particular from RRC_IDLE to RRC_CONNECTED, are also highly relevant for this discussion. It should be noted that the application is unaware of RRC states, and when an application session is configured with QMC measurements this configuration remains active for the duration of the application session.

For QMC for MTSI (Multimedia Telephony Service for IMS), TS 26.114 provides the following description for the application layer (clause 16.3):

"The QoE configuration shall only be checked by the client when each session starts, and thus all logging and reporting criterias for an ongoing session shall be unaffected by any QoE configuration changes received during that session. This also includes evaluation of any filtering criterias, such as geographical filtering, which shall only be done when the session starts. Thus changes to the QoE configuration will only affect sessions started after these configuration changes have been received."

TS 26.114 describes two methods of geographical filtering. The first method is based on network evaluation of geographical criteria (area scope), where the UE is informed about the outcome based on a Within-area indication which informs whether the UE is inside or outside the wanted geographic area. The Within-area indication is not supported in Rel-16 RAN specifications. This method is described in TS 26.114 clause 16.5.1 as follows: "If the MTSI client is informed that it is not inside the area, it shall not start any new QoE measurements even if it has received a valid QoE configuration container, but shall continue measuring for already started sessions."

The second method is based on an XML-encoded LocationFilter evaluated by the UE, and this filter indicates the geographic area(s) or location(s) where quality metric collection is requested (any combination of targeted cell-IDs, polygons and circular areas, see TS 26.114 clause 16.3.1 for details). It can be understood from the text previously cited from TS 26.114 clause 16.3 that the UE will evaluate this location filter only when the MTSI client session starts. This second method (UE-based) seems different from the network-based method in the sense that TS 26.114 doesn't explicitly state that the MTSI client shall continue measuring in case of already started sessions  when the UE is not inside the area. It therefore seems  that  ongoing application sessions may take the LocationFilter into account, but any changes to the QoE configuration will only affect sessions started after these configuration changes have been received. The following statement from SA5 therefore doesn't seem to apply to SA4's UE-based method: "There is also the requirement that the measurements shall continue until the application session stops even if the UE goes outside the specified area." (LS in S5-205347). It also seems difficult to deduce any rationale for such requirement from functional point of view, for both methods (WithinArea, LocationFilter).

Observation 1: There is no explicit requirement that measurements shall continue if the UE goes outside the area indicated by the LocationFilter (TS 26.114 clause 16.3.1), and such requirement would also lack rationale from functional point of view.

A consequence of this observation is that a clarification is needed in clause 6.6 of the TR which currently states:

"In addition, the SA4 requirements for QoE measurements stipulate that the client shall check the QoE configuration only when a session starts. This means that the client shall continue the QoE measurements for an ongoing session even if the UE moves out of the configured area. The SA4 requirements are RAT-independent and shall therefore be applied to the mobility solution for QoE measurement in NR, as well."

We include in annex TP with such clarification, including the scope of the SA4 requirements (application layer) and details related to the WithinArea vs. LocationFilter methods.

## 2.2	Possible way forward for Rel-17 solution

Editor's note 2 is a reminder to further work on fulfilment of SA4's requirements for QoE measurement continuity. RAN3's main focus in this respect has so far been on the impact of mobility (handovers) on the observed QoE, as can be seen in the TR (clause 6.6):

Seamless mobility is a key functionality in NR and its impacts should be measurable at the application layer. To enable measuring the impact of the mobility on the application and users’ QoE, it is required to support QoE measurements in mobility scenarios at least for signalling based QoE.

Measurement reporting continuity in case of transition via RRC_IDLE state has not been discussed so far within the present study item. It is expected that the life cycle of an application client session is not limited to the time duration while UE stays in RRC_CONNECTED mode (e.g. a video streaming application session is presumed kept alive while streaming is paused by the user, however the UE RRC layer may transit to idle mode until resumption of the streaming). Taking into account that existing RRC signalling for LTE QMC configuration mandatorily combines application layer configuration and RRC layer configuration, we believe that RAN2 should confirm whether this signalling can be used to reactivate reporting of QoE measurements from an existing application session when the UE re-enters RRC_CONNECTED state without necessarily triggering new application layer configuration. This confirmation is required to ensure that QoE measurement reporting continues until the application session stops, provided RRC state transitions via idle mode is considered for the QoE monitoring. For some important services for NR QMC monitoring, e.g. based on URLLC, the UE might not transit to idle mode during the life cycle of the application client session, and this might explain focus on mobility scenarios in TS 28.405 but no scenario showing transit via RRC IDLE or RRC INACTIVE.

Observation 2: Existing RRC signalling for LTE QMC configuration mandatorily combines application layer configuration and RRC layer configuration, RAN2 should therefore confirm whether it can be used to reactivate reporting of QoE measurements from an existing application session when the UE re-enters RRC_CONNECTED state without necessarily triggering new application layer configuration.

From network point of view, support for reactivation of QoE measurement reporting can, if this scenario is required, be ensured by signalling based activation, i.e. by keeping the QoE measurement configuration context in the AMF when the UE is in idle mode. Final confirmation on whether and how to resume QoE measurement reporting at the RRC layer for an ongoing application client session when the UE transits from RRC_IDLE to RRC_CONNECTED state may be done in normative phase.

Proposal 1: Rel-17 support for measurement reporting continuity handling following RRC reconnection after idle mode transition is assumed supported by signalling based activation, and can be finally confirmed in normative phase based on feedback from RAN2.

When it comes to measurement reporting continuity in case of mobility, a solution based on signalling based activation will ensure this both in case of intra-node and inter-node mobility. A solution based on legacy mechanism for management-based activation will provide such continuity for intra-node mobility. Taking into account the possibility to rely on signalling-based activation, we believe that RAN specifications based on the legacy MDT solution can be considered to provide sufficient support of SA4's requirements.

Proposal 2: QoE measurement reporting continuity in case of inter-node handover is ensured based on signalling-based activation.

In annex we provide update of the TR reflecting proposals 1 and 2, and by this solving Editor's notes 1, 2 and 4. Decision related to Editor's note 3 (inter-RAT and/or inter-system mobility) may be taken in normative phase, and we therefore include this note into the TR text.

## 2.3	Clarification of deactivation procedures

In clause 6.1.2 "Signalling-based deactivation procedures”, there is a mention: “The NG-RAN node checks whether there is an ongoing session pertaining to the QMC (according to the SA4 requirements)”

Our main concern with this statement is that, as per our understanding of SA4's specification, once configured for QoE an application client session can’t be reconfigured or de-configured. Also, in fig. 6.1.2-1, this check is depicted under the UE APP, and not under the NG-RAN. A possible solution could be to remove this statement from the TR text and the figure.

Also, any reference to SA4 requirements should have an explicit reference to SA4 specification (e.g. in clause 6.2.2 for management based deactivation)..

Proposal 3: Discuss and correct statement on "ongoing session pertaining to the QMC" for deactivation procedures, including explicit reference to SA4 requirement.

# 3	Conclusion

We have made the following observations and proposals:

Observation 1: There is no explicit requirement that measurements shall continue if the UE goes outside the area indicated by the LocationFilter (TS 26.114 clause 16.3.1), and such requirement would also lack rationale from functional point of view.

Observation 2: Existing RRC signalling for LTE QMC configuration mandatorily combines application layer configuration and RRC layer configuration, RAN2 should therefore confirm whether it can be used to reactivate reporting of QoE measurements from an existing application client session when the UE re-enters RRC_CONNECTED state without necessarily triggering new application layer QoE configuration.

Proposal 1: Rel-17 support for measurement reporting continuity handling following RRC reconnection after idle mode transition is assumed supported by signalling based activation, and can be finally confirmed in normative phase based on feedback from RAN2.

Proposal 2: QoE measurement reporting continuity in case of inter-node handover is ensured based on signalling-based activation.

Proposal 3: Discuss and correct statement on "ongoing session pertaining to the QMC" for deactivation procedures, including explicit reference to SA4 requirement.

# Annex	- TP for TR 38.890 v0.2.0

<<< start of changes >>>

## 6.4 	Release of QoE measurement configuration

An NG-RAN node can issue a release of QoE measurement configuration for UEs previously configured for QoE measurement reporting, provided that the session for which the QoE measurements are reported is completed.

Editor’s NOTE: FFS whether RAN is allowed to release an ongoing QoE measurement reporting; to be confirmed by RAN2.

QoE measurement reporting at the RRC layer is released when the UE transits to RRC_IDLE. For e.g. URLLC services it may be assumed that the UE remains in RRC_CONNECTED state for the full duration of the application client session. Whether and how to resume QoE measurement reporting at the RRC layer for an ongoing application client session when the UE transits from RRC_IDLE to RRC_CONNECTED state may be confirmed in normative phase.

<<< next change >>>

## 6.6 	Support for Mobility

Seamless mobility is a key functionality in NR and its impacts should be measurable at the application layer. To enable measuring the impact of the mobility on the application and users’ QoE, it is required to support QoE measurements in intra-node and inter-node mobility scenarios for signalling based QoE, and intra-node mobility scenarios for management based QoE.

In LTE, to support the QoE measurement in mobility scenarios, the QoE configuration is forwarded from the source eNB to the target eNB as part of Trace Activation IE over X2 interface. The same IE is sent over S1 interfaces for mobility scenarios when the X2 interface is not established between the source and target.

In NR, to support mobility for QoE measurements in CONNECTED state, the QoE measurement configuration transfer is supported on the Xn and NG interfaces, inside the Trace Activation IE. To support keeping QoE measurement configuration in INACTIVE state mobility, QoE measurement configuration for a UE can be fetched from the node hosting the UE Context. The RRC procedure for resumption of QoE measurement reporting at the RRC layer for an ongoing application client session when the UE transits from INACTIVE state to CONNECTED state may be confirmed by RAN2 in normative phase.

In addition, the SA4 requirements for QoE measurements stipulate that the client shall check the QoE configuration only when a session starts. The application client shall continue the QoE measurements for an ongoing session even if the UE moves out of the configured area as notified by the WithinArea signalling, but similar requirement is not captured for the LocationFilter method described in TS 26.114 clause 16.3.1. The SA4 requirements apply to the application layer (RAT-independent) and shall also be taken into account for the mobility solution for QoE measurement in NR.

Whether inter-RAT and/or inter-system mobility for QoE measurements should be supported will be determined in normative phase.

<<< end of changes >>>