3GPP TSG-RAN WG3 #111-e	R3-210992

Online, 25 Jan – 05 Feb 2020

Agenda Item:	10.2.1.5

Source:	Nokia (moderator), Nokia Shanghai Bell

Title:	Summary of Offline Discussion on Load Balancing Enhancements for SON (CB #1004)

Document for:	Approval

# Introduction

CB: # 1004_SONMDT_LoadBalancing

- PRBs utilization per slice?

- NUL and SUL reporting?

- Reporting of load information of potential target PSCell from other eNBs in EN-DC?

- Reporting of load of the initial BWP separately from the overall load?

- Information of RRM policy for slicing reporting?

- BWP load reporting?

- Slice specific offset in MSC exchange reporting?

- Per-SSB offset in Mobility Setting Change reporting?

- Reporting of per-cell list of cell identifiers of cells that can be used for resource aggregation

- For reporting over Xn, the lowest TNL Information value (in terms of available capacity) between F1 and NG interfaces is to be reported?

- Which interfaces are applicable when a gNB provides TNL Load Information over Xn and X2?

- May also discuss other issues based on papers submitted

- Try to reach high-level agreements in the first phase, proceed to TPs in the second phase of the email discussion

(Nok - moderator)

Summary of offline disc R3-210992

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## Per-slice resource reporting

At the last meeting, it was agreed to report resource utilisation per slice – but details were left FFS. Now, companies propose a solution for reporting per-slice resource utilisation:

- In a contribution co-signed by numerous companies, including 3 operators, it is proposed to report momentary PRB utilisation (GBR & nGBR) per slice [1-3].

- An operator makes a similar proposal as in (1), but proposes to provide information on the slice policy (shared and dedicated PRBs) – in [16-18].

- Alternatively, it is proposed to report PRB utilisation as percentage of the total available quota, but without exposing the quota (RRM policy) – in [13-14].

Please, provide your comment on the proposal 1 above.

| Company | Comment |
| --- | --- |
| Nokia | Yes, we support this solution (we co-authored the proposal). |

Please, provide your comment on the proposal 2 above.

| Company | Comment |
| --- | --- |
| Nokia | This may be interesting extension to proposal 1 above, but we’d rather gone for the simple solution as in proposal 1 and only later extend it. |

Please, provide your comment on the alternative in proposal 3 above.

| Company | Comment |
| --- | --- |
| Nokia | No, this will not help. Percentage in case of nGBR traffic will likely be most of the time around 100%. Without the information on the RRM policy, this will offer no help. Explicit PRB utilisation, as proposed in 1 above, can help even without revealing RRM policy if used together with the available capacity and compared among all reported slices. |

## NUL and SUL reporting

This was discussed already at the last meeting, but not decided. Several companies propose to support separate load information for SUL:

- In [5-7,12,15], it is porposed to report separately load for SUL.

- In [4], in addition to SUL, also separate load reporting for NUL is proposed.

Please, provide your comment on the proposal 1 above.

| Company | Comment |
| --- | --- |
| Nokia | No, this is no needed. As already discussed, SUL load has hardly any impact on load balancing. |

Please, provide your comment on the extension in proposal 2 above.

| Company | Comment |
| --- | --- |
| Nokia | Not needed. |

## Enhancements to the mobility setting change

Two companies propose amendments to the Mobility Setting Change procedure:

- In [12], mobility setting change per slice is proposed.

- In [13-14], mobility setting change per beam is proposed.

Please, provide your comment on the proposal 1 above:

| Company | Comment |
| --- | --- |
| Nokia | Yes, we are interested to discuss it further. This is another flavour of per-UE-group mobility setting negotiations. This topic was discussed long time ago and then decided to be unnecessary because the source node has enough information to set HO threshold per each UE type. Nonetheless, we could review if anything has changed since early LTE time in this respect. |

Please, provide your comment on the proposal 2 above:

| Company | Comment |
| --- | --- |
| Nokia | No sure if this will work. The UE may report the SSB in the measurement report, but the trigger is common for the whole target cell. So, the source has to schedule several reports from the UE to “catch” the one that may possibly lead to a HO. Also, the purpose is questionable: the node controlling the beams may internally manage the load so per-beam negotiations with neighbours will not be needed. |

## Reporting the load of possible PSCell

One company proposes (continuation from the last meeting) that a node should be enabled to obtain load level at possible SNs (PSCells) connected to a neighbour node [5-7].

Please, provide your comment on the proposal above:

| Company | Comment |
| --- | --- |
| Nokia | Not needed. This can be included in the composite available capacity. This is much simpler than including load of any possibly usable PSCell (DC operation is per UE, so there may be many possible SNs for each UE). |

## Separate load information on the initial BWP

One company proposes to continue the discussion on the load reporting per BWP [11]. This time, it is proposed to report load for the initial BWP only.

Please, provide your comment on the above proposal.

| Company | Comment |
| --- | --- |
| Nokia | Yes, we proposed the solution. This proposal is modified based on the comments at previous meetings. Indeed, for neighbour nodes, only the load at the initial BWP is relevant. This load, depending on the UEs, may be much higher than the overall load of the cell and thus MLB HOs may fail, even if there is capacity in other BWPs. |

## Reporting opportunities for resource aggregation

One company proposes to extend the signalling so that neighbours could understand that certain cells may work in aggregated configuration thus allowing for aggregated capacity [13-14].

Please, provide your comment on the above proposal.

| Company | Comment |
| --- | --- |
| Nokia | This is interesting, but the gain for load balancing is unclear: resource aggregation is RRM policy, so may not be static enough to allow for MLB actions, while static information on “possible” aggregation tell nothing about the situation when MLB is needed. |

## Further clarification of the TNL load information

One company proposes to clarify further that the TNL load information used on X2 and Xn is based on load reported over the NG and F1 interfaces, thus that E1 is rather irrelevant for the TNL load [8-10].

Please, provide your comment on the above proposal.

| Company | Comment |
| --- | --- |
| Nokia | Yes, this could complete the discussion started at the past meetings. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-210148, Simple information on PRB utilisation per slice (Nokia, Nokia Shanghai Bell, ZTE, Deutsche Telekom, British Telecom, China Telecom)

- R3-210149, (TP to TS 38.473, NR_ENDC_SON_MDT_enh-Core) Adding slice to the load information (Nokia, Nokia Shanghai Bell, ZTE, Deutsche Telekom, British Telecom, China Telecom)

- R3-210288, (TP for SON BL CR 38.423) Adding slice to the load information (ZTE, Nokia, Nokia Shanghai Bell, Deutsche Telekom, British Telecom, China Telecom)

- R3-210242, Open issues on load balancing enhancement: SUL load (NEC)

- R3-210306, (TP on SON for 36.423) Discussion on PSCell MLB and SUL PRB usage (CATT)

- R3-210307, (TP on SON for 38.423) Support of SUL PRB usage (CATT)

- R3-210308, (TP on SON for 38.473) Support of SUL PRB usage (CATT)

- R3-210318, Further discussion on the TNL load information (Nokia, Nokia Shanghai Bell)

- R3-210319, (TP to TS 38.423, NR_ENDC_SON_MDT_enh-Core) Further clarification of the TNL load (Nokia, Nokia Shanghai Bell)

- R3-210320, (TP to TS 36.423, NR_ENDC_SON_MDT_enh-Core) Further clarification of the TNL load (Nokia, Nokia Shanghai Bell)

- R3-210321, Load information enhancements for BWP (Nokia, Nokia Shanghai Bell)

- R3-210394, (TP for SON BLCR for 38.423) Load Balancing Enhancements (Huawei)

- R3-210674, (TP for SON BL CR for TS 38.423): MLB enhancements (Ericsson)

- R3-210675, (TP for SON BL CR for TS 36.423): MLB enhancements (Ericsson)

- R3-210780, Issue on load balancing enhancement (LG Electronics)

- R3-210894, Slice PRB related load metric (CMCC)

- R3-210895, TP to SON BLCR 38.423 on slice PRB (CMCC)

- R3-210896, TP to SON BLCR 38.473 on slice PRB (CMCC)